IBM 3270 Credit Card (CC) support code

--------------------------------------

This code is intended for TEST use only and is not representative

of "ship level" software.   Since various machines have different

requirements, such as interrupts and memory allocation, all

environments may not be supported.

The following files are contained on this disk:

IBM 3270 Credit Card Point Enablers

-----------------------------------

These following files are vendor PC specific in nature.  They are

intended to provide a means of configuring or "enabling" the various

hardware PCMCIA chips which control the PCMCIA sockets.  When PCMCIA

Card Services become available in the near future, these enablers will

not be required.

INTL3270.EXE  -  Point Enabler for the Intel 82365 PCIC

SHRP3270.EXE  -  Point Enabler for the Sharp PCMCIA Interface (monochrome PC)

SHRPCOL.EXE   -  Point Enabler for the Sharp PCMCIA Interface (color PC)

TOSH3270.EXE  -  Point Enabler for the Toshiba PCMCIA Interface

Diagnostic Program

------------------

This file will test and exercise the IBM 3270 Credit Card PCMCIA

Adapter hardware.  It will test the card AFTER the PC has been enabled

either by one of the above programs or another user written program.

(This program is the same program that comes with the IBM 3270 Credit

Card package.)

TEST3270.EXE   -  3270 CC Diagnostic V1.0

Card Information Structure

--------------------------

This is the data that is contained in the Flash RAM on the 3270 CC.

This file is included for informational purposes only.  There is no need

to process or manipulate this file.

ATTRRAM1.LST  -  3270 CIS Data (Card Information Structure Data)

***************************************************************************

To attach to a host system, the following procedure should be used.

1. Insert the IBM 3270 Credit Card into the PC's

PCMCIA socket.  The current environment does

not support insertion or removal of the card

once operation has begun.

2. From the DOS prompt or the AUTOEXEC.BAT file,

enable the PCMCIA port by executing the appropriate

software such as:   INTL3270

If the port is not enabled, enter INTL3270 ? for

a list of possible options and their default values.

3. Load and begin your emulator (not included on

this disk.  The IBM Personal Communications/3270

emulator has been tested in both DOS mode and

Windows mode.  IBM OS/2 is not supported at this

time.

Additional Notes

----------------

All of the Point Enablers will attempt to enable the 3270 Credit

Card with Socket Services Version 1.01 first.  If Socket Services

Version 1.01 is not present then the Point Enabler will attempt to

enable the card directly.

***************************************************************************

To test the card with the Diagnostic Program, the following sample procedure should be used:

1. Insert the IBM 3270 Credit Card into the PC's

PCMCIA socket.  The current environment does

not support insertion or removal of the card

once operation has begun.

2. Exit OS/2 or Windows operating systems and

return to the DOS prompt.  Make sure that a 3270

terminal emulator (PC/3270) is not resident in memory.

3. Enable the PCMCIA port by executing the appropriate

software such as:   INTL3270

If the port is not enabled, enter INTL3270 ? for

a list of possible options and their default values.

4. Enter TEST3270 and follow the on screen instructions.

(If options other than the defaults were used to enable the card,

then they need to be specified again when running the Diagnostic Program.

Enter TEST3270 ? for a list of possible options and their default values.)

Additional Notes

----------------

The Diagnostic Program will attempt to enable the 3270 Credit

Card with Socket Services Version 1.01.

If Socket Services Version 1.01 is not present then the Diagnostic Program

will look for a previously enabled 3270 Credit Card (one that was enabled

by a Point Enabler.)

***************************************************************************

Questions?

----------

Contact Bruce Armstrong at IBM in Research Triangle Park, NC.

Phone:.........919-254-2385

FAX:...........919-254-2455

Internet ID:...BRUCEA@RALVM29.VNET.IBM.COM