CHAPTER 1

System Description

###### ATC5200M Mainboard with Socket 7 on board supports 66~100MHz host clock, and is intended to run Intel Pentium CPUs up to 233MHz, AMD K5/K6/K6-2 CPUs, and Cyrix 6x86/6x86MX / M II CPUs.  With  VIA MVP3 chipset on board, it also supports AGP and Ultra DMA  performance for  fast VGA display and data transfer. Below are detailed description of its system specifications and features.

1-1 SYSTEM SPECIFICATION

- Supports Intel Pentium (P54C) CPUS up to 200MHz, Intel Pentium MMX 166~233MHz (P55C), AMD K5, AMD K6, AMD K6-2, Cyrix 6x86MX, MII, 6x86L, IDT Win Chip C6 CPUs.

- Supports CPU Bus Clock from 66MHz to 100MHz and Ratio from 1.5x to 5.5x

- VIA @ VT82C598MVP & VT82C586B (with keyboard control) chipset.

- Built-in Switching Voltage Regulator.(VRM 8.2 SPEC.)

- CPU core voltage support: 2.2V~3.5V; CPU/IO Voltage:3.40V

- Three 168-pin DIMM sockets on board, supporting three banks of 64-bit wide path up to 384MB SDRAM or 768 EDO DRAM (with parity chip ECC support).

- L2 Cache: 512KB, 3.3V

- AGP slot on board; three PCI slots with revision 2.1 interface compliance and two 16-bit ISA slots.

- Dual Master IDE connectors support Ultra DMA/33, up to four devices in two channels for connecting high capacity hard drive, CD-ROM, tape backup etc.

- One USB (Universal Serial Bus) header supporting up to 127 devices.

- AT style keyboard connector and PS/2 mouse header.

- WINBOND W83877TF high-speed Super Multi-I/O chipset.

- Infrared transfer (IrDA TX/RX) Support.

- One FDD port supports following 2 floppy drives: 1.2MB/1.44MB/ 2.88MB.

- Two 16550A fast UARTs compatible serial ports; One DIN connector for KB

- AT & ATX power connectors on board.

- Hardware Dimension :220mm x 230mm (8.66" x 9.0"); BabyAT Form Factor.

1-2  FEATURES HIGHLIGHT

1-2-1  Software Power Off Control (under ATX power supply)

This mainboard supports Software Power Off Control feature through the SMM  code in the BIOS under Windows 95/98, and MS-DOS operation system environment.  It needs ATX power supply to support this performance.

First, you should connect the power switch cable to the connector “PS-ON” on the mainboard. In the BIOS screen of POWER MANAGEMENT SETUP’, choose “User Defined” (or “Min. Power Saving” or “Max. Power Saving”) in ‘Power Manager’ and choose “Yes” in ‘PM Control by APM’.

In Windows 95/98, if you want to power off the system, you just choose “shutdown the computer ?” in the “Shut Down Windows“ from Windows 95/98, then the system power will be off directly and stay at the stand-by status. If you want to restart the system, just press the power switch button, and the system will be powered on.

In case you leave your system idle for several days, you better use “Hardware Power Off” to shut down your system.

1-2-2  AGP (Accelerated Graphics Port) Interface

AGP gives graphics accelerators fast, high throughput direct access to system memory. This allows a graphics accelerator access to more memory than available locally on the graphics card. For example, to display a scene that contains 16MB of textures, a 4MB AGP graphics card could access the PC system memory for the additional 12MB required.

- AGP delivers a peak bandwidth that is 4 times higher than the PCI bus using pipelining, sideband addressing, and more data transfers per clock.

- The current PCI bus supports a data transfer rate up to 132 MB/s, while AGP (at 66MHz) supports up to 533 MB/s!

CHAPTER 2

INSTALLATION

2-1 MAINBOARD INSTALLATION

Before installing the computer, please get ready all components such as CPU, DRAM; peripherals(such as hard drive, keyboard, CD-ROM) and accessories(such as cables, jumper caps, cooling fans, screw driver)etc. Then, install the system as below:

- Plug CPU, heat sink, cooling fan and DRAM modules onto the 5200M mainboard.

2. Set DIP switch based on your CPU configuration.

3. Set jumpers based on your CPU and BIOS configuration.

4. Plug add-on cards into PCI/ISA slots, if needed.

5. Connect the power supply.

6. Connect I/O and other cables to the system.

7. Make sure all components and devices are well connected, turn on the power and

setup System BIOS based on your configuration.

8. Install peripheral devices, add-on card drivers and test them.

If all of above procedures are running successfully, turn it off and screw the chassis cover to the system, and then connect external devices which are cabled to the system.

2-2 CPU CONFIGURATION

5200M Mainboard supports Pentium level CPUs.

To install CPU, please notice CPU pin 1 must align with the ZIF socket 7 pin 1 location. To configure your CPU, please follow the CPU Guide provided by CPU manufacturer to configure the CPU voltage setting and then set up DIP Switch & Jumpers on 5200M Mainboard to suit your CPU type.

###### It is very important you must set up DIP switch SW1, Jumpers JP2, JP3, JP4, JP5 accurately for a normal and stable CPU operation.

To locate these jumpers and SW1, please turn to Page A “Component Location Diagram”.

The following are illustrations of SW1 setting, as well as JP2,3,4, 5 settings.

SW1 setting: for CPU base frequency and CPU Frequency Ratio

CPU Frequency Ratio Settings         CPU Base Frequency Settings

| SW1 | SW1-1 | SW1-2 | SW1-3 | SW1 | SW1-4 | SW1-5 | SW1-6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.0X | OFF | OFF | ON | 66.6MHz | ON | ON | OFF |
| 2.5X | OFF | ON | ON | 75MHz | OFF | ON | ON |
| 3.0X | OFF | ON | OFF | 83.5MHz | OFF | ON | OFF |
| 1.5X;3.5X | OFF | OFF | OFF | 95.25MHz | OFF | OFF | ON |
| 4.0X | ON | OFF | ON | 100MHz | OFF | OFF | OFF |
| 4.5X | ON | ON | ON |  |  |  |  |
| 5.0X | ON | ON | OFF |  |  |  |  |
| 5.5X | ON | OFF | OFF |  |  |  |  |

How to set up DIP switch SW1:

SW1 Settings Illustration:

DIP switch, (i.e. : OFF, OFF, OFF, ON, ON, OFF, OFF)

ON                 DIP

Off

1   2   3   4   5   6   7

#### JP2 & JP3 Setting:

(The setting for frequency-ratio comparing with the PCI clock and the CPU host clock.)

| CPU_CLK/PCI_CLK | JP 2 | JP 3 |
| --- | --- | --- |
| 66.6MHz/33.3MHz | Pin 2-3 | Pin 2-3 |
| 75MHz/30MHz | Pin 1-2 | Pin 1-2 |
| 83.3MHz/33.3MHz | Pin 1-2 | Pin 1-2 |
| 95.25MHz/31.7MHz | Pin 1-2 | Pin 2-3 |
| 100MHz/33.3MHz | Pin 1-2 | Pin 2-3 |

JP4 Setting: For CPU Core voltage

The setting for CPU Ccore Voltage Selection

| Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.2V | Open | Closed | Open | Open |
| 2.3V | Closed | Closed | Open | Open |
| 2.7V | Closed | Closed | Closed | Open |
| 2.8V | Open | Open | Open | Closed |
| 2.9V | Closed | Open | Open | Closed |
| 3.2V | Open | Open | Closed | Closed |
| 3.3V | Closed | Open | Closed | Closed |
| 3.5V | Closed | Closed | Closed | Closed |

JP5 setting

(The setting for Single/Dual Voltage supporting for Pentium level Processor)

| Mode | JP5 (Pin1-2,3-4,5-6) |
| --- | --- |
| Single Voltage Mode(Vcore = Vi/o)* | Open |
| Dual Voltage Mode(Vcore ≠Vi/o)** | Closed |

* P54C mode (Single Voltage Mode)	: supports for Intel Pentium, AMD-

K5,Cyrix 6X86, IDT Win Chip C6.

** P55C mode (Dual Voltage Mode)	: supports for Intel Pentium MMX,

AMD-K6, AMD-K6-2, Cyrix 6X86L,

Cyrix 6X86MX, Cyrix M II.

how to set up jumpers:

1  2  3

1  2  3

Open       Closed

( remove the      ( put on the           Pin1-2              Pin2-3

jumper cap)      jumper cap)

2-2-1 CPU TYPE vs Jumper Setting Illustrations:

- INTEL PENTIUM CPU (P54C)

- P54C VRE : 3.400V~3.600V (CPU Voltage is marked on the back side

of the CPU itself.  V = VRE, S = standard. )

JP4(3.5V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 3.5V | Closed | Closed | Closed | Closed |

- ※ P54C STD : 3.135V ~ 3.600V  (CPU Voltage is marked on the back side

of the CPU itself.  V = VRE, S = standard. )

JP4 (3.3V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 3.3V | Closed | Open | Closed | Closed |

JP5 Setting (Single)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Single Voltage Mode(Vcore = Vi/o)* | Open |

JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP 1 | JP 2 | JP3 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 2-3 | Pin 2-3 | Pin 2-3 |

A-1.  Intel Pentium 100MHz

| CPU Clock | SW1 | Base x Ratio. |
| --- | --- | --- |
| 100MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 1.5 |

A-2.  Intel Pentium 133MHz

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 133MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 2.0 |

A-3.  Intel Pentium 166MHz

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 166MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 2.5 |

A-4.  Intel Pentium 200MHz

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 200MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 3.0 |

B. INTEL PENTIUM MMXTM CPU (P55C)

JP4 (2.8V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.8V | Open | Open | Open | Closed |

JP5 Setting (Dual)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | Closed |

JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP1 | JP2 | JP3 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 2-3 | Pin 2-3 | Pin 2-3 |

B-1.  Intel Pentium MMXTM 166MHz

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 166MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 2.5 |

B-2.  Intel Pentium MMXTM 200MHz

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 200MHz | ON ###### OFF 1 2 3 4 5 6 7 | 66 x 3.0 |

B-3.  Intel Pentium MMXTM 233MHz

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 233MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 3.5 |

C. Cyrix 6x86MX CPU

JP4 (2.9V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.9V | Closed | Open | Open | Closed |

JP5 Setting (Dual)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | Closed |

JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP1 | JP2 | JP3 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 2-3 | Pin 2-3 | Pin 2-3 |

C-1.  Cyrix  6x86MX PR166 @ 66MHz Bus 2x

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| PR166 | ON OFF 1 2 3 4 5 6 7 | 66 x 2.0 |

C-2.  Cyrix  6x86MX PR200 @ 66MHz Bus 2.5x

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| PR200 | ON OFF 1 2 3 4 5 6 7 | 66 x 2.5 |

C-3.  Cyrix  6x86MX PR233 @ 66MHz Bus 3x

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| PR233 | ON OFF 1 2 3 4 5 6 7 | 66 x 3.0 |

C-4.  Cyrix  6x86MX PR233 @ 75MHz Bus 2.5x

JP4 (2.9V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.9V | Closed | Open | Open | Closed |

JP5 Setting (Dual)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | Closed |

JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP1 | JP2 | JP3 |
| --- | --- | --- | --- |
| 75MHz/75MHz/30MHz | Pin 2-3 | Pin 1-2 | Pin 1-2 |

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| PR233 | ON OFF 1 2 3 4 5 6 7 | 75 x 2.5 |

D.  Cyrix MII CPU

D-1  Cyrix MII 300MHz @ 66MHz Bus 3.5x

JP4(2.9V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.9V | Closed | Open | Open | Closed |

#### JP5 Setting (Dual)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | Closed |

#### JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP1 | JP2 | JP3 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 2-3 | Pin 2-3 | Pin 2-3 |

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| MII 300MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 3.5 |

D-2  Cyrix MII 300MHz @ 75MHz Bus 3x

JP4(2.9V)

| #### JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.9V | Closed | Open | Open | Closed |

#### JP5 Setting (Dual)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | Closed |

JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP1 | JP2 | JP3 |
| --- | --- | --- | --- |
| ###### 75MHz/75MHz/30MHz | Pin 2-3 | Pin 1-2 | Pin 1-2 |

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| MII 300MHz | ON OFF 1 2 3 4 5 6 7 | 75 x 3.0 |

E. Cyrix 6x86L CPU (dual voltage)

JP4 (2.8V)

| #### JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.8V | Open | Open | Open | Closed |

#### JP5 Setting (Dual)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | Closed |

JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP1 | JP2 | JP3 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz* | Pin 2-3 | Pin 2-3 | Pin 2-3 |

Cyrix 6x86L PR166+

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| PR166+ | ON OFF 1 2 3 4 5 6 7 | 66 x 2.0 |

F.  AMD-K5 CPU Series

JP4(3.5V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 3.5V | Closed | Closed | Closed | Closed |

#### JP5 Setting (Single)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Single Voltage Mode(Vcore = Vi/o)* | Open |

JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP1 | JP2 | JP3 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz* | Pin 2-3 | Pin 2-3 | Pin 2-3 |

F-1.  AMD-K5 PR100

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| PR100 | ON OFF 1 2 3 4 5 6 7 | 66 x 1.5 |

F-2.  AMD-K5 PR133

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| PR133 | ON OFF 1 2 3 4 5 6 7 | 66 x 1.5 |

F-3.  AMD-K5 PR166

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| PR166 | ON OFF 1 2 3 4 5 6 7 | 66 x 2.5 |

G.  AMD-K6 CPU

JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP1 | JP2 | JP3 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 2-3 | Pin 2-3 | Pin 2-3 |

#### JP5 Setting (Dual)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | Closed |

G-1.  AMD-K6 166MHz

JP4(2.9V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.9V | Closed | Open | Open | Closed |

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| 166MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 2.5 |

G-2.  AMD-K6 200MHz

JP4(2.9V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| Vcore | Closed | Open | Open | Closed |

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| 200MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 3.0 |

G-3a.  AMD-K6 233MHz

JP4(3.2V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 3.2V | Open | Open | Closed | Closed |

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 233MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 3.5 |

G-3b.  AMD-K6-2 233MHz

JP4(2.2V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.2V | Open | Closed | Open | Open |

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 233MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 3.5 |

G-4a.  AMD-K6 266 MHz

JP4(2.2V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.2V | Open | Closed | Open | Open |

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| 266MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 4.0 |

G-4b.  AMD-K6-2 266 MHz

JP4(2.2V)

| #### JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.2V | Open | Closed | Open | Open |

| CPU Clock | SW1 | Base x Ratio |
| --- | --- | --- |
| 266MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 4.0 |

G-5a.  AMD-K6 300MHz

JP4(2.2V)

| #### JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.2V | Open | Closed | Open | Open |

#### JP5 Setting (Dual)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | Closed |

JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP1 | JP2 | JP3 |
| --- | --- | --- | --- |
| ###### 66MHz/66MHz/33MHz | Pin 2-3 | Pin 2-3 | Pin 2-3 |

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 300MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 4.5 |

G-5b.  AMD-K6-2 300MHz

JP4(2.2V)

| #### JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| Vcore | Open | Closed | Open | Open |

#### JP5 Setting (Dual)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o) | Closed |

SW1(Pin7), JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | SW1-pin7 | JP1 | JP2 | JP3 |
| --- | --- | --- | --- | --- |
| 100MHz/100MHz/33MHz* | OFF | Pin 2-3 | Pin 1-2 | Pin 2-3 |
| 100MHz/66MHz/33MHz** | ON | Pin 1-2 | Pin 1-2 | Pin 2-3 |

*	Please use PC-100 SDRAM module (SYNC.)

**	Please use non PC-100 SDRAM module or EDO DIMM module (ASYNC.)

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 300MHz | ON OFF 1 2 3 4 5 6 7 | 100 x 3.0 |

G-5c.  AMD-K6-2 333MHz

JP4(2.2V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.2V | Open | Closed | Open | Open |

#### JP5 Setting (Dual)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o) | Closed |

SW1(Pin7), JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | SW1-pin7 | JP1 | JP2 | JP3 |
| --- | --- | --- | --- | --- |
| 95MHz/95MHz/31.7MHz* | OFF | Pin 2-3 | Pin 1-2 | Pin 2-3 |
| 95MHz/66MHz/31.7MHz** | ON | Pin 1-2 | Pin 1-2 | Pin 2-3 |

*	Please use for PC-100 SDRAM module (SYNC.)

**	Please use non PC-100 SDRAM module or EDO DIMM module (ASYNC.)

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 333MHz | ON OFF 1 2 3 4 5 6 7 | 95 x 3.5 |

G-6.  AMD-K6-2 350MHz

JP4(2.2V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 2.2V | Open | Closed | Open | Open |

#### JP5 Setting (Dual)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | Closed |

SW1(Pin7), JP1 & JP2 Settings

| ###### CPU host clock/DRAM/PCI clock | SW1-pin7 | JP1 | JP2 | JP3 |
| --- | --- | --- | --- | --- |
| 100MHz/100MHz/33MHz* | OFF | Pin 2-3 | Pin 1-2 | Pin 2-3 |
| 100MHz/66MHz/33MHz** | ON | Pin 1-2 | Pin 1-2 | Pin 2-3 |

*	Please use PC-100 SDRAM module (SYNC.)

**	Please use non PC-100 SDRAM module or EDO DIMM module (ASYNC.)

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 350MHz | ON OFF 1 2 3 4 5 6 7 | 100 x 3.5 |

H. IDT Win Chip C6

JP4(3.5V)

| JP4 Vcore | Pin1 & Pin5 | Pin2 & Pin6 | Pin3 & Pin7 | Pin4 & Pin8 |
| --- | --- | --- | --- | --- |
| 3.5V | Closed | Closed | Closed | Closed |

JP5 Setting (Single)

| Mode | JP5 (Pin1-2, 3-4, 5-6) |
| --- | --- |
| Single Voltage Mode(Vcore = Vi/o)* | Open |

H-1.  IDT Win Chip C6 200MHz

JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP1 | JP2 | JP3 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 2-3 | Pin 2-3 | Pin 2-3 |

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 200MHz | ON OFF 1 2 3 4 5 6 7 | 66 x 3.0 |

H-2  IDT Win Chip C6 225MHz

JP1, JP2 & JP3 Settings

| ###### CPU host clock/DRAM/PCI clock | JP1 | JP2 | JP3 |
| --- | --- | --- | --- |
| 75MHz/75MHz/30MHz | Pin 2-3 | Pin 1-2 | Pin 1-2 |

| CPU CLOCK | SW1 | Base x Ratio |
| --- | --- | --- |
| 225MHz | ON OFF 1 2 3 4 5 6 7 | 75 x 3.0 |

2-3 SYSTEM MEMORY INSTALLATION

The 5200M provides three 168-pin DIMM sockets for system memory expansion from 8MB to 768MB. These three DIMMs are arranged to three banks, please refer to page A. Each bank provides 64-bit wide data path.

※ Samples of System Memory Combinations Options ※

| BANK0 | BANK1 | BANK2 | Total Memory |
| --- | --- | --- | --- |
| DIMM 1 | DIMM 2 | DIMM 3 | DIMM 1-3 |
| 32MBx1 | - | - | 32MB |
| - | - | 32MBx1 | 32MB |
| 8MBx1 | 16MBx1 | 16MBx1 | 40MB |
| 32MBx1 | 32MBx1 | - | 64MB |
| - | 32MBx1 | 32MBx1 | 64MB |
| 64MBx1 | - | - | 64MB |
| : | : | : | : |
| - | 64MBx1 | 64MBx1 | 128MB |
| 128MBx1 | - | - | 128MB |
| - | 128MBx1 | - | 128MB |
| - | - | 128MBx1 | 128MB |
| 128MBx1 | 128MBx1 | - | 256MB |
| 128MBx1 | - | 128MBx1 | 256MB |
| - | 128MBx1 | 128MBx1 | 256MB |
| 256MBx1 | - | - | 256MB |
| : | : | : | : |
| 256MBx1 | 256MBx1 | - | 512MB |
| 256MBx1 | - | 256MBx1 | 512MB |
| - | 256MBx1 | 256MBx1 | 512MB |
| 256MBx1 | 256MBx1 | 256MBx1 | 768MB |

2-4 VIA IDE DRIVER SETUP

The VIA IDE drivers are enclosed into the VIA Utility CD. Use this CD ROM for installation of the VIA drivers.

2-4-1  Setup for Windows 9x :

1. Starting Windows system.

2. Insert the VIA Utility CD ROM into your CD-ROM Drive.  If the VIA Driver Setup Main Menu does not appear automatically, just point to “My Computer” and click the CD ROM icon to open the VIA Utility CD ROM directly.

3. In a second, the “VIA Drivers Installation Main Menu” shows up as below:

4.To start VIA drivers setup, point to the “VIA Integrated Setup” and click on it.

5.User-friendly Setup screens will appear one by one , guiding you to finish the complete setup.

6. When the “Setup complete” screen appears, click “Finish” to restart your computer so as to put the just installed drivers into effect.

2-4-2  Setup for Windows NT 4.0

The VIA IDE drivers for WinNT are also enclosed into the VIA Utility CD. Use this CD ROM for installation of the VIA drivers for WinNT.  And the installation procedures are the same as those for Windows 9x. Please refer to “Setup for windows 9x for your Windows NT installation.

CHAPTER 3

Award BIOS SETUP

Award BIOS manufacturer provides access to the system BIOS through the hardware and software on each 5200M mainboard. The hardware consists of a Flash ROM and the software is a group of programs that are installed in the ROMBIOS along with all the other data the BIOS must contain.

The 5200M mainboard will require special driver supplied by the manufacturer to update the BIOS SETUP program. It is a good idea to read the next page for details for update BIOS driver installation or you can ask your system dealer to do it for you.

When the driver has been successfully updated, it is very important to contact your system dealer to change the CMOS settings for your computer. The CMOS settings are shown in the following pages.

NOTE : To clear CMOS you should unplug the power cord, then set 2-3 to clear, put it back to normal position and plug the power cord again.

|  | JP8 |
| --- | --- |
| Normal | 1-2 |
| Clear | 2-3 |

JP8

ISA slots

3-1 UPDATE BIOS PROCEDURE

3-1-1 Updating BIOS

If the BIOS needs to be updated, you can get a CD or diskette with the updated BIOS utility in the package. The BIOS-update diskette or All-In-One CD includes two files:

“awdflash.exe”  --  and 	“awdflash.doc”.

And the new or updated Award BIOS file should be obtained from your system manufacturer or dealer.

The updating procedure is as the following:

- Before updating your BIOS, please load the above-mentioned files to root directory “C” or a floppy drive diskette.

- Boot the system to DOS mode in a normal manner.

- Under “C” or Drive “A” (where your BIOS update files should be), type awdflash.exe and press “Enter”

- A window similar to the following dialog box will instantly appear, on which you should enter the file name of the new or updated BIOS.

(Sample of BIOS Update Dialog Box)

- After you have entered the file name, press “Enter”. And on the “Error Message” row shows “Do you want to save BIOS (Y/N)”.

- Type “N“ but do not press “Enter”.  And the message : " Are you sure to program (Y/N) ?" will appear at once.

- Type “Y” but do not press “Enter”.

- BIOS updating process will start at once until you see the message “Reset System or power off to accomplish update process”.

Press “F1” button to restart your system, and the updating process is complete.

3-2  SYSTEM BIOS CONFIGURATION SETUP

The following pages explain how to set up the system configuration (CMOS) under the Award BIOS. The SETUP program is stored in the Read-Only-Memory (ROM) on the mainboard. To do the SETUP procedure, press the <Del> key when the system is booting up. The following main menu will appear. Please select " STANDARD CMOS SETUP" to enter the next screen.

ROM PCI/ISA BIOS (5200M)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | âàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

The section on the bottom of the main menu explains how to control this screen.  The other section displays the items highlighted in the list.

3-2-1  Standard CMOS Setup

This screen records some basic hardware information, and sets the system clock and error handling. These records can be lost or corrupted if the on-board battery has failed or is weak.

ROM PCI/ISA BIOS (5200M)

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| Date (mm:dd:yy) : Wed, June 05 1998 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time(hh:mm:ss) : 13 : 37 : 14 |  |  |  |  |  |  |  |  |
| HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |  |  |  |  |  |  |
| Primary Master : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Primary Slave : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Secondary Master : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Secondary Slave : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Drive A : 1.44M, 3.5 in. |  |  |  |  |  |  |  |  |
| Drive B : None |  |  | Base Memory : 640K |  |  |  |  |  |
| Floppy 3 Mode Support : Disabled |  |  | Extended memory : 7168K |  |  |  |  |  |
|  |  |  | Other Memory : 384K |  |  |  |  |  |
| Video : EGA/VGA |  |  | ------------------------------------------------ |  |  |  |  |  |
| Halt On: All Errors |  |  | Total Memory : 8192K |  |  |  |  |  |
| ESC : Quit áâàß:Select Item PU/PD/+/- : Modify |  |  |  |  |  |  |  |  |
| F1 : Help (Shift) F2 : Change Color |  |  |  |  |  |  |  |  |

Date  Date format is <day>,<date>,<month>,<year>. 	Press<F3> to show calendar.

| day | The day, from Sun to Sat, determined by the BIOS and is displayed-only |
| --- | --- |
| date | The date, from 1 to 31 |
| month | The month, Jan. through Dec. |
| year | The year, from 1900 to 2099 |

Time  The time format is <hour><minute><second>. The time is calculated based on the 24-hour military-time clock. For example, 1p.m. is 13:00:00.

Primary Master Primary; Slave Secondary Master Secondary Slave

These categories identify the types of the 2 channels that have been installed in the computer. There are 45 predefined types and 4 user definable types for Enhanced IDE BIOS. Type 1 to 45 which are predefined. Type ‘user’ which is user-definable. Press PgUp/PgDn to select a numbered hard disk type or type the number and press <Enter>. If ‘Auto’ selected, the BIOS will detect the HDD & CD-ROM Drive automatically at the POST stage and show the IDE for HDD & CD-ROM Drive. If ‘user’ selected, you will need to know the information listed below. This information should be from your hard disk vender or dealer. Then enter the figure directly and press <Enter>. If the controller of the HDD interface is ESDI, the selection shall be ‘Type 1’; if SCSI, the selection is ‘None’. If no device is installed, select ‘NONE’ and press <Enter>.

| Type | drive type |
| --- | --- |
| SIZE | automatically adjusts |
| CYLS | number of cylinders |
| HEAD | number of heads |
| PRECOMP | write precom |
| LANDZ | landing zone |
| SECTOR | number of sectors |
| MODE | mode type |

Drive A, Drive B  This category identifies the types of floppy disk drive A or drive B that have been installed in the computer.

| None | No floppy drive installed |
| --- | --- |
| 360K, 5.25 in | 5.25“ PC-type 360KB capacity |
| 1.2M, 5.25 in | 5.25“ AT-type 1.2MB capacity |
| 720K, 3.5 in | 3.5“ double-side 720KB capacity |
| 1.44M, 3.5 in | 3.5“ double-side 1.44MB capacity |
| 2.88M, 3.5 in | 3.5“ double-side 2.88MB capacity |

Floppy 3 Mode Support  This is the Japanese standard floppy drive. This standard stores 1.2MB in a 3.5” diskette.

Video  This category selects the type of video adapter used for the primary system monitor. Although secondary monitors are supported, you do not have to select the type in Setup.

| EGA/VGA | Enhanced Graphics Adapter/Video Graphics Array. For EGA, VGA, SEGA, SVGA or PGA monitor adapters. |
| --- | --- |
| CGA 40 | Color Graphics Adapters, power up in 40 column mode. |
| CGA 80 | Color Graphics Adapters, power up in 80 column mode. |
| MONO | Monochrome adapter, includes high resolution monochrome adapters. |

Halt On  This category determines whether the computer will stop if an error is detected during power up.

| No errors | The system boot will not be stopped for any error that may be detected |
| --- | --- |
| All errors | When the BIOS detects a non-fatal error the system will be stopped and you will be prompted |
| All, But Keyboard | The system boot will not stop for a keyboard error, it will stop for all other errors |
| All, But Diskette | The system boot will not stop for a disk error, it will stop for all other errors |
| All, But Disk/Key | The system boot will not stop for a disk or keyboard error, it will stop for all other errors |

Memory This category is displayed only which is determined by POST (Power On Self Test) of the BIOS. Base Memory The POST will determine the amount of base (or conventional) memory installed in the system. The value of the base memory is typically 512K or 640K based on the memory installed on the mainboard. Extended Memory How much extended memory is present during the POST. This is the amount of memory located above 1MB in the CPU‘s memory address map. Other Memory This refers to the memory located in the 640K to 1024K address space. This is memory that can be used for different applications. DOS uses this area to load device drivers in an effort to keep as much base memory free for application programs. The BIOS is the most frequent user of this RAM area since this is where it shadows RAM.

3-2-2  BIOS Features Setup

This screen is a list of system configuration options. Some of them are defaults required by the mainboard's design, others depend on the features of your system.

ROM PCI/ISA BIOS (5200M)

BIOS FEATURES SETUP

AWARD SOFTWARE, INC.

| Virus Warning | : Disabled | Video BIOS Shadow | : Enabled |
| --- | --- | --- | --- |
| CPU Internal Cache | : Enabled | C8000-CBFFF Shadow | : Disabled |
| External Cache | : Enabled | CC000-CFFFF Shadow | : Disabled |
| Quick Power On Self Test | : Enabled | D0000-D3FFF Shadow | : Disabled |
| Boot Sequence | : A,C,SCSI | D4000-D7FFF Shadow | : Disabled |
| Swap Floppy Drive | : Disabled | D8000-DBFFF Shadow | : Disabled |
| Boot Up Floppy Seek | : Enabled | DC000-DFFFF Shadow | : Disabled |
| Boot Up NumLock Status | : On | Cyrix 6x86/MII CPUID | : Enabled |
| Gate A20 Option | : Fast |  |  |
| Memory Parity/ECC Check | : Enabled |  |  |
| Typematic Rate Setting | : Disabled |  |  |
| Typematic Rate(Chars/Sec) | : 6 |  |  |
| Typematic Delay(Msec) | : 250 | Esc : Quit áâàß:Select Item |  |
| Security Option | : Setup | F1 : Help PU/PD/+/- : Modify |  |
| PCI/VGA Palette Snoop | : Disabled | F5 : Old Values (SHIFT) F2 : Color |  |
| OS Select for DRAM>64MB | : Non-OS2 | F6 : Load BIOS Defaults |  |
| Report No FDD For Win95 | : No | F7 : Load Setup Defaults |  |
| RTC Y2K Compliance | : Enabled |  |  |

| Virus Warning When this item is enabled, the Award BIOS will monitor the boot sector and partition table of the hard disk drive for any attempt at modification. If an attempt is made, the BIOS will halt the system and the following error message will appear. Afterwards, if necessary, you will be able to run an antivirus program to locate and remove the problem before any damage is done. |  |  |  |
| --- | --- | --- | --- |
|  | ! WARNING ! Disk boot sector is to be modified Type ‘Y’ to accept write or ‘N’ to abort write Award Software, Inc. |  |  |
|  | Enabled | Activates automatically when the system boots up. If anything attempts to access the boot sector or hard disk, partition table will cause a warning message to appear. |  |
|  | Disabled | No warning message will appear when anything attempts to access the boot sector or hard disk partition table. |  |

Many disk diagnostic programs which attempt to access the boot sector table can cause the above warning message. If you will be running such a program, we recommend that you first disable Virus Protection beforehand.

CPU Internal Cache, External Cache  These two categories speed up memory access. However, it depends on CPU/chipset design. The default value is ‘enabled‘.

Quick Power On Self Test  This category speeds up Power On Self Test after you power up the computer. If you set Enabled, BIOS will shorten or skip some items under check during POST.

Boot Sequence  This category determines which drive to search first for System booting(i.e., DOS). The system will search those drives in order, Ex.: C, CDROM, A: System will first search for hard disk drive then CDROM drive, and the last is floppy disk drive.

Note: C is primary master; D is primary slave; E is secondary master, F is secondary slave.

Swap Floppy Drive  This item allows you to determine whether to enable the swap floppy drive or not. The choice : Enabled/ Disabled

Boot Up Floppy Seek  This item enabled, System will boot with floppy disk track verification: 40tracks/80 tracks. This item disabled, System will not search for the floppy disk type.

| Boot Up NumLock Status This allows you to determine the default state of the numeric keypad. By default, the system boots up with NumLock on. |  |  |
| --- | --- | --- |
| On | Keypad is for numeric keys |  |
| Off | Keypad is for arrow keys |  |

Gate A20 Option  The Gate A20 is a device used to address memory above 1MB. While Gate A20 support can be provided through keyboard, it is much faster for system chipset to provide Gate A20 support.

To set “Normal” for keyboard, “Fast” for chipset.

Memory Parity/ECC Check  Enabled, this item supports the SDRAM which has the Memory Parity/ECC Check functions.

Typematic Rate Setting	 This determines if the typematic rate is to be used. When disabled, continually holding down a key on your keyboard will generate only one key instance. In other words, the BIOS will only report that the key is down. When the typematic rate is enabled, the BIOS will report as before, but it will then wait a moment, and, if the key is still down, it will begin the report that the key has been depressed repeatedly. For example, you would use such a feature to accelerate cursor movements with the arrow keys. The choice : Enabled/Disabled

Typematic Rate (Chars/Sec)  When the typematic rate is enabled, this item allows you to select the rate at which the keys are repeated. 6 means 6 characters per second.

Typematic Delay (Msec)  When the typematic rate is enabled, this item allows you to select the delay between when the key was first depressed and when the acceleration begins. 250: 250 msec (500,750,1000)

Security Option  This category allows you to limit access to the system and Setup, or just to Setup.

| System | The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt |
| --- | --- |
| Setup | The system will boot, but access to Setup will be denied if the correct password is not entered at the prompt |

To disable the security, select PASSWORD SETTING at Main Menu, and then you will be asked to enter password. Do not type anything and just press <Enter>, it will disable the security. Once the security is disabled, the system will boot and you can enter Setup freely.

PCI/VGA Palette Snoop  It determines whether the MPEG ISA/VESA VGA cards can work with PCI/VGA or not.

| Enabled | When PCI/VGA working with MPEG ISA/VESA VGA Card |
| --- | --- |
| Disabled | When PCI/VGA not working with MPEG ISA/VESA VGA Card |

OS Select for DRAM > 64MB  This item allows you to access the memory that is over 64MB in OS/2. The choice : Non-OS2, OS2.

Report No FDD For WIN 95  Set this item to Yes, BIOS will report FDD to Win95. If in standard CMOS setup, set Drive A to none and set this item to yes. Inside Win95, My Computer and File manager Disk(A:) will show Removable Disk (A:).

RTC Y2K Compliance  “Enabled”, RTC (Real Time Clock)will pass the Y2K Compliance Test.  “Disabled”, RTC will not pass the Y2K Compliance Test.

Video BIOS Shadow  Determines whether video BIOS will be copied to RAM. However it is optional depending on chipset design. Video Shadow will increase the video speed. The choice : Enabled/Disabled

C8000 – CBFFF Shadow; DC000 – DFFFF Shadow  These categories determine whether option ROMs will be copied to RAM. An example of such option ROM would be the support of onboard SCSI. The choice : Enabled/Disabled

Cyrix 6x86/MII CPUID  “Enabled”, Cyrix 6x86/MII MMX function is active.  “Disabled”, Cyrix 6x86/MII MMX function is Closed, so that NetWare System may run without interference.

3-2-3  Chipset Features Setup

This screen controls the setting for the chipset on the mainboard.

ROM PCI/ISA BIOS (5200M)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Bank 0/1 DRAM Timing | : FP/EDO 70ns | OnChip USB |  | : enabled |
| --- | --- | --- | --- | --- |
| Bank 2/3 DRAM Timing | : FP/EDO 70ns | USB Keyboard Support |  | : Disabled |
| Bank 4/5 DRAM Timing | : FP/EDO 70ns | Auto Detect DIMM/PCI Clk |  | : Enabled |
| SRAM Cycle Length | : 2 | Spread Spectrum |  | : Disabled |
| DRAM Read Pipeline | : Disabled |  |  |  |
| Sustained 3T Write | : Enabled |  |  |  |
| Cache Rd+CPU Wt Pipeline | : Disabled |  |  |  |
| Cache Timing | : Fast |  |  |  |
| Video BIOS Cacheable | : Disabled |  |  |  |
| System BIOS Cacheable | : Disabled |  |  |  |
| Memory Hole At 15Mb Addr. | : Disabled |  |  |  |
| AGP Aperture Size (MB) | : 64MB |  |  |  |
|  |  | Esc: Quit | :Select Item |  |
|  |  | F1 : Help | PU/PD/+/-:Modify |  |
|  |  | F5 : Old Values | (Shift)F2 :Color |  |
|  |  | F6 :Load BIOS Defaults |  |  |
|  |  | F7 : Load Setup Defaults |  |  |

Bank 0/1/2/3/4/5 DRAM Timing   The DRAM speed is controlled by the DRAM timing registers. The timing programmed into this register depends on the system design. Slower rates may be required in certain system designs to support loose layouts or slower memory.  i.e. 60ns; 70ns

SDRAM Cycle Length      You should select CAS latency time in HCLKS of 2/2 or 3/3. The system board designer should set the values in this field, depending on the DRAM installed. Do not change the values in this field unless you change specifications of the installed DRAM or the installed CPU..

The choice : 2, .3

DRAM Read Pipeline   Enable/Disable DRAM Read Pipeline Cycle.

Sustained 3T Write       Enabled : set cache to write back mode.

Disabled : set cache to write through mode.

Cache Rd+CPU Wt Pipeline   Enable/Disable Cache Read Write cycle.

Cache Timing      Fast : Better system performance will occur.

Video BIOS Cacheable 	Select Enabled allows caching of the video BIOS ROM at

F0000h-FFFFFh, resulting in better system performance.  However, if any program writes to this memory area, a system error may result.

System BIOS	Select Enabled allows caching of the system BIOS ROM at   F0000h-FFFFFh, resulting in better system performance.  However, if any program writes to this memory area, a system error may result.

Memory Hole At 15Mb Addr.	In order to improve performance, certain space

in memory can be reserved for ISA cards. This memory must be mapped into the memory below 16MB.

AGP Aperture Size (MB)	Select the size of the AGP aperture. The aperture is a portion of the PCI memory address range dedicated for graphics memory address space. Host cycle that hit the aperture range are forwarded to the AGP without any translation. See  www.agpforum.org for AGP information.

Onchip USB & USB Keyboard Support    Enable/Disable USB.  If  USB is “Enabled”, “USB Keyboard” will appear for “Enabled/Disabled” option.

Auto Detec DIMM/PCI Clk  If this item is enabled, the unused DIMM and PCI slot clock will be disabled. If this item is disabled the unused DIMM  and PCI slot will still get the active clock signal.

Spread Spectrum Modulated    Enable / Disable this item the BIOS will Enable / Disable the clock generator spread spectrum .

3-2-4  Power Management Setup

This screen controls the 'green' features of this mainboard.

ROM PCI/ISA BIOS (5200M)

POWER MANAGEMENT SETUP

AWARD SOFTWARE, INC.

| Power Management | : User Defined |  | Primary INTR : ON |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PM Control by APM | : Yes |  | IRQ3 (COM2) |  |  |  | : Primary |  |
| Video Off Option | : Suspend ->OFF |  | IRQ4 (COM1) |  |  |  | : Primary |  |
| Video Off Method | : V/HSync+blank |  | IRQ5 (LPT 2) |  |  |  | : Primary |  |
| Modem Use IRQ | : 3 |  | IRQ6 (Floppy Disk) |  |  |  | : Primary |  |
| Soft-Off by PWRBTN | : Delay 4 sec. |  | IRQ7 (LPT 1) |  |  |  | : Primary |  |
| ** PM Timers | ** |  | IRQ8 (RTC Alarm) |  |  |  | : Disabled |  |
| HDD Power Down | : Disabled |  | IRQ9 (IRQ2 Redir) |  |  |  | : Secondary |  |
| Doze Mode | : Disabled |  | IRQ10 (Reserved) |  |  |  | : Secondary |  |
| Suspend Mode | : Disabled |  | IRQ11 (Reserved) |  |  |  | : Secondary |  |
| ** PM Events | ** |  | IRQ12 (PS/2 Mouse) |  |  |  | : Primary |  |
| VGA | : OFF |  | IRQ13 (Coprocessor) |  |  |  | : Disabled |  |
| LPT & COM | : LPT/COM |  | IRQ14 (Hard Disk) |  |  |  | : Primary |  |
| HDD & FDD | : ON |  | IRQ15 (Reserved) |  |  |  | : Disabled |  |
| DMA/master | : OFF |  | Esc: Quit áâàß |  |  | :Select Item |  |  |
| Modem Ring Resume | : Disabled |  | F1 : Help | PU/PD/+/- : Modify |  |  |  |  |
| RTC Alarm Resume | : Disabled |  | F5 : Old Values |  | (Shift) F2: Color |  |  |  |
| Date (of Month) |  |  | F6 : Load BIOS Defaults |  |  |  |  |  |
| Timer (hh:mm:ss) |  |  | F7 : Load Setup Defaults |  |  |  |  |  |

Power Management 	This category allows you to select the type (or degree) of power saving and is directly related to the following modes : Doze; Standby; Suspend;

HDD Power Down.

| Min. Power Saving | Minimum power management. Doze =1hr.; Standby=1hr.; Suspend=1hr.; HDD Power Down=15min |
| --- | --- |
| Max. Power Saving | Maximum power management only available for SL CPU.Doze=1min.; Standby=1min.;Suspend=1min.;HDD Power Down=1min |
| User Defined | Allows you to set each mode individually. When not disabled, each of the ranges are from 1min. to 1hr. except for HDD Power Down which ranges from 1 to 15min. and disable |

If you would like to use Software Power-off Control function, you cannot choose“ Disabled ”here, and should select “Yes” in PM Control by APM.

PM Control by APM	When enabled, an Advanced Power Management 	device will be activated to enhance the Max. Power Saving Mode and stop the CPU internal clock. If the Max.Power Saving is not enabled, this will be shown as NO.

Video Off Option          When enabled, this feature allows the VGA adapter to operate in a power saving mode.

| Always On | Monitor will remain on during power saving modes. |
| --- | --- |
| Suspend | Monitor blanked when the systems enters the Suspend mode. |
| All Modes | Monitor blanked when the system enters any power saving mode. |

Video Off Method	This determines the manner in which the monitor is blanked.

| V/H SYNC + Blank | This selection will cause the system to turn off the vertical and horizontal sync. ports and write blanks to the video buffer |
| --- | --- |
| Blank Screen | This option only writes blanks to the video buffer |
| DPMS | Initial display power management signaling |

MODEM Use IRQ 		This item determines the IRQ in which the MODEM can be used.  The choice : 3,4,5,7,9,10,11,NA.

Soft-Off by PWR-BTTN    Instant-off : When push the power button, the system power will be off immediately. Delay 4 sec : when push the power button, it will enter suspend mode. We need to push the power button and hold for 4 seconds to turn off the power.

The Following 4 modes are Green PC power saving functions which are only user configurable when ‘User Defined’ power management has been selected.

HDD Power Down	When enabled and after the set time of system inactivity, the

hard disk drive will be powered down while all other devices           remain active.

Doze Mode       	When enabled and after the set time of system inactivity, the CPU           clock will run at slower speed while all other devices will operate                       at full speed.

Suspend Mode	When enabled and after the set time of system inactivity, all

devices except the CPU will be shut off.

VGA/LPT & COM  These are I/O events whose occurrence can prevent the system

HDD & FDD/      from entering a power saving mode or can awaken the system

DMA/master       from such a mode. In fact, the system remains alert for anything

which occurs on a device which is configured as on, even when

the system is in a password down mode. When an I/O device

wants to gain the attention of the operating system, it signals this

by causing an IRQ(Interrupt ReQuest) to occur. When the

operating system is ready to respond to the request, it interrupts

itself and performs the service. When set to off, activity will

neither prevent the system from going into a power management

mode nor awaken it.

Modem Ring Resume    Enabled : when system in suspend mode, it can be wake up by modem.   Disabled : it cannot be wake up by modem.

RTC Alarm Resume  When enabled, two additional lines will be added to the screen for user’s setup: Date (of Month) Alarm; Timer (hh:mm:ss) .

After power off, the system will automatically power on at the specified date and time.

Primary INTR        It enables/disables the IRQ3 to IRQ15 PM events. If Enabled, IRQ3 to IRQ15 will appear for user’s setup.

3-2-5  PNP/PCI Configuration

This screen configures the PCI Bus slots.

ROM PCI/ISA BIOS (5200M)

PNP/PCI CONFIGURATION

AWARD SOFTWARE, INC.

| PNP OS Installed | : No | CPU to PCI Write Buffer |  |  |  | : Enabled |
| --- | --- | --- | --- | --- | --- | --- |
| Resources Controlled by | : Auto | PCI Dynamic Bursting |  |  |  | : Disabled |
| Reset Configuration Data | : Disabled | PCI Master 0 WS Write |  |  |  | : Enabled |
|  |  | PCI Delay Transaction |  |  |  | : Disabled |
| IRQ-3 assigned to | : PCI/ISA PnP | PCI Master Read Prefetch |  |  |  | : Disabled |
| IRQ-4 assigned to | : PCI/ISA PnP | PCI#2 Access #1 Retry |  |  |  | : Disabled |
| IRQ-5assigned to | : PCI/ISA PnP | AGP Master 1 WS Write |  |  |  | : Disabled |
| IRQ-7 assigned to | : PCI/ISA PnP | AGP Master 1 WS Read |  |  |  | : Disabled |
| IRQ-10 assigned to | : PCI/ISA PnP | Assign IRQ For USB |  |  |  | : Enabled |
| IRQ-11 assigned to | : PCI/ISA PnP | Assign IRQ For VGA |  |  |  | : Disabled |
| IRQ12 assigned to | : PCI/ISA PnP |  |  |  |  |  |
| IRQ-14 assigned to | : PCI/ISA PnP | Esc: Quit áâàß |  |  | :Select Item |  |
| IRQ-15 assigned to | : PCI/ISA PnP | F1 : Help | PU/PD/+/- : Modify |  |  |  |
| DMA-0 assigned to | : PCI/ISA PnP | F5 : Old Values |  | (Shift) F2: Color |  |  |
| DMA-1 assigned to | :PCI/ISA PnP | F6 : Load BIOS Defaults |  |  |  |  |
| DMA-3 assigned to | :PCI/ISA PnP | F7 : Load Setup Defaults |  |  |  |  |
| DMA-5 assigned to | :PCI/ISA PnP |  |  |  |  |  |

PNP OS Installed	This item allows you to choose PnP OS or not. If “Yes”, OS will carry out PnP and BIOS will not. If “No”, OS will not do PnP, but BIOS will.

Resources Controlled by	If “PnP OS Installed” is set “No”, the Award Plug and Play BIOS has the capability to automatically configure all of the boot and Plug and Play compatible devices. However, this capability means absolutely nothing unless you are using a Plug and Play OS such as Windows 95. Choices are Auto and Manual.

Reset Configuration Data  This item allows you to determine whether to reset the

configuration data or not.

ACPI I/O Device Node  Enable : reserve a node in memory for ACPI.

CPU to PCI Write Buffer   Enable/Disable CPU to PCI POST Write.

PCI Dynamic Bursting    Enable/Disable PCI burst operation.

PCI Master 0 WS Write    Enable : PCI Master 0 wait state mode.

Disable : PCI Master 1 wait state mode.

PCI Delay Transaction   This chipset has an embedded 32-bit posted write buffer to support delay transactions cycles. Select “enabled” to support compliance with PCI specification version 2.1.

## The choice : Enabled, disabled

PCI Master Read Prefetch   Enable : always prefetch

Disable : prefetch only if enhance command

PCI#2 Access #1 Retry    Disable : PCI#2 will be disconnected until access finished      Enable : PCI#2 will be disconnected if max. retries are attempted without success.

AGP Master 1 WS Write   Enable/Disable AGP master one wait state write.

AGP Master 1 WS Read   Enable/Disable AGP master one wait state read.

Assign IRQ for USB   When this items is enabled, the system will assign an IRQ for USB. If this item is disabled, the USB will not occupy an IRQ; therefore the IRQ of USB will be released for other usage.

Assign IRQ for VGA   When this items is enabled, the system will assign an IRQ for VGA. If this item is disabled, the VGA will not occupy an IRQ; therefore the IRQ of VGA will be released for other usage.

3-2-6  Integrated Peripherals

This section page includes all the items of IDE hard drive and Programmed Input/Output features. See also Section “Chipset Features Setup”.

ROM PCI/ISA BIOS (5200M)

INTEGRATED PERIPHERALS

AWARD SOFTWARE, INC.

| OnChip IDE First Channel | : Enabled | Onboard Parallel Port |  |  | :378/IRQ7 |
| --- | --- | --- | --- | --- | --- |
| OnChip IDE Second Channel | : Enabled | Onboard Parallel Mode |  |  | :SPP |
| IDE Prefetch Mode | : Disabled | ECP Mode Use DMA |  |  | : 3 |
| IDE HDD Block Mode | : Disabled | Parallel Port EPP Type |  |  | : EPP1.9 |
| IDE Primary Master PIO | : Auto |  |  |  |  |
| IDE Primary Slave PIO | : Auto |  |  |  |  |
| IDE Secondary Master PIO | : Auto |  |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |  |
| IDE Primary Master UDMA | : Disabled |  |  |  |  |
| IDE Primary Slave UDMA | : Disabled |  |  |  |  |
| IDE Secondary Master UDMA | : Disabled |  |  |  |  |
| IDE Secondary Slave UDMA | : Disabled |  |  |  |  |
| Init Display First | : PCI Slot |  |  |  |  |
| Onboard FDD Controller | : Enabled |  |  |  |  |
| Onboard Serial Port 1 | : 3F8/IRQ4 | Esc: Quit áâàß |  |  | :Select Item |
| Onboard Serial Port 2 | : 2F8/IRQ3 | F1 : Help | PU/PD/+/- : Modify |  |  |
| UART 2 Mode | : Standard | F5 : Old Values |  | (Shift) F2: Color |  |
| IR Function Duplex | : Full | F6 : Load BIOS Defaults |  |  |  |
| RxD, TxD Active | : Hi, Hi | F7 : Load Setup Defaults |  |  |  |

OnChip IDE First     These 2 items allow you to either enable or disable

Channel             the primary/secondary controller. You might choose to

OnChip IDE Second   disable the controller if you were to add higher performance

Channel             or specialized controller.

IDE Prefetch Mode    Enable/Disable IDE Read Prefetch Buffer.

IDE HDD Block	  This allows your HD controller to use the fast block mode to

Mode	  transfer data to and from your HDD drive

| Enabled | IDE controller uses block mode |
| --- | --- |
| Disabled | IDE controller uses standard mode |

IDE Primary	  PIO (Programmed Input/Output) allows the BIOS to

Master/Slave PIO	  tell the controller what it wants and then let the controller

IDE Secondary	  and the CPU to complete the task by themselves.

Master/Slave PIO	  This is simpler and more faster. Your system supports

five modes, 0 - 4, which primarily differ in timing. When

Auto is selected, the BIOS will select the best available mode.

IDE Primary	  Auto, will support the Ultra DMA function.

Master/Slave UDMA	Disabled, will not support the Ultra DMA function.

IDE Secondary

Master/Slave UDMA

Init AGP            “Enabled”, this item will activate the AGP in the multi-display

Display First         environment it displayed, if “disabled”, and the system has both

AGP and PCI VGA card, the AGP monitor will not display.

KBC input clock	  Let user change the keyboard working clock.

On Board           This item will enable or disable the floppy disk controller.

FDC Controller

On Board           User can select serial port IRQ. If set to 3F8/IRQ4, system will

UART Port 1         assign an IRQ for it. Note : set to Auto is not recommended.

On Board           User can select serial port IRQ. If set to 2F8/IRQ3, system will

UART Port 2	  assign an IRQ for it. Note : set to Auto is not recommended.

OnBoard UART    This lets you select the Infrared mode. Choices are Standard,

2 Mode            HPSIR, and ASKIR. If you choose HPSIR or ASKIR mode,

the screen will show another two lines to let you choose

‘IR Function Duplex’ (Full or Half) and‘RxD TxD Active’

(Hi Lo; Lo Hi; Hi Hi;Lo Lo).

On Board 	Let user select IRQ for parallel port.  When Disabled, the parallel

Parallel Port	port will be disabled

On Board 	Let user select Error Check Mode. Default is recommended not to Parallel Mode	be changed except that user has special request.

ECP Mode	 Select a DMA channel for the port.

Use DMA	 Choices are 3, 1.

3-2-7  IDE HDD Auto Detection

This allows you to detect the IDE hard disk drivers‘ parameters and enter them into ‘Standard CMOS Setup’ automatically.  If the auto-detected parameters displaye on screen do not match the ones that should be used for your hard drive, do not accept them. Press <N> to reject the values and enter the correct ones manually on the Standard CMOS Setup screen.

ROM PCI/ISA BIOS (5200M)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| Hard Disks Type Size CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Primary Master : |  |  |  |  |
|  | Select Primary Master Option (N=Skip) : N |  |  |  |
|  |  | Options Size CYLS Head PRECOMP LANDZ Sector Mode |  |  |
|  |  | 2(Y) 1337 648 64 0 2594 63 LBA |  |  |
|  |  | 1 1339 2595 16 65535 2594 63 NORMAL |  |  |
|  |  | 3 1338 1297 32 65535 2594 63 LARGE |  |  |
| Note : Some OSes (like SCO-UNIX) must use “Normal” for installation |  |  |  |  |
| ESC : Skip |  |  |  |  |

3-2-8  LOAD BIOS DEFAULTS

When your mainboard has problems and needs to troubleshoot the system, you can use this function. When “Load BIOS Default” is highlighted and you press “Enter”, the default values will be re-loaded to the “BIOS Features Setup”, “Chipset Features Setup”, “Power Management Setup” and “PNP/PCI Configuration Setup”. No

effect will be caused on the “Standard CMOS Setup”.

3-2-9  LOAD SETUP DEFAULTS

This allows you to load optimal settings which are stored in the BIOS ROM. To load the default values, highlight “Load Setup Defaults” and press “Enter. Then Setup Defaults values will be re-loaded to “BIOS Features Setup”, “Chipset Features Setup”, “Power Management Setup” and “PNP/PCI Configuration Setup”. There is no effect on the Standard CMOS Setup.

3-2-10  SUPERVISOR PASSWORD / USER PASSWORD

This allows you to set the password. The mainboard defaults with password disabled.

Enter/Change password : Enter the current password, at the prompt, key-in your new password (up to eight alphanumeric characters), press <Enter>. At the next prompt, confirm the new password by typing it again and press <Enter>.

Disable password : Press the <Enter> key instead of entering a new password when the ‘Enter Password’ dialog box appears. A message will appear confirming that the password is disabled.

If you set both supervisor and user passwords, only the supervisor password allows you to enter the BIOS SETUP program.

Reminder: If you forget your password, you must disable the CMOS by turning power off and set JP 8 ‘Closedd‘and then set JP8 “open” again to reload the system.

3-2-11  SAVE & EXIT SETUP

###### This allows you to save the new setting values in the CMOS memory and continue with the booting process.  Select what you want to do, press <Enter>.

3-2-12  EXIT WITHOUT SAVING

This allows you to exit the BIOS setup utility without recording any new values or changing old ones.

Control Key Description ※

| UP ARROW | ↑ | Move to previous item |
| --- | --- | --- |
| DOWN ARROW | ↓ | Move to next item |
| LEFT ARROW | ← | Move to the item in the left hand |
| RIGHT ARROW | → | Move to the item in the right hand |
| Esc KEY | Esc | Main Menu : Quit and not save changes Setup menu : Exit current page and return to main menu |
| PgUp KEY |  | Increase the numeric value or make changes |
| PgDn KEY |  | Decrease the numeric value or make changes |
| F1 KEY | Help | General help |
| F2 KEY | ＜Shift＞+F2 | Change color from total 16 colors |
| F5 KEY | Old Value | Restore the pervious CMOS value from CMOS |
| F6 KEY | Load BIOS default | Load the default CMOS value from BIOS default table |
| F7 KEY | Load setup default | Load Setup default |
| F10 KEY | Save & Exit Setup | Save all the CMOS changes and Exit setup, only for Main Menu |