CHAPTER 2

- INSTALLATION

2-1 INSTALLATION PROCEDURE

Before installing the computer, please prepare all components such as CPU, DRAM; peripherals such as hard drive, keyboard, CD-ROM and accessories such as cables. Then, install the system as following:

-  Put CPU / heat sink (refer to CeleronTM CPU installation guide or see the following setup chapter), and DRAM modules on the mainboard.

-  Plug add-on cards into PCI slots, if needed.

-  Connect cables to peripheral devices, especially for COM2 connector.

-  Make sure all components and devices are well connected, turn on the   power and setup System BIOS based on your configuration.

-  Install peripheral devices, add-on card drivers and test them.

-  If all of above procedures are running successfully, turn the power off and screw the chassis cover to the chassis.

2-2 CPU  INSTALLATION

This mainboard supports PPGA Socket 370 and Slot 1  processors.

The 6430M mainboard has built-in Switching Voltage Regulator to support CPU Vcore autodetection. That is, It is smart enough to detect and recognize the CPU voltage, clock, ratio and enables users to set up the CPU frequency from the BIOS Setup Screen. Users can adjust the frequency through “Frequency / Voltage Control” of the BIOS Setup Screen.

B. How to install CPU for Slot 1:

For Slot 1 CPU installation, please refer to “Page B Slot 1 installation” of this manual.

2-3 SYSTEM MEMORY INSTALLATION

6430M mainboard provides two 3.3V 168-pin DIMM sockets for system memory expansion from 8MB to 512MB SDRAM. These two DIMMs are arranged to two banks, please refer to page A.

| Bank/DIMM | PC-100 Memory Module | Total Memory |
| --- | --- | --- |
| Bank0/DIMM1 | 8/16/32/64/128/256MB | 8MB~256MB |
| Bank1/DIMM2 | 8/16/32/64/128/256MB | 8MB~256MB |
| Total System Memory |  | 8MB~512MB |

DIMM type, Size, parity supported:

  PC-66/100 or higher performance memory module.

  Both parity or non-parity are available.

- 3.3V, single/double-side.

- 8/16/32/64/128/256Mbytes.

- For either 66MHz , 100MHz and 133MHz host bus CPUs, please use 10ns or faster and PC-100 compliant modules .

2-4 INTEL 810 DRIVER INSTALLATION

 INF Files for Windows 95/98 :

- Start Windows 95/98.

- Put the All-In-One CD into your CD-ROM drive.

- Choose Intel 810/820 Inf Files on the Autorun main menu.

- Then follow the instruction on the screen.

(INF Files that enable the Intel 810/820 Chipsets to be recognized by listed operating systems.)

2-5 VGA DRIVER INSTALLATION

Install Intel 810 AGP Driver For Win95/98/NT

- Start Windows 95/98/NT.

- Put the All-In-One CD into your CD-ROM drive.

- Choose Intel 810 AGP Driver on the Autorun main menu.

- Press Next to install the driver.

- Follow the instructions on the screen to complete the installation.

2-6 SOUND DRIVER INSTALLATION

Install Sound Driver For Win95/98/NT

- Start Windows 95/98/NT.

- Put the All-In-One CD into your CD-ROM drive.

- Choose Audio Device Driver on the Autorun main menu.

- Follow the instructions on the screen to finish the installation.
(Please refer to readme.doc file to get more detailed information.)

Install YAMAHA Software Synthesizer For Win95/98/NT

- Start Windows 95/98/NT.

- Put the All-In-One CD into your CD-ROM drive.

- Choose YAMAHA Software Synthesizer on the Autorun main menu.

- Follow the instructions on the screen to finish the installation.
(Please refer to readme.doc file to get more detailed information.)

Note : the sound driver is not supported under DOS operating system.