CHAPTER 3  AWARD BIOS SETUP

Award BIOS manufacturer provides access to the system BIOS through the hardware and software on each mainboard. The hardware consists of a Flash ROM (4MB in 7460M mainboard) and the software is a group of programs that are installed in the ROMBIOS along with all the other data that should be included into the BIOS.

After the BIOS is updated, if you want to clear the old setup data stored in the CMOS, then you can reset CMOS as follow.

NOTE : In case CMOS should be reset, first unplug the power cord, then set Jumper 8 2-3 closed for at least 5 seconds, reset JP8 1-2 closed to normal position and plug the power cord again.

|  | JP8 |
| --- | --- |
| Normal | 1-2 |
| Reset CMOS | 2-3 |

JP8

#### PCI 1 Slot

123

+

+

Normal(1-2 closed)     Reset CMOS(2-3 closed)

3-1 UPDATE BIOS PROCEDURE

If the BIOS needs to be updated, you can get a CD with the updated BIOS utility in the package. The CD includes :

“awdflash.exe”  --  BIOS update utility program

“awdflash.doc”

The update procedure is as following:

- Boot the system to DOS mode in a normal manner.

- Insert the updated CD to drive D (or E).

- Change working directory to CD-ROM drive, D or E, which contains the update BIOS CD. -- Type “d:\” or “e: \”, then press “ENTER”.

- Type “cd flash”, then press “ENTER”.

- Type “awdflash”, then press “ENTER”-- for running the BIOS update utility.

- Type “(update BIOS file name with version number).bin”, ENTER.

- If you do not want to save the old BIOS, type “N” when the screen displays the message : " Do you want to save BIOS (Y/N) ?".

- Type “Y“ when the screen shows the message : " Are you sure to program (Y/N) ?".

- Follow instructions displayed on the screen. DO NOT remove the update BIOS CD from the CD-ROM drive nor turn the system power off until the BIOS update is completed.

- Turn the power off. Clear the data in CMOS according to the procedure described in the previous page.

- Turn the system power on and test that your system is working properly.

3-1-1 UPDATE MICROCODE API

Intel also provides MICROCODE API (Applications Programming Interface) for mainboard user to update data block in BIOS quickly and easily. (You can find this utility in the All-In-One CD in the mainboard package).

The BIOS code on the mainboards contains data that is specific to each silicon stepping of the processor. Integrators must ensure that this BIOS stepping data matches the used processor stepping. When the BIOS does not contain stepping data that matches the processor stepping, integrators must update the data in the BIOS before shipping the system. Historically, the systems have been updated by replacing the entire BIOS with a new revision of BIOS that contains the correct stepping data.

Intel’s BIOS update API allows just the stepping data within the BIOS to be updated as needed. Mainboards that contain a BIOS with the Intel-defined BIOS update API can be quickly and easily updated, if required, without obtaining a complete BIOS upgrade. Using this utility, integrators can easily verify that the correct stepping data is present in all mainboards. However, if the stepping data requires to be updated, the mainboard BIOS must contain the Intel-defined BIOS update API, otherwise a complete BIOS upgrade is required from the mainboard vendor.

Since API program can only be executed under DOS Real Mode, you must enter Real Mode first and load the API program to Drive C.

To load the program to C by following steps:

- Type “ md c:\ api ” and press Enter. Directory “api” is made in Drive C now.

- Insert the Driver/Utility CD into CD ROM Drive E.

- Then type “ copy e:\api\*.* c:\api ” and press Enter.

(API program is loaded to Drive C now.)

(4) Type “ C:\ api \checkup ” to execute this program.

The main menu should now be displayed with the following four options :

1) Check and load update

2) Specify stepping data file [current : pep.pdb]

3) Help

4) Quit without loading update

Select 1 to know the stepping filename, select 2 to load right patch code, then select 1 to update proper patch code. Now, the screen will show the message “please remove the CD from CD-ROM drive”. Then cold boot (mechanical power off) system to continue. For more information, please refer to “CHECKUP.HLP“ file.

3-2  SYSTEM BIOS CONFIGURATION SETUP

The following pages explain how to set up the BIOS configuration under the Award BIOS. The SETUP program is stored in the Read-Only-Memory (ROM) on the mainboard. To do the SETUP procedure, press the <Del> key when the system is booting up. The following main menu will appear. Please select " STANDARD CMOS FEATURE" to enter the next screen.

CMOS Setup Utility - Copyright ( C )  1984-1998

| Standard CMOS Feature Advanced BIOS Feature Advanced Chipset Feature Integrated Peripherals Power Management Setup PnP/PCI Configurations Frequency/Voltage Control | Load Fail-Safe Defaults Load Optimized Defaults Set Supervisor Password Set User Password Save & Exit Setup Exit Without Saving |
| --- | --- |
| Esc : Quit     : Select Item F10 : Save & Exit Setup |  |
| Time, Date, Hard Disk Type…. |  |

The section on the bottom of the main menu explains how to control this screen.  The other section displays the items highlighted in the list.

※ Control Key Description ※

| UP ARROW | ↑ | Move to the previous item |
| --- | --- | --- |
| DOWN ARROW | ↓ | Move to the next item |
| LEFT ARROW | ← | Move to the item on the left |
| RIGHT ARROW | → | Move to the item on the right |
| Esc KEY | Esc | Main Menu : Quit without saving changes Setup menu : Exit current page and return to main menu |
| PgUp KEY |  | Increase the numeric value or make changes |
| PgDn KEY |  | Decrease the numeric value or make changes |
| F1 KEY | Help | General help |
| F5 KEY | Old Value | Restore the pervious CMOS value from CMOS |
| F6 KEY | Load Fail-Safe Defaults | Load the fail-safe default CMOS value from BIOS default table |
| F7 KEY | Load Optimized Defaults | Load the optimized default |
| F10 KEY | Save & Exit Setup | Save all the CMOS changes and Exit setup, only for Main Menu |

STANDARD CMOS FEATURES  This screen records some basic hardware information, and sets the system clock and error handling. These records can be lost or corrupted if the on-board battery has failed or is weak.

| CMOS Setup Utility – Copyright © 1984-1998 Award Software Standard CMOS Features |  |
| --- | --- |
| Date: Mon, Apr 26 1999 Time: 16:19:20 - IDE Primary Master Press Enter None - IDE Primary Slave Press Enter None - IDE Secondary Master Press Enter None - IDE Secondary Slave Press Enter None Drive A 1.44M, 3.5 in. Floppy 3 mode support Disabled Video EGA/VGA Halt On All Errors Based Memory 640K Extended Memory 64512K Total Memory 65536K | Item Help _____________________ Menu Level  Change the day, month, year and century |
| Move Enter: Select +/-/PU/PD: Value F10:Save ESC: Exit F1:General Help F5:Previous Values F6:Fail-safe defaults F7:Optimized Defaults |  |

Date  The date format is <day>,<date>,<month>,<year>. 	Press<F3> to show the calendar.

| day | The day, from Sun to Sat, automatically changes when you set the date. |
| --- | --- |
| date | The date, from 1 to 31 |
| month | The month, Jan. through Dec. |
| year | The year, from 1900 to 2099 |

Time  The time format is <hour><minute><second>. The time is calculated based on the 24-hour military-time clock. For example, 1p.m. is 13:00:00.

Drive A  This category identifies the types of floppy disk drive A  that has been installed in the computer.

| None | No floppy drive installed |
| --- | --- |
| 360K, 5.25 in | 5.25“ PC-type 360KB capacity |
| 1.2M, 5.25 in | 5.25“ AT-type 1.2MB capacity |
| 720K, 3.5 in | 3.5“ double-side 720KB capacity |
| 1.44M, 3.5 in | 3.5“ double-side 1.44MB capacity |
| 2.88M, 3.5 in | 3.5“ double-side 2.88MB capacity |

Floppy 3 Mode Support  This is the Japanese standard floppy drive. This standard stores 1.2MB in a 3.5” diskette.

Video  This category selects the type of primary video adapter used for the system monitor.

| EGA/VGA | Enhanced Graphics Adapter/Video Graphics Array. For EGA, VGA, SEGA, SVGA or PGA monitor adapters |
| --- | --- |
| CGA 40 | Color Graphics Adapters, power up in 40 column mode |
| CGA 80 | Color Graphics Adapters, power up in 80 column mode |
| MONO | Monochrome adapter, includes high resolution monochrome adapters |

Halt On  This category determines whether the computer will stop if an error is detected during power up.

| No errors | The system boot will not be stopped for any error that may be detected. |
| --- | --- |
| All errors | When the BIOS detects a non-fatal error, the system will be stopped and you will be prompted. |
| All, But Keyboard | The system boot will not stop for a keyboard error. It will stop for all other errors. |
| All, But Diskette | The system boot will not stop for a floppy drive error. It will stop for all other errors. |
| All, But Disk/Key | The system boot will not stop for a floppy drive or keyboard error. It will stop for all other errors. |

Memory  This category is displayed only which is determined by POST (Power On Self Test) of the BIOS.

Base Memory  The POST will determine the amount of base (or conventional) memory installed in the system. The value of the base memory is typically 512K or 640K based on the memory installed on the motherboard.

Extended Memory  How much extended memory is present during the POST. This is the amount of memory located above 1MB in the CPU‘s memory address map.

Total Memory  Displays the total memory available in the system.

IDE Primary Master; Primary Slave Secondary Master; Secondary Slave

Press Enter to enter the sub menu of detailed options.

Ex : IDE Primary Master sub menu

| CMOS Setup Utility – Copyright © 1984-1998 Award Software |  |
| --- | --- |
| IDE Primary Master |  |
| IDE HDD Auto-Detection Press Enter IDE Primary Master Auto Capacity 2557 MB Access Mode Auto Cylinder 4956 Head 16 Precomp 0 Landing Zone 4955 Sector 63 | Item Help Menu Level  To auto-detect the HDD’s size, head... on this channel |
| Move Enter: Select +/-/PU/PD: Value F10:Save ESC: Exit F1:General Help F5:Previous Values F6:Fail-safe defaults F7:Optimized Defaults |  |

IDE HDD Auto-Detection  Press Enter to auto-detect the HDD. If detection is successful, it fills the remaining fields on this menu.

IDE Primary Master  If you select ‘Auto’, the BIOS will detect the HDD & CD-ROM Drive automatically at the POST stage and show the IDE for HDD & CD-ROM Drive. If you select ‘manual’, please contact your hard disk vendor or dealer to get the information listed below. Then enter the figure directly and press <Enter>.

The following options are selectable only if the IDE Primary Master item is set to “Manual”.

| Cylinder | number of cylinders |
| --- | --- |
| Head | number of read/write heads |
| Precomp | write precomp |
| Landing Zone | landing zone |
| Sector | number of sectors per track |

Capacity  Displays your disk drive size.

Access Mode  Choose the access mode for the hard disk. The choices: Normal, LBA, Large, Auto.

ADVANCED BIOS FEATURES   This screen is a list of system configuration options. Some of them are defaults required by the mainboard’s design, others depend on the features of your system.

| CMOS Setup Utility – Copyright © 1984 – 1998 Award Software |  |
| --- | --- |
| Advanced BIOS Features |  |
| Virus Warning Disabled CPU L1 Cache Enabled CPU L2 Cache Enabled Quick Power On Self Test Enabled First Boot device Floppy Second Boot device HDD-0 Third Boot device SCSI Boot other device Enabled Swap Floppy Drive Disabled Boot Up Floppy Seek Enabled Boot Up NumLock Status On Gate A20 Option Fast Typematic Rate Setting Disabled Typematic Rate (Chars/Sec) 6 Typematic Delay (Msec) 250 Security Option Setup OS Select For DRAM > 64MB Non-OS2 HDD S.M.A.R.T Capability Disabled Report NO FDD For Win 95 No | Item Help _______________________ Menu Level  Allows you to choose the VIRUS warning feature for IDE Hard Disk boot sector protection. If this function is enabled and someone attempt to write data into this area, BIOS will show a warning message on screen and alarm beep |
| Move Enter: Select +/-/PU/PD: Value F10:Save ESC: Exit F1:General Help F5:Previous Values F6:Fail-safe defaults F7:Optimized Defaults |  |

| Virus Warning When this item is enabled, the Award BIOS will monitor the boot sector and partition table of the hard disk drive for any attempt at modification. If an attempt is made, the BIOS will halt the system and the following error message will appear. Afterwards, if necessary, you will be able to run an anti-virus program to locate and remove the problem before any damage is done. |  |  |  |
| --- | --- | --- | --- |
|  | ! WARNING ! Disk boot sector is to be modified Type ‘Y’ to accept write or ‘N’ to abort write Award Software, Inc. |  |  |
|  | Enabled | Activates automatically when the system boots up. If anything attempts to access the boot sector or hard disk, partition table will cause a warning message to appear. |  |
|  | Disabled | No warning message will appear when anything attempts to access the boot sector or hard disk partition table. |  |

Many disk diagnostic programs which attempt to access the boot sector table can cause the above warning message. If you will be running such a program, we recommend that you first disable Virus Protection beforehand.

CPU L1 Cache, L2 Cache  These two categories speed up memory access. However, it depends on CPU design. The default value is ‘Enabled‘.

Quick Power On Self Test  This category speeds up Power On Self Test after you power up the computer. If you set “Enabled”, BIOS will skip some items under check during POST.

First Boot Device, Second Boot Device, Third Boot Device, Boot Other Device  The BIOS attempts to load the operating system from the devices in the sequence selected in these items. The choices: Floppy, LS/ZIP/, HDD, SCSI, CDROM, Disabled.

Swap Floppy Drive  This item allows you to determine whether to enable the swap floppy drive or not. The choice : Enabled/ Disabled

Boot Up Floppy Seek  During POST, the BIOS will determine if the floppy disk drive installed is 40 tracks (360K) or 80 tracks (720K, 1.2M, 1.44M)

| Enabled | BIOS searches for floppy disk drive to determine if it is 40 or 80 tracks |
| --- | --- |
| ###### Disabled | BIOS will not search for the type of floppy disk drive by track number |

Boot Up NumLock Status  This allows you to determine the default state of the numeric keypad. By default, the system boots up with NumLock on.

| On | Keypad is for numeric keys |
| --- | --- |
| Off | Keypad is for arrow keys |

Gate A20 Option  This entry allows you to select how the gate A20 is handled. The gate A20 is a device used to address memory above 1 MB. Initially, the gate A20 was handled via a pin on the keyboard. Today, while keyboards still provide this support, it is more common and much faster for the system chipset to provide support for gate A20. Normal is keyboard; Fast is chipset.

Typematic Rate Setting	 This determines if the typematic rate is to be used. When disabled, continually holding down a key on your keyboard will generate only one key instance. In other words, the BIOS will only report that the key is down. When the typematic rate is enabled, the BIOS will report as before, but it will then wait a moment, and, if the key is still down, it will begin the report that the key has been depressed repeatedly. For example, you would use such a feature to accelerate cursor movements with the arrow keys. The choice : Enabled/Disabled

Typematic Rate (Chars/Sec)  When the typematic rate is enabled, this section allows you to select the rate at which the keys are repeated. 6 means 6 characters per second.

Typematic Delay (Msec)  When the typematic rate is enabled, this section allows you to select the delay between when the key was first depressed and when the acceleration begins. 250: 250 msec (500,750,1000)

Security Option  This category allows you to limit access to the system and Setup, or just to Setup.

| System | The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt |
| --- | --- |
| Setup | The system will boot, but access to Setup will be denied if the correct password is not entered at the prompt |

To disable the security, select PASSWORD SETTING at Main Menu, and then you will be asked to enter password. Do not type anything and just press <Enter>, it will disable the security. Once the security is disabled, the system will boot and you can enter Setup freely.

OS Select for DRAM > 64MB  This item allows you to access the memory that is over 64MB in OS/2. The choice : Non-OS2, OS2

HDD S.M.A.R.T. Capability  Select Enabled if your Hard disk supports S.M.A.R.T.(Safe-Monitoring Analysis and Reporting Technology) function.

Report No FDD For WIN 95  Set this item to Yes, BIOS will report FDD to Win95. If in standard CMOS setup, set Drive A to none and set this item to yes. Inside Win95, My Computer and File manager Disk(A:) will show Removable Disk (A:).

ADVANCED CHIPSET FEATURES  This screen controls the setting for the chipset on the mainboard. This chipset manages bus speeds and access to system memory resources.

| CMOS Setup Utility – Copyright © 1984 – 1998 Award Software |  |
| --- | --- |
| Advanced Chipset Features |  |
| SDRAM CAS Latency Time 3 SDRAM Cycle Time Tras/Trc 6/8 SDRAM RAS-to-CAS Delay 3 SDRAM RAS Precharge Time 3 System BIOS Cacheable Enabled Video BIOS Cacheable Enabled Memory Hole At 15M-16M Disabled Delay Transaction Disabled On-Chip Video Window Size 64MB * Onboard Display Cache Setting * CAS# Latency 3 Paging Mode Control Close RAS-to-CAS Override By CAS# LT RAS# Timing Slow RAS# Precharge Timing Slow | Item Help _____________________ Menu Level  |
| Move Enter: Select +/-/PU/PD: Value F10:Save ESC: Exit F1:General Help F5:Previous Values F6:Fail-safe defaults F7:Optimized Defaults |  |

SDRAM CAS Latency Time When synchronous DRAM is installed, the number of clock cycles of CAS latency depends on the DRAM timing. The choices: 2,3. “2” gives faster performance and “3” gives more stable performance. SDRAM Cycle Time Tras/Trc Select the number of SCLKs for an access cycle. “5/7” gives faster performance and “6/8” gives more stable performance. SDRAM RAS-to-CAS Delay This field lets you insert a timing delay between the CAS and RAS strobe signals, used when DRAM is written to, read from, or refreshed. “2” gives faster performance and “3” gives more stable performance. This field applies only when synchronous DRAM is installed in the system. SDRAM RAS Prechage Time If an insufficient number of cycles is allowed for the RAS to accumulate its charge before DRAM refresh, the refresh may be incomplete and the DRAM may fail to retain data. This field applies only when synchronous DRAM is installed in the system. System BIOS Cacheable Selecting Enabled allows the caching of the system BIOS ROM area, resulting in better system performance. However, if any program writes to this memory area, a system error may result.

Video BIOS Cacheable Selecting Enabled allows the caching of the video BIOS ROM area, resulting in better system performance. However, if any program writes to this memory area, a system error may result.

Memory Hole At 15M-16M Certain space in memory can be reserved for ICH/ICH0.

Delay Transaction This chipset has an embedded 32-bit posted write buffer to support delay transactions cycles. Select Enabled to support compliance with PCI specification version 2.1. The choice : Enabled, disabled space

On-chip Video Window Size  Determine the size of memory space can be allocated for on-chip graphics device.

The following items are optional for onboard display cache setting:

CAS# Latency  The number of clock cycles of CAS latency depends on the Display Cache timing. The choices: 2,3. “2” gives faster performance and “3” gives more stable performance.

Paging Mode Control  Close: the GMCH will precharge all during the service of a page miss. Open: it will precharge bank during the service of a page miss.

RAS-to-CAS Override  By CAS# LT: the RAS-to-CAS delay timing will be the same as item of CAS# Latency. Override: the value will be fixed in 2.

RAS# Timing  This item controls the active time of RAS# and bank cycle time. “Fast” gives faster performance and “Slow” gives more stable performance.

RAS# Precharge Timing  If an insufficient number of cycles is allowed for the RAS to accumulate its charge before DRAM refresh, the refresh may be incomplete and the DRAM may fail to retain data. “Fast” gives faster performance and “Slow” gives more stable performance.

INTEGRATED PERIPHERALS  This section page includes all the items of IDE hard drive and Programmed Input/Output features. See also Section “Chipset Features Setup”.

| CMOS Setup Utility – Copyright © 1984 – 1998 Award Software |  |
| --- | --- |
| Integrated Peripherals |  |
| OnChip Primary PCI IDE Enabled OnChip Secondary PCI IDE Enabled IDE Primary Master PIO Auto IDE Primary Slave PIO Auto IDE Secondary Master PIO Auto IDE Secondary Slave PIO Auto IDE Primary Master UDMA Auto IDE Primary Slave UDMA Auto IDE Secondary Master UDMA Auto IDE Secondary Slave UDMA Auto USB Controller Enabled USB Keyboard Support Disabled Init Display First PCI Slot AC97 Audio Enabled AC97 Modem Auto IDE HDD Block Mode Enabled Onboard FDC Controller Enabled Onboard Serial Port 1 3F8/IRQ4 Onboard Serial Port 2 2F8/IRQ3 UART Mode Select Normal IR Duplex Mode Half TxD RxD Polarity Active Lo, Hi Onboard Parallel Port 378/IRQ7 Parallel Port Mode ECP ECP Mode Use DMA 3 Game Port Address 201 Midi Port Address Disabled Midi Port IRQ 7 CPU FAN Speed 0RPM Power FAN Speed 0RPM | Item Help ______________________ Menu Level  If your IDE hard drive supports block mode select Enabled for automatic detection of the optimal number of block read/write per sector the drive can support |
|  Move Enter: Select +/-/PU/PD: Value F10:Save ESC: Exit F1:General Help F5:Previous Values F6:Fail-safe defaults F7:Optimized Defaults |  |

On-Chip Primary PCI IDE; On-Chip Secondary PCI IDE  This setup item allows you to either enable or disable the primary/secondary controller. You might choose to disable the controller if you want to add higher performance or specialized controller.

IDE Primary Master/Slave PIO IDE; Secondary Master/Slave PIO

PIO - Programmed Input/Output, it allows the BIOS to tell the controller what it wants and then let the controller and the CPU to complete the task by themselves. This is simpler and faster. Your system supports five modes, 0 - 4, which primarily differ in timing. When Auto is selected, the BIOS will select the best available mode.

IDE Primary Master/Slave UDMA; IDE Secondary Master/Slave UDMA  Auto, will support the Ultra DMA function. Disabled, will not support the Ultra DMA function.

USB Controller  This item allows you to decide to enable/disable the USB controller.

USB Keyboard Support  Enabled will support USB keyboard in Win95 2.1 and NT 5.0 or above operating system.

Init Display First  This item allows you to decide to active whether PCI VGA or on-chip VGA first. The choice: PCI Slot, Onboard.

AC97 Audio; AC 97 Modem  This item allows you to decide to enable/disable the i810 chipset family to support AC97 Audio/Modem.

IDE HDD Block Mode  This allows your HD controller to use the fast block mode to transfer data to and from your HD drive.

| Enabled | IDE controller uses block mode |  |
| --- | --- | --- |
| Disabled | IDE controller uses standard mode |  |
| On Board FDC Controller This item will enable or disable the floppy disk controller. |  |  |
| On Board Serial Port 1 User can select serial port IRQ. If set to Auto, system will assign an IRQ for it. Note : set to Auto is not recommended. |  |  |
| On Board Serial Port 2 User can select serial port IRQ. If set to Auto, system will assign an IRQ for it. Note : set to Auto is not recommended. |  |  |
| UART Mode Select This lets you select the Infrared mode. Choices are Standard, HPIR, and ASKIR. If you choose HPIR or ASKIR mode, the screen will show another two lines to let you choose ‘IR Function Duplex’ (Full or Half) and ‘TxD RxD Polarity Active’ (Hi Lo; Lo Hi; Hi Hi; Lo Lo). On Board Parallel Port Let user select IRQ for parallel port When Disabled, the parallel port will be disabled. Onboard Parallel Mode Let user select error check mode. This item is not recommended to change except user has special request. |  |  |

ECP Mode Use DMA  Select a DMA channel for the port. Choices are 3, 1.

Game Port Address; Midi Port Address  The IO address of Game Port/Midi Port.

CPUFAN / PowerFAN Speed  It shows the running speed of the CPU fan and Power fan. The figure will be changing when the system is running. If you do not install the fan, the figure will show 0.

POWER MANAGEMENT SETUP  This screen controls the 'green' features of this mainboard.

| CMOS Setup Utility – Copyright © 1984 – 1998 Award Software |  |
| --- | --- |
| Power Management Setup |  |
| ACPI function Enabled Power Management User Define Video Off Method DPMS Video Off In Suspend Yes Suspend Type Stop Grant MODEM Use IRQ 3 Suspend Mode Disabled HDD Power Down Disabled Soft-Off by PWRBTN Instant Off CPU PME# Function Disabled Power On by Ring Enabled Resume by Alarm Disabled ** Reload Global Timer Events ** Primary IDE 0 Disabled Primary IDE 1 Disabled Secondary IDE 0 Disabled Secondary IDE 1 Disabled FDD, COM, LPT Port Disabled PCI PIRQ [A-D]# Disabled | Item Help ____________________ Menu Level  |
| Move Enter: Select +/-/PU/PD: Value F10:Save ESC: Exit F1:General Help F5:Previous Values F6:Fail-safe defaults F7:Optimized Defaults |  |

ACPI function  This item is to set the ACPI ( Advanced Configuration Power Interface ) function enabled or disabled.

Power Management  This category allows you to select the type (or degree) of power saving and is directly related to the following modes : Suspend; HDD Power Down.

| ###### Disabled | No power management. It means all 4 modes are disabled. |
| --- | --- |
| Min. Power Saving | Minimum power management. Doze =1hr.; Standby=1hr.; Suspend=1hr.; HDD Power Down=15min |
| Max. Power Saving | Maximum power management only available for SL CPU. Doze=1min.; Standby=1min.; Suspend=1min.;HDD Power Down=1min |
| User Define | Allows you to set each mode individually. When not disabled, each of the ranges are from 1min. to 1hr. except for HDD Power Down which ranges from 1 to 15min. and disable |

| Video Off Method This determines the manner in which the monitor is blanked. |  |
| --- | --- |
| V/H SYNC + Blank | This selection will cause the system to turn off the vertical and horizontal sync. ports and write blanks to the video buffer. |
| Blank Screen | This option only writes blanks to the video buffer. |
| DPMS | Initial of “Display Power Management Signaling”. |

Video off In Suspend  This determines the manner in which the monitor is blanked.

Suspend Type  Select the suspend type.

| Stop Grant | Only CPU enters sleeping mode |
| --- | --- |
| Full on Suspend | Both CPU and chipset enter sleeping mode |

MODEM Use IRQ  This item determines the IRQ in which the MODEM can be used. The choice : 3,4,5,7,9,10,11,NA.

Suspend Mode  When enabled and after the set time of system inactivity, all devices except the CPU will be shut off.

HDD Power Down When enabled and after the set time of system inactivity, the hard disk drive will be powered down while all other devices remain active.

Soft-Off by PWR-BTTN Instant-off : When push the power button, the system power will be off immediately. Delay 4 sec : when push the power button, it will enter suspend mode. We need to push the power button and hold it for 4 seconds to turn off the power.

PCI PME# Function With PCI card supporting PCI Power Management capability and this item setting to Enabled, the system can be waked up by PCI card. PowerOn by Ring Enabled: when system is in suspend mode, it can be waked up by modem. Disabled: it cannot be waked up by modem. Resume by Alarm When Enabled, two additional lines will be added to the screen :Date (of Month) Alarm; Time (hh:mm:ss) Alarm to let user set the desired date and time. After power off, the system will automatically power on at the specified date and time. Reload Global Timer Events When enabled, an event occurring on each device listed below restarts the global time for Standby mode. IRQ [3-7, 9-15], NMI; Primary IDE 0; Primary IDE 1; Secondary IDE0; Secondary IDE1; Floppy Disk; Serial Port; Parallel Port.

PNP/PCI CONFIGURATION   This screen configures the PCI Bus slots.

| CMOS Setup Utility – Copyright © 1984-1998 Award Software |  |
| --- | --- |
| PnP/PCI Configurations |  |
| Reset Configuration Data Disabled Resources Controlled By Auto - IRQ Resources Press Enter - DMA Resources Press Enter - Memory Resources Press Enter PCI/VGA Palette Snoop Disabled | Item Help -------------------- Menu Level  Default is Disabled. Select Enabled to reset Extended System Configuration Data(ESCD) when you exit Setup if you have installed a new add-on and the system reconfiguration has caused such a serious conflict that the OS cannot boot |
| Move Enter: Select +/-/PU/PD: Value F10:Save ESC: Exit F1:General Help F5:Previous Values F6:Fail-safe defaults F7:Optimized Defaults |  |

Reset Configuration Data This item allows you to determine whether to reset the configuration data or not.

Resource Controlled by  The Award Plug and Play BIOS has the capability to automatically configure all of the boot and Plug & Play compatible devices. However, this capability means absolutely nothing unless you are using a Plug and Play OS such as Windows 95. Choices are Auto and Manual.

IRQ Resources; DMA Resources  When resources are controlled manually, assign each system interrupt/DMA channel a type, depending on the type of device using the interrupt/DMA channel.

IRQ3/4/5/7/9/1011/12/14/15, DMA0/1/3/5/6/7 Assign to  This item allows you to determine the IRQ/DMA assigned to the ISA bus and is not available to any PCI slot. Choices are Legacy ISA and PCI/ISA PnP.

PCI/VGA Palette Snoop  It determines whether the MPEG ISA/VGA cards can work with PCI/VGA or not.

| Enabled | When PCI/VGA working with MPEG ISA/VGA Card |
| --- | --- |
| Disabled | When PCI/VGA not working with MPEG ISA/VGA Card |

FREQUENCY / VOLTAGE CONTROL  This screen controls the 'green' features of this mainboard.

| CMOS Setup Utility – Copyright © 1984-1998 Award Software |  |
| --- | --- |
| Frequency/Voltage Control |  |
| Auto Detect DIMM/PCI CLK Disabled Spread Spectrum Modulated Disabled CPU/SDRAM/PCI Clock Default CPU Ratio X 3 | Item Help -------------------- Menu Level  |
|  Move Enter: Select +/-/PU/PD: Value F10:Save ESC: Exit F1:General Help F5:Previous Values F6:Fail-safe defaults F7:Optimized Defaults |  |

Auto Detect DIMM/PCI ClK If this item is enabled, the unused DIMM and PCI slot clock will be disabled. If this item is disabled the unused DIMM and PCI slot will still get the active clock signal. Spread Spectrum Enable / Disable this item, the BIOS will Enable / Disable the clock generator spread spectrum .

CPU/SDRAM/PCI Clock Use this item to set CPU base clock frequency which includes 70/105/35、75/112/37、83/125/41、90/90/30、95/95/31、100/100/33、101/101/33、105/105/35、114/114/38、120/120/40、124/124/41、133/133/33、133/133/44、140/140/35 and 150/150/37MHz. Note: After you change the CPU Clock Frequency and the system can not start, please do the following procedures: - Turn the system off firstly. - Turn on the system. Then press and hold the “ Insert ” key at boot. - Select the proper figure in the item of CPU/SDRAM/PCI Clock. Save and Exit Setup. CPU Ratio Use this item to set CPU ratio which includes x 3、x 3.5、x 4、x 4.5、x 5、x 5.5、x 6、x 6.5、x 7、x 7.5 and x 8.

LOAD FAIL-SAFE DEFAULTS  When your mainboard has problems and needs to trouble shoot the system, you can use this function. The default values loaded only affect the BIOS Features Setup, Chipset Features Setup, Power Management Setup and PNP/PCI Configuration Setup. There is no effect on the Standard CMOS Setup. To use this function, select it from main menu and press <Enter>. A line will appear on the screen asking if you want to load the fail-safe default values. Press <Yes> and <Enter> then the BIOS default values will be loaded.

LOAD OPTIMIZED DEFAULTS	 This allows you to load optimal settings which are stored in the BIOS ROM. The default values loaded only affect the BIOS Features Setup, Chipset Features Setup, Power Management Setup and PNP/PCI Configuration Setup. There is no effect on the Standard CMOS Setup. To use this function, select it from main menu and press <Enter>.	A line will appear on the screen asking if you want to load the optimized default values. Press <Yes> and <Enter> then the Setup default values will be loaded.

SUPERVISOR PASSWORD / USER PASSWORD  This allows you to set the password. The mainboard defaults with password disabled. If you set both supervisor and user passwords, only the	supervisor password allows you to enter the BIOS SETUP program.

Enter/Change password : Enter the current password. And at the prompt, key-in your new password (up to eight alphanumeric characters), then press <Enter>. At the next prompt, confirm the new password by typing it again and press <Enter>.

Disable password : Press the <Enter> key instead of entering a new password when the ‘Enter Password’ dialog box appears. A message will appear for confirming that the password is disabled.

|  | CAUTION: If you forget your password, you must clear CMOS first and restart system in order to set up password again. See “Reset CMOS” on Page 12. |
| --- | --- |

SAVE & EXIT SETUP	 This allows you to save the new setting values in the CMOS memory and continue with the booting process. If you select this item and press 'Y', then these records will be saved in the CMOS memory on the mainboard. It will be checked every time you turn your computer on.

EXIT WITHOUT SAVING	 This allows you to exit the BIOS setup utility without recording any new values or changing old ones.