Table of Contents

JUMPER LOCATION DIAGRAM	A

CONNECTOR LOCATION DIAGRAM	B

CHAPTER 1   INTRODUCTION	2

1-1 SYSTEM FEATURES	3

1-2 CHECK LIST OF THE PACKING	4

CHAPTER 2   INSTALLATION	5

2-1 CPU INSTALLATION	5

2-2 SYSTEM MEMORY INSTALLATION	8

2-3 SRAM INSTALLATION	9

2-4 OTHER JUMPERS SETTING	10

2-5 IDE DRIVER INSTALLATION	11

CHAPTER 3   AWARD BIOS SETUP	12

3-1 UPDATE BIOS PROCEDURES	13

3-2 AWARD SYSTEM BIOS CONFIGURATION SETUP	14

APPENDIX A 	22

CHAPTER 1  INTRODUCTION

This manual introduce how to set-up and upgrade this mainboard. It's an overview of the layout and features of the mainboard, and provides necessary information for you to change the configuration

or system environment.

This manual is divided into two sections :

PART ONE includes page A and page B about this mainboard's layout diagram. You should refer them when you set-up and

upgrade your system hardware.

PART TWO includes three chapters as following:

Chapter 1 is an overview of the mainboard features and this

mainboard's packing contents.

Chapter 2 explains how to upgrade and change hardware

configuration including memory, CPU, and lists jumper settings

and connectors.

Chapter 3 is the user's guides of AWARD BIOS setup utility,

and Flash ROM BIOS update procedure. The menu showed in

this chapter are default setting.

Your system dealer will set up the mainboard according to your demand of computer. It means your mainboard's current setting may not be the same as the defaults shown in this user's manual.  If you would like to change your configuration, please ask your

dealer firstly, be sure this will not against your system warranty. Or

ask for your dealer to do it for you.

Intel is a registered trademark of Intel Corporation.

REMARK

MS-DOS® is a registered trademark of Microsoft Corporation.

IBM ,IBM PC ,IBM AT and IBM XT are registered trademark of  International

Business Machines Corporations.

All other brands and product names are trademarks registered trademarks of their respective companies.

1-1 SYSTEM FEATURES

: 	CPU	Supports INTEL Pentium up to 200MHz and later,

and AMD5k86 CPUs with 1 ZIF socket 7.

:  	Chipset	INTEL 82430FX PCIset and Winbond high-speed

multi I/O chipset.

: 	SRAM	Supports two Synchronous SRAM chips on board

and one Synchronous COAST (Cache On A STick)

slot for Pipeline Burst SRAM module.

: 	SRAM	Supports maximum 512KB Synchronous SRAM.

On board 256KB and upgradable by using CAOST

module size of 256KB / 512KB.

: 	DRAM	Using four 72-pin SIMM sockets, provides two banks

of 64-bit wide path up to 128MB addressing page

mode DRAMs.

: 	DRAM	Supports EDO DRAM and mixing of EDO SIMMs

with fast page mode DRAM SIMMs among different 		banks.

: 	Slot	Four PCI master slots and four 16-bit ISA slots.

: 	IDE	Dual IDE connectors support up to four devices

in two channels for connection of high capacity

hard disk, CD-ROM, tape backup etc.. PCI IDE

controller supports PIO mode 0 to mode 4. Auto

detecting of installed IDE devices through BIOS utility.

: 	Green	"True" Green power management for CPU stop 			clock state.

: 	I/O	On board high-speed Multi-I/O connectors:

- one FDC port supports two devices up to 2.88MB

- two 16550A fast UARTs compatiable serial ports

- one EPP/ECP mode parallel port

The selections of all primary and secondary ISA

addressing of   COM1 - 4, LPT 1-3 and FDC disable

or enable from BIOS SETUP configuration please

refer to "CHIPSET FEATURES SETUP" in the 			chapter three.

: 	Dimension	Board dimension : 220mm x 260mm , 8.66" x 10.23"

1-2 CHECK LIST OF THE PACKING

The mainboard comes securely packed in a durable box and shipping carton. If any of the above items are missed or damaged, please contact your system dealer.

Each mainboard containing:

Q'TY     	Description

1                 Mainboard 	: ATC-1000.

1	Diskette	: Enhanced IDE driver (3.5").

1                	Cable 	: Enhanced IDE connector.

1               	Cable 	: F.D.D. connector.

1	Cable 	: Serial port.

1               	Cable 	: Serial/Parallel.

1             	Manual 	: User`s manual.

NOTE : Leave the mainboard in its original packing until you are ready to install it.

CHAPTER 2  INSTALLATION

2-1 CPU  INSTALLATION

ATC-1000 can support INTEL Pentium up to 200MHz and later, and AMD 5k86 CPUs. For installation, please notice CPU pin 1

must align with the ZIF socket 7 Pin 1 location.

2-1-1 CPU TYPE SELECTION

CPUJP13

INTELSTD*open (3.3V)

VRE*close (3.45V)

AMD5k86close (3.45V)

*	STD 	◆3.135V - 3.600V

VRE 	◆3.40V - 3.60V

(The fourth line of the mark on the underside of the

processor contains a code that identifies the voltage

level type. V is VRE, S is standard.)

2-1-2 CPU CLOCK SETTING

A. INTEL PENTIUM CPU

INTERNAL CPU CLOCKJP9JP10JP11JP12Ext.x Fre.

75MHzclosecloseopenopen50x1.5

90MHzcloseopenopenopen60x1.5

100MHzopencloseopenopen66x1.5

120MHzcloseopencloseopen60x2.0

133MHzopenclosecloseopen66x2.0

150MHzcloseopencloseclose60x2.5

166MHzopenclosecloseclose66x2.5

180MHzcloseopenopenclose60x3.0

200MHzopencloseopenclose66x3.0

B. AMD 5k86 CPU

INTERNAL CPU CLOCKJP9JP10JP11JP12Ext.x Fre.

AMD5k86-P75

SSA/5-75ABRclosecloseopenopen50x1.5

AMD5k86-P90

SSA/5-90ABQcloseopenopenopen60x1.5

AMD5k86-P100

SSA/5-100ABQ＊opencloseopenopen66x1.5

※ The AMD-K5 series CPUs are dual voltage processors, please

contact your dealer for jumper setting.

＊ If AMD provides dual voltage of AMD5k86-P100 SSA/5-100xxx,

please contact your dealer for jumper setting.

2-1-3 CPU CLOCK SETTING

The following setting is for CPU speed higher than 2-1-1.

EXTERNAL CPU CLOCKJP9JP10

50(MHz)closeclose

60(MHz)closeopen

66(MHz)openclose

INTERNAL CPU CLOCKJP11JP12

INTEL EXTERNAL CLOCK X 1.5openopen

INTEL EXTERNAL CLOCK X 2.0closeopen

INTEL EXTERNAL CLOCK X 2.5closeclose

INTEL EXTERNAL CLOCK X 3.0openclose

2-2 SYSTEM MEMORY INSTALLATION

ATC-1000 provides four 72-pin SIMM sockets for system memory expansion from 4MB to 128MB. These four SIMMs are arranged to two banks, Bank0 (SIMM 3, 4) and  Bank1(SIMM 1, 2), please refer to page A,B. Each bank provides 64-bit wide data path.

This mainboard can accept either Fast Page Mode or EDO Mode (Extended Data Out) DRAM, with a speed at least 70 nanosecond. You should plug DRAM modules into two sockets (same bank) or four sockets at one time. Each pair of modules must be the same size, type and speed; no matter single-side or double-side module. This mainboard supports mixing of EDO SIMMs with fast page

mode DRAM SIMMs among different banks.

※ System Memory Combinations Options ※

BANK0BANK1Total Memory

SIMM 3, 4SIMM 1, 2SIMM 1- 4

2MBx2-4MB

-2MBx24MB

4MBx2-8MB

-4MBx28MB

8MBx2-16MB

-8MBx216MB

4MBx24MBx216MB

16MBx2-32MB

-16MBx232MB

8MBx28MBx232MB

32MBx2 -64MB

-32MBx264MB

16MBx216MBx264MB

32MBx232MBx2128MB

2-3 SRAM  INSTALLATION

ATC-1000 is built-in 256KB Sync. Pipeline Burst SRAM on board and provides a Sync. SRAM module in COAST slot for further

expansion.

The on board TAG SRAM ( location U24) is for on board Sync. SRAM only. The upgraded Pipeline Burst SRAM module must use

the kind of module with TAG SRAM.

※ System Memory Combinations Options ※

SRAM TYPESRAM SIZEDATA SRAMTAG SRAM

Synchronous

on board256KB 32K32 x 2pcs  8K8,16K8 or

32K8 x 1pc

Synchronous module256KB 32K32 x 2pcs 16K8 or

32K8 x 1pc

※ Upgrade Procedure :

For upgrading your Sync. SRAM from 256KB to 512KB by using COAST Module, if the KIT-256 has TAG SRAM already, you should remove the Tag RAM Chip (U24) firstly, then plug-in the KIT-256 module. If the KIT-256 has no TAG SRAM, and the on board TAG SRAM is 16K8 or 32K8, then just plug-in the KIT-256 module. (If the on board TAG SRAM is 8K8, then you should change it to 16K8

or 32K8 firstly.)

You must use KIT-256 for upgrading your system to 512KB Sync. SRAM , please contact your system dealer about its information.

There are no jumper setting and BIOS value adjustments for

upgrading the SRAM size.

2-4 OTHR JUMPERS AND CONNECTORS

The location of following jumpers and connectors please refer to page A and B.

ConnectorFunctionRemark

J2 IDE LED Pin 1 is '+'  **

J3 Turbo LED Pin 1 is '+'  **

J5 Reset  Hardware Reset Switch

J6 KEY-LOCK KeyLock Connector

J7 SPK PC Speaker Connector

J8 FAN +12V Cooling Fan

Connector, Pin 1 is '+' *

※Turbo Switch is fixed to open mode for the TURBO function.

*  If you plugged wrong side, the cooling fan should be failed.

** If you plugged wrong side, the light will be weaker, but the IDE

devices are still workable.

JumperFunctionRemark

JP5 ATBUS CLK 1-2: PCI CLK/3 for external

CPU clock 50MHz

2-3: PCI CLK/4 for external

CPU clock 60, 66MHz

JP8 CMOS open: Normal＊

close: Clear CMOS

＊is default setting

＊＊is factory default setting (fixed)

ConnectorFunctionRemark

CN1 PS/2 Mouse Optional

CN2 COM 1 Serial Port Cable

Connector

CN3 COM 2 Serial Port Cable

Connector

CN4 FDC Floppy Disk Drive Cable

Connector

CN5 LPT Parallel Port Cable

Connector

CN7 IDE1 Primary 1st IDE Drive Cable

Connector

CN6 IDE2 Secondary 2nd IDE Drive Cable

Connector

※  When you plug a cable into the above I/O connectors, you should let the pin 1 edge of the cable is at the pin 1 end of the connector.

2-5 IDE DRIVER INSTALLATION

The IDE driver installation procedure is printed on the label of diskette as following :

Setup for DOS/Windows :

1. Starting MS-Windows 3.1 (or 3.11)

2. Select "RUN", then type "A:\drivers\setup.exe"

3. Exit MS-Windows, turn power off; then turn power on.

CHAPTER 3  AWARD BIOS

SETUP

This chapter explains the system BIOS setup, and how to update new BIOS. All BIOS screens showed on following pages are default values, your system dealer will set up these values according to

your demand of computer.

ATC-1000 uses Flash ROM (+5V) to make BIOS easier to be updated by the floppy disk-based program. and to committe

Microsoft Windows 95 plug & play feature.

※  JP8 Setting is for Update System BIOS

openNORMAL＊

closeCLEAR CMOS

＊is default setting

3-1 UPDATE BIOS PROCEDURES

If the BIOS should be updated, you can get the new BIOS diskette from your system dealer. The BIOS diskette includes 3 files they are "awdflash.exe" an execution file, it's BIOS update utility; the new version BIOS file with a file name about its version number; and "readme.txt" a text readme file. The update procedures are as

following:

1. Boot from hard disk drive C, then change to floppy A (or B).

2. Insert the updated diskette to drive A (or B), press "awdflash" to

run the BIOS update utility.

3. Press new BIOS file name to install the new BIOS.

4. Press 'N' when the screen shows the message : " Do you want to

save BIOS (Y/N) ?".

5. Press 'Y' when the screen shows the message : " Are you sure to

program (Y/N) ?".  Then the BIOS updated is finished.

6. Exit the utility and turn off the power, set JP8 to 'close' to

clear the CMOS. After 10 seconds, set JP8 to 'open' (remove

the jumper cap), become the normal status.

7. Turn the power on and test your system workable or not.

8. If your installation fail, re-install the new BIOS again as above

procedures. Or call your system dealer for help.

3-2 AWARD SYSTEM BIOS CONFIGURATION

SETUP

The following pages explain how to set up the system configuration (CMOS) under the AWARD BIOS. The SETUP program is stored in the Read-Only-Memory (ROM) on the mainboard. To do the SETUP procedure, press the <Del> key when the system is booting up. The following main menu will appear.

Please select " STANDARD CMOS SETUP" to enter the next

screen.

ROM PCI/ISA BIOS (2A59CA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUPPASSWORD SETTING

BIOS FEATURES SETUPIDE HDD AUTO DETECTION

CHIPSET FEATURES SETUPSAVE & EXIT SETUP

POWER MANAGEMENT SETUPEXIT WITHOUT SAVING

PCI CONFIGURATION  SETUP

LOAD BIOS DEFAULTS

LOAD SETUP DEFAULTS

ESC: Quitáâàß:Select Item

F10: Save & Exit Setup(Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

The section on the bottom of the main menu explains how to control this screen.  The other one section displays the items highlighted in the list.

This screen records, some basic hardware information, set the

system clock and error handling. These records can be lost or

corrupted if the on-board battery  is fail or  weak.

ROM PCI/ISA BIOS (2A59CA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUPPASSWORD SETTING

BIOS FEATURES SETUPIDE HDD AUTO DETECTION

CHIPSET FEATURES SETUPSAVE & EXIT SETUP

POWER MANAGEMENT SETUPEXIT WITHOUT SAVING

PCI CONFIGURATION  SETUP

LOAD BIOS DEFAULTS

LOAD SETUP DEFAULTS

ESC: Quitáâàß:Select Item

F10: Save & Exit Setup(Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

ROM PCI/ISA BIOS (2A59CA29)

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

Date (mm:dd:yy)  : Sat, Jan  6 1996

Time(hh:mm:ss)   : 13 : 37 : 14

HARD DISKS      TYPE    SIZE  CYLS  HEAD  PRECOMP LANDZ SECTOR MODE

Primary Master       : AUTO   0       0           0            0                 0            0        AUTO

Primary Slave         : AUTO   0       0           0            0                 0            0        AUTO

Secondary Master  : AUTO   0       0           0            0                 0            0          AUTO

Secondary Slave    : AUTO   0       0           0            0                 0            0           AUTO

Drive A : 1.44M,  3.5  in. Base Memory           :        640K

Drive B : None Extended memory    :   64512K

Other Memory            :        384K

Video  : EGA/VGA  ------------------------------------------------

Halt On: All Errors   Total Memory             :   65536K

ESC  :  Quit                 áâàß:Select Item                 PU/PD/+/- : Modify

F1     :  Help                (Shift) F2 : Change Color

This screen is a list of system configuration options. Some of them are defaults required by the mainboard's design, others depend on the features of your  system.

ROM PCI/ISA BIOS (2A59CA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUPPASSWORD SETTING

BIOS FEATURES SETUPIDE HDD AUTO DETECTION

CHIPSET FEATURES SETUPSAVE & EXIT SETUP

POWER MANAGEMENT SETUPEXIT WITHOUT SAVING

PCI CONFIGURATION  SETUP

LOAD BIOS DEFAULTS

LOAD SETUP DEFAULTS

ESC: Quitáâàß:Select Item

F10: Save & Exit Setup(Shift) F2 : Change Color

Virus, Protection, Boot Sequence

ROM PCI/ISA BIOS(2A59CA29)

BIOS FEATURES SETUP

AWARD SOFTWARE , INC.

Virus Warning: DisabledVideo BIOS Shadow: Enabled

CPU Internal Cache: EnabledC8000-CBFFF Shadow: Disabled

External Cache: EnabledCC000-CFFFF Shadow: Disabled

Quick Power On Self Test: DisabledD0000-D3FFF Shadow: Disabled

Boot Sequence: A, CD4000-D7FFF Shadow: Disabled

Swap Floppy Drive: DisabledD8000-DBFFF Shadow: Disabled

Boot Up Floppy Seek: EnabledDC000-DFFFF Shadow: Disabled

Boot Up NumLock Status: On

Boot Up System Speed: High

Gate A20 Option: Fast

Memory Parity Check: Enabled

Typematic Rate Setting: Disabled

Typematic Rate(Chars/Sec): 6Esc : Quit       áâàß:Select Item

Typematic Delay(Msec): 250F1   : Help           PU/PD/+/- : Modify

Security Option: SetupF5  : Old Values  (SHIFT)F2 : Color

PS/2 mouse function control: EnabledF6   : Load BIOS Defaults

PCI/VGA Palette Snoop: DisabledF7   : Load Setup Defaults

OS/2 select for DRAM>64MB: Non-OS2

This screen controls the setting for the chipset on the mainboard.

ROM PCI/ISA BIOS (2A59CA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUPPASSWORD SETTING

BIOS FEATURES SETUPIDE HDD AUTO DETECTION

CHIPSET FEATURES SETUPSAVE & EXIT SETUP

POWER MANAGEMENT SETUPEXIT WITHOUT SAVING

PCI CONFIGURATION  SETUP

LOAD BIOS DEFAULTS

LOAD SETUP DEFAULTS

ESC: Quitáâàß:Select Item

F10: Save & Exit Setup(Shift) F2 : Change Color

AT Clock, DRAM Timings, ....

ROM PCI/ISA BIOS (2A59CA29)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

DRAM  RAS# Precharge Time: 4PCI Concurrency :  Enabled

DRAM R/W Leadoff Timing: 8/6*PCI Streaming    :  Enabled

DRAM  RAS To CAS Delay: 3*PCI Bursting:  Enabled

DRAM Read Burst Timing: x2222*Onboard FDC Control:  Enabled

DRAM Write Burst Timing: x3333*Onboard Serial Port  1:  COM1/3F8

System BIOS Cacheable: DisabledOnboard Serial Port  2:  COM2/2F8

Video BIOS Cacheable: DisabledOnboard Parallel Port:  378H/IRQ7

8-bit I/O Recovery Time: 3*Onboard Parallel Mode:  EPP/SPP

16-bit I/O Recovery Time: 2*

Memory Hole At 15M-16M: DisabledParallel Port EPP Type:  EPP1.9

IDE HDD Block Mode: Enabled

IDE 32-bit Transfer Mode: Enabled

IDE Primary Master PIO: Auto

IDE Primary Slave PIO: Auto

IDE Secondary Master PIO: AutoEsc: Quit:Select Item

IDE Secondary Slave PIO: AutoF1  : HelpPU/PD/+/-:Modify

On-Chip Primary PCI IDE: EnabledF5  : Old  Values(Shift)F2  :Color

On-Chip Secondary PCI IDE: EnabledF6  :Load BIOS Defaults

PCI Slot IDE 2nd Channel: EnabledF7  :Load Setup Defaults

This screen controls the 'green' features of this mainboard.

ROM PCI/ISA BIOS (2A59CA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUPPASSWORD SETTING

BIOS FEATURES SETUPIDE HDD AUTO DETECTION

CHIPSET FEATURES SETUPSAVE & EXIT SETUP

POWER MANAGEMENT SETUPEXIT WITHOUT SAVING

PCI CONFIGURATION  SETUP

LOAD BIOS DEFAULTS

LOAD SETUP DEFAULTS

ESC: Quitáâàß:Select Item

F10: Save & Exit Setup(Shift) F2 : Change Color

Sleep Timer, Suspend Timer, .....

ROM PCI/ISA BIOS (2A59CA29)

POWER MANAGEMENT SETUP

AWARD SOFTWARE, INC.

Power Management: DisabledIRQ3  (COM 2): OFF

PM Control by APM: YesIRQ4  (COM 1): OFF

Video Off Method: V/H SYNCIRQ5  (LPT 2): OFF

+BlankIRQ6  (Floppy Disk): OFF

IRQ7  (LPT 1): OFF

Doze Mode: DisabledIRQ8  (RTC Alarm): OFF

Standby Mode: DisabledIRQ9  (IRQ2 Redir): OFF

Suspend Mode: DisabledIRQ10 (Reserved): OFF

HDD Power Down: DisabledIRQ11 (Reserved): OFF

IRQ12 (PS/2 Mouse): OFF

IRQ3  (Wake-Up Event): OFFIRQ13 (Coprocessor): OFF

IRQ4  (Wake-Up Event): OFFIRQ14 (Hard Disk): OFF

IRQ8  (Wake-Up Event): OFFIRQ15 (Reserved): OFF

IRQ12(Wake-Up Event): OFF

Esc: Quit     áâàß:Select Item

Power Down ActivitiesF1  : Help           PU/PD/+/- : Modify

COM Ports Accessed: OFFF5  : Old Values  (Shift) F2: Color

LPT Ports Accessed: OFFF6  : Load BIOS Defaults

Drive Ports Accessed: OFFF7  : Load Setup Defaults

This screen configures the PCI Bus slots.

ROM PCI/ISA BIOS (2A59CA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUPPASSWORD SETTING

BIOS FEATURES SETUPIDE HDD AUTO DETECTION

CHIPSET FEATURES SETUPSAVE & EXIT SETUP

POWER MANAGEMENT SETUPEXIT WITHOUT SAVING

PCI CONFIGURATION  SETUP

LOAD BIOS DEFAULTS

LOAD SETUP DEFAULTS

ESC: Quitáâàß:Select Item

F10: Save & Exit Setup(Shift) F2 : Change Color

IRQ Settings, Latency Timers, ......

ROM PCI/ISA BIOS (2A59CA29)

PCI CONFIGURATION SETUP

AWARD SOFTWARE, INC.

PnP BIOS Auto-Config: Disabled

Slot  1  Using INT#: AUTO

Slot  2  Using INT#: AUTO

Slot  3  Using INT#: AUTO

Slot  4  Using INT#: AUTO

1st  Available IRQ: 10

2nd Available IRQ: 11

3rd  Available IRQ: 9

4th  Available IRQ: 5

PCI IRQ Actived By: Level

PCI IDE IRQ Map To: PCI-AUTO

Primary     IDE INT#: AEsc: Quit      áâàß:Select Item

Secondary IDE INT#: BF1  : Help               PU/PD/+/- : Modify

F5  : Old Values (Shift) F2: Color

F6  : Load BIOS Defaults

F7  : Load Setup Defaults

The last step is 'save and exit'. If you select this item and press 'Y',

then these  records will be saved in the CMOS memory on the

mainboard. It will be checked every  time when you turn your computer

on.

ROM PCI/ISA BIOS(2A59CA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUPPASSWORD SETTING

BIOS FEATURES SETUPIDE HDD AUTO DETECTION

CHIPSET FEATURES SETUPSAVE & EXIT SETUP

POWER MANAGEMENT SETUPEXIT WITHOUT SAVING

PCI CONFIGURATION  SETUP

LOAD BIOS DEFAULTS

LOAD SETUP DEFAULTS

ESC: Quitáâàß:Select Item

F10: Save & Exit Setup(Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

ROM PCI/ISA BIOS (2A59CA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUPPASSWORD SETTING

BIOS FEATURES SETUPIDE HDD AUTO DETECTION

CHIPSET FEATURES SETUPSAVE & EXIT SETUP

POWER MANAGEMENT SETUPEXIT WITHOUT SAVING

PCI CONFIGURATION  SETUPSAVE to CMOS and EXIT (Y/N):Y

LOAD BIOS DEFAULTS

LOAD SETUP DEFAULTS

ESC: Quitáâàß:Select Item

F10: Save & Exit Setup(Shift) F2 : Change Color

Auto-Configure HDD: Sector, Cylinder, Head .....

※ Control Key Description ※

UP ARROW↑Move to previous item

DOWN ARROW↓Move to next item

LEFT ARROW←Move to the item in the left hand

RIGHT ARROW→Move to the item in the right hand

Esc KEYEscMain Menu : Quit and not save changes

Setup menu : Exit current page and return to main menu

PgUp KEYIncrease the numeric value or make changes

PgDn KEYDecrease the numeric value or make changes

F1 KEYHelpGeneral help

F2 KEY＜Shift＞+F2Change color from total 16 colors

F5 KEYOld ValueRestore the pervious CMOS value from CMOS

F6 KEYLoad BIOS defaultLoad the default CMOS value from BIOS default table

F7 KEYLoad setup defaultLoad Setup default

F10 KEYSave & Exit SetupSave all the CMOS changes and Exit setup, only for Main Menu

## APPENDIX A

### ※※TECHNICAL SUPPORT REQUEST FORM※※

If the mainboard doesn't function properly, please complete the following

informationand return it to your system dealer. If the further information is

needed, please attach this separating sheets.

Model No :  ATC-1000      Date of  Purchase :  ______________

Serial No : ___________________

HARDWARE :

BRANDMODELSPEEDQ'TY

CPU

Co-processor

SIM Module

TAG SRAM

Sync. Cache

Chipset Brand  :__________  Model  :__________

Cache Memory :________KB  SIMM  :_______MB (__ EDO, __FastPage)

Bus Interface :  PCI x ______ ,   ISA x ______

Hard Disk Interface Controller : __ __ IDE, ____ SCSI

Hard Disk Brand :___________, Model :_________, Capacity :________

Display Controller Brand :_____________,Model :__________

Controller Chip Brand :_____________,Model :__________

SOFTWARE:

SYSTEM BIOS:____ <A>AMI, <W>AWARD

Version _________  Date Code _____________

Keyboard BIOS: Brand __________

Other Add-on Cards Information:

Add-on CardBus InterfaceModel Remark

Error Description :