Table of Contents

JUMPER COMPONENT LOCATION DIAGRAM	A

CHAPTER 1   INTRODUCTION	2

1-1 SYSTEM FEATURES	3

1-2 CHECK LIST OF THE PACKING	4

CHAPTER 2   INSTALLATION	5

2-1 INSTALLATION PROCEDURES 	5

2-2 CPU INSTALLATION	6

2-3 SYSTEM MEMORY INSTALLATION	12

2-4 SRAM INSTALLATION	14

2-5 OTHER JUMPERS AND CONNECTORS

DESCRIPTION 	15

2-6 IDE DRIVER INSTALLATION	20

CHAPTER 3   AWARD BIOS SETUP	21

3-1 UPDATE BIOS PROCEDURES	22

3-2 AWARD SYSTEM BIOS CONFIGURATION SETUP	23

APPENDIX A 	47

CHAPTER 1 INTRODUCTION

This manual introduces how to configure the ATC-1020 mainboard for different environment. It's an overview of the layout and features of the mainboard, and also provides information for

you to change the configuration or system environment.

This manual is divided into two sections :

PART ONE includes page A and two chapters as following:

Page A contains layout diagram of the mainboard. Please refer it when you configure the system.

Chapter 1 is an overview of the mainboard features and packing

contents.

Chapter 2 describes how to upgrade and to change hardware

configurations such as memory size, CPU type, and lists of

jumper settings and connectors.

PART TWO includes chapter 3 which contains Award BIOS description.

Chapter 3 is the user's guide of AWARD BIOS setup utility, Flash ROM BIOS update procedure. The menu showed in this chapter are default settings.

Your system dealer will set up the mainboard according to your demand of computer. It means that the current settings of your mainboard may not be the same as the defaults shown in this user's manual.  If you need to change your configuration, please ask your dealer firstly, be sure this will not against your system warranty. Or ask for your dealer to do it for you.

REMARK

Intel®  is a registered trademark of Intel Corporation.

MS-DOS® is a registered trademark of Microsoft Corporation.

All other brands and product names are trademarks registered trademarks of their respective companies.

1-1 SYSTEM FEATURES

- Pentium level CPU operating at 75 MHz to 200MHz with 321-pin ZIF socket 7 and scalability to accept processor in the future.

- INTEL 82430VX PCIset.

- Using four 72-pin SIM sockets, provides two banks of 64-bit wide path up to 128MB addressing page mode DRAMs.

- Supporting two types of DRAM including EDO (Extended Data Out), or FPM (Fast Page Mode).

- Supporting four PCI bus master revision 2.1, 5V interface compliant and three 16-bit ISA slots.

- Dual Master IDE connectors support up to four devices in two channels for connection of high capacity hard disk drive, CD-ROM, tape backup etc..

- AT style keyboard connector and PS/2 mouse connector.

- Winbond 83877 high-speed Multi-I/O chipset:

- Supporting Infrared transfer (IrDA TX/RX) connection.

- One FDC port supports two devices up to 2.88MB

- Two 16550A fast UARTs compatiable serial ports

- One EPP/ECP mode parallel port

- Hardware Dimension is 220mm x 280mm (8.66" x 11.02") with four layers designed.

1-2 CHECK LIST OF THE PACKING

The mainboard comes securely packed in a durable box and shipping carton. If any of the above items are missing or damaged, please contact your supplier.

Each mainboard containing:

Q'TY     	Description

1         	Mainboard 	: ATC-1020.

1	Diskette	: Enhanced IDE driver (3.5").

1         	Cable 	: Enhanced IDE connector.

1         	Cable 	: F.D.D. connector.

1	Cable 	: Serial port.

1         	Cable 	: Serial/Parallel.

1         	Manual 	: User`s manual.

NOTE : Leave the mainboard in its original packing until you are ready to install it.

CHAPTER 2  INSTALLATION

2-1 INSTALLATION PROCEDURE

Before installing the computer, please prepare all components such as CPU, DRAM; peripherals such as hard disk devices, keyboard, CD-ROM drive; and accessories such as cables. Then, install the system as following :

1. Plug CPU, heat sink, cooling fan, and DRAM modules in the

mainboard.

2. Set jumpers based on your configuration.

3. Plug add-on cards in PCI/ISA slots.

4. Connect cables to peripherals, power supply..

5. Make sure all components and devices are well connected,

turn on the power and setup System BIOS based on your

configuration.

6. Install peripheral, add-on card drivers and test them.

7. If all of above procedures are success, turn-off the power then

plug all of them into your computer case.

2-2 CPU  INSTALLATION

ATC-1020 supports Pentium level CPU up to 200MHz. For installation, please notice CPU pin 1 must align with the ZIF socket 7 Pin 1 location. Before you install or upgrade your CPU, please read CPU guide from CPU manufacturer to make sure the CPU voltage specification. Then choose the right installation in the 2-2-1 based on your CPU type/brand and follow the description to setup jumpers. If your CPU is not in the list of 2-2-1, please refer to 2-2-2 and 2-2-3 for installation.

2-2-1 CPU TYPE SELECTION

A. INTEL PENTIUM CPU (P54C)

| JP11 | 1-2 |
| --- | --- |

| CPU Core Voltage | VRE | STD |
| --- | --- | --- |
| JP14 | 1-2** | 3-4* |

| INTERNAL CPU CLOCK | JP5 | JP6 | JP9 | JP10 | Ext.x Frq. |
| --- | --- | --- | --- | --- | --- |
| 75MHz | open | open | 1-2 | 1-2 | 50x1.5 |
| 90MHz | open | close | 1-2 | 1-2 | 60x1.5 |
| 100MHz | close | open | 1-2 | 1-2 | 66x1.5 |
| 120MHz | open | close | 1-2 | 2-3 | 60x2.0 |
| 133MHz | close | open | 1-2 | 2-3 | 66x2.0 |
| 150MHz | open | close | 2-3 | 2-3 | 60x2.5 |
| 166MHz | close | open | 2-3 | 2-3 | 66x2.5 |
| 180MHz | open | close | 2-3 | 1-2 | 60x3.0 |
| 200MHz | close | open | 2-3 | 1-2 | 66x3.0 |

B. INTEL PENTIUM CPU (P55C)

Besides CPU clock setting, for P55C (MMX) CPU you have to set JP11,JP14 based on its CPU I/O voltage and core voltage.

| JP11 | 2-3 |
| --- | --- |

Set 2-3 for dual voltage mode.

| CPU Core Voltage | 3.5V | 3.3V | 2.9V | 2.8V | 2.7V |
| --- | --- | --- | --- | --- | --- |
| JP14 | 1-2 | 3-4 | 5-6 | 7-8 | 9-10 |

Currently, Intel P55C is released 2.8V core voltage only.

Please confirm this with your system supplier before you

install P55C CPU.

| INTERNAL CPU CLOCK | JP5 | JP6 | JP9 | JP10 | Ext.x Frq. |
| --- | --- | --- | --- | --- | --- |
| 166MHz | close | open | 2-3 | 2-3 | 66x2.5 |
| 180MHz | open | close | 2-3 | 1-2 | 60x3.0 |
| 200MHz | close | open | 2-3 | 1-2 | 66x3.0 |

The location of JP11 and JP14

C. Cyrix 6x86/6x86L CPU

Cyrix 6x86 (028) CPU

| CPU | JP11 | JP14 |
| --- | --- | --- |
| Cyrix 6x86 (028) | 1-2 | 1-2 |

| INTERNAL CPU CLOCK | JP5 | JP6 | JP9 | JP10 | Ext.x Frq. |
| --- | --- | --- | --- | --- | --- |
| PR120+ @ 100MHz | open | open | 1-2 | 2-3 | 50x2.0 |
| PR150+ @ 120MHz | open | close | 1-2 | 2-3 | 60x2.0 |
| PR166+ @ 133MHz | close | open | 1-2 | 2-3 | 66x2.0 |

Cyrix 6x86L CPU (dual voltage)

| CPU | JP11 | JP14 |
| --- | --- | --- |
| Cyrix 6x86L | 2-3 | 7-8 |

| INTERNAL CPU CLOCK | JP5 | JP6 | JP9 | JP10 | Ext.x Frq. |
| --- | --- | --- | --- | --- | --- |
| PR120+ @ 100MHz | open | open | 1-2 | 2-3 | 50x2.0 |
| PR133+ @ 110MHz | close | close | 1-2 | 2-3 | 55x2.0 |
| PR150+ @ 120MHz | open | close | 1-2 | 2-3 | 60x2.0 |
| PR166+ @ 133MHz | close | open | 1-2 | 2-3 | 66x2.0 |

D. AMD-K5 CPU

| CPU | JP11 | JP14 |
| --- | --- | --- |
| AMD K5 | 1-2 | 1-2 |

| CPU CLOCK RATING | JP5 | JP6 | JP9 | JP10 | Ext.x Frq. |
| --- | --- | --- | --- | --- | --- |
| PR75ABR 75MHz | open | open | 1-2 | 1-2 | 50x1.5 |
| PR90ABQ 90MHz | open | close | 1-2 | 1-2 | 60x1.5 |
| PR100ABQ 100MHz | close | open | 1-2 | 1-2 | 66x1.5 |
| PR120ABQ 90MHz | open | close | 1-2 | 1-2 | 60x1.5 |
| PR133ABQ 100MHz | close | open | 1-2 | 1-2 | 66x1.5 |
| PR166ABQ | close | open | 2-3 | 2-3 | 66x2.5 |

E. AMD-K6 CPU

AMD K6-PR166/PR200

| JP11 | JP14 |
| --- | --- |
| 2-3 | 5-6 |

| INTERNAL CPU CLOCK | JP5 | JP6 | JP9 | JP10 | Ext.x Frq. |
| --- | --- | --- | --- | --- | --- |
| AMD K6-PR166 | close | open | 2-3 | 2-3 | 66x2.5 |
| AMD K6-PR200 | close | open | 2-3 | 1-2 | 66x3.0 |

2-2-2 CPU VOLTAGE SETTING

JP11 is for setting single or dual CPU voltage mode. JP14 is used for various CPU voltage value types, please refer to CPU's marking on/under CPU and its user's manual.

| CPU MODE | Single | Dual |
| --- | --- | --- |
| JP11 | 1-2 | 2-3 |

| CPU CORE VOLTAGE |  | JP14 |
| --- | --- | --- |
| INTEL | STD | 3-4 |
| (P54C) | VRE | 1-2 |
| P55C | 2.8V | 7-8 |
| Cyrix | 6x86(028) | 1-2 |
| Cyrix | 6x86L | 7-8 |
| AMD | K5 | 1-2 |
| AMD | K6(PR166/200) | 5-6 |

2-2-3 CPU CLOCK SETTING

The following setting is for new release CPUs.

| EXTERNAL CPU CLOCK | JP5 | JP6 |
| --- | --- | --- |
| 50(MHz) | open | open |
| 55(MHz) | close | close |
| 60(MHz) | open | close |
| 66(MHz) | close | open |

External

CPU Clock :

| INTERNAL CPU CLOCK | JP9 | JP10 |
| --- | --- | --- |
| INTEL EXTERNAL CLOCK X 1.5 | 1-2 | 1-2 |
| INTEL EXTERNAL CLOCK X 2.0 | 1-2 | 2-3 |
| INTEL EXTERNAL CLOCK X 2.5 | 2-3 | 2-3 |
| INTEL EXTERNAL CLOCK X 3.0 | 2-3 | 1-2 |
| INTEL EXTERNAL CLOCK X 3.5 | 1-2 | 1-2 |

Internal

CPU Clock :

2-3 SYSTEM MEMORY INSTALLATION

ATC-1020 provides four 72-pin SIM sockets for system memory expansion from 4MB to 128MB. These four SIMs are arranged to two banks, Bank0 (SIM 1, 2) and  Bank1(SIM 3, 4), please refer to page A. Each bank provides 64-bit wide data path.

The mainboard accepts Fast Page Mode DRAM, and EDO Mode (Extended Data Out) DRAM, with a speed at least 70 nanosecond. You should plug DRAM modules into two sockets (same bank) or four sockets at one time. Each pair of modules must be the same size, type, and speed; no matter single-side or double-side module. Please plug in Bank 0 firstly if you only have 2 modules. The mainboard supports mixing of EDO and fast page mode DRAM among different banks, please plug EDO in Bank 0.

Also this mainboard provides one optional 168-pin DIMM socket for Synchronous DRAM expansion. It's not recommended to install the  SDRAM and EDO or Fast Paged mode memory within a system. Changing EDO/FPM DRAM to SDRAM, you don't have to adjust jumper setting or BIOS value, nor change SDRAM to EDO/FPM DRAM. ( Please make sure the SDRAM plugged-in fully, to prevent contact loss.)

System Memory Combinations Options

| BANK0 | BANK1 | Total Memory |
| --- | --- | --- |
| SIM 1, 2 | SIM 3, 4 | SIM 1- 4 |
| 2MBx2 | - | 4MB |
| - | 2MBx2 | 4MB |
| 4MBx2 | - | 8MB |
| - | 4MBx2 | 8MB |
| 8MBx2 | - | 16MB |
| - | 8MBx2 | 16MB |
| 4MBx2 | 4MBx2 | 16MB |
|  | - continue - |  |
| BANK0 | BANK1 | Total Memory |
| SIM 1, 2 | SIM 3, 4 | SIM 1- 4 |
| 4MBx2 | 8MBx2 | 24MB |
| 8MBx2 | 4MBx2 | 24MB |
| 16MBx2 | - | 32MB |
| - | 16MBx2 | 32MB |
| 8MBx2 | 8MBx2 | 32MB |
| 4MBx2 | 16MBx2 | 40MB |
| 16MBx2 | 4MBx2 | 40MB |
| 8MBx2 | 16MBx2 | 48MB |
| 16MBx2 | 8MBx2 | 48MB |
| 32MBx2 | - | 64MB |
| - | 32MBx2 | 64MB |
| 16MBx2 | 16MBx2 | 64MB |
| 4MBx2 | 32MBx2 | 72MB |
| 32MBx2 | 4MBx2 | 72MB |
| 8MBx2 | 32MBx2 | 80MB |
| 32MBx2 | 8MBx2 | 80MB |
| 16MBx2 | 32MBx2 | 96MB |
| 32MBx2 | 16MBx2 | 96MB |
| 32MBx2 | 32MBx2 | 128MB |

2-4 SRAM  INSTALLATION

ATC-1020 is built-in 256KB or 512KB Sync. Pipeline Burst SRAM on board and provides a COAST slot for further expansion.

Cache Memory Combinations Options

| SRAM TYPE | SRAM SIZE | DATA SRAM | TAG SRAM |
| --- | --- | --- | --- |
| Synchronous | 256KB | 32K32 x 2pcs | 16K8 or |
| on board | 512KB | 64K32 x 2pcs | 32K8 x 1pc |
| Synchronous module | 256KB | 32K32 x 2pcs | 16K8 or 32K8 x 1pc |

Upgrade Procedure :

For upgrading your Sync. SRAM from 256KB to 512KB by using COAST Module, you should plug-in the KIT-256 module, then set JP17 to "open" ( disable the original TAG on board). You must use KIT-256 for upgrading your system to 512KB Sync. SRAM , please contact your system dealer about its information. It does not

need any BIOS value adjustments for upgrading the SRAM size.

| On Board TAG | JP17 |
| --- | --- |
| Disable | open |
| Enable | close |

(If your module is without TAG RAM, please set

JP17 "close".)

2-5 OTHER JUMPERS AND

CONNECTORS  DESCRIPTION

The locations of following jumpers are indicated in page A.

| Jumper | Function | Remark |
| --- | --- | --- |
| JP8 | CMOS | open: Normal close: Clear CMOS |
| JP7 | ATBUS CLK | open: PCI CLK/3 for external CPU clock 50MHz close: PCI CLK/4 for external CPU clock 60, 66MHz |

is default setting     **is factory default setting (fixed)

The locations of following connectors are indicated in page A. When you plug a cable into the following I/O connectors, you should have the pin 1 edge of the cable aligned with the pin 1 end of the connector.

CONN1 : speaker, keyboard lock, reset, SMI,

turbo LED, and IDE LED connectors.

FAN1 : CPU cooling fan connector. Wire with

+12V voltage (most likely red wire) must be

plugged into pin2, and GROUND wires (must

likely black wires) must be plugged into pin1

and/or pin3. Please confirm the wire color re-

presentation with your supplier.

CAUTION: Plug wire into wrong connector will

DAMAGE fan and mainboard.

IR1 : Infrared module connector.

USB1 : USB connector; Universal Serial Bus;this

is used to connect USB devices through

an optional dual head cable with a iron plane.

CAUTION: Plug wire into wrong connector

will DAMAGE USB devices and mainboard.

KB1 : 5-pin AT style compatible keyboard

connector.

PS/2 : PS/2 mouse connector, which is used to

connect an optional cable.

pin1 : data

pin2 : N/C

pin3 : GND

pin4 : VCC

pin5 : clock

pin6 : N/C

PW1 : +5 Voltage power supply connector.

COM1/COM2

COM1/COM2 : these two connectors are used

to connect serial port cables.

| pin | signal name |
| --- | --- |
| 1 | DCD |
| 2 | Serial In |
| 3 | Serial Out |
| 4 | DTR |
| 5 | GND |
| 6 | DSR |
| 7 | RTS |
| 8 | CTS |
| 9 | RI |
| 10 | N/C |

FDC : this connector is used to connect floppy

disk drive through cable.

| pin | signal | pin | signal |
| --- | --- | --- | --- |
| 2 | RWC- | 20 | STEP- |
| 4 | reserved | 22 | Write Data |
| 6 | FDEDIN | 24 | Write Gate |
| 8 | Index- | 26 | Track 00- |
| 10 | Motor EnableA- | 28 | Write Protect- |
| 12 | Drive Sele.B- | 30 | Read Data- |
| 14 | Drive Sele.A- | 32 | Side 1 Sele.- |
| 16 | Motor EnableB- | 34 | DisketteChange |
| 18 | DIR- |  |  |
| All of odd pins are ground |  |  |  |

LPT

LPT : this connector is used to connect parallel

port cable.

| pin | signal | pin | signal |
| --- | --- | --- | --- |
| 1 | STROBE- | 10 | ACK- |
| 2 | Data Bit 0 | 11 | BUSY |
| 3 | Data Bit 1 | 12 | PE |
| 4 | Data Bit 2 | 13 | SLCT |
| 5 | Data Bit 3 | 14 | Auto Feed- |
| 6 | Data Bit 4 | 15 | ERROR- |
| 7 | Data Bit 5 | 16 | INIT- |
| 8 | Data Bit 6 | 17 | SLCT IN- |
| 9 | Data Bit 7 | 26 | N/C |
| pin18 -- pin25 are ground |  |  |  |

IDE1/IDE2 : these two connectors are used to

connect IDE devices through IDE cables, up to 4 devices.

| pin | signal | pin | signal |
| --- | --- | --- | --- |
| 1 | Reset IDE | 21 | DDRQ0(1) |
| 2 | GND | 22 | GND |
| 3 | Host Data 7 | 23 | I/O Write- |
| 4 | Host Data 8 | 24 | GND |
| 5 | Host Data 6 | 25 | I/O Read- |
| 6 | Host Data 9 | 26 | GND |
| 7 | Host Data 5 | 27 | IORDY |
| 8 | Host Data 10 | 28 | N/C |
| 9 | Host Data 4 | 29 | DDAK0-(1-) |
| 10 | Host Data 11 | 30 | GND |
| 11 | Host Data 3 | 31 | IRQ14* |
| 12 | Host Data 12 | 32 | IOCS16- |
| 13 | Host Data 2 | 33 | Addr 1 |
| 14 | Host Data 13 | 34 | N/C |
| 15 | Host Data 1 | 35 | Addr 0 |
| 16 | Host Data 14 | 36 | Addr 2 |
| 17 | Host Data 0 | 37 | ChipSele. 1P- |
| 18 | Host Data 15 | 38 | ChipSele 3P- |
| 19 | GND | 39 | Activity |
| 20 | Key | 40 | GND |

2-6 IDE DRIVER INSTALLATION

The IDE driver installation procedure is printed on the label of diskette as following :

Setup for DOS/Windows :

1. Starting MS-Windows 3.1 (or 3.11)

2. Select Program Manager, File/RUN, then type "A:\setup.exe"

3. Exit MS-Windows, turn power off; then turn power on.

Setup for Windows 95 :

1. Starting Windows 95

2. Select "My Computer"; select "Control Panel"; select "System"; then select "Device Manager", "Hard Disk Controllers".

3. Double-click to Remove default driver, restart computer.

4. Follow the instructions on your screen to install new IDE driver

we offer in the 3.5" diskette

5. Exit Windows 95, turn power off; then turn power on.

For more information, please refer to Windows 95 manual.

Make sure your HDD should follow ATA standard, and your CD-ROM should follow ATAPI standard. When you plug-in the IDE devices, please plug your first and second devices into IDE 1 port (Master then Slave), then plug third and forth devices into IDE 2 port. If you have CD-ROM driver, please set it behind hard disk devices as the last device. For example, if you have 2 HDDs and 1 CD-ROM, you should set HDD1 and HDD2 in IDE1 Master and Slave, set CD-ROM in IDE 2 Master.

CHAPTER 3  AWARD BIOS

SETUP

This chapter explains the system BIOS setup, and how to update new BIOS. All BIOS screens showed in the following pages are default values, your system dealer will set up these

values according to your demand of computer.

ATC-1020 uses Flash ROM to make BIOS easier to be updated by the floppy disk-based program and to committe

Microsoft Windows 95 plug & play feature.

If you would like to update CMOS date also, please set JP8.

| open | NORMAL |
| --- | --- |
| close | CLEAR CMOS |

is default setting

JP19 Setting is for Update System BIOS by using Intel

Flash ROM only.

JP9 : Intel Flash ROM update

| 1-2 | Enable |
| --- | --- |
| 2-3 | Disable |

is default setting

When you want to update Intel Flash ROM

you should set JP19 to '1-2'.

After you finish updated, set to '2-3' again.

If the mainboard uses non-Intel Flash ROM, then JP19 will

be wired (2-3) in factory.

3-1 UPDATE BIOS PROCEDURES

If the BIOS needs to be updated, it can be obtained on a diskette from your system supplier. The BIOS diskette includes 3 files:

"awdflash.exe"  --  BIOS update utility program

"awdflash.doc"

"(update BIOS filename with version number).rom"

The update procedures are as following:

- Boot the system to DOS mode in a normal manner.

- Insert the updated diskette to drive A (or B).

- Change working directory to floppy drive, A or B, which contains the update BIOS diskette. --  Type "a:\" or "b:\", "ENTER".

- Run the BIOS update utility  --  Type "awdflash", "ENTER".

- Type "(update BIOS file name with version number).rom", ENTER.

- Type "N" when the screen displays the message : " Do you want to save BIOS (Y/N) ?".

- Type "Y" when the screen shows the message : " Are you sure to  program (Y/N) ?".

- Follow instructions displayed on the screen.  DO NOT remove the update BIOS diskette from the floppy drive nor turn the system power off until the BIOS update is completed.

- Exit the utility and turn the system power off, set JP8 to 'close' to clear the CMOS. After about 10 seconds, set JP8 to 'open' (remove the jumper cap ), become the normal status.

- Turn the system power on and test your system working properly or not.

3-2 AWARD SYSTEM BIOS CONFIGURATION

SETUP

The following pages explain how to set up the system configuration (CMOS) under the AWARD BIOS. The SETUP program is stored in the Read-Only-Memory (ROM) on the mainboard. Enter the SETUP procedure, press the <Del> key when the system is booting up. The following main menu will appear. Please select " STANDARD CMOS SETUP" to enter the next screen.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |  |  |
| --- | --- | --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |  |  |
| CHIPSET FEATURES SETUP | USER PASSWORD |  |  |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |  |  |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |  |  |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |  |  |
| LOAD SETUP DEFAULTS |  |  |  |
| ESC: Quit | áâàß:Select Item |  |  |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |  |  |
| Time, Date, Hard Disk Type ..... |  |  |  |

The section on the bottom of the main menu explains how to control this screen.  The other one section displays the items highlighted in the list.

This screen records some basic hardware information, and set the system clock and error handling. These records can be lost or corrupted if the on-board battery  is failed or  weak.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |  |
| --- | --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |  |
| CHIPSET FEATURES SETUP | USER PASSWORD |  |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |  |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |  |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |  |
| LOAD SETUP DEFAULTS |  |  |
| ESC: Quit | áâàß:Select Item |  |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |  |
| Time, Date, Hard Disk Type ..... |  |  |

ROM PCI/ISA BIOS (2A59GA29)

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| Date (mm:dd:yy) : Fri, Jun 14 1996 |  |  |
| --- | --- | --- |
| Time(hh:mm:ss) : 13 : 7 : 14 |  |  |
| HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |
| Primary Master : Auto 0 0 0 0 0 0 Auto |  |  |
| Primary Slave : Auto 0 0 0 0 0 0 Auto |  |  |
| Secondary Master : Auto 0 0 0 0 0 0 Auto |  |  |
| Secondary Slave : Auto 0 0 0 0 0 0 Auto |  |  |
| Drive A : 1.44M, 3.5 in. | Base Memory : 640K |  |
| Drive B : None | Extended memory : 64512K |  |
|  | Other Memory : 384K |  |
| Video : EGA/VGA | ------------------------------------------------ |  |
| Halt On: All Errors | Total Memory : 65536K |  |
| ESC : Quit áâàß:Select Item PU/PD/+/- : Modify |  |  |
| F1 : Help (Shift) F2 : Change Color |  |  |

Date	mm is month, dd is date, yy is year.

| date | from 1 to 31 |
| --- | --- |
| month | from Jan. to Dec. |
| year | from 1900 to 2099 |

Time	hh is hour, mm is minute, ss is second.

| hh | from 0 to 23 (24-hour military -time) |
| --- | --- |
| mm | from 1 to 59 |
| ss | from 1 to 59 |

Primary Master	The categories identify the types of 2 channels

Primary Slave	that have been installed in the computer. There

Secondary Master	are 45 predefined types and 4 user definable

Secondary Slave	types are for Enhanced IDE BIOS. Type 1 to 45

are predefined. Type 'user' is user-definable.

Press PgUp/PgDn to select a numbered hard

disk type or type the number and press<Enter>.

If you select 'Auto' BIOS will auto-detect the

HDD & CD-ROM Drive at the POST stage and

showing the IDE for HDD & CD-ROM Drive. If

you select 'user', you will need to know the

information listed below. Enter the information 		directly from the keyboard and press <Enter>. 		This information should be from your hard disk

vender or dealer. If the controller of HDD

interface is ESDI, the selection shall be 'Type			1'; is SCSI, the selection shall be 'None'.

If the device has not been installed select

'NONE' and press <Enter>.

| Type | drive type |
| --- | --- |
| SIZE | automatically adjusts |
| CYLS | number of cylinders |
| HEAD | number of heads |
| PRECOMP | write precom |
| LANDZ | landing zone |
| SECTOR | number of sectors |
| MODE | mode type |

Drive A	The category identifies the types of floppy disk

Drive B	drive A or drive B that have been installed in 			the computer.

| None | No floppy drive installed |
| --- | --- |
| 360K, 5.25 in | 5.25" PC-type 360KB capacity |
| 1.2M, 5.25 in | 5.25" AT-type 1.2MB capacity |
| 720K, 3.5 in | 3.5" double-side 720KB capacity |
| 1.44M, 3.5 in | 3.5" double-side 1.44MB capacity |
| 2.88M, 3.5 in | 3.5" double-side 2.88MB capacity |

Video	The category selects the type of video adapter

used for the primary system monitor. Although

secondary monitors are supported, you do not

have to select the type in Setup.

Halt On	The category determines whether the computer

will stop if an error is detectd during power up.

| No errors | When the BIOS detects a non-fatal error the system will be stopped and you will be prompted |
| --- | --- |
| All errors | The system boot will not be stopped for any error that may be detected |
| All, But Keyboard | The system boot will not stop for a keyboard error, it will stop for all other errors |
| All, But Diskette | The system boot will not stop for a disk error, it will stop for all other errors |
| All, But Disk/Key | The system boot will not stop for a disk or keyboard error, it will stop for all other errors |

Memory	The category is display-only which is determined

by POST (Power On Self Test) of the BIOS.

Base Memory  The value of the base memory

is typically 512K or 640K based on the memory

installed on the mainboard.

Extended Memory  How much extended

memory is present during the POST. This is

the amount of memory located above 1MB in

the CPU's memory address map.

Other Memory  This refers to the memory

located in the 640K to 1024K address space.

The BIOS is the most frequent user of this RAM

area since this is where it shadows RAM.

This screen is a list of system configuration options. Some of them are defaults required by the mainboard's design, others depend on the features of your system.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |  |
| --- | --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |  |
| CHIPSET FEATURES SETUP | USER PASSWORD |  |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |  |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |  |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |  |
| LOAD SETUP DEFAULTS |  |  |
| ESC: Quit | áâàß:Select Item |  |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |  |
| Virus, Protection, Boot Sequence |  |  |

ROM PCI/ISA BIOS (2A59GA29)

BIOS FEATURES SETUP

AWARD SOFTWARE, INC.

| Virus Warning | : Disabled | Video BIOS Shadow | : Enabled |
| --- | --- | --- | --- |
| CPU Internal Cache | : Enabled | C8000-CBFFF Shadow | : Disabled |
| External Cache | : Enabled | CC000-CFFFF Shadow | : Disabled |
| Quick Power On Self Test | : Disabled | D0000-D3FFF Shadow | : Disabled |
| Boot Sequence | : A, C | D4000-D7FFF Shadow | : Disabled |
| Swap Floppy Drive | : Disabled | D8000-DBFFF Shadow | : Disabled |
| Boot Up Floppy Seek | : Enabled | DC000-DFFFF Shadow | : Disabled |
| Boot Up NumLock Status | : On |  |  |
| Boot Up System Speed | : High |  |  |
| Gate A20 Option | : Fast |  |  |
| Typematic Rate Setting | : Disabled |  |  |
| Typematic Rate(Chars/Sec) | : 6 | Esc : Quit áâàß:Select Item |  |
| Typematic Delay(Msec) | : 250 | F1 : Help PU/PD/+/- : Modify |  |
| Security Option | : Setup | F5 : Old Values (SHIFT)F2 : Color |  |
| PCI/VGA Palette Snoop | : Disabled | F6 : Load BIOS Defaults |  |
| OS/2 select for DRAM>64MB | : Non-OS2 | F7 : Load Setup Defaults |  |
| Report No FDD for Win95 | : No |  |  |

Virus Warning	When this item is enabled, the BIOS will

monitor the boot sector and partition table of the

hard disk drive for any attempt at modification.

If an attempt is made, the BIOS will halt the

system and the following error message will

appear. Many disk diagnostic programs which

attempt to access the boot sector table can

cause the above warning message. If you will

be running such a program, we recommend

that you first disable Virus Protection beforehand.

! WARNING ! Disk boot sector is to be modified Type 'Y' to accept write or 'N' to abort write Award Software, Inc.

| Enabled | Activates automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector or hard disk partition table. |
| --- | --- |
| Disabled | No warning message will appear when anything attempts to access the boot sector or hard disk partition table. |

CPU	These two categories speed up memory access.

Internal Cache	However, it depends on CPU/chipset design.

External Cache	The default value is'enabled'.

Quick Power On	This category speeds up Power On Self Test

Self Test	after you power up the computer. If you set 			Enabled, BIOS will shorten or skip some check

items during POST.

Boot Sequence	This category determines which drive to search

first for the disk operating system (i.e.,DOS).

| C,A | System will first search for HDD then FDD |
| --- | --- |
| A,C | System will first search for FDD then HDD |
| CDROM,C, A | System will first search for CDROM then HDD, and next is FDD |
| C,CDROM, A | System will first search for HDD then CDROM, and next is FDD |

Swap Floppy	This item allows you to determine whether

Drive 	enable the swap floppy drive or not.

Boot Up Floppy 	During POST, BIOS will determine if the floppy

Seek	disk drive installed is 40 tracks (360K) or 80

tracks (720K, 1.2M, 1.44M)

| Enabled | BIOS searchs for floppy disk drive to determine if it is 40 or 80 tracks |
| --- | --- |
| Disabled | BIOS will not search for the type of floppy disk drive by track number |

Boot Up 	This allows you to determine the default state of

NumLock Status	the numeric keypad. By default, the system

boots up with NumLock on.

Boot Up System 	Selects the default system speed - the normal

Speed	operating speed at power up.

Gate A20	This entry allows you to select how the gate

Option	A20 is handled. The gate A20 is a device used

to address memory above 1 MB. Normal is

keyboard; Fast is chipset.

Typematic Rate	This determines if the typematic rate is to be used.

Setting	When disabled, continually holding down a key

on your keyboard will generate only one instance.

Typematic Rate 	When the typematic rate is enabled, this section

(Chars/Sec)	allows you select the rate at which the keys are

accelerated.

| 6 | 6 characters per second |
| --- | --- |
| 8 | 8 characters per second |
| 10 | 10 characters per second |
| 12 | 12 characters per second |
| 15 | 15 characters per second |
| 20 | 20 characters per second |
| 24 | 24 characters per second |
| 30 | 30 characters per second |

Typematic Delay 	When the typematic rate is enabled, this section

(Msec)	allows you select the delay between when the

key was first depressed and when the

acceleration begins.

| 250 | 250 msec |
| --- | --- |
| 500 | 500 msec |
| 750 | 750 msec |
| 1000 | 1000 msec |

Security Option 	This category allows you to limit access to the

system and Setup, or just to Setup

| System | The system will not boot and access to Setup will be denIed if the correct password is not entered at the prompt |
| --- | --- |
| Setup | The system will boot, but access to Setup will be denIed if the correct password is not entered at the prompt |

To disable security, select PASSWORD SETTING

at Main Menu and then you will be asked to enter

password. Do not type anything and just press

<Enter>, it will disable security. Once the security

is disabled, the system will boot and you can enter 		Setup freely.

PCI/VGA 	It determines whether the MPEG ISA/VESA

Palette Snoop	VGA cards can work with PCI/VGA or not.

| Enabled | When PCI/VGA working with MPEG ISA/VESA VGA Card |
| --- | --- |
| Disabled | When PCI/VGA not working with MPEG ISA/VESA VGA Card |

OS Select for 	This item allows you to access the memory

DRAM > 64MB	that over 64MB in OS/2

Video BIOS	Determines whether video BIOS will be copied to

Shadow	RAM. However it is optional depending on

chipset design. Video Shadow will increase the

video speed.

C8000 - CBFFF 	These categories determine whether option

Shadow	ROMs will be copied to RAM. An example of

DC000 - DFFFF	such option ROM would be support of onboard

Shadow 	SCSI.

This screen controls the setting for the chipset on the mainboard.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |  |
| --- | --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |  |
| CHIPSET FEATURES SETUP | USER PASSWORD |  |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |  |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |  |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |  |
| LOAD SETUP DEFAULTS |  |  |
| ESC: Quit | áâàß:Select Item |  |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |  |
| AT Clock, DRAM Timmings ..... |  |  |

ROM PCI/ISA BIOS (2A59GA29)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Auto Configuration | : Enabled | Delay Transaction |  | : Enabled |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DRAM Timing | : 70ns |  |  |  |  |  |
| DRAM RAS# Precharge Time | : 4 |  |  |  |  |  |
| DRAM R/W Leadoff Timing | : 6 |  |  |  |  |  |
| Fast RAS to CAS Delay | : 3 |  |  |  |  |  |
| DRAM Read Burst (EDO/FP) | : x333/X444 |  |  |  |  |  |
| DRAM Write Burst Timing | : x333 |  |  |  |  |  |
| Fast MA to RAS# Delay CLK | : 1 |  |  |  |  |  |
| Fast EDO Path Select | : Disabled |  |  |  |  |  |
| Refresh RAS# Assertion | : 5 Clks |  |  |  |  |  |
| ISA Bus Clock | : PCICLK/4 |  |  |  |  |  |
| System BIOS Cacheable | : Disabled |  |  |  |  |  |
| Video BIOS Cacheable | : Disabled |  |  |  |  |  |
| 8-bit I/O Recovery Time | : 3 | Esc: Quit | :Select Item |  |  |  |
| 16-bit I/O Recovery Time | : 2 | F1 : Help | PU/PD/+/-:Modify |  |  |  |
| Memory Hole At 15M-16M | : Disabled | F5 : Old Values | (Shift)F2 :Color |  |  |  |
| Peer Concurrency | : Enabled | F6 :Load BIOS Defaults |  |  |  |  |
| Passive Release | : Enabled | F7 :Load Setup Defaults |  |  |  |  |

Auto	Pre-defined values for DRAM, cache... timing

Configuration	according to CPU type & system clock. When

this item is enabled, the pre-defined items will

become SHOW-ONLY.

DRAM Timing	The DRAM speed is controlled by the DRAM

timing Registers. The timings programmed into

this register are dependent on the system design.

DRAM RAS#	DRAM must continually be refreshed or it will lose

Precharge Time	its data. Normally, DRAM is refreshed entirely as

the result of a single request. This option allows

you to determine the number of CPU clocks

allocated for the Row Address Strobe to

accumulate its charge before the DRAM is

refreshed. If insufficient time is allowed, refresh

may be incomplete and data lost.

Fast RAS# to	When DRAM is refreshed, both rows and columns

CAS# Delay	are addressed separately. This setup item allows

you to determine the timing of the transition from

RAS to Column Address Strobe (CAS).

DRAM Read	This sets the timing for burst mode read (or writes)

Burst (EDO/FP) 	from DRAM. Burst read and write requests are

DRAM Write		generated by the CPU in four separate parts.

Burst Timing		The first part provides the location within the

DRAM where the read or write is to take place

while the remaining three parts provide the

actual data. The lower the timing numbers, the

faster the system will address memory.

ISA Bus Clock	This item allows you to select the PCI clock type.

Choices are PCI CLK/3; PCI CLK/4

System BIOS	When enabled, accesses to the system BIOS

Cacheable 	ROM addressed at F0000H-FFFFFH are cached,

provided that the cache controller is enabled.

Video BIOS 	As with changing the system BIOS above,

Cacheable 	enabling the Video BIOS cache will cause

access to video BIOS addressed at C0000H to

C7FFFH to be cached, if the cache controller is

also enabled.

8-bit I/O 	The recovery time is the length of time, measured

Recovery Time 	in CPU clocks, which the system will delay after

the completion of an I/O request. This item allows

you to determine the recovery time allowed for 8-

bit I/O. Choices are from NA, 1 to 8 CPU clocks.

16-bit I/O	This item allows you to determine the recovery

Recovery Time 	time allowed for 16-bit I/O. Choices are from NA,

1 to 4 CPU clocks.

Memory Hole At	In order to improve performance, certain space

15M-16M	in memory can be reserved for ISA cards. This

memory must be mapped into the memory

below 16MB.

This screen controls the 'green' features of this mainboard.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |  |
| --- | --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |  |
| CHIPSET FEATURES SETUP | USER PASSWORD |  |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |  |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |  |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |  |
| LOAD SETUP DEFAULTS |  |  |
| ESC: Quit | áâàß:Select Item |  |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |  |
| Sleep Timer, Suspend Timer, ..... |  |  |

ROM PCI/ISA BIOS (2A59GA29)

POWER MANAGEMENT SETUP

AWARD SOFTWARE, INC.

| Power Management | : Disabled | IRQ3 (COM 2) |  |  | : OFF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PM Control by APM | : Yes | IRQ4 (COM 1) |  |  | : OFF |  |  |  |
| Video Off Method | : V/H SYNC | IRQ5 (LPT 2) |  |  | : OFF |  |  |  |
|  | +Blank | IRQ6 (Floppy Disk) |  |  | : OFF |  |  |  |
|  |  | IRQ7 (LPT 1) |  |  | : OFF |  |  |  |
| Doze Mode | : Disabled | IRQ8 (RTC Alarm) |  |  | : OFF |  |  |  |
| Standby Mode | : Disabled | IRQ9 (IRQ2 Redir) |  |  | : OFF |  |  |  |
| Suspend Mode | : Disabled | IRQ10 (Reserved) |  |  | : OFF |  |  |  |
| HDD Power Down | : Disabled | IRQ11 (Reserved) |  |  | : OFF |  |  |  |
|  |  | IRQ12 (PS/2 Mouse) |  |  | : OFF |  |  |  |
|  |  | IRQ13 (Coprocessor) |  |  | : OFF |  |  |  |
| *Wake Up Events In Doze & Standby* |  | IRQ14 (Hard Disk) |  |  | : OFF |  |  |  |
| IRQ3 (Wake-Up Event) | : OFF | IRQ15 (Reserved) |  |  | : OFF |  |  |  |
| IRQ4 (Wake-Up Event) | : OFF |  |  |  |  |  |  |  |
| IRQ8 (Wake-Up Event) | : OFF | Esc: Quit áâàß |  |  | :Select Item |  |  |  |
| IRQ12(Wake-Up Event) | : OFF | F1 : Help | PU/PD/+/- : Modify |  |  |  |  |  |
|  |  | F5 : Old Values |  | (Shift) F2: Color |  |  |  |  |
|  |  | F6 : Load BIOS Defaults |  |  |  |  |  |  |
|  |  | F7 : Load Setup Defaults |  |  |  |  |  |  |

Power 	This category allows you to select the type (or

Management	degree) of power saving and is dirctly related to

the following modes : Doze; Standby; Suspend;

HDD Power Down.

| Disabled | No power management. Disables all 4 modes |
| --- | --- |
| Min. Power Saving | Minimum power management. Doze =1hr.; Standby=1hr.; Suspend=1hr.; HDD Power Down=15min |
| Max. Power Saving | Maximum power management only available for SL CPU's.Doze=1min.; Standby=1min.;Suspend=1min.;HDD Power Down=1min |
| User Defined | Allows you to set each mode indivi-dually. When not disabled, each of the ranges are from 1min. to 1hr. except for HDD Power Down which ranges from 1 to 15min. and disable |

PM Control 	When enabled, an Advanced Power Management 	by APM	device will be activated to enhance the Max.

Power Saving Mode and stop the CPU internal

clock. If the Max. Power Saving is not enabled,

this will be present to NO.

Video Off	This determines the manner in which the monitor

Method	is blanked.

| V/H SYNC + Blank | This selection will cause the system to turn off the vertical and horizontal sync. ports and write blanks to the video buffer |
| --- | --- |
| Blank Screen | This option only writes blanks to the video buffer |
| DPMS | Initial display power management signaling |

The Following 4 modes are Green PC power saving function which are only user configuration when 'User Defined' power management has been selected.

Doze Mode	When enabled and after the set time of system

inactivity, the CPU clock will run at slower speed

while all other devices still operate at full speed

Standby Mode	When enabled and after the set time of system

inactivity, the fixed disk drive and the video

would be shut off while all other devices still

operate at full speed

Suspend Mode	When enabled and after the set time of system

inactivity, all devices expect the CPU will be shut

off

HDD Power Down	When enabled and after the set time of system

inactivity, the hard disk drive will be powered

down while all other devices remain active

Wake Up Events

In Doze & Standby

Power Down &	These are I/O events whose occurrence can

Resume Events	prevent the system from entering a power saving

mode or can awaken the system from such a

IRQ3 ~ IRQ15	mode. In effect, the system remains alert for

anything which occurs to a device which is

configured as On, even when the system is in a

power down mode. When an I/O device wants to

gain the attention of the operating system, it

signals this by causing an IRQ (Interrupt ReQuests)

to occur. When the operating system is ready to

respond to the request, it interrupts itself and

performs the service. When set off, activity will

neither prevent the system from going into a power

management mode nor awaken it.

This screen configures the PCI Bus slots.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |  |
| --- | --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |  |
| CHIPSET FEATURES SETUP | USER PASSWORD |  |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |  |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |  |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |  |
| LOAD SETUP DEFAULTS |  |  |
| ESC: Quit | áâàß:Select Item |  |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |  |
| IRQ Settings, Latency Timers, ..... |  |  |

ROM PCI/ISA BIOS (2A59GA29)

PNP/PCI CONFIGURATION

AWARD SOFTWARE, INC.

| Resources Controlled by | : Auto | PCI IRQ Actived by |  | : Level |
| --- | --- | --- | --- | --- |
| Reset Configuration Data | : Disabled | PCI IDE IRQ Map to |  | : PCI-AUTO |
|  |  | Primary IDE INT# |  | : A |
|  |  | Secondary IDE INT# |  | : B |
|  |  | Esc: Qui áâàß |  | :Select Item |
|  |  | F1 : Help | PU/PD/+/- : Modify |  |
|  |  | F5 : Old Values | (Shift) F2: Color |  |
|  |  | F6 : Load BIOS Defaults |  |  |
|  |  | F7 : Load Setup Defaults |  |  |

Resource	The Award Plug and Play BIOS has the capacity

Controlled by	to automatically configure all of the boot and Plug

and Play compatible devices. However, this

capability means absolutely nothing unless you

are using a Plug and Play OS such as Windows 95

Choices are Auto and Manual

Reset Config-	This item allows you to determine reset the

uration Data	configuration data or not.

PCI IRQ	This sets the method by which the PCI bus reco-

Actived By	gnizes that an IRQ service is being requested by a

device. Under all circumstances, you should retain

the default configuration unless advised otherwise

by  your system's manufacturer.

Choices are Level and Edge

PCI IDE IRQ	This allows you to configure your system to the

Map To	type of IDE disk controller in use. If you have

equipped your system with a PCI controller,

changing this allows you to specify which slot has

the controller and which PCI interrupt (A,B,C,D) is

associated with the connected hard disk. Select

'PCI Auto' allows the system to automatically

determine how your IDE disk system is configured.

This section page includes all the items of IDE hard drive and Programmed

Input/Output features. See also Section "Chipset FeaturesSetup".

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |  |
| --- | --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |  |
| CHIPSET FEATURES SETUP | USER PASSWORD |  |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |  |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |  |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |  |
| LOAD SETUP DEFAULTS |  |  |
| ESC: Quit | áâàß:Select Item |  |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |  |
| Time, Date, Hard Disk Type ..... |  |  |

ROM PCI/ISA BIOS (2A59GA29)

INTEGRATED PERIPHERALS

AWARD SOFTWARE, INC.

| IDE HDD Block Mode | : Enabled | ECP Mode Use DMA |  |  | : 3 |
| --- | --- | --- | --- | --- | --- |
| IDE 32-bit Transfer Mode | : Enabled |  |  |  |  |
| IDE Primary Master PIO | : Auto |  |  |  |  |
| IDE Primary Slave PIO | : Auto |  |  |  |  |
| IDE Secondary Master PIO | : Auto |  |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |  |
| On-Chip Primary PCI IDE | : Enabled |  |  |  |  |
| On-Chip Secondary PCI IDE | : Enabled |  |  |  |  |
| PCI Slot IDE 2nd Channel | : Enabled |  |  |  |  |
| USB Controller | : Disabled |  |  |  |  |
| Onboard FDD Controller | : Enabled |  |  |  |  |
| Onboard Serial Port 1 | : Auto |  |  |  |  |
| Onboard Serial Port 2 | : Auto | Esc: Quit áâàß |  | :Select Item |  |
| UART 2 Mode | : Standard | F1 : Help | PU/PD/+/- : Modify |  |  |
| Onboard Parallel Port | : 378H/IRQ7 | F5 : Old Values | (Shift) F2: Color |  |  |
| Onboard Parallel Mode | : ECP | F6 : Load BIOS Defaults |  |  |  |
|  |  | F7 : Load Setup Defaults |  |  |  |

IDE HDD Block	This allows your HD controller to use the fast block

Mode	mode to transfer data to and from your HD drive

| Enabled | IDE controller uses block mode |
| --- | --- |
| Disabled | IDE controller uses standard mode |

IDE Primary	PIO - Programmed Input/Output, it allows the

Master/Slave PIO	BIOS to tell the controller what it wants and then

IDE Secondary	let the controller and the CPU perform the complete

Master/Slave PIO	task by themselves. This simpler and more faster.

Your system supports five mods, 0 - 4, which

primarily differ in timing. When Auto is selected,

the BIOS will select the best available mode.

On-Chip Primary	This setup item allows you either to enable or

PCI IDE 	disable the primary/secondary controller. You might

On-Chip Second- 	choose to disable the controller if you were to add

ary PCI IDE 	a higher performance or specialized controller.

PCI Slot IDE 	This item allows you designate an IDE controller

2nd Channel 	controller board insert into one of the physical PCI

slots as your secondary IDE

| Enabled | External IDE controller designated as the secondary controller |
| --- | --- |
| Disabled | No IDE controller occupying a PCI slot |

The last step is 'save and exit'. If you select this item and press 'Y', then these records will be saved in the CMOS memory on the mainboard. It will be checked every time when you turn your computer on.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |  |
| --- | --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |  |
| CHIPSET FEATURES SETUP | USER PASSWORD |  |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |  |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |  |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |  |
| LOAD SETUP DEFAULTS |  |  |
| ESC: Quit | áâàß:Select Item |  |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |  |
| Time, Date, Hard Disk Type ..... |  |  |

| STANDARD CMOS SETUP |  | INTEGRATED PERIPHERALS |  |  |
| --- | --- | --- | --- | --- |
| BIOS FEATURES SETUP |  | SUPERVISOR PASSWORD |  |  |
| CHIPSET FEATURES SETUP |  | USER PASSWORD |  |  |
| POWER MANAGEMENT SETUP |  | IDE HDD AUTO DETECTION |  |  |
| PNP/PCI CONFIGURATION |  | SAVE & EXIT SETUP |  |  |
| LOAD BIOS DEFAULTS |  | EXIT WITHOUT SAVING |  |  |
| LOAD SETUP DEFAULTS | SAVE to CMOS and EXIT (Y/N):Y |  |  |  |
| ESC: Quit |  | áâàß:Select Item |  |  |
| F10: Save & Exit Setup |  | (Shift) F2 : Change Color |  |  |
| Auto-Configure HDD: Sector, Cylinder, Head ..... |  |  |  |  |

LOAD BIOS DEFAULTS

When your mainboard has problems and need to debug or 		troubleshoot the system, you can use this function.

The default values loaded only affect the BIOS Features

Setup, Chipset Features Setup, Power Management

Setup and PNP/PCI Configuration Setup. There is no

effect on the Standard CMOS Setup. To use this function,

select it from main menu and press <Enter>. A line will

appear on the screen asking if you want to load the BIOS

default values. Press <Yes> and <Enter> then the BIOS

default values will be loaded.

LOAD SETUP DEFAULTS

This allows you load optimal settings which are stored in

the BIOS ROM. The default values loaded only affect the

BIOS Features Setup, Chipset Features Setup, Power

Management Setup and PNP/PCI Configuration Setup.

There is no effect on the Standard CMOS Setup. To use

this function, select it from main menu and press <Enter>.

A line will appear on the screen asking if you want to load

the Setup default values. Press <Yes> and <Enter> then

the Setup default values will be loaded.

SUPERVISOR PASSWORD / USER PASSWORD

This allows you to set the password. the mainboard 	defaults password disabled.

Enter/Change password : Enter the current password, at

the prompt key-in your new password (up to eight

alphanumeric characters), press <Enter>. At the next

prompt, confirm the new password by typing it and press

<Enter> again.

Disable password : Press <Enter> key instead of

entering a new password when the 'Enter Password'

dialog box appears. A message will appear confirming that

the password is disable.

If you set both supervisor and user passwords, only the

supervisor password allows you to enter the BIOS SETUP

program.

CAUTION :If you forget your password, you must disable

the CMOS by turning power off and set JP8 'close'. Then

reload the system. Please refer to page 13.

IDD HDD AUTO DETECTION

This allows you to detect IDE hard drivers' parameters

and enter them into 'Standard CMOS Setup' automatically.

If the auto-detected parameters displayed do not

match the ones that should be used for your hard drive,

do not accept them. Press <N> to reject the values

and enter the correct ones manually on the Standard

CMOS Setup screen.

SAVE & EXIT SETUP

This allows you to save the new setting values in the

CMOS memory and continue with the booting process.

Select what you want to do, press <Enter>.

EXIT WITHOUT SAVING

This allows you to exit the BIOS setup utility without

recording any new values or changing old ones.

Control Key Description

| UP ARROW |  | Move to previous item |
| --- | --- | --- |
| DOWN ARROW |  | Move to next item |
| LEFT ARROW |  | Move to the item in the left hand |
| RIGHT ARROW |  | Move to the item in the right hand |
| Esc KEY | Esc | Main Menu : Quit and not save changes Setup menu : Exit current page and return to main menu |
| PgUp KEY |  | Increase the numeric value or make changes |
| PgDn KEY |  | Decrease the numeric value or make changes |
| F1 KEY | Help | General help |
| F2 KEY | Shift+F2 | Change color from total 16 colors |
| F5 KEY | Old Value | Restore the pervious CMOS value from CMOS |
| F6 KEY | Load BIOS default | Load the default CMOS value from BIOS default table |
| F7 KEY | Load setup default | Load Setup default |
| F10 KEY | Save & Exit Setup | Save all the CMOS changes and Exit setup, only for Main Menu |

APPENDIX A

TECHNICAL SUPPORT REQUEST FORM

If the mainboard doesn't function properly, please complete the following

information and return it to your system dealer. If the further information

is needed, please attach this separating sheets.

Model No :  ATC-1020      Date of  Purchase :  ______________

Serial No : ___________________

HARDWARE :

|  | BRAND | MODEL | SPEED | Q'TY |
| --- | --- | --- | --- | --- |
| CPU |  |  |  |  |
| SIM Module |  |  |  |  |
| PB. Cache on Board |  |  |  |  |
| TAG SRAM |  |  |  |  |
| Sync. SRAM Module |  |  |  |  |

SIMM  :_______MB (__ EDO, __FastPage)

Hard Disk Interface Controller : __ __ IDE, ____ SCSI

Hard Disk Brand :___________, Model :_________, Capacity :________

Display Controller Brand :_____________,Model :__________

Controller Chip Brand :_____________,Model :__________

AWARD SYSTEM BIOS: Version _______  Date Code _____________

Keyboard BIOS: Brand __________

Other Add-on Cards Information:

| Add-on Card | Bus Interface | Model | Remark |
| --- | --- | --- | --- |