Table of Contents

JUMPER COMPONENT LOCATION DIAGRAM	A

µCHAPTER 1   INTRODUCTION	2

1-1 SYSTEM FEATURES	3

1-2 SOFTWARE POWER OFF CONTROL	4

1-3 CHECK LIST OF THE PACKING	5

CHAPTER 2   INSTALLATION	6

2-1 INSTALLATION PROCEDURES 	6

2-2 CPU INSTALLATION	7

2-3 SYSTEM MEMORY INSTALLATION	16

2-4 SRAM INSTALLATION	18

2-5 OTHER JUMPERS AND CONNECTORS 
       DESCRIPTION 	19

2-6 IDE DRIVER INSTALLATION	23

CHAPTER 3   AWARD BIOS SETUP	24

3-1 UPDATE BIOS PROCEDURES	25

3-2 AWARD SYSTEM BIOS CONFIGURATION SETUP	26

APPENDIX A 	52

CHAPTER 1 INTRODUCTION

This manual describes to configure the ATC-5000 mainboard for different environments. It's an overview of the layout and features of the mainboard, and also provides information for you to change the configuration or system environment.

This manual is divided into two sections :

PART ONE   includes page A and two chapters as following:

Page A contains layout diagram of the mainboard.

Please refer to it when you configure the system.

Chapter 1    is an overview of the mainboard features and packing

contents.

Chapter 2    describes how to upgrade and to change hardware

configurations such as memory size, CPU type, and

lists of jumper settings and connectors.

PART TWO  includes chapter 3 which contains Award BIOS

description..

Chapter 3    is the user's guide of the Award BIOS setup utility and

Flash ROM BIOS. The menu shown in this chapter are the

default settings.

Your system dealer will set up the mainboard according to your demand of the computer. It means that the current settings of your mainboard may not be the same as the defaults shown in this user's manual. If you need to change your configuration, please ask your dealer first. Be sure this will not void your system warranty, or ask your dealer to do it for you.

REMARK

Intel®  is a registered trademark of Intel Corporation.

All other brands and product names are trademarks registered trademarks of their respective companies.

1-1 SYSTEM FEATURES

Intel Pentium (P54C) CPU operating at 90MHz to 200MHz     ie. Intel Pentium MMXTM 166~233MHz, AMD K5, AMD K6,      Cyrix 6x86MXTM, 6x86L.

INTEL 82430TX PCIset.

Using four 72-pin SIMM sockets, provides two banks of 64-bit wide path up to 256MB addressing page mode DRAMs.

Using two 168-pin DIMM sockets for 3.3V SDRAM or EDO DRAM module.

Supporting two types of DRAM including EDO (Extended Data Out), or FPM (Fast Page Mode).

Built-in 256KB or 512KB PB SRAM on board

Supporting four PCI bus master revision 2.1, 5V interface compliant and three 16-bit ISA slots.

Dual Master IDE connectors support Ultra UMA/33 (33MB/sec transfer rate), up to four devices in two channels for connection of high capacity hard disk drive, CD-ROM, tape backup etc..

AT style keyboard connector and PS/2 mouse connector.

SMC Ultra Multi-I/O chipset, integrated Keyboard controller.

Supporting Infrared transfer (IrDA TX/RX) connection.

One FDC port supports two devices up to 2.88MB

Two 16550A fast UARTs compatiable serial ports

One EPP/ECP mode parallel port

Supporting Dual channel USB connection.

Built-in Switching Voltage Regulator

Supports ACPI Power Management - OSPM (OS directed power management)

Optional ATX power connector support Soft-OFF Control.

PCB Dimension is 220mm x 260mm (8.66" x 10.24") with four layers designed.

1-2 SOFT-OFF CONTROL (option)

The mainboard design supports Soft-Off Control feature through the SMM code in the BIOS under Windows 95, Windows 3.1x, and MS-DOS operation system environment. It needs to use ATX power supply. (please contact your supplier for this option)

Firstly, you should connect power switch cable (provided by ATX case supplier) to the connector JP7 (next to system BIOS socket) on the mainboard.

In Windows 95, if you would like to soft power off the system, you just choose “shutdown the computer ?” in “Shut Down Windows“ from Windows 95, then push the Power Switch* (on ATX case), then system will be in soft off status directly. If you would like to restart the system, just press the button of the power switch, and the system will be power on.

- * Default value (Power management of BIOS screen) is “Instant-off‘, you can change to ’Delay 4 Sec.‘, then you should push the Power Switch at least 4 seconds otherwise the system will be under SMI mode only.

- In Windows 3.1x or MS-DOS, you should copy the program of “down.com” (you can find it in the diskette of IDE driver) into hard drive. When you would like to power off the system, just run this “down.com”, then the system will be shutdown and stay in standby status.

- Note : If you will leave your system several days, we suggest you use hardware mechanical power off to shutdown your system.

- 1-3 CHECK LIST OF THE PACKING

- The mainboard comes securely packed in a durable box and shipping carton. If any of the above items are missing or damaged, please contact your supplier.

- Each mainboard containing:

- Q'TY     	Description

- 1       	Mainboard	: ATC-5000.

- 1	Diskette		: Enhanced IDE driver (3.5").

- 1         	Cable 		: Enhanced IDE connector.

- 1         	Cable 		: F.D.D. connector.

- 1	Cable 		: Serial port.

- 1         	Cable 		: Serial/Parallel.

- 1         	Manual 		: User`s manual.

- NOTE : Leave the mainboard in its original packing until you are ready to install it.

- CHAPTER 2 INSTALLATION

- 2-1 INSTALLATION PROCEDURE

- Before installing the computer, please prepare all components such as CPU, DRAM; peripherals such as hard disk devices, keyboard, CD-ROM drive; and accessories such as cables. Then, install the system as following :

- 1. Plug CPU, heat sink, cooling fan, and DRAM modules in

- the mainboard.

- 2. Set jumpers based on your configuration.

- 3. Plug add-on cards in PCI/ISA slots.

- 4. Connect cables to peripherals, power supply..

- 5. Make sure all components and devices are well

- connected, turn on the power and setup System BIOS

- based on your configuration.

- 6. Install peripheral, add-on card drivers and test them.

- 7. If all of above procedures are success, turn-off the power

- then plug all of them into your computer case.

- 2-2 CPU  INSTALLATION

- ATC-5000 supports one Pentium level CPU.

- It can auto detect single or dual CPU voltage power supply type without jumper setting. For installation, please notice CPU pin 1 must align with the ZIF socket 7 Pin 1 location. Before you install or upgrade your CPU, please read CPU guide from CPU manufacturer to make sure the CPU voltage specification. Then choose the right installation in section 2-2-1 based on your CPU type/brand and follow the description to setup jumpers. If your CPU is not in the list of 2-2-1, please refer to 2-2-2 and 2-2-3 for installation.

- 2-2-1 CPU TYPE SELECTION

A. INTEL PENTIUM CPU (P54C)

¡ P54C VRE : 3.400V ~ 3.600V (The fourth line of the mark on the

under-side of the processor contains a code that identifies

the voltage level type. V is VRE, S is standard. )

JP6

1-2

INTERNAL CPU CLOCK

JP3

JP2

Ext.x Frq.

120MHz

1-2, 5-6

2-3, 4-5

60x2.0

133MHz

1-2, 4-5

2-3, 4-5

66x2.0

150MHz

1-2, 5-6

2-3, 5-6

60x2.5

166MHz

1-2, 4-5

2-3, 5-6

66x2.5

180MHz

1-2, 5-6

1-2, 5-6

60x3.0

200MHz

1-2, 4-5

1-2, 5-6

66x3.0

JP3

1ꅃꅃ4

2ꅃꅃ5

3ꅃꅃ6

JP2

1ꅃꅃꅃ4 7

2ꅃꅃꅃ5 8

3ꅃꅃꅃ6 9

¡ P54C STD : 3.135V ~ 3.600V (The fourth line of the mark on the

under-side of the processor contains a code that identifies

the voltage level type. V is VRE, S is standard. )

JP6

3-4

INTERNAL CPU CLOCK

JP3

JP2

Ext.x Frq.

120MHz

1-2, 5-6

2-3, 4-5

60x2.0

133MHz

1-2, 4-5

2-3, 4-5

66x2.0

150MHz

1-2, 5-6

2-3, 5-6

60x2.5

166MHz

1-2, 4-5

2-3, 5-6

66x2.5

180MHz

1-2, 5-6

1-2, 5-6

60x3.0

200MHz

1-2, 4-5

1-2, 5-6

66x3.0

Intel Pentium CPU, the first letter after ႑/႒ denotes voltage type.

xxxxxxxxx

xxxxxxxxx

xxxxxxxxx

Sxxxx/SMU

JP3

1ꅃꅃ4

2ꅃꅃ5

3ꅃꅃ6

JP2

1ꅃꅃꅃ4 7

2ꅃꅃꅃ5 8

3ꅃꅃꅃ6 9

B. INTEL PENTIUM MMXTM CPU (P55C)

JP6

9-10

INTERNAL CPU CLOCK

JP3

JP2

Ext.x Frq.

166MHz

1-2, 4-5

2-3, 5-6

66x2.5

200MHz

1-2, 4-5

1-2, 5-6

66x3.0

233MHz

1-2, 4-5

1-2, 4-5

66x3.5

JP3

1ꅃꅃ4

2ꅃꅃ5

3ꅃꅃ6

JP2

1ꅃꅃꅃ4 7

2ꅃꅃꅃ5 8

3ꅃꅃꅃ6 9

C. Cyrix 6x86(028) CPU

JP6

1-2

INTERNAL CPU CLOCK

JP3

JP2

Ext.x Frq.

PR150+ @ 120MHz

1-2, 5-6

2-3, 4-5

60x2.0

PR166+ @ 133MHz

1-2, 4-5

2-3, 4-5

66x2.0

6x86-PRxx+GP

xxxMHz

3.52V(028)

The bottom line of the mark on the processor contains a code 028 that identifies the voltage level type. If the code is 016 or others, or no marking then please contact your dealer.

JP3

1ꅃꅃ4

2ꅃꅃ5

3ꅃꅃ6

JP2

1ꅃꅃꅃ4 7

2ꅃꅃꅃ5 8

3ꅃꅃꅃ6 9

D. Cyrix 6x86L CPU (dual voltage)

JP6

9-10

INTERNAL CPU CLOCK

JP3

JP2

Ext.x Frq.

PR150+ @ 120MHz

1-2, 5-6

2-3, 4-5

60x2.0

PR166+ @ 133MHz

1-2, 4-5

2-3, 4-5

66x2.0

PR200+

2-3, 4-5

2-3, 4-5

75x2.0

6x86L-PRxx+GP

xxxMHz

2.8V

The mark on the processor is shown as a sample on the left. The code-name 6x86L is dual voltage mode processor.

JP3

1ꅃꅃ4

2ꅃꅃ5

3ꅃꅃ6

JP2

1ꅃꅃꅃ4 7

2ꅃꅃꅃ5 8

3ꅃꅃꅃ6 9

E. AMD-K5 CPU

JP6

1-2

INTERNAL CPU CLOCK

JP3

JP2

Ext.x Frq.

PR90

1-2, 5-6

1-2, 4-5

60x1.5

PR100

1-2, 4-5

1-2, 4-5

66x1.5

PR120

1-2, 5-6

1-2, 4-5

60x1.5

PR133

1-2, 4-5

1-2, 4-5

66x1.5

PR166

1-2, 4-5

2-3, 5-6

66x2.5

F. AMD-K6 CPU

INTERNAL CPU CLOCK

JP6

JP3

JP2

Ext.x Frq.

PR166

7-8

1-2, 4-5

2-3, 5-6

66x2.5

PR200

7-8

1-2, 4-5

1-2, 5-6

66x3.0

PR233

5-6

1-2, 4-5

1-2, 4-5

66x3.5

JP2

1ꅃꅃꅃ4 7

2ꅃꅃꅃ5 8

3ꅃꅃꅃ6 9

JP3

1ꅃꅃ4

2ꅃꅃ5

3ꅃꅃ6

2-2-2 CPU VOLTAGE SETTING

ATC-5000 can auto detect single or dual CPU voltage supply type without jumper setting. JP6 is used for various CPU voltage value types, please refer to CPU‘s marking on/under CPU.

¡CPU Core Voltage Type : JP6

INTEL STD(P54C)

INTEL VRE(P54C)

10  9

8  7

6  5

4  3

2  1

10  9

8  7

6  5

4  3

2  1

AMD K5

INTEL MMXTM (P55C)

10  9

8  7

6  5

4  3

2  1

10  9

8  7

6  5

4  3

2  1

CYRIX 6x86

CYRIX 6x86L

10  9

8  7

6  5

4  3

2  1

10  9

8  7

6  5

4  3

2  1

AMD-K6/PR233

AMD-K6/PR166,200

10  9

8  7

6  5

4  3

2  1

10  9

8  7

6  5

4  3

2  1

2-2-3 CPU CLOCK SETTING

EXTERNAL CPU CLOCK

JP3

60(MHz)

1-2, 5-6

66(MHz)

1-2, 4-5

75(MHz)

2-3, 4-5

60MHz          66MHz          75MHz

JP3  1

2

3

456

JP3

JP3

INTERNAL CPU CLOCK

JP2

INTEL EXTERNAL CLOCK X 1.5

1-2, 4-5

INTEL EXTERNAL CLOCK X 2.0

2-3, 4-5

INTEL EXTERNAL CLOCK X 2.5

2-3, 5-6

INTEL EXTERNAL CLOCK X 3.0

1-2, 5-6

INTEL EXTERNAL CLOCK X 3.5

1-2, 4-5

x 1.5       x 2.0        x 2.5        x 3.0       x 3.5

JP2  1

2

3

4

5

6

2-3 SYSTEM MEMORY INSTALLATION

ATC-5000 provides four 72-pin SIMM sockets for system memory expansion from 4MB to 256MB. These four SIMMs are arranged to two banks, Bank0 (SIM 1, 2) and  Bank1(SIM 3, 4), please refer to page A. Each bank provides 64-bit wide data path.

The mainboard accepts Fast Page Mode DRAM, and EDO Mode (Extended Data Out) DRAM, with a speed no slower than 70 nanosecond. You should plug DRAM modules into two sockets (same bank) or four sockets at one time. Each pair of modules in the same bank must be the same size, type, and speed; no matter single-side or double-side module. Please plug in Bank 0 firstly if you only have 2 modules. The mainboard supports mixing of EDO and fast page mode DRAM among different banks, please plug EDO in Bank 0.

Also this mainboard provides two optional 168-pin DIMM sockets for 3.3V SDRAM or 3.3V EDO DRAM expansion. You should plug SDRAM/DRAM module into each DIMM sockets (as a bank) or two sockets at one time.

CAUTION: It‘s not recommended to install the 3.3V SDRAM and 5V EDO or Fast Paged mode memory within a system. The 72-pin DRAM cannot work with 168-pin DRAM in the same time. Changing EDO/FPM DRAM to SDRAM, you don‘t have to adjust jumper setting or BIOS value, nor change SDRAM to EDO/FPM DRAM. (¡ Please make sure the SDRAM plugged-in fully, to prevent contact loss.)

¡ System Memory Combinations Options ¡

BANK0

BANK1

Total Memory

SIM 1, 2

SIM 3, 4

SIM 1- 4

4MBx2

-

8MB

-

4MBx2

8MB

8MBx2

-

16MB

-

8MBx2

16MB

4MBx2

4MBx2

16MB

4MBx2

8MBx2

24MB

8MBx2

4MBx2

24MB

16MBx2

-

32MB

-

16MBx2

32MB

8MBx2

8MBx2

32MB

4MBx2

16MBx2

40MB

16MBx2

4MBx2

40MB

8MBx2

16MBx2

48MB

16MBx2

8MBx2

48MB

32MBx2

-

64MB

-

32MBx2

64MB

16MBx2

16MBx2

64MB

4MBx2

32MBx2

72MB

32MBx2

4MBx2

72MB

8MBx2

32MBx2

80MB

32MBx2

8MBx2

80MB

16MBx2

32MBx2

96MB

32MBx2

16MBx2

96MB

32MBx2

32MBx2

128MB

64MBx2

-

128MB

-

64MBx2

128MB

- continue -

4MBx2

64MBx2

136MB

64MBx2

4MBx2

136MB

8MBx2

64MBx2

144MB

64MBx2

8MBx2

144MB

16MBx2

64MBx2

160MB

64MBx2

16MBx2

160MB

32MBx2

64MBx2

192MB

64MBx2

32MBx2

192MB

64MBx2

64MBx2

256MB

*128MBx2

-

256MB

-

*128MBx2

256MB

* Please confirm this with your supplier firstly.

2-4 SRAM  INSTALLATION

ATC-5000 is built-in 512KB Sync. Pipeline Burst SRAM on board

2-5 OTHER JUMPERS AND

CONNECTORS  DESCRIPTION

When you plug a cable into the following I/O connectors, you should have the pin 1 edge of the cable aligned with the pin 1 end of the connector.

CONN1 : speaker, power LED, reset, SMI,

turbo LED, and IDE LED connectors.

IDE-LED TB-LED  SMI   RST

KEYLOCK       SPK

+       +

SPK : speaker

VCC

GND

GND

Speaker

KEYLOCK (    Power LED connector)

GND (power)

N/C

Power LED +

RST : Reset connector

GND

Reset Signal

SMI : SMI lead

SMI Signal

GND

TB-LED : Turbo LED indicator

+5V

GND

IDE-LED : IDE devices indicator LED

connector. IDE-LED stays ON indicates    +5V  on-board IDE devices in operation. If

GND plug wire into wrong connector, color

of LED will be lighter and the IDE dvices

can still function properly.

FAN1 : CPU cooling fan connector. Wire with +12V

1     GND 2    +12V

3     GND

voltage (most likely red wire) must be plugged

into pin2, and GROUND wires (most likely

black wires) must be plugged into pin1 and/or

pin3. please confirm the wire color represtation

with your supplier.

CAUTION: Plug wire into wrong connector will

DAMAGE fan and mainboard.

1 +5V

2N/C

3IRRX

4GND

5IRTX

IR1 : Infrared module connector.

USB1 : USB connector; Universal Serial Bus;this

(red)

VCC GND

D1 -D0 +

D1+D0 -

GNDVCC

OC0OC1

(black)

is used to connect USB devices through an

optional dual head cable with a iron plane.

OC0 and OC1 are used to mention the status

of the USB power supply lines.

CAUTION: Plug wire into wrong connector

will DAMAGE USB devices and mainboard.

USB

KB1 : 5-pin AT style keyboard connector.

PS/2 Mouse: connector, which is used to

4 6

2

1

35

connect an optional cable.

pin1 : data

pin2 : N/C

pin3 : GND

pin4 : VCC

pin5 : clock

pin6 : N/C

PW1 : +5 Voltage power supply connector.(P8,P9)

PG   +12V    GND          +5V

+5V   -12V          -5V

PW2 : ATX mode +3.3/5/12V power supply

connector.(optional)

13.3V6+5V113.3V16GND23.3V7GND12-12V17GND3GND8PWRGD13GND18-5V4+5V95VSB14*PS_ON19+5V5GND10+12V15GND20+5V* PS_ON : Soft-Off power control

1           10

11             20

COM1/COM2 : these two connectors are

1 2

34

56

78

910

used to connect serial port cables.

pin

signal name

1

DCD

2

Serial In

3

Serial Out

4

DTR

5

GND

6

DSR

7

RTS

COM1/COM2

8

CTS

9

RI

10

N/C

FDC : this connector is used to connect floppy

12

3334

disk drive through cable.

pin

signal

pin

signal

2

RWC-

20

STEP-

4

reserved

22

Write Data

6

FDEDIN

24

Write Gate

8

Index-

26

Track 00-

10

Motor EnableA-

28

Write Protect-

12

Drive Sele.B-

30

Read Data-

14

Drive Sele.A-

32

Side 1 Sele.-

16

Motor EnableB-

34

Di

sketteCh

an

ge

8

DIR-

All of odd pins are ground(except pin 5)

LPT : this

| c |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

onnector is used to connect parallel

114

215

1326

port cable.

| pi |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

n

signal

pinsignal

1

STROBE-

10

ACK-

2

Data Bit 0

11

B

| US |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

Y

3

Data Bit 1

12

PE

4

Data Bit 2

13

SLCT

5

Data Bit 3

14

A

| ut |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

o Feed-

6

Data Bit 4

15

ERROR-

7

Data Bit 5

16

INIT-

8

Data

| B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

it 6

17

SLCT IN-

9

Data Bit 7

26

N/C

pin18 -- pin25 are groun

| d |  |  |  |
| --- | --- | --- | --- |

IDE1/IDE2 : these two connectors are used to

connect max. 4

| de |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

vices through IDE cable.

12

34

3940

pin

signalpinsignal

1

Reset IDE

21

DDRQ0(1)

2

GND

22

GND

3

Host Data 7

23

I/O Write-

4

Host Data 8

24

GND

5

Host Data 6

25

I/O Read-

6Host Data 9

26

GND

7

Host Data 5

27

IORDY

8

Host Data 10

28

N/C

9

Host Data 4

29

DDAK0-(1-)

10

Host Data 11

30

GND

LPT

11

Host Data 3

31

IRQ14*

12

Host Data 12

32

IOCS16-

13

Host Data 2

33

Addr 1

*

IDE1: pin31 is IRQ14; IDE2: pin31 is IRQ15

or MIRQ0

14

Host Data 13

34

N/C

15

Host Data 1

35

Addr 0

16

Host Data 14

36

A

dd

r 2

17

Ho

st Data 0

37

ChipSele. 1P-

18

Host Data 15

38

ChipSele 3P-

19

GND

39

Activity

| 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

0

Key

40

GND

2-6 IDE DRIVER INSTALLATION

Setup for Windows 95 : (Windows 95 version 950/950a/950b)

1. Starting Windows 95

2. Select “START”, “RUN”.

3. Install INF.EXE before you install IDE driver, please refer to

readme file.

4. Type “A:\WIN95\SETUP.EXE”.

5. Restart computer, then follow the instructions on your

screen to install new IDE driver we offer in the 3.5“ diskette

6. Exit Windows 95, turn power off; then turn power on.

After installation, the screen will show a yellow ¡, please ignore it.

(The other platforms please refer to readme file.)

Make sure your HDD should follow ATA standard, and your CD-ROM should follow ATAPI standard. When you plug-in the IDE devices, please plug your first and second devices into IDE 1 port (Master then Slave), then plug third and forth devices into IDE 2 port. If you have CD-ROM drive, please set it behind hard disk devices as the last device. For example, if you have 2 HDDs and 1 CD-ROM, you should set HDD1 and HDD2 in IDE1 Master and Slave, set CD-ROM in IDE 2 Master. Some of the brands devices combination may not work under this sequence, you can try to re-arrange the devices sequence, or contact your vendor.

Primary MasterPrimary SlaveSecondary MasterSecondary SlaveATA no ATAPIATAATAPI disk & CD-ROMATAATAPI use only one cableATAATAPIATAPI CD-ROM and a tape

or two CD-ROMs

CHAPTER 3  AWARD BIOS

SETUP

This chapter explains the system BIOS setup, and how to update new BIOS. All BIOS screens showed in the following pages are default values, your system dealer will set up these

values according to your demand of computer.

ATC-5000 uses Flash ROM to make BIOS easier to be updated by the floppy disk-based program and to committe Microsoft

Windows 95 plug & play feature.

The update CMOS process is mechanical power off (or unplug the power cord) to ensure there is no power to the mainboard, firstly. Set JP8 to ‘open’, then the CMOS is cleared. Reset is set JP8 to ‘close’, power on the power switch (or plug the power cord).

JP8

Nomal  Clear

close

NORMAL¡

open

CLEAR CMOS

¡is default setting

3-1 UPDATE BIOS PROCEDURES

If the BIOS needs to be updated, it can be obtained on a diskette from your system supplier. The BIOS diskette includes 3 files:

“awdflash.exe”  --  BIOS update utility program

“awdflash.doc”

“(update BIOS filename with version number).bin”

The update procedures are as following:

Boot the system to DOS mode in a normal manner.

Insert the updated diskette to drive A (or B).

Change working directory to floppy drive, A or B, which contains the update BIOS diskette. --  Type “a:\” or “b:\”, “ENTER”.

Run the BIOS update utility  --  Type “awdflash”, “ENTER”.

Type “(update BIOS file name with version n

| um |  |  |
| --- | --- | --- |

ber).bin”, ENTER.

Type “N” when the screen d

| is |  |  |
| --- | --- | --- |

plays the message : " Do you want to save BIOS (Y/N) ?".

Type “Y“ when the screen shows the message : " Are you sure to  program (Y/N) ?".

Follow instructions displayed on the screen.  DO NOT remove the update BIOS diskette from the floppy drive nor turn the system power off until the BIOS update is completed.

Turn the system power on and test your system working properly or not.

3-2 AWARD SYSTEM BIOS

CONFIGURATION SETUP

- The following pages explain how to set up the system configuration (CMOS) under the AWARD BIOS. The SETUP program is stored in the Read-Only-Memory (ROM) on the mainboard. Enter the SETUP procedure, press the <Del> key when the system is booting up. The following main menu will appear. Please select " STANDARD CMOS SETUP" to enter the next screen.

- ROM PCI/ISA BIOS (2A59IA2A)

- CMOS SETUP UTILITY

- AWARD SOFTWARE, INC.

- STANDARD CMOS SETUP

- INTEGRATED PERIPHERALS

- BIOS FEATURES SETUP

- SUPERVISOR PASSWORD

- CHIPSET FEATURES SETUP

- USER PASSWORD

- POWER MANAGEMENT SETUP

- IDE HDD AUTO DETECTION

- PNP/PCI CONFIGURATION

- SAVE & EXIT SETUP

- LOAD BIOS DEFAULTS

- EXIT WITHOUT SAVING

- LOAD SETUP DEFAULTS

- ESC: Quit

- áâàß:Select Item

- F10: Save & Exit Setup

- (Shift) F2 : Change Color

- Time, Date, Hard Disk Type .....

- The section on the bottom of the main menu explains how to control this screen.  The other one section displays the items highlighted in the list.

- This screen records some basic hardware information, and set the system clock and error handling. These records can be lost or corrupted if the on-board battery  is failed or  weak.

- ROM PCI/ISA BIOS (2A59IA2A)

- CMOS SETUP UTILITY

- AWARD SOFTWARE, INC.

- STANDARD CMOS SETUP

- INTEGRATED PERIPHERALS

- BIOS FEATURES SETUP

- SUPERVISOR PASSWORD

- CHIPSET FEATURES SETUP

- USER PASSWORD

- POWER MANAGEMENT SETUP

- IDE HDD AUTO DETECTION

- PNP/PCI CONFIGURATION

- SAVE & EXIT SETUP

- LOAD BIOS DEFAULTS

- EXIT WITHOUT SAVING

- LOAD SETUP DEFAULTS

- ESC: Quit

- áâàß:Select Item

- F10: Save & Exit Setup

- (Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

ROM PCI/ISA BIOS (2A59IA2A)

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

Date (mm:dd:yy) : Fri, Feb  14 1997

Time(hh:mm:ss)   : 13 : 7 : 14

HARD DISKS  TYPE   SIZE  CYLS  HEAD  PRECOMP LANDZ SECTOR MODE

Primary Master  : Auto

0

0

0

0      0

0

Auto

Primary Slave   : Auto

0

0

0

0      0

0

Auto

Secondary Master : Auto

0

0

0

0      0

0

Auto

Secondary Slave  : Auto

0

0

0

0      0

0

Auto

Drive A : 1.44M,  3.5  in.

Base Memory           :        640K

Drive B : None

Extended memory       :      64512K

Floopy 3 Mode support : Disabled

Other Memory            :        384K

------------------------------------------------

Video  : EGA/VGA

Total Memory             :      65536K

Halt On: All Errors

ESC  :  Quit                 áâàß:Select Item       PU/PD/+/- : Modify

F1   :  Help                (Shift) F2 : Change Color

Date	mm is month, dd is date, yy is year.

date

from 1 to

| 31 |  |  |
| --- | --- | --- |

month

from Jan. to Dec.

year

from 1900 to 2099

Time	hh is hour, mm is mi

| nu |  |  |
| --- | --- | --- |

te, ss is second.

hh

from 0 to 23 (24-hour military -time)

mm

from 1

| to |  |  |
| --- | --- | --- |

59

ss

from 1 to 59

Primary Master	The categories identify the types of 2

Primary Sla

| ve |  |  |
| --- | --- | --- |

channels that have been installed in the

Secondary Master	computer. There are 45 p

| re |  |  |
| --- | --- | --- |

defined types

Secondary Slave	and 4 user definable types are for Enhanced

I

| DE |  |  |
| --- | --- | --- |

BIOS. Type 1 to 45 are predefined. Type

‘user’is user- definable. Press PgUp/PgDn

to select a numbered hard disk type or type

the number and press<Enter>. If you select

‘Auto’ BIOS will auto-detect the HDD & CD-

ROM Drive at the POST stage and showing

the IDE for HDD & CD-ROM Drive. If you s

| el |  |  |
| --- | --- | --- |

ect

‘user’, you will need to know the information

listed below. Enter the information directly

from the keyboard and press <Enter>.

This information should

b

e from your hard disk 			vender or dealer. If the controller of HDD

interface is ESDI, the selection shall be  				‘Type	1’; is SCSI, the selection shall be

‘None’. If the device has not been installed

select‘NONE’ and press <Enter>.

Type

drive type

SIZE

automatically adjusts

CYLS number of cylinders

HEAD

number of heads

PRECOMP

write precom

LANDZ

landing zone

SECTOR

number of sectors

MODE

mode type

Drive A	The category identifies the types of floppy

Drive B	disk drive A or drive B that have been 				installed in the computer.

None

No floppy drive installed

360K, 5.25 in

5.25“ PC-type 360KB capacity

1.2M, 5.25 in

5.25“ AT-type 1.2MB capacity

720K, 3.5 in

3.5“ double-side 720KB capacity

1.44M, 3.5 in

3.5“ double-side 1.44MB capacity

2.88M, 3.5 in

3.5“ double-side 2.88MB capacity

Floopy 3 mode   This is the Japanese standard floppy drive.

| su |  |  |
| --- | --- | --- |

pport         The standard stores 1.2MB in a 3.5“ diskette

Video	The category se

| le |  |  |
| --- | --- | --- |

cts the type of video adapter

used for the primary system monitor. Altho

| ug |  |  |
| --- | --- | --- |

h

secondary monitors are supported, you do not

have to select the 	type in Setup.

Halt

| O |  |  |
| --- | --- | --- |

n	The category determines whether the computer

will stop if an error is detected d

| ur |  |  |
| --- | --- | --- |

ing power up.

No errors

When the BIOS detects a non-fatal error the system w

| il |  |  |
| --- | --- | --- |

l be stopped and you will be prompted

Al

| l |  |  |
| --- | --- | --- |

errors

The system boot will not be stopped for any error that may be detected

All, But

Keyboard

The system boot will not stop for a keyboard error, it will stop for all other errors

All, But

Diskette

The system boot will not stop for a disk error, it will

| s |  |  |
| --- | --- | --- |

top for all other errors

All, But

Disk/Key

The system boot will not stop for a disk or keybo

| ar |  |  |
| --- | --- | --- |

d error, it will stop for all other errors

Memory	The catego

ry

is display-only which is deter-

mined by POST (Power On Self Test) of the

BIOS. Base Memory  The value of the base

memory is typicall

| y 512K or 640K based on the memory installed on the mainboard. Extended Memory How much extended memory is present during the POST. This is the amount of memory located above 1MB in the CPU‘s memory address map. Other Memory This refers to the memory located in the 640K to 1024K address space. The BIOS is the most frequent user of this RAM area since this is where it shadows RAM. This screen is a list of system configuration options. Some of them are defaults required by the mainboard's design, others depend on the features of your system. ROM PCI/ISA BIOS (2A59IA2A) CMOS SETUP UTILITY AWARD SOFTWARE, INC. STANDARD CMOS SETUP INTEGRATED PERIPHERALS BIOS FEATURES SETUP SUPERVISOR PASSWORD CHIPSET FEATURES SETUP USER PASSWORD POWER MANAGEMENT SETUP IDE HDD AUTO DETECTION PNP/PCI CONFIGURATION SAVE & EXIT SETUP |  |  |
| --- | --- | --- |
| LOAD BIOS DEFAULTS EXIT WITHOUT SAVING LOAD SETUP DEFAULTS ESC: Quit áâàß:Select Item F10: Save & Exit Setup (Shift) F2 : Change Color Virus, Protection, Boot Sequence ROM PCI/ISA BIOS (2A59IA2A) BIOS FEATURES SETUP AWARD SOFTWARE, INC. Virus Warning : Disabled Video BIOS Shadow : Enabled CPU Internal Cache : Enabled C8000-CBFFF Shadow : Disabled External Cache : Enabled CC000-CFFFF Shadow : Disabled Quick Power On Self Test : Disabled D0000-D3FFF Shadow : Disabled Boot Sequence : A, C, SCSI D4000-D7FFF Shadow : Disabled Swap Floppy Drive : Disabled D8000-DBFFF Shadow : Disabled Boot Up Floppy Seek : Enabled DC000-DFFFF Shadow : Disabled | Boot Up NumLock Status : On Boot Up System Speed : High Typematic Rate Setting : Disabled Typemat |  |

ic Rate(Chars/Sec)

: 6

Esc : Quit   áâ

| àß:Select Item Typematic Delay(Msec) : 250 |  |  |
| --- | --- | --- |
| F1 : Help PU/PD/+/- : Modify Security Option : |  |  |

Setup

F5  : Old Values  (SHIFT)F2 : Color

PCI/VGA Palette Snoop

: Disabled

F6   : Load BIOS

| Defaults OS/2 select for DRAM>64MB : Non-OS2 F7 : Load Setup Defaults Report No FDD for WIN 95 : NO Virus Warning When this item is enabled, the BIOS will |  |  |
| --- | --- | --- |

monitor the boot sector and partition table

of the hard disk drive for any attempt at

modification. If an attempt is made, the BIOS

will halt the system and the following error

message will appear. Many disk diagnostic

programs which attempt to access the boot

sector table can cause the above warning

message. If you will be running such a

program, we recommend that you first 					disable Virus Protection beforehand.

! WARNING !

Disk boot sector is to be modified

Type ‘Y’ to accept write or ‘N’ to abort write

Award Software, Inc.

Enabled

Activates automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector or hard disk partition table.

Disabled

No warning message will appear when anything attempts to access the boot sector or hard disk partition table.

CPU	These two categories speed up memory

Internal Cache	access.However, it depends on CPU/

External Cache	chipset design.The default value

Quick Power On	is‘enabled‘.This category speeds up Power

Self Test	On Self Test after you power up the computer. 			If you set Enabled, BIOS will shorten or skip

some check items during POST.

Swap Floppy	This item allows you to determine whether

Drive 	enable the swap floppy drive or not.

Boot Up Floppy 	During POST, BIOS will determine if the

Seek	floppy disk drive installed is 40 tracks (360K)

or 80 tracks (720K, 1.2M, 1.44M)

Enabled

BIOS searchs for floppy disk drive to determine if it is 40 or 80 tracks

Disabled

BIOS will not search for the type of floppy disk drive by track number

Boot Up 	This allows you to determine the default

NumLock Status	state of the numeric keypad. By default,

the system boots up with NumLock on.

Boot Up System 	Selects the default system speed - the

Speed	normal operating speed at power up.

Typematic Rate	This determines if the typematic ra

| te is to be used. Setting When disabled, continually holding down a key on your keyboard will generate only one instance. Typematic Rate When the typematic rate is enabled, this (Chars/Sec) section allows you select the rate at which the keys are accelerated. 6 6 characters per second |  |  |
| --- | --- | --- |
| 8 8 characters per second 10 10 characters per second 12 12 characters per second 15 |  |  |

15 characters per second

20

20 characters per second

24

24 characters per second

30

30 characters per second

Typematic Delay 	When the typematic rate is enabled, this

(Msec)	section allows you select the delay between

when the key was first depressed and

| when the acceleration begins. 250 250 msec 500 500 msec 750 |  |  |
| --- | --- | --- |
| 750 msec 1000 1000 msec Security Option This category allows you to limit access to the system and Setup, or just to Setup System The system will not boot and access to Setup will be denIed if the correct password is not entered at the prompt Setup The system will boot, but access to Setup will be denIed if the correct password is not entered at the prompt |  |  |
| To disable security, select PASSWORD SETTING at Main Menu and then you will be asked to ent |  |  |

er password. Do not type anything and

just press <Enter>, it will disable security. Once

the security is disabled, the system will boot 				and you can enter Setup freely.

PCI/VGA 	It determines whether the MPEG ISA/VESA

Palette Snoop	VGA cards can work with PCI/VGA or not.

Enabled

When PCI/VGA working with MPEG ISA/VESA VGA Card

Disabled

When PCI/VGA not working with MPEG ISA/VESA VGA Card

OS Select for 	This item allows you to access the memory

DRAM > 64MB	that over 64MB in OS/2

Video BIOS	Determines whether video BIOS will be

Shadow	copied to RAM. However it is optional

depending on chipset design. Video

Shadow will increase the video speed.

C8000 - CBFFF 	These categories determine whether

Shadow	option ROMs will be copied to RAM. An

DC000 - DFFFF	example of such option ROM would be

| Shadow support of onboard SCSI. This screen controls the setting for the chipset on the mainboard. ROM PCI/ISA BIOS (2A59IA2A) CMOS SETUP UTILITY AWARD SOFTWARE, INC. STANDARD CMOS SETUP INTEGRATED PERIPHERALS BIOS FEATURES SETUP SUPERVISOR PASSWORD CHIPSET FEATURES SETUP USER PASSWORD POWER MANAGEMENT SETUP IDE HDD AUTO DETECTION PNP/PCI CONFIGURATION SAVE & EXIT SETUP LOAD BIOS DEFAULTS EXIT WITHOUT SAVING LOAD SETUP DEFAULTS ESC: Quit áâàß:Select Item F10: Save & Exit Setup (Shift) F2 : Change Color AT Clock, DRAM Timmings ..... ROM PCI/ISA BIOS (2A59IA2A) CHIPSET FEATURES SETUP AWARD SOFTWARE, INC. Auto Configuration : Enabled DRAM Timing : 70ns DRAM Leadoff Timing : 10/6/4 DRAM Read Burst (EDO/FP) : x333/X444 DRAM Write Burst Timing : x333 Fast EDO Lead Off : Disabled Refresh RAS# Assertion : 5 Clks Fast RAS to CAS Delay : 3 DRAM Page Id |  |  |
| --- | --- | --- |

le Timer

: 2 Clks

DRAM Enhanced Paging

: Enabled

Fast MA to RAS# Delay

: 2 Clks

SDRAM (CAS Lat/RAS-to-CAS)

: 3/3

SDRAM Speculative Read

: Disabled

System BIOS Cacheable

: Disabled

Video BIOS Cacheable

: Disabled

Esc: Quit

:Select Item

8-bit I/O Recovery Time

: NA

F1  : Help

PU/PD/+/-:Modify

16-bit I/O Recovery Time

: NA

F5  : Old Values

(Shift)F2  :Color

Memory Hole At 15M-16M

: Disabled

F6  :Load BIOS Defaults

PCI 2.1conpliance

: Disabled

F7  :Load Setup Defaults

Auto	Pre-defined values for DRAM, cache..timing

Configuration	according to CPU type & system clock.

When this item is enabled, the pre-defined

items will become SHOW-ONLY.

DRAM Timing	The DRAM speed is controlled by the DRAM

timing Registers. The timings programmed

into this register are dependent on the 				system design.

DRAM RAS#	DRAM must continually be refreshed or it will

Precharge Time	lose its data. Normally, DRAM is refreshed

entirely as the result of a single request. This

option allows you to determine the number

of CPU clocks allocated for the Row

Address Strobe to accumulate its charge

before the DRAM is refreshed. If insufficient

time is allowed, refresh may be incomplete 			and data lost.

Fast RAS# to	When DRAM is refreshed, both rows and

CAS# Delay	columns are addressed separately. This

setup item allows you to determine the

timing of the transition from RAS to Column 			Address Strobe (CAS).

DRAM Read	This sets the timing for burst mode read (or

Burst (EDO/FP) 	writes)from DRAM. Burst read and write

DRAM Write		requests are generated by the CPU in four

Burst Timing		separate parts. The first part provides the

location within the DRAM where the read or

write is to take pl

| ac |  |  |
| --- | --- | --- |

e while the remaining

three parts provide the actual data. The

lower the timin

| g |  |  |
| --- | --- | --- |

numbers, the faster the 				system will address memory.

ISA Bus

| Cl |  |  |
| --- | --- | --- |

ock	This item allows you to select the PCI clock

type. Choices are PCI CLK/3; PCI CLK/4

System BIOS	When enabled, accesses to the system BIOS

Cacheable 	ROM addressed at

| F |  |  |
| --- | --- | --- |

0000H-FFFFFH are

cached, provided that the cache controller 			is enabled.

Video BIOS 	As with changing the system

| BI |  |  |
| --- | --- | --- |

OS above,

Cacheable 	enabling the Video BIOS cache will cause

access to video BIOS addressed at C0000H

to C7FFFH to be cached, if the cache

controller is also enabled.

8-bit I/O 	The recovery time is the length of time,

Recovery Time 	measured in CP

| U |  |  |
| --- | --- | --- |

clocks, which the system will

delay after the completion of an I/O request.

This item allows y

| ou |  |  |
| --- | --- | --- |

to determine the

recovery time allowed for 8-bit I/O. Choices

ar

e from NA, 1 to 8 CPU clocks.

16-bit I/O	This item allows you to determine the

Recovery Time 	recovery time allowed for 16-bit I/O.

Cho

| ices are from NA, 1 to 4 CPU clocks. Memory Hole At In order to improve performance, certain 15M-16M space in memory can be reserved for ISA cards. This memory must be mapped into the memory below 16MB. This screen controls the 'green' features of this mainboard. *** Refer to Page 4; * are only enabled under ATX power environment ROM PCI/ISA BIOS (2A59IA2A) CMOS SETUP UTILITY AWARD SOFTWARE, INC. STANDARD CMOS SETUP INTEGRATED PERIPHERALS BIOS FEATURES SETUP SUPERVISOR PASSWORD CHIPSET FEATURES SETUP USER PASSWORD POWER MANAGEMENT SETUP IDE HDD AUTO DETECTION |  | PNP/PCI CONFIGURATION SAVE & EXIT SETUP LOAD BIOS DEFAULTS EXIT WITHOUT SAVING LOAD SETUP DEFAULTS ESC: Quit áâàß:Select Item F10: Save & Exit Setup (Shift) F2 : Change Color Sleep Timer, Suspend Timer, ..... ROM PCI/ISA BIOS (2A59IA2A) POWER MANAGEMENT SETUP AWARD SOFTWARE, INC. Power Management : Disabled *Reload Global Timer Events* |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PM Control by APM : Yes IRQ {3-7, 9-15}, NMI : Disabled Video Off Method : V/H SYNC Primary IDE 0 : Disabled +Blank Primary IDE 1 : Disabled Video Off After : Standby Secondary IDE 0 : Disabled Doze Mode : Disabled Secondary IDE 1 : Disabled Standby Mode : Disabled Floppy Disk : Disabled Suspend Mode : Disabled Serial Port : Enabled HDD Power Down : Disabled |  | Parallel Port : Disabled Throttle Duty Cycle : 62.5% ZZ Active in Suspend : Disabled VGA Active Monitor : Enabled Soft-Off by PWR-BTTN* : Instant-off*** CPU FAN Off In Suspend : Enabled |  |  |  |
| Resume by Alarm* : Disabled Esc: Quit áâàß :Select Item Resume by Ring* : Disabled |  | F1 : Help PU/PD/+/- : Modify F5 : Old values (Shift) F2: Color *Break Events From Suspend* F6 : L |  |  |  |

oad BIOS Defaults

IRQ 8 Clock Event

: Disabled

F7  : Load Setup Defaults

Power 	This category allows you to select the type

Management	(or degree) of power saving and is directly

related to the following modes : Doze;

Standby; Suspend; HDD Power Down.

Disabled

No power management. Disables all 4 modes

Min. Power

Saving

Minimum power management. Doze =1hr.; Standby=1hr.; Suspend=1hr.; HDD Power Down=15min

Max.

Power

Saving

Maximum power management only available for SL CPU‘s.Doze=1min.; Standby=1min.;Suspend=1min.;HDD Power Down=1min

User

Defined

Allows you to set each mode indivi-dually. When not disabled, each of the ranges are from 1min. to 1hr. except for HDD Power Down which ranges from 1 to 15min. and disable

PM Control 	When enabled, an Advanced Power

by APM	Management device will be activated to

enhance the Max. Power Saving Mode and

stop the CPU internal clock. If the Max. Power

Saving is not enabled, this will be present to NO.

Video Off This determines the manner in which the Method monitor is blanked. V/H SYNC + Blank This selection will cause the system to turn off the vertical and horizontal sync. ports and write blanks to the video buffer Blank

Sc

| reen This option only writes blanks to the video buffer DPMS Initial display power management signaling The Following 4 modes are Green PC power saving function which are only user configuration when ‘User Defined’ power management has been selected. Doze Mode When enabled and after the set time of system inactivity, the CPU clock will run at slower speed while all other devices still operate at full speed Standby Mode When enabled and after the set time of system inactivity, the fixed disk drive and the video would be shut off while all other devices still |  |  |
| --- | --- | --- |

operate at full speed

Suspend Mode	When enabled and after the set time of

system inactivity, all devices expect the CPU

will be shut off

HDD Power Down	When enabled and after the set time of system

inactivity, the hard disk drive will be powered

down while all other devices remain active

Wake Up Events	In Doze & Standby

Power Down &	These are I/O events whose occurrence can

Resume Events	prevent the system from entering a power

saving mode or can awaken the system from

IRQ3 ~ IRQ15	such a mode. In effect, the system remains

alert for anything which occurs to a device

which is configured as On, even when the

system is in a power down mode. When an

I/O device wants to gain the attention of the

operating system, it signals this by causing an

IRQ (Interrupt ReQuests) to occur. When the

operating system is ready to respond to the

request, it interrupts itself and performs the

service. When set off, activity will neither

prevent the system from going into a power 				management mode nor awaken it.

This screen configures the PCI Bus slots.

ROM PCI/ISA BIOS (2A59IA2A)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRA

| TED PERIPHERALS BIOS FEATURES SETUP SUPERVISOR PASSWORD CHIPSET FEATURES SETUP USER PASSWORD POWER MANAGEMENT SETUP IDE HDD AUTO DETECTION PNP/PCI CONFIGURATION SAVE & EXIT SETUP | LOAD BIOS DEFAULTS EXIT WITHOUT SAVING LOAD SETUP DEFAULTS ESC: Quit áâàß:Select Item F10: Save & Exit Setup (Shift) F2 : Change Color IRQ |  |
| --- | --- | --- |

Settings, Latency Timers, .....

ROM PCI/ISA BIOS (2A59IA2A)

PNP/PCI CONFIGURATION

AWARD SOFTWARE, INC.

PNP OS Installed

: No

PCI IDE IRQ Map to

: PCI-AUTO

Resources Controlled by

: AutoPrimary IDE INT#

: A

Reset Configuration Data

: Disabled

Secondary IDE INT#

: B

Esc: Quit   áâàß

:Select Item

F1 : Help

PU/PD/+/- : Modify

F5 : Old Values

(Shift) F2: Color

F6  : Load BIOS Defaults

F7  : Load Setup Defaults

Resource	The Award Plug and Play BIOS has the

Controlled by	capacity to automatically configure all of the

boot and Plug and Play compatible devices.

However, this capability means absolutely

nothing unless you are using a Plug and Play

OS such as Windows 95 Choices are Auto 				and Manual

Reset Config-	This item allows you to determine reset the

uration Data	configuration data or not.

PCI IRQ	This sets the method by which the

| PCI bus Actived By recognizes that an IRQ service is being requested by a device. Under all circumstances, you should retain the default configuration unless advised otherwise by your system‘s manufacturer. Choices are Level and Edge PCI IDE IRQ This allows you to configure your system to the Map To type of IDE disk controller in use. If you have equipped your system with a PCI controller, changing this allows you to specify which slot has the con |  |  |
| --- | --- | --- |

troller and which PCI interrupt

(A,B,C,D) is associated with the connected

hard disk. Select ‘PCI Auto’ allows the system

to automatically determine how your IDE disk 				system is configured.

This section page includes all the items of IDE hard drive and

Programmed Input/Output features. See also Section “Chipset Features Setup”.

ROM PCI/ISA BIO

| S (2A59IA2A) CMOS SETUP UTILITY AWARD SOFTWARE, INC. STANDARD CMOS SETUP INTEGRATED PERIPHERALS BIOS FEATURES SETUP SUPERV |  |  |
| --- | --- | --- |

ISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC:

| Quit áâàß:Select Item F10: Save & Exit Setup (Shift) F2 : Change Color Time, Date, Hard Disk Type ..... ROM PCI/ISA BIOS (2A59IA2A) INTEGRATED PERIPHERALS AWARD SOFTWARE, INC. IDE HDD Block Mode : Disabled Parallel Port Mode ECP IDE Primary Master PIO : Auto ECP Mode Use DMA : 3 IDE Primary Slave PIO : Auto IDE Secondary Master PIO : Auto IDE Secondary S |  |  |
| --- | --- | --- |

lave PIO

: Auto

IDE Primary Master UDMA

: Disabled

IDE Primary Slave UDMA

: Disabled

IDE Secondary Master UDMA

: Disabled

IDE Secondary Slave UDMA

: Disabled

On-Chip Primary PCI IDE

: Enabled

On-Chip Secondary PCI IDE

: Enabled

USB Keyboard Support: Disabled

Onboard FDC Controller

: Enabled

Esc: Quit    áâàß

:Select Item

Onboard Serial Port 1

: Auto

F1 : Help

PU/PD/+/- : Modify

Onboard Serial Port 2

: AutoF5 : Old Values

(Shift) F2: Color

UART 2 Mode

: Standard

F6  : Load BIOS Defaults

Onboard Parallel Port

: 378H/IRQ7

F7  : Load Setup Defaults

IDE HDD Block	This allows your HD controller to use the fast

Mode	block mode t

| o transfer data to and from your HD drive Enabled IDE controller uses block mode Disabled IDE controller uses standard mode IDE Primary PIO - Programmed Input/Output, it allows the Master/Slave PIO BIOS to tell the controller what it w |  |  |
| --- | --- | --- |

ants and

IDE Secondary	then let the controller and the CPU perform the

Master/Slave PIO	complete task by themselves. This simpler and

more faster. Your system supports five mods, 0 -

4, which primarily differ in timing. When Auto is

selected,the BIOS will select the best available 			mode.

On-Chip Primary	This setup item allows you either to enable or

PCI IDE 	disable the primary/secondary controller. You

On-Chip Second- 	might choose to disable the controller if you

ary PCI IDE 	were to add a higher performance or 						specialized controller.

PCI Slot IDE 	This item allows you designate an IDE controller

2nd Channel 	controller board insert into one of the physical

PCI slots as your secondary IDE

Enabled

External IDE controller designated as the secondary controller

Disabled

No IDE controller occupying a PCI slot

ROM PCI/ISA BIOS (2A59IA2A)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUPINTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

Time, Date,

| Ha |  |  |
| --- | --- | --- |

rd Disk Type .....

ROM PCI/ISA BIOS (2A59IA2A)

CMOS SETUP UTILITY

AWAR

| D |  |  |
| --- | --- | --- |

SOFTWARE, INC.

Hard Disks  Type  Size  CYLS  HEAD  PRECOMP  LANDZ  SECTOR  MODE

Primary

| M |  |  |
| --- | --- | --- |

ster :

Select Primary Master  Option (N=Skip) : N

Options  Size  CYLS  Hea

| d |  |  |
| --- | --- | --- |

PRECOMP  LANDZ  Sector  Mode

2(Y)  1337   648  64       0     2594   63

LBA

1     1339  2595  16  65535

| 2 |  |  |
| --- | --- | --- |

594   63 NORMAL

3     1338  1297  32  65535     2594   63  LARGE

Note : Some Oses (like SCO-UNIX) must use “Normal” for installation

ESC : Skip

The last step is 'save and exit'. If you select this item and press

'Y', then these  records will b

| e |  |  |
| --- | --- | --- |

saved in the CMOS memory on

the mainboard. It will be checked every time when you turn your

comp

| ut |  |  |
| --- | --- | --- |

er on.

ROM PCI/ISA BIOS (2A59IA2A)

CMOS SETUP UTILITY

AWARD S

OF

TWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANA

| GEMENT SETUP IDE HDD AUTO DETECTION PNP/PCI CONFIGURATION SAVE & EXIT SETUP LOAD BIOS DEFAULTS EXIT WITHOUT SAVING LOAD SETUP DEFAULTS ESC: Quit áâàß:Select Item F10: Save & Exit Setup (Shift) F2 : Change Color Time, Date, Hard Disk Type ..... STANDARD CMOS SETUP INTEGRATED PERIPHERALS BIOS FEATURES SETUP SUPERVISOR PASSWORD CHIPSET FEATURES SETUP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| USER PASSWORD POWER MANAGEMENT SETUP IDE HDD AUTO DETECTION PNP/PCI CONFIGURATION SAVE & EXIT SETUP LOAD BIOS DEFAULTS EXIT WITHOUT SAVING LOAD SETUP DEFAULTS SAVE to CMOS and EXIT (Y/N):Y ESC: Quit áâàß:Select Item F10: Save & Exit Setup (Shift) F2 : Change Color Auto-Configure HDD: Sector, Cylinder, Head ..... LOAD BIOS DEFAULTS When your mainboard has problems and need to debug or troubleshoot the system, you can use this function. The default values loaded only affect the BIOS Features Setup, Chipset Features Setup, Power Management Setup and PNP/PCI Configuration Setup. There is no effect on the Standard CMOS Setup.To use this function,select it from main menu and press <Enter>. A line will appear on the screen asking if you want to load the BIOS default values. Press <Yes> and <Enter> then the BIOS default values will be loaded. LOAD SETUP DEFAULTS This allows you load optimal settings which are stored in the BI |  |  |  |  |  |

OS ROM to get better performance. The default

values loaded only affect the BIOS Features Setup,

Chipset Features Setup, Power Management Setup

and PNP/PCI Configuration Setup.There is no effect on

the Standard CMOS Setup. To use this function, select

it from main menu and press <Enter>.A line will appear

on the screen asking if you want to load the Setup

default values. Press <Yes> and <Enter> then the 	Setup default values will be loaded.

SUPERVISOR PASSWORD / USER PASSWORD

This allows you to set the password. the mainboard 	defaults password disabled.

Enter/Change password : Enter the current

password,at the prompt key-in your new password

(up to eight alphanumeric characters), press <Enter>.

At the next prompt, confirm the new password by

typing it and press <Enter> again.

Disable password : Press <Enter> key instead of

entering a new password when the ‘Enter Password’

dialog box appears. A message will appear

confirming that the password is disable.

If you set both supervisor and user passwords, only the

supervisor password allows you to enter the BIOS

SETUP program.

CAUTION :If you forget your password, you must

disable the CMOS by turning power off and set JP8

‘open‘. Then reload the system.

IDD HDD AUTO DETECTION

This allows you to detect IDE hard drives‘

parameters and enter them into ‘Standard CMOS 	Setup’ 	automatically.

If the auto-detected parameters displayed do not

match the ones that should be used for your hard

drive, do not accept them. Press <N> to reject the

values and enter the correct ones manually on

the Standard CMOS Setup screen.

SAVE & EXIT SETUP

This allows you to save the new setting values in

the CMOS memory and continue with the booting

process. Select what you want to do, press <Enter>.

EXIT WITHOUT SAVING

This allows you to exit the BIOS setup utility without

recording any new values or changing old ones.

¡ Control Key Description ¡

UP ARROW

¡

Move to previous item

DOWN ARROW

¡

Move to next item

LEFT ARROW

¡

Move to the item in the left hand

RIGHT ARROW

¡

Move to the item in the right hand

Esc KEY

Esc

Main Menu : Quit and not save changes

Setup menu : Exit current page and return to main menu

PgUp KEY

Increase the numeric value or make changes

PgDn KEY

Decrease the numeric value or make changes

F1 KEY

Help

General help

F2 KEY

¡Shift¡+F2

Change color from total 16 colors

F5 KEY

Old Value

Restore the pervious CMOS value from CMOS

F6 KEY

Load BIOS default

Load the default CMOS value from BIOS default table

F7 KEY

Load setup default

Load Setup default

F10 KEY

Save & Exit Setup

Save all the CMOS changes and Exit setup, only for Main Menu

APPENDIX A

¡¡TECHNICAL SUPPORT REQUEST FORM¡¡

If the mainboard doesn't function properly, please complete the following

information and return it to your system dealer. If the further information

is needed, please attach this separating sheets.

Model No :  ATC-5000      Date of  Purchase :  ______________

Serial No : ___________________

HARDWARE :

BRAND

MODEL

SPEED

Q'TY

CPU

SIM Module

PB SRAM on Board

TAG SRAM

Sync. SRAM Module

SIMM  :_______MB (__ EDO, __FastPage)

Hard Disk Interface Controller : __ __ IDE, ____ SCSI

Hard Disk Brand :___________, Model :_________, Capacity :________

Display Controller Brand :_____________,Model :__________

Controller Chip Brand :_____________,Model :__________

AWARD SYSTEM BIOS: Version _______  Date Code _____________

Keyboard BIOS: Brand __________

Other Add-on Cards Information:

Add-on Card

Bus Interface

Model

Remark

Error Description :