Table of Contents

JUMPER COMPONENT LOCATION DIAGRAM	A

µCHAPTER 1   INTRODUCTION	2

1-1 SYSTEM FEATURES	3

1-2 SOFTWARE POWER OFF CONTROL	4

1-3 CHECK LIST OF THE PACKING	5

CHAPTER 2   INSTALLATION	6

2-1 INSTALLATION PROCEDURES 	6

2-2 CPU INSTALLATION	7

2-3 SYSTEM MEMORY INSTALLATION	15

2-4 SRAM INSTALLATION	17

2-5 OTHER JUMPERS AND CONNECTORS 
       DESCRIPTION 	17

2-6 IDE DRIVER INSTALLATION	22

CHAPTER 3   AWARD BIOS SETUP	23

3-1 UPDATE BIOS PROCEDURES	24

3-2 AWARD SYSTEM BIOS CONFIGURATION SETUP	25

APPENDIX A 	49

CHAPTER 1 INTRODUCTION

This manual describes how to configure the ATC-5020 mainboard for different environment. It's an overview of the layout and features of the mainboard, and also provides information for changing the configuration or system environment.

This manual is divided into two sections :

PART ONE  includes page A and two chapters as following:

Page A contains layout diagram of the mainboard.

Please refer to it when you configure the system.

Chapter 1    is an overview of the mainboard features and

packing contents.

Chapter 2    describes how to upgrade and to change

hardware configurations such as memory size, CPU

type, and lists of jumper settings and connectors.

PART TWO  includes chapter 3 which contains Award BIOS

description.

Chapter 3    is the user's guide of Award BIOS setup utility,

Flash ROM BIOS update procedure. The figure

described in this chapter are default settings.

Your system dealer will set up the mainboard according to your demand of computer. It means that the current settings of your mainboard may not be the same as the defaults shown in this user's manual.  If you need to change your configuration, please ask your dealer first, be sure it doesn‘t void your system warranty. Or ask for your dealer to do it for you.

REMARK

Intel®  is a registered trademark of Intel Corporation. All other brands and product names are trademarks registered trademarks of their respective companies.

1-1 SYSTEM FEATURES

- Intel Pentium( (P54C) CPU operating at 90MHz to 233MHz      Intel Pentium MMXTM 166~233MHz, AMD-K5, AMD-K6,      Cyrix 6x86L, M2.

- INTEL 82430TX PCIset.

- Using four 72-pin SIMM sockets, provides two banks of 64-bit wide path up to 256MB addressing page mode DRAMs.

- Using two 168-pin DIMM sockets for 3.3V SDRAM or EDO module.

- Supporting two types of DRAM including EDO (Extended Data Out), or FPM (Fast Page Mode).

- Built-in 256KB or 512KB PB SRAM on board

- Support four PCI bus master revision 2.1, 5V interface compliant slots and four 16-bit ISA slots.

- Dual Master IDE connectors support Ultra UMA/33, up to four devices in two channels for connection of high capacity hard disk drive, CD-ROM, tape backup etc..

- PS/2 keyboard connector and PS/2 mouse cable connector.

- Support Dual channel USB connection.

- SMC Ultra Multi-I/O chipset:

- Support Infrared transfer (IrDA TX/RX) connection.

- One FDC port supports two devices up to 2.88MB

- Two 16550A fast UARTs compatiable serial ports

- One EPP/ECP mode parallel port

- Built-in Switching Voltage Regulator

- Supports ACPI Power Management - OSPM (OS directed power management)

- Form Factor is 30.5mm x 21.0mm (9.6" x 8.26") with four layers designed.

1-2 SOFT-OFF CONTROL

The mainboard design supports Soft-Off Control feature through the SMM code in the BIOS under Windows 95, Windows 3.1x, and MS-DOS operation system environment. It needs to use ATX power supply. (please contact your supplier for this option)

Firstly, you should connect power switch cable (provided by ATX case supplier) to the connector JP7 (next to system BIOS socket) on the mainboard.

In Windows 95, if you would like to soft power off the system, you just choose “shutdown the computer ?” in “Shut Down Windows“ from Windows 95, then push the Power Switch* (on ATX case), then system will be in soft off status directly. If you would like to restart the system, just press the button of the power switch, and the system will be power on.

* Default value (Power management of BIOS screen) is “Instant-off‘, you can change to ’Delay 4 Sec.‘, then you should push the Power Switch at least 4 seconds otherwise the system will be under SMI mode only.

In Windows 3.1x or MS-DOS, you should copy the program of “down.com” (you can find it in the diskette of IDE driver) into hard drive. When you would like to power off the system, just run this “down.com”, then the system will be shutdown and stay in standby status.

Note : If you will leave your system several days, we suggest you use hardware mechanical power off to shutdown your system.

1-3 CHECK LIST OF THE PACKING

The mainboard comes securely packed in a durable box and shipping carton. If any of the above items are missing or damaged, please contact your supplier.

Each mainboard containing:

Q'TY     	Description

1       	Mainboard	: ATC-5020.

1	Diskette	: Enhanced IDE driver (3.5").

1         Cable 	: Enhanced IDE connector.

1         	Cable 	: F.D.D. connector.

1         	Manual 	: User`s manual.

NOTE : Leave the mainboard in its original packing until you are ready to install it.

CHAPTER 2 INSTALLATION

2-1 INSTALLATION PROCEDURE

Before installing the computer, please prepare all components such as CPU, DRAM; peripherals such as hard disk devices, keyboard, CD-ROM drive; and accessories such as cables. Then, install the system as following :

1. Plug CPU, heat sink, cooling fan, and DRAM modules in

the mainboard.

2. Set jumpers based on your configuration.

3. Plug add-on cards in PCI/ISA slots.

4. Connect cables to peripherals, power supply..

5. Make sure all components and devices are well

connected. Turn on the power and setup System BIOS

according to your system.

6. Install peripheral, add-on card drivers and test them.

7. If all of above procedures are success, turn-off the power

then plug all of them into your computer case.

2-2 CPU  INSTALLATION

ATC-5020 supports Pentium level CPU up to 233MHz. For installation, please notice CPU pin 1 must align with the ZIF socket 7 Pin 1 location. Before you install or upgrade your CPU, please read CPU guide from CPU manufacturer to make sure the CPU voltage specification. Then choose the right installation in the 2-2-1 based on your CPU type/brand and follow the description to setup jumpers. If your CPU is not in the list of 2-2-1, please refer to 2-2-2 and 2-2-3 for installation.

2-2-1 CPU TYPE SELECTION

A. INTEL PENTIUM CPU (P54C)

¡ P54C VRE : 3.400V ~ 3.600V (The fourth line of the mark on the

under-side of the processor contains a code that identifies

the voltage level type. V is VRE, S is standard. )

JP2

1-2

INTERNAL CPU CLOCK

JP6

JP4

Ext.x Frq.

120MHz

2-3

2-3, 4-560x2.0

133MHz

1-2

2-3, 4-5

66x2.0

150MHz

2-3

2-3, 5-6

60x2.5

166MHz

1-2

2-3, 5-6

66x2.5

180MHz

2-3

1-2, 5-6

60x3.0

200MHz

1-2

1-2, 5-6

66x3.0

JP6

1ꅃꅃ4

2ꅃꅃ5

3ꅃꅃ6

JP4

1ꅃꅃꅃ4 7

2ꅃꅃꅃ5 8

3ꅃꅃꅃ6 9

¡ P54C STD : 3.135V ~ 3.600V (The fourth line of the mark on the

under-side of the processor contains a code that identifies

the voltage level type. V is VRE, S is standard. )

JP2

3-4

INTERNAL CPU CLOCK

JP6

JP4

Ext.x Frq.

120MHz

2-3

2-3, 4-5

60x2.0

133MHz

1-2

2-3, 4-5

66x2.0

150MHz

2-3

2-3, 5-6

60x2.5

166MHz

1-2

2-3, 5-6

66x2.5

180MHz

2-3

1-2, 5-6

60x3.0

200MHz

1-2

1-2, 5-6

66x3.0

Intel Pentium CPU, the first letter after ႑/႒ denotes voltage type.

xxxxxxxxx

xxxxxxxxx

xxxxxxxxx

Sxxxx/SMU

JP6

1ꅃꅃ4

2ꅃꅃ5

3ꅃꅃ6

JP4

1ꅃꅃꅃ4 7

2ꅃꅃꅃ5 8

3ꅃꅃꅃ6 9

B. INTEL PENTIUM CPU (P55C)

JP2

9-10

INTERNAL CPU CLOCK

JP6

JP4

Ext.x Frq.

166MHz

1-2

2-3, 5-6

66x2.5

200MHz

1-2

1-2, 5-6

66x3.0

233MHz

1-2

1-2, 4-5

66x3.5

C. Cyrix 6x86 CPU

JP2

1-2

INTERNAL CPU CLOCK

JP6

JP4

Ext.x Frq.

PR150+ @ 120MHz

2-3

2-3, 4-5

60x2.0

PR166+ @ 133MHz

1-2

2-3, 4-5

66x2.0

6x86-Pxxx+GP

xxxMHz

3.52V(028)

The button line of the mark on the processor contains a code 028 that identifies the voltage level type. If the code is 016 or others, or no marking then please contact your dealer.

D. Cyrix 6x86L CPU

JP2

9-10

INTERNAL CPU CLOCK

JP6

JP4

Ext.x Frq.

PR150+ @ 120MHz

2-3

2-3, 4-5

60x2.0

PR166+ @ 133MHz

1-2

2-3, 4-5

66x2.0

6x86L-P150+GP

120MHz

2.8V

The mark on the processor contains as the sample in the left. The code-name 6x86L is dual voltage mode processor,

E. AMD-K5 CPU

JP2

1-2

INTERNAL CPU CLOCK

JP6

JP4

Ext.x Frq.

PR100ABQ

1-21-2, 4-5

66x1.5

PR120ABQ

2-3

1-2, 4-5

60x1.5

PR133ABQ

1-2

1-2, 4-5

66x1.5

PR166ABQ

1-2

2-3, 5-6

66x2.5

AMD-K5(

PR100

AMD-K5-PR100ABQ

100MHz

Product Name

P-rating

Internal CPU Frequency

F. AMD-K6 CPU

INTERNAL CPU CLOCK

JP2

JP6

JP4

Ext.x Frq.

PR166

7-8

1-2

2-3, 5-6

66x2.5

PR200

7-8

1-2

1-2, 5-6

66x3.0

PR233

5-6

1-21-2, 4-5

66x3.5

2-2-2 CPU VOLTAGE SETTING

ATC-5020 can auto detect single or dual CPU voltage power suppply type, JP2 is used for various CPU voltage value types.

¡CPU Core Voltage Type : JP2

INTEL STD(P54C)

INTEL VRE(P54C)

10  9

8  7

6  5

4  3

2  1

10  9

8  7

6  5

4  3

2  1

AMD-K5

INTEL MMXTM (P55C)

10  9

8  7

6  5

4  3

2  1

10  9

8  7

6  5

4  3

2  1

CYRIX 6x86

CYRIX 6x86L

10  9

8  7

6  5

4  3

2  1

10  9

8  7

6  5

4  3

2  1

AMD-K6/PR233

AMD-K6/PR166,200

10  9

8  7

6  5

4  3

2  1

10  9

8  7

6  5

4  3

2  1

2-2-3 CPU CLOCK SETTING

EXTERNAL CPU CLOCK

JP6

60(MHz)

2-3

66(MHz)

1-2

66MHz                   60MHz

JP6  1

2

3

JP6

INTERNAL CPU CLOCK

JP4

INTEL EXTERNAL CLOCK X 1.5

1-2, 4-5

INTEL EXTERNAL CLOCK X 2.0

2-3, 4-5

INTEL EXTERNAL CLOCK X 2.5

2-3, 5-6

INTEL EXTERNAL CLOCK X 3.0

1-2, 5-6

INTEL EXTERNAL CLOCK X 3.5

1-2, 4-5

x 1.5       x 2.0        x 2.5        x 3.0       x 3.5

JP4  1

2

3

4

5

6

2-3 SYSTEM MEMORY INSTALLATION

ATC-5020 provides four 72-pin SIMM sockets for system memory expansion from 8MB to 256MB. These four SIMMs are arranged to two banks, Bank0 (SIM 1, 2) and  Bank1(SIM 3, 4), please refer to page A. Each bank provides 64-bit wide data path.

The mainboard accepts Fast Page Mode DRAM, and EDO Mode (Extended Data Out) DRAM, with a speed no slower than 70 nanosecond. You should plug DRAM modules into two sockets (same bank) or four sockets at one time. Each pair of modules in the same bank must be the same size, type, and speed; no matter single-side or double-side module. Please plug in Bank 0 firstly if you only have 2 modules. The mainboard supports mixing of EDO and fast page mode DRAM among different banks, please plug EDO in Bank 0.

Also this mainboard provides two optional 168-pin DIMM sockets for 3.3V SDRAM or 3.3V EDO DRAM expansion. You should plug SDRAM/DRAM module into each DIMM sockets (as a bank) or two sockets at one time.

CAUTION: It‘s not recommended to install the 3.3V SDRAM and 5V EDO or Fast Paged mode memory within a system. The 72-pin DRAM cannot work with 168-pin DRAM in the same time. Changing EDO/FPM DRAM to SDRAM, you don‘t have to adjust jumper setting or BIOS value, nor change SDRAM to EDO/FPM DRAM. (¡ Please make sure the SDRAM plugged-in fully, to prevent contact loss.)

¡ System Memory Combinations Options ¡

BANK0

BANK1

Total Memory

SIM 1, 2

SIM 3, 4

SIM 1- 4

4MBx2

-

8MB

-

4MBx2

8MB

8MBx2-

16MB

-

8MBx2

16MB

4MBx2

4MBx2

16MB

4MBx2

8MBx2

24MB

8MBx2

4MBx2

24MB

16MBx2

-

32MB

-

16MBx2

32MB

8MBx2

8MBx2

32MB

4MBx2

16MBx2

40MB

16MBx2

4MBx2

40MB

8MBx2

16MBx2

48MB

16MBx2

8MBx2

48MB

32MBx2

-

64MB

-

32MBx2

64MB

16MBx2

16MBx2

64MB

4MBx2

32MBx2

72MB

32MBx2

4MBx2

72MB

8MBx2

32MBx2

80MB

32MBx2

8MBx2

80MB

16MBx2

32MBx2

96MB

32MBx2

16MBx296MB

32MBx2

32MBx2

128MB

64MBx2

-

128MB

-

64MBx2

128MB

- continue -

4MBx2

64MBx2

136MB

64MBx2

4MBx2

136MB

8MBx2

64MBx2

144MB

64MBx2

8MBx2

144MB

16MBx2

64MBx2

160MB

64MBx2

16MBx2

160MB

32MBx2

64MBx2

192MB

64MBx2

32MBx2

192MB

64MBx2

64MBx2

256MB

*128MBx2

-

256MB

-

*128MBx2

256MB

* Please confirm this with your supplier firstly.

2-4 SRAM  INSTALLATION

ATC-5020 is built-in 512KB Sync. Pipeline Burst SRAM on board

2-5 OTHER JUMPERS AND

CONNECTORS  DESCRIPTION

When you plug a cable into the following I/O connectors, you should have the pin 1 edge of the cable aligned with the pin 1 end of the connector.

CONN1 : speaker, power LED, reset, SMI,

turbo LED, and IDE LED connectors.

IDE-LED TB-LED  SMI   RST

POWER LED    SPK

+       +

SPK : speaker

VCC

GND

GND

Speaker

Power LED connector

GND (power)

N/C

Power LED +

RST : Reset connector

GND

Reset Signal

SMI : SMI lead

SMI Signal

GND

TB-LED : Turbo LED indicator, LED on

when system runs higher speed

+5V (next to CPU)

GND

IDE-LED : IDE devices indicator LED

connector. IDE-LED stays ON indicates    +5V  on-board IDE devices in operation. If

GND plug wire into wrong connector, color

of LED will be lighter and the IDE dvices

can still function properly.

FAN1 : CPU cooling fan connector. Wire with

1 GND

2+12V

3GND

+12V voltage (most likely red wire) must

be plugged into pin2, and GROUND wires

(must likely black wires) must be plugged

into pin1and/or pin3. Please confirm the

wire color re-presentation with your supplier.

CAUTION: Plug wire into wrong connector will

DAMAGE fan and mainboard.

1 +5V

2N/C

3IRRX

4GND

5IRTX

IR1 : Infrared module connector.

USB1 : USB connector; Universal Serial Bus;this

(red)

VCC GND

D1 -D0 +

D1+D0 -

GNDVCC

(black)

is used to connect USB devices through

an optional dual head cable with a iron plane.

CAUTION: Plug wire into wrong connector

will DAMAGE USB devices and mainboard.

PS/2 KB1 : 6-pin PS/2 keyboard connector.

PS/2 Mouse: connector, which is used to

4 6

2

1

35

connect an optional cable.

pin1 : data

pin2 : N/C

pin3 : GND

pin4 : VCC

pin5 : clock

pin6 : N/C

PW1 : ATX mode +3.3/5/12V power supply

connector.

13.3V6+5V113.3V16GND23.3V7GND12-12V17GND3GND8PWRGD13GND18-5V4+5V95VSB14*PS_ON19+5V5GND10+12V15GND20+5V* PS_ON : Soft-Off power control

1          10

11           20

COM1/COM2 : these two connectors are

1 2

34

56

78

910

used to connect serial port cables.

pin

signal name

1

DCD

2

Serial In

3

Serial Out

4

DTR

5

GND

6

DSR

7 RTS

COM1/COM2

8

CTS

9

RI

10

N/C

FDC : this connector is used to connect floppy

12

3334

disk drive through cable.

pin

signal

pin

signal

2

RWC-

20

STEP-

4

reserved

22

Write Data

6

FDEDIN

24

Write Gate

8

Index-

26

Track 00-

10

Motor EnableA-

28

Write Protect-

12

Drive Sele.B-

30

Read Data-

14

Drive Sele.A-

32

Side 1 Sele.-

16

Motor EnableB-

34

DisketteChange

8

DIR-

All of odd pins are ground

LPT : this connector is used to connect parallel

114

215

1326

port cable.

pin

signal

pin

signal

1

STROBE-

10

ACK-

2Data Bit 0

11

BUSY

3

Data Bit 1

12

PE

4

Data Bit 2

13

SLCT

5

Data Bit 3

14

Auto Feed-

6

Data Bit 4

15

ERROR-

7

Data Bit 5

16

INIT-

8

Data Bit 6

17

SLCT IN-

9

Data Bit 7

26

N/C

pin18 -- pin25 are ground

IDE1/IDE2 : these two connectors are used to

connect max. 4 devices through IDE cable.

12

34

3940

pin

signal

pin

signal

1

Reset IDE

21

DDRQ0(1)

2

GND

22

GND

3

Host Data 723

I/O Write-

4

Host Data 8

24

GND

5

Host Data 6

25I/O Read-

6

Host Data 9

26

GND

7

Host Data 5

27

IORDY

8

Host Data 10

28

N/C

9

Host Data 4

29

DDAK0-(1-)

LPT

10

Host Data 11

30

GND

11

Host Data 3

31

IRQ14*

12

Host Data 12

32

IOCS16-

13

Host Data 2

33

Addr 1

*

IDE1: pin31 is IRQ14; IDE2: pin31 is IRQ15

or MIRQ0

14

Host Data 13

34N/C

15Host Data 1

35

Addr 0

16

Host Data 14

36

Addr 2

17

Host Data 0

37

ChipSele. 1P-

18

Host Data 15

38

ChipSele 3P-

19

GND

39

Activity

20

Key

40

GND

2-6 IDE DRIVER IN

| STALLATION The IDE driver installation procedure is as following : Setup for DOS/Windows : 1. Starting MS-Windows 3.1 (or 3.11) 2. Select Program Manager, File/RUN, then type "A:\setup.exe" 3. Exit MS-Windows, turn power off; then tu |  |  |
| --- | --- | --- |

rn power on.

Setup for Windows 95 : (Windows 95 version 950/950a/950b)

1. Starting Windows 95

2. Select “START”, “RUN”.

3. Type “A:\WIN95\SETUP.EXE”.

4. Restart computer, then follow the instructions on yo

| ur screen to install new IDE driver we offer in the 3.5“ diskette 5. Exit Windows 95, turn power off; then turn power on. Make sure your HDD follow ATA standard, and your CD-ROM should follow ATAPI standard. When you plug-in the IDE devices, please plug your first and second devices into IDE 1 port (Master then Slave), then plug third and forth devices into IDE 2 port. If you have CD-ROM driver, please set it aafter hard disk devices as the last device. For example, if you have 2 HDDs and 1 CD-ROM, you should set HDD1 and HDD2 in IDE1 Master and Slave, set CD-ROM in IDE 2 Master. Some of the brands devices co |  |  |  |
| --- | --- | --- | --- |

mbination may not work under this sequence, you can try to re-arrange the devices sequence, or contact your vendor.

Primary MasterPrimary SlaveSecondary MasterSecondary SlaveATA no ATAPIATAATAPI disk & CD-ROMATAATAPI use only one cableATAATAPIATAPI CD-ROM and a tape or two CD-ROMs

CHAPTER 3  AWARD BIOS

SETUP

Thi

| s chapter explains the system BIOS setup, and how to update new BIOS. All BIOS screens showed in the following pages are default values, your system dealer will set up these values according to your demand of computer. ATC-5020 uses Flash ROM to make BIOS easier to be updated by the floppy disk-based program and to committe Microsoft Windows 95 plug & play feature. The update CMOS process is mechanical power off (or unplug the power cord) to ensure there is no power to the mainboard, firstly. Set JP8 to ‘open’, then the CM |  |  |  |
| --- | --- | --- | --- |

OS is cleared. Reset is set JP8 to ‘close’, power on the power switch (or plug the power cord).

JP8

Nomal  Clear

close

NORMAL¡

open

CLEAR CMOS

¡is default setting

3-1 UPDATE BIOS

|  | PROCEDURES If the BIOS needs to be updated, it can be obtained on a diskette from your system supplier. The BIOS diskette includes: “awdflash.exe” -- BIOS update utility program “awdflash.doc” “(update BIOS filename with version number).rom” The update procedures are as following: Boot the system to DOS mode in a normal manner. Insert the updated diskette to drive A (or B). - Change working directory to floppy drive, A or B, which contains the update BIOS diskette. -- Type “a:\” or “b:\”, “ENTER”. - Run the BIOS update utility -- Type “awdflash”, “ENTER”. - Type “(update BIOS file name with version number).rom”, ENTER. - Type “N” when the screen displays the message : " Do you want to save BIOS (Y/N) ?". - Type “Y“ when the screen shows the message : " Are you sure to program (Y/N) ?". - Follow instructions displayed on the screen. DO NOT remove the update BIOS diskette from the floppy drive nor turn the system power off until the BIOS update is completed. - Turn the system power on and test your system working properly or not. 3-2 AWARD SYSTEM BIOS CONFIGURATION SETUP The following pages explain how to set up the system configuration (CMOS) under the AWARD BIOS. The SETUP program is stored in the Read-Only-Memory (ROM) on the mainboard. Enter the SETUP procedure, press the <Del> key when the system is booting up. The following main menu will appear. Please select " STANDARD CMOS SETUP" to enter the next screen. ROM PCI/ISA BIOS (2A59IA2A) CMOS SETUP UTILITY AWARD SOFTWARE, INC. STANDARD CMOS SETUP INTEGRATED PERIPHERALS BIOS FEATURES SETUP SUPERVISOR PASSWORD CHIPSET FEATURES SETUP USER PASSWORD POWER MANAGEMENT SETUP IDE HDD AUTO DETECTION PNP/PCI CONFIGURATION SAVE & EXIT SETUP LOAD BIOS DEFAULTS EXIT WITHOUT SAVING LOAD SETUP DEFAULTS ESC: Quit áâàß:Select Item F10: Save & Exit Setup (Shift) F2 : Change Color Time, Date, Hard Disk Type ..... The section on the bottom of the main menu explains how to control this |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

screen.  The other one section displays the items highlighted in the list.

This screen records some basic hardware information, and set the system cl

| oc |  |  |
| --- | --- | --- |

k and error handling. These records can be lost or corrupted if the on-board battery is fail

| ed |  |  |
| --- | --- | --- |

or weak.

ROM PCI/ISA BIOS (2A59IA2A)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STAN

| DA |  |  |
| --- | --- | --- |

RD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWE

| R |  |  |
| --- | --- | --- |

MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

ROM PCI/ISA BIOS (2A59IA2A)

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

D

ate (mm:dd:yy) : Fri, Feb  14 1997

Time(hh:mm:ss)   : 13 : 7 : 14

HARD DISKS  TYPE   SIZE  CYLS  HEAD  PRECOMP LANDZ SECTOR MODE

Primary Master  : Auto

0

0

0

0      0

0

Auto

Primary Slave   : Auto

0

0

0

0      0

0

Auto

Secondary Master : Auto

0

0

0

0      0

0

Auto

Secondary Slave  : Auto

0

0

0

0      00Auto

Drive A : 1.44M,  3.5  in.

Base Memory           :        640K

Drive B : None

Extended memory       :      64512K

Floppy 3 Mode support : Disabled

Other Memory            :        384K

------------------------------------------------

Video  : EGA/VGA

Total Memory             :      65536K

Halt On: All Errors

ESC  :  Quit                 áâàß:Select Item       PU/PD/+/- : Modify

F1   :  Help                (Shift) F2 : Change Color

Date	mm is month

| , |  |  |
| --- | --- | --- |

dd is date, yy is year.

date

from 1 to 31

month

from Jan. to Dec.

year

fro

| m |  |  |
| --- | --- | --- |

1900 to 2099

Time	hh is hour, mm is minute, ss is second.

hh

from 0 t

| o |  |  |
| --- | --- | --- |

23 (24-hour military -time)

mm

from 1 to 59

ss

from 1 to 59

Primary Master	The cate

| go |  |  |
| --- | --- | --- |

ries identify the types of 2

Primary Slave	channels that have been installed in the

| S |  |  |
| --- | --- | --- |

econdary Master	computer. There are 45 predefined types

Secondary Slave	and 4

| u |  |  |
| --- | --- | --- |

ser definable types are for Enhanced

IDE

| B |  |  |
| --- | --- | --- |

IOS. Type 1 to 45 are predefined. Type

‘user’is user- definable. Press PgUp/PgDn

to select a numbered hard disk type or type

the number and press<Enter>. If you select

‘Auto’ BIOS will auto-detect the HDD & CD-

ROM Drive at the POST stage and showing

the IDE for HDD & CD-ROM Drive. If you select

‘user’, you will need to know the device

information listed below. Enter the information

directly  from the keyboard and press <Enter>. 		                  This information should be from your hard disk 			vender or dealer.

| If the controller of HDD interface is ESDI, the selection shall be ‘Type 1’; is SCSI, the selection shall be ‘None’. If the device has not been installed select‘NONE’ and press <Enter>. Type drive type SIZE automatically adjusts CYLS number of cylinders HEAD number of heads PRECOMP write precom LANDZ landing zone SECTOR number of sectors MODE mode type Drive A The category identifies the types of floppy Drive B disk drive A or drive B that have been installed in the computer. None No floppy drive installed 360K, 5.25 in 5.25“ PC-type 360KB capacity 1.2M, 5.25 in 5.25“ AT-type 1.2MB capacity 720K, 3.5 in 3.5“ double-side 720KB capacity 1.44M, 3.5 in 3.5“ double-side 1.44MB capacity 2.88M, 3.5 in 3.5“ double-side 2.88MB capacity Floppy 3 mode This is the Japanese standard floppy device. support The standard stores 1.2MB in a 3.5“ diskette Video The category selects the type of video adapter used for the primary system monitor. Although secondary monitors are supported, you do not have to select the type in Setup. Halt On The category determines whether the computer will stop if an error is detected during power up. No errors When the BIOS detects a non-fatal error the system will be stopped and you will be prompted All errors The system boot will not be stopped for any error that may be detected All, But Keyboard The system boot will not stop for a keyboard error, it will stop for all other errors All, But Diskette The system boot will not stop for a disk error, it will stop for all other errors All, But Disk/Key The system boot will not stop for a disk or keyboard error, it will stop for all other errors Memory The category is display-only which is deter- mined by POST (Power On Self Test) of the BIOS. Base Memory The value of the base memory is typically 512K or 640K based on the memory installed on the mainboard. Extended Memory How much extended memory is present during the POST. This is the amount of memory located above 1MB in the CPU‘s memory address map. Other Memory This refers to the memory located in the 640K to 1024K address space. The BIOS is the most frequent user of this RAM area since this is where it shadows RAM. This screen is a list of system configuration options. Some of them are defaults required by the mainboard's design, others depend on the features of your system. ROM PCI/ISA BIOS (2A59IA2A) CMOS SETUP UTILITY AWARD SOFTWARE, INC. STANDARD CMOS SETUP INTEGRATED PERIPHE |  |
| --- | --- |

RALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

Virus, Protection, Boot Sequence

ROM PCI/ISA BIOS (2A59IA2A)

BIOS FEATURES SETUP

AWARD SOFTWARE, INC.

Virus Warning

: Disabled

Video BIOS Shadow

: Enabled

CPU Internal Cache

: Enabled

C8000-CBFFF Shadow

: Disabled

External Cache

: Enabled

CC000-CFFFF Shadow

: Disabled

Quick Power On Self Test

: Disabled

D0000-D3FFF Shadow

: Disabled

Boot Sequence

: A, C, SCSI

D4000-D7FFF Shadow

: Disabled

Swap Floppy Drive

: Disabled

D8000-DBFFF Shadow

: Disabled

Boot Up Floppy Seek

: Enabled

DC000-DFFFF Shadow

: Disabled

Boot Up NumLock Status

: On

Boot Up System Speed

: High

Typematic Rate Setting

: Disabled

Typematic Rate(Chars/Sec)

: 6

Esc : Quit   áâàß:Select Item

Typematic Delay(Msec)

: 250

F1   : Help    PU/PD/+/- : Modify

Security Option

: Setup

F5  : Old Values  (SHIFT)F2 : Color

PCI/VGA Palette Snoop

: Disabled

F6   : Load BIOS Defaults

OS/2 select for DRAM>64MB

: Non-OS2

F7   : Load Setup Defaults

Report No FDD for WIN 95

: NO

Virus Warning	When this item is enabled, the BIOS will

monitor the boot sector and partition table

of the hard disk drive for any attempt at

modification. If an attempt is made, the BIOS

will halt the system and the following error

message will appear. Man

| y |  |  |
| --- | --- | --- |

disk diagnostic

programs which attempt to access the boot

sector table can cau

| se |  |  |
| --- | --- | --- |

the above warning

message. If you will be running such a

program, we

| re |  |  |
| --- | --- | --- |

commend that you first 					disable Virus Protection beforehand.

! WARNING !

Disk boot sec

| to |  |  |
| --- | --- | --- |

r is to be modified

Type ‘Y’ to accept write or ‘N’ to abort write

Award Software, I

| nc |  |  |
| --- | --- | --- |

.

Enabled

Activates automatically when the system boots up causing a warni

| ng |  |  |
| --- | --- | --- |

message to appear when anything attempts

| to |  |  |
| --- | --- | --- |

access the boot sector or hard disk partition table.

Disabled

No warning message will appear when anything attempts to access the boot sector or hard disk partition table.

CPU	These two categories speed up memory

Internal Cache	access.However, it depend

| s |  |  |
| --- | --- | --- |

on CPU/

External Cache	chipset design.The default value

Quick Power On	is‘enabled‘.This categ

| or |  |  |
| --- | --- | --- |

y speeds up Power

Self Test	On Self Test after you power up the com

pu

ter. 			If you set Enabled, BIOS will shorten or skip

some check items during POST.

Swap Floppy	This item allows you to determine whether

| Drive enable the swap floppy drive or not. Boot Up Floppy During POST, BIOS will determine if the Seek floppy disk drive installed is 40 tracks (360K) or 80 tracks (720K, 1.2M, 1.44M) Enabled BIOS searchs for floppy disk drive to determine if it is 40 or 80 tracks Disabled BIOS will not search for the type of floppy disk drive by track number Boot Up This allows you to determine the default NumLock Status state of the numeric keypad. By default, the system boots up with NumLock on. Boot Up System Selects the default system speed - the Speed normal operating speed at power up. Typematic Rate This determines if the typematic rate is to be used. Setting When disabled, continually holding down a key on your keyboard will generate only one instance. Typematic Rate When the typematic rate is enabled, this (Chars/Sec) section allows you select the rate at which the keys are accelerated. 6 6 characters per second |  | 8 8 characters per second 10 10 characters per second 12 | 12 characters per second |  |  |
| --- | --- | --- | --- | --- | --- |
| 15 15 characters per second 20 20 characters per second |  | 24 24 characters per second 30 30 characters per second Typematic Delay When the typematic rate is enabled, this (Msec) section allows you select the delay between when the key was first depressed and when the acceleration begins. 250 250 msec 500 500 msec 750 750 msec 1000 1000 msec Security Option This category allows you to limit access to the system and Setup, or just to Setup System The system will not boot and access to Setup will be denIed if the correct password is not entered at the prompt Setup The system will boot, but access to Setup will be denIed if the correct password is not entered at the prompt To disable security, select PASSWORD SETTING at Main Menu and then you will be asked to enter password. Do not type anything and just press <Enter>, it will disable secu |  |  |  |

rity. Once

the security is disabled, the system will boot 				and you can enter Setup freely.

PCI/VGA 	It determines whether the MPEG ISA/VESA

Palette Snoop	VGA cards can work with PCI/VGA or not.

Enabled

When PCI/VGA working with MPEG ISA/VESA VGA Card

Disabled

When PCI/VGA not working with MPEG ISA/VESA VGA Card

OS Select for 	This item allows you to access the memory

DRAM > 64MB	that over 64MB in OS/2

Video BIOS	Determines whether video BIOS will be

Shadow	copied to RAM. However it is optional

depending on chipset design. Video

Shadow will increase the video speed.

C8000 - CBFFF 	These categories determine whether

Shadow	option ROMs will be copied to RAM. An

DC000 - DFFFF	example of such option ROM would be

Shadow 	support of onboard SCSI.

This screen controls the setting for the chipset on the mainboard.

ROM PCI/ISA BIOS (2A59IA2A)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPH

ERALS BIOS FEATURES SETUP SUPERVISOR PASSWORD CHIPSET FEATURES SETUP USER PASSWORD POWER MANAGEMENT SETUP IDE HDD AUTO DETECTION PNP/PCI CONFIGURATION SAVE & EXIT SETUP LOAD BIOS DEFAULTS EXIT WITHOUT SAVING LOAD SETUP DEFAULTS

| ESC: Quit áâàß:Select Item F10: Save & Exit Setup (Shift) F2 : Change Color AT Clock, DRAM Timmings ..... ROM PCI/ISA BIOS (2A59IA2A) CHIPSET FEATURES SETUP AWARD SOFTWARE, INC. Auto Configuration : Enabled DRAM Timing : 70ns | DRAM Leadoff Timing : 10/6/4 DRAM Read Burst (EDO/FP) : x333/X444 DRAM Write Burst Timing : x333 Fast EDO Lead Off : Disabled Refresh RAS# Assertion : 5 Clks Fast RAS to CAS Delay : 3 DRAM Page Idle Timer : 2 Clks DRAM |  |
| --- | --- | --- |

Enhanced Paging

: Enabled

Fast MA to RAS# Delay

: 2 Clks

SDRAM (CAS Lat/RAS-to-CAS)

: 3/3

SDRAM Speculative Read

: Disabled

System BIOS Cacheable

: Disabled

Video BIOS Cacheable

: Disabled

Esc: Quit

:Select Item

8-bit I/O Recovery Time

: NA

F1  : Help

PU/PD/+/-:Modify

16-bit I/O Recovery Time

: NA

F5  : Old Values

(Shift)F2  :Color

Memory Hole At 15M-16M

: Disabled

F6  :Load BIOS Defaults

PCI 2.1conpliance

: Disabled

F7  :Load Setup Defaults

Auto	Pre-defined values for DRAM, cache..timing

Configuration	according to CPU type & system clock.

When this item is enabled, the pre-defined

items will become SHOW-ONLY.

DRAM Timing	The DRAM speed is controlled by the DRAM

timing Registers. The timings programmed

into this register are dependent on the 				system design.

DRAM RAS#	DRAM must continually be refreshed or it will

Precharge Time	lose its data. Normally, DRAM is refreshed

entirely as the result of a single request. This

option allows you to determine the number

of CPU clocks allocated for the Row

Address Strobe to accumulate its charge

before the DRAM is refreshed. If insufficient

time is allowed, refresh

| may be incomplete and data lost. Fast RAS# to When DRAM is refreshed, both rows and CAS# Delay columns are addressed separately. This setup item allows you to determine the timing of the transition from RAS to Column Address Strobe (CAS). DRAM Read This sets the timing for burst mode read (or Burst (EDO/FP) wr |  |  |
| --- | --- | --- |

ites)from DRAM. Burst read and write

DRAM Write		requests are generated by the CPU in four

Burst Timing		separate parts. The first part provides the

location within the DRAM where the read or

write is to take place while the remaining

three parts provide the actual data. The

lower the timing numbers, the faster the 				system will address memory.

ISA Bus Clock	This item allows you to select the PCI clock

type. Choices are PCI CLK/3; PCI CLK/4

System BIOS	When enabled, accesses to the system BIOS

Cacheable 	ROM addressed at F0000H-FFFFFH are

cached, provided that the cache controller 			is enabled.

Video BIOS 	As with changing the system BIOS above,

Cacheable 	enabling the Video BIOS cache will cause

access to video BIOS addressed at C0000H

to C7FFFH to be cached, if the cache

controller is also enabled.

8-bit I/O 	The recovery time is the length of time,

Recovery Time 	measured in CPU clocks, which the system will

delay after the completion of an I/O request.

This item allows you to determine the

recovery time allowed for 8-bit I/O. Choices 			are from NA, 1 to 8 CPU clock

| s. 16-bit I/O This item allows you to determine the Recovery Time recovery time allowed for 16-bit I/O. Choices are from NA, 1 to 4 CPU clocks. Memory Hole At In order to improve performance, certain 15M-16M space in memory can be reserved for ISA cards. This memory must be mapped into the memory below 16MB. This screen controls the 'green' features of this mainboard. *** Refer to Page 4 ROM PCI/ISA BIOS (2A59IA2A) CMOS SETUP UTILITY AWARD |  |  |
| --- | --- | --- |

SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10

| : Save & Exit Setup (Shift) F2 : Change Color Sleep Timer, Suspen |  |  |
| --- | --- | --- |

d Timer, .....

ROM PCI/ISA BIOS (2A59IA2A)

POWER MANAGEMENT SETUP

AWARD SOFTWARE, INC.

Power Management

: Disabled

*Reload Global Timer Events*

PM Control by APM

: Yes

IRQ {3-7, 9-15}, NMI

: Disabled

Video

| Off Method : V/H SYNC Primary IDE 0 : Disabled +Blank Primary IDE 1 : Disabled Video Off After : Standby Secondary IDE 0 : Disabled Doze Mode : Disabled Secondary IDE 1 : Disabled Standby Mode : Disabled Floppy Disk : Disabled Suspend Mode : Disabled Serial Port : Enabled HDD Power Down : Disabled Parallel Port : Disabled Throttle Duty Cycle : 62.5% ZZ Active in Suspend : Disabled VGA Active Monitor : Enabled Soft-Off by PWR-BTTN : Instant-off*** |  |
| --- | --- |

CPUFAN Off In Suspend

: Enabled

Resume by Alarm

: Disabled

Esc: Quit   áâàß

:Select Item

Resume by Ring

: Disabled

F1  : Help

PU/PD/+/- : Modify

F5  : Old values

(Shift) F2: Color

*Break Events From Suspend*

F6  : Load BIOS Defaults

IRQ 8 Clock Event

: Disabled

F7  : Load Setup Defaults

Power 	This category allows you to select the type

Management	(or degree) of power saving and is directly

related to the following modes : Doze;

Standby; Suspend; HDD Power Down.

Disabled

No power management. Disables all 4 modes

Min. Power

Saving

Minimum power management. Doze =1hr.; Standby=1hr.; Suspend=1hr.; HDD Power Down=15min

Max.

Power

Sav

| ing Maximum power management only available for SL CPU‘s.Doze=1min.; Standby=1min.;Suspend=1min.;HDD Power Down=1min |  |
| --- | --- |
| User Defined Allows you to set each mode indivi-dually. When not disabled, each of the ranges are from 1min. to 1hr. except for |  |

HDD Power Down which ranges from 1 to 15min. and disable

PM Control 	When enabled, an Advanced Power

by APM	Management device will be activated to

enhance the Max. Power Saving Mode and

stop the CPU internal clock. If the Max. Power

Saving is not enabled, this will be present to NO.

Video Off	This determines the manner in which the

Method	monitor is blanked.

V/H SYNC

+ Blank

This selection will cause the system to turn off the vertical and horizontal sync. ports and write blanks to the video buffer

Blank

ScreenThis option only writes blanks to the video buffer

DPMS

Initial display power management signaling

The Following 4 modes are Green PC power saving function which are only user configuration when ‘User Defined’ power management has been selected.

Doze Mode	When enabled and after the set time of

system inactivity, the CPU clock will run at

slower speed while all other devices still 				operate at full speed

Standby Mode	When enabled and after the set time of

system inactivity, the fixed disk drive and the

video would be shut off while all other

devices still operate at full speed

Suspend Mode	When enabled and after the set time of

system inactivity, all devices expect the CPU

will be shut off

HDD Power Down	When enabled and after the se

| t |  |  |
| --- | --- | --- |

time of system

inactivity, the hard disk drive will be powered

down while all

| ot |  |  |
| --- | --- | --- |

her devices remain active

Wake Up Events	In Doze & Standby

Power Down

| & |  |  |
| --- | --- | --- |

These are I/O events whose occurrence can

Resume Events	prevent the system from entering a

| p |  |  |
| --- | --- | --- |

ower

saving mode or can awaken the system from

IRQ3 ~ IRQ15	such a mode. In effec

| t, |  |  |
| --- | --- | --- |

the system remains

alert for anything which occurs to a device

which is c

| on |  |  |
| --- | --- | --- |

figured as On, even when the

system is i

| n |  |  |
| --- | --- | --- |

a power down mode. When an

I/O device wants to gain the attention of the

operating system, it signals this by causing an

IRQ (Interrupt ReQuests) to occur. When the

operating system is ready to respond to the

request, it interrupts itself and performs

| th |  |  |
| --- | --- | --- |

e

service. When set off, activity will neither

prevent the system from going into a power

| ma |  |  |
| --- | --- | --- |

nagement mode nor awaken it.

This screen configures the PCI B

us

slots.

ROM PCI/ISA BIOS (2A59IA2A)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPE

| RVISOR PASSWORD CHIPSET FEATURES SETUP USER PASSWORD POWER MANAGEMENT SETUP IDE HDD AUTO DETECTION |  | PNP/PCI CONFIGURATION SAVE & EXIT SETUP LOAD BIOS DEFAULTS EXIT WITHOUT SAVING LOAD SETUP DEFAULTS ESC: Quit áâàß:Select Item F10: Save & Exit Setup (Shift) F2 : Change Color IRQ Settings, Latency Timers, ..... ROM PCI/ISA BIOS (2A59IA2A) PNP/PCI CONFIGURATION AWARD SOFTWARE, INC. PNP OS Installed : No | PCI IDE IRQ Map to : PCI-AUTO Resources Controlled by : Auto Primary IDE INT# : A Reset Configuration Data : Disabled Secondary IDE INT# |  |  |
| --- | --- | --- | --- | --- | --- |
| : B Esc: Quit áâàß :Select Item F1 : Help PU/PD/+/- : Modify |  |  | F5 : Old Values (Shift) F2: Color F6 : Load BIOS Defaults F7 : Load Setup Defaults Resource The Award Plug and Play BIOS has the Controlled by capacity to automatically configure all of the boot and Plug and Play compatible devices. However, this capability means absolutely nothing unless you are using a Plug and Play OS such as Windows 95 Choices are Auto and Manual Reset Config- This item allows you to determine reset the uration Data configuration data or not. PCI IRQ This sets the method by which the PCI bus Actived By recognizes that an IRQ service is being requested by a device. Under all circumstances, you should r |  |  |

etain the default configuration

unless advised otherwise by your system‘s

manufacturer. Choices are Level and Edge

PCI IDE IRQ	This allows you to configure your system to the

Map To	type of IDE disk controller in use. If you have

equipped your system with a PCI controller,

changing this allows you to specify which slot

has the controller and which PCI interrupt

(A,B,C,D) is associated with the connected

hard disk. Select ‘PCI Auto’ allows the system

to automatically determine how your IDE disk 				system is configured.

This section page includes all the items of IDE hard drive and

Programmed Input/Output features. See also Section “Chipset Features Setup”.

ROM PCI/ISA BIOS (2A59IA2A)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

ROM PCI/ISA BIOS (2A59IA2A)

INTEGRATED PERIPHERALS

AWARD SOFTWARE, INC.

IDE HDD Block Mode

: Disabled

Parallel Port Mode

ECP

IDE Primary Master PIO

: Auto

ECP Mode Use DMA

: 3

IDE Primary Slave PIO

: Auto

IDE Secondary Master PIO

: Auto

IDE Secondary Slave PIO

: Auto

IDE Primary Master UDMA

: Disabled

IDE Primary Slave UDMA

: Disabled

IDE Secondary Master UDMA

: Disabled

IDE Secondary Slave UDMA

: Disabled

On-Chip Primary PCI IDE

: Enabled

On-Chip Secondary PCI IDE

: Enabled

USB Keyboard Support

: Disabled

Onboard FDC Controller

: Enabled

Esc: Quit    áâàß

:Select Item

Onboard Serial Port 1

: Auto

F1 : Help

PU/PD/+/- : Modify

Onboard Serial Port 2

: Auto

F5 : Old Values

(Shift) F2: Color

UART 2 Mode

: StandardF6  : Load BIOS Defaults

Onboard Parallel Port

: 378H/IRQ7

F7  : Load Setup Defaults

IDE HDD Block	This allows your HD controller to use the fast

Mode	block mode to transfer data to and from your 				HD drive

Enabled

IDE controller uses block mode

Disabled

IDE controller uses standard mode

IDE Primary	PIO - Programmed Input/Output, it allows the

Master/Slave PIO	BIOS to tell the controller what it wants and

IDE Secondary	then let the controller and the CPU perform the

Master/Slave PIO	complete task by themselves. This simpler and

more faster. Your system supports five mods, 0 -

4, which primarily differ in timing. When Auto is

selected,the BIOS will select the best available 			mode.

On-Chip Primary	This setup item allows you either to enable or

PCI IDE 	disable the primary/secondary controller. You

On-Chip Second- 	might choose to disable the controller if you

ary PCI IDE 	were to add a higher performance or 						specialized controller.

PCI Slot IDE 	This item allows you designate an IDE controller

2nd Channel 	controller board insert into one of the physical

PCI slots as your secondary IDE

Enabled

External IDE controller designated as the secondary controller

Disabled

No IDE controller occupying a PCI slot

ROM PCI/ISA BIOS (2A59IA2A)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

ROM PCI/ISA BIOS (2A59IA2A)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

Hard Disks  Type  Size  CYLS  HEAD  PRECOMP  LANDZ  SECTOR  MODE

Primary Mster :

Select Primary Master  Option (N=Skip) : N

Options  Size  CYLS  Head  PRECOMP  LANDZ  Sector  Mode

2(Y)  1337   648  64       0     2594   63   LBA

1     1339  2595  16  65535     2594   63 NORMAL

3     1338  1297  32  65535     2594   63  LARGE

Note : Some Oses (like SCO-UNIX) must use “Normal” for installation

ESC : Skip

The last step is 'save and exit'. If you select this item and press

'Y', then these  records will be saved in the CMOS memory on

the mainboard. It will be checked every time when you turn your

computer on.

ROM PCI/ISA BIOS (2A59IA2A)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SE

| TU |  |  |
| --- | --- | --- |

P

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS D

| EF |  |  |
| --- | --- | --- |

AULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

SAVE to CMOS and EXIT (Y/N)

| :Y |  |  |
| --- | --- | --- |

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2

| : |  |  |
| --- | --- | --- |

Change Color

Auto-Configure HDD: Sector, Cylinder, Head .....

LOAD

| B |  |  |
| --- | --- | --- |

IOS DEFAULTS

When your mainboard has pro

| bl |  |  |
| --- | --- | --- |

ems and need to 	debug or 	troubleshoot the system, you can use this

function.The default values loaded only affect the

BIOS Features Setup, Chipset Features Setup, Power

Management Setup and PNP/PCI Configuration

Setup. There is no effect on the Standard CM

| OS |  |  |
| --- | --- | --- |

Setup.To use this function,select it from main menu

and press <Enter>. A line will appear on th

| e screen asking if you want to load the BIOS default values. Press <Yes> |  |
| --- | --- |

and <Enter> then the BIOS default values 	will be loaded.

LOAD SETUP DEFAULTS

This allows you load optimal settings which are stored

in the

| BIOS ROM. The default values loaded only affect the BIOS Features Setup, Chipset Features Setup, Power Management Setup and PNP/PCI Configuration Setup.There is no effect on the Standard CMOS Setup. To use this function, select it from main menu and press <Enter>.A line will appear on the screen asking if you want to load the Setup default values. Press <Yes> and <Enter> then the Setup default values will be loaded. SUPERVISOR PASSWORD / USER PASSWORD This allows you to set the password. the mainboard defaults password disabled. Enter/Change password : Enter the current password,at the prompt key-in your new password (up to eight alphanumeric characters), press <Enter>. At the next prompt, confirm the new password by typing it and press <Enter> again. Disable password : Press <Enter> key instead of entering a new password when the ‘Enter Password’ dialog box appears. A message will appear confirming that the password is disable. If you set both supervisor and user passwords, only the supervisor password allows you to enter the BIOS SETUP program. CAUTION :If you forget your password, you must disable the CMOS by turning power off and set JP16 ‘close‘. Then reload the system. Please refer to page 23. IDD HDD AUTO DETECTION This allows you to detect IDE hard drivers‘ parameters and enter them into ‘Standard CMOS Setup’ automatically. If the auto-detected parameters displayed do not match the ones that should be used for your hard drive, do not accept them. Press <N> to reject the values and enter the correct ones manually on the Standard CMOS Setup screen. SAVE & EXIT SETUP This allows you to save the new setting values in the CMOS memory and continue with the booting process. Select what you want to do, press <Enter>. EXIT WITHOUT SAVING This allows you to exit the BIOS setup utility without recording any new values or changing |  |  |  |  |
| --- | --- | --- | --- | --- |

old ones.

¡ Control Key Description ¡

UP ARROW

¡

Move to previous item

DOWN ARROW

¡Move to next item

LEFT ARROW

¡

Move to the ite

| m |  |  |  |  |
| --- | --- | --- | --- | --- |

in the left hand

RIGHT ARROW

¡

Move to the item in the right h

| an |  |  |  |  |
| --- | --- | --- | --- | --- |

d

Esc KEY

Esc

Main Menu : Quit and not save changes

Setup menu : Exit current page and retur

| n |  |  |  |  |
| --- | --- | --- | --- | --- |

to main menu

PgUp KEY

Increase the numeric value or make changes

PgDn KEY

Decrease the numeric

| v |  |  |  |  |
| --- | --- | --- | --- | --- |

alue or make changes

F1 KEY

Help

General help

F2 KEY

¡Shift¡+F2

Change color from total 16 colors

F5 KEY

Old Value

Restore the pervious CMOS value from CMOS

F6 KEY

Load BIOS default

Load the default CMOS v

| al |  |  |  |  |
| --- | --- | --- | --- | --- |

ue from BIOS default table

F7 KEY

Load setup default

Load Setup default

F10 KEY

Save & Exit Setup

Sa

| ve |  |  |  |  |
| --- | --- | --- | --- | --- |

## all the CMOS changes and Exit setup, only for Main Menu

## APPENDIX A

### ¡¡TECHNICAL SUPPORT REQUEST FORM¡¡

If the mainboard doesn't function properly, please complete the following

information and return it to your system dealer. If the further information

is needed, please attach this separating sheets.

Model No :  ATC-5020      Date of  Purchase :  ______________

Serial No : ___________________

HARDWARE :

BRAND

MODEL

SPEED

Q'TY

CPU

SIM Module

PB SRAM on Board

TAG SRAM

SDRAM Module

SIMM  :_______MB (__ EDO, __FastPage)

Hard Disk Interface Controller : __ __ IDE, ____ SCSI

Hard Disk Brand :___________, Model :_________, Capacity :________

Display Controller Brand :_____________,Model :__________

Controller Chip Brand :_____________,Model :__________

AWA

| RD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

SYSTEM BIOS: Versio

| n |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

_______  Date Code _____________

| Ke |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

yboard BIOS: Brand __________

Other

| Ad |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

d-on Cards Information:

Add-o

| n |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Card

Bus Interface

Model

Remark

Error Description :