CONTENTS

INDEX	A

HOW TO INSTALL THE SOCKET 7 PROCESSOR ON

THE ATC-5220 MAINBOARD………………………………..B

HOW TO USE THIS MANUAL……………………………….2

INTRODUCTION TO THE ATC-5220 MAINBOARD

HARDWARE(Including easy installation)…………………….3

INSIDE THE ATC-5220 MAINBOARD PACKAGE………...9

CH

APTER 1   Introduction…………………………………..10

1-1 Soft-Off-Control	11

1-2 Wake-On-LAN	.12

CHAPTER 2   Installation	13

2-1 Installation Procedure 	.13

2-2 CPU Installation	.14

2-3 System Memory Installation……………………………………..37

2-4 Connectors Description	.38

2-5 IDE Driver Installation	.43

2-6 VIA MVP3 AGP VGA Driver Installation	.43

CHAPTER 3   Award BIOS Setup	44

3-1 Update BIOS Procedure	.45

3-2 Award System BIOS Configuration Setup	.46

Appendix A ……………………………………………………..A

##### HOW TO USE THIS MANUAL

To obtain maximum use from this manual it is suggested:

Read Page A COMPONENT LOCATION DIAGRAM where you find the mainboard layout diagram. Please refer to it when you configure the system.

Read about an overview of the mainboard features, packing contents, and  how to upgrade as well as to change hardware configurations such as memory size, CPU type, jumper settings lists and connectors in the following categories:

INTRODUCTION TO THE ATC-5220 MAINBOARD HARDWARE

INSIDE THE ATC-5220 MAINBOARD PACKAGE

Chapter 1  Introduction

Chapter 2  Installation

When you have finished reading of both chapter 1 and chapter 2, turn to Chapter 3 Award BIOS Setup where you will find the update BIOS procedure and the further information which is stored in the SETUP is the system hardware configuration.

Your system dealer will set up the mainboard according to your demand of the computer. It means that the current settings of your mainboard may not be the same as the defaults shown in this user's manual. If you need to change your configuration, please ask your dealer first. Be sure this will not void your system warranty, or ask your dealer to do it for you.

###### The product , ATC-5220 mainboard , quick description as following :

###### ATC-5220 mainboard supports 100MHz host clock Super 7 processor, ie. AMD K6-2, Cyrix M II. Of course , ATC-5220 mainboard also supports 66 ~ 83.3 MHz host clock Socket 7 processor, ie. Intel Pentium & Pentium processor with MMX technology, AMD K5/K6 , Cyrix 6X86/6X86MX , and IDT Winchip.

###### ATC-5220 mainboard, VIA MVP3 chipset, supports AGP slot for best        performance of VGA display in 3D application .

REMARK

Intel®  is a registered trademark of Intel Corporation.

All other brands and product names are trademarks registered trademarks of their respective companies.

##### INTRODUCTION TO THE ATC-5220 MAINBOARD

HARDWARE(Including easy installation)

###### ATC-5220 mainboard supports 100MHz host clock Super 7 processors, ie. AMD  K6-2, AMD K6-III, Cyrix MII. , also with Socket 7 processor from Intel , AMD , Cyrix and IDT . The AGP slot is the second important feature for this product .

###### It is important to remember that you must be set Jumper JP5, Jumper JP6 , Jumper JP7, Jumper JP13, DIP switch SW1 and SW2 accurately, so that you can power up your system , correctly.

###### The followings are the description of these important DIP switch & jumpers :

SW1,JP7,JP5,JP6	: The completed jumper group to setup CPU and system

frequency.

SW2, JP13			: The completed jumper group to setup CPU working

voltage.

More detailed information as following:

SW1					: This DIP switch is to setup the CPU host clock

Speed(Front Side Bus Speed) & the times of

multiple with CPU host clock, for your installing

CPU frequency.

JP5 & JP6(pair)	: The jumper-pair are to select the Synchronous mode

or the Asynchronous mode , comparing with CPU host

clock and DRAM clock.

The Synchronous Mode	: The speed of the DRAM

clock is same as the CPU host

clock, ie.

The CPU host clock is 100 MHz.

The DRAM clock is 100 MHz.

or,

The CPU host clock is 66 MHz.

The DRAM clock is 66 MHz.

The Asynchronous Mode		: The speed of the DRAM

clock is not same as the CPU host

clock, ie.

The CPU host clock is 100 MHz.

The DRAM clock is 66 MHz.

JP7				: This jumper is to setup the frequency-ratio comparing

With the PCI clock and the CPU host clock.

Set JP7 to Pin2-3      :The ratio is 2 : 1 , ie.

The CPU host clock is 66MHz.

The PCI clock is 33 MHz.

Set JP7 to Pin1-2      :The ratio is 3 : 1 , ie.

The CPU host clock is 100 MHz.

The PCI clock is 33 MHz.

SW2				: The Vcore, CPU core voltage selection, ranging from

2.2V, 2.8V, 2.9V, 3.2V, 3.3V and 3.5V.

JP13			: The selection of the Single voltage for CPU(P54C

compatible) or the Dual voltage for CPU(P55C

compatible).

2-3, 5-6 close		: the single voltage support

1-2, 4-5 close		: the dual voltage support

The following DIP switch & Jumper charts are the effective information for you to setup correct CPU and total system speed , when installing your system with ATC-5220 mainboard :

SW1 & JP7 setting

(The setting for CPU host clock , Times of multiple clock & PCI clock ratio)

| SW1 | SW1-1 | SW1-2 | SW1-3 | JP7 | SW1 | SW1-4 | SW1-5 | SW1-6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66 | OFF | OFF | OFF | 2-3* | 2.0x | ON | OFF | OFF |
| 68.5 | ON | ON | ON | 2-3* | 2.5x | ON | ON | OFF |
| 75 | OFF | ON | OFF | 2-3* | 3.0x | OFF | ON | OFF |
| 83.3 | ON | ON | OFF | 2-3* | 3.5x | OFF | OFF | OFF |
| 95 | ON | OFF | ON | 1-2** | 4.0x | ON | OFF | ON |
| 100 | OFF | OFF | ON | 1-2** | 4.5x | ON | ON | ON |
| 112 | OFF | ON | ON | 1-2** | 5.0x | OFF | ON | ON |

*  JP7: 2-3 close : the PCI clock ratio is 1 : 2 , it means :

If the CPU host clock is lower than 75 MHz ,

it Is recommend to setup as 1 : 2.

** JP 7 : 1-2 close : the PCI clock ratio is 1 : 3 , it means :

If the CPU host clock is higher than 83.3 MHz ,

It is recommend to setup as 1 : 3.

#### JP5 & JP6 Setting

|  | JP 5 | JP 6 |
| --- | --- | --- |
| ASYNC.* | 2-3 | 1-2 |
| SYNC.** | 1-2 | 2-3 |

* ASYNC. mode				: When the CPU host clock is 100 MHz , and the DRAM

clock is 66 MHz .

** SYNC. mode 				: 1. When the CPU host clock is 100 MHz , and the

SDRAM clock is followed PC-100 specification .

- When the CPU host clock is 60 ~ 83.3 MHz , and

the DRAM/SDRAM clock is not PC-100 spec. .

SW 2 Setting

| Vcore | SW2-1 | SW2-2 | SW2-3 | SW2-4 | SW2-5 |
| --- | --- | --- | --- | --- | --- |
| 2.2V | OFF | ON | OFF | OFF | OFF |
| 2.8V | OFF | OFF | OFF | ON | OFF |
| 2.9V | ON | OFF | OFF | ON | OFF |
| 3.2V | OFF | OFF | ON | ON | OFF |
| 3.3V | ON | OFF | ON | ON | OFF |
| 3.5V | ON | ON | ON | ON | OFF |

JP13 setting

(The setting for Single/Dual Voltage supporting for Pentium level Processor)

| Mode | JP13 |  |
| --- | --- | --- |
| Single Voltage Mode(Vcore = Vi/o)* | 2-3 close | 5-6 close |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 close | 4-5 close |

* P54C mode (Single Voltage Mode)	: supports for Intel Pentium, AMD-K5,

Cyrix 6x86, IDT Win Chip C6.

** P55C mode (Dual Voltage Mode)	: supports for Intel Pentium MMX ,

AMD-K6, AMD-K6-2, Cyrix 6x86L,

Cyrix 6x86MX, Cyrix M II.

The following two examples will show you how to set JP 5, JP 6, JP7, JP13,

DIP_Switch SW1 & SW2 for most often usage CPU type .

The first example is for Intel Pentium® MMX 233 MHz processor.

The second example is for AMD-K6-2® 300 MHz (100 MHz host clock * 3) processor.

Example 1

Intel PentiumTM MMX 233MHz

|  | JP 5 | JP 6 |
| --- | --- | --- |
| Synchronous (CPU host clock & DRAM clock are the same) | Pin 1-2 | Pin 2-3 |

| CPU HOST CLOCK / PCI CLOCK | JP 7 |
| --- | --- |
| 66MHz/33MHz | Pin 2-3 |

SW2 (2.8V)

SW2

ON OFF 1 2 3 4

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 233MHz | ON OFF 1 2 3 4 5 6 | 66 x 3.5 |

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 close | 4-5 close |

Example 2

AMD-K6-2® 300MHz

|  | JP 5 | JP 6 |
| --- | --- | --- |
| Synchronous (CPU host clock & DRAM clock are same speed) * | Pin 1-2 | Pin 2-3 |
| Asynchronous (CPU host clock & DRAM clock are different speed) ** | Pin 2-3 | Pin 1-2 |

* Synchronous mode 	: The SDRAM module follows PC-100 spec.

** Asynchronous mode: The DRAM/SDRAM module isn’t PC-100 spec.

| CPU HOST CLOCK / PCI CLOCK | JP 7 |
| --- | --- |
| 100MHz/33MHz | Pin 1-2 |

SW2 (2.2V)

SW2

ON OFF 1 2 3 4

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 300MHz | ON OFF 1 2 3 4 5 6 | 100 x 3.0 |

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 close | 4-5 close |

###### ATC-5220 mainboard supports 100MHz host clock Super 7 processors, ie. AMD K6-2, Cyrix MII. , also with Socket 7 processor from intel , AMD , Cyrix  and IDT . The AGP slot is the second important feature for this product .

###### Each ATC-5220 mainboard supports or contains the following components:

- Supports Intel Pentium (P54C) CPU operating from 75MHz to 200MHz,

Intel Pentium MMXTM 166~233MHz (P55C), AMD K5, AMD K6, AMD K6-2

Cyrix 6x86MXTM, MII, 6x86L.

- VIA @ VT82C598AT.

- Using three 168-pin DIMM sockets, provides three banks of 64-bit wide path up to 384MB SDRAM or 768 EDO DRAM (with parity chip ECC support).

- Built-in Switching Voltage Regulator.(VRM 8.2 SPEC.)

- Supports CPU core voltage range from 2.0V up to 3.5V.

- Supports one AGP slot, four PCI slots with revision 2.1 interface compliant and two 16-bit ISA slots.

- Dual Master IDE connectors support Ultra DMA/33, up to four devices in two channels for connecting of high capacity hard drive, CD-ROM disk drive, tape backup etc..

- Two USB (Universal Serial Bus) connectors support up to 127 peripheral devices.

- PS/2 keyboard connector and PS/2 mouse connector.

- SMC-FDC37M669 high-speed Super Multi-I/O chipset.

- Supports Infrared transfer (IrDA TX/RX) connection.

- One FDC port supports two devices up to 2.88MB.

- Two 16550A fast UARTs compatible serial ports.

- One EPP/ECP mode parallel printer port.

- Hardware Dimension is 305mm x 180mm(12" x 7.08"); ATX Form Factor.

- ATX power supply connector.

INSIDE THE ATC-5220 MAINBOARD PACKAGE

The mainboard comes securely packed in a durable box and shipping carton. If any of the following items are missing or damaged, please contact your supplier.

Each mainboard contains:

Q'TY     	Description

1             Mainboard       : ATC-5220.

1             CD  	      	  : Enhanced IDE driver

Award system BIOS Update Utility

VIA MVP3 AGP VGA Vgart.vxd

Utility

VIA IRQ Routing Utility

VIA MVP3 ACPI compliant utility

1             Cable 	         : Enhanced IDE cable.

1             Cable 	         : F.D.D. cable.

1             Manual 	         : User manual. (English version)

CHAPTER 1  INTRODUCTION

###### It is important to remember that you must be set Jumper JP5, Jumper JP6 , Jumper JP7, Jumper JP13, DIP switch SW1 and SW2 accurately, so that you can power up your system , correctly.

###### The followings are the description of these important DIP switch & jumpers :

SW1,JP7,JP5,JP6	: The completed jumper group to setup CPU and system

frequency.

SW2, JP13		: The completed jumper group to setup CPU working

voltage.

###### The followings are the locations of these important DIP switch & jumpers :

JP5                                                   JP13(RED

(YELLOW JUMPER)                    SW1                JUMPER)

AGP slot             JP6             JP7                SW2

(YELLOW      (YELLOW

JUMPER)       JUMPER)

1-1 SOFTWARE POWER OFF CONTROL

The mainboard design supports Software Power Off Control feature through the SMM code in the BIOS under Windows 95/98, and MS-DOS operation system environment. This is Intel ATX form factor feature and you should use ATX power supply.

First, you should connect the power switch cable (provided by the ATX case supplier) to the connector “PS-ON” on the mainboard.  In the BIOS screen of POWER MANAGEMENT SETUP’, choose “User Defined” (or “Min. Power Saving” or “Max. Power Saving”) in ‘Power Manager’ and choose “Yes” in ‘PM Control by APM’.

In Windows 95/98, if you would like to power off the system, you just choose “shutdown the computer ?” in the “Shut Down Windows“ from Windows 95/98, then the system power will be off directly, and become the stand-by status. If you would like to restart the system, just press the power switch button, and the system will be powered on.

Note : If you will leave your system for several days, we suggest you use hardware power off to shutdown your system.

1-2 Wake-On-LAN

The remote Wake-On-LAN mode of operation is a mechanism that uses Advanced Micro Device Magic Packet technology to power up a sleeping workstation on the network. This mechanism is accomplished when the LAN card receives a specific packet of information, called a Magic Packet, addressed to the node on the network. For additional protection, Secure ON is an optional security feature that can be added to the Magic Packet that requires a password to power up the sleeping workstation. When the LAN card is in remote Wake-On-LAN mode, main system power can be shut down leaving power only for the LAN card and auxiliary power recondition.

The LAN card performs no network activities while in the remote Wake-On-LAN mode of operation. It only monitors the network for receipt of a Magic Packet. If a Magic Packet is addressed to the LAN card on the network, the LAN card wake up the system. If the Secure ON feature has been enabled, the password added to the Magic Packet is also verified prior to waking up the system.

WOL LAN card will provide a 3-pin line to connect the WOL connector on the mainboard.

CAUTION :

For Wake-on-LAN, the +5V standby line for the power supply must capable of

Delievring +5V ±5% at 720mA. Failure to provide adequate standby current when

implementing Wake-on-LAN, can damage the power supply.

Before you enable Wake-on-LAN function, first check your power supply specification to meet the above requirement or not.

CHAPTER 2  INSTALLATION

2-1 INSTALLATION PROCEDURE

Before installing the computer, please prepare all components such as CPU, DRAM; peripherals such as hard drive, keyboard, CD-ROM disk drive and accessories such as cables. Then, install the system as following:

- Plug CPU, heat sink, cooling fan and DRAM modules into the ATC-5220 mainboard.

2. Set jumpers based on your configuration.

3. Set DIP switch based on your configuration.

4. Plug add-on cards into PCI/ISA slots, if needed.

5. Connect the power supply.

6. Connect I/O and other cables to the system.

7. Make sure all components and devices are well connected, turn on the power and

setup System BIOS based on your configuration.

8. Install peripheral devices, add-on card drivers and run the system configuration

program.

If all of above procedures are running successfully, turn it off and screw the chassis cover to the case, and then connect external devices which are cabled to the system.

2-2 CPU  INSTALLATION

ATC-5220 supports one Pentium level CPU.

For installation, please notice CPU pin 1 must align with the ZIF socket 7 pin 1 location. Before you install or upgrade your CPU, please read CPU guide from CPU manufacturer to make sure the CPU voltage specification. Then choose the right installation in section 2-2-1 based on your CPU type / brand and follow the description to setup DIP Switch & Jumpers.

ATC-5220 uses the following DIP Switch & Jumpers for the user to install CPU easily.

SW1 (1-3）for External clock and SW1（4-6）for Frequency_Ratio.

SW2 for different CPU voltage value;

JP13 for Single/Dual CPU Voltage selection.

JP7 for CPU host clock/PCI clock ratio selection , 2 : 1 or 3 : 1 .

JP5 & JP6 for CPU host clock/DRAM clock ratio selection , Asynchronous or Synchronous .

The following DIP switch & Jumper charts are the effective information for you to setup correct CPU and total system speed , when installing your system with

ATC-5220 mainboard :

SW1 & JP7 setting

(The setting for CPU host clock , Times of multiple clock & PCI clock ratio)

| SW1 | SW1-1 | SW1-2 | SW1-3 | JP7 | SW1 | SW1-4 | SW1-5 | SW1-6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66 | OFF | OFF | OFF | 2-3* | 2.0x | ON | OFF | OFF |
| 68.5 | ON | ON | ON | 2-3* | 2.5x | ON | ON | OFF |
| 75 | OFF | ON | OFF | 2-3* | 3.0x | OFF | ON | OFF |
| 83.3 | ON | ON | OFF | 2-3* | 3.5x | OFF | OFF | OFF |
| 95 | ON | OFF | ON | 1-2** | 4.0x | ON | OFF | ON |
| 100 | OFF | OFF | ON | 1-2** | 4.5x | ON | ON | ON |
| 112 | OFF | ON | ON | 1-2** | 5.0x | OFF | ON | ON |

*  JP7: 2-3 closed : the PCI clock ratio is 1 : 2 , it means :

If the CPU host clock is lower than 75 MHz,

it Is recommend to setup as 1 : 2.

** JP 7 : 1-2 closed : the PCI clock ratio is 1 : 3 , it means :

If the CPU host clock is higher than 83.3 MHz,

it is recommend to setup as 1 : 3.

#### JP5 & JP6 Setting

|  | JP 5 | JP 6 |
| --- | --- | --- |
| ASYNC.* | 2-3 | 1-2 |
| SYNC.** | 1-2 | 2-3 |

* ASYNC. mode				: When the CPU host clock is 100 MHz , and the DRAM

clock is 66 MHz .

** SYNC. mode 				: 1. When the CPU host clock is 100 MHz , and the

SDRAM clock is followed PC-100 specification .

2. When the CPU host clock is 60 ~ 83.3 MHz , and

the DRAM/SDRAM clock is not PC-100 spec. .

SW 2 Setting

| Vcore | SW2-1 | SW2-2 | SW2-3 | SW2-4 |
| --- | --- | --- | --- | --- |
| 2.2V | OFF | ON | OFF | OFF |
| 2.3V | ON | ON | OFF | OFF |
| 2.4V | OFF | OFF | ON | OFF |
| 2.7V | ON | ON | ON | OFF |
| 2.8V | OFF | OFF | OFF | ON |
| 2.9V | ON | OFF | OFF | ON |
| 3.2V | OFF | OFF | ON | ON |
| 3.3V | ON | OFF | ON | ON |
| 3.5V | ON | ON | ON | ON |

JP13 setting

(The setting for Single/Dual Voltage supporting for Pentium level Processor)

| Mode | JP13 |  |
| --- | --- | --- |
| Single Voltage Mode(Vcore = Vi/o)* | 2-3 closed | 5-6 closed |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 closed | 4-5 closed |

* P54C mode (Single Voltage Mode)	: supports for Intel Pentium, AMD-K5,

Cyrix 6x86, IDT Win Chip C6.

** P55C mode (Dual Voltage Mode)	: supports for Intel Pentium MMX ,

AMD-K6, AMD-K6-2, Cyrix 6x86L,

Cyrix 6x86MX, Cyrix M II.

2-2-1 CPU TYPE SELECTION

A. INTEL PENTIUM CPU (P54C)

- P54C VRE : 3.400V~3.600V (The fourth line of the mark on the under-side

of the processor contains a code that identifies the voltage level

type. V is VRE, S is standard. )

SW2(3.5V)

SW2

ON OFF 1 2 3 4

※ P54C STD : 3.135V ~ 3.600V (The fourth line of the mark on the

under-side of the processor contains a code that identifies

the voltage level type. V is VRE, S is standard. )

SW2(3.3V)

SW2

ON OFF 1 2 3 4

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin2-3” position.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 1-2 | Pin 2-3 | Pin 2-3 |

| Mode | JP13 |  |
| --- | --- | --- |
| Single Voltage Mode(Vcore = Vi/o)* | 2-3 closed | 5-6 closed |

Intel Pentium 100MHz

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 100MHz | ON OFF 1 2 3 4 5 6 | 66 x 1.5 |

Intel Pentium 133MHz

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 133MHz | ON OFF 1 2 3 4 5 6 | 66 x 2.0 |

Intel Pentium 166MHz

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 166MHz | ON OFF 1 2 3 4 5 6 | 66 x 2.5 |

Intel Pentium 200MHz

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 200MHz | ON OFF 1 2 3 4 5 6 | 66 x 3.0 |

B. INTEL PENTIUM MMXTM CPU (P55C)

SW2(2.8V)

SW2

ON OFF 1 2 3 4

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin2-3” position.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 1-2 | Pin 2-3 | Pin 2-3 |

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 closed | 4-5 closed |

B-1.  Intel PentiumTM MMX 166MHz

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 166MHz | ON OFF 1 2 3 4 5 6 | 66 x 2.5 |

B-2.  Intel PentiumTM MMX 200MHz

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 200MHz | ON ###### OFF 1 2 3 4 5 6 | 66 x 3.0 |

B-3.  Intel PentiumTM MMX 233MHz

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 233MHz | ON OFF 1 2 3 4 5 6 | 66 x 3.5 |

C. Cyrix 6x86MX CPU

SW2 (2.9V)

SW2

ON OFF 1 2 3 4

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin2-3” position.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
|  | Pin 1-2 | Pin 2-3 | Pin 2-3 |

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 closed | 4-5 closed |

C-1.  Cyrix  6x86MX PR166 @ 66MHz Bus 2x

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| PR166 | ON OFF 1 2 3 4 5 6 | 66 x 2.0 |

C-2.  Cyrix  6x86MX PR200MHz @ 66MHz Bus 2.5x

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| PR200 | ON OFF 1 2 3 4 5 6 | 66 x 2.5 |

C-3.  Cyrix  6x86MX PR233 @ 66MHz Bus 3x

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| PR233 | ON OFF 1 2 3 4 5 6 | 66 x 3.0 |

C-4.  Cyrix  6x86MX PR233 @ 75MHz Bus 2.5x

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| PR233 | ON OFF 1 2 3 4 5 6 | 75 x 2.5 |

D.  Cyrix MII CPU

D-1  Cyrix MII 300MHz @ 66MHz Bus 3.5x

SW2(2.9V)

SW2

ON OFF 1 2 3 4

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin2-3” position.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 1-2 | Pin 2-3 | Pin 2-3 |

#### JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 closed | 4-5 closed |

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| MII 300MHz | ON OFF 1 2 3 4 5 6 | 66 x 3.5 |

D-2  Cyrix MII 75MHz Bus

SW2(2.9V)

SW2

ON OFF 1 2 3 4

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin2-3” position.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 75MHz/75MHz/37.5MHz | Pin 1-2 | Pin 2-3 | Pin 2-3 |

#### JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 closed | 4-5 closed |

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| MII 300MHz | ON OFF 1 2 3 4 5 6 | 75 x 3.0 |
| MII 366MHz | ON OFF 1 2 3 4 5 6 | 75 x 4.0 |

D-3  Cyrix MII 83MHz Bus

SW2(2.9V)

SW2

ON OFF 1 2 3 4

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin1-2” position.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 83.3MHz/83.3MHz/27.7MHz | Pin 1-2 | Pin 2-3 | Pin 1-2 |

#### JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 closed | 4-5 closed |

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| MII 333MHz | ON OFF 1 2 3 4 5 6 | 83 x 3.0 |
| MII 366MHz | ON OFF 1 2 3 4 5 6 | 83 x 3.5 |

D-4  Cyrix MII 100MHz Bus

SW2(2.9V)

SW2

ON OFF 1 2 3 4

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin1-2” position for PC-100 SDRAM module or set JP5, JP6 and JP7 to “Pin2-3, Pin1-2 and Pin1-2” position for non PC-100 SDRAM module or EDO DIMM module.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 100MHz/100MHz/33MHz* | Pin 1-2 | Pin 2-3 | Pin1-2 |
| 100MHz/66MHz/33MHz** | Pin 2-3 | Pin 1-2 | Pin 1-2 |

*	Please use for PC-100 SDRAM module

**	Please use non PC-100 SDRAM module or EDO DIMM module

#### JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 closed | 4-5 closed |

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| MII 366MHz | ON OFF 1 2 3 4 5 6 | 100 x 2.5 |
| MII 380MHz | ON OFF 1 2 3 4 5 6 | 100 x 3.0 |

 If your CPU type is not included in this section, please set SW1 according to your CPU working frequency. ( Please refer to page 14,15.)

E. Cyrix 6x86L CPU (dual voltage)

SW2 (2.8V)

SW2

ON OFF 1 2 3 4

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin2-3” position.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
|  | Pin 1-2 | Pin 2-3 | Pin 2-3 |

#### JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 closed | 4-5 closed |

E-1.   Cyrix 6x86L PR166+

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| PR166+ | ON OFF 1 2 3 4 5 6 | 66 x 2.0 |

F.  AMD-K5 CPU Series

SW2(3.5V)

SW2

ON OFF 1 2 3 4

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin2-3” position.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz* | Pin 1-2 | Pin 2-3 | Pin2-3 |

#### JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Single Voltage Mode(Vcore = Vi/o)* | 2-3 closed | 5-6 closed |

F-1.  AMD-K5 PR100

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| PR100 | ON OFF 1 2 3 4 5 6 | 66 x 1.5 |

F-2.  AMD-K5 PR133

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| PR133 | ON OFF 1 2 3 4 5 6 | 66 x 1.5 |

F-3.  AMD-K5 PR166MHz

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| PR166 | ON OFF 1 2 3 4 5 6 | 66 x 2.5 |

G.  AMD-K6 CPU

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin2-3” position.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 1-2 | Pin 2-3 | Pin 2-3 |

#### JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 closed | 4-5 closed |

G-1.  AMD-K6 166MHz

SW2(2.9V)

SW2

ON OFF 1 2 3 4

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 166MHz | ON OFF 1 2 3 4 5 6 | 66 x 2.5 |

G-2.  AMD-K6 200MHz

SW2(2.9V)

SW2

ON OFF 1 2 3 4

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 200MHz | ON OFF 1 2 3 4 5 6 | 66 x 3.0 |

G-3a.  AMD-K6 233MHz

SW2(3.2V)

SW2

ON OFF 1 2 3 4

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 233MHz | ON OFF 1 2 3 4 5 6 | 66 x 3.5 |

G-3b.  AMD-K6-2 233MHz

SW2(2.2V)

SW2

ON OFF 1 2 3 4

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 233MHz | ON OFF 1 2 3 4 5 6 | 66 x 3.5 |

G-4  AMD-K6/K6-2 266 MHz, AMD-K6 300MHz

SW2(2.2V)

SW2

ON OFF - 2 3 4

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 266MHz | ON OFF 1 2 3 4 5 6 | 66 x 4.0 |
| 300MHz | ON OFF 1 2 3 4 5 6 | 66 x 4.5 |

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin2-3” position.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 1-2 | Pin 2-3 | Pin2-3 |

#### JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 closed | 4-5 closed |

G-5  AMD-K6-2 333MHz (option)

SW2(2.2V)

SW2

ON OFF 1 2 3 4

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 333MHz | ON OFF 1 2 3 4 5 6 | 95 x 3.5 |

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin1-2” position for PC-100 SDRAM module or set JP5, JP6 and JP7 to “Pin2-3, Pin1-2 and Pin1-2” position for non PC-100 SDRAM module or EDO DIMM module.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 95MHz/95MHz/31.7MHz* | Pin 1-2 | Pin 2-3 | Pin1-2 |
| 95MHz/66MHz/31.7MHz** | Pin 2-3 | Pin 1-2 | Pin 1-2 |

*	Please use for PC-100 SDRAM module

**	Please use non PC-100 SDRAM module or EDO DIMM module

#### JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o) | 1-2 closed | 4-5 closed |

G-6  AMD-K6-2 300, 350, 400MHz

SW2(2.2V)

SW2

ON OFF 1 2 3 4

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 300MHz | ON OFF 1 2 3 4 5 6 | 100 x 3.0 |
| 350MHz | ON OFF 1 2 3 4 5 6 | 100 x 3.5 |
| 400MHz | ON OFF 1 2 3 4 5 6 | 100 x 4.0 |

#### JP5, JP6 & JP7 Setting

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 100MHz/100MHz/33MHz* | Pin 1-2 | Pin 2-3 | Pin1-2 |
| 100MHz/66MHz/33MHz** | Pin 2-3 | Pin 1-2 | Pin 1-2 |

*	Please use for PC-100 SDRAM module

**	Please use non PC-100 SDRAM module or EDO DIMM module

#### JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o) | 1-2 closed | 4-5 closed |

G-7.  AMD-K6-III 400MHz, AMD-K6-2, K6-III 450MHz

SW2(2.4V)

SW2

ON OFF 1 2 3 4

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 400MHz | ON OFF 1 2 3 4 5 6 | 100 x 4.0 |
| 450MHz | ON OFF 1 2 3 4 5 6 | 100 x 4.5 |

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin1-2“ position for PC-100 SDRAM module or set JP5, JP6 and JP7 to “Pin2-3, Pin1-2 and Pin1-2“ position for non PC-100 SDRAM module or EDO DIMM module.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 100MHz/100MHz/33MHz* | Pin 1-2 | Pin 2-3 | Pin1-2 |
| 100MHz/66MHz/33MHz** | Pin 2-3 | Pin 1-2 | Pin 1-2 |

*	Please use for PC-100 SDRAM module

**	Please use non PC-100 SDRAM module or EDO DIMM module

#### JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Dual Voltage Mode(Vcore ≠Vi/o)** | 1-2 closed | 4-5 closed |

H. IDT Win Chip 2

SW2(3.5V)

SW2

ON OFF 1 2 3 4

JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Single Voltage Mode(Vcore = Vi/o)* | 2-3 closed | 5-6 closed |

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin2-3” position.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 66MHz/66MHz/33MHz | Pin 1-2 | Pin 2-3 | Pin 2-3 |

IDT Win Chip 2 200, 233MHz

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 200MHz | ON OFF 1 2 3 4 5 6 | 66 x 3.0 |
| 233MHz | ON OFF 1 2 3 4 5 6 | 66 x 3.5 |

IDT Win Chip 2 225MHz

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 225MHz | ON OFF 1 2 3 4 5 6 | 75 x 3.0 |

SW2(3.5V)

SW2

ON OFF 1 2 3 4

JP13 Setting

| Mode | JP13 |  |
| --- | --- | --- |
| Single Voltage Mode(Vcore = Vi/o)* | 2-3 closed | 5-6 closed |

The settings of JP5, JP6 and JP7 are depending on the CPU host clock. Before you do the following CPU settings, please set JP5, JP6 and JP7 to “Pin1-2, Pin2-3 and Pin1-2“ position for PC-100 SDRAM module or set JP5, JP6 and JP7 to “Pin2-3, Pin1-2 and Pin1-2“ position for non PC-100 SDRAM module or EDO DIMM module.

| ###### CPU host clock/DRAM/PCI clock | JP 5 | JP 6 | JP 7 |
| --- | --- | --- | --- |
| 100MHz/100MHz/33MHz* | Pin 1-2 | Pin 2-3 | Pin1-2 |
| 100MHz/66MHz/33MHz** | Pin 2-3 | Pin 1-2 | Pin 1-2 |

*	Please use for PC-100 SDRAM module

**	Please use non PC-100 SDRAM module or EDO DIMM module

IDT Win Chip 2 300MHz

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 300MHz | ON OFF 1 2 3 4 5 6 | 100 x 2.5 |

2-3 SYSTEM MEMORY INSTALLATION

The ATC-5220 provides three 168-pin DIMM sockets for system memory expansion from 8MB to 768MB. These three DIMMs are arranged to three banks, please refer to page A. Each bank provides 64-bit wide data path.

※ Samples of System Memory Combinations Options ※

| BANK0 | BANK1 | BANK2 | Total Memory |
| --- | --- | --- | --- |
| DIMM 1 | DIMM 2 | DIMM 3 | DIMM 1-3 |
|  | 16MBx1 | 16MBx1 | 32MB |
| 16MBx1 | - | 16MBx1 | 32MB |
| 32MBx1 | - | - | 32MB |
| - | 32MBx1 | - | 32MB |
| - | - | 32MBx1 | 32MB |
| 8MBx1 | 16MBx1 | 16MBx1 | 40MB |
| 32MBx1 | 32MBx1 | - | 64MB |
| - | 32MBx1 | 32MBx1 | 64MB |
| 64MBx1 | - | - | 64MB |
| - | 64MBx1 | - | 64MB |
| : | : | : | : |
| - | 64MBx1 | 64MBx1 | 128MB |
| 128MBx1 | - | - | 128MB |
| - | 128MBx1 | - | 128MB |
| - | - | 128MBx1 | 128MB |
| 128MBx1 | 128MBx1 | - | 256MB |
| 128MBx1 | - | 128MBx1 | 256MB |
| - | 128MBx1 | 128MBx1 | 256MB |
| 256MBx1 | 256MBx1 | - | 512MB |
| 256MBx1 | - | 256MBx1 | 512MB |
| - | 256MBx1 | 256MBx1 | 512MB |
| 256MBx1 | 256MBx1 | 256MBx1 | 768MB |

2-4 CONNECTORS  DESCRIPTION

The locations of following connectors are indicated in page A. When you plug wires into the following connector of CONN1, you should have the pin 1 edge of the wires align with the pin 1 end of the connector.

CONN1 : speaker, keyboard lock, reset, SMI,

turbo LED, and IDE LED connectors.

SPK    PWR LED

RST     SMI  TB-LED IDE-LED

+       +

SPK : speaker

Speaker

GND

GND

VCC

Power LED connector

Power LED +

N/C

GND (power)

Keylock

GND

PS ON

RST : Reset connector

Reset Signal

GND

#### Socket 7

SMI : SMI lead

GND

SMI Signal

TB-LED : Turbo LED indicator, LED on

when system runs higher speed.

GND

+5V

IDE-LED : IDE devices indicator LED

connector. IDE-LED stays ON indicates

LED signal  on-board IDE devices in operation. The

+5V        red wire of the HDD connector must be

connected to the pin which is +5V.

Pin 12                 PS_ON : Power Button

Pin 24                 Pin 12 : PS_ON      Pin 24 : +5VSB

SOCKET 7

FAN1 : CPU cooling fan connector. Wire with

+12V voltage (most likely red wire) must be

plugged into pin2, and GROUND wires

(most likely black wire) must be plugged into

pin1. Please confirm the wire color

re-presentation with your supplier.

CAUTION: Plug the wire into wrong connector will

FAN 1

DAMAGE fan and mainboard.

1      GND

- +12V

IR1 : Infrared module connector.

1 	+5V

- FIRRX

- IRRX

4	GND

5	IRTX

Socket 7

JP 11

1  2  3    Pin 1-2 : Interlave mode

Pin 2-3 : Linear Burst

#### Socket 7    JP 11

10    20       PW1 : ATX mode +3.3/5/12V power supply

connector.

| 1 | 3.3V | 6 | +5V | 11 | 3.3V | 16 | GND |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 3.3V | 7 | GND | 12 | -12V | 17 | GND |
| 3 | GND | 8 | PWRGD | 13 | GND | 18 | -5V |
| 4 | +5V | 9 | 5VSB | 14* | PS_ON | 19 | +5V |
| 5 | GND | 10 | +12V | 15 | GND | 20 | +5V |

* PS_ON : Soft-Off power control

1    11

COM1/COM2 : these two connectors are

used to connect serial port cables.

Socket 7

| pin 1 6 2 7 3 8 4 9 5 | Signal name |
| --- | --- |
| 1 | NDCDA/B |
| 2 | NSINA/B |
| 3 | NSOUTA/B |
| 4 | NDTRA/B |
| 5 | GND |
| 6 | NDSRA/B |
| 7 | NRTSA/B |
| 8 | NCTSA/B |
| 9 | NRIA/B |

A is COM1, B is COM2

Socket 7

FDC1 : this connector is used to connect the floppy

drive through a cable.

| pin | signal | pin | signal |
| --- | --- | --- | --- |
| 2 | RWC- | 20 | STEP- |
| 4 | Reserved | 22 | Write Data |
| 6 34 33 2 1 | FDEDIN | 24 | Write Gate |
| 8 | Index- | 26 | Track 00- |
| 10 | Motor EnableA- | 28 | Write Protect- |
| 12 | Drive Sele.B- | 30 | Read Data- |
| 14 | Drive Sele.A- | 32 | Side 1 Sele.- |
| 16 | Motor EnableB- | 34 | DisketteChange |
| 18 | DIR- |  |  |
| All of odd pins are ground |  |  |  |

Socket 7    LPT : this connector is used to connect parallel

port cable.

| Pin | Signal | pin | Signal |
| --- | --- | --- | --- |
| 1 | STROBE- | 10 | ACK- |
| 2 | Data Bit 0 | 11 | BUSY |
| 3 | Data Bit 1 | 12 | PE |
| 4 25 13 13 1 | Data Bit 2 | 13 | SLCT |
| 5 | Data Bit 3 | 14 | Auto Feed- |
| 6 | Data Bit 4 | 15 | ERROR- |
| 7 | Data Bit 5 | 16 | INIT- |
| 8 | Data Bit 6 | 17 | SLCT IN- |
| 9 | Data Bit 7 |  |  |
| ###### Pin18 -- pin25 are GND |  |  |  |

LPT1  COM1  COM2

PS/2 Mouse

Keyboard conn.

USB 1   USB 0

IDE1/IDE2 : these two connectors are used to

connect IDE devices through IDE cables,

a total of 4 devices can be connected.

40      39

4        3

2	1

| pin | signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Reset IDE | 21 | DDRQ0(1) |
| 2 | GND | 22 | GND |
| 3 | Host Data 7 | 23 | I/O Write- |
| 4 | Host Data 8 | 24 | GND |
| 5 | Host Data 6 | 25 | I/O Read- |
| 6 | Host Data 9 | 26 | GND |
| 7 | Host Data 5 | 27 | IORDY |
| 8 | Host Data 10 | 28 | N/C |
| 9 | Host Data 4 | 29 | DDAK0- (1-) |
| 10 | Host Data 11 | 30 | GND |
| 11 | Host Data 3 | 31 | IRQ14* |
| 12 | Host Data 12 | 32 | IOCS16- |
| 13 | ###### Host Data 2 | 33 | Addr 1 |
| 14 | Host Data 13 | 34 | N/C |
| 15 Socket 7 | Host Data 1 | 35 | Addr 0 |
| 16 | Host Data 14 | 36 | Addr 2 |
| 17 | Host Data 0 | 37 | ChipSele.1P- |
| 18 | Host Data 15 | 38 | ChipSele.3P- |
| 19 | GND | 39 | Activity |
| 20 | Key | 40 | GND |

* IDE1 : pin31 is IRQ14;

IDE2 : pin31 is IRQ15 or MIRQ0

2-5 IDE DRIVER INSTALLATION

The IDE driver installation procedures are in the following :

Setup from Windows 95/98 :

1. Starting Windows 95/98.

2. Put All-In-One CD into your CD-ROM drive.

3. In “My Computer” Windows, double clicking “VIA” icon.

4. Choose “IDE driver”.

5. Follow the screen instructions to complete the installation.

Setup from WinNT :

- Starting WinNT.

- Put All-In-One CD into your CD-ROM drive.

- Choose “VIA MVP3 Integrated Installation”.

- Choose “IDE driver”.

- Follow the screen instructions to complete the installation.

- In “control panel” Windows, click “SCSI Adapters” twice.

- In “SCSI Adapters” Windows, choose “Drivers”.

- Choose “Add”.

- In manufacturers, select “additional models”, then in SCSI adapter, select “VIA bus master IDE drivers”, then choose “OK”.

- Restart your computer.

2-6 VIA MVP3 AGP VGA Driver Installation

1. Put All-In-One CD into your CD-ROM drive.

- In “My computer” Windows, choose VxD driver.

- Follow the screen instructions to complete the installation.

Remark :

Please install two VIA utilities from our CD, “ VIA Routing Utility “ & “ VIA MVP3 ACPI Compliant utility ”, when you install Windows95/98 on your system, in order to get best compatibility.

CHAPTER 3  Award BIOS SETUP

Award BIOS manufacturer provides access to the system BIOS through the hardware and software on each ATC-5220 mainboard. The hardware consists of a Flash ROM and the software is a group of programs that are installed in the ROMBIOS along with all the other data the BIOS must contain.

The ATC-5220 mainboard will require special driver supplied by the manufacturer to update the BIOS SETUP program. It is a good idea to read the next page for details for update BIOS driver installation or you can ask your system dealer to do it for you.

When the driver has been successfully updated, it is very important to contact your system dealer to change the CMOS settings for your computer. The CMOS settings are shown in the following pages.

NOTE : To clear CMOS you should unplug the power cord, then set 2-3 to clear, put it back to normal position and plug the power cord again.

|  | JP8 |
| --- | --- |
| Normal | 1-2 |
| Clear | 2-3 |

Normal CMOS              Clear CMOS

ISA slots  SOCKET 7  JP8

3-1 UPDATE BIOS PROCEDURE

If the BIOS needs to be updated, you can get a diskette with the updated BIOS driver from your system supplier or from your All-In-One CD(in “flash” directory). The BIOS updated driver includes :

“awdflash.exe”  --  BIOS update utility program

“awdflash.doc”

“(update BIOS filename with version number).bin”

The update procedure is in the following:

- Boot the system to DOS mode in a normal manner.

- Insert the updated diskette to drive A (or B), or put All-In-One CD into your CD-ROM drive.

- Change working directory to floppy drive, A or B, or change working directory to your CD-ROM drive, D or E, which contains the update BIOS driver. --  Type “a:\” or “b:\”, “ENTER” or “d:\” or “e:\”, “ENTER”.

- Run the BIOS update utility  --  Type “cd flash”, then press “ENTER“ key. After that , type “awdflash”, then press “ENTER” key.

- Type “(update BIOS file name with version number).bin”, ENTER.

- If you do not want to save the old BIOS Type “N” when the screen displays the message : " Do you want to save BIOS (Y/N) ?".

- Type “Y“ when the screen shows the message : " Are you sure to program (Y/N) ?".

- Follow instructions displayed on the screen.  DO NOT remove the update BIOS diskette from the floppy drive nor turn the system power off until the BIOS update is completed.

- Turn the power off. Clear the data in CMOS according to the procedure described in the previous page.

- Turn the system power on and test that your system is working properly.

3-2  Award SYSTEM BIOS

CONFIGURATION SETUP

The following pages explain how to set up the system configuration (CMOS) under the Award BIOS. The SETUP program is stored in the Read-Only-Memory (ROM) on the mainboard. To do the SETUP procedure, press the <Del> key when the system is booting up. The following main menu will appear. Please select " STANDARD CMOS SETUP" to enter the next screen.

ROM PCI/ISA BIOS (ATC-5220)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | âàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

The section on the bottom of the main menu explains how to control this screen.  The other section displays the items highlighted in the list.

This screen records some basic hardware information, and sets the system clock and error handling. These records can be lost or corrupted if the on-board battery has failed or is weak.

ROM PCI/ISA BIOS (ATC-5220)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (ATC-5220)

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| Date (mm:dd:yy) : Wed, June 05 1998 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time(hh:mm:ss) : 13 : 37 : 14 |  |  |  |  |  |  |  |  |
| HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |  |  |  |  |  |  |
| Primary Master : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Primary Slave : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Secondary Master : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Secondary Slave : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Drive A : 1.44M, 3.5 in. |  |  |  |  |  |  |  |  |
| Drive B : None |  |  | Base Memory : 640K |  |  |  |  |  |
| Floppy 3 Mode Support : Disabled |  |  | Extended memory : 7168K |  |  |  |  |  |
|  |  |  | Other Memory : 384K |  |  |  |  |  |
| Video : EGA/VGA |  |  | ------------------------------------------------ |  |  |  |  |  |
| Halt On: All Errors |  |  | Total Memory : 8192K |  |  |  |  |  |
| ESC : Quit áâàß:Select Item PU/PD/+/- : Modify |  |  |  |  |  |  |  |  |
| F1 : Help (Shift) F2 : Change Color |  |  |  |  |  |  |  |  |

Date	The date format is <day>, <date><month><year>.

Press<F3> to show the calendar.

| Day | The day, from Sun to Sat, determined by the BIOS and is display-only |
| --- | --- |
| Date | The date, from 1 to 31 |
| Month | The month, Jan. through Dec. |
| Year | The year, from 1900 to 2099 |

Time	The time format is <hour><minute><second>. The time is

calculated based on the 24-hour military-time clock.

For example, 1p.m. is 13:00:00.

Primary Master	These categories identify the types of the 2 channels

Primary Slave	that have been installed in the computer. There

Secondary Master	are 45 predefined types and 4 user definable

Secondary Slave	types are for Enhanced IDE BIOS. Type 1 to 45

are predefined. Type ‘user’ is user-definable.

Press PgUp/PgDn to select a numbered hard

disk type or type the number and press<Enter>.

If you select ‘Auto’, the BIOS will auto-detect the

HDD & CD-ROM Drive at the POST stage and

show the IDE for HDD & CD-ROM Drive. If

you select ‘user’, you will need to know the

information listed below. Enter the information 						directly from the keyboard and press <Enter>.

This information should be from your hard disk

vender or dealer. If the controller of the HDD

interface is ESDI, the selection shall be ‘Type 1’;

if SCSI, the selection shall be ‘None’.

If no device is installed select

‘NONE’ and press <Enter>.

| Type | drive type |
| --- | --- |
| SIZE | Automatically adjusts |
| CYLS | number of cylinders |
| HEAD | number of heads |
| PRECOMP | write precom |
| LANDZ | landing zone |
| SECTOR | number of sectors |
| MODE | mode type |

Drive A	This category identifies the types of floppy disk drive

Drive B	A or drive B that have been installed in the computer.

| None | No floppy drive installed |
| --- | --- |
| 360K, 5.25 in | 5.25“ PC-type 360KB capacity |
| 1.2M, 5.25 in | 5.25“ AT-type 1.2MB capacity |
| 720K, 3.5 in | 3.5“ double-side 720KB capacity |
| 1.44M, 3.5 in | 3.5“ double-side 1.44MB capacity |
| 2.88M, 3.5 in | 3.5“ double-side 2.88MB capacity |

Floppy 3 Mode	This is the Japanese standard floppy drive. This standard

Support	stores 1.2MB in a 3.5” diskette

Video	This category selects the type of video adapter used for the

primary system monitor. Although secondary monitors are

supported, you do not have to select the type in Setup.

| EGA/VGA | Enhanced Graphics Adapter/Video Graphics Array. For EGA, VGA, SEGA, SVGA or PGA monitor adapters |
| --- | --- |
| CGA 40 | Color Graphics Adapters, power up in 40 column mode |
| CGA 80 | Color Graphics Adapters, power up in 80 column mode |
| MONO | Monochrome adapter, includes high resolution monochrome adapters |

Halt On	This category determines whether the computer

will stop if an error is detected during power up.

| No errors | The system boot will not be stopped for any error that may be detected |
| --- | --- |
| All errors | When the BIOS detects a non-fatal error the system will be stopped and you will be prompted |
| All, But Keyboard | The system boot will not stop for a keyboard error, it will stop for all other errors |
| All, But Diskette | The system boot will not stop for a disk error, it will stop for all other errors |
| All, But Disk/Key | The system boot will not stop for a disk or keyboard error, it will stop for all other errors |

Memory	          This category is display-only which is determined by POST

(Power On Self Test) of the BIOS.

Base Memory  The POST will determine the amount of base

(or conventional) memory installed in the system. The value of

the base memory is typically 512K or 640K based on the

memory installed on the motherboard.

Extended Memory  How much extended memory is present

during the POST. This is the amount of memory located above

1MB in the CPU‘s memory address map.

Other Memory  This refers to the memory located in the 640K

to 1024K address space. This is memory that can be used for

different applications. DOS users this area to load device drivers

in an effort to keep as much base memory free for application

programs. The BIOS is the most frequent user of this RAM area

since this is where it shadows RAM.

This screen is a list of system configuration options. Some of them are defaults required by the mainboard's design, others depend on the features of your system.

ROM PCI/ISA BIOS (ATC-5220)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Virus, Protection, Boot Sequence |  |

ROM PCI/ISA BIOS (ATC-5220)

BIOS FEATURES SETUP

AWARD SOFTWARE, INC.

| Virus Warning | : Disabled | Video BIOS Shadow | : Enabled |
| --- | --- | --- | --- |
| CPU Internal Cache | : Enabled | C8000-CBFFF Shadow | : Disabled |
| External Cache | : Enabled | CC000-CFFFF Shadow | : Disabled |
| Quick Power On Self Test | : Enabled | D0000-D3FFF Shadow | : Disabled |
| Boot Sequence | : A,C,SCSI | D4000-D7FFF Shadow | : Disabled |
| Swap Floppy Drive | : Disabled | D8000-DBFFF Shadow | : Disabled |
| Boot Up Floppy Seek | : Enabled | DC000-DFFFF Shadow | : Disabled |
| Boot Up NumLock Status | : On |  |  |
| Gate A20 Option | : Fast |  |  |
| Memory Parity/ECC Check | : Enabled |  |  |
| Typematic Rate Setting | : Disabled |  |  |
| Typematic Rate(Chars/Sec) | : 6 |  |  |
| Typematic Delay(Msec) | : 250 | Esc : Quit áâàß:Select Item |  |
| Security Option | : Setup | F1 : Help PU/PD/+/- : Modify |  |
| PCI/VGA Palette Snoop | : Disabled | F5 : Old Values (SHIFT) F2 : Color |  |
| OS Select for DRAM>64MB | : Non-OS2 | F6 : Load BIOS Defaults |  |
| Report No FDD For Win95 | : No | F7 : Load Setup Defaults |  |

Virus Warning	When this item is enabled, the Award BIOS will monitor the

boot sector and partition table of the hard disk drive for any

attempt at modification. If an attempt is made, the BIOS will

halt the system and the following error message will appear.

Afterwards, if necessary, you will be able to run an antivirus

program to locate and remove the problem before any damage

is done.

! WARNING ! Disk boot sector is to be modified Type ‘Y’ to accept write or ‘N’ to abort write Award Software, Inc.

| Enabled | Activates automatically when the system boots up, if anything attempts to access the boot sector or hard disk partition table will cause a warning message to appear. |
| --- | --- |
| Disabled | No warning message will appear when anything attempts to access the boot sector or hard disk partition table. |

Many disk diagnostic programs which attempt to

access the boot sector table can cause the above warning

message. If you will be running such a program, we

recommend that you first disable Virus Protection beforehand.

CPU	These two categories speed up memory access.

Internal Cache	However, it depends on CPU/chipset design.

External Cache	The default value is ‘enabled‘.

Quick Power On	This category speeds up Power On Self Test

Self Test	after you power up the computer. If you set 							Enabled, BIOS will shorten or skip some checked

items during POST.

Boot Sequence	This category determines which drive is to search

first for the Disk Operating System (i.e., DOS).

| #### A, C, SCSI | System will first search for floppy disk drive then hard disk drive, and the next is SCSI device. |
| --- | --- |
| #### C, A, SCSI | System will first search for hard disk drive then floppy disk drive, and the next is SCSI device. |
| #### C, CDROM, A | System will first search for hard disk drive then CDROM drive, and the next is floppy disk drive. |
| #### CDROM, C, A | System will first search for CDROM drive then hard disk drive, and the next is floppy disk drive. |
| #### D, A, SCSI | System will first search for secondary hard disk drive then floppy disk drive, and the next is SCSI device. |
| #### E, A, SCSI | System will first search for third hard disk drive then floppy disk drive, and the next is SCSI device. |
| #### F, A, SCSI | System will first search for fourth hard disk drive then floppy disk drive, and the next is SCSI device. |
| #### SCSI, A, C | System will first search for SCSI device then floppy disk drive, and the next is hard disk drive. |
| #### SCSI, C, A | System will first search for SCSI device then hard disk drive, and the next is floppy disk drive. |
| #### C only | System will search for hard disk drive only. |
| #### LS/ZIP, C | System will first search for LS120 or IOMEGA (ZIP) drive, and the next is hard disk drive. |

C is primary master; D is primary slave;

E is secondary master, F is secondary slave

Swap Floppy	This item allows you to determine whether to

Drive 	enable the swap floppy drive or not.

The choice : Enabled/ Disabled

Boot Up Floppy 	During POST, the BIOS will determine if the floppy

Seek	disk drive installed is 40 tracks (360K) or 80

tracks (720K, 1.2M, 1.44M)

| Enabled | BIOS searches for floppy disk drive to determine if it is 40 or 80 tracks |
| --- | --- |
| Disabled | BIOS will not search for the type of floppy disk drive by track number |

Boot Up 	This allows you to determine the default state of

NumLock Status	the numeric keypad. By default, the system

boots up with NumLock on.

| On | Keypad is numeric keys |
| --- | --- |
| Off | Keypad is arrow keys |

Gate A20	This entry allows you to select how the gate A20 is handled.

Option	The gate A20 is a device used to address memory above

1 MB. Initially, the gate A20 was handled via a pin on the

keyboard. Today, while keyboards still provide this support, it

is more common, and much faster, for the system chipset to

provide support for gate A20.

Normal is keyboard; Fast is chipset.

Memory Parity/     Select parity, ECC, or Disabled, depending on the type of

ECC Check        DRAM installed in your system.

The choice : ECC, Parity, Disabled

Typematic Rate	This determines if the typematic rate is to be used. When disabled,

Setting	continually holding down a key on your keyboard will generate

only one key instance. In other words, the BIOS will only report

that the key is down. When the typematic rate is enabled, the

BIOS will report as before, but it will then wait a moment, and, if

the key is still down, it will begin the report that the key has been

depressed repeatedly. For example, you would use such a feature

to accelerate cursor movements with the arrow keys.

The choice : Enabled/Disabled

Typematic Rate 	When the typematic rate is enabled, this section

(Chars/Sec)	allows you select the rate at which the keys are repeated.

| 6 | 6 characters per second |
| --- | --- |
| 8 | 8 characters per second |
| 10 | 10 characters per second |
| 12 | 12 characters per second |
| 15 | 15 characters per second |
| 20 | 20 characters per second |
| 24 | 24 characters per second |
| 30 | 30 characters per second |

Typematic Delay 	When the typematic rate is enabled, this section

(Msec)	allows you select the delay between when the

key was first depressed and when the

acceleration begins.

| 250 | 250 msec |
| --- | --- |
| 500 | 500 msec |
| 750 | 750 msec |
| 1000 | 1000 msec |

Security Option 	This category allows you to limit access to the

system and Setup, or just to Setup.

| System | The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt |
| --- | --- |
| Setup | The system will boot, but access to Setup will be denied if the correct password is not entered at the prompt |

To disable security, select PASSWORD SETTING

at Main Menu and then you will be asked to enter

password. Do not type anything and just press

<Enter>, it will disable security. Once the security

is disabled, the system will boot and you can enter 						Setup freely.

PCI/VGA Palette   It determines whether the MPEG ISA/VESA VGA

Snoop            cards can work with PCI/VGA or not.

| ###### Enabled | When PCI/VGA working with MPEG ISA/VESA VGA Card |
| --- | --- |
| Disabled | When PCI/VGA not working with MPEG ISA/ VESA VGA Card |

OS Select for 	This item allows you to access the memory that is over

DRAM > 64MB	64MB in OS/2.

The choice : Non-OS2 or OS2

Report No FDD 	Set this item to Yes BIOS will report FDD to Win95.

For WIN 95	If in standard CMOS setup, set Drive A to none, and set

this item to yes. Inside Win95, My Computer and File

manager Disk(A:) will show Removable Disk (A:).

Video BIOS	Determines whether video BIOS will be copied to RAM.

Shadow	However it is optional depending on chipset design.

Video Shadow will increase the video speed.

The choice : Enabled/Disabled

C8000 - CBFFF 	These categories determine whether option ROMs will

Shadow	be copied to RAM. An example of such option ROM

DC000 - DFFFF	would be the support of onboard SCSI.

Shadow 	The choice : Enabled/Disabled

This screen controls the setting for the chipset on the mainboard.

ROM PCI/ISA BIOS (ATC-5220)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| AT Clock, DRAM Timings, ....... |  |

ROM PCI/ISA BIOS (ATC-5220)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Bank 0/1 DRAM Timing | : FP/EDO 60ns | OnChip USB |  | : Disabled |
| --- | --- | --- | --- | --- |
| Bank 2/3 DRAM Timing | : FP/EDO 60ns | Auto Detect DIMM/PCI ClK |  | : Enabled |
| Bank 4/5 DRAM Timing | : FP/EDO 60ns | Spread Spectrum Modulatd |  | : Disabled |
| SRAM Cycle Length | : 3 |  |  |  |
| DRAM Read Pipeline | : Disabled |  |  |  |
| Sustained 3T Write | : Enabled |  |  |  |
| Cache Rd+CPU Wt Pipeline | : Disabled |  |  |  |
| Cache Timing | : Fast |  |  |  |
| Video BIOS Cacheable | : Disabled |  |  |  |
| System BIOS Cacheable | : Disabled |  |  |  |
| Memory Hole At 15Mb Addr. | : Disabled |  |  |  |
| AGP Aperture Size (MB) | : 128MB |  |  |  |
|  |  | Esc: Quit | :Select Item |  |
|  |  | F1 : Help | PU/PD/+/-:Modify |  |
|  |  | F5 : Old Values | (Shift)F2 :Color |  |
|  |  | F6 :Load BIOS Defaults |  |  |
|  |  | F7 : Load Setup Defaults |  |  |

Bank 0/1/2/3/4/5    The DRAM speed is controlled by the DRAM timing

DRAM Timing      registers. The programmed into this register are depend on

the system design. Slower rates may be required in certain

system designs to support loose layouts or slower memory.

i.e. 60ns; 70ns

SDRAM Cycle      You should select CAS latency time in HCLKS of 2/2 or 3/3.

Length            The system board designer should set the values in this field,

depending on the DRAM installed. Do not change the values

in this field unless you change specifications of the installed

DRAM or the installed CPU..

The choice : 2, .3

DRAM Read       Enable/Disable DRAM Read Pipeline Cycle.

Pipeline

Sustained 3T       Enabled : set cache to write back mode.

Write             Disabled : set cache to write through mode.

Cache Rd+CPU     Enable/Disable Cache Read Write cycle.

Wt Pipeline

Cache Timing      Fastest : better system performance will occur.

Video BIOS 	Select Enabled allows caching of the video BIOS ROM at

Cacheable 	F0000h-FFFFFh, resulting in better system performance.

However, if any program writes to this memory area, a system

error may result.

System BIOS	Select Enabled allows caching of the system BIOS ROM at

Cacheable 	F0000h-FFFFFh, resulting in better system performance.

However, if any program writes to this memory area, a system

error may result.

Memory Hole At	In order to improve performance, certain space

15Mb Addr.	in memory can be reserved for ISA cards. This memory

must be mapped into the memory below 16MB.

AGP Aperture	Select the size of the AGP aperture. The aperture is a portion of

Size (MB) 	the PCI memory address range dedicated for graphics memory

address space. Host cycle that hit the aperture range are

forwarded to the AGP without any translation. See

www.agpforum.org for AGP information.

Onchip USB       Enable/Disable USB.

Auto Detect        If this item is enabled, the unused DIMM and PCI slot clock

DIMM/PCI ClK     will be disabled. If this item is disabled the unused DIMM

and PCI slot will still get the active clock signal.

Spread Spectrum   Enable / Disable this item the BIOS will Enable / Disable

Modulated        the clock generator spread spectrum .

This screen controls the 'green' features of this mainboard.

ROM PCI/ISA BIOS (ATC-5220)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Sleep Timer, Suspend Timer, ..... |  |

ROM PCI/ISA BIOS (ATC-5220)

POWER MANAGEMENT SETUP

AWARD SOFTWARE, INC.

| Power Management | : User Defined |  | Primary INTR : ON |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PM Control by APM | : Yes |  | IRQ3 (COM2) |  |  |  | : Primary |  |
| Video Off Option | : Suspend ->OFF |  | IRQ4 (COM1) |  |  |  | : Primary |  |
| Video Off Method | : DPMS |  | IRQ5 (LPT 2) |  |  |  | : Primary |  |
| Modem Use IRQ | : 3 |  | IRQ6 (Floppy Disk) |  |  |  | : Primary |  |
| Soft-Off by PWRBTN | : Instant-Off |  | IRQ7 (LPT 1) |  |  |  | : Primary |  |
| ** PM Timers | ** |  | IRQ8 (RTC Alarm) |  |  |  | : Disabled |  |
| HDD Power Down | : Disabled |  | IRQ9 (IRQ2 Redir) |  |  |  | : Primary |  |
| Doze Mode | : Disabled |  | IRQ10 (Reserved) |  |  |  | : Primary |  |
| Suspend Mode | : Disabled |  | IRQ11 (Reserved) |  |  |  | : Primary |  |
| ** PM Events | ** |  | IRQ12 (PS/2 Mouse) |  |  |  | : Primary |  |
| VGA | : OFF |  | IRQ13 (Coprocessor) |  |  |  | : Disabled |  |
| LPT & COM | : LPT/COM |  | IRQ14 (Hard Disk) |  |  |  | : Primary |  |
| HDD & FDD | : ON |  | IRQ15 (Reserved) |  |  |  | : Disabled |  |
| DMA/master | : OFF |  | Esc: Quit áâàß |  |  | :Select Item |  |  |
| Modem Ring Resume | : Disabled |  | F1 : Help | PU/PD/+/- : Modify |  |  |  |  |
| RTC Alarm Resume | : Disabled |  | F5 : Old Values |  | (Shift) F2: Color |  |  |  |
|  |  |  | F6 : Load BIOS Defaults |  |  |  |  |  |
|  |  |  | F7 : Load Setup Defaults |  |  |  |  |  |

Power 	This category allows you to select the type (or

Management	degree) of power saving and is directly related to

the following modes : Doze; Standby; Suspend;

HDD Power Down.

| Min. Power Saving | Minimum power management. Doze =1hr.; Standby=1hr.; Suspend=1hr.; HDD Power Down=15min |
| --- | --- |
| Max. Power Saving | Maximum power management only available for SL CPU.Doze=1min.; Standby=1min.;Suspend=1min.;HDD Power Down=1min |
| User Defined | Allows you to set each mode individually. When not disabled, each of the ranges are from 1min. to 1hr. except for HDD Power Down which ranges from 1 to 15min. and disable |

If you would like to use Software Power-off Control

function, you cannot choose“ Disabled ”here, and

should select “Yes” in PM Control by APM.

PM Control 	When enabled, an Advanced Power Management 	device

by APM	will be activated to enhance the Max. Power Saving

Mode and stop the CPU internal clock. If the Max.

Power Saving is not enabled, this will be shown as NO.

Video Off         When enabled, this feature allows the VGA adapter to operate

Option           in a power saving mode.

| Always On | Monitor will remain on during power saving modes. |
| --- | --- |
| Suspend | Monitor blanked when the systems enters the Suspend mode. |
| Susp, Stby | Monitor blanked when the system enters Suspend or Standby mode. |
| All Modes | Monitor blanked when the system enters any power saving mode. |

Video Off	This determines the manner in which the monitor is blanked.

Method

| V/H SYNC + Blank | This selection will cause the system to turn off the vertical and horizontal sync. ports and write blanks to the video buffer |
| --- | --- |
| Blank Screen | This option only writes blanks to the video buffer |
| DPMS | Initial display power management signaling |

MODEM Use IRQ 		This item determines the IRQ in which the MODEM can be used.

The choice : 3,4,5,7,9,10,11,NA.

Soft-Off by         Instant-off : When push the power button, the system power will

PWR-BTTN	be off immediately. Delay 4 sec : when push the power button, it

will enter suspend mode. We need to push the power button and

hold for 4 seconds to turn off the power.

The Following 4 modes are Green PC power saving functions which are only user configurable when ‘User Defined’ power management has been selected.

HDD Power Down	When enabled and after the set time of system inactivity, the

hard disk drive will be powered down while all other devices

remain active.

Doze Mode	When enabled and after the set time of system inactivity, the

CPU clock will run at slower speed while all other devices

still operate at full speed.

Suspend Mode	When enabled and after the set time of system inactivity, all

devices except the CPU will be shut off.

VGA/LPT & COM  These are I/O events whose occurrence can prevent the system

HDD & FDD/      from entering a power saving mode or can awaken the system

DMA/master       from such a mode. In effect, the system remains alert for anything

which occurs on a device which is configured as on, even when

the system is in a password down mode. When an I/O device

wants to gain the attention of the operating system, it signals this

by causing an IRQ(Interrupt ReQuest) to occur. When the

operating system is ready to respond to the request, it interrupts

itself and performs the service. When set to off, activity will

neither prevent the system from going into a power management

mode nor awaken it.

Modem Ring         Enabled : when system in suspend mode, it can be wake up

Resume             by modem.

Disabled : it cannot be wake up by modem.

RTC Alarm          When enabled, two additional lines will added to the screen :

Resume             Date (of Month) Alarm; Time (hh:mm:ss) Alarm to let user set

The desired date and time. After power off, the system will

automatic power on at the specified date and time.

Primary INTR        It enables/disables the IRQ3 to IRQ15 PM events.

This screen configures the PCI Bus slots.

ROM PCI/ISA BIOS (ATC-5220)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| IRQ Settings, Latency Timers, ..... |  |

ROM PCI/ISA BIOS (ATC-5220)

PNP/PCI CONFIGURATION

AWARD SOFTWARE, INC.

| PNP OS Installed | : No | CPU to PCI Write Buffer |  |  |  | : Enabled |
| --- | --- | --- | --- | --- | --- | --- |
| Resources Controlled by | : Auto | PCI Dynamic Bursting |  |  |  | : Disabled |
| Reset Configuration Data | : Disabled | PCI Master 0 WS Write |  |  |  | : Enabled |
| ACPI I/O Device Node | : Enabled | PCI Delay Transaction |  |  |  | : Disabled |
|  |  | PCI Master Read Prefetch |  |  |  | : Disabled |
|  |  | PCI#2 Access #1 Retry |  |  |  | : Disabled |
|  |  | AGP Master 1 WS Write |  |  |  | : Disabled |
|  |  | AGP Master 1 WS Read |  |  |  | : Disabled |
|  |  | PCI IDE IRQ Map To |  |  |  | : PCI-AUTO |
|  |  | Primary IDE INT# |  |  |  | : A |
|  |  | Primary IDE INT# |  |  |  | : B |
|  |  | Assign IRQ For USB |  |  |  | : Disabled |
|  |  | Assign IRQ For VGA |  |  |  | : Enabled |
|  |  | Esc: Quit áâàß |  |  | :Select Item |  |
|  |  | F1 : Help | PU/PD/+/- : Modify |  |  |  |
|  |  | F5 : Old Values |  | (Shift) F2: Color |  |  |
|  |  | F6 : Load BIOS Defaults |  |  |  |  |
|  |  | F7 : Load Setup Defaults |  |  |  |  |

PNP OS Installed	This item allows you to determine PnP OS or not.

Choices are Yes or No.

Resources	The Award Plug and Play BIOS has the capability to

Controlled By	automatically configure all of the boot and Plug and Play

compatible devices. However, this capability means absolutely

nothing unless you are using a Plug and Play OS such as

Windows 95. Choices are Auto and Manual.

Reset Config-	This item allows you to determine whether to reset the

uration Data	configuration data or not.

ACPI I/O Device    Enable : reserve a node in memory for ACPI.

Node

CPU to PCI        Enable/Disable CPU to PCI POST Write.

Write Buffer

PCI Dynamic      Enable/Disable PCI burst operation.

Bursting

PCI Master        Enable : PCI Master 0 wait state mode.

0 WS Write        Disable : PCI Master 1 wait state mode.

PCI Delay         This chipset has an embedded 32-bit posted write buffer to support

Transaction        deadly transactions cycles. Select “enabled” to support compliance

with PCI specification version 2.1.

## The choice : Enabled, disabled space

PCI Master        Enable : always prefetch

Read Prefetch      Disable : prefetch only if enhance command

PCI#2 Access      Disable : PCI#2 will be disconnected until access finished

#1 Retry          Enable : PCI#2 will be disconnected if max. retries are attempted

without success.

AGP Master       Enable/Disable AGP master one wait state write.

1 WS Write

AGP Master       Enable/Disable AGP master one wait state read.

1 WS Read

PCI IDE IRQ	This allows you to configure your system to the type of IDE disk

Map To	controller in use. By default, Setup assumes that your controller is

Primary IDE INT#		an ISA device rather than a PCI controller. The most apparent

Secondary IDE		difference is the type of slot being used. If you have equipped your

INT#		system with a PCI controller, changing this allows you to specify

which slot holds the controller and which PCI interrupt (A,B,C,D)

is associated with the connected hard disk. This setting refers to the

hard disk drive itself, rather than individual partitions. Since each

IDE controller supports two separate hard drives, you can select the

INT# for each. Again, you will note that the primary has a lower

interrupt than the secondary as described in “ lot x Using INT#”

above. Select ‘PCI Auto’ allows the system to automatically

determine how your IDE disk system is configured.

Assign IRQ for     When this items is enabled, the system will assign an IRQ for

USB              USB. If this item is disabled, the USB will not occupy an

IRQ; therefore the IRQ of USB will be released for other

usage.

Assign IRQ for     When this items is enabled, the system will assign an IRQ for

VGA              VGA. If this item is disabled, the VGA will not occupy an

IRQ; therefore the IRQ of VGA will be released for other

usage.

This section page includes all the items of IDE hard drive and Programmed

Input/Output features. See also Section “Chipset Features Setup”.

ROM PCI/ISA BIOS (ATC-5220)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (ATC-5220)

INTEGRATED PERIPHERALS

AWARD SOFTWARE, INC.

| OnChip IDE First Channel | : Enabled | Onboard Parallel Port |  |  | : 378/IRQ7 |
| --- | --- | --- | --- | --- | --- |
| Onchip IDE Second Channel | : Enabled | Onboard Parallel Mode |  |  | : ECP |
| IDE Prefetch Mode | : Enabled | ECP Mode Use DMA |  |  | : 3 |
| IDE HDD Block Mode | : Enabled |  |  |  |  |
| IDE Primary Master PIO | : Auto |  |  |  |  |
| IDE Primary Slave PIO | : Auto |  |  |  |  |
| IDE Secondary Master PIO | : Auto |  |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |  |
| IDE Primary Master UDMA | : Auto |  |  |  |  |
| IDE Primary Slave UDMA | : Auto |  |  |  |  |
| IDE Secondary Master UDMA | : Auto |  |  |  |  |
| IDE Secondary Slave UDMA | : Auto |  |  |  |  |
| Init AGP Display First | : Disabled |  |  |  |  |
| Onboard FDC Controller | : Enabled |  |  |  |  |
| Onboard UART 1 | : 3F8/IRQ4 | Esc: Quit áâàß |  |  | :Select Item |
| Onboard UART 2 | : 2F8/IRQ3 | F1 : Help | PU/PD/+/- : Modify |  |  |
| Onboard UART 2 Mode | : Standard | F5 : Old Values |  | (Shift) F2: Color |  |
|  |  | F6 : Load BIOS Defaults |  |  |  |
|  |  | F7 : Load Setup Defaults |  |  |  |

OnChip IDE First     This setup item allows you to either enable or disable

Channel             the primary/secondary controller. You might choose to

OnChip IDE Second   disable the controller if you were to add higher performance

Channel             or specialized controller.

IDE Prefetch Mode    Enable/Disable IDE Read Prefetch Buffer.

IDE HDD Block	  This allows your HD controller to use the fast block mode to

Mode	  transfer data to and from your HDD drive

| Enabled | IDE controller uses block mode |
| --- | --- |
| Disabled | IDE controller uses standard mode |

IDE Primary	PIO - Programmed Input/Output, it allows the BIOS to

Master/Slave PIO	tell the controller what it wants and then let the controller

IDE Secondary	and the CPU to complete the task by themselves.

Master/Slave PIO	This is simpler and more faster. Your system supports

five modes, 0 - 4, which primarily differ in timing. When

Auto is selected, the BIOS will select the best available mode.

IDE Primary	Auto, will support the Ultra DMA function.

Master/Slave UDMA	Disabled, will not support the Ultra DMA function.

IDE Secondary

Master/Slave UDMA

Init AGP          This item will activate the AGP in the multi-display environment

Display First       , it displayed, if disabled, and the system has both AGP and PCI

VGA card, the AGP monitor will not display.

KBC input clock	Let user change the keyboard working clock.

On Board 	This item will enable or disable the floppy disk controller.

FDC Controller

On Board 	User can select serial port IRQ. If set to 3F8/IRQ4, system will

UART Port 1       assign an IRQ for it. Note : set to Auto is not recommended.

On Board 	User can select serial port IRQ. If set to 2F8/IRQ3, system will

UART Port 2	assign an IRQ for it. Note : set to Auto is not recommended.

OnBoard UART    This lets you select the Infrared mode. Choices are Standard,

2 Mode           HPIR, and ASKIR. If you choose HPIR or ASKIR mode,

the screen will show another two lines to let you choose

‘IR Function Duplex’ (Full or Half) and‘RxD TxD Active’

(Hi Lo; Lo Hi; Hi Hi;Lo Lo).

On Board 	Let user select IRQ for parallel port, when Disabled, the parallel

Parallel Port	port will be disabled

On Board 	Let user select error check mode. This item is not recommended to

Parallel Mode	change except user has special request.

ECP Mode	 Select a DMA channel for the port.

Use DMA	 Choices are 3, 1.

ROM PCI/ISA BIOS (ATC-5220)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (ATC-5220)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| Hard Disks Type Size CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Primary Master : |  |  |  |  |
|  | Select Primary Master Option (N=Skip) : N |  |  |  |
|  |  | Options Size CYLS Head PRECOMP LANDZ Sector Mode |  |  |
|  |  | 2(Y) 1337 648 64 0 2594 63 LBA |  |  |
|  |  | 1 1339 2595 16 65535 2594 63 NORMAL |  |  |
|  |  | 3 1338 1297 32 65535 2594 63 LARGE |  |  |
| Note : Some OSes (like SCO-UNIX) must use “Normal” for installation |  |  |  |  |
| ESC : Skip |  |  |  |  |

The last second step is 'save and exit'. If you select this item and press 'Y', then these records will be saved in the CMOS memory on the mainboard. It will be checked every time you turn your computer on.

ROM PCI/ISA BIOS (ATC-5220)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (ATC-5220)

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP |  | INTEGRATED PERIPHERALS |  |
| --- | --- | --- | --- |
| BIOS FEATURES SETUP |  | PASSWORD SETTING |  |
| CHIPSET FEATURES SETUP |  | IDE HDD AUTO DETECTION |  |
| POWER MANAGEMENT SETUP |  | SAVE & EXIT SETUP |  |
| PNP/PCI CONFIGURATION |  | EXIT WITHOUT SAVING |  |
| LOAD BIOS DEFAULTS | SAVE to CMOS and EXIT (Y/N):Y |  |  |
| ESC: Quit |  | áâàß:Select Item |  |
| F10: Save & Exit Setup |  | (Shift) F2 : Change Color |  |
| Save Data to CMOS & Exit SETUP |  |  |  |

LOAD BIOS DEFAULTS

When your mainboard has problems and needs to

trouble shoot the system, you can use this function.

The default values loaded only affect the BIOS Features

Setup, Chipset Features Setup, Power Management

Setup and PNP/PCI Configuration Setup. There is no

effect on the Standard CMOS Setup. To use this function,

select it from main menu and press <Enter>. A line will

appear on the screen asking if you want to load the BIOS

default values. Press <Yes> and <Enter> then the BIOS

default values will be loaded.

LOAD SETUP DEFAULTS

This allows you to load optimal settings which are stored in

the BIOS ROM. The default values loaded only affect the

BIOS Features Setup, Chipset Features Setup, Power

Management Setup and PNP/PCI Configuration Setup.

There is no effect on the Standard CMOS Setup. To use

this function, select it from main menu and press <Enter>.

A line will appear on the screen asking if you want to load

the Setup default values. Press <Yes> and <Enter> then

the Setup default values will be loaded.

SUPERVISOR PASSWORD / USER PASSWORD

This allows you to set the password. The mainboard defaults with

password disabled.

Enter/Change password : Enter the current password, at

the prompt, key-in your new password (up to eight

alphanumeric characters), press <Enter>. At the next

prompt, confirm the new password by typing it again and press

<Enter>.

Disable password : Press the <Enter> key instead of

entering a new password when the ‘Enter Password’

dialog box appears. A message will appear confirming that

the password is disabled.

If you set both supervisor and user passwords, only the

supervisor password allows you to enter the BIOS SETUP

program.

CAUTION :If you forgot your password, you must disable

the CMOS by turning power off and set JP 8 to ‘close‘. And then

open reload the system.

IDE HDD AUTO DETECTION

This allows you to detect the IDE hard disk drivers‘ parameters

and enter them into ‘Standard CMOS Setup’ automatically.

If the auto-detected parameters displayed do not

match the ones that should be used for your hard drive,

do not accept them. Press <N> to reject the values

and enter the correct ones manually on the Standard

CMOS Setup screen.

SAVE & EXIT SETUP

This allows you to save the new setting values in the

CMOS memory and continue with the booting process.

Select what you want to do, press <Enter>.

EXIT WITHOUT SAVING

This allows you to exit the BIOS setup utility without

recording any new values or changing old ones.

※ Control Key Description ※

| UP ARROW | ↑ | Move to previous item |
| --- | --- | --- |
| DOWN ARROW | ↓ | Move to next item |
| LEFT ARROW | ← | Move to the item in the left hand |
| RIGHT ARROW | → | Move to the item in the right hand |
| Esc KEY | Esc | Main Menu : Quit and not save changes Setup menu : Exit current page and return to main menu |
| PgUp KEY |  | Increase the numeric value or make changes |
| PgDn KEY |  | Decrease the numeric value or make changes |
| F1 KEY | Help | General help |
| F2 KEY | ＜Shift＞+F2 | Change color from total 16 colors |
| F5 KEY | Old Value | Restore the pervious CMOS value from CMOS |
| F6 KEY | Load BIOS default | Load the default CMOS value from BIOS default table |
| F7 KEY | Load setup default | Load Setup default |
| F10 KEY | Save & Exit Setup | Save all the CMOS changes and Exit setup, only for Main Menu |

APPENDIX A

※※TECHNICAL SUPPORT REQUEST FORM※※

If the mainboard doesn't function properly, please complete the following information and return it to your system dealer. If the further information is needed, please attach it.

Model No :  ATC-5220      Date of Purchase :  ______________

Serial No : ___________________

HARDWARE :

|  | BRAND | MODEL | SPEED | Q'TY |
| --- | --- | --- | --- | --- |
| DIM Module |  |  |  |  |

CPU SPEED : ________ MHz

DRAM :______MB (        SDRAM/EDO DRAM)

Hard Disk Interface Controller : _____ IDE, ____ SCSI

Hard Disk Brand :___________, Model :_________, Capacity :________

Display Controller Brand :_____________,Model :__________

Controller Chip Brand :_____________,Model :__________

SOFTWARE:

Award SYSTEM BIOS: Version _________  Date Code _____________

Other Add-on Cards Information:

| Add-on Card | Bus Interface | Model | Remark |
| --- | --- | --- | --- |

Error Description

160-5220-020000-90308