Table of Contents

COMPONENT LOCATION DIAGRAM	A

INDEX	1

CHAPTER 1   INTRODUCTION	2

1-1 SYSTEM FEATURES	3

1-2 SOFTWARE POWER OFF CONTROL	4

1-3 CHECK LIST OF THE PACKAGING	5

CHAPTER 2   INSTALLATION	6

2-1 INSTALLATION PROCEDURES 	6

2-2 CPU INSTALLATION	7

2-3 SYSTEM MEMORY INSTALLATION	9

2-4 CONNECTORS DESCRIPTION	11

2-5 IDE DRIVER INSTALLATION	16

CHAPTER 3   Award BIOS SETUP	17

3-1 UPDATE BIOS PROCEDURES	18
	3-1-1 UPDATE Award SYSTEM BIOS (FLASH EPROM) 
	3-1-2 UPDATE PENTIUM PRO BIOS API

3-2 Award SYSTEM BIOS CONFIGURATION SETUP	20

APPENDIX A 	45

How to install Pentium II ?	..B-1

CHAPTER 1 INTRODUCTION

This manual describes to configure the ATC-6100 mainboard for different environments. It's an overview of the layout and features of the mainboard, and also provides information for you to change the configuration or system environment.

This manual is divided into two sections :

PART ONE   includes page A and two chapters as following:

Page A contains layout diagram of the mainboard.

Please refer to it when you configure the system.

Chapter 1    is an overview of the mainboard features and packing

contents.

Chapter 2    describes how to upgrade and to change hardware

configurations  such as memory size, CPU type, and

lists of jumper settings and connectors.

PART TWO  includes chapter 3 which contains Award BIOS description..

Chapter 3    is the user's guide of the Award BIOS setup utility, Flash ROM

BIOS, and Intel Pentium II BIOS API update procedure. The

menu shown in this chapter are the default settings.

Your system dealer will set up the mainboard according to your demand of the computer. It means that the current settings of your mainboard may not be the same as the defaults shown in this user's manual. If you need to change your  configuration, please ask your dealer first. Be sure this will not void your system warranty, or ask your dealer to do it for you.

REMARK

Intel®  is a registered trademark of Intel Corporation.

MS-DOS® is a registered trademark of Microsoft Corporation.

All other brands and product names are trademarks registered trademarks of their respective companies.

1-1 SYSTEM FEATURES

- INTEL Pentium II CPU operating at 233MHz and 266MHz by using Single Edge Contact (S.E.C.) cartridge (or named Slot 1).

- INTEL Natoma 82440FX PCIset.

- Pentium II CPU with MMXTM Technology, and 512 or 256KB L2 Cache.

- Using six 72-pin SIM sockets, provides three banks of 64-bit wide path up to 768MB addressing page mode DRAMs.

- Supports all types of DRAM included EDO (Extended Data Out), BEDO (Burst EDO), or FPM (Fast Page Mode), with parity chip support ECC (Error Checking and Correcting).

- Built-in Switching Voltage Regulator.

- Supports auto-detect CPU core voltage range 2.1V to 3.5V.

- Supports four PCI revision 2.1 interface compliant and four 16-bit ISA slots.

- Dual Master IDE connectors support up to four devices in two channels for connecting of high capacity hard drive, CD-ROM, tape backup etc..

- Supports the USB (Universal Serial Bus) header.

- PS/2 keyboard connector and PS/2 mouse connector.

- Winbond 83877 high-speed Multi-I/O chipset.

- Supports Infrared transfer (IrDA TX/RX) connection.

- One FDC port supports two devices up to 2.88MB

- Two 16550A fast UARTs compatiable serial ports

- One EPP/ECP mode parallel port

- Hardware Dimension is 245mm x 305mm (9.64" x 12.00").

1-2 SOFTWARE POWER OFF CONTROL

The mainboard design supports Software Power Off Control feature through the SMM code in the BIOS under Windows 95, Windows 3.1x, and MS-DOS operation system environment. This is Intel ATX form factor feature and you should use ATX power supply.

First, you should connect the power switch cable (provided by the ATX case supplier) to the connector JP1 “PS-ON” (next to IDE connector) on the mainboard.  In the BIOS screen of ‘POWER MANAGEMENT SETUP’, choose “User Defined” (or “Min. Power Saving” or “Max. Power Saving”) in ‘Power Manager’ and choose “Yes” in ‘PM Control by APM’.

In Windows 95, if you would like to power off the system, you just choose “shutdown the computer ?” in the “Shut Down Windows“ from Windows 95, then the system power will be off directly, and be the stand-by status. You will find the power LED light blinking. If you would like to restart the system, just press the power switch button, and the system will be powered on.

In Windows 3.1x or MS-DOS, you should copy the program of “down.com” (you can find it in the diskette of the IDE driver) into the hard drive. When you would like to power off the system, just run this “down.com”, then the system will be shutdown and stay in standby status.

Note : If you will leave your system for several days, we suggest you use hardware power off to shutdown your system.

1-3 CHECK LIST OF THE PACKAGING

The mainboard comes securely packed in a durable box and shipping carton. If any of the above items are missing or damaged, please contact your supplier.

Each mainboard contains:

Q'TY     	Description

1        	Mainboard 	: ATC-6100.

1	Retention module (for Pentium II installation)

1	Diskette	: Enhanced IDE driver (3.5")

Award system BIOS

Update Utility

1      	Cable 	: Enhanced IDE connector.

1         Cable 	: F.D.D. connector.

1         Manual 	: User`s manual.

NOTE : Leave the mainboard in its original packaging until you are ready to install it.

CHAPTER 2  INSTALLATION

2-1 INSTALLATION PROCEDURE

Before installing the computer, please prepare all components such as CPU, DRAM; peripherals such as hard drive, keyboard, CD-ROM; and accessories such as cables. Then, install the system as following:

1. Plug CPU/ heat sink (refer to Pentium II installation guide),

and DRAM modules in the mainboard.

2. Set DIP switch based on your configuration.

3. Plug add-on cards in PCI/ISA slots.

4. Connect cables to peripherals, power supply.

5. Make sure all components and devices are well connected,

turn on the power and setup System BIOS based on your

configuration.

6. Install peripherals, add-on card drivers and test them.

7. If all of above procedures are success, turn-off the power then

plug all of them into your computer case.

2-2 CPU  INSTALLATION

ATC-6100 supports INTEL Pentium II CPU cartridge.

2-2-1 CPU TYPE SELECTION

| INTERNAL CPU CLOCK | SW1 | Ext.x Frq. |
| --- | --- | --- |
| 233MHz | ON OFF 1 2 3 4 | 66x3.5 |
| 266MHz | ON OFF 1 2 3 4 | 66x4.0 |
| 300MHz | ON OFF 1 2 3 4 | 66x4.5 |
| 333MHz | ON OFF 1 2 3 4 | 66x5.0 |

SW1

1

2

3

4

ON   DIP

1   2   3   4

Slot 1

DIP switch, ( ex. : ON,ON,ON,OFF)

ON : switch to the ‘ON’ position.

OFF : switch to the ‘OFF’ position.

2-2-2 CPU VOLTAGE SETTING

The ATC-6100 mainboard has built-in VID (Voltage IDentify) function to auto-detect various CPU voltages, you do not need to adjust the CPU voltage setting when you install the Pentium II CPU card on this mainboard.

2-3 SYSTEM MEMORY INSTALLATION

The ATC-6100 provides six 72-pin SIMM sockets for system memory expansion from 8MB to 768MB. These six SIMMs are arranged to three banks, Bank0 (SIM 1, 2), Bank1 (SIM 3, 4), and Bank2 (SIM 5, 6), please refer to page A. Each bank provides 64-bit wide data path.

The mainboard accepts Fast Page Mode DRAM, EDO Mode (Extended Data Out) DRAM, ECC (with parity-chip) DRAM,and BEDO(Burst EDO) DRAM, with a speed of at least 70 nanosecond. You should plug DRAM modules into two sockets (same bank) or six sockets at one time. Each pair of modules must be the same size, type, and speed; no matter single-side or double-side module.

Please plug in Bank 0 first if you only have 2 modules. The mainboard supports mixing of EDO SIMMs with fast page mode DRAM SIMMs among different banks; please plug EDO in Bank 0, if you only have 2 EDO modules.

¡ Samples of System Memory Combinations Options ¡

| BANK0 | BANK1 | BANK2 | Total Memory |
| --- | --- | --- | --- |
| SIM 1, 2 | SIM 3, 4 | SIM 5, 6 | SIM 1- 6 |
| 4MBx2 | - | - | 8MB |
| - | 4MBx2 | 4MBx2 | 16MB |
| 8MBx2 | - | - | 16MB |
| - | 8MBx2 | 8MBx2 | 32MB |
| 4MBx2 | 4MBx2 | 4MBx2 | 24MB |
| 8MBx2 | 4MBx2 | 4MBx2 | 32MB |
| 16MBx2 | - | - | 32MB |
| 4MBx2 | 8MBx2 | 8MBx2 | 40MB |
| 8MBx2 | 8MBx2 | 8MBx2 | 48MB |
| 16MBx2 | 4MBx2 | 4MBx2 | 48MB |
| - | 16MBx2 | 16MBx2 | 64MB |
| 16MBx2 | 8MBx2 | 8MBx2 | 64MB |
|  | - continue - |  |  |
| 32MBx2 | - | - | 64MB |
| 4MBx2 | 16MBx2 | 16MBx2 | 72MB |
| 8MBx2 | 16MBx2 | 16MBx2 | 80MB |
| 32MBx2 | 4MBx2 | 4MBx2 | 80MB |
| 16MBx2 | 16MBx2 | 16MBx2 | 96MB |
| 32MBx2 | 8MBx2 | 8MBx2 | 96MB |
| - | 32MBx2 | 32MBx2 | 128MB |
| 64MBx2 | - | - | 128MB |
| 4MBx2 | 32MBx2 | 32MBx2 | 136MB |
| - | 64MBx2 | 64MBx2 | 256MB |
| : | : | : | : |
| : | : | : | : |
| 128MBx2 | 128MBx2 | 128MBx2 | 768MB |

2-4 CONNECTORS  DESCRIPTION

The locations of following connectors are indicated in page A. When you plug a cable into the following I/O connectors, you should have the pin 1 edge of the cable align with the pin 1 end of the connector.

CONN1 : speaker, keyboard lock, reset, SMI,

turbo LED, and IDE LED connectors.

SPK          POWER LED

RST   SMI    TB-LED IDE-LED

+       +

SPK : speaker

Speaker

GND

GND

VCC

Power LED connector

Power LED +

N/C

GND (power)

RST : Reset connector

Reset Signal

GND

SMI : SMI lead

GND

SMI Signal

TB-LED : Turbo LED indicator, LED on

when system runs higher speed

GND

+5V (next to CPU)

IDE-LED : IDE devices indicator LED

connector. IDE-LED stays ON indicates GND   on-board IDE devices in operation. If

+5V    plug wire into wrong connector, color of

LED will be lighter and the IDE dvices can

still function properly.

FAN1 : CPU cooling fan connector. Wire with

+12V voltage (most likely red wire) must be

plugged into pin2, and GROUND wires

1 GND

2+12V

3GND

(most likely black wires) must be plugged into

pin1and/or pin3. Please confirm the wire color

re-presentation with your supplier.

CAUTION: Plug wire into wrong connector will

DAMAGE fan and mainboard.

1 +5V

2N/C

3IRRX

4GND

5IRTX

IR1 : Infrared module connector.

USB1 : USB connector; Universal Serial Bus;this

(red)

VCC GND

D1 -D0 +

D1+D0 -

GNDVCC

(black)

is used to connect USB devices through an

optional dual head cable with a iron plane.

CAUTION: Plug wire into wrong connector

will DAMAGE USB devices and mainboard.

PS/2 KB : 6-pin PS/2 style compatible keyboard

connector.

PS/2 mouse : PS/2 mouse connector, which is

used to connect an optional cable.

4 6

2

1

35

pin1 : data

pin2 : N/C

pin3 : GND

pin4 : VCC

pin5 : clock

pin6 : N/C

PW1 : ATX mode +3.3/5/12V power supply

connector.

13.3V6+5V113.3V16GND23.3V7GND12-12V17GND3GND8PWRGD13GND18-5V4+5V95VSB14*PS_ON19+5V5GND10+12V15GND20+5V* PS_ON : Soft-Off power control

1            10

11             20

COM1/COM2 : these two connectors are used to

1 2

34

56

78

910

connect serial port cables.

| pin | signal name |
| --- | --- |
| 1 | DCD |
| 2 | Serial In |
| 3 | Serial Out |
| 4 | DTR |
| 5 | GND |
| 6 | DSR |
| 7 | RTS |
| 8 | CTS |
| 9 | RI |
| 10 | N/C |

FDC1 : this connector is used to connect the floppy

12

3334

drive through a cable.

| pin | signal | pin | signal |
| --- | --- | --- | --- |
| 2 | RWC- | 20 | STEP- |
| 4 | reserved | 22 | Write Data |
| 6 | FDEDIN | 24 | Write Gate |
| 8 | Index- | 26 | Track 00- |
| 10 | Motor EnableA- | 28 | Write Protect- |
| 12 | Drive Sele.B- | 30 | Read Data- |
| 14 | Drive Sele.A- | 32 | Side 1 Sele.- |
| 16 | Motor EnableB- | 34 | DisketteChange |
| 18 | DIR- |  |  |
| All of odd pins are ground |  |  |  |

LPT1 : this connector is used to connect the parallel

port cable.

114

215

1326

| pin | signal | pin | signal |
| --- | --- | --- | --- |
| 1 | STROBE- | 10 | ACK- |
| 2 | Data Bit 0 | 11 | BUSY |
| 3 | Data Bit 1 | 12 | PE |
| 4 | Data Bit 2 | 13 | SLCT |
| 5 | Data Bit 3 | 14 | Auto Feed- |
| 6 | Data Bit 4 | 15 | ERROR- |
| 7 | Data Bit 5 | 16 | INIT- |
| 8 | Data Bit 6 | 17 | SLCT IN- |
| 9 | Data Bit 7 | 26 | N/C |
| pin18 -- pin25 are ground |  |  |  |

IDE1/IDE2 : these two connectors are used to

connect IDE devices through IDE cables,

12

34

3940

a total of 4 devices can be connected.

| pin | signal | pin | signal |
| --- | --- | --- | --- |
| 1 | Reset IDE | 21 | DDRQ0(1) |
| 2 | GND | 22 | GND |
| 3 | Host Data 7 | 23 | I/O Write- |
| 4 | Host Data 8 | 24 | GND |
| 5 | Host Data 6 | 25 | I/O Read- |
| 6 | Host Data 9 | 26 | GND |
| 7 | Host Data 5 | 27 | IORDY |
| 8 | Host Data 10 | 28 | N/C |
| 9 | Host Data 4 | 29 | DDAK0- (1-) |
| 10 | Host Data 11 | 30 | GND |
| 11 | Host Data 3 | 31 | IRQ14* |
| 12 | Host Data 12 | 32 | IOCS16- |
| 13 | Host Data 2 | 33 | Addr 1 |
| 14 | Host Data 13 | 34 | N/C |
| 15 | Host Data 1 | 35 | Addr 0 |
| 16 | Host Data 14 | 36 | Addr 2 |
| 17 | Host Data 0 | 37 | ChipSele.1P- |
| 18 | Host Data 15 | 38 | ChipSele.3P- |
| 19 | GND | 39 | Activity |
| 20 | Key | 40 | GND |

* IDE1 : pin31 is IRQ14;

IDE2 : pin31 is IRQ15 or MIRQ0

2-5 IDE DRIVER INSTALLATION

The IDE driver installation procedure is as following :

Setup for DOS/Windows :

1. Starting MS-Windows 3.1 (or 3.11)

2. Select Program Manager, File/RUN, then type

"A:\setup.exe"

3. Exit MS-Windows, turn power off; then turn power on.

Setup for Windows 95 : (Windows 95 version 950/950a/950b)

1. Starting Windows 95

2. Select “START”, “RUN”.

3. Type “A:\WIN95\SETUP.EXE”.

4. Restart computer, then follow the instructions on your

screen to install new IDE driver we offer in the 3.5“ diskette

5. Exit Windows 95, turn power off; then turn power on.

Make sure your HDD should follow ATA standard, and your CD-ROM should follow ATAPI standard. When you plug-in the IDE devices, please plug your first and second devices into IDE 1 port (Master then Slave), then plug third and forth devices into IDE 2 port. If you have CD-ROM driver, please set it behind hard disk devices as the last device. For example, if you have 2 HDDs and 1 CD-ROM, you should set HDD1 and HDD2 in IDE1 Master and Slave, set CD-ROM in IDE 2 Master. Some of the brands devices combination may not work under this sequence, you can try to re-arrange the devices sequence, or contact your vendor.

Primary MasterPrimary SlaveSecondary MasterSecondary SlaveATA no ATAPIATAATAPI disk & CD-ROMATAATAPI use only one cableATAATAPIATAPI CD-ROM and a tape

or two CD-ROMs

CHAPTER 3  Award BIOS SETUP

This chapter explains the system BIOS setup, and how to update to a new BIOS. All BIOS screens shown in the following pages are default values, your system dealer will set up these values according to your demand of the computer.

ATC-6100 uses Flash ROM to make the BIOS easily to be updated by the floppy disk-based program. and to meet Microsoft Windows 95 plug & play feature.

¡  JP10 Setting is for Update System BIOS

| JP10 Nomal Clear open | NORMAL¡ |
| --- | --- |
| close | CLEAR CMOS |

¡is default setting

NOTE :

To clear CMOS while

power is on, otherwise the

‘clear’ will not work.

3-1 UPDATE BIOS PROCEDURES

If the BIOS needs to be updated, it can be obtained on a diskette from your system supplier. The BIOS diskette includes :

“awdflash.exe”  --  BIOS update utility program

“awdflash.doc”

“(update BIOS filename with version number).rom”

The update procedure is as following:

- Boot the system to DOS mode in a normal manner.

- Insert the updated diskette to drive A (or B).

- Change working directory to floppy drive, A or B, which contains the update BIOS diskette. --  Type “a:\” or “b:\”, “ENTER”.

- Run the BIOS update utility  --  Type “awdflash”, “ENTER”.

- Type “(update BIOS file name with version number).rom”, ENTER.

- Type “N” when the screen displays the message : " Do you want to save BIOS (Y/N) ?".

- Type “Y“ when the screen shows the message : " Are you sure to program (Y/N) ?".

- Follow instructions displayed on the screen.  DO NOT remove the update BIOS diskette from the floppy drive nor turn the system power off until the BIOS update is completed.

- Exit the utility and turn the system power off, set JP10 to 'close' to clear the CMOS. After about 10 seconds, set JP10 to 'open' (remove the jumper cap ) to return to normal status.

- Turn the system power on and test that your system is working properly.

3-1-2 UPDATE PENTIUM II BIOS API

Intel also provides BIOS API(Applications Programming Interface) for Pentium II processor-based mainboard user to update data block in BIOS quickly and easily. (You can find this utility in the 3.5“ diskette in the package).

The BIOS code on the Pentium II processor-based mainboards contains data that is specific to each silicon stepping of the processor. Integrators must ensure that this BIOS stepping data matches the processor stepping used. When the BIOS does not contain stepping data that matchs the processorstepping, integrators must update the data in the BIOS before shipping the system. Historically, Pentium II systems have been updated by replacing the entire BIOS with a new revision of BIOS that contains the correct stepping data.

Intel‘s BIOS update API allows just the stepping data within the BIOS to be updated as needed. Mainboards that contain a BIOS with the Intel-defined BIOS update API can be quickly and easily updated, if required, without obtaining a complete BIOS upgrade. Using this utility, integrators can easily verify that the correct stepping data is present in all Pentium II processor-based mainboards. However, if the stepping data requires updating, the mainboard BIOS must contain the Intel-defined BIOS update API, otherwise a complete BIOS upgrade is required from the mainboard vendor.

Yould should set your diskette to “write protect, then type

A:\>cd api; A:\api>checkup1

The main menu should now be displayed, showing the following four options :

1) Check and load update

2) Specify stepping data file [current : pep.pdb]

3) Help

4) Quit without loading update

Select 1) to know the stepping filename, select 2) to load right patch code, then select 1) to update proper patch code. Now, the screen will show the message “please remove floppy diskette from floppy disk drive”. Then cold boot (mechanical power off) system to continue. For more information, please refer to “CHECKUP.HLP“ file.

3-2  AWARD SYSTEM BIOS

CONFIGURATION SETUP

The following pages explain how to set up the system configuration (CMOS) under the AWARD BIOS. The SETUP program is stored in the Read-Only-Memory (ROM) on the mainboard. To do the SETUP procedure, press the <Del> key when the system is booting up. The following main menu will appear. Please select " STANDARD CMOS SETUP" to enter the next screen.

ROM PCI/ISA BIOS (2A69HA2B)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

The section on the bottom of the main menu explains how to control this screen.  The other section displays the items highlighted in the list.

This screen records some basic hardware information, and sets the system clock and error handling. These records can be lost or corrupted if the on-board battery has failed or is weak.

ROM PCI/ISA BIOS (2A69HA2B)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (2A69HA2B)

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| Date (mm:dd:yy) : Fri, Jun 14 1996 |  |  |
| --- | --- | --- |
| Time(hh:mm:ss) : 13 : 37 : 14 |  |  |
| HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |
| Primary Master : Auto 0 0 0 0 0 0 Auto |  |  |
| Primary Slave : None 0 0 0 0 0 0 ---- |  |  |
| Secondary Master : None 0 0 0 0 0 0 ----- |  |  |
| Secondary Slave : None 0 0 0 0 0 0 ---- |  |  |
| Drive A : 1.44M, 3.5 in. | Base Memory : 640K |  |
| Drive B : None | Extended memory : 7168K |  |
|  | Other Memory : 384K |  |
| Video : EGA/VGA | ------------------------------------------------ |  |
| Halt On: All Errors | Total Memory : 8192K |  |
| ESC : Quit áâàß:Select Item PU/PD/+/- : Modify |  |  |
| F1 : Help (Shift) F2 : Change Color |  |  |

Date	mm is month, dd is date, yy is year.

| date | from 1 to 31 |
| --- | --- |
| month | from Jan. to Dec. |
| year | from 1900 to 2099 |

Time	hh is hour, mm is minute, ss is second.

| hh | from 0 to 23 (24-hour military -time) |
| --- | --- |
| mm | from 0 to 59 |
| ss | from 0 to 59 |

Primary Master	These categories identify the types of the 2 channels

Primary Slave	that have been installed in the computer. There

Secondary Master	are 45 predefined types and 4 user definable

Secondary Slave	types are for Enhanced IDE BIOS. Type 1 to 45

are predefined. Type ‘user’ is user-definable.

Press PgUp/PgDn to select a numbered hard

disk type or type the number and press<Enter>.

If you select ‘Auto’ the BIOS will auto-detect the

HDD & CD-ROM Drive at the POST stage and

show the IDE for HDD & CD-ROM Drive. If

you select ‘user’, you will need to know the

information listed below. Enter the information 					directly from the keyboard and press <Enter>.

This information should be from your hard disk

vender or dealer. If the controller of the HDD

interface is ESDI, the selection shall be ‘Type	1’;

if SCSI, the selection shall be ‘None’.

If no device is installed select

‘NONE’ and press <Enter>.

| Type | drive type |
| --- | --- |
| SIZE | automatically adjusts |
| CYLS | number of cylinders |
| HEAD | number of heads |
| PRECOMP | write precom |
| LANDZ | landing zone |
| SECTOR | number of sectors |
| MODE | mode type |

Drive A	This category identifies the types of floppy disk

Drive B	drive A or drive B that have been installed in the computer.

| None | No floppy drive installed |
| --- | --- |
| 360K, 5.25 in | 5.25“ PC-type 360KB capacity |
| 1.2M, 5.25 in | 5.25“ AT-type 1.2MB capacity |
| 720K, 3.5 in | 3.5“ double-side 720KB capacity |
| 1.44M, 3.5 in | 3.5“ double-side 1.44MB capacity |
| 2.88M, 3.5 in | 3.5“ double-side 2.88MB capacity |

Video	This category selects the type of video adapter

used for the primary system monitor. Although

secondary monitors are supported, you do not

have to select the type in Setup.

Halt On	This category determines whether the computer

will stop if an error is detectd during power up.

| No errors | When the BIOS detects a non-fatal error the system will be stopped and you will be prompted |
| --- | --- |
| All errors | The system boot will not be stopped for any error that may be detected |
| All, But Keyboard | The system boot will not stop for a keyboard error, it will stop for all other errors |
| All, But Diskette | The system boot will not stop for a disk error, it will stop for all other errors |
| All, But Disk/Key | The system boot will not stop for a disk or keyboard error, it will stop for all other errors |

Memory	This category is display-only which is determined

by POST (Power On Self Test) of the BIOS.

Base Memory  The value of the base memory

is typically 512K or 640K based on the memory

installed on the mainboard.

Extended Memory  How much extended

memory is present during the POST. This is

the amount of memory located above 1MB in

the CPU‘s memory address map.

Other Memory  This refers to the memory

located in the 640K to 1024K address space.

The BIOS is the most frequent user of this RAM

area since this is where it shadows RAM.

This screen is a list of system configuration options. Some of them are defaults required by the mainboard's design, others depend on the features of your system.

ROM PCI/ISA BIOS (2A69HA2B)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Virus, Protection, Boot Sequence |  |

ROM PCI/ISA BIOS (2A69HA2B)

BIOS FEATURES SETUP

AWARD SOFTWARE, INC.

| Virus Warning | : Disabled | Video BIOS Shadow | : Enabled |
| --- | --- | --- | --- |
| CPU Internal Cache | : Enabled | C8000-CBFFF Shadow | : Disabled |
| External Cache | : Enabled | CC000-CFFFF Shadow | : Disabled |
| Quick Power On Self Test | : Disabled | D0000-D3FFF Shadow | : Disabled |
| Boot Sequence | : A,C,SCSI | D4000-D7FFF Shadow | : Disabled |
| Swap Floppy Drive | : Disabled | D8000-DBFFF Shadow | : Disabled |
| Boot Up Floppy Seek | : Enabled | DC000-DFFFF Shadow | : Disabled |
| Boot Up NumLock Status | : On |  |  |
| Boot Up System Speed | : High |  |  |
| Gate A20 Option | : Fast |  |  |
| Typematic Rate Setting | : Disabled |  |  |
| Typematic Rate(Chars/Sec) | : 6 | Esc : Quit áâàß:Select Item |  |
| Typematic Delay(Msec) | : 250 | F1 : Help PU/PD/+/- : Modify |  |
| Security Option | : Setup | F5 : Old Values (SHIFT)F2 : Color |  |
| PCI/VGA Palette Snoop | : Disabled | F6 : Load BIOS Defaults |  |
| OS/2 select for DRAM>64MB | : Non-OS2 | F7 : Load Setup Defaults |  |
| Report No FDD for WIN 95 | : No |  |  |

Virus Warning	When this item is enabled, the BIOS will

monitor the boot sector and partition table of the

hard disk drive for any attempt at modification.

If an attempt is made, the BIOS will halt the

system and the following error message will

appear. Many disk diagnostic programs which

attempt to access the boot sector table can

cause the above warning message. If you will

be running such a program, we recommend

that you first disable Virus Protection beforehand.

! WARNING ! Disk boot sector is to be modified Type ‘Y’ to accept write or ‘N’ to abort write Award Software, Inc.

| Enabled | Activates automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector or hard disk partition table. |
| --- | --- |
| Disabled | No warning message will appear when anything attempts to access the boot sector or hard disk partition table. |

CPU	These two categories speed up memory access.

Internal Cache	However, it depends on CPU/chipset design.

External Cache	The default value is‘enabled‘.

Quick Power On	This category speeds up Power On Self Test

Self Test	after you power up the computer. If you set 						Enabled, BIOS will shorten or skip some checked

items during POST.

Boot Sequence	This category determines which drive to search

first for the disk operating system (i.e.,DOS).

Choices are A,C, SCSI; C, A, SCSI;

C, CDROM, A; CDROM, C, A; D, A, SCSI;

E, A, SCSI; F, A, SCSI; SCSI, A, C; SCSI, C, A;

and C only.

For example :

| CDROM,C, A | System will first search for CDROM then HDD, and next is FDD |
| --- | --- |
| C,CDROM, A | System will first search for HDD then CDROM, and next is FDD |

C is primary master; D is primary slave;

E is secondary master,F is secondary slave

Swap Floppy	This item allows you to determine whether to

Drive 	enable the swap floppy drive or not.

Boot Up Floppy 	During POST, the BIOS will determine if the floppy

Seek	disk drive installed is 40 tracks (360K) or 80

tracks (720K, 1.2M, 1.44M)

| Enabled | BIOS searchs for floppy disk drive to determine if it is 40 or 80 tracks |
| --- | --- |
| Disabled | BIOS will not search for the type of floppy disk drive by track number |

Boot Up 	This allows you to determine the default state of

NumLock Status	the numeric keypad. By default, the system

boots up with NumLock on.

Boot Up System 	Selects the default system speed - the normal

Speed	operating speed at power up.

Gate A20	This entry allows you to select how the gate

Option	A20 is handled. The gate A20 is a device used

to address memory above 1 MB. Normal is

keyboard; Fast is chipset.

Typematic Rate	This determines if the typematic rate is to be used.

Setting	When disabled, continually holding down a key

on your keyboard will generate only one key instance.

Typematic Rate 	When the typematic rate is enabled, this section

(Chars/Sec)	allows you select the rate at which the keys are

repeated.

| 6 | 6 characters per second |
| --- | --- |
| 8 | 8 characters per second |
| 10 | 10 characters per second |
| 12 | 12 characters per second |
| 15 | 15 characters per second |
| 20 | 20 characters per second |
| 24 | 24 characters per second |
| 30 | 30 characters per second |

Typematic Delay 	When the typematic rate is enabled, this section

(Msec)	allows you select the delay between when the

key was first depressed and when the

acceleration begins.

| 250 | 250 msec |
| --- | --- |
| 500 | 500 msec |
| 750 | 750 msec |
| 1000 | 1000 msec |

Security Option 	This category allows you to limit access to the

system and Setup, or just to Setup

| System | The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt |
| --- | --- |
| Setup | The system will boot, but access to Setup will be denied if the correct password is not entered at the prompt |

To disable security, select PASSWORD SETTING

at Main Menu and then you will be asked to enter

password. Do not type anything and just press

<Enter>, it will disable security. Once the security

is disabled, the system will boot and you can enter 					Setup freely.

PCI/VGA 	It determines whether the MPEG ISA/VESA

Palette Snoop	VGA cards can work with PCI/VGA or not.

| Enabled | When PCI/VGA working with MPEG ISA/VESA VGA Card |
| --- | --- |
| Disabled | When PCI/VGA not working with MPEG ISA/VESA VGA Card |

OS Select for 	This item allows you to access the memory

DRAM > 64MB	that is over 64MB in OS/2

Video BIOS	Determines whether video BIOS will be copied to

Shadow	RAM. However it is optional depending on

chipset design. Video Shadow will increase the

video speed.

C8000 - CBFFF 	These categories determine whether option

Shadow	ROMs will be copied to RAM. An example of

DC000 - DFFFF	such option ROM would be the support of onboard

Shadow 	SCSI.

Report No FDD 	For Windows 3.1x users set ‘NO’ (default); for

For WIN 95	Windows 95 users set ‘NO’ or ’YES’.

This screen controls the setting for the chipset on the mainboard.

ROM PCI/ISA BIOS (2A69HA2B)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| AT Clock, DRAM Timmings, ....... |  |

ROM PCI/ISA BIOS (2A69HA2B)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Auto Configuration | : Enabled | 8-bit I/O Recovery Time |  | : 3 |
| --- | --- | --- | --- | --- |
| DRAM Speed Selection | : 70 ns | 16-bit I/O Recovery Time |  | : 2 |
| DRAM RAS# Precharge Time | : 4 | Memory Hole At 15M-16M |  | : Disabled |
| MA Additional Wait State | : Disabled | DRAM Fast Leadoff |  | : Disabled |
| RAS# To CAS# Delay | : Enabled | Passive Release |  | : Enabled |
| DRAM Read Burst (B/E/F) | : x2/3/4/ | Delayed Transaction |  | : Disabled |
| DRAM Write Burst (B/E/F) | : x3/3/3 |  |  |  |
| ISA BusClock | : PCICLK/4 |  |  |  |
| DRAM Refresh Queue | : Disabled |  |  |  |
| DRAM RAS Only Refresh | : Enabled |  |  |  |
| DRAM ECC/PARITY Select | : Disabled |  |  |  |
| Fast DRAM Refresh | : Disabled |  |  |  |
| Read-Around-Write | : Disabled |  |  |  |
| PCI Burst Write Combine | : Disabled |  |  |  |
| PCI-to-DRAM Pipeline | : Disabled | Esc: Quit | :Select Item |  |
| CPU-to-PCI Write Post | : Disabled | F1 : Help | PU/PD/+/-:Modify |  |
| CPU-to-PCI IDE Posting | : Disabled | F5 : Old Values | (Shift)F2 :Color |  |
| System BIOS Cacheable | : Disabled | F6 :Load BIOS Defaults |  |  |
| Video RAM Cacheable | : Disabled |  |  |  |

Auto	Pre-defined values for DRAM, cache... timing

Configuration	according to CPU type & system clock. When

this item is enabled, the pre-defined items will

become SHOW-ONLY.

DRAM Speed	The DRAM speed is controlled by the DRAM

Selection	timing Registers. The timings programmed into

this register are dependent on the system design.

DRAM RAS#	DRAM must continually be refreshed or it will lose

Precharge Time	its data. Normally, DRAM is refreshed entirely as

the result of a single request. This option allows

you to determine the number of CPU clocks

allocated for the Row Address Strobe to

accumulate its charge before the DRAM is

refreshed. If insufficient time is allowed, refresh

may be incomplete and data lost.

RAS# to	When DRAM is refreshed, both rows and columns

CAS# Delay	are addressed separately. This setup item allows

you to determine the timing of the transition from

RAS to Column Address Strobe (CAS).

DRAM Read	This sets the timing for burst mode read (or

Burst (B/E/F) 	writes)from DRAM. Burst read and write

DRAM Write		requests are generated by the CPU in four

Burst (B/E/F)		separate parts. The first part provides the

location within the DRAM where the read or write

is to take place while the remaining three parts

provide the actual data. The lower the timing

numbers, the faster the system will address memory.

ISA Bus Clock	This item allows you to select the PCI clock type.

Choices are PCI CLK/3; PCI CLK/4

System BIOS	When enabled, accesses to the system BIOS

Cacheable 	ROM addressed at F0000H-FFFFFH are cached,

provided that the cache controller is enabled.

Video BIOS 	As with changing the system BIOS above,

Cacheable 	enabling the Video BIOS cache will cause

access to video BIOS addressed at C0000H to

C7FFFH to be cached, if the cache controller is

also enabled.

8 Bit I/O 	The recovery time is the length of time, measured

Recovery Time 	in CPU clocks, which the system will be delay after

the completion of an I/O request. This item allows

you to determine the recovery time allowed for 8-

bit I/O. Choices are from NA, 1 to 8 CPU clocks.

16 Bit I/O	This item allows you to determine the recovery

Recovery Time 	time allowed for 16-bit I/O. Choices are from NA,

1 to 4 CPU clocks.

Memory Hole At	In order to improve performance, certain space

15M-16M	in memory can be reserved for ISA cards. This memory

must be mapped into the memory below 16MB.

DRAM ECC/	During CPU reads of the DRAM, the 430HX

Parity Select 	provides error checking and correction or

parity of the data.

This screen controls the 'green' features of this mainboard.

ROM PCI/ISA BIOS (2A69HA2B)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Sleep Timer, Suspend Timer, ..... |  |

ROM PCI/ISA BIOS (2A69HA2B)

POWER MANAGEMENT SETUP

AWARD SOFTWARE, INC.

| Power Management | : UserDefined |  | *Power Down & Resume Events* |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PM Control by APM | : Yes |  | IRQ3 (COM 2) |  |  |  |  | : OFF |  |
| Video Off Method | : V/H SYNC |  | IRQ4 (COM 1) |  |  |  |  | : OFF |  |
|  | +Blank |  | IRQ5 (LPT 2) |  |  |  |  | : OFF |  |
| Modem Use IRQ | : 3 |  | IRQ6 (Floppy Disk) |  |  |  |  | : OFF |  |
|  |  |  | IRQ7 (LPT 1) |  |  |  |  | : OFF |  |
| Doze Mode | : Disabled |  | IRQ8 (RTC Alarm) |  |  |  |  | : OFF |  |
| Standby Mode | : Disabled |  | IRQ9 (IRQ2 Redir) |  |  |  |  | : OFF |  |
| Suspend Mode | : Disabled |  | IRQ10 (Reserved) |  |  |  |  | : OFF |  |
| HDD Power Down | : Disabled |  | IRQ11 (Reserved) |  |  |  |  | : OFF |  |
|  |  |  | IRQ12 (PS/2 Mouse) |  |  |  |  | : OFF |  |
|  |  |  | IRQ13 (Coprocessor) |  |  |  |  | : OFF |  |
| *Wake Up Events In Doze & Standby* |  |  | IRQ14 (Hard Disk) |  |  |  |  | : OFF |  |
| IRQ3 (Wake-Up Event) |  | : OFF | IRQ15 (Reserved) |  |  |  |  | : OFF |  |
| IRQ4 (Wake-Up Event) |  | : OFF | Esc: Quit áâàß |  |  |  | :Select Item |  |  |
| IRQ8 (Wake-Up Event) |  | : OFF | F1 : Help | PU/PD/+/- : Modify |  |  |  |  |  |
| IRQ12 (Wake-Up Event) |  | : OFF | F5 : Old Values |  | (Shift) F2: Color |  |  |  |  |
|  |  |  | F6 : Load BIOS Defaults |  |  |  |  |  |  |

Power 	This category allows you to select the type (or

Management	degree) of power saving and is directly related to

the following modes : Doze; Standby; Suspend;

HDD Power Down.

| Disabled | No power management. Disables all 4 modes |
| --- | --- |
| Min. Power Saving | Minimum power management. Doze =1hr.; Standby=1hr.; Suspend=1hr.; HDD Power Down=15min |
| Max. Power Saving | Maximum power management only available for SL CPU‘s.Doze=1min.; Standby=1min.;Suspend=1min.;HDD Power Down=1min |
| User Defined | Allows you to set each mode indivi-dually. When not disabled, each of the ranges are from 1min. to 1hr. exect for HDD Power Down which ranges from 1 to 15min. and disable |

If you would like to use Software Power-off

Control function, you cannot choose

“Disabled ”here, and should select “Yes” in

PM Control by APM.

PM Control 	When enabled, an Advanced Power Management

by APM	device will be activated to enhance the Max.

Power Saving Mode and stop the CPU internal

clock. If the Max. Power Saving is not enabled,

this will be shown as NO.

Video Off	This determines the manner in which the monitor

Method	is blanked.

| V/H SYNC + Blank | This selection will cause the system to turn off the vertical and horizontal sync. ports and write blanks to the video buffer |
| --- | --- |
| Blank Screen | This option only writes blanks to the video buffer |
| DPMS | Initial display power management signaling |

The Following 4 modes are Green PC power saving function which are only user configurable when ‘User Defined’ power management has been selected.

Doze Mode	When enabled and after the set time of system

inactivity, the CPU clock will run at slower speed

while all other devices still operate at full speed

Standby Mode	When enabled and after the set time of system

inactivity, the fixed disk drive and the video

would be shut off while all other devices still

operate at full speed

Suspend Mode	When enabled and after the set time of system

inactivity, all devices except the CPU will be shut

off

HDD Power Down	When enabled and after the set time of system

inactivity, the hard disk drive will be powered

down while all other devices remain active

Wake Up Events

In Doze & Standby

Power Down &	These are I/O events whose occurrence can

Resume Events	prevent the system from entering a power saving

mode or can awaken the system from such a

IRQ3 ~ IRQ15	mode. In effect, the system remains alert for

anything which occurs on a device which is

configured as On, even when the system is in a

powered down mode. When an I/O device wants to

gain the attention of the operating system, it

signals this by causing an IRQ (Interrupt ReQuests)

to occur. When the operating system is ready to

respond to the request, it interrupts itself and

performs the service. When set to off, activity will

neither prevent the system from going into a power

management mode nor awaken it.

This screen configures the PCI Bus slots.

ROM PCI/ISA BIOS (2A69HA2B)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| IRQ Settings, Latency Timers, ..... |  |

ROM PCI/ISA BIOS (2A69HA2B)

PNP/PCI CONFIGURATION

AWARD SOFTWARE, INC.

| Resources Controlled by | : Auto | PCI IRQ Actived by |  |  |  | : Level |
| --- | --- | --- | --- | --- | --- | --- |
| Reset Configuration Data | : Disabled | PCI IDE IRQ Map to |  |  |  | : PCI-AUTO |
|  |  | Primary IDE INT# |  |  |  | : A |
|  |  | Secondary IDE INT# |  |  |  | : B |
|  |  | Esc: Quit áâàß |  |  | :Select Item |  |
|  |  | F1 : Help | PU/PD/+/- : Modify |  |  |  |
|  |  | F5 : Old Values |  | (Shift) F2: Color |  |  |
|  |  | F6 : Load BIOS Defaults |  |  |  |  |

Resource	The Award Plug and Play BIOS has the capability

Controlled by	to automatically configure all of the boot and Plug

and Play compatible devices. However, this

capability means absolutely nothing unless you

are using a Plug and Play OS such as Windows 95

Choices are Auto and Manual

Reset Config-	This item allows you to determine whether to reset the

uration Data	configuration data or not.

PCI IRQ	This sets the method by which the PCI bus reco-

Actived By	gnizes that an IRQ service is being requested by a

device. Under all circumstances, you should retain

the default configuration unless advised otherwise

by  your system‘s manufacturer.

Choices are Level and Edge.

PCI IDE IRQ	This allows you to configure your system to the

Map To	type of IDE disk controller in use. If you have

equipped your system with a PCI controller,

changing this allows you to specify which slot holds

the controller and which PCI interrupt (A,B,C,D) is

associated with the connected hard disk. Select

‘PCI Auto’ allows the system to automatically

determine how your IDE disk system is configured.

This section page includes all the items of IDE hard drive and Programmed

Input/Output features. See also Section “Chipset Features Setup”.

ROM PCI/ISA BIOS (2A69HA2B)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (2A69HA2B)

INTEGRATED PERIPHERALS

AWARD SOFTWARE, INC.

| IDE HDD Block Mode | : Enabled | USB Controller |  |  | : Disabled |
| --- | --- | --- | --- | --- | --- |
| IDE Primary Master PIO | : Auto |  |  |  |  |
| IDE Primary Slave PIO | : Auto |  |  |  |  |
| IDE Secondary Master PIO | : Auto |  |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |  |
| On-Chip Primary PCI IDE | : Enabled |  |  |  |  |
| On-Chip Secondary PCI IDE | : Enabled |  |  |  |  |
| PCI Slot IDE 2nd Channel | : Enabled |  |  |  |  |
| Onboard FDD Controller | : Enabled |  |  |  |  |
| Onboard Serial Port 1 | : Auto |  |  |  |  |
| Onboard Serial Port 2 | : Auto | Esc: Quit áâàß |  |  | :Select Item |
| UART 2 Mode | : Standard | F1 : Help | PU/PD/+/- : Modify |  |  |
|  |  | F5 : Old Values |  | (Shift) F2: Color |  |
| Onboard Parallel Port | : 378H/IRQ7 | F6 : Load BIOS Defaults |  |  |  |
| Onboard Parallel Mode | : ECP |  |  |  |  |
| ECP Mode Use DMA | : 3 |  |  |  |  |

IDE HDD Block	This allows your HD controller to use the fast block

Mode	mode to transfer data to and from your HD drive

| Enabled | IDE controller uses block mode |
| --- | --- |
| Disabled | IDE controller uses standard mode |

PCI Slot IDE 	This item allows you designate an IDE controller

2nd Channel 	board inserted into one of the physical PCI slots as

your secondary IDE

| Enabled | External IDE controller designated as the secondary controller |
| --- | --- |
| Disabled | No IDE controller occupying a PCI slot |

On-Chip Primary	This setup item allows you to either enable or

PCI IDE 	disable the primary/secondary controller. You might

On-Chip Second- 	choose to disable the controller if you were to add

ary PCI IDE 	a higher performance or specialized controller.

IDE Primary	PIO - Programmed Input/Output, it allows the

Master/Slave PIO	BIOS to tell the controller what it wants and then

IDE Secondary	let the controller and the CPU to complete the

Master/Slave PIO	task by themselves. This is simpler and more faster.

Your system supports five mods, 0 - 4, which

primarily differ in timing. When Auto is selected,

the BIOS will select the best available mode.

UART 2 Mode	This lets you select the Infrared mode. Choices are

Standard, HPIR, and ASKIR. If you choose HPIR or

ASKIR mode, the screen will show another two

lines to let you choose ‘IR Function Duplex’ (Full

or Half) and‘RxD TxD Active’ (Hi Lo; Lo Hi; Hi Hi;

Lo Lo).

ROM PCI/ISA BIOS (2A69HA2B)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (2A69HA2B)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| Hard Disks Type Size CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Primary Mster : |  |  |  |  |
|  | Select Primary Master Option (N=Skip) : N |  |  |  |
|  |  | Options Size CYLS Head PRECOMP LANDZ Sector Mode |  |  |
|  |  | 2(Y) 1337 648 64 0 2594 63 LBA |  |  |
|  |  | 1 1339 2595 16 65535 2594 63 NORMAL |  |  |
|  |  | 3 1338 1297 32 65535 2594 63 LARGE |  |  |
| Note : Some OS (like SCO-UNIX) must use “Normal” for installation |  |  |  |  |
| ESC : Skip |  |  |  |  |

The last step is 'save and exit'. If you select this item and press 'Y', then these  records will be saved in the CMOS memory on the mainboard. It will be checked every time you turn your computer on.

ROM PCI/ISA BIOS (2A69HA2B)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (2A69HA2B)

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP |  | INTEGRATED PERIPHERALS |  |
| --- | --- | --- | --- |
| BIOS FEATURES SETUP |  | PASSWORD SETTING |  |
| CHIPSET FEATURES SETUP |  | IDE HDD AUTO DETECTION |  |
| POWER MANAGEMENT SETUP |  | SAVE & EXIT SETUP |  |
| PNP/PCI CONFIGURATION |  | EXIT WITHOUT SAVING |  |
| LOAD BIOS DEFAULTS | SAVE to CMOS and EXIT (Y/N):Y |  |  |
| ESC: Quit |  | áâàß:Select Item |  |
| F10: Save & Exit Setup |  | (Shift) F2 : Change Color |  |
| Save Data to CMOS & Exit SETUP |  |  |  |

LOAD BIOS DEFAULTS

When your mainboard has problems and need to

troubleshoot the system, you can use this function.

The default values loaded only affect the BIOS Features

Setup, Chipset Features Setup, Power Management

Setup and PNP/PCI Configuration Setup. There is no

effect on the Standard CMOS Setup. To use this function,

select it from main menu and press <Enter>. A line will

appear on the screen asking if you want to load the BIOS

default values. Press <Yes> and <Enter> then the BIOS

default values will be loaded.

LOAD SETUP DEFAULTS

This allows you to load optimal settings which are stored in

the BIOS ROM. The default values loaded only affect the

BIOS Features Setup, Chipset Features Setup, Power

Management Setup and PNP/PCI Configuration Setup.

There is no effect on the Standard CMOS Setup. To use

this function, select it from main menu and press <Enter>.

A line will appear on the screen asking if you want to load

the Setup default values. Press <Yes> and <Enter> then

the Setup default values will be loaded.

SUPERVISOR PASSWORD / USER PASSWORD

This allows you to set the password. The mainboard defaults with

password disabled.

Enter/Change password : Enter the current password, at

the prompt, key-in your new password (up to eight

alphanumeric characters), press <Enter>. At the next

prompt, confirm the new password by typing it again and press

<Enter>.

Disable password : Press the <Enter> key instead of

entering a new password when the ‘Enter Password’

dialog box appears. A message will appear confirming that

the password is disabled.

If you set both supervisor and user passwords, only the

supervisor password allows you to enter the BIOS SETUP

program.

CAUTION :If you forgot your password, you must disable

the CMOS by turning power off and set JP10 to ‘close‘. And then

open reload the system.

IDE HDD AUTO DETECTION

This allows you to detect the IDE hard disk drivers‘ parameters

and enter them into ‘Standard CMOS Setup’ automatically.

If the auto-detected parameters displayed do not

match the ones that should be used for your hard drive,

do not accept them. Press <N> to reject the values

and enter the correct ones manually on the Standard

CMOS Setup screen.

SAVE & EXIT SETUP

This allows you to save the new setting values in the

CMOS memory and continue with the booting process.

Select what you want to do, press <Enter>.

EXIT WITHOUT SAVING

This allows you to exit the BIOS setup utility without

recording any new values or changing old ones.

¡ Control Key Description ¡

| UP ARROW | ¡ | Move to previous item |
| --- | --- | --- |
| DOWN ARROW | ¡ | Move to next item |
| LEFT ARROW | ¡ | Move to the item in the left hand |
| RIGHT ARROW | ¡ | Move to the item in the right hand |
| Esc KEY | Esc | Main Menu : Quit and not save changes Setup menu : Exit current page and return to main menu |
| PgUp KEY |  | Increase the numeric value or make changes |
| PgDn KEY |  | Decrease the numeric value or make changes |
| F1 KEY | Help | General help |
| F2 KEY | ¡Shift¡+F2 | Change color from total 16 colors |
| F5 KEY | Old Value | Restore the pervious CMOS value from CMOS |
| F6 KEY | Load BIOS default | Load the default CMOS value from BIOS default table |
| F7 KEY | Load setup default | Load Setup default |
| F10 KEY | Save & Exit Setup | Save all the CMOS changes and Exit setup, only for Main Menu |

APPENDIX A

¡¡TECHNICAL SUPPORT REQUEST FORM¡¡

If the mainboard doesn't function properly, please complete the following

information and return it to your system dealer. If the further information is

needed, please attach it.

Model No :  ATC-6100      Date of  Purchase :  ______________

Serial No : ___________________

HARDWARE :

|  | BRAND | MODEL | SPEED | Q'TY |
| --- | --- | --- | --- | --- |
| SIM Module |  |  |  |  |

CPU SPEED : ________ MHz

DRAM :______MB (__ EDO, __FastPage, __ECC)

Hard Disk Interface Controller : _____ IDE, ____ SCSI

Hard Disk Brand :___________, Model :_________, Capacity :________

Display Controller Brand :_____________,Model :__________

Controller Chip Brand :_____________,Model :__________

SOFTWARE:

AWARD SYSTEM BIOS: Version _________  Date Code _____________

Keyboard BIOS: Brand __________

Other Add-on Cards Information:

| Add-on Card | Bus Interface | Model | Remark |
| --- | --- | --- | --- |

| Error Description : |  |
| --- | --- |