CONTENTS

COMPONENT LOCATION DIAGRAM……………………A

CHECK LIST OF THE PACKAGING………………………B

TABLE OF CONTENTS……………………………………….1

HOW TO USE THIS MANUAL……………….……………... 2

CH

APTER 1   Introduction…………………………………..3

1-1 System features	3

1-2 Soft-Off-Control	5

1-3 Wake-On-LAN	.6

1-4 Keyboard & PS/2 Mouse Power On (JP9)……………...………...7

1-5 SB-Link Sideband Signals (JP3)………………………………… 7

1-6 Thermister (option)	.7

1-7 LDCM ( LANDesk Client Manager, option)………………..……8

CHAPTER 2   Installation	8

2-1 Installation Procedure 	.8

2-2 CPU Installation	.9

2-3 System Memory Installation……………………………………..15

2-4 Connectors Description	16

2-5 IDE Driver Installation	21

2-6 VGA & Sound Driver Installation………………………………..22

CHAPTER 3   Award BIOS Setup…………………………36

3-1 Update BIOS Procedure	37
3-1-1 Update PentiumII microcode API…………………………….38
3-2 Award System BIOS Configuration Setup	39

Appendix A  How to Install PentiumII & Celeron CPU?

Appendix B  How to Setup Thermister ?

Appendix C  Technical Support Request Form

##### HOW TO USE THIS MANUAL

To obtain maximum use from this manual it is suggested:

Read Page A COMPONENT LOCATION DIAGRAM and Page B I/O Panel Layout Diagram where you find the mainboard and I/O panel layout diagrams. Please refer to it when you configure the system.

Read Page B CHECK LIST OF THE PACKAGING where you find the packing contents.

Read about an overview of the mainboard features, and how to upgrade as well as to change hardware configurations such as memory size, CPU type, jumper settings lists and connectors in the following categories:

Chapter 1  Introduction

Chapter 2  Installation

When you have finished reading of both chapter 1 and chapter 2, turn to Chapter 3 Award BIOS Setup where you will find the update BIOS procedure and the further information which is stored in the SETUP is the system hardware configuration.

The back of this manual contains APPENDIX A which shows you how to setup INTEL PentiumII and CeleronTM CPU, APPENDIX B which shows you how to setup thermister and APPENDIX C which is a list of request form for technical support.

Your system dealer will set up the mainboard according to your demand of the computer. It means that the current settings of your mainboard may not be the same as the defaults shown in this user's manual. If you need to change your configuration, please ask your dealer first. Be sure this will not void your system warranty, or ask your dealer to do it for you.

REMARK

Intel®  is a registered trademark of Intel Corporation.

All other brands and product names are trademarks registered trademarks of their respective companies.

CHAPTER 1  INTRODUCTION

The ATC-6510 mainboard has ATI RAGETM IIC 3D VGA chipset comes with 4MB SGRAM and Yamaha YMF 715E-S 3D enhanced controller.The ATI RAGETM IIC 3D VGA chipset using AGP interface technique which allows users could have 3D display resolution without plug any of VGA cards and the Yamaha YMF 715E-S 3D enhanced controller using ISA interface which could let your system has 3D stereo sound.

  - SYSTEM FEATURES

- INTEL PentiumII & CeleronTM CPU operating from 233MHz up to 333MHz.

- INTEL 440EX AGPset.

- ATI RageTM IIC 3D AGP VGA chipset comes with 4MB SGRAM.

- Yamaha YMF 715E-S 3D enhanced controller.

- Using two 168-pin DIMM sockets, provides two banks of 64-bit wide path up to 512MB SDRAM or EDO DRAM.

- Built-in Switching Voltage Regulator.(VRM 8.2 SPEC.)

- Supports auto-detect CPU core voltage range 1.8V to 3.5V.

- Supports three PCI slots with revision 2.1 interface compliant and one 16-bit ISA slot.

- Dual Master IDE connectors support Ultra DMA/33, up to four devices in two channels for connecting of high capacity hard drive, CD-ROM disk drive, tape backup etc..

- Supports the USB (Universal Serial Bus) connector.

- PS/2 keyboard connector and PS/2 mouse connector.

- Winbond 83977 high-speed Ultra Multi-I/O chipset.

- Supports Infrared transfer (IrDA TX/RX) connection.

- One FDC port supports two devices up to 2.88MB.

- Two 16550A fast UARTs compatible serial ports.

- One EPP/ECP mode parallel printer port.

- MicroATX Form Factor ; Hardware Dimension is 244mm x 244mm. (9.6" x 9.6")

1-2 SOFTWARE POWER OFF CONTROL

The mainboard design supports Software Power Off Control feature through the SMM code in the BIOS under Windows 95, and MS-DOS operation system environment.

First, you should connect the power switch cable (provided by the MicroATX case supplier) to the connector “PS-ON” on the mainboard.  In the BIOS screen of POWER MANAGEMENT SETUP’, choose “User Defined” (or “Min. Power Saving” or “Max. Power Saving”) in ‘Power Manager’ and choose “Yes” in ‘PM Control by APM’.

In Windows 95, if you would like to power off the system, you just choose “shutdown the computer ?” in the “Shut Down Windows“ from Windows 95, then the system power will be off directly, and become the stand-by status. You will find the power LED light is not blinking (see the table below for the operating status). If you would like to restart the system, just press the power switch button, and the system will be powered on.

Note : If you will leave your system for several days, we suggest you use hardware power off to shutdown your system.

| ### Status | Power LED Light | Turbo LED Light |
| --- | --- | --- |
| Software power off control | Blinking | Light off |
| APM mode | Light on | Light on |
| System running | Light on | Light on |

1-3 Wake-On-LAN

The remote Wake-On-LAN mode of operation is a mechanism that uses Advanced Micro Device Magic Packet technology to power up a sleeping workstation on the network. This mechanism is accomplished when the LAN card receives a specific packet of information, called a Magic Packet, addressed to the node on the network. For additional protection, Secure ON is an optional security feature that can be added to the Magic Packet that requires a password to power up the sleeping workstation. When the LAN card is in remote Wake-On-LAN mode, main system power can be shut down leaving power only for the LAN card and auxiliary power recondition.

The LAN card performs no network activities while in the remote Wake-On-LAN mode of operation. It only monitors the network for receipt of a Magic Packet. If a Magic Packet is addressed to the LAN card on the network, the LAN card wake up the system. If the Secure ON feature has been enabled, the password added to the Magic Packet is also verified prior to waking up the system.

WOL LAN card will provide a 3-pin line to connect the WOL connector on the mainboard.

CAUTION :

For Wake-on-LAN, the +5V standby line for the power supply must capable of

Delievring +5V ±5% at 720mA. Failure to provide adequate standby current when

implementing Wake-on-LAN, can damage the power supply.

Before you enable Wake-on-LAN function, first check your power supply specification to meet the above requirement or not.

1-4 Keyboard & PS/2 Mouse Power On (JP9)

The ATC-6220 mainboard built-in Jumper 9 may have two easily and conveniently ways to power up the system. Users may power on the system by typing a password,

or power on the system by double-clicking the PS/2 mouse.

In addition to satisfying different users will have many different needs, the ATC-6220 mainboard provides four different methods to power up the system:

- KB Power On Password:

This function is available for ATX power only. User can select the power on method to

power on the system. The options are the following :Password : when user select this

option, it will show “Enter Password:”, after user type password, it will show “ Confirm

Password:”, user should type the same password to confirm it. After power off, if user

types the correct password, the system will power on.

- Hot Key Power On:

When user select this option, it will show another line as Hot Key Power ON: Ctrl-F1”’

select any one you like. After power off, if user key in the control-F, it will power up

the system.

- Mouse Left, Mouse Right :

It will power up the system by double click the left or right mouse. Note do not move

your mouse between the first click and the second click.

4. BUTTON ONLY :

Only the power button can power the system.

| JP 9 | KB & PS/2 Wakeup |
| --- | --- |
| 1-2 | Disable |
| 2-3 | Enable |

(a) Disable              (b) Enable

ISA Slots

JP 9

1-5 SB-Link Sideband Signals (JP3)

In order to migrate the legacy Sound Blaster compatible audio to the PCI bus, EMU8008 incorporates a pair of SB-Link request/grant sideband signals (PCPCIR        EQ#A and PCPCISIRQ) to interface to the PCI bus. SB-Link is a mechanism that was defined and developed by Creative as a PCI solution where the system can have the same quality of sound as an ISA sound card when running games under MS-DOS environment.

ISA slot    PCI slots              JP3 2x3 sideband header, sound will also

has a 2x3 header and a cable.

JP 3

1-6 Thermister (option)

This means that user can monitor the CPU temperature through thermister.

When setting up the thermister, the Bios will load the CPU temperature automatically. There is a choice of the warning beep sound if the user set the option on. If the CPU temperature overheated, the user will get the notice from the thermister. This time you should shut down computer and check your devices. Or you can connect with you dealer.

Therefore, protecting and monitoring the CPU temperature is the thermister’s job.

1-7 LDCM （LANDesk Client Manager）(option)

ATC-6220 built-in controller support Intel LDCM. LDCM can satisfy users who want manageable systems that can interact automatically with the user. Client manager is the answer, enabling both administrators and clients to manage systems. The features of LDCM are as following :

- View system inventory

- Client Manager enables you to view hundreds of inventoried items. Some of these items are software related, while many others are hardware related.

- View DMI-compliant component information

- Client manager enables you to view component information that is compliant with the Desktop Manager Interface（DMI）. This means you can manage third-party DMI-compliant components not included with Client manager.

- Back up and restore system configuration files.

- Client Manager enables you back up and restore system configuration files. Whenever you plan on changing the system configuration , you can make a backup set. If the system no longer works correctly, after you the change the system configuration , you can simply restore the system configuration with the backup set.

- Troubleshoot

- Since Client manager enables you to view the system inventory, you can easily troubleshoot system problems.

- Receive notifications for system events

- Client manager enables you to receive notification of certain system events. For example, if the system is running low on virtual memory, you are notified of the potential problem.

- Transfer files to and from client workstations

- As an administrator, you have the ability to transfer files to and from client workstation. This is helpful, for example, when you need to update a client workstation driver.

- Remotely reboot client workstations

- Administrator also have the ability to remotely reboot a workstation . This is helpful when you want system configuration changes to take effect.

CHAPTER 2  INSTALLATION

2-1 INSTALLATION PROCEDURE

Before installing the computer, please prepare all components such as CPU, DRAM; peripherals such as hard drive, keyboard, CD-ROM disk drive and accessories such as cables. Then, install the system as following:

- Plug CPU/ heat sink (refer to PentiumII & CeleronTM installation guide, Appendix A), and DRAM modules on the mainboard.

2. Set DIP switch based on your configuration.

3. Plug add-on cards into PCI/ISA slots, if needed.

4. Connect cables to peripherals, specially for VGA and Sound connector.

5. Make sure all components and devices are well connected, turn on the power and

setup System BIOS based on your configuration.

6. Install peripherals, add-on card drivers and test them.

7. If all of above procedures are success, turn-off the power then plug all of them into

your computer case.

2-2 CPU  INSTALLATION

The 6510 mainboard supports both INTEL PentiumII and CeleronTM CPU.

2-2-1 CPU TYPE SELECTION (by BIOS)

- Press the <Del> key when the system is booting up.

- The following main menu will appear.

ROM PCI/ISA BIOS (ATC-6510)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| AT Clock, DRAM Timings, ....... |  |

- Select “ CHIPSET FEATURES SETUP ”

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| AT Clock, DRAM Timings, ....... |  |

4. Then the following screen will appear.

ROM PCI/ISA BIOS (ATC-6510)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Auto Configuration | : Enabled | SDRAM CAS latency Time |  | : 3 |  |
| --- | --- | --- | --- | --- | --- |
| DRAM Speed Selection | : 60 ns | CPU Clock Ratio |  | : 2.0x |  |
| MA Wait State | : Slow | Auto Detect DIMM/PCI ClK |  | : Enabled |  |
| EDO RAS# To CAS# Delay | : 3 | CPU Clock Frequency |  | : 66MHz |  |
| EDO RAS# Precharge Time | : 3 | Spread Spectrum Modulated |  | : Disabled |  |
| EDO DRAM Read Burst | : x333 | *Following menus are |  | optional* |  |
| EDO DRAM Write Burst | : x222 | *CPU Warning Temperature |  | : Disabled |  |
| CPU-To-PCI IDE Posting | : Disabled | *Current System Temperature |  | : 20 o C/ 68 o F |  |
| System BIOS Cacheable | : Disabled | *Current CPU Temperature |  | : 20 o C/ 68 o F |  |
| Video BIOS Cacheable | : Disabled | *Current ChassisFAN Speed |  | : 0 PRM |  |
| Video RAM Cacheable | : Disabled | *Current PWRFAN Speed |  | : 0 PRM |  |
| 8 Bit I/O Recovery Time | : 3 | *Current CPUFAN Speed |  | : 0 RPM |  |
| 16 Bit I/O Recovery Time | : 2 | *Vcore : 2.80V VTT(V) |  | : 1.47V |  |
| Memory Hole At 15M-16M | : Disabled | *+3.3V : 3.45V +5V |  | : 4.91V |  |
| Passive Release | : Enabled | *+12V : 12.22V –12V |  | : -11.76 |  |
| Delayed Transaction | : Disabled | *-5V : -5.16V |  |  |  |
| AGP Aperture Size (MB) | : 128 | Esc: Quit | :Select Item |  |  |
| SDRAM RAS-to-CAS Delay | : Slow | F1 : Help | PU/PD/+/-:Modify |  |  |
| SDRAM RAS Precharge Time | : Slow | F5 : Old Values |  |  | (Shift)F2 :Color |
|  |  | F6 :Load BIOS Defaults |  |  |  |
|  |  | F7 :Load Setup Defaults |  |  |  |

※※ NOTE

If an incorrect CPU frequency is set through the BIOS, a system failure may occur. This system failure can be solved by pressing the “Insert” key on the keyboard to clear up the previous set frequency (i.e. back to 133MHz (66MHz x 2.0) or the default frequency), and then the power button to restart the system and reset the value in BIOS screen. Or user can clear CMOS to reset the BIOS value.

Also the ATC-6510 mainboard supports keyboard power on or mouse power on function, (refer to ‘Integrated Peripherals’, page48,49). When you set ‘keyboard power on’, you can select set password or not. When you set password, you should key-in the password to power on the system. If you forget the password, you should clear CMOS to reset the system.

5a. Intel PentiumII 233MHz (66 x 3.5)

ROM PCI/ISA BIOS (ATC-6510)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Auto Configuration | : Enabled | SDRAM CAS latency Time |  | : 3 |  |
| --- | --- | --- | --- | --- | --- |
| DRAM Speed Selection | : 60 ns | CPU Clock Ratio |  | : 3.5x |  |
| MA Wait State | : Slow | Auto Detect DIMM/PCI ClK |  | : Enabled |  |
| EDO RAS# To CAS# Delay | : 3 | CPU Clock Frequency |  | : 66MHz |  |
| EDO RAS# Precharge Time | : 3 | Spread Spectrum Modulated |  | : Disabled |  |
| EDO DRAM Read Burst | : x333 | *Following menus are |  | optional* |  |
| EDO DRAM Write Burst | : x222 | *CPU Warning Temperature |  | : Disabled |  |
| CPU-To-PCI IDE Posting | : Disabled | *Current System Temperature |  | : 20 o C/ 68 o F |  |
| System BIOS Cacheable | : Disabled | *Current CPU Temperature |  | : 20 o C/ 68 o F |  |
| Video BIOS Cacheable | : Disabled | *Current ChassisFAN Speed |  | : 0 PRM |  |
| Video RAM Cacheable | : Disabled | *Current PWRFAN Speed |  | : 0 PRM |  |
| 8 Bit I/O Recovery Time | : 3 | *Current CPUFAN Speed |  | : 0 RPM |  |
| 16 Bit I/O Recovery Time | : 2 | *Vcore : 2.80V VTT(V) |  | : 1.47V |  |
| Memory Hole At 15M-16M | : Disabled | *+3.3V : 3.45V +5V |  | : 4.91V |  |
| Passive Release | : Enabled | *+12V : 12.22V –12V |  | : -11.76 |  |
| Delayed Transaction | : Disabled | *-5V : -5.16V |  |  |  |
| AGP Aperture Size (MB) | : 128 | Esc: Quit | :Select Item |  |  |
| SDRAM RAS-to-CAS Delay | : Slow | F1 : Help | PU/PD/+/-:Modify |  |  |
| SDRAM RAS Precharge Time | : Slow | F5 : Old Values |  |  | (Shift)F2 :Color |
|  |  | F6 :Load BIOS Defaults |  |  |  |
|  |  | F7 :Load Setup Defaults |  |  |  |

5b. Intel PentiumII & CeleronTM  266MHz (66 x 4.0)

ROM PCI/ISA BIOS (ATC-6510)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Auto Configuration | : Enabled | SDRAM CAS latency Time |  | : 3 |  |
| --- | --- | --- | --- | --- | --- |
| DRAM Speed Selection | : 60 ns | CPU Clock Ratio |  | : 4.0x |  |
| MA Wait State | : Slow | Auto Detect DIMM/PCI ClK |  | : Enabled |  |
| EDO RAS# To CAS# Delay | : 3 | CPU Clock Frequency |  | : 66MHz |  |
| EDO RAS# Precharge Time | : 3 | Spread Spectrum Modulated |  | : Disabled |  |
| EDO DRAM Read Burst | : x333 | *Following menus are |  | optional* |  |
| EDO DRAM Write Burst | : x222 | *CPU Warning Temperature |  | : Disabled |  |
| CPU-To-PCI IDE Posting | : Disabled | *Current System Temperature |  | : 20 o C/ 68 o F |  |
| System BIOS Cacheable | : Disabled | *Current CPU Temperature |  | : 20 o C/ 68 o F |  |
| Video BIOS Cacheable | : Disabled | *Current ChassisFAN Speed |  | : 0 PRM |  |
| Video RAM Cacheable | : Disabled | *Current PWRFAN Speed |  | : 0 PRM |  |
| 8 Bit I/O Recovery Time | : 3 | *Current CPUFAN Speed |  | : 0 RPM |  |
| 16 Bit I/O Recovery Time | : 2 | *Vcore : 2.80V VTT(V) |  | : 1.47V |  |
| Memory Hole At 15M-16M | : Disabled | *+3.3V : 3.45V +5V |  | : 4.91V |  |
| Passive Release | : Enabled | *+12V : 12.22V –12V |  | : -11.76 |  |
| Delayed Transaction | : Disabled | *-5V : -5.16V |  |  |  |
| AGP Aperture Size (MB) | : 128 | Esc: Quit | :Select Item |  |  |
| SDRAM RAS-to-CAS Delay | : Slow | F1 : Help | PU/PD/+/-:Modify |  |  |
| SDRAM RAS Precharge Time | : Slow | F5 : Old Values |  |  | (Shift)F2 :Color |
|  |  | F6 :Load BIOS Defaults |  |  |  |
|  |  | F7 :Load Setup Defaults |  |  |  |

5c. Intel PentiumII & CeleronTM  300MHz (66 x 4.5)

ROM PCI/ISA BIOS (ATC-6510)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Auto Configuration | : Enabled | SDRAM CAS latency Time |  | : 3 |  |
| --- | --- | --- | --- | --- | --- |
| DRAM Speed Selection | : 60 ns | CPU Clock Ratio |  | : 4.5x |  |
| MA Wait State | : Slow | Auto Detect DIMM/PCI ClK |  | : Enabled |  |
| EDO RAS# To CAS# Delay | : 3 | CPU Clock Frequency |  | : 66MHz |  |
| EDO RAS# Precharge Time | : 3 | Spread Spectrum Modulated |  | : Disabled |  |
| EDO DRAM Read Burst | : x333 | *Following menus are |  | optional* |  |
| EDO DRAM Write Burst | : x222 | *CPU Warning Temperature |  | : Disabled |  |
| CPU-To-PCI IDE Posting | : Disabled | *Current System Temperature |  | : 20 o C/ 68 o F |  |
| System BIOS Cacheable | : Disabled | *Current CPU Temperature |  | : 20 o C/ 68 o F |  |
| Video BIOS Cacheable | : Disabled | *Current ChassisFAN Speed |  | : 0 PRM |  |
| Video RAM Cacheable | : Disabled | *Current PWRFAN Speed |  | : 0 PRM |  |
| 8 Bit I/O Recovery Time | : 3 | *Current CPUFAN Speed |  | : 0 RPM |  |
| 16 Bit I/O Recovery Time | : 2 | *Vcore : 2.80V VTT(V) |  | : 1.47V |  |
| Memory Hole At 15M-16M | : Disabled | *+3.3V : 3.45V +5V |  | : 4.91V |  |
| Passive Release | : Enabled | *+12V : 12.22V –12V |  | : -11.76 |  |
| Delayed Transaction | : Disabled | *-5V : -5.16V |  |  |  |
| AGP Aperture Size (MB) | : 128 | Esc: Quit | :Select Item |  |  |
| SDRAM RAS-to-CAS Delay | : Slow | F1 : Help | PU/PD/+/-:Modify |  |  |
| SDRAM RAS Precharge Time | : Slow | F5 : Old Values |  |  | (Shift)F2 :Color |
|  |  | F6 :Load BIOS Defaults |  |  |  |
|  |  | F7 :Load Setup Defaults |  |  |  |

5d. Intel PentiumII & CeleronTM  333MHz (66 x 5.0)

ROM PCI/ISA BIOS (ATC-6510)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Auto Configuration | : Enabled | SDRAM CAS latency Time |  | : 3 |  |
| --- | --- | --- | --- | --- | --- |
| DRAM Speed Selection | : 60 ns | CPU Clock Ratio |  | : 5.0x |  |
| MA Wait State | : Slow | Auto Detect DIMM/PCI ClK |  | : Enabled |  |
| EDO RAS# To CAS# Delay | : 3 | CPU Clock Frequency |  | : 66MHz |  |
| EDO RAS# Precharge Time | : 3 | Spread Spectrum Modulated |  | : Disabled |  |
| EDO DRAM Read Burst | : x333 | *Following menus are |  | optional* |  |
| EDO DRAM Write Burst | : x222 | *CPU Warning Temperature |  | : Disabled |  |
| CPU-To-PCI IDE Posting | : Disabled | *Current System Temperature |  | : 20 o C/ 68 o F |  |
| System BIOS Cacheable | : Disabled | *Current CPU Temperature |  | : 20 o C/ 68 o F |  |
| Video BIOS Cacheable | : Disabled | *Current ChassisFAN Speed |  | : 0 PRM |  |
| Video RAM Cacheable | : Disabled | *Current PWRFAN Speed |  | : 0 PRM |  |
| 8 Bit I/O Recovery Time | : 3 | *Current CPUFAN Speed |  | : 0 RPM |  |
| 16 Bit I/O Recovery Time | : 2 | *Vcore : 2.80V VTT(V) |  | : 1.47V |  |
| Memory Hole At 15M-16M | : Disabled | *+3.3V : 3.45V +5V |  | : 4.91V |  |
| Passive Release | : Enabled | *+12V : 12.22V –12V |  | : -11.76 |  |
| Delayed Transaction | : Disabled | *-5V : -5.16V |  |  |  |
| AGP Aperture Size (MB) | : 128 | Esc: Quit | :Select Item |  |  |
| SDRAM RAS-to-CAS Delay | : Slow | F1 : Help | PU/PD/+/-:Modify |  |  |
| SDRAM RAS Precharge Time | : Slow | F5 : Old Values |  |  | (Shift)F2 :Color |
|  |  | F6 :Load BIOS Defaults |  |  |  |
|  |  | F7 :Load Setup Defaults |  |  |  |

2-3 SYSTEM MEMORY INSTALLATION

The ATC-6510 provides two 168-pin DIMM sockets for system memory expansion from 8MB to 512MB. These two DIMMs are arranged to two banks,

please refer to page A. Each bank provides 64-bit wide data path.

※ Samples of System Memory Combinations Options ※

| BANK0 | BANK1 | Total Memory |
| --- | --- | --- |
| DIMM 1 | DIMM 2 | DIMM 1-2 |
| 8MBx1 | - | 8MB |
| - | 8MBx1 | 8MB |
| 8MBx1 | 8MBx1 | 16MB |
| 16MBx1 | - | 16MB |
| - | 16MBx1 | 16MB |
| 8MBx1 | 16MBx1 | 24MB |
| 16MBx1 | 8MBx1 | 24MB |
| 16MBx1 | 16MBx1 | 32MB |
| 32MBx1 | - | 32MB |
| - | 32MBx1 | 32MB |
| 32MBx1 | 32MBx1 | 64MB |
| 32MBx1 | 64MBx1 | 96MB |
| 64MBx1 | 32MBx1 | 96MB |
| 64MBx1 | 64MBx1 | 128MB |
| 128MBx1 | - | 128MB |
| - | 128MBx1 | 128MB |
| 128MBx1 | 128MBx1 | 256MB |
| : | : | : |
| 256MBx1 | 256MBx1 | 512MB |

2-4 CONNECTORS  DESCRIPTION

The locations of following connectors are indicated in page A. When you plug wires into the following connector of CONN1, you should have the pin 1 edge of the wires align with the pin 1 end of the connector.

CONN1 : speaker, keyboard lock, reset, SMI, turbo LED,

IDE LED and PS_ON connectors.

SPK          KEYLOCK

RST  EXTSMI  TB-LED IDE-LED

+       +

SPK : speaker

Speaker

GND

GND

VCC

Power LED connector

Power LED +

N/C

GND (power)

Keylock

GND

PS ON

RST : Reset connector

Reset Signal

GND

#### Slot 1

SMI : SMI lead

GND

SMI Signal

TB-LED : Turbo LED indicator, LED on

when system runs higher speed.

GND

+5V

IDE-LED : IDE devices indicator LED

connector. IDE-LED stays ON indicates

LED signal  on-board IDE devices in operation. The

+5V        red wire of the HDD connector must be

connected to the pin which is +5V.

Pin 12                 PS_ON : Power Button

Pin 24                 Pin 12 : PS_ON      Pin 24 : +5VSB

(3) CPU FAN

(2) Chassis FAN

Slot 1

FAN : CPU cooling fan connector. Wire with

+12V voltage (most likely red wire) must be

plugged into pin2, and GROUND wires

(most likely black wire) must be plugged into

pin3. Please confirm the wire color

(1) POWER FAN

re-presentation with your supplier.

1      GND  CAUTION: Plug the wire into wrong connector will

2      +12V     DAMAGE fan and mainboard.

3      Sense     	FAN1 for Power Fan.

FAN2 for Chassis Fan.

FAN3 for CPU Fan.

IR1 : Infrared module connector.

1 	+5V

- FIRRX

- IRRX

4	GND

5	IRTX

Slot 1

PW1 : ATX mode +3.3/5/12V power supply

20             11        connector.

| 10 1 1 | 3.3V | 6 | +5V | 11 | 3.3V | 16 | GND |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 3.3V | 7 | GND | 12 | -12V | 17 | GND |
| 3 | GND | 8 | PWRGD | 13 | GND | 18 | -5V |
| 4 | +5V | 9 | 5VSB | 14* | PS_ON | 19 | +5V |
| 5 | GND | 10 | +12V | 15 | GND | 20 | +5V |

* PS_ON : Soft-Off power control

COM2 : This connector is used to connect

serial port cables.

Slot 1

| pin 10 5 9 4 8 3 7 2 6 1 | Signal name |
| --- | --- |
| 1 | NDCDA |
| 2 | NSINA |
| 3 | NSOUTA |
| 4 | NDTRA |
| 5 | GND |
| 6 | NDSRA |
| 7 | NRTSA |
| 8 | NCTSA |
| 9 | NRIA |

Slot 1

FDC1 : this connector is used to connect the floppy

drive through a cable.

| pin | signal | Pin | signal |
| --- | --- | --- | --- |
| 2 | RWC- | 20 | STEP- |
| 4 34 33 2 1 | Reserved | 22 | Write Data |
| 6 | FDEDIN | 24 | Write Gate |
| 8 | Index- | 26 | Track 00- |
| 10 | Motor EnableA- | 28 | Write Protect- |
| 12 | Drive Sele.B- | 30 | Read Data- |
| 14 | Drive Sele.A- | 32 | Side 1 Sele.- |
| 16 | Motor EnableB- | 34 | DisketteChange |
| 18 | DIR- |  |  |
| All of odd pins are ground |  |  |  |

Slot 1    LPT : this connector is used to connect parallel

port cable.

| Pin | Signal | pin | Signal |
| --- | --- | --- | --- |
| 1 | STROBE- | 10 | ACK- |
| 2 | Data Bit 0 | 11 | BUSY |
| 3 | Data Bit 1 | 12 | PE |
| 4 25 13 13 1 | Data Bit 2 | 13 | SLCT |
| 5 | Data Bit 3 | 14 | Auto Feed- |
| 6 | Data Bit 4 | 15 | ERROR- |
|  | Data Bit 5 | 16 | INIT- |
| 8 | Data Bit 6 | 17 | SLCT IN- |
| 9 | Data Bit 7 |  |  |
| ###### Pin18 -- pin25 are GND |  |  |  |

LPT1  COM1  VGA  MIDI/Game Port

PS/2 Mouse

Keyboard conn.

Audio Line In

Audio Mic In

USB 1   USB 0   Audio Line Out

IDE1/IDE2 : these two connectors are used to

connect IDE devices through IDE cables,

a total of 4 devices can be connected.

40      39

4        3

2	1

| pin | signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Reset IDE | 21 | DDRQ0(1) |
| 2 | GND | 22 | GND |
| 3 | Host Data 7 | 23 | I/O Write- |
| 4 | Host Data 8 | 24 | GND |
| 5 | Host Data 6 | 25 | I/O Read- |
| 6 | Host Data 9 | 26 | GND |
| 7 | Host Data 5 | 27 | IORDY |
| 8 | Host Data 10 | 28 | N/C |
| 9 | Host Data 4 | 29 | DDAK0- (1-) |
| 10 | Host Data 11 | 30 | GND |
| 11 | Host Data 3 | 31 | IRQ14* |
| 12 | Host Data 12 | 32 | IOCS16- |
| 13 | ###### Host Data 2 | 33 | Addr 1 |
| 14 | Host Data 13 | 34 | N/C |
| 15 Slot 1 | Host Data 1 | 35 | Addr 0 |
| 16 | Host Data 14 | 36 | Addr 2 |
| 17 | Host Data 0 | 37 | ChipSele.1P- |
| 18 | Host Data 15 | 38 | ChipSele.3P- |
| 19 | GND | 39 | Activity |
| 20 | Key | 40 | GND |

* IDE1 : pin31 is IRQ14;

IDE2 : pin31 is IRQ15 or MIRQ0

2-5 IDE DRIVER INSTALLATION

The IDE driver installation procedure is as following :

Setup for Windows 95 :

1. Starting Windows 95

2. Put the All-In-One CD into your CD-ROM drive.

3. Select “START”, “RUN”.

4. Type “D:\WIN95\SETUP.EXE”.

(If your CD-ROM is not the drive D, type the appropriate letter instead.)

5. Restart computer, then follow the instructions on your screen to install

new IDE driver we offer in the All-In-One CD.

6. Exit Windows 95, turn power off; then turn power on.

After installation, the screen will show a yellow ！, please ignore it.

(The other platforms please refer to readme file.)

2-6 VGA & SOUND DRIVER INSTALLATION

###### The All-In-One CD contains one “readme.txt” and three (“ATI AGP Driver”, “Yamaha 715 ISA driver” , “Yamaha 724 PCI driver”, “IDE driver”, “A-Trend Utility”, “Microcode API driver”) software. The readme file describes the ATI

###### AGP and Yamaha sound card on board installation utility usages file organization

###### of the CD. The GUI installation items are described in the following:

Read me :

When user clicks this item, it will call notepad to open the readme file. The readme file describes the GUI installation items.

###### Install ATI AGP Driver :

###### When user click this item, it will automatically install the AGP Driver, and ask user to restart the system.

Install Yamaha 715 ISA Driver :

Before click this item, please refer to your user manual to make sure your on board sound card is Yamaha 715 ISA. When user click this item, it will call notepad to open a proper readme file. Inside the readme file, it will guide users to go through the

driver installation procedures.

###### Install Yamaha 724 PCI Driver :

###### Before click this item, please refer to your user manual to make sure your on board sound card is Yamaha 724 PCI. When user click this item, it will call notepad to open a proper readme file. Inside the readme file, it will guide users to go through the driver installation procedures.

Browse this CD :

It will allow users to browse the installation CD-ROM.

Exit :

Click this item will exit the installation program.

2-6-1 Installing ATI AGP Driver For WIN95

1. Choose “Install ATI AGP Driver”.

Fig. 1

- In ATI Installation Windows, choose “Easy Install (Display drivers and ATI

Player)”.

Fig. 2

3. Choose “Next”.

Fig. 3

4. Choose “I Agree with this Agreement”.

Fig. 4

5. Choose “ Express Setup”.

Fig. 5

6. All files are copied to the system.

Fig. 6

7. Choose “OK”.

Fig. 7

8. Choose “Yes” to restart your computer.

Fig. 8

2-6-2 Installing ATI AGP Driver For WinNT

1. Choose “Install ATI AGP Driver”.

Fig. 1

2. In Display Properties Windows, choose “setting” and then choose “OK”.

Fig. 2

3. Choose “Change”.

4. Choose “Have Disk”.

Fig. 3

Fig. 4

Fig. 4

5. Type “D:\ATiAGP\WINNT, and then choose “OK”.

Fig. 5

6. Type “ATI Technologies Inc. 3D PRO”, then choose “OK”.

Fig. 6

7. Choose “OK”.

Fig. 7

8. Choose “Change”.

Fig. 8

9. Choose “close”, then restart your computer.

Fig. 9

CHAPTER 3  Award BIOS SETUP

Award BIOS manufacturer provides access to the system BIOS through the hardware

and software on each ATC-6510 mainboard. The hardware consists of a Flash ROM

and the software is a group of programs that are installed in the ROMBIOS along with all the other data the BIOS must contain.

The ATC-6510 mainboard will require special driver supplied by the manufacturer to update the BIOS SETUP program. It is a good idea to read the next page for details for update BIOS driver installation or you can ask your system dealer to do it for you.

When the driver has been successfully updated, it is very important to contact your system dealer to change the CMOS settings for your computer. The CMOS settings are shown in the following pages.

NOTE : To clear CMOS you should unplug the power cord, then set 2-3 to clear, put it back to normal position and plug the power cord again.

|  | JP8 |
| --- | --- |
| Normal | 1-2 |
| Clear | 2-3 |

Normal CMOS              Clear CMOS

ISA slot   PCI slots   JP 8

3-1 UPDATE BIOS PROCEDURE

If the BIOS needs to be updated, you can get a CD with the updated BIOS

from your system supplier. The BIOS diskette includes :

“awdflash.exe”  --  BIOS update utility program

“awdflash.doc”

“(update BIOS filename with version number).bin”

The update procedure is as following:

- Boot the system to DOS mode in a normal manner.

- Insert the updated diskette to drive D (or E).

- Change working directory to floppy drive, D or E, which contains the update BIOS diskette. --  Type “d:\” or “e:\”, “ENTER”.

- Run the BIOS update utility  --  Type “awdflash”, “ENTER”.

- Type “(update BIOS file name with version number).bin”, ENTER.

- If you do not want to save the old BIOS Type “N” when the screen displays the message : " Do you want to save BIOS (Y/N) ?".

- Type “Y“ when the screen shows the message : " Are you sure to program (Y/N) ?".

- Follow instructions displayed on the screen.  DO NOT remove the update BIOS CD from the CD-ROM drive nor turn the system power off until the BIOS update is completed.

- Turn the power off. Clear the data in CMOS according to the procedure described in the previous page.

- Turn the system power on and test that your system is working properly.

3-1-1 UPDATE PENTIUMII MICROCODE API

Intel also provides MICROCODE API(Applications Programming Interface) for PentiumII processor-based mainboard user to update data block in BIOS quickly and easily. (You can find this utility in the All-In-One CD in the mainboard package).

The BIOS code on the PentiumII processor-based mainboards contains data that is specific to each silicon stepping of the processor. Integrators must ensure that this BIOS stepping data matches the processor stepping used. When the BIOS does not contain stepping data that matches the processor stepping, integrators must update the data in the BIOS before shipping the system. Historically, PentiumII systems have been updated by replacing the entire BIOS with a new revision of BIOS that contains the correct stepping data.

Intel‘s BIOS update API allows just the stepping data within the BIOS to be updated as needed. Mainboards that contain a BIOS with the Intel-defined BIOS update API can be quickly and easily updated, if required, without obtaining a complete BIOS upgrade. Using this utility, integrators can easily verify that the correct stepping data is present in all PentiumII processor-based mainboards. However, if the stepping data requires updating, the mainboard BIOS must contain the Intel-defined BIOS update API, otherwise a complete BIOS upgrade is required from the mainboard vendor.

Put the All-In-One CD into your CD-ROM drive, e.g. drive D, and then type

D:\>”ENTER”, and type \api\checkup3.

The main menu should now be displayed, showing the following four options :

1) Check and load update

2) Specify stepping data file [current : pep.pdb]

3) Help

4) Quit without loading update

Select 1 to know the stepping filename, select 2 to load right patch code, then select 1 to update proper patch code. Now, the screen will show the message “please remove floppy diskette from floppy disk drive”. Then cold boot (mechanical power off) system to continue. For more information, please refer to “CHECKUP.HLP“ file.

3-2  Award SYSTEM BIOS

CONFIGURATION SETUP

The following pages explain how to set up the system configuration (CMOS) under the Award BIOS. The SETUP program is stored in the Read-Only-Memory (ROM) on the mainboard. To do the SETUP procedure, press the <Del> key when the system is booting up. The following main menu will appear. Please select " STANDARD CMOS SETUP" to enter the next screen.

ROM PCI/ISA BIOS (ATC-6510)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | âàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

The section on the bottom of the main menu explains how to control this screen.  The other section displays the items highlighted in the list.

This screen records some basic hardware information, and sets the system clock and error handling. These records can be lost or corrupted if the on-board battery has failed or is weak.

ROM PCI/ISA BIOS (ATC-6510)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (ATC-6510)

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| Date (mm:dd:yy) : Wed, May 20 1998 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time(hh:mm:ss) : 13 : 37 : 14 |  |  |  |  |  |  |  |  |
| HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |  |  |  |  |  |  |
| Primary Master : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Primary Slave : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Secondary Master : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Secondary Slave : Auto | 0 | 0 |  |  | 0 | 0 0 | 0 | Auto |
| Drive A : 1.44M, 3.5 in. |  |  |  |  |  |  |  |  |
| Drive B : None |  |  | Base Memory : 640K |  |  |  |  |  |
| Floppy 3 Mode Support : Disabled |  |  | Extended memory : 7168K |  |  |  |  |  |
|  |  |  | Other Memory : 384K |  |  |  |  |  |
| Video : EGA/VGA |  |  | ------------------------------------------------ |  |  |  |  |  |
| Halt On: All Errors |  |  | Total Memory : 8192K |  |  |  |  |  |
| ESC : Quit áâàß:Select Item PU/PD/+/- : Modify |  |  |  |  |  |  |  |  |
| F1 : Help (Shift) F2 : Change Color |  |  |  |  |  |  |  |  |

Date	The date format is <day>, <date><month><year>.

Press<F3> to show the calendar.

| Day | The day, from Sun to Sat, determined by the BIOS and is display-only |
| --- | --- |
| Date | The date, from 1 to 31 |
| Month | The month, Jan. through Dec. |
| Year | The year, from 1900 to 2099 |

Time	The time format is <hour><minute><second>. The time is

calculated based on the 24-hour military-time clock.

For example, 1p.m. is 13:00:00.

Primary Master	These categories identify the types of the 2 channels

Primary Slave	that have been installed in the computer. There

Secondary Master	are 45 predefined types and 4 user definable

Secondary Slave	types are for Enhanced IDE BIOS. Type 1 to 45

are predefined. Type ‘user’ is user-definable.

Press PgUp/PgDn to select a numbered hard

disk type or type the number and press<Enter>.

If you select ‘Auto’, the BIOS will auto-detect the

HDD & CD-ROM Drive at the POST stage and

show the IDE for HDD & CD-ROM Drive. If

you select ‘user’, you will need to know the

information listed below. Enter the information 						directly from the keyboard and press <Enter>.

This information should be from your hard disk

vender or dealer. If the controller of the HDD

interface is ESDI, the selection shall be ‘Type 1’;

if SCSI, the selection shall be ‘None’.

If no device is installed select

‘NONE’ and press <Enter>.

| Type | drive type |
| --- | --- |
| SIZE | Automatically adjusts |
| CYLS | number of cylinders |
| HEAD | number of heads |
| PRECOMP | write precom |
| LANDZ | landing zone |
| SECTOR | number of sectors |
| MODE | mode type |

Drive A	This category identifies the types of floppy disk drive

Drive B	A or drive B that have been installed in the computer.

| None | No floppy drive installed |
| --- | --- |
| 360K, 5.25 in | 5.25“ PC-type 360KB capacity |
| 1.2M, 5.25 in | 5.25“ AT-type 1.2MB capacity |
| 720K, 3.5 in | 3.5“ double-side 720KB capacity |
| 1.44M, 3.5 in | 3.5“ double-side 1.44MB capacity |
| 2.88M, 3.5 in | 3.5“ double-side 2.88MB capacity |

Floppy 3 Mode	This is the Japanese standard floppy drive. This standard

Support	stores 1.2MB in a 3.5” diskette

Video	This category selects the type of video adapter used for the

primary system monitor. Although secondary monitors are

supported, you do not have to select the type in Setup.

| EGA/VGA | Enhanced Graphics Adapter/Video Graphics Array. For EGA, VGA, SEGA, SVGA or PGA monitor adapters |
| --- | --- |
| CGA 40 | Color Graphics Adapters, power up in 40 column mode |
| CGA 80 | Color Graphics Adapters, power up in 80 column mode |
| MONO | Monochrome adapter, includes high resolution monochrome adapters |

Halt On	This category determines whether the computer

will stop if an error is detected during power up.

| No errors | The system boot will not be stopped for any error that may be detected |
| --- | --- |
| All errors | When the BIOS detects a non-fatal error the system will be stopped and you will be prompted |
| All, But Keyboard | The system boot will not stop for a keyboard error, it will stop for all other errors |
| All, But Diskette | The system boot will not stop for a disk error, it will stop for all other errors |
| All, But Disk/Key | The system boot will not stop for a disk or keyboard error, it will stop for all other errors |

Memory	          This category is display-only which is determined by POST

(Power On Self Test) of the BIOS.

Base Memory  The POST will determine the amount of base

(or conventional) memory installed in the system. The value of

the base memory is typically 512K or 640K based on the

memory installed on the motherboard.

Extended Memory  How much extended memory is present

during the POST. This is the amount of memory located above

1MB in the CPU‘s memory address map.

Other Memory  This refers to the memory located in the 640K

to 1024K address space. This is memory that can be used for

different applications. DOS users this area to load device drivers

in an effort to keep as much base memory free for application

programs. The BIOS is the most frequent user of this RAM area

since this is where it shadows RAM.

This screen is a list of system configuration options. Some of them are defaults required by the mainboard's design, others depend on the features of your system.

ROM PCI/ISA BIOS (ATC-6510)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Virus, Protection, Boot Sequence |  |

ROM PCI/ISA BIOS (ATC-6510)

BIOS FEATURES SETUP

AWARD SOFTWARE, INC.

| Virus Warning | : Disabled | Video BIOS Shadow | : Enabled |
| --- | --- | --- | --- |
| CPU Internal Cache | : Enabled | C8000-CBFFF Shadow | : Disabled |
| External Cache | : Enabled | CC000-CFFFF Shadow | : Disabled |
| Quick Power On Self Test | : Enabled | D0000-D3FFF Shadow | : Disabled |
| Boot Sequence | : A,C,SCSI | D4000-D7FFF Shadow | : Disabled |
| Swap Floppy Drive | : Disabled | D8000-DBFFF Shadow | : Disabled |
| Boot Up Floppy Seek | : Enabled | DC000-DFFFF Shadow | : Disabled |
| Boot Up NumLock Status | : On |  |  |
| Boot Up System Speed | : High |  |  |
| Gate A20 Option | : Fast |  |  |
| Typematic Rate Setting | : Disabled |  |  |
| Typematic Rate(Chars/Sec) | : 6 | Esc : Quit áâàß:Select Item |  |
| Typematic Delay(Msec) | : 250 | F1 : Help PU/PD/+/- : Modify |  |
| Security Option | : Setup | F5 : Old Values (SHIFT) F2 : Color |  |
| PCI/VGA Palette Snoop | : Disabled | F6 : Load BIOS Defaults |  |
| Assign IRQ For VGA | : Enabled | F7 : Load Setup Defaults |  |
| OS Select for DRAM>64MB | : Non-OS2 |  |  |
| Report No FDD For Win95 | : No |  |  |

Virus Warning	When this item is enabled, the Award BIOS will monitor the

boot sector and partition table of the hard disk drive for any

attempt at modification. If an attempt is made, the BIOS will

halt the system and the following error message will appear.

Afterwards, if necessary, you will be able to run an antivirus

program to locate and remove the problem before any damage

is done.

! WARNING ! Disk boot sector is to be modified Type ‘Y’ to accept write or ‘N’ to abort write Award Software, Inc.

| Enabled | Activates automatically when the system boots up, if anything attempts to access the boot sector or hard disk partition table will cause a warning message to appear. |
| --- | --- |
| Disabled | No warning message will appear when anything attempts to access the boot sector or hard disk partition table. |

Many disk diagnostic programs which attempt to

access the boot sector table can cause the above warning

message. If you will be running such a program, we

recommend that you first disable Virus Protection beforehand.

CPU	These two categories speed up memory access.

Internal Cache	However, it depends on CPU/chipset design.

External Cache	The default value is ‘enabled‘.

Quick Power On	This category speeds up Power On Self Test

Self Test	after you power up the computer. If you set 							Enabled, BIOS will shorten or skip some checked

items during POST.

Boot Sequence	This category determines which drive is to search

first for the Disk Operating System (i.e., DOS).

| #### A, C, SCSI | System will first search for floppy disk drive then hard disk drive, and the next is SCSI device. |
| --- | --- |
| #### C, A, SCSI | System will first search for hard disk drive then floppy disk drive, and the next is SCSI device. |
| #### C, CDROM, A | System will first search for hard disk drive then CDROM drive, and the next is floppy disk drive. |
| #### CDROM, C, A | System will first search for CDROM drive then hard disk drive, and the next is floppy disk drive. |
| #### D, A, SCSI | System will first search for secondary hard disk drive then floppy disk drive, and the next is SCSI device. |
| #### E, A, SCSI | System will first search for third hard disk drive then floppy disk drive, and the next is SCSI device. |
| #### F, A, SCSI | System will first search for fourth hard disk drive then floppy disk drive, and the next is SCSI device. |
| #### SCSI, A, C | System will first search for SCSI device then floppy disk drive, and the next is hard disk drive. |
| #### SCSI, C, A | System will first search for SCSI device then hard disk drive, and the next is floppy disk drive. |
| #### C only | System will search for hard disk drive only. |
| #### LS/ZIP, C | System will first search for LS120 or IOMEGA (ZIP) drive, and the next is hard disk drive. |

C is primary master; D is primary slave;

E is secondary master, F is secondary slave

Swap Floppy	This item allows you to determine whether to

Drive 	enable the swap floppy drive or not.

The choice : Enabled/ Disabled

Boot Up Floppy 	During POST, the BIOS will determine if the floppy

Seek	disk drive installed is 40 tracks (360K) or 80

tracks (720K, 1.2M, 1.44M)

| Enabled | BIOS searches for floppy disk drive to determine if it is 40 or 80 tracks |
| --- | --- |
| Disabled | BIOS will not search for the type of floppy disk drive by track number |

Boot Up 	This allows you to determine the default state of

NumLock Status	the numeric keypad. By default, the system

boots up with NumLock on.

| On | Keypad is numeric keys |
| --- | --- |
| Off | Keypad is arrow keys |

Boot Up System 	Selects the default system speed - the normal

Speed	operating speed at power up.

| High | Set the speed to high |
| --- | --- |
| Low | Set the speed to low |

Gate A20	This entry allows you to select how the gate A20 is handled.

Option	The gate A20 is a device used to address memory above

1 MB. Initially, the gate A20 was handled via a pin on the

keyboard. Today, while keyboards still provide this support, it

is more common, and much faster, for the system chipset to

provide support for gate A20.

Normal is keyboard; Fast is chipset.

Typematic Rate	This determines if the typematic rate is to be used. When disabled,

Setting	continually holding down a key on your keyboard will generate

only one key instance. In other words, the BIOS will only report

that the key is down. When the typematic rate is enabled, the

BIOS will report as before, but it will then wait a moment, and, if

the key is still down, it will begin the report that the key has been

depressed repeatedly. For example, you would use such a feature

to accelerate cursor movements with the arrow keys.

The choice : Enabled/Disabled

Typematic Rate 	When the typematic rate is enabled, this section

(Chars/Sec)	allows you select the rate at which the keys are repeated.

| 6 | 6 characters per second |
| --- | --- |
| 8 | 8 characters per second |
| 10 | 10 characters per second |
| 12 | 12 characters per second |
| 15 | 15 characters per second |
| 20 | 20 characters per second |
| 24 | 24 characters per second |
| 30 | 30 characters per second |

Typematic Delay 	When the typematic rate is enabled, this section

(Msec)	allows you select the delay between when the

key was first depressed and when the

acceleration begins.

| 250 | 250 msec |
| --- | --- |
| 500 | 500 msec |
| 750 | 750 msec |
| 1000 | 1000 msec |

Security Option 	This category allows you to limit access to the

system and Setup, or just to Setup.

| System | The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt |
| --- | --- |
| Setup | The system will boot, but access to Setup will be denied if the correct password is not entered at the prompt |

To disable security, select PASSWORD SETTING

at Main Menu and then you will be asked to enter

password. Do not type anything and just press

<Enter>, it will disable security. Once the security

is disabled, the system will boot and you can enter 						Setup freely.

PCI/VGA 	It determines whether the MPEG ISA/VESA

Palette Snoop	VGA cards can work with PCI/VGA or not.

| Enabled | When PCI/VGA working with MPEG ISA/VESA VGA Card |
| --- | --- |
| Disabled | When PCI/VGA not working with MPEG ISA/VESA VGA Card |

Assign IRQ for     When this items is enabled, the system will assign an IRQ for

VGA              VGA. If this item is disabled, the VGA will not occupy an

IRQ; therefore the IRQ of VGA will be released for other

usage.

OS Select for 	This item allows you to access the memory that is over

DRAM > 64MB	64MB in OS/2.

The choice : Non-OS2 or OS2

Report No FDD 	Set this item to Yes BIOS will report FDD to Win95.

For WIN 95	If in standard CMOS setup, set Drive A to none, and set

this item to yes. Inside Win95, My Computer and File

manager Disk(A:) will show Removable Disk (A:).

Video BIOS	Determines whether video BIOS will be copied to RAM.

Shadow	However it is optional depending on chipset design.

Video Shadow will increase the video speed.

The choice : Enabled/Disabled

C8000 - CBFFF 	These categories determine whether option ROMs will

Shadow	be copied to RAM. An example of such option ROM

DC000 - DFFFF	would be the support of onboard SCSI.

Shadow 	The choice : Enabled/Disabled

This screen controls the setting for the chipset on the mainboard.

ROM PCI/ISA BIOS (ATC-6510)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| AT Clock, DRAM Timings, ....... |  |

ROM PCI/ISA BIOS (ATC-6510)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Auto Configuration | : Enabled | SDRAM CAS latency Time |  | : 3 |
| --- | --- | --- | --- | --- |
| DRAM Speed Selection | : 60 ns | CPU Clock Ratio |  | : 3.0x |
| MA Wait State | : Slow | Auto Detect DIMM/PCI ClK |  | : Enabled |
| EDO RAS# To CAS# Delay | : 3 | CPU Clock Frequency |  | : 66MHz |
| EDO RAS# Precharge Time | : 3 | Spread Spectrum Modulated |  | : Disabled |
| EDO DRAM Read Burst | : x333 | *CPU Warning Temperature |  | : Disabled |
| EDO DRAM Write Burst | : x222 | *Current System Temperature |  | : 20 o C/ 68 o F |
| CPU-to-PCI IDE Posting | : Disabled | *Current CPU Temperature |  | : 20 o C/ 68 o F |
| System BIOS Cacheable | : Disabled | *Current CPUFAN Speed |  | : 0 PRM |
| Video BIOS Cacheable | : Disabled | *Current ChassisFAN Speed |  | : 0 PRM |
| Video RAM Cacheable | : Disabled | *Current PWRFAN Speed |  | : 0 PRM |
| 8-bit I/O Recovery Time | : 3 | *Vcore : 2.80V VTT(V) |  | : 1.47V |
| 16-bit I/O Recovery Time | : 2 | *+3.3V : 3.45V +5V |  | : 4.91V |
| Memory Hole At 15M-16M | : Disabled | *+12V : 12.22V –12V |  | : -11.76 |
| Passive Release | : Enabled | *-5V : -5.16V |  |  |
| Delayed Transaction | : Disabled |  |  |  |
| AGP Aperture Size (MB) | : 128 | Esc: Quit | :Select Item |  |
| SDRAM RAS-to-CAS Delay | : Slow | F1 : Help | PU/PD/+/-:Modify |  |
| SDRAM RAS Precharge Time | : Slow | F5 : Old Values | (Shift)F2 :Color |  |
|  |  | F6 :Load BIOS Defaults |  |  |
|  |  | F7 : Load Setup Defaults |  |  |

Auto	The first chipset settings deal with CPU access to dynamic

Configuration		random access memory (DRAM). The default timings have

been carefully chosen and should only be altered if data is

being lost. Such a scenario might well occur if your system had

mixed speed DRAM chips installed so that greater delays may

be required to preserve the integrity of the data held in the

slower memory chips.

DRAM Speed	The DRAM speed is controlled by the DRAM timing Registers.

Selection	The timings programmed into this register are dependent on

the system design. Slower rates may be required in certain system

designs to support loose layouts or slower memory.

i.e. 60ns; 70ns

MA Wait State	This item allows you to select MA Wait State.

The choice : Fast, Slow

EDO RAS# to	This sets the relative delay between the row and column address

CAS# Delay	strobes from DRAM (EDO). The choice : 2, 3

EDO RAS#	Defines the length of time for Row Address Strobe from DRAM

Precharge Time		(EDO) is allowed to precharge.

The choice : 3,4

EDO DRAM	This sets the timing for burst mode read (or writes)from DRAM

Read Burst 	(EDO). Burst read and write requests are generated by the CPU

EDO DRAM		in four separate parts. The lower the timing numbers, the faster

Write Burst 		the system will address memory.

i.e. x222 is read/write DRAM timings are 2-2-2-2

DRAM Data	Select parity, ECC, or Disabled, depending on the type of

Integrity Mode	DRAM installed in your system.

The choice : ECC, Parity, Disabled

CPU-To-PCI 	Select Enabled to post write cycles from the CPU to the PCI

# IDE posting	IDE interface. IDE access are posted in the CPU to PCI buffers,

# for cycle optimization.

# The choice : Enabled, disabled

System BIOS	Select Enabled allows caching of the system BIOS ROM at

Cacheable 	F0000h-FFFFFh, resulting in better system performance.

However, if any program writes to this memory area, a system

error may result.

Video BIOS 	Select Enabled allows caching of the video BIOS ROM at

Cacheable 	F0000h-FFFFFh, resulting in better system performance.

However, if any program writes to this memory area, a system

error may result.

Video RAM 	Select Enabled allows caching of the video RAM, resulting in

Cacheable 	better system performance. However, if any program writes to

this memory area, a system error may result.

8 Bit I/O 	The recovery time is the length of time, measured in CPU

Recovery Time 	clocks, which the system will be delay after the completion of

an I/O request. This delay takes place because the CPU is

operating so much faster than the input/output bus that the CPU

must be delayed to allow for the completion of the I/O.This item

allows you to determine the recovery time allowed for 8- bit

I/O. Choices are from NA, 1 to 8 CPU clocks.

16 Bit I/O	This item allows you to determine the recovery

Recovery Time 	time allowed for 16-bit I/O. Choices are from NA,

1 to 4 CPU clocks.

Memory Hole At	In order to improve performance, certain space

15M-16M	in memory can be reserved for ISA cards. This memory

must be mapped into the memory below 16MB.

Passive Release	When Enabled, CPU to PCI bus accesses are allowed during

passive release. Otherwise, the arbiter only accepts another PCI

master access to local DRAM.

The choice : Enabled, disabled

Delayed	This chipset has an embedded 32-bit posted write buffer to

Transaction	support delay transactions cycles. Select Enabled to support

compliance with PCI specification version 2.1.

The choice : Enabled, disabled space

AGP Aperture	Select the size of the AGP aperture. The aperture is a portion of

Size (MB) 	the PCI memory address range dedicated for graphics memory

address space. Host cycle that hit the aperture range are

forwarded to the AGP without any translation. See

www.agpforum.org for AGP information.

The choice 4, 8, 16, 32, 64, 128, 256

SDRAM RAS-to-	You can select RAS to CAS Delay time (CAS latency time) in

CAS Delay 	HCLKs of 2/2 or 3/3. The system board designer should set the

SDRAM CAS	values in this field, depending on the DRAM installed. Do not

latency Time	change the values in this field unless you change specifications

of the installed DRAM or the installed CPU. The choice: 2,3

SDRAM RAS	Defines the length of time for Row Address Strobe is allowed to

Precharge Time 	precharge.

CPU Clock Ratio  	Use this option to set CPU clock ratio. After change the CPU

Clock Ratio, if the system can not start , please see “ CPU Clock

Frequency “ item to solve the problem.

Auto Detect        If this item is enabled, the unused DIMM and PCI slot clock

DIMM/PCI ClK     will be disabled. If this item is disabled the unused DIMM

and PCI slot will still get the active clock signal.

Spread Spectrum   Enable / Disable this item the BIOS will Enable / Disable

Modulated        the clock generator spread spectrum .

CPU Clock         Use this item to set CPU base clock frequency.

Frequency        	After change the CPU Clock Frequency, if the system can

not start, then turn power off, press and hold “ Insert ” key,

power on until screen show up.

※ ※ NOTE  Press “ Del ” to enter BIOS setup and set the CPU clock

frequency / CPU clock ratio to correct value.

The following functions are optional, and they show only when the monitoring IC exists on the mainboard. Please confirm this with your supplier.

CPU Warning     When this item is enabled, we can set the CPU warning

Temperature      temperature . If the CPU temperature is higher than the

setting temperature, the system will beep.

Current System 	This field displays the current system temperature, if your

Temperature   	computer contains a monitoring system.

Current CPU 	It shows the current system temperature.

Temperature

# Current CPUFAN	 It shows the running speed of the system fan, Chassis fan and

/ChassisFAN/ 	power fan. The value will be changing when the system is

PWRFAN Speed 	running. If you do not install the fan, the value will show 0.

This screen controls the 'green' features of this mainboard.

ROM PCI/ISA BIOS (ATC-6510)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Sleep Timer, Suspend Timer, ..... |  |

ROM PCI/ISA BIOS (ATC-6510)

POWER MANAGEMENT SETUP

AWARD SOFTWARE, INC.

| Power Management | : User Defined |  |  | *Reload Global Timer Events* |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PM Control by APM | : Yes |  |  | IRQ [3-7, 9-15], NMI |  |  |  | :Disabled |  |
| Video Off Method | : DPMS |  |  | Primary IDE 0 |  |  |  | :Disabled |  |
| Video Off After | : Standby |  |  | Primary IDE 1 |  |  |  | :Disabled |  |
| Modem Use IRQ | : 3 |  |  | Secondary IDE 0 |  |  |  | :Disabled |  |
| Doze Mode | : Disabled |  |  | Secondary IDE 1 |  |  |  | :Disabled |  |
| Standby Mode | : Disabled |  |  | Floppy Disk |  |  |  | :Disabled |  |
| Suspend Mode | : Disabled |  |  | Serial Port |  |  |  | :Enabled |  |
| HDD Power Down | : Disabled |  |  | Parallel Port |  |  |  | :Disabled |  |
| Throttle Duty Cycle | : 62.5% |  |  |  |  |  |  |  |  |
| ZZ Active in Suspend | : Disabled |  |  |  |  |  |  |  |  |
| VGA Active Monitor | : Enabled |  |  |  |  |  |  |  |  |
| Soft-Off by PWR-BTTN : Instant-Off |  |  |  |  |  |  |  |  |  |
| CPUFAN Off In Suspend |  | : Enabled |  |  |  |  |  |  |  |
| Resume by Ring |  | : Disabled |  | Esc: Quit áâàß |  |  | :Select Item |  |  |
| Resume by Alarm |  | : Disabled |  | F1 : Help | PU/PD/+/- : Modify |  |  |  |  |
| Wake Up On LAN |  | : Enabled |  | F5 : Old Values |  | (Shift) F2: Color |  |  |  |
| IRQ 8 Break Suspend |  | : Disabled |  | F6 : Load BIOS Defaults |  |  |  |  |  |
|  |  |  |  | F7 : Load Setup Defaults |  |  |  |  |  |

Power 	This category allows you to select the type (or

Management	degree) of power saving and is directly related to

the following modes : Doze; Standby; Suspend;

HDD Power Down.

| Min. Power Saving | Minimum power management. Doze =1hr.; Standby=1hr.; Suspend=1hr.; HDD Power Down=15min |
| --- | --- |
| Max. Power Saving | Maximum power management only available for SL CPU.Doze=1min.; Standby=1min.;Suspend=1min.;HDD Power Down=1min |
| User Defined | Allows you to set each mode individually. When not disabled, each of the ranges are from 1min. to 1hr. except for HDD Power Down which ranges from 1 to 15min. and disable |

If you would like to use Software Power-off Control

function, you cannot choose“ Disabled ”here, and

should select “Yes” in PM Control by APM.

PM Control 	When enabled, an Advanced Power Management 	device

by APM	will be activated to enhance the Max. Power Saving

Mode and stop the CPU internal clock. If the Max.

Power Saving is not enabled, this will be shown as NO.

Video Off	This determines the manner in which the monitor is blanked.

Method

| V/H SYNC + Blank | This selection will cause the system to turn off the vertical and horizontal sync. ports and write blanks to the video buffer |
| --- | --- |
| Blank Screen | This option only writes blanks to the video buffer |
| DPMS | Initial display power management signaling |

Video off After      When enabled, this feature allows the VGA adapter to

operate in a power saving mode.

| N/A | Monitor will remain on during power saving modes. |
| --- | --- |
| Suspend | Monitor blanked when the systems enters the Suspend mode. |
| Standby | Monitor blanked when the system enters Standby mode. |
| Doze | Monitor blanked when the system enters any power saving mode. |

MODEM Use IRQ 		This item determines the IRQ in which the MODEM can be used.

The choice : 3,4,5,7,9,10,11,NA.

The Following 4 modes are Green PC power saving functions which are only user configurable when ‘User Defined’ power management has been selected.

Doze Mode	When enabled and after the set time of system inactivity, the

CPU clock will run at slower speed while all other devices

still operate at full speed.

Standby Mode	When enabled and after the set time of system inactivity, the

fixed disk drive and the video would be shut off while all

other devices still operate at full speed.

Suspend Mode	When enabled and after the set time of system inactivity, all

devices except the CPU will be shut off.

HDD Power Down	When enabled and after the set time of system inactivity, the

hard disk drive will be powered down while all other devices

remain active.

Throttle Duty Cycle When the system enters Doze mode, the CPU clock runs only

part of time. You may select the percent of time that the

clock runs.

ZZ Active in       When enabled, the ZZ signal is active during Suspend mode.

Suspend

VGA Active        When Enabled, any video active restarts the global timer for

Monitor           standby mode.

Soft-Off by        Instant-off : When push the power button, the system power will

PWR-BTTN	be off immediately. Delay 4 sec : when push the power button, it

will enter suspend mode. We need to push the power button and

hold for 4 seconds to turn off the power.

CPUFAN Off 	Enabled : under suspend mode, the CPU FAN will be turn off.

In Suspend	 Disabled : suspend mode will not turn off CPU FAN.

Resume by Ring    Enabled : when system in suspend mode, it can be wake up by

modem. Disabled : it cannot be wake up by modem.

Resume by Alarm	 When Enabled, two additional lines will be added to the screen :

Date (of Month) Alarm; Time (hh:mm:ss) Alarm to let user

set the desired date and time. After power off, the system will

automatic power on at the specified date and time.

Wake Up On LAN   Enabled : If you have installed LDCM administrator software, and

any client side is powered off, you can wake up by LAN through

the LDCM mechanism.

Disabled : You cannot wake up by LAN.

IRQ 8 Break       When enabled, the device which occupies the IRQ8 can wake up

Suspend	 the system.

This screen configures the PCI Bus slots.

ROM PCI/ISA BIOS (ATC-6510)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| IRQ Settings, Latency Timers, ..... |  |

ROM PCI/ISA BIOS (ATC-6510)

PNP/PCI CONFIGURATION

AWARD SOFTWARE, INC.

| PNP OS Installed | : No | PCI IDE IRQ Map to |  |  |  | : PCI-AUTO |
| --- | --- | --- | --- | --- | --- | --- |
| Resources Controlled by | : Auto | Primary IDE INT# |  |  |  | : A |
| Reset Configuration Data | : Disabled | Secondary IDE INT# |  |  |  | : B |
|  |  | Esc: Quit áâàß |  |  | :Select Item |  |
|  |  | F1 : Help | PU/PD/+/- : Modify |  |  |  |
|  |  | F5 : Old Values |  | (Shift) F2: Color |  |  |
|  |  | F6 : Load BIOS Defaults |  |  |  |  |
|  |  | F7 : Load Setup Defaults |  |  |  |  |

PNP OS Installed	This item allows you to determine PnP OS or not.

Choices are Yes or No.

Resource	The Award Plug and Play BIOS has the capability to

Controlled by	automatically configure all of the boot and Plug and Play

compatible devices. However, this capability means absolutely

nothing unless you are using a Plug and Play OS such as

Windows 95. Choices are Auto and Manual.

Reset Config-	This item allows you to determine whether to reset the

uration Data	configuration data or not.

IRQ3/4/5/7/9/10	This item allows you to determine the IRQ/DMA assigned to

11/12/14/15,	the ISA bus and is not available to any PCI slot.

DMA0/1/3/5/6/7	Choices are Legacy ISA and PCI/ISA PnP.

Assign to

PCI IDE IRQ	This allows you to configure your system to the type of IDE disk

Map To	controller in use. By default, Setup assumes that your controller is

Primary IDE INT#		an ISA device rather than a PCI controller. The most apparent

Secondary IDE		difference is the type of slot being used. If you have equipped your

INT#		system with a PCI controller, changing this allows you to specify

which slot holds the controller and which PCI interrupt (A,B,C,D)

is associated with the connected hard disk. This setting refers to the

hard disk drive itself, rather than individual partitions. Since each

IDE controller supports two separate hard drives, you can select the

INT# for each. Again, you will note that the primary has a lower

interrupt than the secondary as described in “ lot x Using INT#”

above. Select ‘PCI Auto’ allows the system to automatically

determine how your IDE disk system is configured.

Used MEM base	This item allows you to determine which basic address will

addr	not be occupied by PCI card and leave these address for

some special ISA card used only.

Choices are C800, CC00, D000, D400, D800, DC00.

Used MEM Length	This item determines the memory length of address which is for

some special ISA Card used. Choices are 8K, 64K, 32K, 16K.

This section page includes all the items of IDE hard drive and Programmed

Input/Output features. See also Section “Chipset Features Setup”.

ROM PCI/ISA BIOS (ATC-6510)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (ATC-6510)

INTEGRATED PERIPHERALS

AWARD SOFTWARE, INC.

| IDE HDD Block Mode | : Enabled | Onboard Parallel Port |  |  | : 378/IRQ7 |
| --- | --- | --- | --- | --- | --- |
| IDE Primary Master PIO | : Auto | Onboard Parallel Mode |  |  | : ECP |
| IDE Primary Slave PIO | : Auto | ECP Mode Use DMA |  |  | : 3 |
| IDE Secondary Master PIO | : Auto |  |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |  |
| IDE Primary Master UDMA | : Auto |  |  |  |  |
| IDE Primary Slave UDMA | : Auto |  |  |  |  |
| IDE Secondary Master UDMA | : Auto |  |  |  |  |
| IDE Secondary Slave UDMA | : Auto |  |  |  |  |
| On-Chip Primary PCI IDE | : Enabled |  |  |  |  |
| On-Chip Secondary PCI IDE | : Enabled |  |  |  |  |
| Onboard Audio Chip | : Enabled |  |  |  |  |
| USB Keyboard Support | : Disabled |  |  |  |  |
| Init AGP Display First | : Disabled | Esc: Quit áâàß |  |  | :Select Item |
| Power On Function | : Button Only | F1 : Help | PU/PD/+/- : Modify |  |  |
| KBC input clock | : 8 MHz | F5 : Old Values |  | (Shift) F2: Color |  |
| Onboard FDC Controller | : Enabled | F6 : Load BIOS Defaults |  |  |  |
| Onboard Serial Port 1 | : 3F8/IRQ4 | F7 : Load Setup Defaults |  |  |  |
| Onboard Serial Port 2 | : 3F8/IRQ3 |  |  |  |  |
| UART Mode Select | : Normal |  |  |  |  |

IDE HDD Block	This allows your HD controller to use the fast block

Mode	mode to transfer data to and from your HD drive

| Enabled | IDE controller uses block mode |
| --- | --- |
| Disabled | IDE controller uses standard mode |

IDE Primary	PIO - Programmed Input/Output, it allows the BIOS to

Master/Slave PIO	tell the controller what it wants and then let the controller

IDE Secondary	and the CPU to complete the task by themselves.

Master/Slave PIO	This is simpler and more faster. Your system supports

five modes, 0 - 4, which primarily differ in timing. When

Auto is selected, the BIOS will select the best available mode.

IDE Primary	Auto, will support the Ultra DMA function.

Master/Slave UDMA	Disabled, will not support the Ultra DMA function.

IDE Secondary

Master/Slave UDMA

On-Chip Primary	This setup item allows you to either enable or disable the

PCI IDE 	primary/secondary controller. You might choose to disable

On-Chip Second- 	the controller if you were to add higher performance

ary PCI IDE 	or specialized controller.

Onboard Audio     When enabled, the system will automatically detect sound card,

Chip              if your system has a sound card on the mainboard.

When disabled, the system will not detect the sound card.

USB Keyboard 	Enabled will support USB keyboard in Win95 2.1 and NT 5.0 or

Support	 above operating system.

Init AGP          This item will activate the AGP in the multi-display environment

Display First       , it displayed, if disabled, and the system has both AGP and PCI

VGA card, the AGP monitor will not display.

Power On 	KB Power On Password: this function is available for ATX power

Function	only. User can select the power on method to power on the system.

The options are the following :Password : when user select this

option, it will show “Enter Password:”, after user type password, it

will show “ Confirm Password:”, user should type the same

password to confirm it. After power off, if user types the correct

password, the system will power on.

Hot Key : when user select this option, it will show another line

lines as Hot Key Power ON: Ctrl-F1”’ select any one you like.

After power off, if user key in the control-F, it will power up the

system.

Mouse Left, Mouse Right : it will power up the system by double

click the left or right mouse. Note do not move your mouse between

the first click and the second click.

BUTTON ONLY : only the power button can power the system.

KBC input clock	Let user change the keyboard working clock.

On Board 	This item will enable or disable the floppy disk controller.

FDC Controller

On Board 	User can select serial port IRQ. If set to Auto, system will assign

Serial Port 1	an IRQ for it. Note : set to Auto is not recommended.

On Board 	User can select serial port IRQ. If set to Auto, system will

Serial Port 2	assign an IRQ for it. Note : set to Auto is not recommended.

UART Mode	This lets you select the Infrared mode. Choices are Standard,

Select	HPIR, and ASKIR. If you choose HPIR or ASKIR mode,

the screen will show another two lines to let you choose

‘IR Function Duplex’ (Full or Half) and‘RxD TxD Active’

(Hi Lo; Lo Hi; Hi Hi;Lo Lo).

On Board 	Let user select IRQ for parallel port, when Disabled, the parallel

Parallel Port	port will be disabled

On Board 	Let user select error check mode. This item is not recommended to

Parallel Mode	change except user has special request.

ECP Mode	 Select a DMA channel for the port.

Use DMA	 Choices are 3, 1.

ROM PCI/ISA BIOS (ATC-6510)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (ATC-6510)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| Hard Disks Type Size CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Primary Master : |  |  |  |  |
|  | Select Primary Master Option (N=Skip) : N |  |  |  |
|  |  | Options Size CYLS Head PRECOMP LANDZ Sector Mode |  |  |
|  |  | 2(Y) 1337 648 64 0 2594 63 LBA |  |  |
|  |  | 1 1339 2595 16 65535 2594 63 NORMAL |  |  |
|  |  | 3 1338 1297 32 65535 2594 63 LARGE |  |  |
| Note : Some OSes (like SCO-UNIX) must use “Normal” for installation |  |  |  |  |
| ESC : Skip |  |  |  |  |

The last step is 'save and exit'. If you select this item and press 'Y', then these records will be saved in the CMOS memory on the mainboard. It will be checked every time you turn your computer on.

ROM PCI/ISA BIOS (ATC-6510)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | SAVE & EXIT SETUP |
| LOAD BIOS DEFAULTS | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |
| ESC: Quit | áâàß:Select Item |
| F10: Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type ..... |  |

ROM PCI/ISA BIOS (ATC-6510)

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP |  | INTEGRATED PERIPHERALS |  |
| --- | --- | --- | --- |
| BIOS FEATURES SETUP |  | PASSWORD SETTING |  |
| CHIPSET FEATURES SETUP |  | IDE HDD AUTO DETECTION |  |
| POWER MANAGEMENT SETUP |  | SAVE & EXIT SETUP |  |
| PNP/PCI CONFIGURATION |  | EXIT WITHOUT SAVING |  |
| LOAD BIOS DEFAULTS | SAVE to CMOS and EXIT (Y/N):Y |  |  |
| ESC: Quit |  | áâàß:Select Item |  |
| F10: Save & Exit Setup |  | (Shift) F2 : Change Color |  |
| Save Data to CMOS & Exit SETUP |  |  |  |

LOAD BIOS DEFAULTS

When your mainboard has problems and needs to

trouble shoot the system, you can use this function.

The default values loaded only affect the BIOS Features

Setup, Chipset Features Setup, Power Management

Setup and PNP/PCI Configuration Setup. There is no

effect on the Standard CMOS Setup. To use this function,

select it from main menu and press <Enter>. A line will

appear on the screen asking if you want to load the BIOS

default values. Press <Yes> and <Enter> then the BIOS

default values will be loaded.

LOAD SETUP DEFAULTS

This allows you to load optimal settings which are stored in

the BIOS ROM. The default values loaded only affect the

BIOS Features Setup, Chipset Features Setup, Power

Management Setup and PNP/PCI Configuration Setup.

There is no effect on the Standard CMOS Setup. To use

this function, select it from main menu and press <Enter>.

A line will appear on the screen asking if you want to load

the Setup default values. Press <Yes> and <Enter> then

the Setup default values will be loaded.

SUPERVISOR PASSWORD / USER PASSWORD

This allows you to set the password. The mainboard defaults with

password disabled.

Enter/Change password : Enter the current password, at

the prompt, key-in your new password (up to eight

alphanumeric characters), press <Enter>. At the next

prompt, confirm the new password by typing it again and press

<Enter>.

Disable password : Press the <Enter> key instead of

entering a new password when the ‘Enter Password’

dialog box appears. A message will appear confirming that

the password is disabled.

If you set both supervisor and user passwords, only the

supervisor password allows you to enter the BIOS SETUP

program.

CAUTION :If you forgot your password, you must disable

the CMOS by turning power off and set JP 8 to ‘close‘. And then

open reload the system.

IDE HDD AUTO DETECTION

This allows you to detect the IDE hard disk driver’s parameters

and enter them into ‘Standard CMOS Setup’ automatically.

If the auto-detected parameters displayed do not

match the ones that should be used for your hard drive,

do not accept them. Press <N> to reject the values

and enter the correct ones manually on the Standard

CMOS Setup screen.

SAVE & EXIT SETUP

This allows you to save the new setting values in the

CMOS memory and continue with the booting process.

Select what you want to do, press <Enter>.

EXIT WITHOUT SAVING

This allows you to exit the BIOS setup utility without

recording any new values or changing old ones.

※ Control Key Description ※

| UP ARROW | ↑ | Move to previous item |
| --- | --- | --- |
| DOWN ARROW | ↓ | Move to next item |
| LEFT ARROW | ← | Move to the item in the left hand |
| RIGHT ARROW | → | Move to the item in the right hand |
| Esc KEY | Esc | Main Menu : Quit and not save changes Setup menu : Exit current page and return to main menu |
| PgUp KEY |  | Increase the numeric value or make changes |
| PgDn KEY |  | Decrease the numeric value or make changes |
| F1 KEY | Help | General help |
| F2 KEY | ＜Shift＞+F2 | Change color from total 16 colors |
| F5 KEY | Old Value | Restore the pervious CMOS value from CMOS |
| F6 KEY | Load BIOS default | Load the default CMOS value from BIOS default table |
| F7 KEY | Load setup default | Load Setup default |
| F10 KEY | Save & Exit Setup | Save all the CMOS changes and Exit setup, only for Main Menu |

APPENDIX C

※※TECHNICAL SUPPORT REQUEST FORM※※

If the mainboard doesn't function properly, please complete the following information and return it to your system dealer. If the further information is needed, please attach it.

Model No :  ATC-6510      Date of Purchase :  ______________

Serial No : ___________________

HARDWARE :

|  | BRAND | MODEL | SPEED | Q'TY |
| --- | --- | --- | --- | --- |
| DIM Module |  |  |  |  |

CPU SPEED : ________ MHz

DRAM :______MB (        SDRAM/EDO DRAM)

Hard Disk Interface Controller : _____ IDE, ____ SCSI

Hard Disk Brand :___________, Model :_________, Capacity :________

Display Controller Brand :_____________,Model :__________

Controller Chip Brand :_____________,Model :__________

SOFTWARE:

Award SYSTEM BIOS: Version _________  Date Code _____________

Other Add-on Cards Information:

| Add-on Card | Bus Interface | Model | Remark |
| --- | --- | --- | --- |

Error Description

160-6510-010000-80520

CHECK LIST OF THE PACKAGING

The mainboard comes securely packed in a durable box or shipping carton. If any of the following items are missing or damaged, please contact your supplier.

Each mainboard contains:

Q'TY     	Description

1             Mainboard        : ATC-6510.

1             CD     	      : LDCM, Enhanced IDE driver

Award system BIOS Update Utility

1             Retention mechanism (for PentiumII installation)

1             Retention mechanism (for CeleronTM installation)
      2             Key (for PentiumII & CeleronTM installation)

1             Cable 	         : Enhanced IDE cable.

1             Cable 	         : F.D.D. cable.

1             Cable 	         : Thermister cable (option).

1             Manual 	         : User manual. (English version)

1                               English LDCM user manual (option)