Table of Contents

JUMPER COMPONENT LOCATION DIAGRAM	A

µCHAPTER 1   INTRODUCTION	2

1-1 SYSTEM FEATURES	3

1-2 CHECK LIST OF THE PACKING	4

CHAPTER 2   INSTALLATION	5

2-1 INSTALLATION PROCEDURES 	5

2-2 CPU INSTALLATION	6

2-3 SYSTEM MEMORY INSTALLATION	12

2-4 SRAM INSTALLATION	14

2-5 OTHER JUMPERS AND CONNECTORS 
       DESCRIPTION 	15

2-6 IDE DRIVER INSTALLATION	20

CHAPTER 3   AWARD BIOS SETUP	21

3-1 UPDATE BIOS PROCEDURES	22

3-2 AWARD SYSTEM BIOS CONFIGURATION SETUP	23

APPENDIX A 	47

CHAPTER 1 INTRODUCTION

This manual introduces how to configure the ATC-1020 mainboard for different environment. It's an overview of the layout and features of the mainboard, and also provides information for

you to change the configuration or system environment.

This manual is divided into two sections :

PART ONE includes page A and two chapters as following:

Page A contains layout diagram of the mainboard. Please refer it when you configure the system.

Chapter 1 is an overview of the mainboard features and packing

contents.

Chapter 2 describes how to upgrade and to change hardware

configurations such as memory size, CPU type, and lists of

jumper settings and connectors.

PART TWO includes chapter 3 which contains Award BIOS description.

Chapter 3 is the user's guide of AWARD BIOS setup utility, Flash ROM BIOS update procedure. The menu showed in this chapter are default settings.

Your system dealer will set up the mainboard according to your demand of computer. It means that the current settings of your mainboard may not be the same as the defaults shown in this user's manual.  If you need to change your configuration, please ask your dealer firstly, be sure this will not against your system warranty. Or ask for your dealer to do it for you.

REMARK

Intel®  is a registered trademark of Intel Corporation.

MS-DOS® is a registered trademark of Microsoft Corporation.

All other brands and product names are trademarks registered trademarks of their respective companies.

1-1 SYSTEM FEATURES

Pentium level CPU operating at 75 MHz to 200MHz with 321-pin ZIF socket 7 and scalability to accept processor in the future.

INTEL 82430VX PCIset.

Using four 72-pin SIM sockets, provides two banks of 64-bit wide path up to 128MB addressing page mode DRAMs.

Supporting two types of DRAM including EDO (Extended Data Out), or FPM (Fast Page Mode).

Supporting four PCI bus master revision 2.1, 5V interface compliant and three 16-bit ISA slots.

Dual Master IDE connectors support up to four devices in two channels for connection of high capacity hard disk drive, CD-ROM, tape backup etc..

AT style keyboard connector and PS/2 mouse connector.

Winbond 83877 high-speed Multi-I/O chipset:

Supporting Infrared transfer (IrDA TX/RX) connection.

One FDC port supports two devices up to 2.88MB

Two 16550A fast UARTs compatiable serial ports

One EPP/ECP mode parallel port

Hardware Dimension is 220mm x 280mm (8.66" x 11.02") with four layers designed.

1-2 CHECK LIST OF THE PACKING

The mainboard comes securely packed in a durable box and shipping carton. If any of the above items are missing or damaged, please contact your supplier.

Each mainboard containing:

Q'TY     	Description

1         	Mainboard 	: ATC-1020.

1	Diskette	: Enhanced IDE driver (3.5").

1         	Cable 	: Enhanced IDE connector.

1         	Cable 	: F.D.D. connector.

1	Cable 	: Serial port.

1         	Cable 	: Serial/Parallel.

1         	Manual 	: User`s manual.

NOTE : Leave the mainboard in its original packing until you are ready to install it.

CHAPTER 2  INSTALLATION

2-1 INSTALLATION PROCEDURE

Before installing the computer, please prepare all components such as CPU, DRAM; peripherals such as hard disk devices, keyboard, CD-ROM drive; and accessories such as cables. Then, install the system as following :

1. Plug CPU, heat sink, cooling fan, and DRAM modules in the

mainboard.

2. Set jumpers based on your configuration.

3. Plug add-on cards in PCI/ISA slots.

4. Connect cables to peripherals, power supply..

5. Make sure all components and devices are well connected,

turn on the power and setup System BIOS based on your

configuration.

6. Install peripheral, add-on card drivers and test them.

7. If all of above procedures are success, turn-off the power then

plug all of them into your computer case.

2-2 CPU  INSTALLATION

ATC-1020 supports Pentium level CPU up to 200MHz. For installation, please notice CPU pin 1 must align with the ZIF socket 7 Pin 1 location. Before you install or upgrade your CPU, please read CPU guide from CPU manufacturer to make sure the CPU voltage specification. Then choose the right installation in the 2-2-1 based on your CPU type/brand and follow the description to setup jumpers. If your CPU is not in the list of 2-2-1, please refer to 2-2-2 and 2-2-3 for installation.

2-2-1 CPU TYPE SELECTION

A. INTEL PENTIUM CPU (P54C)

JP11

1-2

Intel Pentium CPU,

the first letter after ႑/႒ denotes voltage type.

CPU Core Voltage

VRE

STD

xxxxxxxxx

xxxxxxxxx

xxxxxxxxx

Sxxxx/SMU

JP14

1-2**

3-4*

*  STD : 3.135V ~ 3.600V

** VRE : 3.400V ~ 3.600V

(The fourth line of the mark on the under-side

of the processor contains a code that identifies

the voltage level type. V is VRE, S is standard. )

INTERNAL CPU CLOCK

JP5

JP6

JP9

JP10

Ext.x Frq.

75MHz

open

open

1-2

1-2

50x1.5

90MHz

open

close

1-2

| 1- |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

2

60x1.5

100MHz

close

open

1-2

1-2

66x1.5

120MHz

open

clos

| e |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

1-2

2-3

60x2.0

133MHz

close

open1-2

2-366x2.0

150MHz

open

cl

| os |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

e

2-3

2-3

60x2.5

166MHz

close

open

2-3

2-3

66x2.5

180MHz

open

clos

| e |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

2-3

1-2

60x3.0

200MHz

close

open

2-31-2

66x3.0

B. INTEL PENTIUM

| CP |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

U (P55C)

Besides CPU clock setting, for P55C (MMX) CPU you hav

| e |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

to set JP11,JP14 based on its CPU I/O voltage and core voltage.

| JP |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

11

2-3

Set 2-3 for dual voltage mode.

CPU Core Voltage

3.5V

3.3

| V |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

2.9V

2.8V

2.7V

JP14

1-2

3-4

5-6

7-8

9-10

Currently, Intel P55C i

| s |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

released 2.8V core voltage only.

Please confirm this with your s

| ys |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

tem supplier before you

install P55C CPU.

INTERNAL CPU CLOCK

JP5

JP6

JP9

JP10

Ext.x Frq.

166MHz

close

open

2-3

2-3

66x2.5

180MHz

open

close

2-3

1-2

60x3.0

200MHz

close

open

2-3

1-2

66x3.0

¡The location of JP11 and JP14

JP11

JP14

C. Cyrix 6x86/6x86L CPU

¡Cyrix 6x86 (028) CPU

CPUJP11

JP14

Cyrix 6

| x8 |  |  |
| --- | --- | --- |

6 (028)

1-2

1-2

INTERNAL CPU CLOCK

JP5

JP6

JP9

JP10

Ext.x Frq.

PR120+ @ 100MHz

open

open

1-2

2-350x2.0

PR150+ @ 120MHz

open

close

1-2

2-3

60x2.0

| P |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R166+ @ 133MHz

close

open

1-2

2-3

66x2.0

(Cyrix logo)

6x86-PRxxx+GP

xxxMHz

3.52V(028)

The button line of the mark on the processor contains a code 028 that identifies the voltage level type. If the code is 016 or others, or no marking then please contact your dealer.

¡Cyrix 6x86L CPU (dual voltage)

CPU

JP11

JP14

Cyrix 6x86L

2-3

7-8

INTERNAL CPU CLOCK

JP5

JP6

JP9

JP10

Ext.x Frq.

PR120+ @ 100MHz

open

open

1-2

2-3

50x2.0

PR133+ @ 110MHz

close

close

1-2

2-3

55x2.0

PR150+ @ 120MHz

open

close

1-2

2-3

60x2.0

PR166+ @ 133MHz

close

| open 1-2 2-3 66x2.0 (Cyrix logo) 6x86L-PR150+GP 120MHz 2.8V The mark on the processor contains as the sample in the left. The code-name 6x86L is dual voltage mode processor, you should set JP11 to 2-3. D. AMD-K5 CPU CPU JP11 JP14 |  |  | AMD K5 1-2 1-2 CPU CLOCK RATING JP5 JP6 JP9 | JP10 Ext.x Frq. PR75ABR 75MHz open open 1-2 1-2 50x1.5 PR90ABQ 90MHz open close 1-2 1-2 |  | 60x1.5 |  | PR100ABQ 100MHz close |  | open 1-2 1-2 66x1.5 PR120ABQ 90MHz open close 1-2 1-2 60x1.5 PR133ABQ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

100MHz

close

open

1-2

1-2

66x1.5

PR166ABQ

close

open

2-3

2-3

66x2.5

E. AMD-K6 CPU

AMD K6-PR166/PR200

JP11

JP14

2-3

5-6

INTERNAL CPU CLOCK

JP5

JP6

JP9

JP10

Ext.x Frq.

AMD K6-PR166

close

open

2-32-3

66x2.5

AMD K6-PR200

close

open

2-31-2

66x3.0

AMD-K5(

PR100

AMD-K5-PR100ABQ

100MHz

Product Name

P-rating

Internal CPU Frequency

2-2-2 CPU VOLTAGE SETTING

JP11 is for setting single or dual CPU voltage mode. JP14 is used for various CPU voltage value types, please refer to CPU‘s marking on/under CPU and its user’s manual.

CPU MODE

Single

Dual

JP11

1-2

2-3

P54C VRE

Cyrix 6x86

AMD K5

1

3

5

7

9

CPU CORE VOLTAGE

JP14

P54C STD

INTEL

STD

3-4

AMD K6

(PR166/200)

(P54C)

VRE

1-2

P55C

Cyrix 6x86L

P55C

2.8V

| 7 |  |  |  |  |
| --- | --- | --- | --- | --- |

-8

Cyrix

6x86(028)

1-2

Cyrix

6x86L

7-8

AMD

K5

1-2

AMD

K6(PR166/200)

5-6

JP5

JP6

JP11

JP14

JP9

JP10

2-2-3 CPU CLOCK SETTING

The following setting is for new release CPUs.

EXTERNAL CPU CLOCK

JP5

JP6

50(MHz)

open

open

55(MHz)

close

close

60(MHz)

open

close

66(MHz)

close

open

JP5 JP6 JP5 JP6 JP5 JP6 JP5 JP6

50MHz55Mhz60MHz66MHz

External

CPU Clock :

INTERNAL

| C |  |  |  |  |
| --- | --- | --- | --- | --- |

PU CLOCK

JP9

JP10

INTEL EXTERNAL CLOCK

| X |  |  |  |  |
| --- | --- | --- | --- | --- |

1.5

1-2

1-2

INTEL EXTERNAL CLOCK X 2.

| 0 |  |  |
| --- | --- | --- |

1-2

2-3

INTEL EXTERNAL CLOCK X 2.5

2-

| 3 |  |  |
| --- | --- | --- |

2-

| 3 INTEL EXTERNAL CLOCK X 3.0 2-3 1-2 INTEL EXTERNAL CLOCK X 3.5 1-2 1-2 JP9 JP10JP9 JP10JP9 JP10JP9 JP10 JP9 JP10 x1.5x2.0x2.5x3.0x3.5 Internal CPU Clock : 2-3 SYSTEM MEMORY INSTALLATION ATC-1020 provides four 72-pin SIM sockets for system memory expansion from 4MB to 128MB. These four SIMs are arranged to two banks, Bank0 (SIM 1, 2) and Bank1(SIM 3, 4), please refer to page A. Each bank provides 64-bit wide data path. The mainboard accepts Fast Page Mode DRAM, and EDO Mode (Extended Data Out) DRAM, with a speed at least 70 nanosecond. You should plug DRAM modules into two sockets (same bank) or four sockets at one time. Each pair of modules must be the same size, type, and speed; no matter single-side or double-side module. Please plug in Bank 0 firstly if you only have 2 modules. The mainboard supports mixing of EDO and fast page mode DRAM among different banks, please plug EDO in Bank 0. Also this mainboard provides one optional 168-pin DIMM socket for Synchronous DRAM expansion. It‘s not recommended to install the SDRAM and EDO or Fast Paged mode memory within a system. Changing EDO/FPM DRAM to SDRAM, you don‘t have to adjust jumper setting or BIOS value, nor change SDRAM to EDO/FPM DRAM. (¡ Please make sure the SDRAM plugged-in fully, to prevent contact loss.) ¡ System Memory Combinations Options ¡ BANK0 BANK1 Total Memory SIM 1, 2 SIM 3, 4 SIM 1- 4 2MBx2 - 4MB - 2MBx2 4MB 4MBx2 - 8MB - 4MBx2 8MB 8MBx2 - 16MB - 8MBx2 16MB 4MBx2 4MBx2 16MB - continue - BANK0 BANK1 Total Memory SIM 1, 2 SIM 3, 4 SIM 1- 4 4MBx2 8MBx2 24MB 8MBx2 4MBx2 24MB 16MBx2 - 32MB - 16MBx2 32MB 8MBx2 8MBx2 32MB 4MBx2 16MBx2 40MB 16MBx2 4MBx2 40MB 8MBx2 16MBx2 48MB 16MBx2 8MBx2 48MB 32MBx2 - 64MB - 32MBx2 64MB 16MBx2 16MBx2 64MB 4MBx2 32MBx2 72MB 32MBx2 4MBx2 72MB 8MBx2 32MBx2 80MB 32MBx2 8MBx2 80MB 16MBx2 32MBx2 96MB 32MBx2 16MBx2 96MB 32MBx2 32MBx2 128MB 2-4 SRAM INSTALLATION ATC-1020 is built-in 256KB or 512KB Sync. Pipeline Burst SRAM on board and provides a COAST slot for further expansion. ¡ Cache Memory Combinations Options ¡ SRAM TYPE SRAM SIZE DATA SRAM TAG SRAM Synchronous 256KB 32K32 x 2pcs 16K8 or on board 512KB 64K32 x 2pcs 32K8 x 1pc Synchronous module 256KB 32K32 x 2pcs 16K8 or 32K8 x 1pc ¡ Upgrade Procedure : For upgrading your Sync. SRAM from 256KB to 512KB by using COAST Module, you should plug-in the KIT-256 module, then set JP17 to “open” ( disable the original TAG on board). You must use KIT-2 |  |  |
| --- | --- | --- |

56 for upgrading your system to 512KB Sync. SRAM , ple

| as |  |  |  |  |
| --- | --- | --- | --- | --- |

e contact your system de

| al |  |  |  |  |
| --- | --- | --- | --- | --- |

er about its information

| . |  |  |  |  |
| --- | --- | --- | --- | --- |

It does not

need any BIO

| S |  |  |  |  |
| --- | --- | --- | --- | --- |

value adjustments for up

| gr |  |  |  |  |
| --- | --- | --- | --- | --- |

ading the SRAM size.

| JP17 Enable Disable |  |  |  |  |
| --- | --- | --- | --- | --- |

On Board TAG

JP17

Disa

| bl |  |  |  |  |
| --- | --- | --- | --- | --- |

e

open

Enable

close

(If your m

| od |  |  |  |  |
| --- | --- | --- | --- | --- |

ule is without TAG RAM, please

| s |  |  |  |  |
| --- | --- | --- | --- | --- |

et

J

| P1 |  |  |  |  |
| --- | --- | --- | --- | --- |

7 “clo

| se |  |  |  |  |
| --- | --- | --- | --- | --- |

”.)

2-5 OTHER JUMPERS AND

CONNE

| CT |  |  |  |  |
| --- | --- | --- | --- | --- |

ORS  DESCRIPTION

The locations of following jumper

| s |  |  |  |  |
| --- | --- | --- | --- | --- |

are indicated in page A.

Jumper

F

| un |  |  |  |  |
| --- | --- | --- | --- | --- |

ction

Remark

JP8

CMOS open: Nor

| ma |  |  |  |  |
| --- | --- | --- | --- | --- |

l¡

close: Clear CMOS

JP7

ATBUS CLK

open: PCI CLK/3 for external

CPU cl

| oc |  |  |  |  |
| --- | --- | --- | --- | --- |

k 50MHz

close: PCI CLK/4 for external

CPU clock 60, 66MHz

¡is default s

| et |  |  |  |  |
| --- | --- | --- | --- | --- |

ting     **is factory default sett

| in |  |  |  |  |
| --- | --- | --- | --- | --- |

g (fixed)

The locations of fol

| lo |  |  |  |  |
| --- | --- | --- | --- | --- |

wing connectors are indicated in pag

| e |  |  |  |  |
| --- | --- | --- | --- | --- |

A. When you plug a cable into the fo

| ll |  |  |  |  |
| --- | --- | --- | --- | --- |

owing I/O connectors, you should hav

| e |  |  |  |  |
| --- | --- | --- | --- | --- |

the pin 1 edge of the cable al

| ig |  |  |  |  |
| --- | --- | --- | --- | --- |

ned with the pin 1 end of the

| co |  |  |  |  |
| --- | --- | --- | --- | --- |

nnector.

CONN1 : speaker, keyboard l

| oc |  |  |  |  |
| --- | --- | --- | --- | --- |

k, reset, SMI,

turbo LED

| , |  |  |  |  |
| --- | --- | --- | --- | --- |

and IDE LED connectors.

SPK            KEYLOCK

RST   SMI   TB-LED IDE-LED

+       +

SPK : speaker

Speaker

GND

GND

VCC

KEYLOCK : keyboard lock switch and

power LED connector

Power LED +

N/C

GND (power)

Keylock

GND (keylock)

RST : Reset connector

Reset Signal

GND

SMI : SMI lead

GND

SMI Signal

TB-LED : Turbo LED indicator, LED on

when system runs higher speed

GND

+5V (near to CPU)

IDE-LED : IDE devices indicator LED

connector, LED stays ON indicates

on-board IDE devices in operation.

GND* If plug wire into wrong connector,

+5V color of LED will be lighter and the IDE

(next to CPU)   devices can still function properly.

FAN1 : CPU cooling fan connector

| . |  |  |  |  |
| --- | --- | --- | --- | --- |

Wire with

1 GND

2+12V

3GND

+12V voltage (most l

| ik |  |  |  |  |
| --- | --- | --- | --- | --- |

ely red wire) must be

plugged into p

| in |  |  |  |  |
| --- | --- | --- | --- | --- |

2, and GROUND wires (must

likely black wires) must be plugged into pin1

and/or pin3. Please confirm the wire color re-

presentation with your supplier.

CAUTION: Plug wire into wrong connector will

DAMAGE fan and mainboard.

1 +5V

2N/C

3IRRX

4GND

5IRTX

IR1 : Infrared module connector.

USB1 : USB connector; Universal Serial Bus;this

(red)

VCC GND

D1 -D0 +

D1+D0 -

GNDVCC

(black)

is used to connect USB devices through

an optional dual head cable with a iron plane.

CAUTION: Plug wire into wrong conn

| ec |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

tor

will DAMAGE USB devices and mainboard.

KB1 : 5-pin AT style compatible keyboard

connector.

PS/2 : PS/2 mouse connector, which is used to

connect an optional cable

4 6

2

1

35

.

pin1 : data

pin2 : N/C

pin3 : GND

pin4 : VCC

pin5 : clock

pin6 : N/C

PW1 : +5 Voltage power supply connector.

PG   +12V    GND          +5V

+5V   -12V          -5V

COM1/COM2

COM1/COM2 : these two connectors are used

1 2

34

56

78

910

to connect serial port cables.

pin signal name

1

DCD

2

Serial In

3

Serial Out

4

DTR

5

GND

6

DSR

| 7 RTS 8 | CTS 9 RI 10 N/C FDC : this connector is used to connect floppy 12 3334 disk drive through cable. pin signal pin signal |  |
| --- | --- | --- |
| STEP-2 4 reserved 22 Write Data 6 FDEDIN 24 Write Gate RWC- 20 |  |  |

8

Index-

26

Track 00-

10

Motor EnableA-

28

Write Protect-

12

Drive Sele.B-

30

Read Data-

14

Drive Sele.A-

32

Side 1 Sele.-

16

Motor EnableB-

34

DisketteChange

18

DIR-

All of odd

| pins are ground | LPT LPT : this connector is used to connect parallel 114 215 1326 port cable. pin signal pin signal 1 STROBE- 10 | ACK- 2 Data Bit 0 11 BUSY 3 Data Bit 1 12 PE | 4 |
| --- | --- | --- | --- |
| Data Bit 2 13 SLCT 5 Data Bit 3 14 Auto Feed- |  |  |  |
| 6 Data Bit 4 15 ERROR- 7 Data Bit 5 16 INIT- 8 Data Bit 6 17 SLCT IN- 9 Data Bit 7 26 N/C pin18 -- pin25 are ground IDE1/IDE2 : these two connectors are used to connect IDE devices through IDE cables, up to 4 devices. 12 34 3940 pin signal pin signal 1 Reset IDE 21 DDRQ0(1) 2 GND 22 GND 3 Host Data 7 23 I/O Write- 4 Host Data 8 24 GND 5 Host Data 6 25 I/O R |  |  |  |

ead-

6Host Data 9

26

GND

7

Host Data 5

27

IORDY

8

Host Data 10

28

N/C

9

Host Data 4

29

DDAK0-(1-)

10

Host Data 11

30

GND

11

Host Data 3

31

IRQ14*

12

Host Data 12

32

IOCS16-

13

| Host Data 2 33 Addr 1 * IDE1: pin31 is IRQ14; IDE2: pin31 is IRQ15 or MIRQ0 14 Host Data 13 34 N/C 15 Host Data 1 35 Addr 0 16 Host Data 14 36 Addr 2 17 Host Data 0 37 ChipSele. 1P- | 18 Host Data 15 | 38 ChipSele 3P- 19 GND 39 Activity |  |
| --- | --- | --- | --- |
| 20 | Key 40 GND 2-6 IDE DRIVER INSTALLATION The IDE driver installation procedure is printed on the label of diskette as following : Setup for DOS/Windows : 1. Starting MS-Windows 3.1 (or 3.11) 2. Select Program Manager, File/RUN, then type "A:\setup.exe" 3. Exit MS-Windows, turn power off; then turn power on. Setup for Windows 95 : 1. Starting |  |  |

Windows 95

2. Select “My Computer”; select “Control Panel”; select “System”; then select “Device Manager”, “Hard Disk Controllers”.

3. Double-click to Remove default driver, restart computer.

4. Follow the instruction

| s on your screen to install new IDE driver we offer in the 3.5“ diskette 5. Exit Windows 95, turn power off; then turn power on. ¡For more information, please refer to Windows 95 manual. Make sure your HDD should follow ATA standard, and your CD-ROM should follow ATAPI standard. When you plug-in the IDE devices, please plug your first and second devices into IDE 1 port (Master then Slave), then plug third and forth devices into IDE 2 port. If you have CD-ROM driver, please set it behind hard disk devices as the last device. For example, if you have 2 HDDs and 1 CD-ROM, you should set HDD1 and HDD2 in IDE1 Master and Slave, set CD-ROM in IDE 2 Master. Primary MasterPrimary SlaveSecondary MasterSecondary SlaveATA no ATAPIATAATAPI disk & CD-ROMATAATAPI use only one cableATAATAPIATAPI CD-ROM and a tape or two CD-ROMs CHAPTER 3 AWARD BIOS SETUP This chapter explains the system BIOS setup, and how to update new BIOS. All BIOS screens showed in the following pages are default values, your system dealer will set up these values according to your demand of computer. ATC-1020 uses Flash ROM to make BIOS easier to be updated by the floppy disk-based program and to committe Microsoft Win |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

dows 95 plug & play feature.

If you would like to update CMOS date also, please set JP8.

JP8

Nomal  Clear

open

NORMAL¡

close

CLEAR CMOS

¡is default setting

¡  JP19 Setting is for Update System BIOS by using Intel

Flash ROM only.

JP9 : Intel Flash ROM update

JP19

Enable Disable

1-2

Enable

2-3

Disable¡

¡is default setting

When you want to update Intel Flash ROM

you should set JP19 to ‘1-2‘.

After you finish updated, set to ‘2-3’ again.

¡ If the mainboard uses non-Intel Flash ROM, then JP19 will

be wired (2-3) in factory.

3-1 UPDATE BIOS PROCEDURES

If the BIOS needs to be updated, it can be obtained on a diskette from your system supplier. The BIOS diskette includes 3 files:

“awdflash.exe”  --  BIOS update utility program

“awdflash.doc”

“(update BIOS filename with version number).rom”

The update procedures are as following:

Boot the system to DOS mode in a normal manner.

Insert the updated diskette to drive A (or B).

Change working directory to floppy drive, A or B, which contains the update BIOS diskette. --  Type “a:\” or “b:\”, “ENTER”.

Run the BIOS update utility  --  Type “awdflash”, “ENTER”.

Type “(update BIOS file name with version number).rom”, ENTER.

Type “N” when the screen displays the message : " Do you want to save BIOS (Y/N) ?".

Type “Y“ when the screen shows the message : " Are you sure to  program (Y/N) ?".

Follow instructions displayed on the screen.  DO NOT remove the update BIOS diskette from the floppy drive nor turn the system power off until the BIOS update is completed.

Exit the utility and turn the system power off, set JP8 to 'close' to clear the CMOS. After about 10 seconds, set JP8 to 'open' (remove the jumper cap ), become the normal status.

Turn the system power on and test your system working properly or not.

3-2 AWARD SYSTEM BIOS CONFIGURATION

SETUP

The following pages explain how to set up the system configuration (CMOS) under the AWARD BIOS. The SETUP program is stored in the Read-Only-Memory (ROM) on the mainboard. Enter the SETUP procedure, press the <Del> key when the system is booting up. The following main menu will appear. Please select " STANDARD CMOS SETUP" to enter the next screen.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

The section on the bottom of the main menu explains how to control thi

| s |  |  |
| --- | --- | --- |

screen.  The other one section displays the items highlighted in the list.

| T |  |  |
| --- | --- | --- |

his screen records some basic hardware information, and set the system clo

| ck |  |  |
| --- | --- | --- |

and error handling. These records can be lost or corrupted if the on-board battery  is fail

| ed |  |  |
| --- | --- | --- |

or  weak.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STA

| ND |  |  |
| --- | --- | --- |

ARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POW

| ER |  |  |
| --- | --- | --- |

MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

ROM PCI/ISA BIOS (2A59GA29)

STANDARD CMOS SETUP

AWARD SOFTWARE, IN

C.

Dat

e (mm:dd:yy) : Fri, Jun 14 1996 Time(hh:mm:ss) : 13 : 7 : 14 HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE Primary Master : Auto 0 0 0 0 0 0 Auto Primary Slave : Auto 0 0 0 0 0 0 Auto Secondary Master : Auto 0 0 0 0 0 0 Auto Secondary Slave : Auto 0 0 0 0 0 0 Auto

Drive A : 1.44M, 3.5 in. Base Memory : 640K Drive B : None Extended memory : 64512K Other Memory : 384K Video : EGA/VGA ------------------------------------------------ Halt On: All Errors Total Memory : 65536K ESC : Quit áâàß:Select Item PU/PD/+/- : Modify F1 : Help (Shift) F2 : Change Color Date mm is month, dd is date, yy is year. date from 1 to 31 month from Ja

n. to Dec.

year

from 1900 to 2099

Time	hh is hour, mm is minute, ss is second.

hh

fr

| om 0 to 23 (24-hour military -time) mm from 1 to 59 | ss from 1 to 59 Primary Master The categories identify the types of 2 channels Primary |  |
| --- | --- | --- |

Slave	that have been installed in the computer. There

Secondary Master	are 45 predefined typ

| es and 4 user definable Secondary Slave types are for Enhanced IDE BIOS. Type 1 to 45 are predefined. Type ‘user’ is user-definable. Press PgUp/PgDn to selec |  |  |
| --- | --- | --- |

t a numbered hard

disk type or type the number and press<Enter>.

If you select ‘Auto’ BIOS will auto-detect the

HDD & CD-ROM Drive at the POST stage and

showing the IDE for HDD & CD-ROM Drive. If

you select ‘user’, you will need to know the

information listed below. Enter the information 		directly from the keyboard and press <Enter>. 		This information should be from your hard disk

vender or dealer. If the controller of HDD

interface is ESDI, the selection shall be ‘Type			1’; is SCSI, the selection shall be ‘None’.

If the device has not been installed select

‘NONE’ and press <Enter>.

Type

drive type

SIZE

automatically adjusts

CYLS

number of cylinders

HEAD

number of heads

PRECOMP

write precom

LANDZ

landing zone

SECTOR

number of sectors

MODE

mode type

Drive A	The category identifies the types of floppy disk

Drive B	drive A or drive B that have been installed in 			the computer.

None

No floppy drive installed

360K, 5.25 in

5.25“ PC-type 360KB capacity

1.2M, 5.25 in

5.25“ AT-type 1.2MB capacity

720K, 3.5 in

3.5“ double-side 720KB capacity

1.44M, 3.5 in

3.5“ double-side 1.44MB capacity

2.88M, 3.5 in

3.5“ double-side 2.88MB capacity

Video	The category selects the type of video adapter

used for the primary system monitor. Although

secondary monitors are supported, you do not

have to select the type in Setup.

Halt On	The category determines whether the computer

will stop if an error is detectd during power up.

No errors

When the BIOS detects a non-fatal error the system will be stopped and you will be prompted

All errors

The system boot will not be stopped for any error that may be detected

All, But

Keyboard

The system boot will not stop for a keyboard error, it will stop for all other errors

All, But

Diskette

The system boot will not stop for a disk error, it will stop for all oth

| er errors | All, But Disk/Key The system boot will not stop for a disk or keyboard error, it will stop for all other errors Memory The category is display-only which is determined by POST (Power On Self Test) of the BIOS. Base Memory The value of the base memory is typically 512K or 640K based on the memory installed on the mainboard. Extended Memory How much ex |  |
| --- | --- | --- |

tended

memory is present during the POST. This is

the amount of memory located above 1MB in

the CPU‘s memory address map.

Other Memory  This refers to the memory

located in the 640K to 1024K address space.

The BIOS is the most frequent user of this RA

|  | M area since this is where it shadows RAM. This screen is a list of system configuration options. Some of them are defaults required by the mainboard's design, others depend on the features of your system. ROM PCI/ISA BIOS (2A59GA29) CMOS SETUP UTILITY AWARD SOFTWARE, INC. STANDARD CMOS SETUP INTEGRATED PERIPHERALS BIOS FEATURES SETUP SUPERVISOR PASSWORD CHIPSET FEATURES SETUP USER PASSWORD POWER MANAGEMENT SETUP IDE HDD AUTO DETECTIO |  |  |  |
| --- | --- | --- | --- | --- |

N

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVI

| NG |  |  |
| --- | --- | --- |

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save &

| Ex |  |  |
| --- | --- | --- |

it Setup

(Shift) F2 : Change Color

Virus, Protection, Boot Sequence

ROM PCI/ISA

| B |  |  |
| --- | --- | --- |

IOS (2A59GA29)

BIOS FEATURES SETUP

AWARD SOFTWARE, INC.

Virus Warning

: Disabl

| ed |  |  |
| --- | --- | --- |

Video BIOS Shadow

: Enabled

CPU Interna

| l |  |  |
| --- | --- | --- |

Cache

: Enabled

C8000-CBFFF Shadow

: Disabled

External Cache

: Enabled

CC000-CFFFF Shadow

: Disabled

Quick Power On Self Test

: Disabled

D0000-D3FFF Shadow

: Disabled

Boot Sequence

: A, C

D4000-D7FFF Shadow

: Disabled

Swap Floppy Drive

: Disabled

D8000-DBFF

| F |  |  |
| --- | --- | --- |

Shadow

: Disabled

Boot Up Floppy Seek

: Enabled

DC000-DFFFF Shadow

: Disabled

Boot Up NumLock St

| at |  |  |
| --- | --- | --- |

us

: On

Boot Up System Speed

: High

Gate A20 Option

: Fast

Typematic Rate Setting

: Disabled

Typematic Rate(Chars/Sec

| ) : 6 Esc : Quit áâàß:Select Item Typematic Delay(Msec) : 250 F1 : Help PU/PD/+/- : Modify Security Option : Setup F5 : Old Values (SHIFT)F2 : Color PCI/VGA Palette Snoop : Disabled F6 : Load BIOS Defaults OS/2 select for DRAM>64MB : Non-OS2 F7 : Load Setup Defaults Report No FDD for Win95 : No |  | Virus Warning When this item is enabled, the BIOS will monitor the boot sector and partition table of the hard disk drive for any attempt at modification. If an attempt is made, the BIOS will halt the system and the following error message will appear. Many disk diagnostic programs which attempt to access the boot sector table can cause the above warning message. If you will be running such a program, we recommend that you first disable Virus Protection beforehand. ! WARNING ! Disk boot sector is to be modified Type ‘Y’ to accept write or ‘N’ to abort write Award Software, Inc. Enabled Activates automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector or hard disk partition table. Disabled | No warning message will appear when anything attempts to access the boot sector or hard disk partition table. CPU These two categories speed up memory access. Internal Cache However, it depends on CPU/chipset design. External Cache The default value is‘enabled‘. Quick Power On This category speeds up Power On Self Test Self Test after you power up the computer. If you set Enabled, BIOS will shorten or skip some ch |  |
| --- | --- | --- | --- | --- |

ec

| k items during POST. Boot Sequence This category determines which drive to search first for the disk operating system (i.e.,DOS). C,A System will first search for HDD then FDD A,C System will first search for FDD then HDD CDROM,C, A System will first search for CDROM then HDD, and next is FDD C,CDROM, A System will first search for HDD then CDROM, and next is FDD Swap Floppy This item allows you to determine whether Drive enable the swap floppy drive or not. Boot Up Floppy During POST, BIOS will determine if the floppy Seek disk drive installed is 40 tracks (360 |  |  |
| --- | --- | --- |

K) or 80

tracks (720K, 1.2M, 1.44M)

Enabled

BIOS searchs for floppy disk drive to determine if it is 40 or 80 tracks

Disabled

BIOS will not search for the type of floppy disk drive by track number

Boot Up 	This allows you to determine the default state of

NumLock Status	the numeric keypad. By default, the system

boots up with NumLock on.

Boot Up System 	Selects the default system speed - the normal

Speed	operating speed at power up.

Gate A20	This entry allows you to select how the gate

Option	A20 is handled. The gate A20 is a device used

to address memory above 1 MB. Normal is

keyboard; Fast is chipset.

Typematic Rate	This determines if the typematic rate is to be used.

Setting	When disabled, continually holding down a key

on your keyboard will generate only one instance.

Typematic Rate 	When the typematic rate is enabled, this section

(Chars/Sec)	allows you select the rate at

| which the keys are accelerated. 6 6 characters per second 8 8 characters per second 10 10 characters per second 12 12 characters per second 15 15 characters per second 20 20 characters per second | 24 24 characters per second 30 30 characters per second Typematic Delay When the typematic rate is enabled, this section (Msec) allows you select the delay between when the key was first depressed and when the acceleration begins. 250 250 msec 500 500 msec |  |
| --- | --- | --- |

750

750 msec

1000

1000 msec

Security Option 	This category allows you to limit access to the

system and Setup, or just to Setup

System

The system will not boot and access to Setup will be denIed if the correct password is not entered at the prompt

Setup

The system will boot, but access to Setup will be denIed if the correct password is not entered at the prompt

To disable security, select PASSWORD SETTING

at Main Menu and then you will be asked to enter

password. Do not

| type anything and just press <Enter>, it will disable security. Once the security is disabled, the system will boot and you can enter Setup freely. PCI/VGA It determines whether the MPEG ISA/VESA Palette Snoop VGA cards can work with PCI/VGA or not. Enabled When PCI/VGA working with MPEG ISA/VESA VGA Card Disabled When PCI/VGA not working with MPEG ISA/VESA VGA Card | OS Select for This item allows you to access the memory DRAM > 64MB that over 64MB in OS/2 Video BIOS Determines whether video BIOS will be copied to Shadow RAM. However it is optional depending on chipset design. Video |  |
| --- | --- | --- |

Shadow will increase the

video speed.

C8000 - CBFFF 	These categories determine whether option

Shadow	ROMs will be copied to RAM. An example of

DC000 - DFFFF	such option ROM would be support of onboard

Shadow 	SCSI.

This screen controls the setting for the chipset on the mainboard.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEF

| AU |  |  |
| --- | --- | --- |

LTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

AT Clock, DRAM Timmings

| . |  |  |
| --- | --- | --- |

....

ROM PCI/ISA BIOS (2A59GA29)

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

Auto

| C |  |  |
| --- | --- | --- |

onfiguration

: Enabled

Delay Transaction

: Enabled

DRAM Timing

: 70ns

DRAM RAS# Precharge Time

: 4

DRAM R/W

| Le |  |  |
| --- | --- | --- |

adoff Timing

: 6

| Fast RAS to CAS Delay : 3 DRAM Read Burst (EDO/FP) : x333/X444 DRAM Write Burst Timing : x333 Fast MA to RAS# Delay CLK : 1 Fast EDO Path Select : Disabled Refresh RAS# Assertion : 5 Clks ISA Bus Clock : PCICLK/4 |  | System BIOS Cacheable : Disabled |  |  | Video BIOS Cacheable : Disabled 8-bit I/O Recovery Time : 3 Esc: Quit |
| --- | --- | --- | --- | --- | --- |
| :Select Item 16-bit I/O Recovery Time : 2 F1 : Help PU/PD/+/-:Modify |  | Memory Hole At 15M-16M : Disabled F5 : Old Values | (Shift)F2 :Color Peer Concurrency : Enabled F6 :Load BIOS Defaults Passive Release : Enabled F7 :Load Setup Defaults Auto Pre-defined values for DRAM, cache... timing Configuration according to CPU type & system clock. When this item is enabled, the pre-defined items will become SHOW-ONLY. DRAM Timing The DRAM speed is controlled by the DRAM timing Registers. The timings programmed into this register are dependent on the system design. DRAM RAS# DRAM must continually be refreshed or it will lose Precharge Time its data. Normally, DRAM is refreshed entirely as the result of a single request. This option allows you to determine the number of CPU clocks allocated for the Row Address Strobe to accumulate its charge before the DRAM is refreshed. If insufficient time is allowed, refresh may be incomplete and data lost. Fast RAS# to When DRAM is refreshed, both rows and columns CAS# Delay are addressed separately. This setup item allows you to determine the timing of the transition from RAS to Column Address Strobe (CAS). DRAM Read This sets the timing for burst mode read (or writes) Burst (EDO/FP) from DRAM. Burst read and write requests are DRAM Write generated by the CPU in four separate parts. Burst Timing The first part provides the location within the DRAM where the read or write is to take place while the remaining three parts provide the actual data. The lower the timing numbers, the faster the system will address memory. ISA Bus Clock This item allows you to select the PCI clock type. Choices are PCI CLK/3; PCI CLK/4 System BIOS When enabled, accesses to the system BIOS Cacheable ROM addressed at F0000H-FFFFFH are cached, provided that the cache controller is enabled. Video BIOS As with changing the system BIOS above, Cacheable enabling the Video BIOS cache will cause access to video BIOS addressed at C0000H to C7FFFH to be cached, if the cache controller is also enabled. 8-bit I/O The recovery time is the length of time, measured Recovery Time in CPU clocks, which the system will delay after the completion of an I/O request. This item allows you to determine the recovery time allowed for 8- bit I/O. Choices are from NA, 1 to 8 CPU clocks. 16-bit I/O This item allows you to determine the recovery Recovery Time time allowed for 16-bit I/O. Choices are from NA, 1 to 4 CPU clocks. Memory Hole At In order to improve performance, certain space 15M-16M in memory can be reserved for ISA cards. This memory must be mapped into the memory below 16MB. This screen controls the 'green' features of this mainboard. ROM PCI/ISA BIOS (2A59GA29) CMOS SETUP UTILITY AWARD SOFTWARE, INC. STANDARD CMOS SETUP INTEGRATED PERIPHERALS BIOS FEATURES SETUP SUPERVISOR PASSWORD CHIPSET FEATURES SETUP USER PASSWORD POWER MANAGEMENT SETUP IDE HDD AUTO DETECTION PNP/PCI CONFIGURATION SAVE & EXIT SETUP LOAD BIOS |  |  |

DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

Sleep Timer, Suspe

| nd |  |  |
| --- | --- | --- |

Timer, .....

ROM PCI/ISA BIOS (2A59GA29)

POWER MANAGEMENT SETUP

AWARD SOFTWARE, INC.

Power Management

: Disabled

IRQ3  (COM 2)

: OFF

PM Control by APM

: Yes

| IRQ4 (COM 1) : OFF Video Off Method : V/H SYNC IRQ5 (LPT 2) : OFF +Blank IRQ6 (Floppy Disk) : OFF IRQ7 (LPT 1) : OFF Doze Mode : Disabled IRQ8 (RTC Alarm) : OFF Standby Mode : Disabled IRQ9 (IRQ2 Redir) : OFF Suspend Mode : Disabled IRQ10 (Reserved) : OFF HDD Power Down : Disabled IRQ11 (Reserved) : OFF IRQ12 (PS/2 Mouse) : OFF IRQ13 (Coprocessor) : OFF *Wake Up Events In Doze & Standby* IRQ14 (Hard Disk) : OFF IRQ3 (Wake-Up Event) : OFF IRQ15 (Reserved) : OFF IRQ4 (Wake-Up Event) : OFF IRQ8 (Wake-Up Event) : OFF Esc: Quit áâàß :Select Item IRQ12(Wake-Up Event) : OFF F1 : Help PU/PD/+/- : Modify | F5 : Old Values (Shift) F2: Color |  | F6 : Load BIOS Defaults |  | F7 : Load Setup Defaults Power This category allows you to select the type (or Management degree) of power saving and is dirctly related to the following modes : Doze; Standby; Suspend; HDD Power Down. Disabled No power management. Disables all 4 modes | Min. Power Saving Minimum power management. Doze =1hr.; Standby=1hr.; Suspend=1hr.; HDD Power Down=15min Max. Power Saving Maximum power management only available for SL CPU‘s.Doze=1min.; Standby=1min.;Suspend=1min.;HDD Power Down=1min User Defined Allows you to set each mode indivi-dually. When not disabled, each of the ranges are from 1min. to 1hr. except for HDD Power Down which ranges from 1 to 15min. and disable PM Control When enabled, an Advanced Power Management by APM device will be activated to enhance the Max. Power Saving Mode and stop the CPU internal clock. If the Max. Power Saving is not enabled, this will be present to NO. Video Off This determines the manner in which the monitor Method is blanked. V/H SYNC + Blank This selection will cause the system to turn off the vertical and horizontal sync. ports and write blanks to the video buffer Blank Screen This option only writes blanks to the video buffer DPMS Initial display power management signaling The Following 4 modes are Green PC power saving function which are only user config |
| --- | --- | --- | --- | --- | --- | --- |

uration when ‘User Defined’ power management has been selected.

Doze Mode	When enabled and after the set time of system

inactivity, the CPU clock will run at slower speed

while all other devices still operate at full speed

Standby Mode	When enabled and after the set time of system

inactivity, the fixed disk drive and the video

would be shut off while all other devices still

operate at full speed

Suspend Mode	When enabled and after the set time of system

inactivity, all devices expect the CPU will be shut

off

HDD Power Down	When enabled and after the set time of system

inactivity, the hard disk drive will

| be powered down while all other devices remain active Wake Up Events In Doze & Standby Power Down & These are I/O events whose occurrence can Resume Events prevent the system from entering a power saving mode or can awaken the system from such a IRQ3 ~ IRQ15 mode. In effect, the system remains alert for anything which occurs to a device which is configured as On, even when the system is in a power down mode. When an I/O device wants to gain the attention of the operating system, it signals thi |  |  |
| --- | --- | --- |

s by causing an IRQ (Interrupt ReQuests)

to occur. When the operating system is ready to

respond to the request, it interrupts itself and

performs the service. When set off, activity will

neither prevent the system from going into a power

management mode nor awaken it.

This screen configures the PCI Bus slots.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

IRQ Settings, Latency Timers, .....

ROM PCI/ISA BIOS (2A59GA29)

PNP/PCI CONFIGURATION

AWARD SOFTWARE, INC.

Resources Controlled by

: Auto

PCI IRQ Actived by

: Level

Reset Configuration Data

: Disabled

PCI IDE IRQ Map to

: PCI-AUTO

Primary IDE INT#

: A

Seco

| ndary IDE INT# : B Esc: Qui áâàß :Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2: Color F6 : Load BIOS Defaults F7 : Load Setup Defaults Resource The Award Plug and Play BIOS has the capacity Controlled by to automatically configure all of the boot and Plug and Play compatible devices. However, this capability means absolutely nothing unless you are using a Plug and Play OS such as Windows 9 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

5

Choices are Auto and Manual

Reset Config-	This item allows you to determine reset the

uration Data	configuration data or not.

PCI IRQ	This sets the method by which the PCI bus reco-

Actived By	gnizes that an IRQ service is being requested by a

device. Under all circumstances, you should retain

the default configuration unless advised otherwise

by  your system‘s manufacturer.

Choices are Level and Edge

PCI IDE IRQ	This allows you to configure your system to the

Map To	type of IDE disk controller in use. If you have

equipped your system with a PCI controller,

changing this allows you to specify which slot has

the controller and which PCI interrupt (A,B,C,D) is

associated with the connected hard disk. Select

‘PCI Auto’ allows the system to automatically

determine how your IDE disk system is configured.

This section page includes all the items of IDE hard drive and Programmed

Input/Output features. See also Section “Chipset FeaturesSetup”.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

ROM PCI/ISA BIOS (2A59GA29)

INTEGRATED PERIPHERALS

AWARD SOFTWARE, I

| NC. IDE HDD Block Mode : Enabled ECP Mode Use DMA : 3 |  | IDE 32-bit Transfer Mode : Enabled IDE Primary Master PIO : Auto IDE Primary Slave PIO : Auto IDE Secondary Master PIO : Auto | IDE Secondary Slave PIO : Auto On-Chip Primary PCI IDE : Enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| IDE HDD Block This allows your HD controller to use the On-Chip Secondary PCI IDE : Enabled PCI Slot IDE 2nd Channel : Enabled USB Controller : Disabled |  | Onboard FDD Controller : Enabled Onboard Serial Port 1 : Auto | Onboard Serial Port 2 |  | : Auto Esc: Quit áâàß :Select Item UART 2 Mode : Standard |
| : 378H/IRQ7F1 : Help F5 : Old Values (Shift) F2: Color Onboard Parallel Mode : ECP F6 : Load BIOS Defaults F7 : Load Setup Defaults PU/PD/+/- : Modify Onboard Parallel Port |  |  |  |  |  |

fast block

Mode	mode to transfer data to and from your HD drive

Enabled

IDE controller uses block mode

Disabled

IDE controller uses standard mode

IDE Primary	PIO - Programmed Input/Output, it allows the

Master/Slave PIO	BIOS to tell the

| controller what it wants and then IDE Secondary let the controller and the CPU perform the complete Master/Slave PIO task by themselves. This simpler and more faster. You |  |  |
| --- | --- | --- |

r system supports five mods, 0 - 4, which

primarily differ in timing. When Auto is selected,

the BIOS will select the best available mode.

On-Chip Primary	This setup item allows you either to enable or

PCI IDE 	disable the primary/secondary controller. You might

On-Chip Second- 	choose to disable the controller if you were to add

ary PCI IDE 	a higher performance or specialized controller.

PCI Slot IDE 	This item allows you designate an IDE controller

2nd Channel 	controller board insert into one of the physical PCI

slots as your secondary IDE

Enabled

External IDE controller designated as the secondary controller

Disabled

No IDE controller occupying a PCI slot

The last step is 'save and exit'. If you select this item and press 'Y', then these records will be saved in the CMOS memory on the mainboard. It will be checked every time when you turn your computer on.

ROM PCI/ISA BIOS (2A59GA29)

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

STANDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSWORD

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD A

| UT |  |  |
| --- | --- | --- |

O DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WI

| TH |  |  |
| --- | --- | --- |

OUT SAVING

LOAD SETUP DEFAULTS

ESC: Quit

áâàß:Select Item

F1

| 0: |  |  |
| --- | --- | --- |

Save & Exit Setup

(Shift) F2 : Change Color

Time, Date, Hard Disk Type .....

S

| TA |  |  |
| --- | --- | --- |

NDARD CMOS SETUP

INTEGRATED PERIPHERALS

BIOS FEATURES SETUP

SUPERVISOR PASSW

| OR |  |  |
| --- | --- | --- |

D

CHIPSET FEATURES SETUP

USER PASSWORD

POWER MANAGEMENT SETUP

IDE HDD AUTO DETECTION

PNP/PCI CONFIGURATION

SAVE & EXIT SETUP

LOAD BIOS DEFAULTS

EXIT WITHOUT SAVING

LOAD SETUP DEFAULTS

SAVE to CMOS and EXIT (Y/N):Y

ESC: Quit

áâàß:Select Item

F10: Save & Exit Setup

(Shift) F2 :

| Ch |  |  |
| --- | --- | --- |

ange Color

Auto-Configure HDD: Sector, Cylinder, Head .....

LOAD BIOS DEFAULTS

When your mainboard has problems and need to debug or 		troubleshoot the system, you can use this function.

The default values loaded only affect the BIOS Features

Setup, Chipset Features Setup, Power Management

Setup and PNP/PCI Configuration Setup. There is no

effect on the Standard CMOS Setup. To use this function,

select it from main menu and press <Enter>. A line will

appear on the screen asking if you want to load the BIOS

default values. Press <Yes> and <Enter> then the BIOS

default values will be loaded.

LOAD SETUP DEFAULTS

This allows you load optimal settings which are stored in

the BIOS ROM. The default values loaded only affect the

BIOS Features Setup, Chipset Features Setup, Power

Management Setup and PNP/PCI Configuration Setup.

There is no effect on the Standard CMOS Setup. To use

this function, select it from main menu and press <Enter>.

A line will appear on the screen asking if you want to load

the Setup default values. Press <Yes> and <Enter> then

the Setup default values will be loaded.

SUPERVISOR PASSWORD / USER PASSWORD

This allows you to set the password. the mainboard 	defaults password disabled.

Enter/Change password : Enter the current password, at

the prompt key-in your new password (up to eight

alphanumeric characters), press <Enter>. At the next

prompt, confirm the new password by typing it and press

<Enter> again.

Disable password : Press <Enter> key instead of

entering a new password when the ‘Enter Password’

dialog box appears. A message will appear confirming that

the password is disable.

If you set both supervisor and user passwords, only the

supervisor password allows you to enter the BIOS SETUP

program.

CAUTION :If you forget your password, you must disable

the CMOS by turning power off and set JP8 ‘close‘. Then

reload the system. Please refer to page 13.

IDD HDD AUTO DETECTION

This allows you to detect IDE hard drivers‘ parameters

and enter them into ‘Standard CMOS Setup’ automatically.

If the auto-detected parameters displayed do not

match the ones that should be used for your hard drive,

do not accept them. Press <N> to reject the values

and enter the correct ones manually on the Standard

CMOS Setup screen.

SAVE & EXIT SETUP

This allows you to save the new setting values in the

CMOS memory and continue with the booting process.

Select what you want to do, press <Enter>.

EXIT WITHOUT SAVING

This allows you to exit the BIOS setup utility without

recording any new values or changing old ones.

¡ Control Key Description ¡

UP ARROW

¡

Move to previous item

DOWN ARROW

¡

Move to next item

LEFT ARROW

¡

Move to the item in the left hand

RIGHT ARROW

¡

Move to the item in the right hand

Esc KEY

Esc

Main Menu : Quit and not save

changes

Setup menu : Exit current page and return to main menu

PgUp KEY

Increase the numeric value or make changes

PgDn KEY

Decrease the numeric value or make changes

F1 KEY

Help

General help

F2 KEY

¡Shift¡+F2

Change color fr

| om |  |  |  |  |
| --- | --- | --- | --- | --- |

total 16 colors

F5 KEY

Old Value

Restore the pervious CMOS value from CMOS

F6 KEY

Load BIOS default

Loa

| d |  |  |  |  |
| --- | --- | --- | --- | --- |

the default CMOS value from BIOS default table

F7 KEY

| Lo |  |  |  |  |
| --- | --- | --- | --- | --- |

ad setup default

Load Setup default

F10 KEY

Save & Exit Setup

Save all the CMOS changes and Exit setup

| , |  |  |  |  |
| --- | --- | --- | --- | --- |

only for Main Menu

APPENDIX A

¡¡TECHNICAL SUPPORT REQUEST FORM¡¡

If the mainboard doesn't function properly,

| pl |  |  |  |  |
| --- | --- | --- | --- | --- |

ease complete the following

information and return it to your system dealer. If the further information

is needed, please attach this separating sheets.

Model

| N |  |  |  |  |
| --- | --- | --- | --- | --- |

o :  ATC-1020      Date of  Purchase :  ______________

Serial No : ___________________

HARDWA

| RE |  |  |  |  |
| --- | --- | --- | --- | --- |

:

BRAND

MODEL

SPEED

Q'TY

CPU

SIM Module

PB. Cache on Board

TAG SRAM

Sy

| nc |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

. SRAM Module

SIMM  :__

| __ |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

___MB (__ EDO, __FastPage)

Hard Disk Interface

| Co |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

ntroller : __ __ IDE, ____ SCSI

Hard Disk Brand :___________, Model :_________, Capacity :________

Display Controller Brand :_____________,Model :__________

Controller Chip Brand :_____________,Model :__________

AWARD SYSTEM BIOS: Version _______  Date Code _____________

Keyboard BIOS: Brand __________

Other Add-on Cards Information:

Add-on Card

Bus Interface

Model

Remark

Error Description :