Direct I/O using Advantech 32-bit DLL Driver Ver 1.xx for Windows 95 and Windows NT

How to install 32-bit DLL driver

Please follow below procedure to install Advantech 32-bit DLL Driver and setup a supported device in the driver for your first time.

- When you download the 32-bit driver from Advantech WWW server, www.advantech-usa.com/techhome/tech1.htm. In tech1.htm page, there is a list of driver for each product. As you get any file from above server, please unzip the file before you install.

DLL3295.100		: Advantech Driver for Windows 95 Ver 1.00

DLL32NT.100		: Advantech Driver for Windows NT Ver 1.00

- The driver supports 32-bit Operating System environment, Windows 95/NT, and 100 means the version number of Advantech 32-bit DLL driver. It is in ZIP file format which needs to be uncompressed by any available WINZIP utility. Because the default file type for WINZIP is ‘ZIP’, you should use full filename in the command line. For example,

- Pkunzip -d DLL32NT.100

- After you unzip the 32-bit DLL driver, please use SETUP.EXE on /DISK1 directory to install the driver. When it comes to below diagram, you should select correct Operating System for your current working PC.

- After you finish the driver installation, you should run Device Installation (DEVINST.EXE) under Advantech Driver for NT (for Windows NT) / Advantech Driver for 95 folder to setup supported devices. Please refer to DEVINST.HLP for more information.

New Device Installation Utility for Windows NT and 95

- The necessary files for programming:

Below files are located in \Program Files\Advantech\Adsapi\ directory after you install 32-bit DLL driver for Windows 95/NT.

- Below is required DLL files for Direct I/O.

\WinNT\ADSAPI32.DLL	: Include Uniform function API to call to specific function for Windows NT.

\Windows\ADSAPI32.DLL	: Include Uniform function API to call to specific function for Windows 95.

\WinNT\System32\ADSIO.DLL	: DLL driver to call to Device/Kernal driver for Widnows NT.

\WinNT\System32\Drivers\ADSIO.SYS	:.Device/Kernal driver to access register of Hardware for Widnows NT.

\Windows\System\ADSIO.DLL	: DLL driver to call to Device/Kernal driver for Widnows 95.

\Windows\System\ADSIO.SYS	:.Device/Kernal driver to access register of Hardware for Widnows 95.

\AdsapiVINST.EXE	: Device Instllation utility to install ‘Unlisted Board for Direct I/O access’.

\Adsapi\MANUAL.DOC	: User’s Manual for Advantech 32-bit DLL driver.

Examples\VC\LIB\ADSAPI32.LIB	: Header library of ADSAPI32.DLL for Visual C++.

Examples\VC \LIB\ADSAPIBC.LIB	: Header library of ADSAPI32.DLL for Borland C++.

Examples\VC \LIB\ADSAPIBC.LIB	: Header library of ADSAPI32.DLL for Borland C++  Builder.

\Include \DRIVER.H	: Header file for Visual C++, Borland C++ /Builder.

\Include \OS.H	: Header file for Visual C++, Borland C++ /Builder.

\Include \DRIVER.PAS	: Header library of ADSAPI32.DLL for Borland Delphi.

\Include \DRIVER.BAS	: Header library of ADSAPI32.DLL for Visual Basic.

How to install device through Device Installation

Please follow below procedure to setup any Advantech DA&C products.

- Launch Device Installation utility from  Advantech Driver for 95 or Advantech Driver for NT folder to setup direct I/O device. You will see below diagram as you launch it.

- Select Device... item from Setup menu, it will shows below I/O Device Installation dialog. Press Add>> button to show lists of device and then press Install button to add a device. Here the Unlisted Boards for Direct I/O Access item is your only choice to perform direct I/O access.

- Enter Direct I/O (43) in Description (You can enter any string as you want) , 43 is Port’s Address, any value (Here I type in 4) for span range for this device, and select a free IRQ Channel.

- After you finish device setup, press close button to quit this utility. Now it is ready for you to perform direct I/O access in your application. Now it is ready for coding.

How to compile in Mircosoft Visual C++ V2.0 or above?

- Please new a Project which you should set it in using class library.

- Import \Program Files\Advantech\Examples\VC\LIB\ADSAPI32.LIB.

- Add your filename of source code into this project.

How to compile in Borland C++ Builder, Borlanc C++ V5.0 or above?

- Please new a Project which you should set it in using class library, don’t use OWL.

- Import \Program Files\Advantech\Examples\BC\LIB\ADAPIBC.LIB for Borland C++ or  \Program Files\Advantech\Examples\BCB\LIB\ADAPIBC.LIB for Borland C++ Builder. The Header Libraries for BC++/BC++ Builder are the same, but it is different from the .LIB file for VC++. If you want to call to any DLL by BC++/BC++ Builder, you can apply IMPLIB utility to create new .LIB files.

- Syntax: IMPLIB [options]   libname[.lib]   dllname[.dll]

- Example:

- 1)  IMPLIBΔC:\Program Files\Advantech\Adsapi\Examples\BC\LIB \ADSAPIBC.LIB

- ΔC:\WinNT\ADSAPI32.DLL

- 2) IMPLIBΔC:\Program Files\Advantech\Adsapi\Examples\BC\LIB \ADSAPIBC.LIB ΔC:\Windows\ADSAPI32.DLL

- Add your filename of source code into this project.

- You configure the Data Alignment to Quad Word (8-Byte) in the Processor item of 32-bit Compiler setup of Project Options. Because the default Data Alignment of Visual C++, Visual Basic and Delphi are in Quad Word (8-Byte), you don’t have to change the configuration in those programming language environment.

How to compile in Mircosoft Visual Basic V4.0 or above?

- Please new a Project which you should set it in using class library.

- Add \Include\DRIVER.BAS into this project.

- Add your filename of source code into this project.

How to compile in Borland Delphi V2.0 or above?

- Please Open a Project which you should set it in using class library.

- Add \Include\DRIVER.PAS into this project.

- Add your filename of source code into this project.The Direct I/O Functions’ description and its declaration for above programming language:

Function Description

DRV_outp	: Output a byte to a hardware port.

DRV_outpw	: Output a word to a hardware port.

DRV_inp	: Read a byte from a hardware port.

DRV_inpw	: Read a word from a hardware port.

Borland C++ Builder, Borland C++ V5.0 or above, Visual C++ V2.0 or above

FEXPORT LRESULT FTYPE DRV_outp(ULONG DeviceNum, USHORT port, USHORT ByteData);

FEXPORT LRESULT FTYPE DRV_outpw(ULONG DeviceNum, USHORT port, USHORT WordData);

FEXPORT LRESULT FTYPE DRV_inp(ULONG DeviceNum, USHORT port, USHORT far * ByteData);

FEXPORT LRESULT FTYPE DRV_inpw(ULONG DeviceNum, USHORT port, USHORT far * WordData);

Borland Delphi V2.0 or above

Function DRV_outp(DeviceNum : Longint; port : Word; ByteData : Word) : Longint; stdcall;

Function DRV_outpw(DeviceNum : Longint; port : Word; ByteData : Word) : Longint; stdcall;

Function DRV_inp(DeviceNum : Longint; port : Word; var ByteData : Word) : Longint; stdcall;

Function DRV_inpw(DeviceNum : Longint; port : Word; var ByteData : Word) : Longint; stdcall;

Visual Basic V4.0 or above

Declare Function DRV_outp Lib "adsapi32.dll" (ByVal DeviceNum As Long, ByVal port As Integer, ByVal ByteData As Long) As Long

Declare Function DRV_outpw Lib "adsapi32.dll" (ByVal DeviceNum As Long, ByVal port As Integer, ByVal ByteData As Long) As Long

Declare Function DRV_inp Lib "adsapi32.dll" (ByVal DeviceNum As Long, ByVal port As Integer, ByteData As Long) As Long

Declare Function DRV_inpw Lib "adsapi32.dll" (ByVal DeviceNum As Long, ByVal port As Integer, ByteData As Long) As Long

- DRV_outp, DRV_outpw

- status = DRV_outp(DeviceNum, port, ByteData)

- status = DRV_outpw(DeviceNum, port, ByteData)

- Purpose

- Output a byte/word data to the port’s address

- Parameters

| - Name | - Direction | - Type | - Range | - Description |
| --- | --- | --- | --- | --- |
| - DeviceNum | - Output | - long | - depend on your setup in device installation | - device’s number |
| - port | - Output | - unsigned short | - port ~ - (port+span-1) | - absolute address |
| - ByteData | - Output | - unsigned short | - 65535 | - output entries |

- Return: 1) SUCCESS (0) if Successful

- 2) MemoryAllocateFailed if Not Enough Memory

- ConfigDataLost if Configuration Data Lost

- CreateFileFailed if Create Low Level Driver Failed

- InvalidDriverHandle if Invalid Driver Handle Failed

- WRTInvalidHandleValue if An error occurred while starting the device

- WRTFileNotFound if The device has not been created

- WRTInvalidHandle if The handle passed to the function is not a valid

- WRTTooManyCmds if The logic commands have created an apparent endless loop

- WRTInvalidParameter if Passed to the driver contains an invalid parameter

- WRTNoAccess if Attempts to access a port which has not been defined in DEVINST

Example:

glDevNum = 0;	// The device number which you set in 43 Hex

gwPort = 43; 	// The real I/O  address which you want to access

if((ErrCde = DRV_outp(glDevNum, gwPort, gwDoState)) != SUCCESS)

{

DRV_GetErrorMessage(ErrCde, (LPSTR) szErrMsg);

MessageBox(hDlg, (LPCSTR) szErrMsg, "Driver Message", MB_OK);

return 0;

- }

DRV_inp, DRV_inpw

- status = DRV_inp(DeviceNum, port, ByteData)

- status = DRV_inpw(DeviceNum, port, ByteData)

- Purpose

- Read a byte/word data from the port’s address

- Parameters

| - Name | - Direction | - Type | - Range | - Description |
| --- | --- | --- | --- | --- |
| - DeviceNum | - Output | - long | - depend on your setup in device installation | - device’s number |
| - port | - Output | - unsigned short | - port ~ - (port+span-1) | - absolute address |
| - ByteData | - input | - unsigned short | - 65535 | - input entries |

- Return:     1) SUCCESS (0) if Successful

- 2) MemoryAllocateFailed if Not Enough Memory

- ConfigDataLost if Configuration Data Lost

- CreateFileFailed if Create Low Level Driver Failed

- InvalidDriverHandle if Invalid Driver Handle Failed

- WRTInvalidHandleValue if An error occurred while starting the device

- WRTFileNotFound if The device has not been created

- WRTInvalidHandle if The handle passed to the function is not a valid

- WRTTooManyCmds if The logic commands have created an apparent endless loop

- WRTInvalidParameter if Passed to the driver contains an invalid parameter

- WRTNoAccess if Attempts to access a port which has not been defined in DEVINST

Example:

glDevNum = 1;	// The device number which you set in 443 Hex

gwPort = 1091; 	// The real I/O  address which you want to access, 1091(Dec) = 443 (Hex)

if((ErrCde = DRV_inp(glDevNum, gwPort, (USHORT far *)&gwDoState)) != SUCCESS)

{

DRV_GetErrorMessage(ErrCde, (LPSTR) szErrMsg);

MessageBox(hDlg, (LPCSTR) szErrMsg, "Driver Message", MB_OK);

return 0;

}

Case Study I : Program Watchdog Timer on MIC-2350 (Reset)

To enable the watchdog timer, you must write a program which writes I/O port address 443 (HEX) at a regular intervals. The output data is a timer value. You can write from 01(HEX)  to 3C(HEX), and the related timer is 1 second to 60 seconds. Please refer to WATCHDOG.MAK for more information. (Watchdog Jumper is set to Reset)

- Please follow above I/O device installation operation procedure to setup the HEX 43 and HEX 443 devices.

- Output DataValue to HEX 443 to start/reset the watchdog timer.

- ErrCde = DRV_outp(1, HEX443, DataValue)

- Output DataValue to HEX 443 to disable the watchdog timer.

- ErrCde = DRV_inp(0, HEX43, DataValue)

- Case Study II : Program Digital Input / Output port on PPC-100 series

The PPC-100 provides 4 bit digital input channels and 4-bit digital output channels. To write the DIO channels, you must write 0 ~ 15 to I/O port address 430 (HEX). To read the status of DIO channels, you must read from I/O port address 430 (HEX).

- Please follow above I/O device installation operation procedure to setup the HEX 430. Because you need one port only, please set the value of port to 1 and its Port Address Hex 430.

- Output DataValue (0 ~ 15) to HEX 430 to enable some digital output bits.

- ErrCde = DRV_outp(1, 430, DataValue)

- Output DataValue to HEX 430 to read 4 digital output bits.

- ErrCde = DRV_inp(0, HEX43, DataValue)

- DataValue = DataValue AND 0x0f	‘ read 4 bits only