How to demo 6 channels 48-bits/line output by PCM-3724?

- Please run “setup.exe” in disk1 to install the Win95 driver.

In the dialog box “Advantech Device Installation”, select “Setup”, see Fig.1

Fig.1

- Then see “in the list of Devices”, select Advantech PCM-3724 then select “install”. See Fig.2.

Fig.2

# Fig.3

- Select “Setup”, you can see “Advantech PCM-3724 Digital I/O Card Setup” dialog. See Fig.3.

There are two channels, Channel1 and Channel2 with each 3 Ports.  For each port, it can control 8 bits digital I/O.  So PCM-3724 can control total 48 digital I/O line.

5. Select “OK” to finish the driver setup.

6.Go to the driver directory, For example, C:\ADVANAPI\examples\c\DIGOUT to run the demo program “digout.exe”. See Fig.4.

# Fig.4

7.Double click the “digout” icom to run “digout.exe”, in “Advantech Driver Demo: Digital Output” box, select “Setting”. See Fig.5.

# Fig.5

8.See “Parameter Settings” box in Fig.5, you can find Device is PCM-3724I/O=300H (Default setting). In the option “Channel”, You can select 0 to 5 value one time, represent 6 different channels.

9.Don’t care about “Mask”, keep the value ff.  Select “Channel 0” and “OK” go back to “Advantech Driver Demo: Digital Output” box.

10. Select “Run” you will see “Output Settings” box. See Fig.6.  In option “Output Bit” select 1.  Then press “Write Byte”, it means now we output 1 Byte (=8 bits) signal in J3/Port1, from PA00 to PA07.  So you can detect about +5 Volt DC in those 8 pins (Pin33 ~48).  And in the mean time other pins should without any voltage except pin49 and 50.

# Fig.6

11. Repeat step 8 but change the Channel value from 1 to 5 each time and follow the same step 9 and 10, you can find the signal output in both two port, total 48 bits I/O demo.