Most systems are already configured by the manufacturer or the dealer.  There is no need to run Setup when starting the computer unless you get a Run Setup message.

The Setup program loads configuration values into the battery-backed nonvolatile memory called CMOS RAM.  This memory area is not part of the system RAM.

|  | If you repeatedly receive Run Setup messages, the battery may be bad. In this case, the system cannot retain configuration values in CMOS. Ask a qualified technician for assistance. |
| --- | --- |

Before running Setup, have the following information ready:

- Diskette drive type  The standard type is either a 5.25-inch 1.2-MB or a 3.5-inch 1.44-MB diskette drive.

- IDE hard disk drive type  The drive information is on a label pasted on your IDE drive or in the documentation supplied by the vendor.

# 2.1	Entering Setup

To enter Setup, press the key combination ++.

|  | You must press ++ while the system is booting. This key combination does not work during any other time. |
| --- | --- |

The BIOS Utility main menu then appears:

| BIOS Utility |  |  |  |
| --- | --- | --- | --- |
| Basic System Configuration Advanced System Configuration PnP/PCI System Configuration Power Saving Configuration System Security Load Setup Default Settings |  |  |  |
| ¯¬® = Move Highlight Bar, ¿ = Select, Esc = Exit and Reboot |  |  |  |
|  |  | The parameters on the screens show default values. These values may not be the same as those in your system. The grayed items (denoted with asterisks) on the screens have fixed settings and are not user-configurable. |  |

# 2.2	Basic System Configuration

Select Basic System Configuration to input configuration values such as date, time, and disk types.

The following screen shows the Basic System Configuration menu.

Basic System Configuration Page 1/2

Date [MM/DD/YY] Time [HH:MM:SS] Diskette Drive A [xx-MB xx-inch] Diskette Drive B [xx-MB xx-inch] Cylinder Head Sector IDE Drive 0 (xxx MB) [Auto] xx xx xx IDE Drive 1 (xxx MB) [Auto] xx xx xx IDE Drive 2 [Auto] xx xx xx IDE Drive 3 [Auto] xx xx xx Total Memory [ 8] MB

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

The command line at the bottom of the menu tells you how to highlight items, change settings, and move from one screen to another.

Press  or  on the cursor-edit keypad to highlight the desired parameter.

Press  or  to select the desired option for a parameter.

Press  to move to the next page or  to return to the previous page.

Press  to exit the configuration menu.

The following screen shows page 2 of the Basic System Configuration menu.

Basic System Configuration Page 2/2

Enhanced IDE Features Hard Disk Block Mode [Enabled ] Advanced PIO Mode [Enabled ] Hard Disk Size > 504MB [Enabled ] Hard Disk 32-Bit Access [Enabled ] Large Memory Support Mode [Normal ] Num Lock After Boot [Enabled ] Memory Test [Disabled] Quiet Boot [Disabled] Configuration Table [Enabled ]

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

The following sections explain the different parameters and their settings.

## 2.2.1	Date and Time

The real-time clock keeps the system date and time.  After setting the date and time, you need not enter them every time you turn on the system.  As long as the internal battery remains good (approximately seven years) and connected, the clock continues to keep the date and time accurately even when the power is off.

Date

Highlight the items on the date parameter and press  or  to set the date following the month-day-year format.

Valid values for month, day, and year are:

- Month	1  to 12

- Day	1  to 31

- Year	00  to 99

Time

Highlight the items on the time parameter and press  or  to set the time following the hour-minute-second format.

Valid values for hour, minute, and second are:

- Hour	00 to 23

- Minute	00 to 59

- Second	00 to 59

## 2.2.2	Diskette Drives

To enter the configuration value for the first diskette drive (drive A), highlight the Diskette Drive A parameter.  Press  or  key to view the options and select the appropriate value.

Possible settings for the Diskette Drive parameters:

- [        None        ]

- [360  KB,   5.25-inch]

- [1.2  MB,   5.25-inch]

- [720  KB,    3.5-inch]

- [1.44 MB,    3.5-inch]

- [2.88 MB,    3.5-inch]

Follow the same procedure for Diskette Drive B.  Choose None if you do not have a second diskette drive.

## 2.2.3	IDE Drives

Move the highlight bar to the IDE Drive 0 parameter to configure the first IDE drive (drive C).  Press  or  to display the IDE drive types with their respective values.  Select the type that corresponds to your IDE hard disk drive.  Follow the same procedure for the other IDE drives, if any.  Choose None if you do not have other drives.

### Selecting the “Auto” Option

If you do not know the exact type of your IDE hard disk drive, select the option Auto.  During the power-on self-test (POST), when the system performs self-testing and self-initialization before loading the operating system and applications, the BIOS utility automatically determines your IDE drive type.  You can see the drive type and its values when you enter the BIOS Utility.

Cylinder	Head	Sector

IDE Drive 0 (xx MB)	[Auto]	xx	xx	xx

If desired, you can save the values under the option User.

Cylinder	Head	Sector

IDE Drive 0 (xx MB)	[User]	xx	xx	xx

The next time you boot the system, the BIOS utility does not have to auto-configure your IDE drive as it detects the saved disk information during POST.

|  | We recommend that you copy the IDE disk drive values and keep them in a safe place in case you have to reconfigure the disk in the future. |
| --- | --- |

Follow the same procedure to auto-configure other IDE drives.

### Selecting the “User” Option

There are cases when you cannot use the option Auto, instead you have to select User.  Choose the User option when you have installed an IDE hard disk that was previously formatted but does not use the disk native parameters or structure, that is, the disk type may be in the IDE hard disk types list but the number of cylinders, heads, and sectors differ.

Follow these steps to configure an IDE hard disk with the User option:

1.	Highlight an IDE drive parameter.

2.	Select the option User and press .

3.	Type in the number of cylinders, heads, and sectors of the drive under the appropriate columns.

|  | Be sure to have the correct IDE hard disk drive information beforehand. |
| --- | --- |

4.	Choose Yes when asked if you want to save CMOS data.

## 2.2.4	Total Memory

The system automatically detects the total amount of onboard memory during the POST and sets this parameter accordingly.  If you install additional memory, the system automatically adjusts this parameter to display the new memory size.

## 2.2.5	Enhanced IDE Features

### Hard Disk Block Mode

This function enhances disk performance depending on the hard disk in use.  If you set this parameter to Enabled, it allows data transfer in block (multiple sectors) by increasing the data transfer rate to 256 bytes per cycle.  If your system does not boot after enabling this parameter, change the setting to Disabled.  This parameter is normally set to Enabled.

### Advanced PIO Mode

Enabling this parameter allows for faster data recovery and read/write timing that reduces hard disk activity time.  This results to better hard disk performance.

To utilize this feature, your hard disk must support the advanced PIO mode.  If not, set this parameter to Disabled.  Refer to your hard disk documentation for information about the advanced PIO mode.

### Hard Disk Size > 504 MB

This enhanced IDE feature works only under DOS and Windows 3.x environments.  If enabled, it allows you to use a hard disk with a capacity of more than 504 MB.  This is made possible through the Logical Block Address (LBA) mode translation.  Other operating systems require this parameter to be set to Disabled.

### Hard Disk 32-bit Access

Enabling this parameter improves system performance by allowing the use of the 32-bit hard disk access.  This enhanced IDE feature works only under DOS, Windows 3.x and Novell NetWare.  If your software or hard disk does not support this function, set this parameter to Disabled .

## 2.2.6	Large Memory Support Mode

The available settings for this parameter are Normal and Advanced.  Set this parameter to Advanced if you are working under Windows NT 3.1 environment.  The setting allows you to support an extended memory higher than 64 MB.  The default setting is Normal.

## 2.2.7	Num Lock After Boot

This parameter allows you to activate the Num Lock function upon booting.  The default setting is Enabled.

## 2.2.8	Memory Test

When set to Enabled, this parameter allows the system to perform a RAM test during the POST routine.  When set to Disabled, the system detects only the memory size and bypasses the test routine.  The default setting is Disabled .

## 2.2.9	Quiet Boot

This parameter enables or disables the quiet boot function.  When set to Enabled, BIOS is in graphical mode and displays only an identification logo during POST and while booting.  After which the screen displays the operating system prompt (such as DOS) or logo (such as Windows 95).  If any error occurred while booting, the system automatically switches to the text mode.

Even if your setting is Enabled, you may also switch to the text mode while booting by pressing  after you hear a beep that indicates the activation of the keyboard.

When set to Disabled, BIOS is in the conventional text mode where you see the system initialization details on the screen.

## 2.2.10	Configuration Table

This parameter allows you to display the configuration table after POST but before booting.  The configuration table gives a summary of the hardware devices and settings that BIOS detected during POST.  A sample configuration table appears below.

| CPU ID : Pentium CPU Clock : 200 MHz Math Coprocessor: Installed IDE Drive 0 : xxx MB IDE Drive 1 : xxx MB IDE Drive 2 : xxx MB IDE Drive 3 : xxx MB Diskette Drive A: xx-MB, xx-inch Diskette Drive B: None Serial Port(s) : 3F8h, 2F8h Parallel Port : 378h | Base Memory: : xxx KB Extended Memory: xxxx KB Shadow RAM : xxx KB Internal Cache : xxx KB, Enabled External Cache : xxx KB, Enabled Pointing Device: None SIMM 1-2 Type : Fast Page Mode SIMM 3-4 Type : EDO SIMM 5-6 Type : None USB HC : Disabled |
| --- | --- |

# 2.3	Advanced System Configuration

The Advanced System Configuration option allows you to configure the advanced system memory functions.

|  | Do not change any settings in the Advanced Configuration if you are not a qualified technician to avoid damaging the system. |
| --- | --- |

The following screen shows page one the Advanced System Configuration parameters.

Advanced System Configuration Page 1/2

Internal Cache (CPU Cache) [Enabled ] External Cache [Enabled ] E0000h - FFFFFh (System BIOS) [Cacheable] C0000h - C7FFFh (Video BIOS) [Cacheable] USB Host Controller [Disabled ]

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

The following screen shows page two the Advanced System Configuration parameters.

Advanced System Configuration Page 2/2

Memory at 15MB - 16MB Reserved for [System ]Use DRAM Leadoff Timing (Read/Write/RAS##Pre) [7/6/4] RAS to CAS Delay [3 ] DRAM Write Burst Timing [X-3-3-3] DRAM Read Burst Timing (EDO/FPM) [X-2-2-2/X-3-3-3] ECC/Parity Mode Selection [Disabled]

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

## 2.3.1	Internal Cache (CPU Cache)

This parameter enables or disables the first-level cache memory.  The default setting is Enabled.

## 2.3.2	External Cache

This parameter enables or disables the second-level cache memory.  The default setting is Enabled.

## 2.3.3	E0000h - FFFFFh (System BIOS)

Set this parameter to Cacheable if you want the system BIOS to run directly from the cache memory, thus allowing your system to function faster.  Set this to Non-cacheable to run the system BIOS from RAM.  The default is Cacheable.

## 2.3.4	C0000h - C7FFFh (Video BIOS)

Set this parameter to Cacheable if you want the video BIOS to run directly from the cache memory, thus allowing your system to function faster.  Set this to Non-cacheable to run the video BIOS from RAM.  The default is Cacheable.

## 2.3.5	USB Host Controller

This parameter lets you enable or disable the USB controller.  The USB function allows your system to support additional peripherals.  The default setting is Disabled.

## 2.3.6	Memory at 15MB - 16MB

This parameter allows you to reserve a memory range for the use of either the system or an expansion board to prevent memory address conflicts.

Before setting this parameter, check your add-on card manual to determine if your add-on card needs this memory space.  If not, set this parameter to System Use .

## 2.3.7	DRAM Leadoff Timing (Read/Write/RAS#Pre)

Before data can be accessed, the core logic chip must issue the Memory Address signal, column address strobe (CAS) signal and row address strobe (RAS) signal to the DRAM.  However, these signals are not issued at the same time.  The period it takes for the core logic chip to charge the RAS signal before issuance is called the RAS precharge time.  The time difference between the issuance of RAS and CAS signals is called the leadoff time.

The leadoff time varies for read, write and RAS signal precharge actions.  Some DRAMs may even require a longer delay to access data.  This parameter allows you to set the leadoff time.   The available settings are 7/6/4, 7/6/3, 6/5/4, and 6/5/3.  The default setting is 7/6/4.

## 2.3.8	RAS to CAS Delay

This function allows you to set the wait state between the RAS and CAS signals.  The available settings are 3 and 2 clocks.  The default setting is 3 clocks.

However, if you install DRAMs with EDO (Extended Data Output) function, you must change the setting to 2 clocks.  This will give you a better system performance.

## 2.3.9	DRAM Write Burst Timing

This parameter adjusts the write wait-state between the L2 and the DRAM cache.  The L2 cache is processed through write-back method and each cache write process consists of four continuous cache write cycles.  Therefore, this setting has four numbers to adjust.

The available settings are X-4-4-4, X-3-3-3, and X-2-2-2.  The value of x depends on the DRAM Leadoff Timing parameter setting. Faster DRAMs require shorter wait states.  The default setting for this parameter is X-3-3-3.

## 2.3.10	DRAM Read Burst Timing (EDO/FPM)

This parameter adjusts the read wait-state between the L2 and the DRAM cache.  Similar to the DRAM Write Burst Timing, this parameter also has four numbers to adjust.  Everytime the CPU reads a L2 cache miss, it reads four continuous memory cycles on four continuous addresses from the DRAM cache.

The read burst timing varies for Extended Data Output (EDO) and Fast Page Mode (FPM) DRAMs.  The available settings are X-3-3-3/
X-4-4-4, X-2-2-2/X-3-3-3, and X-4-4-4/X-4-4-4.  The value of x depends on the DRAM Leadoff Timing parameter setting.  Faster DRAMs require shorter wait states.  The default setting for this parameter is X-2-2-2/X-3-3-3.

## 2.3.11	ECC/Parity Mode Selection

This parameter allows you to enable or disable the ECC and parity check feature.  The selections for this parameter are Parity, ECC and Disabled.  Take note that Fast-page mode SIMMs supports both ECC and parity mode while EDO SIMMs supports only ECC mode.  Select Disabled if you installed SIMMs without parity.

# 2.4	PCI System Configuration

The PCI System Configuration allows you to specify the settings for your PCI devices.

PCI System Configuration Page 1/1

PCI IRQ Setting [ Auto ] INTA INTB INTC INTD *PCI Slot 1 [--] [--] [--] [--] *PCI Slot 2 [--] [--] [--] [--] *PCI Slot 3 [--] [--] [--] [--] *PCI Slot 4 [--] [--] [--] [--] PCI IRQ Sharing [Yes] VGA Palette Snoop [Disabled] Plug & Play OS [Yes] Reset Resources Assignment [No]

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

## 2.4.1	PCI IRQ Setting

This parameter allows for Auto or Manual configuration of PCI devices.  If you use plug-and-play (PnP) devices, you can keep the default setting Auto.  The system then automatically configures the PnP devices.  If your PCI device is not a PnP, you can manually assign the interrupt for each of the device.

|  | Refer to your manual for technical information about the PCI card. |
| --- | --- |

### PCI Slots

These parameters allow you to specify the appropriate interrupt for each of the PCI devices.  You can assign IRQ5, IRQ9, IRQ10, IRQ11, or IRQ15 to the slots.

Press  or  to move between fields.  Press  or  to select options.

## 2.4.2	PCI IRQ Sharing

Setting this parameter to Yes allows you to assign the same IRQ to two different devices.  To disable the feature, select No.

|  | Enable this parameter only if there are no IRQs available to assign for the remaining device function. |
| --- | --- |

## 2.4.3	VGA Palette Snoop

This parameter permits you to use the palette snooping feature if you installed more than one VGA card in the system.

The VGA palette snoop function allows the control palette register (CPR) to manage and update the VGA RAM DAC (Digital Analog Converter, a color data storage) of each VGA card installed in the system.  The snooping process lets the CPR send a signal to all the VGA cards so that they can update their individual RAM DACs.  The signal goes through the cards continuously until all RAM DAC data have been updated.  This allows display of multiple images on the screen.

|  | Some VGA cards have required settings for this feature. Check your VGA card manual before setting this parameter. |
| --- | --- |

## 2.4.4	Plug & Play OS

When this parameter is set to Yes, BIOS initializes only PnP boot devices such as SCSI cards.  When set to No, BIOS initializes all PnP boot and non-boot devices such as sound cards.

|  | Set this parameter to Yes only if your operating system is Windows 95. |
| --- | --- |

## 2.4.5	Reset Resources Assignment

Set this parameter to Yes to avoid IRQ conflict when installing non-PnP or PnP ISA cards.  This clears all resource assignments and allows BIOS to reassign resources to all installed PnP devices the next time the system boots.  After clearing the resource data, the parameter resets to No.

Refer to section 1.8 for instructions on installing and configuring ISA cards.

# 2.5	Power Saving Configuration

The Power Saving Configuration parameters are configurable only if your system supports the power management feature.

The following screen shows the Power Saving Configuration parameters and their default settings:

Power Saving Configuration Page 1/1

Power Management Mode [Enabled ] IDE Hard Disk Standby Timer [15] Minute(s) System Standby Timer [15] Minute(s) System Suspend Timer [15] Minute(s) Monitored Activities IRQ 3 [Disabled] IRQ 9 [Disabled] IRQ 4 [Disabled] IRQ 10 [Disabled] IRQ 5 [Disabled] IRQ 11 [Disabled] IRQ 6 [Enabled ] IRQ 12 [Enabled ] IRQ 7 [Disabled] IRQ 14 [Enabled ] IRQ 8 [Disabled] IRQ 15 [Disabled]

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

## 2.5.1	Power Management Mode

This parameter allows you to reduce power consumption.  When this parameter is set to Enabled, you can configure the IDE hard disk and system timers.  Setting to Disabled deactivates the power management feature and all the timers.

### IDE Hard Disk Standby Timer

This parameter allows the hard disk to enter standby mode after inactivity of 1 to 15 minutes, depending on your setting.  When you access the hard disk again, allow 3 to 5 seconds (depending on the hard disk) for the disk to return to normal speed.  Set this parameter to Off if your hard disk does not support this function.

### System Standby Timer

This parameter sets the system to a "fast-on" power saving mode.  It automatically enters into the standby mode after a specified period of inactivity.  Any keyboard or mouse action, or any enabled monitored activities occurring through the IRQ channels resume system operation.  See section 2.5.2.

### System Suspend Timer

This parameter sets the system to the lowest power saving mode.  It automatically enters into the suspend mode after a specified period of inactivity.

In suspend mode, the CPU clock stops.  Any keyboard or mouse action, or any enabled monitored activities occurring through the IRQ channels resume system operation.  See section 2.5.2.

## 2.5.2	Monitored Activities

The IRQ items under this parameter allow you to monitor system activities occurring through the IRQ channels to determine whether or not to enter power saving mode.

For example, if you assign IRQ 3 to a fax/modem and you set this item to Enabled, any fax/modem activity wakes up the system from standby mode.

# 2.6	System Security Setup

The Setup program has a number of security features to prevent unauthorized access to the system and its data.

The following screen appears if you enter the Setup program and select System Security.

System Security Page 1/1

Disk Drive Control Diskette Drive [ Normal ] Hard Disk Drive [ Normal ] System Boot Drive [Drive A then C] Boot from CD-ROM [Disabled] Onboard Communication Ports Serial Port 1 Base Address [ 3F8h ] Serial Port 2 Base Address [ 2F8h ] Parallel Port Base Address [378 (IRQ 7)] Operation Mode [Standard Parallel Port (SPP)] Mode ECP DMA Channel [-] Onboard PS/2 Mouse (IRQ 12) [Enabled ] Setup Password [ None ] Power On Password [ None ]

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

## 2.6.1	Disk Drive Control

The disk drive control features allow you to enable or disable the read/write functions of a disk drive.  These features can also control the diskette drive or the hard disk drive boot function to prevent loading operating systems or other programs from a certain drive while the other drives are operational.

Table 2-1 lists the drive control settings and their corresponding functions.

Table 2-1	Drive Control Settings

| Diskette Drive |  |
| --- | --- |
| Setting | Description |
| Normal | Diskette drive functions normally |
| Write Protect All Sectors | Disables the write function on all sectors |
| Write Protect Boot Sector | Disables the write function only on the boot sector |
| Disabled | Disables all diskette functions |
| Hard Disk Drive |  |
| Setting | Description |
| Normal | Hard disk drive functions normally |
| Write Protect All Sectors | Disables the write function on all sectors |
| Write Protect Boot Sector | Disables the write function only on the boot sector |
| Disabled | Disables all hard disk functions |
| System Boot Drive |  |
| Setting | Description |
| Drive A then C | The system checks drive A first. If there is a diskette in the drive, the system boots from drive A. Otherwise, it boots from drive C. |
| Drive C then A | The system checks drive C first. If there is a hard disk (drive C) installed, the system boots from drive C. Otherwise, it boots from drive A. |
| C: | The system always boots from drive C. |
| A: | The system always boots from drive A. |

Table 2-1	Drive Control Settings  (continued)

| Boot from CD-ROM |  |
| --- | --- |
| Setting | Description |
| Enabled | The system checks for a bootable CD in the CD-ROM. If a CD is present, the system boots from the CD-ROM. Otherwise, it boots from the drive specified in the System Boot drive parameter. |
| Disabled | System boots from the drive specified in the System Boot Drive parameter. |

## 2.6.2	Onboard Communication Ports

### Serial Port 1 Base Address

This parameter allows you to set the serial port 1 logical base address.

Table 2-2	Serial Port 1 Settings

| Setting | Description |
| --- | --- |
| 3F8h | Serial port 1 with address 3F8h using IRQ4 |
| 2F8h | Serial port 1 with address 2F8h using IRQ3 |
| 3E8h | Serial port 1 with address 3E8h using IRQ4 |
| 2E8h | Serial port 1 with address 2E8h using IRQ3 |
| Disabled | Disables serial port 1 |

### Serial Port 2 Base Address

This parameter allows you to set the serial port 2 logical base address.

Table 2-3	Serial Port 2 Settings

| Setting | Description |
| --- | --- |
| 3F8h | Serial port 2 with address 3F8h using IRQ4 |
| 2F8h | Serial port 2 with address 2F8h using IRQ3 |
| 3E8h | Serial port 2 with address 3E8h using IRQ4 |
| 2E8h | Serial port 2 with address 2E8h using IRQ3 |
| Disabled | Disables serial port 2 |

|  | If you assign 3F8h to serial port 1, you may only assign 2F8h or 2E8h to serial port 2. If you assign 2F8h to serial port 1, you may only assign 3F8h or 3E8h to serial port 2. |
| --- | --- |

### Parallel Port Base Address

The system has one parallel port.  Table 2-4 lists the options for selecting the parallel port address.  You also have the option to disable the parallel port.

Table 2-4	Parallel Port Settings

| Setting | Function |
| --- | --- |
| 3BCh (IRQ 7) | Corresponds to the parallel port with address 3BCh |
| 378h (IRQ 7) | Corresponds to the parallel port with address 378h |
| 278h (IRQ 5) | Corresponds to the parallel port with address 278h |
| Disabled | Disables the parallel port |

To deactivate the parallel port, select the Disabled option.  If you install an add-on card that has a parallel port whose address conflicts with the parallel port onboard, the system automatically disables the onboard functions.

Check the parallel port address on the add-on card and change the address to one that does not conflict.

#### Operation Mode

This item allows you to set the operation mode of the parallel port.  Table 2-5 lists the different operation modes.

Table 2-5	Parallel Port Operation Mode Settings

| Setting | Function |
| --- | --- |
| Standard Parallel Port (SPP) | Allows normal speed one-way operation |
| Standard and Bidirectional | Allows normal speed operation in a two-way mode |
| Enhanced Parallel Port (EPP) | Allows bidirectional parallel port operation at maximum speed |
| Extended Capabilities Port (ECP) | Allows parallel port to operate in bidirectional mode and at a speed higher than the maximum data transfer rate |

#### ecp dma channel

This item becomes active only if you select Extended Capabilities Port (ECP) as the operation mode.  It allows you to assign DMA channel 1 or DMA channel 3 for the ECP parallel port function (as required in Windows 95).

## 2.6.3	Onboard PS/2 Mouse (IRQ 12)

This parameter enables or disables the onboard PS/2 mouse.  When set to Enabled, it allows you to use the onboard PS/2 mouse assigned with IRQ12.  When set to Disabled, it deactivates the mouse and makes IRQ12 available for use of other devices.

## 2.6.4	Setup Password

The Setup Password prevents unauthorized access to the BIOS utility.

### Setting a Password

1.	Make sure that JP1 is set to pins 2-3 (bypass password).

|  | You cannot enter the BIOS utility if a Setup password does not exist and JP1 is set to pins 1-2 (password check enabled). By default, JP1 is set to pins 2-3 (bypass password). |
| --- | --- |

2.	Enter BIOS utility and select System Security.

3.	Highlight the Setup Password parameter and press the  or  key.  The password prompt appears:

4.	Type a password.  The password may consist of up to seven characters.

|  | Be very careful when typing your password because the characters do not appear on the screen. |
| --- | --- |

5.	Press .  A prompt asks you to retype the password to verify your first entry.

6.	Retype the password then press .

After setting the password, the system automatically sets the Setup Password parameter to Present.

7.	Press  to exit the System Security screen and return to the main menu.

8.	Press  to exit the BIOS utility.  A dialog box appears asking if you want to save the CMOS data.

9.	Select Yes to save the changes and reboot the system.

10.	While rebooting, turn off the system then open the housing.

11.	Set JP1 to pins 1-2 to enable the password function.

The next time you want to enter the BIOS utility, you must key-in your Setup password.

### Changing or Removing the Setup Password

Should you want to change your setup password, do the following:

1.	Enter the BIOS utility and select System Security.

2.	Highlight the Setup Password parameter.

3.	Press  or  to display the password prompt and key-in a new password.

or

Press  or  and select None to remove the existing password.

4.	Press  to exit the System Security screen and return to the main menu.

5.	Press  to exit the BIOS utility.  A dialog box appears asking if you want to save the CMOS data.

6.	Select Yes to save the changes.

### Bypassing the Setup Password

If you forget your setup password, you can bypass the password security feature by hardware.  Follow these steps to bypass the password:

1.	Turn off and unplug the system.

2.	Open the system housing and short pins 2-3 of JP1 to bypass the password function.

3.	Turn on the system and enter the BIOS utility.  This time, the system does not require you to type in a password.

|  | You can either change the existing Setup password or remove it by selecting None. Refer to the previous section for the procedure. |
| --- | --- |

## 2.6.5	Power On Password

The Power On Password secures your system against unauthorized use.  Once you set this password, you have to type it whenever you boot the system.  To set this password, enter the BIOS utility, select System Security, then highlight the Power On Password parameter.  Follow the same procedure as in setting the Setup password.

|  | Make sure to set JP1 to 1-2 to enable the Power On password. |
| --- | --- |

# 2.7	Load Setup Default Settings

Use this option to load the default settings for the optimized system configuration.  When you load the default settings, some of the parameters are grayed-out with their fixed settings.  These grayed parameters are not user-configurable.

The following dialog box appears when you select Load Setup Default Settings from the main menu.

Load Setup Default Settings
  Are you sure

	[Yes]	[No]

Select Yes to load the default settings.

# 2.8	Leaving Setup

Examine the system configuration values.  When you are satisfied that all the values are correct, write them down.  Store the recorded values in a safe place.  In the future, if the battery loses power or the CMOS chip is damaged, you will know what values to enter when you rerun Setup.

Press  to leave the system configuration setup.  If there is any change in the BIOS utility functions, the following screen appears:

Do you want to save CMOS data?

	[Yes]	[No]

Use the arrow keys to select your response.  Select Yes to store the new data in CMOS.  Select No to retain the old configuration values. Press  .