Most systems are already configured by the manufacturer or the dealer.  There is no need to run Setup when starting the computer unless you get a Run Setup message.

The Setup program loads configuration values into the battery-backed nonvolatile memory called CMOS RAM.  This memory area is not part of the system RAM.

|  | If you repeatedly receive Run Setup messages, the battery may be bad. In this case, the system cannot retain configuration values in CMOS. Ask a qualified technician for assistance. |
| --- | --- |

Before running Setup, have the following information ready:

·	Floppy drive type  xe "diskette drive types"§The standard type is either a 5.25-inch 1.2-MB or a 3.5-inch 1.44-MB floppy drive.

·	IDE hard disk drive type  The drive information is on the label pasted on your IDE drive or in the documentation supplied by the vendor.

# 2.1	Entering Setupxe "Enter setup"§

To enter Setup, press the key combination  + + .

|  | You must press + + while the system is booting. This key combination does not work during any other time. |
| --- | --- |

The BIOS Utility Main menu xe "BIOS main menu"§then appears:

| BIOS Utility |  |  |  |
| --- | --- | --- | --- |
| System Information Basic Configuration Advanced Configuration System Security Power Management Exit Setup Utility |  |  |  |
| ¯¬® = Move highlight bar, ¿ = Select, Esc = Exit |  |  |  |
|  |  | The parameters on the screens show default values. These values may not be the same as those in your system. The grayed items on the screens have fixed settings and are not user-configurable. |  |

# 2.2	System Information

The following screen appears if you select the System Information from the Main menu.

System Information Page 1/2

Processor Pentium(R) Processor Processor Speed 166 MHz L1 Cache Size 16 KB L2 Cache Size 512 KB Floppy Drive A 1.44 MB, 3.5-inch Floppy Drive B None IDE Primary Channel Master Hard Disk, 519 MB IDE Primary Channel Slave None IDE Secondary Channel Master None IDE Secondary Channel Slave CD-ROM Total Memory xxx MB DIMM 1 SDRAM DIMM 2 None DIMM 3 None

PgDn/PgUp = Move Screen, Esc = Back to Main Menu

The System Information menu shows the current basic configuration of your system.

The command line at the bottom xe "BIOS bottom command lines"§of the menu tells you how to move from one screen to another and return to the Main menu.

Press   to move to the next page or   to return to the previous page.

Press   to return to the Main menu.

The following screen shows page 2 of the System Information menu.

System Information Page 2/2

Serial Port 1 3F8h, IRQ 4 Serial Port 2 2F8h, IRQ 3 Parallel Port 378h, IRQ 7 PS/2 Mouse Installed System BIOS Version V.x.x System BIOS ID xx-xx xx xx INTA INTB INTC INTD PCI Slot 1 [--] [--] [--] [--] PCI Slot 2 [--] [--] [--] [--] PCI Slot 3 [--] [--] [--] [--]

PgDn/PgUp = Move Screen, Esc = Back to Main Menu

The following sections explain the parameters.

## 2.2.1	Processor

The Processor parameter specifies the type of processor currently installed in your system. The system can support Intel Pentium P54C/P55C processors.

## 2.2.2	Processor Speed

The Processor Speed parameter specifies the speed of the CPU currently installed in your system.  The system can support CPUs running from 75 MHz to 233 MHz.

## 2.2.3	L1 Cache Size

This parameter specifies the first-level or the internal memory (i.e., the memory integrated into the CPU) size.

## 2.2.4	L2 Cache Size

This parameter specifies the external cache memory size currently supported by the system.

## 2.2.5	Floppy Drive A

This parameter specifies the system’s current floppy drive A settings.  For information on how to configure the floppy drives, see section 2.3.2.

## 2.2.6	Floppy Drive B

This parameter specifies the system’s current floppy drive B settings. For information on how to configure the floppy drives, see section 2.3.2.

## 2.2.7	IDE Primary Channel Master

This parameter specifies the current configuration of the IDE device connected to the master port of the primary IDE channel. For information on how to configure the IDE devices drives, see section 2.3.3.

## 2.2.8	IDE Primary Channel Slave

This parameter specifies the current configuration of the IDE device connected to the slave port of the primary IDE channel. For information on how to configure the IDE devices drives, see section 2.3.3.

## 2.2.9	IDE Secondary Channel Master

This parameter specifies the current configuration of the IDE device connected to the master port of the secondary IDE channel. For information on how to configure the IDE devices drives, see section 2.3.3.

## 2.2.10	IDE Secondary Channel Slave

This parameter specifies the current configuration of the IDE device connected to the slave port of the secondary IDE channel. For information on how to configure the IDE devices drives, see section 2.3.3.

## 2.2.11	Total Memory

This parameter specifies the total amount xe "system total memory"§of onboard memory.   The memory size is automatically detected by BIOS during the POST.  If you install additional memory, the system automatically adjusts this parameter to display the new memory size.

### DIMM 1

This parameter indicates the type of DRAM installed in DIMM 1.  The None setting indicates that there is no DRAM installed.  For the location of DIMM 1, refer to Figure 1-1.

### DIMM 2

This parameter indicates the type of DRAM installed in DIMM 2.  The None setting indicates that there is no DRAM installed.  For the location of DIMM 2, refer to Figure 1-1.

### DIMM 3

This parameter indicates the type of DRAM installed in DIMM 3.  The None setting indicates that there is no DRAM installed.  For the location of DIMM 3, refer to Figure 1-1.

## 2.2.12	Serial Port 1

This parameter shows the serial port 1 address and IRQ settings.

## 2.2.13	Serial Port 2

This parameter shows the serial port 2 address and IRQ settings.

## 2.2.14	Parallel Port

This parameter shows the parallel port address and IRQ settings.

## 2.2.15	PS/2 Mouse

The BIOS utility automatically detects if there is a mouse connected to your system.  If there is, this parameter displays the Installed setting.  Otherwise, this is set to None.

## 2.2.16	System BIOS Version

This parameter specifies the version of the BIOS.

## 2.2.17	System BIOS ID

This parameter specifies the identification number of the BIOS.

## 2.2.18	PCI Slot 1/2/3

These parameters specify the auto-assigned interrupt for each of the PCI devices.

# 2.3	Basic System Configuration

Select Basic System Configuration xe "Basic System Configuration"§to input configuration values such as date, time, and disk types.

The following screen shows the Basic System Configuration menu.

Basic Configuration Page 1/1

Date [MM/DD/YY] Time [HH:MM:SS] Floppy Drive A [xx-MB xx-inch] Floppy Drive B [xx-MB xx-inch] ØIDE Primary Channel Master ØIDE Primary Channel Slave ØIDE Secondary Channel Master ØIDE Secondary Channel Slave ØBoot Options

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

## 2.3.1	Date and Time

The real-time clock keeps the system date and timexe "RTC:Date"§.  After setting the date and time, you do not need to enter them every time you turn on the system.  As long as the internal battery remains good (approximately seven years) and connected, the clock continues to keep the date and time accurately even when the power is off.

Date

Highlight the items on the Date parameter and press   or   to set the date following the month-day-year format.

Valid values for month, day, and year are:

·	Month	1  to 12

·	Day	1  to 31

·	Year	00  to 99

Time

Highlight the items on the Time xe "RTC:Time"§parameter and press   or   to set the time following the hour-minute-second format.

Valid values for hour, minute, and second are:

·	Hour	00 to 23

·	Minute	00 to 59

·	Second	00 to 59

## 2.3.2	Floppy Drives

To enter the configuration value for the first floppy drive (drive A), highlight the Floppy Drive A parameter.  Press   or   key to view the options and select the appropriate value.

Possible settings for the Floppy Drive xe "diskette drive configuration"§parameters:

·	[        None        ]

·	[360  KB,   5.25-inch]

·	[1.2  MB,   5.25-inch]

·	[720  KB,    3.5-inch]

·	[1.44 MB,    3.5-inch]

Follow the same procedure for Floppy Drive B.  Choose None if you do not have a second floppy drive.

## 2.3.3	IDE Drives

To configure the IDE drives connected to your system, select the parameter that represents the channel and port where the desired hard disk to configure is connected.   The options are:

### IDE Primary Channel Master

This parameter lets you configure the hard disk drive connected to the master port of IDE channel 1.

### IDE Primary Channel Slave

This parameter lets you configure the hard disk drive connected to the slave port of IDE channel 1.

### IDE Secondary Channel Master

This parameter lets you configure the hard disk drive connected to the master port of IDE channel 2.

### IDE Secondary Channel Slave

This parameter lets you configure the hard disk drive connected to the slave port of IDE channel 2.

The following screen appears if you select any of the IDE Drive parameters:

IDE Primary Channel Master Page 1/1

Type [ Auto ] Cylinder [ ] Head [ ] Sector [ ] Size [ ] MB Hard Disk Block Mode [ Auto ] Advanced PIO Mode [ Auto ] Hard Disk Size > 504MB [ Auto ] Hard Disk 32 Bit Access [Enabled ] CD-ROM Drive DMA Mode [Disabled ]

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

#### Type

This parameter lets you specify the type of hard disk installed in your system.  If you want BIOS to automatically configure your hard disk, select Auto.   If you know your hard disk type, you can enter the setting manually.

Setting this parameter also sets the Cylinder, Head, Sector, and Size parameter.

#### CylinderS

This parameter specifies the number of cylinders of your hard disk, and is automatically set depending on your Type parameter setting.

#### head

This parameter specifies the number of heads of your hard disk, and is automatically set depending on your Type parameter setting.

#### SECTors

This parameter specifies the number of sectors of your hard disk, and is automatically set depending on your Type parameter setting.

#### Size

This parameter specifies the size of your hard disk, in MB.  This is automatically set depending on your Type parameter setting.

### Enhanced IDE Features

#### Hard Disk Block Mode

This function xe "Hard Disk Block Mode"§enhances disk performance depending on the hard disk in use.  If you set this parameter to Auto, the BIOS utility automatically detects if the installed hard disk drive supports the Block Mode function.  If supported, it allows data transfer in block (multiple sectors) at a rate of 256 bytes per cycle.  To disregard the feature, change the setting to Disabled.

#### Advanced PIO Mode

When set to Auto, the BIOS utility automatically detects if the installed hard disk supports the function.  If supported, it xe "Advanced PIO Mode"§ allows for faster data recovery and read/write timing that reduces hard disk activity time.  This results to better hard disk performance.  To disregard the feature, change the setting to Disabled.

#### Hard Disk Size > 504 MB

When set to Auto, the BIOS utility automatically detects if the installed hard disk supports the function.  If supported, it allows you to use a hard disk with a capacity of more than 504 MB.  This is made possible through the Logical Block Address (LBA) mode translation.  However, enhanced IDE feature xe "hdd size>504MB"§ works only under DOS and Windows 3.x, 95  environment.  Other operating systems require this parameter to be set to Disabled.

#### Hard Disk 32-bit Access

Enabling this parameterxe "hard disk 32-bit access"§  improves system performance by allowing the use of the 32-bit hard disk access.  This enhanced IDE feature works only under DOS, Windows 3.x, 95, and Novell NetWare.  If your software or hard disk does not support this function, set this parameter to Disabled .

#### CD-ROM Drive DMA Mode

Set this parameter to Enabled to enable the DMA mode for the CD-ROM drive.  This improves the system performance since it allows direct memory access to the CD-ROM.  To deactivate the function, set the parameter to Disabled .

## 2.3.4	Boot Options

This option allows you to specify your preferred setting for bootup.

The following screen appears if you select Boot Options from the Basic Configuration menu:

Boot Options Page 1/1

Fast Boot [Auto ] Silent Boot [Enabled ] Num Lock After Boot [Enabled ] Memory Test [Disabled] System Boot Drive [Drive A Then C] Boot From CD-ROM [Disabled]

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

### Fast Boot

This parameter allows the system to boot faster by skipping some POST routines.  The default setting is Auto.

### Silent Boot

This parameter enables or disables the Silent Boot function.  When set to Enabled, BIOS is in graphical mode and displays only an identification logo during POST and while booting.  After which the screen displays the operating system prompt (such as DOS) or logo (such as Windows 95).  If any error occurred while booting, the system automatically switches to the text mode.

Even if your setting is Enabled, you may also switch to the text mode while booting by pressing   after you hear a beep that indicates the activation of the keyboard.

When set to Disabled, BIOS is in the conventional text mode where you see the system initialization details on the screen.

### Num Lock After Boot

This parameter allows you to activate the Num Lock function upon booting.  The default setting is Enabled.

### Memory Test

When set to Enabled, this parameter allows the system to perform a RAM test during the POST routine.  When set to Disabled, the system detects only the memory size and bypasses the test routine.  The default setting is Disabled.

### System Boot Drive

This parameter allows you to specify the system search sequence.  The selections are:

·	Drive A then C:  The system checks drive A first.  If there is a diskette in the drive, the system boots from drive A.  Otherwise, it boots from drive C.

·	Drive C then A:  The system checks drive C first.  If there is a hard disk (drive C) installed, the system  boots from drive C.  Otherwise, it boots from drive A.

·	C:  The system always boots from drive C.

·	A:  The system always boots from drive A.

### Boot from CD-ROM

When set to Enabled, the system checks for a bootable CD in the CD-ROM drive.  If a CD is present, the system boots from the CD-ROM; otherwise, it boots from the drive specified in the System Boot Drive parameter.

When set to Disabled, the system boots from the drive specified in the System Boot Drive parameter.

# 2.4	Advanced Configuration

The Advanced Configuration xe "Advanced System Configuration"§option allows you to configure the advanced system memory functions.

|  | Do not change any settings in the Advanced Configuration if you are not a qualified technician to avoid damaging the system. |
| --- | --- |

The following screen shows the Advanced Configuration parameters.

Advanced Configuration Page 1/1

Memory at 15MB-16MB Reserved for [System] PCI IRQ Sharing [Yes] VGA Palette Snoop [Disabled] Plug and Play OS [Yes] ØOnboard Peripheral Configuration

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

## 2.4.1	Memory at 15MB-16MB Reserved For

To prevent memory address xe "Memory at 15MB-16MB Reserved For"§conflicts between the system and expansion boards, reserve this memory range for the use of either the system or an expansion board.

## 2.4.2	PCI IRQ Sharing

Setting this parameter xe "IRQ sharing"§ to Yes allows you to assign the same IRQ to two different devices.  To disable the feature, select No.

|  | If there are no IRQs available to assign for the remaining device function, we recommend that you enable this parameter. |
| --- | --- |

## 2.4.3	VGA Palette Snoop

This parameter permits you to use the palette snooping feature if you installed more than one VGA card in the system.

The VGA palette snoop xe "VGA palette snooping"§function allows the control palette register (CPR) to manage and update the VGA RAM DAC (Digital Analog Converter, a color data storage) of each VGA card installed in the system.  The snooping process lets the CPR send a signal to all the VGA cards so that they can update their individual RAM DACs.  The signal goes through the cards continuously until all RAM DAC data have been updated.  This allows display of multiple images on the screen.

|  | Some VGA cards have required settings for this feature. Check your VGA card manual before setting this parameter. |
| --- | --- |

## 2.4.4	Plug and Play OS

When this parameter is set to Yes, BIOS initializes only PnP boot devices such as SCSI cards.  When set to No, BIOS initializes all PnP boot and non-boot devices such as sound cards.

|  | Set this parameter to Yes only if your operating system is Windows 95. |
| --- | --- |

## 2.4.5	Onboard Peripheral Configuration

The Onboard Peripheral Configuration allows you to configure the onboard communication ports and the onboard devices.  Selecting this option from the Advanced menu displays the following screen:

Onboard Devices Configuration Page 1/1

Serial Port 1 [Enabled ] Base Address [378h] IRQ [ 4 ] Serial Port 2 [Disabled] Base Address [----] IRQ [----] Parallel Port [Enabled ] Base Address [378h] IRQ [ 7 ] Operation Mode [Bidirectional] ECP DMA Channel [ - ] Floppy Disk Controller [Enabled] IDE Controller [Both] PS/2 Mouse Controller [Enabled] USB Host Controller [Enabled] USB Legacy Mode [Disabled]

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

### Serial Port 1

This parameter allows you to enable or disable the serial port 1.

#### Base Address

This function lets you set a logical base address for serial port 1.  The options are:

·	3F8h

·	2F8h

·	3E8h

·	2E8h

#### IRQ

This function lets you assign an interrupt for serial port 1.  The options are IRQ 4 and 11.

|  | The Base Address and IRQ parameters are configurable only if Serial Port 1 is enabled. |
| --- | --- |

### Serial Port 2

This parameter allows you to enable or disable the serial port 2.

#### Base Address

This function lets you set a logical base address for serial port 2.  The options are:

·	3F8h

·	2F8h

·	3E8h

·	2E8h

#### IRQ

This function lets you assign an interrupt for serial port 2.  The options are IRQ 3 and 10.

|  | The Base Address and IRQ parameters are configurable only if Serial Port 2 is enabled. |
| --- | --- |

### Parallel Port

This parameter allows you to enable or disable the parallel port.

#### Base Address

This function lets you set a logical base address for the parallel port.  The options are:

·	3BCh

·	378h

·	278h

#### IRQ

This function lets you assign an interrupt for the parallel port.  The options are IRQ 5 and 7.

|  | The Base Address and IRQ parameters are configurable only if Parallel Port is enabled. |
| --- | --- |

#### Operation Mode

This item allows you to set the operation mode of the parallel port.  Table 2-1 lists the different operation modes.

Table 2-1	Parallel Port Operation Mode Settingsxe "parallel port operation modes"§

| Setting | Function |
| --- | --- |
| Standard Parallel Port (SPP) | Allows normal speed one-way operation |
| Standard and Bidirectional | Allows normal speed operation in a two-way mode |
| Enhanced Parallel Port (EPP) | Allows bidirectional parallel port operation at maximum speed |
| Extended Capabilities Port (ECP) | Allows parallel port to operate in bidirectional mode and at a speed higher than the maximum data transfer rate |

#### ecp dma channel

This item becomes active only if you select Extended Capabilities Port (ECP) as the operation mode.  It allows you to assign DMA channel 1 or DMA channel 3 for the ECP parallel port function (as required in Windows 95).

### Floppy Disk Controller

This parameter lets you enable or disable the onboard floppy disk controller.

### Onboard IDE Controller

This parameter lets you enable or disable the IDE controller on board.

### PS/2 Mouse Controller

This parameter enables or disables the onboard PS/2 mousexe "IRQ12"§.  When enabled, it allows you to use the onboard PS/2 mouse assigned with IRQ12.  When disabled, it deactivates the mouse and makes IRQ12 available for use of other devices.

### USB Host Controller

This parameter lets you enable or disable the USB controller on board. When enabled, it activates the USB function of the system.  When disabled, it also deactivates the function.

#### USB Legacy Mode

This function, when enabled, lets you use a USB keyboard in DOS environment.  Set this to Disabled to deactivate the USB keyboard function in DOS environment.

# 2.5	System Security Setup

The Setup program has a number of security features to prevent unauthorized access to the system and its data.

The following screen appears if you select System Security from the Main menu:xe "System Security Setup"§

System Security Page 1/1

Disk Drive Control Floppy Drive [ Normal ] Hard Disk Drive [ Normal ] Setup Password [ None ] Power On Password [ None ] Operation Mode [ Normal ]

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

## 2.5.1	Disk Drive Control

The disk drive control features allow you to control the floppy drive or the hard disk drive boot function to prevent loading operating systems or other programs from a certain drive while the other drives are operational.

Table 2-2 lists the drive control settingsxe "drive control settings"§ and their corresponding functions.

Table 2-2	Drive Control Settings

| Floppy Drive |  |
| --- | --- |
| Setting | Description |
| Normal | Floppy drive functions normally |
| Write Protect All Sectors | Disables the write function on all sectors |
| Write Protect Boot Sector | Disables the write function only on the boot sector |
| Hard Disk Drive |  |
| Setting | Description |
| Normal | Hard disk drive functions normally |
| Write Protect All Sectors | Disables the write function on all sectors |
| Write Protect Boot Sector | Disables the write function only on the boot sector |

## 2.5.2	Setup Password

The Setup Password prevents unauthorized access to the BIOS utility.

### Setting a Passwordxe "Setup password:setting"§

1.	Make sure that switch 6 of SW1 is set to Off (bypass password).

|  | You cannot enter the BIOS utility if a Setup password does not exist and switch 6 of SW1 is set to On (password check enabled). By default, switch 6 of SW1 is set to Off (bypass password). |
| --- | --- |

2.	Enter BIOS utility and select System Security.

3.	Highlight the Setup Password parameter and press the   or   key.  The password prompt appears:

4.	Type a password.  The password may consist of up to seven characters.

|  | Be very careful when typing your password because the characters do not appear on the screen. |
| --- | --- |

5.	Press  .  A prompt asks you to retype the password to verify your first entry.

6.	Retype the password then press  .

After setting the password, the system automatically sets the Setup Password parameter to Present.

7.	Press   to exit the System Security screen and return to the Main menu.

8.	Press   to exit the BIOS utility.  A dialog box appears asking if you want to save the CMOS data.

9.	Select Yes to save the changes and reboot the system.

10.	While rebooting, turn off the system then open the housing.

11.	Set switch 6 of SW1 to On to enable the password function.

The next time you want to enter the BIOS utility, you must key-in your Setup password.

### Changing or Removing the Setup Passwordxe "Setup password:removing"§xe "Setup password:changing"§

Should you want to change your setup password, do the following:

1.	Enter the BIOS utility and select System Security.

2.	Highlight the Setup Password parameter.

3.	Press   or   to display the password prompt and key-in a new password.

or

Press   or   and select None to remove the existing password.

4.	Press   to exit the System Security screen and return to the Main menu.

5.	Press   to exit the BIOS utility.  A dialog box appears asking if you want to save the CMOS data.

6.	Select Yes to save the changes.

### Bypassing the Setup Passwordxe "Setup password:bypassing"§

If you forget your setup password, you can bypass the password security feature by hardware.  Follow these steps to bypass the password:

1.	Turn off and unplug the system.

2.	Open the system housing and switch 6 of SW1 is set to Off to bypass the password function.

3.	Turn on the system and enter the BIOS utility.  This time, the system does not require you to type in a password.

|  | You can either change the existing Setup password or remove it by selecting None. Refer to the previous section for the procedure. |
| --- | --- |

## 2.5.3	Power-on Password

The Power-on Password xe "Power On password"§secures your system against unauthorized use.  Once you set this password, you have to type it whenever you boot the system.  To set this password, enter the BIOS utility, select System Security, then highlight the Power-on Password parameter.  Follow the same procedure as in setting the Setup password.

|  | Make sure to set switch 6 of SW1 to On to enable the Power-on password. |
| --- | --- |

### Operation Mode

This function lets you enable or disable the password prompt display.  When set to Normal, the password prompt appears before system boot.  When set to Keyboard Lock, the password prompt do not appear; however, the keyboard will be locked after system boot and will remain locked until the correct password is entered.

# 2.6	Power Management

The Power Management menu xe "Power Management "§ lets you configure the system power-management feature.

The following screen shows the Power Management parameters and their default settings:

Power Management Page 1/1

Power Management Mode [Enabled ] IDE Hard Disk Standby Timer [OFF ] System Sleep Timer [OFF ] Stop CPU Clock in Sleep State [Yes] Power Switch < 4 sec. . [Suspend] Schedule Resume from Suspend [Disabled] Resume Time [--:--:--]

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

## 2.6.1	Power Management Mode

This parameter allows you to reduce power consumption.  When this parameter is set to Enabled, you can configure the IDE hard disk and system timers.  Setting to Disabled deactivates the power-management feature and all the timers.

### IDE Hard Disk Standby Timer

This parameter allows the hard disk to enter standby xe "power management modes"§xe "power management modes:IDE hdd standby mode"§mode after inactivity of 1 to 15 minutes, depending on your setting.  When you access the hard disk again, allow 3 to 5 seconds (depending on the hard disk) for the disk to return to normal speed.  Set this parameter to OFF if your hard disk does not support this function.

### System Sleep Timer

This parameter sets the system to the lowest power-saving xe "power management modes:suspend mode"§mode.  It automatically enters into the sleep or the suspend mode after a specified period of inactivity.  Any keyboard or mouse action detected resume system operation.

#### Stop CPU Clock in Sleep State

If you want to stop the CPU clock when the system enters the sleep or suspend mode, set this parameter to Yes.  If not, then select No.

## 2.6.2	Power Switch < 4 sec.

When set to Power-off, the system automatically turns off when the power switch is pressed for less than 4 seconds.  When set to Suspend, the system enters the suspend mode.

## 2.6.3	Schedule Resume from Suspend

This option lets you enable or disable the automatic system resume function.  This function allows you to specify the time when to resume the system from suspend mode.  You can specify the time in the Resume Time parameter.

### Resume Time

This parameter lets you specify the time when to resume the system from suspend mode to normal mode.  The time setting is in hour-minute-second format.

This parameter is configurable only when the Schedule Resume function is enabled.

# 2.7	Exit Setup Utility

To exit the BIOS utility, select Exit Setup Utility from the Main menu.  The following screen appears:

Exit Setup Utility Page 1/1

Save CMOS Settings and Exit Load Previous Settings and Exit Load Default Settings Save CMOS Settings Load Previous Settings

¯¬® = Move highlight bar, ¿ = Select,Esc = Back to Main Menu

## 2.7.1	Save CMOS Settings and Exit

Select this option if you want to save the current CMOS settings and exit the BIOS utility.

## 2.7.2	Load Previous Settings and Exit

Select this option to cancel the current changes made to the BIOS settings, reload the previous settings and exit the BIOS utility after reload.

## 2.7.3	Load Default Settings

This option loads the default settings for the optimized system configurationxe "loading setup defaults"§.    After loading the settings, you return to the Main menu.  Press   to leave the BIOS utility.

## 2.7.4	Save CMOS Settings

Select this option to save the current BIOS settings, including your recent modifications.  After saving,  you return to the Main menu.  Press   to leave the BIOS utility.

## 2.7.5	Load Previous Settings

This option cancels all modifications that you have made in the system configuration and reloads your previous settings.  After reload, you return to the Main menu.  Press   to leave the BIOS utility.