The V55 is a high-performance system board with a 64-bit architecture.  It supports the P54C, P55C, Cyrix M1 (P120+, P150+, P166+ - single and dual voltage), AMD K5 (PR166), and AMD K6 (PR200) CPUs.  The system board utilizes the Peripheral Component Interconnect (PCI) local bus architecture that maximizes the system performance by enabling high-speed peripherals to match the speed of the microprocessor with its 120 MB or 132 MB per second transfer rate in burst mode.

Two DRAM banks composed of four 72-pin sockets come with the board to support single- and double-density SIMMs for a maximum system memory of 128 MB.  The SIMM sockets accommodate both the standard page mode and extended data output (EDO) SIMMs.  An exclusive slot comes with the board to allow a cache upgrade of up to 512 KB using a pipeline burst cache module.

The two onboard PCI-enhanced IDE interfaces with a zero-wait state and 16.6 MB per second transfer rate support up to four IDE devices.  Onboard I/O interfaces comprise of two UART 16550 serial ports, a parallel port with ECP/EPP feature, and PS/2 keyboard and mouse ports.  An AT keyboard is an option.

# 1.1	Major Features

The system board has the following major features:

- A zero-insertion force (ZIF) socket for P54C, P55C, Cyrix M1 (P120+, P150+, P166+ - single and dual voltage), AMD K5 (PR166), and AMD K6 (PR200) CPUs

- Two DRAM banks composed of four 72-pin SIMM sockets that support 4/8/16/32-MB 60/70 ns SIMMs (use of 60 ns DRAMs recommended; 60 ns DRAMs meet the chipset default DRAM timing)

- Three ISA and four PCI bus expansion slots

- 256/512-KB onboard pipeline-burst second-level cache.  If the board comes with 256-KB cache, you may upgrade it to 512 KB by installing a UMC UM61 3232AF-7 256K cache module.

- 128-KB Flash ROM for system BIOS

- Two PCI-enhanced IDE interfaces that support up to four IDE devices

- System clock/calendar with 256-byte CMOS RAM

- PS/2 keyboard and mouse ports (optional AT keyboard port)

# 1.2	Layout

Figure 1-1 shows the locations of the system board major components.

| 1 SIMM sockets 2 CPU voltage regulator 3 CPU socket 4 CPU voltage regulator 5 Cache module socket 6 Pipeline burst second-level cache | 7 PS/2 keyboard port 8 PS/2 mouse port 9 PCI expansion slots 10 Battery 11 ISA expansion slots 12 BIOS |
| --- | --- |

Figure 1-1	System Board Layout

# 1.3	Jumpers and Connectors

## 1.3.1	Jumper and Connector Locations

Figure 1-2 shows the jumper and connector locations on the system board.

Figure 1-2	System Board Jumper and Connector Locations

|  | The blackened pin of a jumper or connector represents pin 1. |
| --- | --- |

## 1.3.2	Jumper Settings

Table 1-1 lists the system board jumpers with their corresponding settings and functions.

Table 1-1	Jumper Settings

| Jumper | Setting | Function |
| --- | --- | --- |
| ROM Type JP4 | 1-2 2-3* | EPROM Flash ROM |
| ROM Brand JP5 | 1-2 2-3* 3-4 | NC 29EE010 28F001 |
| CPU Core Voltage JP6 | 1-2 2-3* | 3.2V 2.8V |
| CPU I/O Voltage JP7 | 1-2 2-3* | 3.5V 3.3V |
| L2 Cache Size JP8, JP9 | 1-2 2-3* | 512 KB 256 KB |
| L2 Cache Mode JP10 | 1-2 2-3* | Reserved Normal operation |
| Clearing CMOS JP11 | Open Shorted | Normal operation Clear CMOS |

### Clearing CMOS

In case you forget your password, short JP11 to clear the CMOS.  There is no physical jumper on the location of JP11.  Instead, you will find two solder pads representing JP11.

Follow these steps to short JP11:

1.	Remove the system battery from its socket.

2.	Use a wire to short the two solder pads for about 10 seconds.

3.	Re-insert the system battery into the socket.

### Selecting a CPU

A multifunction switch marked S1 comes onboard to allow selection of the CPU clock frequency and core/clock ratio depending on the CPU type.

Figure 1-3 shows the S1 multifunction switch.

Figure 1-3	S1 Multifunction Switch

Table 1-2 lists the settings and functions of the S1 multifunction switch.

Table 1-2	S1 Settings for CPU Selection

| CPU Freq. (MHz) | Host Bus Freq. (MHz) | Switch Settings SW1 SW2 SW3 SW4 | Clock Ratio |
| --- | --- | --- | --- |
| Intel P54C/P55C, AMD K5 /K6 |  |  |  |
| 233 | 66 | Off Off On Off | 7/2 |
| 200 | 66 | On Off On Off | 3 |
| 180 | 60 | On Off Off On | 3 |
| 166 | 66 | On On On Off | 5/2 |
| 150 | 60 | On On Off On | 5/2 |
| 133 | 66 | Off On On Off | 2 |
| 120 | 60 | Off On Off On | 2 |
| 100 | 66 | Off Off On Off | 3/2 |
| 90 | 60 | Off Off Off On | 3/2 |
| 75 | 50 | Off Off On On | 3/2 |
| Reserved | -- | -- -- -- -- | -- |
| Cyrix M1/IBM 6x86 |  |  |  |
| P166+ | 66 | Off On On Off | 2 |
| P150+ | 60 | Off On Off On | 2 |
| P120+ | 50 | Off On On On | 2 |

|  | Note that the 233 MHz CPU frequency applies to the Intel Pentium CPU only. |
| --- | --- |

## 1.3.3	Connector Functions

Table 1-3 lists the different connectors on the system board and their respective functions.

Table 1-3	Connector Functions

| Connector | Function |
| --- | --- |
| CN1 | Standby power connector |
| CN2 | Serial port 2 connector |
| CN3 | Serial port 1 connector |
| CN4 | Parallel port connector |
| CN5 | Diskette drive connector |
| CN6 | IDE 2 connector |
| CN8 | Power connector |
| CN9 | IDE 1 connector |
| CN10 | PS/2 keyboard port |
| CN11 | PS/2 mouse port |
| CN13 | CPU power shorting block |
| CN14 | CPU fan connector |
| CN16 | Multifunction connector |
| CN17 | External suspend switch |
| CN18 | Software shutdown switch |
| CN19 | IrDA connector (optional) |
| CN20 | IDE LED connector |

The multifunction connector CN16 accommodates the front panel connectors for speaker, LEDs, keylock, and reset.  Figure 1-4 shows the CN16 pin assignments for each of the front panel connectors.

Figure 1-4	20-Pin Multifunction Connector  (CN16)

# 1.4	ESD Precautions

Always observe the following electrostatic discharge (ESD) precautions before installing a system component:

1.	Do not remove a component from its antistatic packaging until you are ready to install it.

2.	Wear a wrist grounding strap before handling electronic components.  Wrist grounding straps are available at most electronic component stores.

|  | Do not attempt the procedures described in the following sections unless you are a qualified technician. |
| --- | --- |

# 1.5	Memory Upgrade

The system board comes with four 72-pin SIMM sockets that support 4-MB and 16-MB single-density SIMMs and 8-MB and 32-MB double-density SIMMs.  Tables 1-4 and 1-5 list the possible memory configurations.

## 1.5.1	Memory Configurations

Table 1-4	Memory Configurations (64-bit)

| Bank 0 |  | Bank 1 |  | Total |
| --- | --- | --- | --- | --- |
| SIMM-1 | SIMM-2 | SIMM-3 | SIMM-4 | Memory |
| 4 MB | 4 MB |  |  | 8 MB |
|  |  | 4 MB | 4 MB | 8 MB |
| 8 MB | 8 MB |  |  | 16 MB |
|  |  | 8 MB | 8 MB | 16 MB |
| 4 MB | 4 MB | 4 MB | 4 MB | 16 MB |
| 4 MB | 4 MB | 8 MB | 8 MB | 24 MB |
| 8 MB | 8 MB | 4 MB | 4 MB | 24 MB |
| 8 MB | 8 MB | 8 MB | 8 MB | 32 MB |
| 16 MB | 16 MB |  |  | 32 MB |
|  |  | 16 MB | 16 MB | 32 MB |
| 4 MB | 4 MB | 16 MB | 16 MB | 40 MB |
| 16 MB | 16 MB | 4 MB | 4 MB | 40 MB |
| 8 MB | 8 MB | 16 MB | 16 MB | 48 MB |
| 16 MB | 16 MB | 8 MB | 8 MB | 48 MB |
| 16 MB | 16 MB | 16 MB | 16 MB | 64 MB |
| 32 MB | 32 MB |  |  | 64 MB |
|  |  | 32 MB | 32 MB | 64 MB |
| 4 MB | 4 MB | 32 MB | 32 MB | 72 MB |
| 32 MB | 32 MB | 4 MB | 4 MB | 72 MB |
| 8 MB | 8 MB | 32 MB | 32 MB | 80 MB |
| 32 MB | 32 MB | 8 MB | 8 MB | 80 MB |
| 16 MB | 16 MB | 32 MB | 32 MB | 96 MB |
| 32 MB | 32 MB | 16 MB | 16 MB | 96 MB |
| 32 MB | 32 MB | 32 MB | 32 MB | 128 MB |

The system also supports 32-bit memory configurations.  This feature allows installation of only one SIMM in a bank or in one configuration.  It also allows random installation of SIMMs in any of the sockets.  Table 1-5 lists the 32-bit memory configurations.

Table 1-5	Memory Configurations  (32-bit)

| Bank 0 |  | Bank 1 |  | Total |
| --- | --- | --- | --- | --- |
| SIMM-1 | SIMM-2 | SIMM-3 | SIMM-4 | Memory |
| 4 MB* |  |  |  | 4 MB |
| 4 MB* | 4 MB* | 4 MB* |  | 12 MB |
| 4 MB | 4 MB | 8 MB |  | 16 MB |
| 4 MB | 4 MB | 16 MB |  | 24 MB |
| 4 MB | 4 MB | 32 MB |  | 40 MB |
| 8 MB |  |  |  | 8 MB |
| 8 MB | 8 MB | 4 MB |  | 20 MB |
| 8 MB | 8 MB | 8 MB |  | 24 MB |
| 8 MB | 8 MB | 16 MB |  | 32 MB |
| 8 MB | 8 MB | 32 MB |  | 48 MB |
| 16 MB |  |  |  | 16 MB |
| 16 MB | 16 MB | 4 MB |  | 36 MB |
| 16 MB | 16 MB | 8 MB |  | 40 MB |
| 16 MB | 16 MB | 16 MB |  | 48 MB |
| 16 MB | 16 MB | 32 MB |  | 64 MB |
| 32 MB |  |  |  | 32 MB |
| 32 MB | 32 MB | 4 MB |  | 68 MB |
| 32 MB | 32 MB | 8 MB |  | 72 MB |
| 32 MB | 32 MB | 16 MB |  | 80 MB |
| 32 MB | 32 MB | 32 MB |  | 96 MB |

|  | The system does not support ECC and parity check features while in 32-bit access. |
| --- | --- |

## 1.5.2	Installing a SIMM

Follow these steps to install a SIMM:

1.	Carefully slip a SIMM at a 45° angle into a socket making sure that the curved edge indicating the pin 1 of the SIMM matches pin 1 of the socket.

|  | A SIMM fits only in one direction. If you slip in a SIMM but would not completely fit, you may have inserted it the wrong way. Reverse the orientation of the SIMM. |
| --- | --- |

2.	Gently push the SIMM to a vertical position until the pegs of the socket slip into the holes on the SIMM, and the holding clips lock the SIMM into position.  The SIMM should be at a 90° angle when installed.

Hole

Peg

Pin 1 Indicator (curved edge)

1

2

Figure 1-5	Installing a SIMM

## 1.5.3	Removing a SIMM

Follow these steps to remove a SIMM:

1.	Press the holding clips on both sides of the SIMM outward to release it.

2.	Move the SIMM to a 45° angle.

Holding Clip

3.	Pull the SIMM out of the socket.

1

2

3

Figure 1-6	Removing a SIMM

|  | Always remove SIMMs from the socket labeled SIMM4, then SIMM3, and so on. |
| --- | --- |

## 1.5.4	Reconfiguring the System

You must enter Setup after installing or removing SIMMs to reconfigure the system.

Follow these steps to reconfigure the system:

1.	Turn the system on.  A memory error message appears, indicating that the total memory does not match the value stored in CMOS.

2.	Press  +  +  to enter Setup.  A warning message appears indicating an incorrect memory configuration.

3.	Press  twice to exit and reboot the system.

The system boots with the new memory configuration.

# 1.6	IDE Hard Disk Support

The system board supports four IDE hard disks, or any other IDE devices, through the two onboard PCI IDE interfaces.  See Figure 1-1 for the location.

Follow the instructions in the housing installation manual on how to install a hard disk in the system.  Connect the cables according to the IDE hard disk configuration in Table 1-6.

Table 1-6	IDE Hard Disk Configuration

| IDE Connector | Master | Slave |
| --- | --- | --- |
| Channel 1 | Hard disk 0 | Hard disk 1 |
| Channel 2 | Hard disk 2 | Hard disk 3 |

# 1.7	CPU Installation

The system board comes with a zero-insertion force (ZIF) CPU socket for easy installation.

Follow these steps to install a Pentium CPU:

1.	Set JP6 to the correct CPU core voltage, JP7 to the CPU I/O voltage, and S1 for the CPU frequency.  Refer to Tables 1-1 and 1-2 for the appropriate settings and to your CPU documentation for more information.

2.	If you install a dual-voltage CPU, remove the shorting block on CN13.  See your CPU documentation for the specifications.

3.	Lift up the CPU socket lever.

4.	Insert the CPU into the socket.  Make sure that the notched corner of the CPU matches the pin 1 indicator on the socket.

5.	Push down the socket lever.

STEP  1

STEP  2

STEP  3

Notched Corner

Pin 1 Indicator

Figure 1-7	Installing a Pentium CPU

# 1.8	Second-level Cache Configuration

The system board has an onboard 256-KB or 512-KB synchronous second-level cache.  The 160-pin cache module slot that comes with the board supports a pipeline burst SRAM module.  If the board comes with a 256-KB cache, you can  upgrade it to 512 KB by installing a UMC UM61 3232AF-7 256K cache module.

Follow these steps to install a cache module:

1.	Position the cache module on top of the slot such that its component side faces the CPU.

2.	Insert the cache module carefully until the golden finger of the module fit into place.

Figure 1-8	Installing a Cache Module

3.	Set jumpers JP8 and JP9 to the correct second-level cache size.

|  | Do not forget this step. The system will not work properly if JP8 and JP9 are not set properly. |
| --- | --- |

# 1.9	Installing ISA Cards

Both PnP and non-PnP ISA cards require specific IRQs.  When installing ISA cards, make sure that the IRQs required by these cards are not previously assigned to PCI devices to avoid resource conflicts.

Follow these steps when installing ISA cards:

- Remove all PnP cards installed in the system, if any.

- Enter BIOS utility and set the Reset Resource Assignment parameter to Yes to clear the resource data assigned to the PnP devices.  Refer to section 2.6.5.

- Install non-PnP ISA cards.

- Turn on the system.

- Use Windows 95 or ICU to manually assign the appropriate IRQs to the cards.  This ensures that BIOS will not use the resources assigned to the non-PnP ISA cards.

|  | BIOS detects and configures only PnP cards. |
| --- | --- |

6.	Turn off the system.

7.	Install PnP ISA and PCI cards.

8.	Turn on the system.  This time PnP BIOS automatically configures the PnP ISA and PCI cards with the remaining free IRQs.

# 1.10	Error Messages

Do not continue using the computer if you receive an error message of any type.  Note the message and take corrective action.  This section explains the different types of error messages and corresponding corrective measures.

There are two general types of error messages:

- Software

- System

## 1.10.1	Software Error Messages

Software error messages are returned by your operating system or application.  These messages typically occur after you boot the operating system or when you run your applications.  If you receive this type of message, consult your application or operating system manual for help.

## 1.10.2	System Error Messages

A system error message indicates a problem with the computer itself.  A message of this type normally appears during the power-on self-test, before the operating system prompt appears.

Table 1-7 lists the system error messages.

Table 1-7	System Error Messages

| Message | Action |
| --- | --- |
| CMOS Battery Error | Replace the battery chip or contact your dealer. |
| CMOS Checksum Error | Load the BIOS default settings or contact your dealer. |
| Diskette Drive Controller Error or Not Installed | Check the diskette drive cables if properly connected. |
| Diskette Drive Error | Diskette drive may be defective. If it is, replace the drive. |
| Diskette Drive A Type Mismatch | Run Setup and select the proper drive type. |
| Diskette Drive B Type Mismatch | Run Setup and select the proper drive type. |
| Equipment Configuration Error | Modify the memory configuration to agree with one of the options in Tables 1-4 and 1-5. |
| Hard Disk Controller Error | Run Setup. |
| Hard Disk 0 Error | Check all cable connections. Replace hard disk. |
| Hard Disk 1 Error | Check all cable connections. Replace hard disk. |
| Keyboard Error or No Keyboard Connected | Check and connect the keyboard to the system unit. |
| Keyboard Interface Error | Replace the keyboard or contact your dealer. |

Table 1-7	System Error Messages  (continued)

| Message | Action |
| --- | --- |
| Memory Error at: MMMM:SSSS:OOO (W:XXXX, R:YYYY) where: M: MB, S: Segment, O: Offset, X/Y: write/read pattern | Check SIMMs on the system board. Contact your dealer. |
| CPU Clock Mismatch | Run Setup. Check if the CPU clock is correct. If correct, exit Setup and reboot the system. If the error message reappears, ask for technical assistance. |
| Onboard Serial Port 1 Conflict | Run Setup and disable the port. |
| Onboard Serial Port 2 Conflict | Run Setup and disable the port. |
| Onboard Parallel Port Conflict | Run Setup and disable the port. |
| Pointing Device Error | Check and connect pointing device. |
| Pointing Device Interface Error | Replace the pointing device or contact your dealer. |
| Press F1 key to continue or Ctrl+Alt+Esc for Setup | Press or ++ to enter Setup. |
| Real Time Clock Error | Contact your dealer. |

## 1.10.3	Correcting Error Conditions

As a general rule, if an error message says "Press F1 to continue," it is caused by a configuration problem, which can be easily corrected.  An equipment malfunction is more likely to cause a fatal error, i.e., an error that causes complete system failure.

Here are some corrective measures for error conditions:

1.	Run Setup.  You must know the correct configuration values for your system before you enter Setup, which is why you should write them down when the system is correctly configured.  An incorrect configuration is a major cause of power-on error messages, especially for a new system.

2.	Remove the system unit cover.  Check that the jumpers on the system board and any expansion boards are set correctly.

3.	If you cannot access a new disk, it may be because your disk is not properly formatted.  Format the disk first using the FDISK and FORMAT commands.

4.	Check that all connectors and boards are securely plugged in.

If you go through the corrective steps above and still receive an error message, the cause may be an equipment malfunction.

If you are sure that your configuration values are correct and your battery is in good condition, the problem may lie in a damaged or defective chip.

In both cases, contact an authorized service center for assistance.

*	Default setting

*	May also be installed in the other sockets.  The same applies to the other
	configurations.