The AP41 is a 486-based system board that utilizes the PCI/ISA architecture.  It supports the 486SX, DX, DX2, and DX4 series microprocessors.  It has four ISA-AT and three PCI slots for future expansion.  The system memory is expandable to 128 MB by adding single in-line memory modules (SIMMs).  The second-level cache is upgradable to 512 KB and supports both the write-back and write-through modes.

Two SiS application-specific integrated circuit (ASIC) chipsets with built-in two-channel, PCI mode 4 IDE controller and a super I/O controller are also incorporated in the design to further enhance system performance.  The board measures 220 mm x 250 mm (baby-AT size).

# Specifications

| Microprocessor | 5V CPUs Intel SL-enhanced Intel SX/DX/DX2 AMD DX/DX2 Cyrix DX/DX2 Intel P24D Intel P24T |
| --- | --- |
|  | 3.45V CPUs Intel DX4 AMD DX2 AMD DX4/DX4-S Cyrix DX2/DX4/5X86 TI DX2 |
| Maximum Memory | 128 MB |
| SIMM Sockets | Four 72-pin, 32-bit |
| ASICs | SiS85C496 SiS85C497 |
| Bus Architecture | ISA, PCI |
| Expansion Slots | Four ISA slots, Three PCI slots |
| Ports | One parallel port (ECP/EPP) Two serial ports Two-channel PCI mode 4 IDE One floppy disk connector |
| Onboard Battery | CR2032 3.0V lithium battery |
| RTC | 146818 built-in 85C497 |
| Secondary Cache | 128 KB/256 KB/512 KB |
| BIOS | AMI Plug-and-Play WinBIOS |
| Board Size | 220 mm x 250 mm (baby-AT) |

# Board Layout

1.	Keyboard connector	11.	HDD LED connector

2.	COM1 connector	12.	BIOS

3.	COM2 connector	13.	ISA slots

4.	Power connector	14.	PCI slots

5.	72-pin SIMM sockets	15.	Keyboard controller

6.	ZIF-type CPU socket	16.	IDE2 connector

7.	2-pin fan connector	17.	IDE1 connector

8.	4-pin fan connector	18.	FDD connector

9.	Tag SRAM	19.	Battery

10.	Second-level cache	20.	Parallel port connector

# System Board Parts

## Microprocessor

The AP41 system board supports the Intel SL-enhanced, Intel 486SX/DX/DX2/DX4, AMD 486DX/DX2/DX4/DX4-S, Cyrix 486DX/DX2/DX4/5X86 and TI 486DX2 series microprocessors.  Chapter 2 gives details on how to upgrade the processor.

## ASICs

The two ASICs onboard are the SiS85C496 and SiS85C497.  The SiS85C496 acts as the PCI and CPU memory controller (PCM).  It integrates the host bridge, the cache and main memory DRAM controller, the PCI bridge, the built-in IDE controller and the Fast Slow Link (FS-Link) bus.

The SiS85C497 is the AT-bus controller and megacell (ATM) component that acts as an interface the PCI/CPU/memory bus and the ISA bus.

## AMI BIOS

The AMI BIOS (basic input-output system) resides in the flash ROM chip.  This contains the program that performs the power-on self-tests (POST) upon booting.  During POST, this program activates the peripheral devices, tests onboard memory, and prepares the system for operation.  Chapter 3 gives more information on the AMI BIOS.

## Expansion Slots

The board expansion slots consist of four ISA-AT and three PCI slots.  These expansion slots are the parallel bars on the system board.  There are rows of golden pins inside each slot that serve as a clutch to secure the contacts of expansion boards.  Chapter 2 tells how to install expansion boards.

## DRAM Sockets

The system board has four 72-pin DRAM sockets that expand system memory from 1 MB to a maximum of 128 MB.  These sockets accept single- and double-density SIMMs.  Chapter 2 tells how to install memory modules.

## Two-Channel PCI Mode 4 IDE

The board utilizes the enhanced-integrated drive electronics (E-IDE) interface that improves the data transfer rate.  It also allows the system to support four E-IDE devices, including fixed disks with more than 528-MB capacity.  This feature offers users increased data storage capacity.

## Super I/O Controller

The onboard super I/O controller chip supports two UART 16450/16550-compatible serial ports and a parallel port (SPP, EPP, ECP).  It also accommodates 1.2-/1.44-/2.88-MB disk drives allowing full-range access to 5.25-inch drives with 360-KB or 1.2-MB format and 3.5-inch drives with 720-KB, 1.44-MB or 2.88-MB format.

## Keyboard Connector

The keyboard connector at the rear of the system board allows you to connect any AT-compatible keyboard.

## SRAM

The system board supports 128-KB, 256-KB and 512-KB second-level, write-back and write-through cache.

## Power Management

The AP41 conforms to the power-saving standards of the U.S. Environmental Protection Agency (EPA) Energy Star program.  The system board features four power-saving modes that reduce power consumption.  For more information on the power-saving modes, see Chapter 3.

SPP:  Standard Parallel Port	
EPP:  Enhanced Parallel Port (IEEE 1284 compliant)	
ECP:  Extended Capabilities Port (IEEE 1284 compliant)