AP41

User’s Guide

Copyright Ó 1995 by this company.  All rights reserved.  No part of this publication may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language or computer language, in any form or by any means, electronic, mechanical, magnetic, optical, manual or otherwise, without the prior written permission of this company.

This company makes no representations or warranties, either expressed or implied, with respect to the contents hereof and specifically disclaims any warranties, merchantability or fitness for any particular purpose.  Any software described in this manual is sold or licensed "as is".  Should the programs prove defective following their purchase, the buyer (and not this company, its distributor, or its dealer) assumes the entire cost of all necessary servicing, repair, and any incidental or consequential damages resulting from any defect in the software.  Further, this company reserves the right to revise this publication and to make changes from time to time in the contents hereof without obligation to notify any person of such revision or changes.

AMI is a registered trademark of American Megatrends Inc.

Intel is a registered trademark of Intel Corporation.

PC XT/AT is a registered trademark of International Business Machines Corporation.

Other brand and product names are trademarks and/or registered trademarks of their respective holders.

FCC Class B Radio Frequency
Interference Statement

Note:

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of FCC Rules.  These limits are designed to provide reasonable protection against harmful interference in a residential installation.  This equipment generates, uses, and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications.  However, there is no guarantee that interference will not occur in a particular installation.  If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

1.	Reorient or relocate the receiving antenna.

2.	Increase the separation between the equipment and receiver.

3.	Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

4.	Consult the dealer or an experienced radio/television technician for help.

Notice 1:

The changes or modifications not expressly approved by the party responsible for compliance could void the user's authority to operate the equipment.

Notice 2:

Shielded interface cables, if any, must be used in order to comply with emission limits.

## Purpose and Scope

This manual tells how to install and configure the system board.

## Organization

This manual consists of three chapters and two apendices:

Chapter 1, Features, covers the specifications, layout, and components of the system board.

Chapter 2, Hardware Setup, tells how to set the jumpers, upgrade the CPU and the system memory, install the system board and add expansion cards.

Chapter 3, AMI BIOS, explains the system BIOS and tells how to configure the system by setting the BIOS parameters.

Appendix A, Jumper Summary, gives you a tabular summary of the jumper settings discussed in Chapter 2.

Appendix B, SiS Drivers, tells how to install the drivers supported by SiS.

## Conventions

The following conventions are used in this manual:

| Text entered by user, default settings | Represent text input by the user, default settings and recommended selections |
| --- | --- |
| message displayed | Denotes actual messages that appear on screen |
| a, e, s, etc | Represent the actual keys that you have to press on the keyboard. |
|  | NOTE Gives bits and pieces of additional information related to the current topic. |
|  | WARNING Alerts you to any damage that might result from doing or not doing specific actions. |
|  | CAUTION Suggests precautionary measures to avoid potential hardware or software problems. |
|  | IMPORTANT Reminds you to take specific action relevant to the accomplishment of the procedure at hand. |
|  | TIP Tells how to accomplish a procedure with minimum steps through little shortcuts. |

1	Features

Specifications	1-2

Board Layout	1-3

System Board Parts	1-4

Microprocessor	1-4

ASICs		1-4

AMI BIOS	1-4

Expansion Slots	1-5

DRAM Sockets	1-5

Two-Channel PCI Mode 4 IDE	1-5

Super I/O Controller	1-6

Keyboard Connector	1-6

SRAM		1-6

Power Management	1-6

2	Hardware Setup

CPU Options	2-1

5V CPU Options	2-1

3.45 V CPU Options	2-2

ESD Precautions	2-2

Installing a Microprocessor	2-3

Upgrading the Microprocessor	2-4

Jumper Settings	2-5

Changing the CPU Type	2-7

Setting the Oscillator Frequency	2-14

Setting the Flash ROM Type	2-14

Selecting the ECP DMA Channel	2-15

Enabling the FDC and Super I/O Chip	2-15

Selecting the Cache Size	2-16

Clearing the CMOS	2-18

Memory Configuration	2-18

Installing a SIMM	2-19

Removing a SIMM	2-20

Connectors	2-21

Multifunction Connector	2-21

Keyboard Connector	2-23

Power Connector	2-24

Break/Suspend Connector	2-24

Fan Power Connectors	2-25

External Battery Connector	2-26

Installation	2-27

Installing the System Board	2-27

Installing Expansion Boards	2-28

3	AMI BIOS

AMI BIOS Setup Main Menu	3-1

Standard CMOS Setup	3-2

Date/Time	3-2

Floppy Drives A and B	3-3

Hard Disk Drives	3-4

Advanced CMOS Setup	3-5

Typematic Rate (Chars./Sec.)	3-7

System Keyboard	3-7

Primary Display	3-7

Above 1 MB Memory Test	3-8

Memory Test Tick Sound	3-8

Hit “Del” Message Display	3-8

Extended BIOS RAM Area	3-8

Wait for F1 If Any Error	3-8

System Boot-up Num Lock	3-9

Numeric Processor Test	3-9

Floppy Drive Seek at Boot	3-9

Floppy Drive Swapping	3-9

System Boot-up Sequence	3-9

System Boot-up CPU Speed	3-10

Turbo Switch Function	3-10

Password Checking	3-10

CPU Selection	3-10

Cache		3-10

Shadow C800, D000, D800, 32K	3-12

Video Shadow	3-12

Main BIOS Cacheable	3-12

Cacheable C800, D000, D800, 32 K	3-12

Video Shadow Cacheable	3-13

Secondary IDE Drives Present	3-13

Primary 1st IDE Block Mode	3-13

Primary 2nd IDE Block Mode	3-13

Secondary 1st IDE Block Mode	3-14

Secondary 2nd IDE Block Mode	3-14

Chipset Setup Mode	3-15

Chipset Features Setup	3-15

Memory Speed (DRAM)	3-17

Cache Speed (SRAM)	3-17

CPU-to-PCI Memory Post Write Buffer	3-18

CPU-to-Memory Burst Write	3-18

DRAM Type (Refresh)	3-18

Power Management Setup	3-22

BIOS Power Management Mode	3-22

Advanced Power Management	3-24

Doze Timer	3-24

Standby Timer	3-24

Suspend Timer	3-24

Wake-up Events	3-25

Monitor Type	3-25

Display Off After	3-25

HDD Off After	3-25

Deturbo After	3-26

Break Switch	3-26

Peripheral Setup	3-26

Programming Mode	3-27

Onboard FDC	3-27

Serial Port 1	3-27

Serial Port 2	3-27

Parallel Port	3-27

IRQ Active	3-28

Parallel Port Mode	3-28

Utility Setup	3-28

Detect Master and Detect Slave	3-28

Color Set	3-29

Default Setup	3-30

Original	3-30

Optimal	3-30

Fail-safe	3-31

Security Setup	3-32

Password	3-32

Anti-virus	3-34

Exit Setup	3-35

NCR SCSI BIOS and Drivers	3-36

Appendix A		Jumper Summary

Appendix B		SiS Drivers