This chapter discusses the CPU options, tells how to set jumpers, upgrade system memory, add expansion boards, and install the system board.

Install the CPU, memory, and set the jumpers before you install the system board inside a system housing.  You may add the other components after installing the board.  Read this chapter to learn about the components before you install them.

# CPU Options

The system board supports both 5V and 3.45V CPUs.  The following tables list the CPU options for this system board.

## 5V CPU Options

| 5V CPU Type |  | Speed (MHz) |
| --- | --- | --- |
| Intel series (SL-enhanced) | 486SX 486DX 486DX2 P24D P24T | 25/33 33 50/66 50/66 63/83 |
| AMD series | 486DX 486DX2 | 33/40 50/66/80 |
| Cyrix | 486DX 486DX2 | 33/40 50/66 |

## 3.45 V CPU Options

| 3.45V CPU Type | Speed (MHz) |
| --- | --- |
| Intel 486DX4 | 75/100 |
| AMD 486DX2 | 66/80 |
| AMD 486DX4 | 100 |
| AMD 486DX4-S | 100/120 |
| Cyrix DX2 | V50/V66/V80 |
| Cyrix DX4 | 100/120 |
| Cyrix 5X86 | 100 |
| TI DX2 | 66/80 |

# ESD Precautions

Electrostatic discharge (ESD) can damage your CPU, disk drives, expansion boards, and other components.  Always observe the following precautions before you install a system component.

1.	Do not remove a component from its protective packaging until you are ready to install it.

2.	Wear a wrist grounding strap and attach it to a metal part of the system unit before handling components.  If a wrist strap is not available, maintain contact with the system unit throughout any procedure requiring ESD protection.

# Installing a Microprocessor

The motherboard has a zero-insertion force microprocessor socket that allows you to install a CPU without using any tools.

Follow these steps to install a CPU in a ZIF-type upgrade socket:

|  | Make sure that the system power is OFF before installing any component. |
| --- | --- |

1.	Pull up the socket lever.

2.	Insert the CPU with the attached heatsink and fan.  Make sure that pin 1 of the CPU aligns with hole 1 of the socket.  The hole on the upper right corner of the center box indicates pin 1.

Pin 1 indicator

Hole 1 indicator

CPU         SOCKET

3.	Pull down the socket lever to lock the CPU into the socket.

4.	Plug the fan cable into the onboard fan connector.

5.	Set the jumpers accordingly.  See the following sections for the correct jumper settings.

# Upgrading the Microprocessor

Follow these steps to upgrade the CPU:

1.	Turn off the system power.

2.	Pull up the socket lever.

3.	Remove the installed CPU.

4.	Install the upgrade CPU.  Refer to the section Installing a Microprocessor for instructions on how to install a CPU.

# Jumper Settings

You have to change the jumper settings when you reconfigure the system.  This section tells how to reset the jumpers.  The figure below shows the jumper locations.

Set a jumper switch as follows:

- To close a jumper switch, insert the plastic jumper cap over two pins of a jumper.

- To open a jumper switch, remove the jumper cap.

The following conventions are used to represent the proper jumper settings.

Open

Closed (2-3 position)

|  | When you open a jumper, attach the plastic jumper cap to one of the pins so you won't lose it. |
| --- | --- |

## Changing the CPU Type

You must set jumpers JP11 to JP23 and JP40 according to the CPU type.  The board supports both 5V and 3.45V CPUs.  Make sure that you know your CPU type and its operating voltage before you begin.  Improper setting of jumpers may result in system failure.

### 5V CPU Type

For Intel processors, you must determine if your CPU is SL-enhanced or not.  To check, simply look for the "&" symbol in the third row on the lower label of the CPU.

The P24D processor looks very similar to an Intel SL-enhanced CPU except for the letter “W” in the third row on the lower label of the CPU.  The letter “W” indicates that the CPU supports the write-back mode.

The following figure shows how to determine an Intel SL-enhanced CPU and a P24D CPU.

SL-ENHANCED

INDICATOR

WRITE-BACK MODE

INDICATOR

#### 5V CPU Type jumper settings

JP11

JP12

JP13

JP14

JP15

JP16

JP17

JP18

JP19

JP20

JP21

JP22

JP23

JP40

### 3.45V CPU Type

#### IntelDX4 CPUs

You must be able to determine your IntelDX4 CPU type before you set the jumpers.  Improper setting may result to system malfunction.

There are two types of 3.45V IntelDX4 CPUs:  one with write-through mode support while the other is with write-back mode support.  To determine an IntelDX4 CPU with write-back internal mode, look for the letter “W” in the third row on the lower label of the CPU.  The letter “W” indicates write-back mode.  An IntelDX4 CPU with write-through internal mode does not have this indication.  See the following figure.

#### AMD DX4 CPUs

The 3.45V AMD DX4 processors come in three types:  AMD DX4 (V8T), AMD DX4-S(SV8T) and AMD DX4-S(SV8B).  The AMD DX4 (V8T) is a non SL-enhanced CPU that supports write-through internal cache mode.  The AMD DX4-S(SV8T) is an SL-enhanced CPU that supports write-through internal mode.  The AMD DX4-S(SV8B) is also an SL-enhanced CPU but with write-back internal cache mode support.

To determine your AMD CPU type, check the second row of your CPU label.  An SL-enhanced AMD DX4 CPU has a letter “S” following the “AMD DX4-100” label.  The last letter (i.e., either “B” or “T”) indicates the CPU cache mode supported.  See the following figure.

AMD 486DX4 (V8T)

(w/out SMM support)

AMD 486DX4-S

(SV8T)

AMD 486DX4-S

(SV8B)

| “T” indicates write-through“S”: SL-enhanced“S”: SL-enhanced mode “T” indicates “B” indicates write-through mode write-back mode |  |  |
| --- | --- | --- |

#### Cyrix DX4 CPUs

There are two types of  Cyrix DX4 CPUs:  Cyrix DX4 with iDX4 pinout and Cyrix DX4 with M7 pinout.

To determine the Cyrix DX4 CPU pinout, check the CPU label.  A Cyrix DX4 with iDX4 pinout has a “DX4 P/O” indication on the second row of the lower CPU label.  A Cyrix DX4 with M7 pinout does not have this indication and the lower CPU label has only two rows.  See the figure below.

iDX4 pinout indication

iDX4 Pinout     M7 Pinout

#### 3.45V CPU Type JUmPer SEttings

JP11

JP12

JP13

JP14

JP15

JP16

JP17

JP18

JP19

JP20

JP21

JP22

JP23

JP40

JP11

JP12

JP13

JP14

JP15

JP16

JP17

JP18

JP19

JP20

JP21

JP22

JP23

JP40

## Setting the Oscillator Frequency

You must reset jumpers JP30, JP31 and JP32 if you change the oscillator frequency.

25 MHz         33 MHz        40 MHz

JP30

JP31

JP32

## Setting the Flash ROM Type

Set jumper JP4 according to the type of Flash ROM in use.  If the system uses a 5V EEPROM (or 5V Flash ROM) instead of a 12V Flash ROM, you must set the jumper to 2-3.  The default setting is 
2-3.

JP4

12V Flash ROM

5V EEPROM or Flash ROM

## Selecting the ECP DMA Channel

The jumpers JP2 and JP3 are used to select the DMA channel for ECP function.  The default channel is DMA 1.

JP2   JP3

DMA 1

DMA 3

## Enabling the FDC and Super I/O Chip

The jumper JP1 is used to enable or disable the floppy disk controller (FDC) and the super I/O chip.  The SMC665 onboard I/O chip supports two serial ports, one parallel port and FDD functions.  Set the jumper to Disabled if you want to use a separate I/O card.

JP1

ENABLED

DISABLED

## Selecting the Cache Size

The system board supports 128-KB, 256-KB, and 512-KB, write-back and write-through second-level cache. See the table below for the possible cache configurations.

The second-level cache is upgradable from 128 KB to 512 KB.  The system supports both write-back and write-through modes.

| Cache Size | TAG SRAM (U31) | Data SRAM | Location |
| --- | --- | --- | --- |
| 128 KB | 16 Kb x 8 or 32 Kb x 8 | 32 Kb x 8 x 4 pcs. | Bank 0 |
| 256 KB | 16 Kb x 8 or 32 Kb x 8 | 32 Kb x 8 x 8 pcs. | Banks 0 and 1 |
| 256 KB | 16 Kb x 8 or 32 Kb x 8 | 64 Kb x 8 x 4 pcs. | Bank 0 |
| 512 KB | 32 Kb x 8 | 64 Kb x 8 x 8 pcs. | Banks 0 and 1 |
| 512 KB | 32 Kb x 8 | 128 Kb x 8 x 4 pcs. | Bank 0 |

Follow these steps to install SRAMs:

1.	Locate the cache sockets on the system board.  See the section Board Layout.

2.	Insert the SRAM chip into the socket.  Align the straight edge of the chip with the straight edge of the socket.  Also, make sure that the cut edge of the chip corresponds to the cut edge of the cache socket.  See the figure below.

|  | Always install SRAMs beginning with Bank 0. |
| --- | --- |

|  | Press the SRAM chip gently but firmly into place. Be careful not to bend the pins. |
| --- | --- |

When upgrading the cache, install the SRAM chips and set jumpers JP24, JP25, JP26, JP27 and JP29 accordingly.  See the following figure.

JP24

JP25

JP26

JP27

JP29

128 KB        256 KB      256 KB         512 KB         512 KB

32 Kb x 4    32 Kb x 8    64 Kb x 4    64 Kb x 8     128 Kb x 4

## Clearing the CMOS

The 3-pin jumper JP37 is used to clear the values in the CMOS.  You need to clear the CMOS if you forget your system password.  To do this, shut off the system power and short pins 2-3 of JP37 for a few seconds.  Then set the jumper to the normal setting by closing pins 1-2 with a jumper cap.  Enter Setup to specify a new password.

NORMAL

JP37

CLEAR CMOS

# Memory Configuration

The system memory is upgradable from 1 MB to 128 MB.   The four 72-pin SIMM sockets accommodate 1-, 4-, 16- and 64-MB single-density SIMMs, and 2-, 8- and 32-MB double-density SIMMs with 70/60-ns DRAM speed.

The table below lists the SIMM types and their corresponding capacities.

| SIMM Type |  |  | Capacity |  |
| --- | --- | --- | --- | --- |
| 256 Kb x 32/36 |  |  | 1 MB |  |
| 512 Kb x 32/36 |  |  | 2 MB |  |
| 1 Mb x 32/36 |  |  | 4 MB |  |
| 2 Mb x 32/36 |  |  | 8 MB |  |
| 4 Mb x 32/36 |  |  | 16 MB |  |
| 8 Mb x 32/36 |  |  | 32 MB |  |
| 16 Mb x 32/36 |  |  | 64 MB |  |
|  |  | The SIMM sockets accept any DRAM combinations that satisfy the 128 MB maximum memory. |  |  |

## Installing a SIMM

|  | Observe the ESD precautions when installing components. |
| --- | --- |

Follow these steps to install a SIMM:

1.	Slip a SIMM at a 45o angle into a socket with the component side facing down.

|  | Be careful when inserting or removing SIMMs. Forcing a SIMM in or out of a socket can damage the socket or the SIMM (or both). |
| --- | --- |

2.	Gently push the SIMM up until the pegs of the socket slip into the holes on the SIMM and the holding clips lock the SIMM into a vertical position.

|  | The SIMM should be at a 90o angle when installed. |
| --- | --- |

## Removing a SIMM

1.	Press the holding clips on both sides of the SIMM outward to release it.

2.	Press the SIMM downward to about a 45o angle.

3.	Gently pull the SIMM out of the socket.

# Connectors

## Multifunction Connector

This 20-pin connector is marked CN5 on the system board.  It supports a number of system functions:  LED, turbo, reset, keylock, and speaker.  Attach the front panel connectors to the corresponding pins as in the illustration below.

Reset        Turbo Switch           Turbo LED

(Green Mode LED)

Speaker          Power LED   Keylock

Some housings have a five-pin connector for the keylock and power LED.  See the following illustration.

Reset          Turbo Switch       Turbo LED

(Green Mode LED)

Speaker            Keylock and Power LED

Other housings may have a 12-pin connector.  If your housing has this type of connector, plug it into CN5 as shown in the following figure.  Make sure that the red wire of the connector connects to pin 11.

Turbo Switch

### Turbo LED/Green Mode LED

You can use the CN5 LED connector either for the Turbo function or for the Green Mode function.  However, you cannot use both functions at the same time.  To use the LED for turbo, you need to connect the turbo switch into the CN5 multifunction connector.  Take note that in turbo mode, the LED is on.  In deturbo mode, the LED is off.

To use the LED for the green function, you need to connect the break switch into JP9 (i.e, the break switch connector).  The green mode LED is always on regardless of the system mode.

## Keyboard Connector

The keyboard connector is a 5-pin, AT-compatible connector.  The following figure shows how to connect a keyboard.

## Power Connector

A standard power supply has two cables with six wires each.  Attach these cables to the power connector on the board in such a way that all the black wires are in the center.

## Break/Suspend Connector

The Break/Suspend connector is a 3-pin connector labeled JP9 on the system board.  Pressing the break switch on the front bezel forces the system to enter the suspend mode.  Pressing any key on the keyboard returns the system to normal mode.

|  | You cannot use the break switch and the turbo switch at the same time. |
| --- | --- |

## Fan Power Connectors

The board comes with one 2-pin and one 4-pin fan power connector.

The 2-pin fan power connector is marked JP43 on the system board.  To connect, plug the connector to its corresponding pin as shown in the following figure.

To connect the 4-pin fan power connector, simply plug it into the 4-pin connector marked JP46 on the system board.

+12V

GND

JP43

+5V         +12V

JP46

## External Battery Connector

The 4-pin external battery connector is marked JP35 on the system board.  This is used to connect an external battery in case your system board does not have an onboard battery or the 146818 RTC/battery.

To connect an external battery, simply plug the battery connector into JP35.  Make sure that the positive end of the connector corresponds to pin 1.  See the illustration below.

JP35

1

1

# Installation

The baby-AT size of the AP41 system board easily fits most housings.  It has mounting holes that conform to the standard system housing.

Some housings may differ slightly in design, requiring additional steps to install the board.  Read the documentation that comes with the housing.

|  | Make sure that you have already installed the system board components like the CPU and memory, and have set the appropriate jumpers before you proceed. |
| --- | --- |

## Installing the System Board

1.	Open the system housing.

2.	Use at least two screws that come with the housing to secure the board.

3.	Attach the power supply cables to the power connector and the front panel connectors to the multifunction connector.  See the section Connectors.

4.	Install any additional components that you have not yet installed.

## Installing Expansion Boards

Install any expansion boards after you have installed the system board into the housing.

Follow these steps to install an expansion board.

1.	Remove the bracket opposite the slot that you want to use.  Save the cover for future use.  Save the screw to secure the expansion board.

2.	Remove the board from its protective packaging.

3.	Gently insert the golden edge of the board into the slot until it fits into place.

Golden Edge

PCI Slot

Golden Edge

ISA Slot

4.	Secure the board bracket with the screw.