# AMI BIOS Setup Main Menu

The AMI BIOS Setup Main Menu appears below.  Press c to enter the system menu.

The AMI BIOS is in Windows form.  You can use either the keyboard or a mouse to move between the items.  To select among the Setup groups, use v to highlight the selected group or simply click on the icon of the selected Setup menu.

To select among the options, you can either use the arrow keys to move the highlight bar or simply click on the icon of the desired option.

After selecting, press e or double-click on the icon to open the menu.

# Standard CMOS Setup

Highlight Setup using v or simply click on the Setup icon.  Select Standard to input configuration values such as the date, time, and disk types.  The Standard CMOS Setup pop-up window appears below:

## Date/Time

To set the date and time, highlight Date/Time and press e or double-click on the Date/Time icon.  The following screen appears:

Use the arrow keys to move among the items.  Press + and - or click the + and - icons to set the current date and time.  Close the window by pressing e or double-clicking the Control menu box in the upper-left corner of the window.

## Floppy Drives A and B

To configure the floppy drive, select Floppy A.  The following values appear on the screen:

After selecting the proper setting, press e or double-click the Control menu box to close the window.

Select Floppy B and follow the same procedure to configure the second floppy drive, if present.

## Hard Disk Drives

Select Master Disk to configure the first hard disk.  The following values appear on the screen:

Press } to move to the next page.  Use w or y to highlight the selected parameter.  If you cannot find your hard disk drive type on the list, select User and enter the disk parameters.  You can also select Utility Setup.  This automatically configures your hard disk.  Refer to the section Utility Setup for more information.

Select ESDI or SCSI, depending the device installed.

If you have two hard disks installed, select Slave Disk and follow the same procedure to configure the second hard disk.

# Advanced CMOS Setup

The window below appears if you select Advanced from the main menu.

The screen above does not show all the parameters of the Advanced Configuration menu.  Use w or y to highlight the desired parameter.  Press } to view the rest of the parameters.  The following screens appear:

## Typematic Rate (Chars./Sec.)

This parameter determines the typematic rate.  The typematic rate settings are 15, 20, 30, and Disabled.  The default setting is 30.  Set it to Disabled to disregard the rate setting.

## System Keyboard

Set this parameter to Present if there is a keyboard connected to the system.  However, some servers may not have keyboards.   Select Absent if there is no keyboard present.

## Primary Display

This function detects the type of VGA in use.  The settings are VGA/EGA, CGA 40 x 25, CGA 80 x 25, Mono, and Absent.  The default setting is VGA/EGA.

## Above 1 MB Memory Test

This parameter allows your system to check all available memory.  Therefore, setting this parameter to Enabled slows down the power-on self-test.  The default setting is Disabled.

## Memory Test Tick Sound

Enabling this parameter lets you hear the tick sound during the memory test.  Disable the parameter to bypass the function.

## Hit “Del” Message Display

Disabling this option prevents the Hit <Del> if you want Setup message from appearing when the system boots.  The settings are Enabled or Disabled.

## Extended BIOS RAM Area

This function allows you to relocate the BIOS from ROM to RAM.  Relocating to RAM enhances system performance as information access is faster than ROM.  The parameter settings are 0:300 and DOS 1K.  The default address is 0:300.

## Wait for F1 If Any Error

When enabled, the BIOS waits for the end user to press l before continuing.  If disabled, the BIOS continues the boot process without waiting for l to be pressed.

## System Boot-up Num Lock

Setting this parameter to On enables the numeric function of the numeric keypad.  Set this parameter to Off to disregard the function.  Disabling the numeric function allows you to use the numeric keypad for cursor control.  The default setting is On.

## Numeric Processor Test

This parameter is non-configurable and auto-detected by the system.  If the CPU in use is a DX CPU, this parameter is automatically set to Enabled.  However, for the SX CPU, the setting is changed to Disabled.  This is because the SX CPU does not have an integral numeric coprocessor.

## Floppy Drive Seek at Boot

When enabled, the BIOS detects whether there is a floppy disk drive installed.  Disable the parameter to bypass the function.

## Floppy Drive Swapping

This parameter allows you to swap floppy drives.  For example, if you have two floppy drives (A and B), you can assign the first drive as drive B and the second drive as drive A or vice-versa.  Disable the parameter to bypass the function.  The default is Disabled.

## System Boot-up Sequence

The settings are C:,A: and A:,C: to specify the system search sequence.  The default setting is A:,C:.

## System Boot-up CPU Speed

Set the system speed to High or Low with this parameter.  High is the default setting.

## Turbo Switch Function

This parameter lets you enable or disable the turbo switch function.  The default setting is Enabled.

## Password Checking

The settings are Setup and Always.  The Setup setting allows the system to boot and use the password to protect the Setup Utility Configuration settings from being tampered with.  The Always setting requires you to enter the password everytime you boot the system.  The default setting is Setup.

## CPU Selection

The available settings for this parameter are Auto, Intel, AMD and Cyrix.  Check your CPU before setting this parameter.  Otherwise, select Auto to automatically configure the CPU.  The default setting is Auto.

## Cache

### External Cache

This parameter lets you enable or disable the secondary cache.  The default setting is Enabled.  Leave it enabled unless you are troubleshooting a problem.

### External Cache Mode

The external cache can be set either to Write-back or Write-through modes.  The Write-back mode is faster than the Write-through mode.  The default is Write-back.

### Cache Tag Width

This parameter lets you set the cache tag width.  The available settings are 8-bit and 7-bit.  The default setting is 8-bit.

### Internal Cache

This parameter lets you enable or disable the internal cache.  The default setting is Enabled.  Leave it enabled unless you are troubleshooting a problem.

### Internal Cache Mode

The internal cache is normally set to Write-through.  However, if you are using a Cyrix DX4 or an Intel P24D CPU, you must change the setting to Write-back for better performance.

### CPU Burst Write

This parameter enables or disables the CPU cache burst-write mode for M7, P24D and P24T.  The default setting is Disabled.

## Shadow C800, D000, D800, 32K

This function is for shadowing other expansion card ROMs.  The default setting for these areas is Disabled.  If you have other expansion cards with ROMs on them, you need to know the specific addresses that the ROMs use to shadow them.  Enable all the ROM Shadow settings if you don't know the addresses.  This ensures the shadowing of any present ROMs.

|  | The F000 and E000 addresses are exclusively shadowed for BIOS. |
| --- | --- |

## Video Shadow

The default setting for this parameter is Enabled.  This copies the video display card BIOS into the system DRAM to improve system performance.

## Main BIOS Cacheable

The default setting for this parameter is Enabled.  This enhances the system performance.  Disabling the parameter prevents the system BIOS from being cached.

## Cacheable C800, D000, D800, 32 K

The default setting for these parameters is Enabled.  This enhances the system performance.  The Disabled setting prevents these addresses from being cached.

## Video Shadow Cacheable

The default setting for this parameter is Enabled.  This copies the video ROM area to RAM and therefore, enhances the system performance.  Disabling the parameter prevents the video BIOS from being cached.

## Secondary IDE Drives Present

This parameter allows you to connect a secondary IDE device to your system.  The default setting is None.

## Primary 1st IDE Block Mode

This function enhances the performance of the primary IDE hard disk connected to your IDE 1 connector.  This parameter is normally set to Disabled.  If enabled, it allows data transfer in block (multiple sectors) by increasing the data transfer rate.  Disable the parameter if your hard disk does not support this feature.

## Primary 2nd IDE Block Mode

This function enhances the performance of the secondary IDE hard disk connected to your IDE 1 connector.  This parameter is normally set to Disabled.  If enabled, it allows data transfer in block (multiple sectors) by increasing the data transfer rate.  Disable the parameter if your hard disk does not support this feature.

## Secondary 1st IDE Block Mode

This function enhances the performance of the primary IDE hard disk connected to your IDE 2 connector.  This parameter is normally set to Disabled.  If enabled, it allows data transfer in block (multiple sectors) by increasing the data transfer rate.  Disable the parameter if your hard disk does not support this feature.

## Secondary 2nd IDE Block Mode

This function enhances the performance of the secondary IDE hard disk connected to your IDE 2 connector.  This parameter is normally set to Disabled.  If enabled, it allows data transfer in block (multiple sectors) by increasing the data transfer rate.  Disable the parameter if your hard disk does not support this feature.

### LBA Mode

This enhanced IDE feature allows you to use a hard disk with a capacity greater than 528 MB.  This is made possible through the Logical Block Address (LBA) mode translation.  Set this parameter to Enabled if your hard disk supports this feature.  The default setting is Disabled.

### 32-bit Transfer

Normally, this parameter is set to Disabled.  Enabling this function improves disk performance by increasing the data transfer rate from 16-bit to 32-bit.

## Chipset Setup Mode

This function allows you to change the Chipset Setup DRAM and SRAM parameters according to the end-user type.  The available settings are End-user and Engineer.  We recommend that you set this parameter to End-user.  See the following section for more details about the Chipset Features Setup.

# Chipset Features Setup

The Chipset Features Setup controls the board's chipset settings.  The controls for this menu are the same as for the previous screen.

The Chipset Setup DRAM and SRAM parameters differ depending on the Chipset Setup Mode setting in the Advanced CMOS Setup.  The following screens appear if you select the Chipset option from the Setup menu and if the Chipset Setup Mode parameter setting is End-user:

The screen below appears if your Chipset Setup Mode parameter setting is Engineer.  Take note of the new parameters that replaced the Memory Speed (DRAM) and the Cache Speed (SRAM) parameters.

See screens 2, 3 and 4 for the rest of the parameters.

|  | This manual describes only the End-user setting parameters. |
| --- | --- |

## Memory Speed (DRAM)

This parameter lets you set the DRAM speed.  The supported DRAM speeds are 70 ns and 60 ns.  The default setting is 70 ns.

## Cache Speed (SRAM)

This parameter lets you set the SRAM cache speed.  The supported DRAM speeds are 15 ns and 20 ns.  The default setting is 20 ns.

## CPU-to-PCI Memory Post Write Buffer

Enable this parameter to control the posting of the CPU-to-memory write data in the posting buffers.  Disable the parameter to deactivate the buffering function.

## CPU-to-PCI Memory Burst Write

Enabling this parameter allows the translation of the host cycles into memory-burst cycles and controls the memory burst-write cycles.  Disabling the function deactivates the memory burst writes.

## DRAM Type (Refresh)

This parameter lets you select the DRAM type that you wish to support.  The options are Normal and Low Power.  The low-power DRAMs are special-type DRAMs.  The default setting is Normal.

### Motherboard IDE

To take advantage of the IDE features of the motherboard, you need to set this parameter to Enabled.  Disable the parameter to bypass the feature.

### Primary Master PIO Mode

If this enhanced IDE parameter is set to Auto, it automatically detects the PIO mode (mode 0, 1, 2, 3, 4) and sets the interface timing of the master drive connected to the primary IDE connector.  If you know your HDD PIO mode, you can set this parameter manually.  The default setting is Auto.

|  | We recommend that you set this parameter to Auto. If you prefer to set the PIO mode manually, make sure that you know your HDD type. |
| --- | --- |

### Primary Slave PIO Mode

If this enhanced IDE parameter is set to Auto, it automatically detects the PIO mode (mode 0, 1, 2, 3, 4) and sets the interface timing of the slave drive connected to the primary IDE connector.  If you know your HDD PIO mode, you can set this parameter manually.  The default setting is Auto.

### Secondary Master PIO Mode

If this enhanced IDE parameter is set to Auto, it automatically detects the PIO mode (mode 0, 1, 2, 3, 4) and sets the interface timing of the master drive connected to the secondary IDE connector.  If you know your HDD PIO mode, you can set this parameter manually.  The default setting is Auto.

### Secondary Slave PIO Mode

If this enhanced IDE parameter is set to Auto, it automatically detects the PIO mode (mode 0, 1, 2, 3, 4) and sets the interface timing of the slave drive connected to the secondary IDE connector.  If you know your HDD PIO mode, you can set this parameter manually.  The default setting is Auto.

|  | For the PIO Mode parameters, we recommend that you select Auto. This automatically detects the correct PIO mode of your HDD and prevents configuration error caused by wrong data entry. |
| --- | --- |

### PCI-IDE Non-compliant Card

This parameter allows you to select your PCI-IDE card slot.  The available selections are Slot 1, Slot 2, Slot 3 and Absent.  Select Absent if you do not have any PCI-IDE cards installed.

### Primary PCI-IDE Card Uses

This parameter lets you assign an INT for the IDE device connected to your primary IDE connector.  The settings are INT A, INT B, INT C, and INT D.  If you do not have any PCI-IDE cards installed in your system, this parameter becomes non-configurable.

### Secondary PCI-IDE Card Uses

This parameter lets you assign an INT for the IDE device connected to your primary IDE connector.  The settings are INT A, INT B, INT C, and INT D.  If you do not have any PCI-IDE cards installed in your system, this parameter becomes non-configurable.

### PCI IRQ Allocation

The settings for this parameter are Auto and Manual.  If you select Auto, it automatically sets the IRQs and INTs for the PCI devices installed in your system.  If you want to configure your PCI device parameters, select Manual.

### PCI Slot 1/2/3 INT Connect

These parameters are configurable only if the PCI IRQ Allocation parameter is set to Manual.  This allows you to set the INT(s) for the PCI devices.  The selections are INT A, INT B, INT C and INT D.  Make sure that you assign a different INT for each PCI device function.

#### IRQ Connect

This parameter is configurable only if the PCI IRQ Allocation parameter is set to Manual.  This allows you to set an IRQ for each PCI device installed in your system.  The selections are IRQ 3~7, 9~12, 14, and 15.  Make sure that you assign a different IRQ for each PCI device to avoid conflict.

#### IRQ Trigger

This parameter lets you set when to trigger the IRQ, i.e., either at the Edge or Level of the signal.

### ISA IRQ Manual Over Ride Parameters 
(IRQ 3, 4, 5, 7, 9, 10, 11, 12, 14 and 15)

These lines allow you to assign the available IRQs to either ISA or PCI devices.

# Power Management Setup

The screen below appears if you select Power Management from the main menu.  To select, highlight Power Mgmt and press e or double-click on the Power Management icon.

## BIOS Power Management Mode

The settings for this parameter depend on the CPU type in use.  If the CPU in use has SMM feature, the available settings for this parameter are Enabled and Disabled.  Select Enabled to take advantage if the power-saving feature.

If the CPU in use is a non-SMM type, you need to assign an IRQ for the power-saving function.  The selections are IRQ 11, IRQ 12, IRQ 15 and Disabled.  Disable the parameter to bypass the feature.

The default setting for this parameter is Disabled.  When disabled, only the screen above appears on the monitor.  When enabled, it allows you to configure the power-saving feature.  The following screens appear if you enable the BIOS Power Management parameter.  Use w or y to highlight the desired parameter.  Press } to view the remaining parameters.

|  | If you are using OS/2, Windows NT, Novell or UNIX operating systems, you must disable the power management function. |
| --- | --- |

## Advanced Power Management

Set this parameter to Enabled to take full advantage of the power-saving feature.  Disable the parameter to bypass the feature.

## Doze Timer

This function reduces the CPU clock.  Full power returns when an activity takes place.  The available settings are 10 sec, 20 sec, 30 sec, 45 sec, 1 Min, 2 Min, 5 Min, 10 Min, 15 Min, 20 Min, 30 Min, 40 Min and Disabled.

## Standby Timer

This function reduces the CPU clock and suspends the video signal.  This activates the power-saving features on your monitor.  Full power returns when an event is detected.  The available settings are 10 sec, 20 sec, 30 sec, 45 sec, 1 Min, 2 Min, 5 Min, 10 Min, 15 Min, 20 Min, 30 Min, 40 Min and Disabled.

## Suspend Timer

The suspend timer stops the CPU internal clock and suspends the video signal.  Full power returns when activity is detected.  The settings are 10 sec, 20 sec, 30 sec, 45 sec, 1 Min, 2 Min, 5 Min, 10 Min, 15 Min, 20 Min, 30 Min, 40 Min and Disabled.  This feature conserves the most energy.

## Wake-up Events

Enabling these parameters allow your system to monitor the I/O activities.  Any activity detected resets the power-management timers and resumes the system to normal mode.

|  | You must enable at least one IRQ activity. Otherwise, the system stays in suspend mode.. |
| --- | --- |

## Monitor Type

This option lets you set the monitor type.  The selections are Green  and Non-Green.

## Display Off After

This parameter lets you set the display-off mode.  The selections are Doze, Suspend, Standby and Disabled.  Once the system enters the specified mode, the monitor screen turns blank.  Monitors with green function allows you to save more power since the system shuts down the VH sync. once it enters the specified power-saving mode.

## HDD Off After

This option lets you specify which mode to "spin down" your hard disk.  The disk returns to full speed once the system returns to normal mode.  The settings are Doze, Suspend, Standby and Disabled.

## Deturbo After

This function lets you specify the mode when to release the system from turbo mode.  The settings are Doze, Suspend, Standby and Disabled.

## Break Switch

This option lets you enable or disable the break switch.  Enabling this parameter allows the system to enter suspend mode and return to normal mode by simply pressing the break switch.  However, enabling the break switch disables your turbo functions.  Disable the parameter to take advantage of the turbo functions.

# Peripheral Setup

This screen appears if you select Peripherals or double-click on the Peripheral Setup icon from the Setup menu.  The Peripheral Setup screen allows you to set up your system peripherals.

## Programming Mode

The settings for this option are Auto and Manual.  The Manual setting allows you to set up the screen items manually.  The Auto setting sets up all the items automatically except for the Parallel Port Mode parameter.

## Onboard FDC

Enabling this function allows you to use the onboard floppy disk controller (FDC).  The default setting is Enabled.

## Serial Port 1

This parameter allows you to set the base address of serial port 1.  The available settings are 3F8H, 2F8H, 3E8H, 2E8H and Disabled.  The default setting is 3F8H.

## Serial Port 2

This parameter allows you to set the base address of serial port 2.  The available settings are 3F8H, 2F8H, 3E8H, 2E8H and Disabled.  The default setting is 2F8H.

## Parallel Port

This parameter allows you to set the base address of the parallel port.  The available settings are 3BCH, 378H, 278H and Disabled. The default setting is 378H.

## Parallel Port Mode

This option lets you set the parallel port mode.  The settings are Normal, SPP, EPP & SPP, ECP and ECP & EPP.  The default setting is Normal.

## Parallel Port Extended Mode

This parameter is configurable only if the Parallel Port Mode parameter is set to Extended.  This enables the extended mode functions of the parallel port.  The selections are EPP, ECP, and ECP & EPP.  The EPP setting allows the parallel port to operate in bidirectional mode at maximum speed.  The ECP setting allows the parallel port to operate in bidirectional mode and at a speed higher than maximum data transfer rate.  The ECP & EPP setting enables the parallel port to support both features.  The default setting is ECP.

# Utility Setup

## Detect Master and Detect Slave

These functions allow your system to automatically configure the hard disk.  This screen appears if you select Detect Master or Detect Slave option.

After a few seconds, the screen below appears showing your disk parameters.  Select Yes to accept the values.

## Color Set

This pop-up window appears if you select Color Set from the Utility Setup menu.

Color Set lets you specify the color of your windows background.  The selections are LCD, Army, Pastel, and Sky.

# Default Setup

Select this option to automatically set your system configuration parameters.  To select, highlight Default and press e.

## Original

This option loads the values that you saved before shutting off the system.  The following prompt appears if you select Original from the Default Setup menu.  Choose Yes to load the original values.

## Optimal

Choose this option and the BIOS configures the system using the best-case values to optimize system performance.  However, these values may not be applicable to your system.  If your system does not boot after choosing this setting, reconfigure it using the Fail-safe settings.  Refer to the following section.

The screen below appears if you choose Optimal from the Default Setup menu.  Select Yes to load the optimum values.

## Fail-safe

Choose this option and the BIOS automatically configures the system using the most stable settings.  These settings are not necessarily the best settings for system performance, but they are safe and stable enough to guarantee that your system will boot.  This is useful if you are having problems with your current system configuration and need to determine the cause.

A prompt appears if you choose Fail-safe from the Default Setup menu.  Select Yes to load the fail-safe values.

# Security Setup

## Password

The system password prevents unauthorized use of your computer.  If you enabled the password feature, it is impossible to boot the computer without entering the password.

To set a password, highlight Password or simply double-click the Password icon.  The following screen appears:

Your password can consist of up to six characters.  The password does not appear on the screen.  WinBIOS prompts you to retype the password.  The following screen appears.

If you forget your password, you must clear the CMOS RAM and reconfigure the system.

To disable the password, press e when prompted for your password.  Press e again when prompted to retype the password.

## Anti-virus

Set this parameter to Enabled to protect the boot sector and partition table of your hard disk from virus intrusion.  Set it to Disabled to bypass the feature.

A prompt appears when you select Anti-virus from the Security Setup menu:

Select Enabled and the screen below appears:

# Exit Setup

To exit Setup, you can either double-click on the Control menu box or simply press ^.  A dialog box appears on the screen.

If you select Save Changes and Exit, BIOS automatically saves all CMOS values before leaving Setup.  Select Do Not Save Changes and Exit to exit Setup without saving the CMOS values.  Select Continue to return to Setup if you want to reconfigure your system.

# NCR SCSI BIOS and Drivers

The NCR 53C810 SCSI BIOS resides on the same flash memory chip as the system BIOS.  To use the onboard NCR BIOS, you need to install an NCR 53C810 SCSI controller card in your system.

All SCSI devices that you install in your system require software drivers.  The NCR SCSI BIOS directly supports SCSI hard disks under DOS, Windows and OS/2.  It also uses DOS-format and SCO UNIX-format support floppy disk device drivers that come with the NCR 53C810 SCSI controller card.  The DOS-format device drivers are for SCSI devices used with DOS, Windows NT, Novell NetWare and OS/2.  The SCO UNIX-format device drivers are for SCSI devices used with SCO UNIX.  These drivers offer higher performance than the direct BIOS support.

To use the device drivers, you must install them in your system hard disk drive and add them to your system configuration files.  For detailed installation instructions, see the README files that come with the drivers.

|  | The system board also supports the AMI Flash Memory Writer Utility that allows you to upgrade the system BIOS. For more information on this utility, contact your local distributor. |
| --- | --- |