This appendix tells how to install the SiS supported drivers using the installation utility.

## DOS, Windows, WFW (Workgroup) and NetWare

To install DOS, Windows, WFW (Workgroup) and NetWare drivers, simply run INSTALL.EXE.

## Windows NT

- From the Windows NT Setup Options menu, select Add/Remove SCSI Adapters.

- Select the Add button in the SCSI Adapter Setup.

- From the Adapter: list dialog box, select Others (Requires a disk from a hardware manufacturer).

- The Insert Diskette dialog box appears.  Insert the SiS driver into drive A and type A:WINNT.

- The screen displays the Select OEM Option dialog box.  Select SiS 32-bit Local-bus IDE Adapter and click on OK.

- The Select SCSI Adapter Option dialog box appears.  Click on Install.  This installs the drivers to the hard disk.

- Installation is successful if the SCSI Adpater Setup dialog box reappears and SiS 32-bit Local-bus IDE Adapter appears on the list of installed drivers.

- Reboot the system.

## OS/2

1.	From the OS/2 desktop, open the OS/2 system.

2.	Open System Setup.

3.	Select Device Driver Install.

4.	Insert the driver diskette into drive A or B.

5.	Change the source directory to A:\OS2 or B:\OS2.

6.	Select Install.

7.	Click on OK.

## SCO UNIX

To install the SCO UNIX drivers, refer to the README.SCO file in the SCO directory.

### Installing SCO UNIX 3.2.X

1.	Install SCO UNIX 3.2.4.X or SCO Open Desktop 2.X using the default SCO UNIX IDE drivers.

2.	Boot your SCO UNIX system and bring it into single-user mode.

3.	Copy the SiS496.TAR file to your /TMP directory using the doscp command.  For example, if you are using drive A, type:

DOSCP A:SIS496.TAR/TMP

4.	Prepare a SCO UNIX formatted diskette.

5.	From the root directory, type the following commands:

- MKDIR/SIS

- CD/SOS

- TAR XVF/TMP/SIS486.TAR

6.	Insert the diskette into the drive and enter the following command:

TAR CVF/DEV/(diskette drive 0 device name)

Your diskette drive 0 device name may be:

- RFD096DS15		5.25 DSHD

- RFD0135DS18		3.5 DSHD

- RFD048DS9		5.25 DSHD

- RFD0135DS9		3.5 DSHD

7.	Start the installation by typing CUSTOM.

8.	Follow the screen instructions to install the SiS IDE driver.  A new kernel replaces the original kernel and the original kernel is renamed as UNIX.OLD.

9.	Reboot your system for the new kernel to take effect.  The new kernel initializes the chip to the new timing setting.