This chapter tells how to set jumpers, upgrade system memory, add expansion boards, and install the system board.

Install the CPU, memory, and set the jumpers before you install the system board inside a system housing.  You may add the other components after installing the board.  Read this chapter to learn about the components before you install them.

# ESD Precautions

Electrostatic discharge (ESD) can damage your CPU, disk drives, expansion boards, and other components.  Always observe the following precautions before you install a system component.

1.	Do not remove a component from its protective packaging until you are ready to install it.

2.	Wear a wrist grounding strap and attach it to a metal part of the system unit before handling components.  If a wrist strap is not available, maintain contact with the system unit throughout any procedure requiring ESD protection.

# Installing a Microprocessor

The motherboard has a zero-insertion force microprocessor socket that allows you to install a Pentium CPU without using any tools.

Follow these steps to install a Pentium CPU in a ZIF-type upgrade socket:

|  | Make sure that the system power is OFF before installing any component. |
| --- | --- |

1.	Attach the heatsink and the fan to the CPU.

2.	Pull up the socket lever.

3.	Insert the CPU with the attached heatsink and fan.  Make sure that pin 1 of the CPU aligns with the hole 1 of the socket.  The notched corner on the CPU indicates pin 1.

4.	Pull down the socket lever to lock the CPU into the socket.

Step 2

Step 3

Step 4

5.	Plug the fan cable into the fan connector.  The board has one 2-pin and one 4-pin fan connector.  See the section Connectors for more details on fan connectors.

6.	Set the jumpers accordingly.  See the following sections for the correct jumper settings.

# Upgrading the Microprocessor

Follow these steps to upgrade the Pentium CPU from 75 MHz to 90,100, 120, 133, 150 or 166 MHz:

1.	Turn off the system power.

2.	Pull up the socket lever.

3.	Remove the installed CPU.

4.	Install the upgrade CPU.  Refer to the section Installing a Microprocessor for instructions on how to install a Pentium CPU.

# Jumper Settings

You have to change the jumper settings when you reconfigure the system.  This section tells how to reset the jumpers.  The figure below shows the jumper locations.

|  | Optional jumpers: JP6 - for PS/2 mouse function JP14, JP16 - for 3.3V or 3.3V/5V Mix Mode SRAM |
| --- | --- |

Set a jumper switch as follows:

- To close a jumper switch, insert the plastic jumper cap over two pins of a jumper.

- To open a jumper switch, remove the jumper cap.

The following conventions are used to represent the proper jumper settings.

Open

Closed  (1-2 position)

## Selecting the CPU Type

The jumpers JP9, JP10 and JP11 let you select the CPU type.  The available settings are as follows:

CPU TYPE         JP9        JP10      JP11

P54C-75

P54C-90

P54C-100

P54C/CS/CQS-120

P54C/CS/CQS-133

P54CS/CQS-150

P54CS/CQS-166

## Selecting the SRAM Type (Optional)

The system board supports both 3.3V and 3.3V/5V Mix Mode SRAMs. Set the onboard switches marked as JP14 and JP16 according to the SRAM type.  See the figure below.

JP14JP16

3.3V SRAM

3.3V/5V Mix Mode SRAM

## Setting the Cache Size

The second-level cache is upgradable from 256 KB to 512 KB.  See the table below for the possible cache configurations.

| Cache Size | TAG SRAM (U25) | Data SRAM (U28, U30, U31, U32, U34, U35, U36, U37) |
| --- | --- | --- |
| 256 KB | 16 Kb x 8 x 1 pc or 32 Kb x 8 x 1 pc | 32 Kb x 8 x 8 pcs |
| 512 KB | 16 Kb x 8 x 1 pc or 32 Kb x 8 x 1 pc | 64 Kb x 8 x 8 pcs |

Follow these steps to install SRAMs:

1.	Locate the cache sockets on the system board.  See the section Board Layout.

2.	Insert the SRAM chip into the socket.  Align the straight edge of the chip with the straight edge of the socket.  Also, make sure that the cut edge of the chip corresponds to the cut edge of the cache socket.  See the figure below.

|  | Press the SRAM chip gently but firmly into place. Be careful not to bend the pins. |
| --- | --- |

You must reset the 3-pin jumper JP12 when you upgrade your cache.  See the figure below for the correct settings.

CACHE SIZE         JP12

256 KB

512 KB

## Selecting the Flash ROM Type

Set the 3-pin jumper JP13 according to the Flash ROM type.  If you use 5V Flash ROM, then you must close pins 1-2 of JP13.  For a 12V Flash ROM, the required setting is 2-3.  The default setting is 1-2.

Flash ROMJP13

5V

12V

## Clearing the CMOS

The 3-pin jumper JP15 is used to clear the values in the CMOS.  You need to clear the CMOS if you forget your system password.  To do this, shut off the system power and short pins 2-3 of JP15 for a few seconds.  Then set the jumper to the normal setting by shorting pins 1-2 with a jumper cap.  Enter Setup to specify a new password.

JP15

Default

Clear CMOS

## Enabling the PS/2 Mouse (Optional)

The 2-pin jumper JP6 lets you enable or disable the PS/2 mouse.  To enable, simply close both pins of the jumper.  To disable, leave the jumper open.

JP6

Enabled

Disabled

## Enabling the Onboard Super I/O Controller

The onboard super I/O controller is SMC37C665GT.  If you want to enable or disable the onboard I/O controller then you must reset the 3-pin jumper marked JP5 on the system board.

JP5

Enabled

Disabled

## Selecting the ECP DMA Channel

The 3-pin jumpers JP3 and JP4 let you select the DMA channel for ECP function.  The available channels are DMA 3 and DMA 1.

JP3         JP4

DMA 3

DMA 1

# Memory Configuration

The system board supports a maximum memory of 128 MB.   The six 72-pin SIMM sockets accommodate 4- and 16-MB single-density SIMMs, and 8- and 32-MB double-density SIMMs, with or without the Extended Data Output (EDO)function.  The EDO feature expands data transfer cycle, thus improves memory performance.  All SIMMs support a DRAM speed of 70/60 ns (or less).  Refer to section Board Layout and see the figure for the location of the SIMMs.

The table below lists the SIMM types and their corresponding capacities.

| SIMM Type | Capacity |
| --- | --- |
| 1 Mb x 32/36 | 4 MB |
| 2 Mb x 32/36 | 8 MB |
| 4 Mb x 32/36 | 16 MB |
| 8 Mb x 32/36 | 32 MB |

The following are the possible SIMM configurations.

| Total Memory | SIMM 0/1 | SIMM 2/3 | SIMM 4/5 |
| --- | --- | --- | --- |
| 8 MB | 4 MB x 2 |  |  |
| 16 MB | 4 MB x 2 | 4 MB x 2 |  |
| 16 MB | 8 MB x 2 |  |  |
| 24 MB | 4 MB x 2 | 4 MB x 2 | 4 MB x 2 |
| 24 MB | 8 MB x 2 | 4 MB x 2 |  |
| 32 MB | 8 MB x 2 | 4 MB x 2 | 4 MB x 2 |
| 32 MB | 8 MB x 2 | 8 MB x 2 |  |
| 32 MB | 16 MB x 2 |  |  |
| 40 MB | 8 MB x 2 | 8 MB x 2 | 4 MB x 2 |
| 40 MB | 16 MB x 2 | 4 MB x 2 |  |
| 48 MB | 16 MB x 2 | 4 MB x 2 | 4 MB x 2 |
| 48 MB | 16 MB x 2 | 8 MB x 2 |  |
| 56 MB | 16 MB x 2 | 8 MB x 2 | 4 MB x 2 |
| 64 MB | 8 MB x 2 | 8 MB x 2 |  |
| 64 MB | 16 MB x 2 | 16 MB x 2 |  |
| 64 MB | 32 MB x 2 |  |  |
| 72 MB | 32 MB x 2 | 4 MB x 2 |  |
| 80 MB | 32 MB x 2 | 4 MB x 2 | 4 MB x 2 |
| 80 MB | 32 MB x 2 | 8 MB x 2 |  |
| 88 MB | 32 MB x 2 | 8 MB x 2 | 4 MB x 2 |
| 96 MB | 32 MB x 2 | 16 MB x 2 |  |
| 104 MB | 32 MB x 2 | 16 MB x 2 | 4 MB x 2 |
| 112 MB | 32 MB x 2 | 8 MB x 2 | 16 MB x 2 |
| 128 MB | 32 MB x 2 | 16 MB x 2 | 16 MB x 2 |
| 128 MB | 32 MB x 2 | 32 MB x 2 |  |

## Installing a SIMM

|  | Observe the ESD precautions when installing components. |
| --- | --- |

Follow these steps to install a SIMM:

1.	Slip a SIMM at a 45o angle into a socket with the component side facing down.  Always install SIMMs beginning with Bank 0.

|  | Be careful when inserting or removing SIMMs. Forcing a SIMM in or out of a socket can damage the socket or the SIMM (or both). |
| --- | --- |

2.	Gently push the SIMM up until the pegs of the socket slip into the holes on the SIMM and the holding clips lock the SIMM into a vertical position.

|  | The SIMM should be at a 90o angle when installed. |
| --- | --- |

## Removing a SIMM

1.	Press the holding clips on both sides of the SIMM outward to release it.

2.	Press the SIMM downward to about a 45o angle.

3.	Gently pull the SIMM out of the socket.

# Connectors

## Multifunction Connector

This 20-pin connector is marked CN14 on the system board.  It supports a number of system functions:  green mode LED, break switch, keylock, and speaker.  Attach the front panel connectors to the corresponding pins as in the illustration below.

Speaker          Power LED   Keylock

Some housings have a five-pin connector for the keylock and power LED.  See the following illustration.

Speaker            Keylock and Power LED

Other housings may have a 12-pin connector.  If your housing has this type of connector, plug it into CN14 as shown in the following figure.  Make sure that the red wire of the connector connects to pin 11.

## Break Switch

The break switch is originally the turbo switch.  It gives the user the option to directly enter the system suspend mode by setting the switch to the on position.  To set, simply press the switch.  Make sure that the break switch is at the off position before you set it to the on position.  To set it to the off position, simply press the switch to release it from the on position.

## Keyboard Connector

The keyboard connector is a 5-pin, AT-compatible connector.  The following figure shows how to connect a keyboard.

|  | The PS/2 keyboard connector is optional. |
| --- | --- |

## Power Connector

A standard power supply has two cables with six wires each.  Attach these cables to the power connector on the board in such a way that all the black wires are in the center.  The power connector is marked CN4 on the system board.

## Fan Connectors

The board has one 4-pin and one 2-pin fan connector.  The 4-pin fan connector is marked CN11 on the system board and the 2-pin is marked CN12.  Figure below shows the pin configuration of each connector.

4-PIN FAN CONNECTOR (CN11)

2-PIN FAN CONNECTOR (CN12)

+12V

+5V

# Installation

The full baby-AT size of the AP5C system board easily fits most housings.  It has mounting holes that conform to the standard system housing.

Some housings may differ slightly in design, requiring additional steps to install the board.  Read the documentation that comes with the housing.

|  | Make sure that you have already installed the system board components like the CPU and memory, and have set the appropriate jumpers before you proceed. |
| --- | --- |

## Installing the System Board

1.	Open the system housing.

2.	Use at least two screws that come with the housing to secure the board.

3.	Attach the power supply cables to the power connector and the front panel connectors to the multifunction connector.  See the section Connectors.

4.	Install any additional components that you have not yet installed.

## Installing Expansion Boards

Install any expansion boards after you have installed the system board into the housing.

Follow these steps to install an expansion board.

1.	Remove the bracket opposite the slot that you want to use.  Save the cover for future use.  Save the screw to secure the expansion board.

2.	Remove the board from its protective packaging.

3.	Gently insert the golden edge of the board into the slot until it fits into place.

Golden Edge

PCI Slot

Golden Edge

ISA Slot

4.	Secure the board bracket with the screw.