This chapter tells how to install the CPU and other components, set jumpers, configure system memory, install the system board, and add expansion boards.

# ESD Precautions

Electrostatic discharge (ESD) can damage your processor, disk drives, expansion boards, and other components.  Always observe the following precautions before you install a system component.

1.	Do not remove a component from its protective packaging until you are ready to install it.

2.	Wear a wrist grounding strap and attach it to a metal part of the system unit before handling components.  If a wrist strap is not available, maintain contact with the system unit throughout any procedure requiring ESD protection.

# Installing a Microprocessor

The motherboard has a zero-insertion force microprocessor socket that allows you to install a CPU without using any tools.

Follow these steps to install a CPU in a ZIF-type upgrade socket:

|  | Make sure that the system power is off before installing any component. |
| --- | --- |

1.	Pull up the socket lever.

2.	Gently insert the CPU.  Make sure that pin 1 of the CPU aligns with hole 1 of the socket.  The dot on the CPU indicates pin 1.  The topmost label indicates the frequency and the cache size that the CPU supports.

Cache size supported

CPU frequency

Hole 1

Pin 1 indicator

3.	Pull down the socket lever to lock the CPU into the socket.

4.	Attach the heatsink and fan to the installed CPU.

Step 1

Step 2

Step 3

Step 4

5.	Plug the fan cable into the fan connector on the system board.  See the section Fan Connector for details.

6.	Set the jumpers accordingly.  See the following sections for the correct jumper settings.

# Upgrading the Microprocessor

Follow these steps to upgrade the CPU:

1.	Turn off the system power.

2.	Pull up the socket lever.

3.	Remove the installed CPU.

4.	Install the upgrade CPU.  Refer to the section Installing a Microprocessor on how to install the CPU.

# Jumper Settings

You have to change the jumper settings when you reconfigure your system.  This section tells how to reset the jumpers.  The figure below shows the jumper locations.

## Selecting the CPU Type

Set the jumper JP12 according to the CPU type installed in your system.

JP12

Standard CPU

OverDrive CPU

(default)

## Selecting the CPU Frequency

The jumpers JP1 and JP3 allow you to select the CPU frequency that you wish to support.  See the figure below for the correct settings.

JP1             JP3

150 MHz

180 MHz

200 MHz

(default)

## Changing the Voltage Regulator Module (VRM) Voltage Setting

To set the VRM voltage, you can either do it manually or use the voltage auto-detect function of the CPU.  However, this depends on the type of the CPU installed in your system.  If your CPU does not support the voltage auto-detect function, you need to set the onboard jumper JP13 according to the figure below.

If your CPU supports the voltage auto-detect function, leave JP13 open.

| Voltage JP13 Voltage JP13 3.5V 3.4V 3.3V 3.2V 3.1V 3.0V 2.9V 2.8V 2.7V 2.6V 2.5V 2.4V 2.3V 2.2V 2.1V VID (default) (auto-detect) |  |
| --- | --- |

## Selecting the Flash ROM Type

The onboard jumper JP9 allows you to select the Flash ROM type that you wish to support.  If a 5V Flash ROM is in use, you must set JP9 to 1-2.  Reset it to 2-3 to support a 12V Flash ROM.

JP9

5V

12V

(default)

## Clearing the CMOS

You need to clear the CMOS if you forget your system password.  To do this, shut off the system power and short JP11 for a few seconds.  Then reset the jumper to normal setting by removing the jumper cap and leaving it open.  Enter Setup to specify a new password.

Normal

Clear CMOS

JP11

(default)

## Enabling the Enhanced IDE Function

The jumper JP2 lets you enable or disable the enhanced IDE function.  To enable, simply close pins 2-3 of JP2.  To disable, reset the jumper to 1-2.

Enabled

Disabled

JP2

(default)

## Enabling the Super I/O Chip

Set the jumper JP6 to 1-2 to enable the super I/O chip controller.  If you want to use a separate I/O card, disable the super I/O controller by resetting JP6 to 2-3.

Enabled

Disabled

JP6

(default)

## Selecting the DMA Channel

The jumpers JP5 and JP7 let you select the DMA channel for ECP function.  The channel selections are DMA 1 and DMA 3.  The following figure shows the correct jumper settings for each channel.

DMA 1

DMA 3

JP5              JP7

(default)

## Enabling the PS/2 Mouse (optional)

The system board may or may not have PS/2 keyboard and mouse connectors.  If your board comes with a PS/2 mouse connector, you must open jumper JP10 to enable the function.  To disable, simply close the jumper with a jumper cap.

Enabled

Disabled

JP10

(default)

# Memory Configuration

The system memory is expandable to 256 MB.   The four 72-pin SIMM sockets onboard accommodate 2, 4, 8, 16, 32 and 64-MB SIMMs with 60, 70, or 80-ns DRAM speed.

The system board supports the special ECC function that enables the system to detect and correct errors in data.  To take advantage of this function, you must install SIMMs with parity into your system.  Installing SIMMs without parity automatically disables the function.

The table below lists the supported SIMM types and their corresponding capacity.

| SIMM Type | Capacity |
| --- | --- |
| 512 Kb x 32/36 | 2 MB |
| 1 Mb x 32/36 | 4 MB |
| 2 Mb x 32/36 | 8 MB |
| 4 Mb x 32/36 | 16 MB |
| 8 Mb x 32/36 | 32 MB |
| 16 Mb x 32/36 | 64 MB |

The system board accepts two types of memory configuration:  one-way interleave (1:1) and two-way interleave (2:1).  The one-way interleave mode requires a single-bank configuration containing the same SIMM type.  The two-way interleave mode requires a double-bank configuration containing the same SIMM type in one-bank.  See the following tables for the possible SIMM configurations:

### One-way Interleave (1:1)

| Bank 0 |  | Bank 1 |  |  | Total |  |
| --- | --- | --- | --- | --- | --- | --- |
| SIMM 0 | SIMM 1 | SIMM 2 |  | SIMM 3 | Memory |  |
| 2 MB | 2 MB |  |  |  | 4 MB |  |
| 4 MB | 4 MB |  |  |  | 8 MB |  |
| 8 MB | 8 MB |  |  |  | 16 MB |  |
| 16 MB | 16 MB |  |  |  | 32 MB |  |
| 32 MB | 32 MB |  |  |  | 64 MB |  |
| 64 MB | 64 MB |  |  |  |  | 128 MB |

### Two-way Interleave (2:1)

| Bank 0 |  | Bank 1 |  |  | Total |  |
| --- | --- | --- | --- | --- | --- | --- |
| SIMM 0 | SIMM 1 | SIMM 2 |  | SIMM 3 | Memory |  |
| 2 MB | 2 MB | 2 MB |  | 2 MB | 8 MB |  |
| 4 MB | 4 MB | 4 MB |  | 4 MB | 16 MB |  |
| 8 MB | 8 MB | 8 MB |  | 8 MB | 32 MB |  |
| 16 MB | 16 MB | 16 MB |  | 16 MB | 64 MB |  |
| 32 MB | 32 MB | 32 MB |  | 32 MB | 128 MB |  |
| 64 MB | 64 MB |  | 64 MB | 64 MB |  | 256 MB |

## Installing a SIMM

|  | Observe the ESD precautions when installing components. |
| --- | --- |

Follow these steps to install a SIMM:

1.	Slip a SIMM at a 45o angle into a socket with the component side facing down.  Always install SIMMs beginning with Bank 0.

|  | Be careful when inserting or removing SIMMs. Forcing a SIMM in or out of a socket can damage the socket or the SIMM (or both). |
| --- | --- |

2.	Gently push the SIMM up until the pegs of the socket slip into the holes on the SIMM and the holding clips lock the SIMM into a vertical position.

|  | The SIMM should be at a 90o angle when installed. |
| --- | --- |

## Removing a SIMM

1.	Press the holding clips on both sides of the SIMM outward to release it.

2.	Press the SIMM downward to about a 45o angle.

3.	Gently pull the SIMM out of the socket.

# Voltage Regulator Module

|  | You must install the CPU first before installing the VRM. See the section Installing a Microprocessor on how to install a CPU. |
| --- | --- |

The system board comes with a separate voltage regulator module that enables the system to support the processor’s operating voltage ranging from 2.1V to 3.5V.  The following sections tell how to install and remove VRM.

## Installing a VRM

|  | Observe the ESD precautions when installing components. |
| --- | --- |

Follow these steps to install a VRM:

1.	Locate the VRM connector on the system board.  See the board layout figure in Chapter 1.

2.	Push the holding clips of the connector outward.

3.	Align the VRM socket to the connector pins.  The holding clips must be facing on the rear side of the VRM.

4.	Press the VRM downward until the holding clips click into place.

## Removing a VRM

1.	Push the holding clips outward to release the VRM.

2.	Gently pull the VRM to remove it from the connector.

# Connectors

## Multifunction Connector

This 20-pin connector is marked CN8 on the system board.  It supports a number of system functions:  LED, turbo, reset, keylock, and speaker.  Attach the front panel connectors to the corresponding pins as in the illustration below.

Reset        Turbo Switch       Turbo LED

Speaker          Power LED   Keylock

Some housings have a five-pin connector for the keylock and power LED.  See the following illustration.

Reset          Turbo Switch    Turbo LED

Speaker            Keylock and Power LED

Other housings may have a 12-pin connector.  If your housing has this type of connector, plug it into CN8 as shown in the following figure.  Make sure that the red wire of the connector connects to pin 11.

Turbo Switch

## Keyboard Connector

The keyboard connector is a 5-pin, AT-compatible connector.  The following figure shows how to connect the keyboard.

|  | PS/2 keyboard connector is optional. |
| --- | --- |

## PS/2 Mouse Connector

The onboard PS/2 mouse connector enables the system to support a PS/2 mouse.  The figure below shows how to connect a PS/2 mouse bracket into the mouse connector on the system board.

## Power Connector

A standard power supply has two cables with six wires each.  Attach these cables to the power connector on the board in such a way that all the black wires are in the center.

## Fan Connector

The two-pin fan connector is marked J6 on the system board.  To connect, plug the connector to its corresponding pin as shown in the following figure.

GND

# Installation

The full baby-AT size of the AP61 system board easily fits most housings.  It has mounting holes that conform to the standard system housing.

Some housings may differ slightly in design, requiring additional steps to install the board.  Read the documentation that comes with the housing.

|  | Make sure that you have already installed the system board components like the CPU and memory, and have set the appropriate jumpers before you proceed. |
| --- | --- |

## Installing the System Board

1.	Open the system housing.

2.	Use at least two screws that come with the housing to secure the board.

3.	Attach the power supply cables to the power connector and the front panel connectors to the multifunction connector.  See the section Connectors.

4.	Install any additional components that you have not yet installed.

## Installing Expansion Boards

Install expansion boards after you have installed the system board into the housing.

Follow these steps to install an expansion board.

1.	Remove the bracket opposite the slot that you want to use.  Save the cover for future use.  Save the screw to secure the expansion board.

2.	Remove the board from its protective packaging.

3.	Gently insert the golden edge of the board into the slot until it fits.

GOLDEN EDGE

GOLDEN EDGE

ISA SLOT

PCI SLOT

4.	Secure the board bracket with the screw.