Appendix   A

Jumper and Connector Summary

CPU Type and Frequency

| CPU Freq. | JP2 | JP3+ | JP4 | JP5 | JP8 |
| --- | --- | --- | --- | --- | --- |
| Intel Pentium (P54C) |  |  |  |  |  |
| 75 MHz | 1-2, 3-4 | 3-4 | 1-2, 3-4, 5-6 | Open | 1-2, 3-4 |
| 90 MHz | 1-2, 3-4 | 3-4 | 1-2, 3-4, 5-6 | Open | 1-2 |
| 100 MHz | 1-2, 3-4 | 3-4 | 1-2, 3-4, 5-6 | Open | 3-4 |
| 120 MHz | 3-4, 5-6 | 3-4 | 1-2, 3-4, 5-6 | Open | 1-2 |
| 133 MHz | 3-4, 5-6* | 3-4 | 1-2, 3-4, 5-6 | Open | 3-4* |
| 150 MHz | 5-6, 7-8 | 3-4 | 1-2, 3-4, 5-6 | Open | 1-2 |
| 166 MHz | 5-6, 7-8 | 3-4 | 1-2, 3-4, 5-6 | Open | 3-4 |
| 200 MHz | 1-2, 7-8 | 3-4 | 1-2, 3-4, 5-6 | Open | 3-4 |
| Intel Pentium (P55C) |  |  |  |  |  |
| 150 MHz | 5-6, 7-8 | 7-8 | Open | 1-2, 3-4 | 1-2 |
| 166 MHz | 5-6, 7-8 | 7-8 | Open | 1-2, 3-4 | 3-4 |
| 200 MHz | 1-2, 7-8 | 7-8 | Open | 1-2, 3-4 | 3-4 |
| Cyrix 6x86 |  |  |  |  |  |
| 120+ | Open | 1-2 | 1-2, 3-4, 5-6 | Open | 1-2, 3-4 |
| 133+ | Open | 1-2 | 1-2, 3-4, 5-6 | Open | Open |
| 150+ | Open | 1-2 | 1-2, 3-4, 5-6 | Open | 1-2 |
| AMD K5 |  |  |  |  |  |
| PR75 | 1-2 | 3-4 | 1-2, 3-4, 5-6 | Open | 1-2, 3-4 |
| PR90 | 1-2 | 3-4 | 1-2, 3-4, 5-6 | Open | 1-2 |
| PR100 | 1-2 | 3-4 | 1-2, 3-4, 5-6 | Open | 3-4 |

CPU Core Voltage

| Core Voltage | JP3 |
| --- | --- |
| 3.52V | 1-2 |
| 3.43V | 3-4 * |
| 2.9V | 5-6 |
| 2.8V | 7-8 |
| 2.7V | 9-10 |
| 2.5V | 11-12 |

VI/O Voltage

| Core Voltage | JP6 # |
| --- | --- |
| 3.52V | 3-4 |
| 3.45V | 1-2 * |

ATX Power Supply

| Power | JP1 |
| --- | --- |
| Off | 1-2 |
| On | 3-4 * |

Cache Size

| Size | JP7 |
| --- | --- |
| 256 KB | 1-2 * |
| 512 KB | 3-4 |

Cache Module Type

| Type | JP16 |
| --- | --- |
| Module without Tag | 1-2 |
| Module with Tag | 2-3 * |

Super I/O Controller

| Super I/O Controller | JP12 |
| --- | --- |
| SMC669 Enabled Disabled | 1-2 * 2-3 |

PS/2 Mouse

| Function | JP14 |
| --- | --- |
| Enabled | Closed * |
| Disabled | Open |

Keyboard Clock

| Clock | JP15 |
| --- | --- |
| ISA clock | 1-2 * |
| 12 MHz | 3-4 |

CMOS

| Function | JP9 |
| --- | --- |
| Normal | 1-2 * |
| Clear CMOS | 2-3 |

Onboard Connectors

| Connector | Function |
| --- | --- |
| CN1 | COM1 connector |
| CN2 | Power connector |
| CN3 | ATX power connector |
| CN4 | PS/2 keyboard connector |
| CN5 | Two-pin fan connector |
| CN6 | Floppy drive connector |
| CN7 | PS/2 mouse connector |
| CN8 | Printer/parallel port |
| CN9 | IDE2 connector |
| CN10 | IDE1 connector |
| CN11 | COM2 port |
| CN12 | IR connector (optional) |
| CN13 | HDD LED connector |
| CN14 | USB connector (optional) |
| CN15 | Multifunction connector |
| JP13 | Reserved for manufacturer use |

+	   Refer to the CPU voltage jumper settings

*	   Default setting

*	  	Default setting

#	   	This jumper is preset by the manufacturer to 1-2.  We recommend you NOT to change the setting.

*	 	Default setting

*	 	Default setting