Chapter   3

AMI BIOS Utility

This chapter tells how to configure the system by setting the BIOS parameters.

# 3.1	Entering the AMI BIOS Setup

To enter the AMI BIOS Setup, press c.  The AMI BIOS Setup Main Menu appears as shown below.

The AMI BIOS is in Windows form.  You can use either the keyboard or a mouse to move between the items.

To select among the Setup menu groups, use v to highlight the selected group or simply click on the icon of the selected Setup menu.

To select among the options, you can either use the arrow keys to move the highlight bar or simply click on the icon of the desired option.

After making your selection, press e or double-click on the icon to open the selected menu option.

|  | You can press h to enter the BIOS Setup screen. This procedure allows you to do the following:: - Resolve an address conflict due to an IRQ address assigned to multiple slots. For more information on IRQ assignment, see the section 3.2.3 (Chipset Features Setup). - Reset to BIOS default settings if the PnP BIOS does not recognize the hardware modifications under Windows 95. |
| --- | --- |

# 3.2	Setup Menu

The screen below shows the Setup menu window.  Use the arrow keys to highlight an option.

## 3.2.1	Standard Setup

The following screen appears if you select Standard from the Setup options:

This menu allows you to input configuration values such as date, time and disk types.

#### Primary Master and Slave/
Secondary Master and Slave

These parameters allow you to configure the hard disks and the IDE devices connected to your IDE connectors.  To configure the hard disk connected to the master port of the primary IDE connector, select Primary Master and press e.  The following screen appears:

To configure the hard disk connected to the slave port of the primary IDE connector, select Primary Slave.

The secondary IDE connector also supports two IDE devices.  To configure the hard disk or the IDE device connected to the master port, select Secondary Master.  Choose Secondary Slave to configure the device connected to the slave port.

The following are the parameters that you need to set to configure your hard disks or the IDE devices:

Type

This parameter lets you set the IDE device type that your system supports.  The options are User, Auto, CD-ROM, Type 1-46, and Not Installed.  Select Auto to automatically configure the installed hard disk or IDE device.  Select CD-ROM or Auto if you have a CD-ROM installed in your system.  If you have an old type HDD installed, you may need to enter the HDD parameters manually.  To do this, you must set this parameter to User.  Set this to Not Installed to bypass the function.

LBA/Large Mode

This enhanced IDE feature allows the system to use a hard disk with a capacity of more than 528 MB.  This is made possible through the Logical Block Address (LBA) mode translation.  Set the parameter to Off to disregard the feature.

This parameter becomes non-configurable when the HDD Type parameter is set to Auto.

Block Mode

This function enhances disk performance depending on the hard disk in use.  If enabled, it allows data transfers in block (multiple sectors) by increasing the data transfer rate to 256 bytes/cycle.  However, if your hard disk does not support this function, set this parameter to Off.

This parameter becomes non-configurable when the HDD Type parameter is set to Auto.

32-bit Mode

Enabling this parameter improves system performance by increasing the hard disk access to 32-bit mode.  However, make sure that your hard disk supports this function before you enable the parameter.  The default setting is Off.

PIO Mode

Setting this parameter to On allows the system to use a faster hard disk drive.  If your hard disk does not support the PIO mode feature, set this parameter to Off.

This parameter becomes non-configurable when the HDD Type parameter is set to Auto.

#### Hard Disk Types

After you have set all the necessary parameters, press e.  A list of the HDD drive parameters appears:

Select your hard disk type.  Press w or y to move among the selections.  After you have made your selection, press e.

If you cannot find your hard disk drive type on the list, select User.  This allows you to enter the disk parameters manually.

#### Date/Time

To set the date and time, highlight Date/Time and press e.  The following screen appears:

Use the arrow keys to move among the items.  Press or click on + or - to set the current time and date.  Press e or double-click on the Control menu box in the upper-left corner of the window.

#### Floppy Drives A and B

To configure the first floppy drive, select Floppy A.  The following values appear on screen:

After selecting the proper setting, press e.

Select Floppy B and follow the same procedure to configure the second floppy drive.

## 3.2.2	Advanced CMOS Setup

The following screen appears if you select the option Advanced from the Setup menu:

The first screen does not show all the parameters of the Advanced Configuration menu.  To scroll down the rest of the parameters, press }. Press w or y to highlight the desired parameter.

|  | Do not change the settings of the Advanced Setup parameters if you are not a qualified technician. Doing so may cause fatal system failure. |
| --- | --- |

### Quick Boot

During the system boot process, the system performs power-on self test (POST) routines.  Enable the parameter if you want to skip some POST routines during boot-up process.  Set this to Disabled to let the system perform all the POST routines and follow the specified boot-up sequence.

### Power-on Delay

This parameter lets you set the POST waiting time for the HDD motor to stabilize before system boot.  The settings are from 1 to 15 seconds and Disabled.  The default setting is Disabled.

### Boot-up Sequence

The boot-up sequence allows you to specify the system search sequence.  The selections are C:, A:, CD-ROM / A:, CD-ROM, C: / A:, C:, CD-ROM / C:, CD-ROM, A: / CD-ROM, A:, C:, and CD-ROM, C:, A:.  If you have a bootable CD-ROM installed, you may set the CD-ROM as the first priority.  The default is A:, C:, CD-ROM.

### Boot-up Numlock

Setting this parameter to On enables the numeric function of the numeric keypad.  Set this parameter to Off to disregard the function.  Disabling the numeric function allows you to use the cursor control numeric keypad.  The default setting is Off.

### Floppy Drive Swap

This parameter allows you to swap floppy drives.  For example, if you have two floppy drives (A and B), you can assign the first drive as drive B and the second drive as drive A or vice-versa.  Disable the parameter to bypass the function.  The default is Disabled.

### Floppy Drive Seek

When enabled, the BIOS detects whether there is a floppy disk drive installed in the system.  Disable the parameter to skip the function.

### Typematic Rate

This parameter determines the number of characters that can be entered per second.  The default is 30.

### System Keyboard

Set this parameter to Present if there is a keyboard connected to the system.  Otherwise, select Absent.

### Primary Display

This function detects the type of VGA in use.  The selections are VGA/EGA, CGA 40 x 25, CGA 80 x 25, Mono, and Absent.  The default setting is VGA/EGA.

### Password Check

This parameter lets you set when to check for the password.  When set to Always, a password prompt appears every time you turn on the computer or when you enter Setup.  When set to Setup, the password prompt appears when you try to enter setup.  The Optimal and Turbo default setting is Setup.

### Parity Check

Set this parameter to Enabled if you install SIMMs with parity in your system.  Otherwise, set this parameter to Disabled.  Since the DRAM can still operate without enabling the parity scheme for SIMMs with parity, this function is normally set to Disabled.

### OS/2 Compatible Mode

Enable the parameter if your system is utilizing an OS/2 operating system and has a memory size of more than 64 MB.  Otherwise, set this to Disabled.  The default setting is Disabled.

### Internal Cache

This function lets you enable or disable the internal cache.

### External Cache

This function lets you enable or disable the external cache.

### System BIOS Cacheable

Enabling this parameter allows you to cache the system BIOS to further system performance.  Leave the parameter setting to its default (i.e., Disabled), to prevent the system BIOS from being cached.

### C000 ~ DC00, 16K Shadow

These parameters are for shadowing expansion cards with ROM.  You need to know the specific addresses that ROMs use to shadow the expansion cards before you set any of these parameters.  If you do not know this information, enable all the ROM shadow settings.  This ensures shadowing of any present ROMs and reduces the available memory .  Select Cached if the data in the chosen addresses are already copied into RAM.  The default setting is Disabled.

|  | The F000 and E000 addresses are exclusively shadowed for BIOS. |
| --- | --- |

## 3.2.3	Chipset Features Setup

The Chipset Features Setup controls the board chipset settings.  The controls for this menu are the same as for the previous screens.

The Chipset Features Setup screen appears as follows.

To scroll down the rest of the parameters, press }.  Use w or y to highlight the desired parameter.

### 430HX Global Features

This option lets you enable or disable the 430HX chip features.  To optimize system performance, set this parameter to Enabled.  Select Disabled to disregard the option.

The default setting is Disabled.

### Memory Hole

This option lets you reserve system memory area for ISA cards.  The settings are 512~640K, 15~16M, and Disabled.

### 8-bit I/O Recovery Time (Sysclk)

This parameter allows you to set the response time of the 8-bit I/O devices connected to your system.  The settings range from 1-7 SYSCLK and Disabled.

### 16-bit I/O Recovery Time (Sysclk)

This parameter allows you to set the response time of the 16-bit I/O devices connected to your system.  The settings range from 1-4 SYSCLK and Disabled.

### DRAM Timing

The selections for this parameter are 60 ns, 70 ns, and Manual.  If you select either 60 ns or 70 ns, the DRAM Timing subparameters become non-configurable since BIOS automatically sets the values.  Select Manual if you want to specify your own parameter settings.

#### DRAM Refresh Rate

This option lets you specify the frequency at which the chipset refreshes the DRAM to avoid data lost.  The selections are 50 MHz, 60 MHz, and 66 MHz.

#### ISA Clock Divisor

This option specifies the ISA clock frequency.  The selections are PCICLK/4 and PCICLK/3.

#### Turbo Read Lead Off

When enabled, BIOS skips the first input register in the DRAM when reading data and therefore, speeds up the data read timings.  Disable the option to bypass the feature.

#### DRAM Read Burst Timing

This parameter sets the timing for burst mode reads from DRAM.  Everytime the CPU reads the second-level cache miss, it reads four continuous memory cycles on four continuous addresses from the DRAM.

The available parameter settings are X-4-4-4, X-3-3-3, and X-2-2-2.  Faster DRAMs require shorter wait states.  The value of X depends on the DRAM Lead-off Timing parameter setting.  The default is X-4-4-4.

#### DRAM Write Burst Timing

This parameter sets the timing for burst mode writes to DRAM.  DRAM burst write requests are generated by the CPU in four continuous addresses.

The available parameter settings are X-4-4-4, X-3-3-3, and X-2-2-2.  Faster DRAMs require shorter wait states.  The value of X depends on the DRAM Lead-off Timing parameter setting.

#### Fast RAS to CAS Delay (clocks)

This option specifies the wait state between the row address strobe (RAS) and column address strobe (CAS) signals.  The available settings are 3 and 2.

#### DRAM Lead-Off Timing (DLT)

This option specifies the lead-off time before data can be accessed.  Some DRAMs may require a longer delay to access data.

#### Speculative Lead Off

Enable the parameter to speed up the data read action by presenting the DRAM controller read request before the controller chip decodes the data to the final memory target (i.e., cache, DRAM or PCI).

#### Turn Around Insertion

Enabling this option allows the CPU to insert one turn-around clock cycle to the MD signals after asserting the MWE signal before enabling the MD buffers.  Set this to Disabled to select the back-to-back DRAM cycles for asserting MWE signal.

### Peer Concurrency

Enable this parameter if you want to activate more than one PCI device.  Otherwise, set this to Disabled.

### Memory Error Check Mode

This parameter allows you to set the DRAM error checking mode.  The options are ECC, Parity, and Disabled.

### PCI 2.1 Compliant

This parameter lets you enable or disable the PIIX3 PCI register delay transaction and passive release functions.  When enabled, the PIIX3 controls the USB operation to make sure that the system complies with the PCI revision 2.1 specification.  Disable the parameter to disregard the function.  The default is Disabled.

### USB Function Enabled

This parameter lets you enable or disable the USB device(s) connected to your system, if any.  The default is Disabled.

|  | The USB function shares INTD with PCI slot 4. Therefore, if you enable the USB function, only PCI cards that do not require IRQ, such as VGA, can be installed in slot 4. The PnP BIOS assigns an IRQ to VGA only if the VGA requests for it. |
| --- | --- |

## 3.2.4	Power Management Setup

To take advantage of the power-management feature, select Power Management from the Setup menu.  The following screen appears:

To scroll down the rest of the parameters, press }.  Use w or y to highlight the desired parameter.

### Power Management/APM

This parameter enables or disables the advanced power-management function.

### Instant On Timeout (Minutes)

This parameter is configurable only if the Power Management/APM parameter is set to Instant On.  This lets you specify when to resume system power after being in power-saving mode for a certain period of time.

### Green Monitor Power Down State

This function lets you set when to power down your green PC monitor.  The options are Standby and Suspend.

### Display Card Power Down Mode

This option allows you to set when to power down your system display card.  The card function returns to full power once the system resumes to normal mode.  The selections are Standby, Suspend, and Disabled.

### Hard Disk Power Down Mode

This option lets you set when to “spin down” your IDE hard disk.  The disk returns to full speed once the system resumes to normal mode.  The available settings are Standby, Suspend, and Disabled.

### Hard Disk Timeout (Minutes)

This option lets you set when to put the hard disk to the specified power-down mode.

### Standby Timeout (Minutes)

This function lets you set when to put the system to standby mode.  In standby mode, the CPU clock slows down.  Any event detected returns the system to full power.  The settings range from 1~15 min.

### Suspend Timeout (Minutes)

This function lets you set when to put the system to suspend mode.  In suspend mode, the CPU clock stops.  Any event detected returns the system to full power.  The settings range from 1~15 min..

### Slow Clock Ratio

When the system enters the standby mode, the CPU clock starts to slow down.  This parameter lets you set the “slow down” clock ratio.

### IRQ 3, 4, 5, 7, 8, 9, 10, 11, 12, 13, 14, and 15

If you set any of these parameters to Monitor, the chipset checks the activity of the selected IRQs.  Any activity detected prevents the system from entering the power-saving mode.

If you set any of these parameters to Wake up, any activity detected from the selected IRQs wakes the system from power-saving mode.

## 3.2.5	PCI/PnP Setup

The PCI/PNP Setup allows you to specify the setting for your PCI devices.  The screen below appears if you select PCI/PnP from the Setup menu.

To scroll down the rest of the parameters, press }.  Use w or y to highlight the desired parameter.

### Plug-and-Play Aware O/S

Enable this parameter only if you have a Plug-and-Play operating system such as Windows 95.  This is to ensure that the PnP cards are initialized during POST and proper resources are allocated to each card by the Plug-and-Play operating system.

### PCI VGA Palette Snoop

PCI devices support the palette snooping technique that enables the device to control access to their palette registers.  Enable this parameter activates the palette snooping function in the PCI VGA devices.  Check your VGA card manual for more information about this function.  The default setting is Disabled.

### PCI IDE Card

This function allows you to select the PCI slots that you want to enable, if there are any offboard PCI IDE card present.   Set this parameter to Auto to automatically configure the installed PCI card.

### PCI IDE Primary IRQ

This parameter lets you assign an IRQ for the IDE device connected to your primary IDE card connector.  The settings are INTA, INTB, INTC, INTD, Hardwired, and Disabled.  If the PCI IDE Card parameter is set to Auto, this parameter becomes non-configurable.

### PCI IDE Secondary IRQ

This parameter lets you assign an IRQ for the IDE device connected to your secondary IDE card connector.  The settings are INTA, INTB, INTC, INTD, Hardwired, and Disabled.  If the PCI IDE Card parameter is set to Auto, this parameter becomes non-configurable.

### PCI Slot 1, 2, 3, and 4 IRQ Priority

These parameters let you specify the appropriate interrupt for each occupied PCI slots.

### DMA Channel 0, 1, 3, 5, 6, and 7

These lines allow you to assign the available DMA channels to either PnP device or Legacy/ISA functions.

### IRQ 3, 4, 5, 7, 9, 10, 11, 14, and 15

These lines allow you to assign the available IRQs to either PCI/PnP or Legacy/ISA devices.

### Reserved Memory Size for ISA

This option lets you specify the memory area reserved for Legacy/ISA devices to avoid conflict.

### Reserved Memory Base for ISA

This option lets you specify the memory base of the specified memory area reserved for Legacy/ISA devices to avoid conflict.

## 3.2.6	Peripheral Setup

Select Peripheral from the Setup menu and the following screen appears.

### Onboard FDC

This parameter enables or disables the floppy drive controller.

### Onboard Serial Port 1

This parameter allows you to select the address for the first serial port. Selecting Disabled deactivates the port.

### Onboard Serial Port 2

This parameter allows you to select the address for the second serial port.  Selecting Disabled deactivates the port.

#### Serial Port 2 Mode

This parameter is configurable only if the Onboard Serial Port 2 parameter is enabled.  This allows you to specify the serial port 2 mode.  The available mode selections are:

- Normal - Sets serial port 2 to operate in normal mode.  This is the default setting.

- HPSIR - Select this setting only if the InfraRed (IR) function is activated (i.e., an IR module is installed in your system).  This setting allows infrared serial communication at a maximum baud rate of 115K baud.

- AskIR - Select this setting only if the IR function is activated (i.e., an IR module is installed in your system).  This setting allows infrared serial communication at a maximum baud rate of 19.2K baud.

### Onboard Parallel Port

This parameter allows you to select the address for the parallel port.  Selecting Disabled deactivates the parallel port.

#### Parallel Port IRQ

This parameter is configurable only if the Onboard Parallel Port is NOT set to Auto.  This allows you to set an IRQ for the parallel port function.  The available IRQ selections are 5 and 7.

#### Parallel Port Mode

This parameter specifies the parallel port mode.  The mode options are SPP, EPP and ECP.

### Onboard IDE

This parameter enables or disables the onboard IDE controller.

# 3.3	Security Setup

The Security window contains the password and anti-virus features.

## 3.3.1	Supervisor Password

The use of password prevents unauthorized use of your computer.  If you set a Supervisor password, the system prompts for this password before granting access to Setup or system boot, depending on the Password Check setting in the Advanced CMOS Setup menu (refer to section 3.2.2).

To set a Supervisor password, select Supervisor from the Security window.  The following screen appears:

Follow these steps to set up a password using the keyboard:

1.	Type in a six-character password using letters, numbers, or a combination of both.  When you type the characters, they appear as asterisks on the password screen boxes.

2.	Press e.

3.	Retype the password when a password confirmation box appears asking you to retype the password.

You may also use the mouse and the characters on the screen to set up a password.

1.	Click on six characters from the password screen.  The characters appear on the boxes as asterisks.

2.	Click on e.

3.	Enter the password when a confirmation box appears.

## 3.3.2	User Password

To set a User password, select User from the Security window.  The following screen appears:

For instructions on how to enter a password, follow the procedures listed in section 3.3.1.

|  | If you set a Supervisor and a User password, you can enter either of the two entries when prompted for a password before system boot or entering Setup. |
| --- | --- |

## 3.3.3	Anti-virus

Select Anti-Virus from the Security window to display the following option box.

The virus protection options allow you to enable or disable the virus protection feature.

# 3.4	Utility Setup

The Utility window lets you change WinBIOS Setup colors and language setting.

## 3.4.1	Color Set

Select Color Set from the Utility window to display the following screen.

Use the arrow keys or simply click an option to select your desired background color for WinBIOS.

## 3.4.2	Language

Select Language from the Utility window to display the following screen.

The system language currently supported is only English.  Therefore, this option is non-configurable and is for display only.

# 3.5	Default Setup

The Default window allows you to select a group of settings for all WinBIOS Setup options.

## 3.5.1	Original

When you select Original, a dialog box prompts you to restore the old values.  Select No to keep your current settings or Yes to restore the original values.

## 3.5.2	Optimal

When you select Optimal, a dialog box prompts you to load the optimal values.  Select No to keep your current settings or Yes to load the optimal values.  We recommend that you select Yes to ensure stable system performance.

## 3.5.3	Turbo

When you select Turbo, a dialog box prompts you to load the Turbo values, such as setting the DRAM speed to 60 ns.  Select No to keep your current settings or Yes to load the Turbo values.

Loading the Turbo values enhance system performance.  However, if instability problem occurs during system operation, we recommend that you load the Optimal values.

# 3.6	Exiting Setup

Carefully check your new settings when you have finished configuring the system.  If correct, write them down and keep the recorded values in a safe place.  If in the future, the battery loses power or the CMOS chip is damaged, you will know what values to enter when you rerun setup.

Press ^ to display the following screen.

Press y or w key then e or simply click on an option to select.  Select Save changes and Exit to save the changes that you made.  Select Do not save changes and Exit to leave setup without saving your changes.  Select Continue if you want to make any more configuration changes.

# 3.7	NCR SCSI BIOS and Drivers

The NCR 53C810 SCSI BIOS resides in the same flash memory chip as the system BIOS.  To use the onboard NCR BIOS, install an NCR 53C810 SCSI controller card in your system.

All SCSI devices that you install in your system require software drivers.  The NCR SCSI BIOS directly supports SCSI hard disks under DOS, Windows and OS/2.  It also uses DOS-format and SCO UNIX-format support floppy disk device drivers that come with the NCR 53C810 SCSI controller card.  The DOS-format device drivers are for SCSI devices used with DOS, Windows NT, Novell NetWare and OS/2.  The SCO UNIX-format device drivers are for SCSI devices used with SCO UNIX.  These drivers offer higher performance than the direct BIOS support.

To use the device drivers, you must install them in your system hard disk drive and add them to your system configuration files.  For detailed installation instructions, see the README files that come with the drivers.

|  | The system board also supports the AMI Flash Memory Writer Utility that allows you to upgrade the system BIOS. For more information on this utility, contact your local distributor. |
| --- | --- |