The system board comes with a NS PC87410 software package that includes two diskettes.  The first diskette contains the software drivers for DOS, Windows 3.1, Windows NT, Novell 3.1X, 4.X and OS/2.  The second diskette contains the software driver for SCO UNIX 3.X.

The following sections tell how to install the device drivers.  For more detailed information, refer to the README file in Diskette 1.

## DOS

We recommend that you install the DOS drivers in order to make use of the special features such as Track Remapping and More than two Drives support.

To install, simply insert Diskette 1 into drive A and type A: e.  Enter INSTALL e.

## Windows v3.1X

1.	Make sure that the DOS drivers are already installed.

2.	Enter Windows.  Click on File Manager.  Run A:\WINDRV/INSTALL.EXE.

The 32-bit Windows driver supports the following:

- A maximum of four drives (two drives per channel)

- Track Remapping, to support a hard disk capacity over than 528 MB

|  | The Microsoft driver does not support the secondary channel and the Track Remapping feature. Therefore, do not enable the 32-bit disk access during installation if the target disk is connected to the secondary channel. The 32-bit disk access will be enabled after the Windows driver installation. |
| --- | --- |

## NetWare 3.11, 3.12 SFT-III 4.00, 4.01

Make sure that the DOS drivers are already installed.  Refer to the NetWare Installation manual for the procedure.

## Windows NT v3.X

1.	Insert Diskette 1 into drive A.

2.	Type A:\INSTALL e.

3.	When prompted where to copy the files, do not select drive C.  Follow the instructions on screen.

4.	After installing Windows NT, enter Windows and click on File Manager.

5.	Click on A:\NTDRV\SETUP.

## OS/2 v2.1X

1.	Insert Diskette 1 into drive A.

2.	Type A:\INSTALL e.

3.	When prompted where to copy the files, do not select drive C.  Follow the screen instructions.

4.	After you install OS/2, click on the Drive Setup icon to complete the instructions.

## SCO UNIX

1.	Insert Diskette 2 into drive A.

2.	Copy the \DOSDRV\DEVICE.LOG to /ETC under the UNIX environment.  To do this, type:

#DOSCP A:\DOSDRV\DEVICE.LOG /ETC e

3.	Type INSTALLPKG and follow the screen instructions.