### Dual-processor/Single-processor Operation Select

| Selection | JP17 | JP18 | JP19 | JP20 | JP21 |
| --- | --- | --- | --- | --- | --- |
| Single processor | Open | Open | 2-3 | 2-3 | 1-2 |
| Dual processor | Closed | Closed | 1-2 | 2-3 | 1-2 |

### Changing the CPU Type and Speed

| CPU Type-Speed | JP16 | JP23 |
| --- | --- | --- |
| P54C-75 | Open | 1-2 |
| P54C-90 | 1-3 | 1-2 |
| P54C-100 | 1-3 | 1-2 |

### Selecting the Primary-level Cache (L1) Mode

| Mode | JP22 |
| --- | --- |
| Write-back | Open |
| Write-through | Closed |

### Selecting the Cache and Size

| Cache Size | JP24 | JP26 |
| --- | --- | --- |
| 256 KB | 1-2 | 2-3 |
| 512 KB | 2-3 | 2-3 |

### Enabling the Second-level Cache Parity

| Function | JP13 |
| --- | --- |
| Enabled | 1-3, 2-4 |
| Disabled | 1-2, 3-4 |

### Selecting the Flash ROM Type

| Flash ROM Type | JP1 |
| --- | --- |
| 5V Flash ROM | 1-2 |
| 12V Flash ROM | 2-3 |

### Enabling the Enhanced IDE Controller

| Function | JP7 |
| --- | --- |
| Enabled | 1-2 |
| Disabled | 2-3 |

### Enabling the Super I/O Chip

| Function | JP6 |
| --- | --- |
| Enabled | 1-2 |
| Disabled | 2-3 |

### Selecting the ECP DMA Channel

| Selection | JP2 | JP3 |
| --- | --- | --- |
| DMA 1 | 1-2 | 1-2 |
| DMA 3 | 2-3 | 2-3 |

### Enabling the SCSI Chip Termination

| Function | JP4 |
| --- | --- |
| Enabled | Open |
| Disabled | Closed |

### Selecting the SCSI Device Size

|  | JP9 | JP10 | JP11 | JP12 |
| --- | --- | --- | --- | --- |
| 8-bit SCSI | 2-3 | 2-3 | Open | Open |
| 16-bit SCSI | 1-2 | 1-2 | Closed | Closed |

### Enabling the PS/2 Mouse (optional)

| Function | JP5 |
| --- | --- |
| Enabled | Open |
| Disabled | Closed |

### Clearing the CMOS

| Function | JP14 |
| --- | --- |
| Normal | Open |
| Clear CMOS | Closed |

### Default Settings for Reserved Jumpers

| Jumper | Default Setting |
| --- | --- |
| JP8 | Open |
| JP15 | Open |
| JP25 | 1-2 |