This chapter explains the Setup utility for the Award BIOS and tells how to configure the system by setting the BIOS parameters.

# Award BIOS Setup Main Menu

The Award BIOS Setup Main Menu appears below.  During booting, a prompt asks you to enter the system menu.  To enter the system menu, press c.

ROM PCI/ISA BIOS 
CMOS SETUP UTILITY
AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP PCI CONFIGURATION SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS |  |  | PASSWORD SETTING IDE HDD AUTO DETECTION SAVE & EXIT SETUP EXIT WITHOUT SAVING |  |
| --- | --- | --- | --- | --- |
| ESC : Quit ¯®¬ : Select Item F10 : Save & Exit Setup (Shift) F2 : Change Color |  |  |  |  |
| Description of each function |  |  |  |  |
|  |  | Choosing Load BIOS Defaults or Load Setup Defaults at this point modifies all applicable settings. |  |  |

The section at the bottom of the screen tells how to control the menu.  Press the arrow keys to move between items, jm to color scheme of the display, ^ to exit, and u to save the changes before exiting.  Another section at the bottom gives a brief description of the highlighted item.

After selecting an item, press e to select or enter a submenu.

# Standard CMOS Setup

Standard CMOS Setup sets the basic system parameters such as the date, time, and the hard disk type.  Press the arrow keys to highlight an item and { or } to select the value for each item.

ROM PCI/ISA BIOS 
STANDARD CMOS SETUP
AWARD SOFTWARE, INC.

Base Memory: 640 K Extended Memory: 15360 K Other Memory: 384 K Total Memory: 16384 K Date (mm:dd:yy) : Wed. Jan 4 1995 Time (hh:mm:ss) : 00:00:00 HARD DISK TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE Primary Master : User 345 790 15 65535 790 57 NORMAL Primary Slave : None 0 0 0 0 0 0 --- Secondary Master : None 0 0 0 0 0 0 --- Secondary Slave : None 0 0 0 0 0 0 --- Drive A: 1.44M, 3.5 in Drive B: None Video : EGA/VGA Halt On: All Errors

ESC : Quit ¯®¬ : Select Item PU/PD/+/- : Modify F1 : Help (Shift) F2 : Change Color

## Date

To set the date, highlight the Date parameter.  Press { or } to set the current date.  The date format is month, date, and year.

## Time

To set the time, highlight the Time parameter.  Press { or } to set the current time in hour, minute, and second format.  The time is based on the 24-hour military clock.

## Hard Disks

The four hard disk parameters listed in the menu are Primary Master, Primary Slave, Secondary Master and Secondary Slave.  These parameters allow you to configure the drives that you connect into your IDE connectors.

To configure, press { or } to select a hard disk drive type, or type the number and press e.  Select User to define your own hard disk type manually.  You can use the IDE HDD Auto-detection function in the Main Menu to automatically configure your hard disk drive parameters.  If your system does not have a hard disk drive, select None and press e.

|  | For an IDE hard disk, we recommend that you use the Auto-detection utility to enter the drive specifications automatically. See the section IDE HDD Auto-detection. Select the User option if you want to enter the specifications manually. |
| --- | --- |

## Floppy Drive Types

Press { or } to select a floppy disk drive type.  The settings are:

- 360K, 5.25 in.

- 1.2M, 5.25 in.

- 720K, 3.5 in.

- 1.44M, 3.5 in.

- 2.88M, 3.5 in.

Select None if you have no floppy drive.

## Video

This parameter allows you to configure the video display card present in your system.  The video settings are:

- EGA/VGA

- CGA 40

- CGA 80

- MONO

Select the adapter type setting that matches your video display card and monitor.

## Error Halt

This parameter enables you to control the system stops in case of error.  The available settings are:

- No Errors

- All Errors

- All, But Keyboard

- All, But Diskette

- All, But Disk/Key

The default setting is All Errors.

## Memory

The Memory parameters are for display only.  These are determined by the BIOS power-on self-test.

### Base Memory

The POST detects the amount of base (or conventional) memory installed in the system.  The value of the base memory is 640 KB for systems with 640 KB or more memory installed on the mainboard.

### Extended Memory

The BIOS detects the amount of extended memory present during the POST.  This is the amount of memory located above 1 MB in the CPU memory address map.

### Expanded Memory

Expanded memory is the memory defined by the Lotus/Intel/Microsoft (LIM) standard as EMS.  Since a number of standard DOS applications can not utilize memory above 640 KB, the Expanded Memory Specification (EMS) swaps the memory which is not utilized by DOS with a section, or frame.  This allows access to the total system memory.  The memory range that can be swapped by EMS depends on the chipset design.

An expanded memory device is required to use memory as expanded memory.

### Other Memory

This refers to the memory located in the 640 KB to 1024 KB address space.  This is the memory that can be used for different applications.  DOS uses this area to load device drivers to keep as much base memory free for application programs.

# BIOS Features Setup

This screen appears when you select BIOS Features Setup from the Main Menu.

ROM PCI/ISA BIOS 
BIOS FEATURES SETUP
AWARD SOFTWARE, INC.

| Virus Warning : Disabled CPU Internal Cache : Enabled External Cache : Enabled Quick Power On Self Test : Disabled Boot Sequence : A, C Swap Floppy Drive : Disabled Boot Up Floppy Seek : Enabled Boot Up NumLock Status : On Boot Up System Speed : High IDE HDD Block Mode : Disabled Gate A20 Option : Fast Memory Parity Check : Disabled Typematic Rate Setting : Disabled | Video BIOS Shadow : Enabled C8000-CBFFF Shadow : Disabled CC000-CFFFF Shadow : Disabled D0000-D3FFF Shadow : Disabled D4000-D7FFF Shadow : Disabled D8000-DBFFF Shadow : Disabled DC000-DFFFF Shadow : Disabled |
| --- | --- |
| Typematic Rate (Chars/Sec) : 6 Typematic Delay (Msec) : 250 Security Option : Setup IDE Second-Channel Control : Enabled | ESC : Quit ¯ ® ¬ : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |

## Virus Warning

Set this parameter to Enabled to activate the warning message.  This feature protects the boot sector and partition table of your hard disk from virus intrusion.  Set it to Disabled to disregard the message.

Any attempt during boot-up to write to the boot sector of the hard disk drive stops the system and the following warning message appears on the screen.  Run an anti-virus program to locate the problem.

! WARNING !
Disk Boot Sector is to be modified
Type "Y" to accept write, or "N" to abort write
Award Software, Inc.

## CPU Internal Cache/External Cache

The CPU Internal Cache/External Cache default setting is Enabled.  Turning the cache off slows down the system.  Leave it enabled unless you are troubleshooting a problem.

## Quick Power-on Self-test

This parameter speeds up the power-on self-test by skipping some items that are normally checked.  The default setting is Disabled.

## Boot Sequence

The settings are C, A and A, C to specify the system search sequence.  The default setting is A, C.

## Swap Floppy Drive

Enabling this parameter tells your system to read the current floppy disk drive in use regardless of the drive designation.  The system may read drive A as drive B or vice-versa.  The default setting is Disabled.

## Boot-up Floppy Seek

Set this parameter to Enabled to detect the type of floppy disk drive by tracks.  Set it to Disabled to disregard searching.  Note that in the Disabled setting, no warning message appears if the drive installed is 360 KB.  The default setting is Enabled.

## Boot-up NumLock Status

Setting this parameter to On enables the numeric function of the numeric keypad.  Set this parameter to Off to disregard the function.  Disabling the numeric function allows you to use the numeric keypad for cursor control.  The default setting is On.

## Boot-up System Speed

Set the system speed to High or Low with this parameter.  The default setting is High.

## IDE HDD Block Mode

This feature enhances hard disk performance by allowing multisector transfers instead of one sector per transfer.  Most IDE drives, except those with old designs, can support this feature.  The default setting is Disabled.

## Gate A20 Option

The settings for this parameter are Normal and Fast.  If the data transfer is controlled by the 8042 chip, set the parameter to Normal.  The Fast setting transfers the control to the ASICs.  The default setting is Fast.

## Memory Parity Check

This parameter lets you enable or disable the optional parity DRAM.  The default setting is Disabled.  If you install SIMMs with parity bit chips, you must set this parameter to Enabled.

## Typematic Rate Setting

Set this parameter to Enabled to determine the typematic rate.  Set it to Disabled to disregard the rate setting.  The default setting is Disabled.

## Typematic Rate

This parameter allows you to control the speed at which the system registers repeated keystrokes.  The typematic rate settings are 6, 8, 10, 12, 15, 20, 24, and 30 chars/sec.  The default setting is 6 chars/sec.

## Typematic Delay

This parameter allows you to control the display time between the first and the second characters.  The typematic delay settings are 250, 500, 750, and 1000 msec.  The default setting is 250 msec.

## Security Option

The security option settings are System and Setup.  The System option limits access to both the System and Setup.  A prompt asking you to enter your password appears on the screen everytime you boot the system.  The Setup option limits access only to setup.  The default setting for this parameter is Setup.

To disable the security option, select Password Setting from the Main Menu and press e.

## IDE Second-channel Control

Set this parameter to Enabled if the system uses the second IDE channel.  Disable the parameter if only the primary IDE channel is in use.

## Video BIOS Shadow

If enabled, this copies the video display card BIOS into the system DRAM to improve system performance.  The default setting for this parameter is Enabled.

## C8000-CBFFF Shadow to DC000-DFFFF Shadow

These six lines are for shadowing other expansion cards with ROMs.  The default setting for these areas is Disabled.  You need to know the specific addresses that ROMs use to shadow the expansion cards.  If you do not know this information, enable all the ROM shadow settings.  This ensures shadowing of any present ROMs.  Also, enabling all the settings reduces the available memory by 640 KB to 1024 KB.

# Chipset Features Setup

This screen appears when you select Chipset Features Setup from the Main Menu.

ROM PCI/ISA BIOS
CHIPSET FEATURES SETUP
AWARD SOFTWARE, INC.

| Auto-Configuration : Enabled DRAM Burst of 4 Refresh : Enabled DRAM RAS# Active : Asserted DRAM Refresh Mode : Normal DRAM R/W Burst Timing : X444/X333 CPU to DRAM write buffer : Enabled CPU to PCI write buffer : Enabled PCI to DRAM write buffer : Enabled PCI Memory Burst Write : Enabled Add Extra Wait for CAS# : Disabled Add Extra Wait for RAS# : Disabled System BIOS Cacheable : Enabled Video BIOS Cacheable : Enabled L2 Cache Zero Wait State : Disabled | Memory Hole at 15 MB Addr: : Disabled |
| --- | --- |
| CPU to PCI Byte Merge : Disabled VGA 128K Range Attribute : Disabled | ESC : Quit ¯ ® ¬ : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |

## Auto-configuration Function

This parameter automatically enters and locks the optimum settings for the chipset.  The Disabled setting unlocks the settings without changing them.  The default setting is Enabled.

|  | Do not change the values of the Chipset parameters unless you are a qualified technician. To configure, simply set the Auto-configuration Function to Enabled |
| --- | --- |

# Power-management Setup

The Power-management Setup controls the mainboard’s green features.

ROM PCI/ISA BIOS 
POWER MANAGEMENT SETUP
AWARD SOFTWARE, INC.

| Power Management : Disabled PM Control by APM : Yes Video Off Method : V/H SYNC+Blank Doze Mode : Disabled Standby Mode : Disabled Suspend Mode : Disabled HDD Power Down : Disabled IRQ3 (Wake-Up Event) : OFF IRQ4 (Wake-Up Event) : OFF IRQ8 (RTC Alarm) : ON IRQ12 (Wake-Up Event) : OFF | IRQ1 (Keyboard) : ON IRQ3 (COM2) : OFF IRQ4 (COM1) : OFF IRQ5 (LPT2) : OFF IRQ6 (Floppy Disk) : OFF IRQ7 (LPT1) : OFF IRQ8 (RTC Alarm) : OFF IRQ9 (IRQ2 Redir) : OFF IRQ10 (Reserved) : OFF IRQ11 (Reserved) : OFF IRQ12 (PS/2 Mouse) : OFF IRQ13 (Coprocessor) : OFF IRQ14 (Hard Disk) : OFF IRQ15 (Reserved) : OFF |
| --- | --- |
| Power Down Activities COM Ports Accessed : ON LPT Ports Accessed : ON Drive Ports Accessed : ON | ESC : Quit ¯ ® ¬ : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |

## Power Management

This function enables you to control the power-saving modes, display turn off and HDD power down.  The four settings are as follows:

| Setting | Description |
| --- | --- |
| Max Saving | Maximize energy conservation by putting the system into power-saving mode after a brief period of system inactivity. The maximum saving default is 1 min. |
| Min Saving | A power-saving assignment which activates after a moderate period of system inactivity. The minimum saving defaults are 15 min and 1 hour. |
| Disabled | Turns off power-saving. |
| User Defined | Allows you to set the power-saving options according to your requirements. |

## PM Control by APM

If your system supports the Advanced Power Management (APM) feature, set this parameter to Yes.

## Video-off Method

If your video display adapter supports the green features, then you can set this parameter to its default setting which is V/H SYNC+Blank.  If not, then set this to Blank Setting.

The default setting saves more power since it turns off the CRT vertical and horizontal scanning once the monitor screen blanks.  However, with non-green monitors, CRT scanning does not stop even if the monitor screen blanks.

|  | Screen-saver software does not work with this feature. It is used only to prevent burning in a static image on the CRT while the monitor is on. A screen saver cannot display while the monitor is shut down to save both energy and the screen. |
| --- | --- |

## Doze Mode, Standby Mode, Suspend Mode

These modes set the period of time after which each of these modes activates.

## HDD Power Down

This time-out setting indicates when to shut down the IDE hard disk.  The settings range from 20 sec. to 15 min. or Disabled.

|  | HDD Power Down does not affect SCSI hard disks. |
| --- | --- |

## IRQ3, 4, 8 and 12

This function allows you to monitor IRQ3, 4, 8 and 12 activities.  An activity detected from any enabled ports returns the system to normal mode.

## Power-down Activities

### COM, LPT, Drive Ports Accessed

These parameters allow you to monitor the activities of the COM, LPT and Drive ports.  An activity detected from any enabled ports returns the system to normal mode.

## IRQ1 to IRQ15

These lines allow you to set the IRQs individually.  An activity from any enabled IRQ returns the system to normal mode.

# PCI Configuration Setup

This screen configures the PCI Bus slots.  All the slots use INTA#.  If you install a card, you should set the card to INTA#.

ROM PCI/ISA BIOS 
PCI CONFIGURATION SETUP
AWARD SOFTWARE, INC.

| PnP BIOS Auto-config : Disabled Slot 1 Using INT# : AUTO Slot 2 Using INT# : AUTO Slot 3 Using INT# : AUTO Onboard SCSI : AUTO 1st Available IRQ : 10 2nd Available IRQ : 11 3rd Available IRQ : 9 4th Available IRQ : 5 PCI IRQ Activated By : Level PCI IDE IRQ Map To: : ISA | Onboard PCI SCSI Chip : Enabled Onboard PCI IDE Chip : Enabled Onboard FDC Controller : Enabled Onboard Serial Port 1 : COM1 Onboard Serial Port 2 : COM2 COM3 & COM 4 Address : 338H, 238H Onboard Parallel Port : 278H Parallel Port Mode : Normal |
| --- | --- |
|  | ESC : Quit ¯ ® ¬ : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |

## PnP BIOS Configuration

Setting this parameter to Enabled automatically sets the available IRQ for each slot.  Setting it to Disabled lets you select the IRQ to assign for each PCI slot and for the onboard SCSI.  The default setting is Disabled.

## Slots 1, 2, and 3 Using INT#

Each PCI slot supports four interrupt pins.  These parameters let you select which interrupt pin to use.  The selections are interrupt pins A, B, C, D and Auto.  However, you must always start with interrupt pin A.  The INTA pin is automatically reserved for master function of the PCI device.  The remaining interrupt pins (INTB, INTC, and INTD) are reserved for multifunction cards.

The default setting for these parameters is Auto.  This automatically sets the system IRQ for the PCI slot.

## Onboard SCSI

This parameter lets you select which interrupt pin to assign for the onboard SCSI.  The selections are interrupt pins A, B, C, D and Auto.  The default setting is Auto.  This automatically sets the system IRQ for the SCSI function.

## Available IRQs

These parameters appear and are configurable only if the PnP BIOS Auto-configuration parameter is disabled.  These let you assign a system IRQ to each PCI slot and for the onboard SCSI.  The available settings are 3, 4, 5, 7, 9, 10, 11, 12, 14, 15 and NA.  The NA setting means no IRQ is assigned to the slot.

Setting the Slot 1/2/3 Using INT# and the Onboard SCSI parameters to Auto automatically sets the IRQ for each slot function.  You may also change the default to another setting.  If you do, make sure that the IRQs do not conflict.

## PCI IRQ Activated By

This option lets the user set when to activate the PCI IRQ.  The available settings are Edge and Level.  The Edge is the start of the signal and Level is the point at which the signal is high.  The default setting for this parameter is Level.

## PCI IDE IRQ Map To

This feature enables you to control the PCI IDE IRQ mapping.  The available options are ISA, PCI-Slot 1, PCI-Slot 2, PCI-Slot 3, PCI-Slot 4 and PCI-Auto.  The default is ISA.

## Onboard PCI SCSI Chip

This parameter lets you enable or disable the onboard PCI SCSI chip.  The default setting is Enabled.

## Onboard PCI IDE Chip

This parameter lets you enable or disable the onboard PCI IDE chip.  The default setting is Enabled.

## Onboard FDC Controller

If enabled, this parameter allows you to connect your floppy disk drives to the onboard floppy disk connector instead of a separate controller card.  Change the setting to Disabled if you want to use a separate controller card.  The default setting is Enabled.

## Onboard Serial Ports 1 and 2

These lines allow you to assign addresses for the board’s serial connectors.  The serial ports can be assigned as follows:

| Port |  |  | Address |  |
| --- | --- | --- | --- | --- |
| COM1 |  |  | 3F8H (onboard serial port 1 default) |  |
| COM2 |  |  | 2F8H (onboard serial port 2 default) |  |
| COM3 |  |  | 338H |  |
| COM4 |  |  | 238H |  |
| Disabled |  |  | Disables the onboard port |  |
|  |  | Make sure that the ports have different assignments. |  |  |

## COM3 & COM4 Address

These lines allow you to set the COM3 and COM4 addresses.  The available options are as follows:

- 3E8H, 2E8H

- 2E8H, 2E0H

- 220H, 228H

- 338H, 238H

The default settings are 338H and 238H.

## Parallel Port

This parameter controls the onboard parallel port connector.  The options for this parameter are:

- 378H

- 3BCH

- 278H

- Disabled

The default setting is 278H.

|  | If you are using an I/O card with a parallel port, make sure that the addresses don’t conflict. |
| --- | --- |

## Parallel Port Mode

The default setting for this function is Normal.  If your system has a parallel interface peripheral device, set this function according to the enhanced mode that your device supports.  The available options are:

- Normal

- EPP (Enhanced Parallel Port)

- ECP (Extended Capabilities Port)

- ECP+EPP

# Load BIOS Defaults

This option loads the default values permanently stored in the BIOS ROM.  If the values specified in the Setup utility become corrupted, these defaults load automatically when you turn on the system power.  These settings are non-optimal and bypass all high-performance features.  This is useful if you are having problems with your current system configuration and need to determine the cause.

The defaults loaded only affect the BIOS Features Setup and Chipset Features Setup screens.  There is no effect on the Standard CMOS Setup.  To use this feature, highlight this option on the main menu and press e.  A line appears on the screen asking if you want to load the BIOS default values.  To load, press Y then e.  Press N to disregard.  See the following screen.

ROM PCI/ISA BIOS (P/I-54SP4)
CMOS SETUP UTILITY
AWARD SOFTWARE, INC.

| Load BIOS Defaults (Y/N)? STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT PCI CONFIGURATION SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | PASSWORD SETTING IDE HDD AUTO DETECTION HDD LOW LEVEL FORMAT SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| ESC : Quit ¯®¬ : Select Item F10 : Save & Exit Setup (Shift) F2 : Change Color |  |
| Loads BIOS Defaults except Standard CMOS SETUP |  |

# Load Setup Defaults

The Load Setup Defaults option loads settings which are a combination of the BIOS defaults and higher-performance settings your system can use as detected during POST.

The auto-configured settings only affect the BIOS Features Setup, Chipset Features Setup and PCI Configuration Setup.  There is no effect on the Standard CMOS Setup.  To use this feature, highlight this option on the main screen and press e.  A line appears on the screen asking if you want to load the Setup default values.  To load, press Y then e.  Press N to disregard.  See the screen below.

ROM PCI/ISA BIOS (P/I-54SP4)
CMOS SETUP UTILITY
AWARD SOFTWARE, INC.

| Load SETUP Defaults (Y/N)? STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT PCI CONFIGURATION SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | PASSWORD SETTING IDE HDD AUTO DETECTION HDD LOW LEVEL FORMAT SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| ESC : Quit ¯®¬ : Select Item F10 : Save & Exit Setup (Shift) F2 : Change Color |  |
| Load SETUP Defaults except Standard CMOS SETUP |  |

# Password Setting

This option lets you set a password.  A password prevents unauthorized use of your computer.

ROM PCI/ISA BIOS
CMOS SETUP UTILITY
AWARD SOFTWARE, INC.

| Enter Password: ****** STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT PCI CONFIGURATION SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | PASSWORD SETTING IDE HDD AUTO DETECTION HDD LOW LEVEL FORMAT SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| ESC : Quit ¯®¬ : Select Item F10 : Save & Exit Setup (Shift) F2 : Change Color |  |
| Change/Set/Disable Password |  |

To set a password, highlight the password type that you want and press e.  At the prompt, type your password.  Your password can consist of 8 alphanumeric characters.  Press e to enter your password.  At the next prompt, re-type your password and press e again to confirm the new password.  After the password entry, the screen automatically reverts to the main screen.

To disable the password, press e when prompted to enter the password.  The screen displays a message confirming that the password has been disabled.

|  | If you use the Password features, the Security Option line in the BIOS Features Setup determines when to enter the password. |
| --- | --- |

# IDE HDD Auto-detection

If your system has an IDE hard drive, you can use this utility to detect its parameters and enter them into the Standard CMOS Setup automatically.

ROM PCI/ISA BIOS
CMOS SETUP UTILITY
AWARD SOFTWARE, INC.

Select Primary Master Option (N=Skip) : N OPTIONSSIZECYLSHEADPRECOMPLANDZSECTORMODE 1 (Y)426826166553582563NORMAL HARD DISK TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE Primary Master

|  | This utility only detects one set of parameters for your IDE hard drive. Some IDE drives can use more than one set of parameters. If your hard disk is formatted using different parameters than those detected, you have to enter the parameters manually. If the parameters listed don't match the ones used to format the disk, the information on that disk won't be accessible. If the auto-detected parameters displayed do not match those that used for your drive, ignore them. Press N to reject the values and enter the correct ones manually from the Standard CMOS Setup screen. |
| --- | --- |

# Save & Exit Setup

This function automatically saves all CMOS values before leaving Setup.

# Exit without Saving

Use this function to exit Setup without saving the CMOS value changes.  Do not use this option if you want to save the new configuration.