This chapter tells how to set jumpers, upgrade system memory, add expansion boards, and install the system board.

Install the CPU, memory, and set the jumpers before you install the board inside a housing.  You may add the other components after installing the board.  Read this chapter to learn about the components before you install them.

# ESD Precautions

Electrostatic discharge (ESD) can damage your CPU, disk drives, expansion boards, and other components.  Always observe the following precautions before you install a system component.

1.	Do not remove a component from its protective packaging until you are ready to install it.

2.	Wear a wrist grounding strap and attach it to a metal part of the system unit before handling components.  If a wrist strap is not available, maintain contact with the system unit throughout any procedure requiring ESD protection.

# Installing a Microprocessor

The motherboard has a zero-insertion force microprocessor socket that allows you to install a Pentium CPU without using any tools.

Follow these steps to install a Pentium CPU in a ZIF-type upgrade socket:

|  | Make sure that the system power is OFF before installing a component. |
| --- | --- |

1.	Attach the heatsink and the fan to the CPU.

2.	Pull up the socket lever.

3.	Insert the CPU with the attached heatsink and fan.  Make sure that pin 1 of the CPU aligns with the hole 1 of the socket.  The notched corner on the CPU indicates pin 1.

4.	Pull down the socket lever to lock the CPU into the socket.

Step 2

Step 3

Step 4

5.	Plug the fan cable into the fan connector.  See the section Connectors for details on the fan connector.

6.	Set the jumpers accordingly.  See the following sections for the correct jumper settings.

# Upgrading the Microprocessor

Follow these steps to upgrade the Pentium CPU from 75 MHz to 90,100, 120, 133, 150 or 166 MHz:

1.	Turn off the system power.

2.	Remove the housing cover and locate the CPU socket on the system board.

3.	Pull up the socket lever.

4.	Remove the installed CPU.

5.	Install the upgrade CPU.  Refer to the section Installing a Microprocessor for instructions on how to install a Pentium CPU.

6.	Set the jumpers accordingly.  See the following sections for the correct jumper settings.

# Jumper Settings

You have to change the jumper settings when you reconfigure the system.  This section tells how to set the jumpers.  The figure below shows the jumper locations.

Set a jumper switch as follows:

- To close a jumper, insert the plastic jumper cap over two pins of a jumper.

- To open a jumper, remove the jumper cap.

The following conventions are used to represent the proper jumper settings:

Open

Closed  (1-2 position)

## Selecting the CPU Type

The jumpers JP9, JP10, JP11 and JP13 let you select the CPU type.  The available settings are as follows:

CPU TYPE    JP9        JP10 JP11    JP13

P54C-75

P54C-90

P54C-100

P54C/CS/CQS-120

P54C/CS/CQS-133

P54CS/CQS-150

P54CS/CQS-166

## Selecting the Memory Mode

The system board supports both the EDO and the Fast-page Mode memory features.  If you want to install memory with EDO features, you must set jumper JP6 to 2-3.  Otherwise, set the jumper to 1-2 for the Fast-page Mode feature.

Mode SelectJP6

Fast Page

EDO

## Setting the Cache Size

The motherboard comes either with a 256-KB pipeline-burst cache or a 256-KB/512-KB asynchronous cache.  The pipeline-burst cache improves system performance by reducing the standard processing time.

See the following figure for the location of the second-level cache.

256-KB pipelined-burst cache

256-KB/512-KB asynchronous cache

See the table below for the various cache configurations.

| Cache Size | Tag SRAM (U36) (SOJ-type, 15 ns) | Data SRAM (U53, U54, U55, U56, U58, U59, U60, U61) |
| --- | --- | --- |
| 256 KB | 32 Kb x 8 x 1 pc. or 16 Kb x 8 x 1 pc. | 32 Kb x 8 x 8 pcs (SOJ type, 15 ns) |
| 512 KB | 32 Kb x 8 x 1 pc. or 16 Kb x 8 x 1 pc. | 64 Kb x 8 x 8 pcs (SOJ type, 15 ns) |
| 256 KB (p.b.) | 32 Kb x 8 x 1 pc or 16 Kb x 8 x 1 pc. | 32 Kb x 32 x 2 pcs (QFP type, 8 ns) |

Also, you must reset jumper JP14 if you upgrade the cache.  See the figure below for the correct jumper settings.

CACHE SIZE           JP14

256 KB

512 KB

## Selecting the Flash ROM Type

Set the jumper JP7 according to the Flash ROM type.  If you use a 5V Flash ROM, then you must close pins 2-3 of JP7.  For a 12V Flash ROM, the required setting is 1-2.  The default setting is 2-3.

Flash ROM TypeJP7

12 V

5 V

## Enabling the Onboard Super I/O Controller

The onboard super I/O controller is SMC37C665GT.  If you want to enable or disable the onboard I/O controller you must reset the jumper marked JP8 on the system board.

JP8

Enabled

Disabled

## Selecting the ECP DMA Channel

The jumpers JP3 and JP4 let you select the DMA channel for ECP function.  The channel selections are DMA 1 and DMA 3.

JP3      JP4

DMA 1

DMA 3

## Enabling the VGA

The VGA chip onboard is S3 Trio64.  To enable the VGA chip, set jumper JP5 to 1-2.  Otherwise, set it to 2-3.

VGA JP5

Enabled

Disabled

## Selecting the Audio Output

You may direct your audio output to line out or speaker out.  If you select speaker out, the audio signal passes through the onboard amplifier module before output.  Selecting line out lets you bypass the amplifier module and allows you to use an external amplifier.

You must set jumpers JP1 and JP2 according to the selected audio output.  See the figure for the proper jumper settings.

Audio Output         JP1        JP2

Line out

Speaker out

## Selecting the Audio I/O Address

The jumpers J1 and J2 let you select the I/O address for audio function.  The available selections are 22XH, 24XH, 26XH, and 28XH.  See the following figure for the corresponding settings for each address.

Audio I/O Address   J1         J2

22XH

24XH

26XH

28XH

## Clearing the CMOS

The jumper JP12 clears the values in the CMOS.  You need to clear the CMOS if you forget your system password.  To do this, shut off the system power and short pins 2-3 of JP12 for a few seconds.  Reset the jumper to the normal setting by shorting pins 1-2 with a jumper cap.  Enter Setup to specify a new password.

JP12

Default  (Normal)

Clearing CMOS

# Memory Configuration

The system board supports a maximum memory of 128 MB.   The four 72-pin SIMM sockets accommodate 4- and 16-MB single-density SIMMs, and 8- and 32-MB double-density SIMMs - with or without the Extended Data Out (EDO) function.  The EDO feature expands data output efficiency (speed), thus improving memory performance.  All SIMMs support a DRAM speed of 70/60 ns (or less).  Refer to the section Board Layout and see the figure for the location of the SIMMs.

The table below lists the SIMM types and their corresponding capacities.

| SIMM Type | Capacity |
| --- | --- |
| 1 Mb x 32/36 | 4 MB |
| 2 Mb x 32/36 | 8 MB |
| 4 Mb x 32/36 | 16 MB |
| 8 Mb x 32/36 | 32 MB |

The following are the possible SIMM configurations.

| Total Memory | SIMM 0/1 | SIMM 2/3 |
| --- | --- | --- |
| 8 MB | 4 MB x 2 |  |
| 16 MB | 4 MB x 2 | 4 MB x 2 |
| 16 MB | 8 MB x 2 |  |
| 32 MB | 8 MB x 2 | 8 MB x 2 |
| 32 MB | 16 MB x 2 |  |
| 40 MB | 16 MB x 2 | 4 MB x 2 |
| 48 MB | 16 MB x 2 | 8 MB x 2 |
| 64 MB | 16 MB x 2 | 16 MB x 2 |
| 64 MB | 32 MB x 2 |  |
| 72 MB | 32 MB x 2 | 4 MB x 2 |
| 80 MB | 32 MB x 2 | 8 MB x 2 |
| 96 MB | 32 MB x 2 | 16 MB x 2 |
| 128 MB | 32 MB x 2 | 32 MB x 2 |

## Installing a SIMM

|  | Observe the ESD precautions when installing components. |
| --- | --- |

Follow these steps to install a SIMM:

1.	Slip a SIMM at a 45o angle into a socket with the component side facing down.  Always install SIMMs beginning with SIMM 0.

|  | Be careful when inserting or removing SIMMs. Forcing a SIMM in or out of a socket can damage the socket or the SIMM (or both). |
| --- | --- |

2.	Gently push the SIMM up until the pegs of the socket slip into the holes on the SIMM and the holding clips lock the SIMM into a vertical position.

|  | The SIMM should be at a 90o angle when installed. |
| --- | --- |

## Removing a SIMM

1.	Press the holding clips on both sides of the SIMM outward to release it.

2.	Press the SIMM downward to about a 45o angle.

3.	Gently pull the SIMM out of the socket.

# Connectors

## Multifunction Connector

This 20-pin connector is marked CN21 on the system board.  It supports a number of system functions:  green mode LED, power LED, break switch, keylock, and speaker.  Attach the front panel connectors to the corresponding pins as in the illustration below.

Speaker          Power LED   Keylock

Some housings have a five-pin connector for the keylock and power LED.  See the following illustration.

Speaker            Keylock and Power LED

Other housings may have a 12-pin connector.  If your housing has this type of connector, plug it into CN21 as shown in the following figure.  Make sure that the red wire of the connector connects to pin 11.

## Break Switch

The break switch gives the user the option to directly enter the system suspend mode by setting the switch to the on position.  To set, simply press the switch.  Make sure that the break switch is in the off position before you set it to the on position.  To set it to the off position, simply press the switch to release it from the on position.

## Keyboard and Mouse Connectors

The board accepts PS/2 keyboard and mouse connectors.  The following figure shows how to connect a keyboard (or mouse).

## Power Connector

A standard power supply has two cables with six wires each.  Attach these cables to the power connector on the board in such a way that all the black wires are in the center.  The power connector is marked CN13 on the system board.

## Fan Connector

The 2-pin fan connector is marked CN20 on the system board.  The figure below shows the pin configuration of the connector.

2-pin fan connector (CN20)

GND

# Installation

The LP5 board conforms to the LPX standard form factor.  It has mounting holes that fit LPX housings.  However, before installing the system board, make sure that the housing accommodates a LPX board with long opening in the rear panel for the onboard connectors.

Some housings may differ slightly in design, requiring additional steps to install the board.  Read the documentation that comes with the housing.

|  | Make sure that you have already installed the system board components like the CPU and memory, and have set the appropriate jumpers before you proceed. |
| --- | --- |

## Installing the System Board

1.	Open the system housing.

2.	The system board comes with a bracket and hex screws.  Attach the bracket to the board with the hex screws.  See the figure below.

3.	Use the screws that come with the housing to secure the board.

4.	Attach the power supply cables to the power connector and the front panel connectors to the multifunction connector.  See the section Connectors.

5.	Install any additional components that you have not yet installed.

## Installing a Riser Card

Follow these steps to install a riser card:

1.	Observe the ESD precautions before removing the riser card from its protective packaging.

2.	Locate the riser card slot on the system board.  See the section Board Layout.

3.	Gently insert the golden edge of the riser card into the slot until it fits.

riser card

motherboard

riser card slot

4.	Make sure that the riser card is properly seated.

## Installing Expansion Boards

Install any expansion boards after you have secured the system board and the riser card in the housing.

Follow these steps to install an expansion board.

1.	Observe the ESD precautions before removing the expansion card from its protective packaging.

2.	Remove the bracket opposite the slot that you want to use.  Save the cover for future use.  Save the screw to secure the expansion board.

3.	Remove the board from its protective packaging.

4.	Gently insert the golden edge of the board into the slot on the riser card until it fits into place.

| PCI card ISA card |  |
| --- | --- |

5.	Secure the board bracket with the screw.

## Applying the Port Indicator Sticker

The board comes with a silver sticker that serves as port indicator.  It has the following indicators on it.

MODEM K/B MOUSE COM1 MIC VOL LINE SPK MIDI/GAME VGA

To apply the sticker:

1.	Remove the protective paper backing from the sticker.

2.	Position the sticker over the rear panel, just above the connectors.  Align each indicator with the connectors on the board.  Press the sticker evenly to adhere.

REAR PANEL

STICKER

CONNECTORS