# AMI BIOS Setup Main Menu

The AMI BIOS Setup Main Menu appears below.  Press c during POST to enter the BIOS Setup.

The AMI BIOS is in Windows form.  You can use either the keyboard or a mouse to move between the items.  To select among the Setup groups, use v to highlight the selected group or simply click on the icon of the selected Setup menu.

To select among the options, you can either use the arrow keys to move the highlight bar or simply click on the icon of the desired option.

After selecting, press e or double-click on the icon to open the menu.

# Standard CMOS Setup

Highlight Setup using v or simply click on the Setup icon.  Select Standard to input configuration values such as the date, time, and disk types.  The Standard CMOS Setup pop-up window appears below:

## Date/Time

To set the date and time, highlight Date/Time and press e or double-click on the Date/Time icon.  The following screen appears:

Use the arrow keys to move among the items.  Press the + and - keys or click the + and - icons to set the current date and time.  Close the window by pressing ^ or double-clicking the Control menu box in the upper-left corner of the window.

## Floppy Drives A and B

To configure the floppy drive, select Floppy A.  The following values appear on the screen:

After selecting the proper setting, press ^ or double-click the Control menu box to close the window.

Select Floppy B and follow the same procedure to configure the second floppy drive, if present.

## Hard Disk Drives

Select Master Hard Disk to configure the first hard disk.  The following values appear on the screen:

If you cannot find your hard disk drive type on the list, select User and enter the disk parameters.  You can also select Utility Setup.  This automatically configures your hard disk.  Refer to the section Utility Setup for more information.

Select ESDI or SCSI depending the device installed.

If you have two hard disks installed, select Slave Disk and follow the same procedure to configure the second hard disk.

# Advanced CMOS Setup

The window below appears if you select the Advanced option.

The screen above does not show all the parameters of the Advanced Configuration menu.  Use w or y to highlight the desired parameter.  Press } to view the rest of the parameters.  The following screens appear:

## Typematic Rate (Chars./Sec.)

This parameter determines the typematic rate.  The typematic rate settings are 15, 20, 30 and Disabled.  The default setting is 30.  Select Disabled to disregard the rate setting.

## System Keyboard

Set this parameter to Present if there is a keyboard connected to the system.  However, some servers may not have keyboards.   Select Absent if there is no keyboard present.

## Primary Display

This function detects the type of VGA in use.  The settings are VGA/EGA, CGA 40 x 25, CGA 80 x 25, Mono, and Absent.  The default setting is VGA/EGA.

## PS/2 Mouse Support

Setting this parameter to Enabled lets you support a PS/2 mouse.  Disable the parameter if you are under the UNIX X-window environment.

## Above 1 MB Memory Test

This parameter allows your system to check all available memory.  Therefore, setting this parameter to Enabled slows down the power-on self-test.  The default setting is Disabled.

## Memory Test Tick Sound

Enabling this parameter lets you hear the tick sound during the memory test.  Disable the parameter to bypass the function.

## Hit “Del” Message Display

This option lets you enable or disable the Hit <Del> if you want Setup message from appearing when the system boots.  The default setting is Enabled.

## Extended BIOS RAM Area

This function allows you to relocate the BIOS from ROM to RAM.  Relocating to RAM enhances system performance as information access is faster than ROM.  The parameter settings are 0:300 and DOS 1K.  The default address is 0:300.

## Wait for F1 If Any Error

When enabled, the BIOS waits for the end user to press l before continuing.  If disabled, the BIOS continues the boot process without waiting for l to be pressed.  The default setting is Enabled.

## System Boot-up Num Lock

Setting this parameter to On enables the numeric function of the numeric keypad.  Set this parameter to Off to disregard the function.  Disabling the numeric function allows you to use the numeric keypad for cursor control.  The default setting is On.

## Floppy Drive Seek at Boot

When enabled, the BIOS detects whether there is a floppy disk drive installed.  Disable the parameter to bypass the function.  The default setting is Disabled.

## Floppy Drive Swapping

This parameter allows you to swap floppy drives.  For example, if you have two floppy drives (A and B), you can assign the first drive as drive B and the second drive as drive A or vice-versa.  Disable the parameter to bypass the function.  The default is Disabled.

## System Boot-up Sequence

The settings are C:, A: and A:, C: to specify the system search sequence.  The default setting is A:,C:.

## Password Checking

The settings are Setup and Always.  The Setup setting allows the system to boot and use the password to protect the Setup Utility Configuration settings from being tampered with.  The Always setting requires you to enter the password everytime you boot the system.  The default setting is Setup.

## Cache Memory

The available selections for this parameter are Internal, Both and Disabled.  Select Internal if you want to enable the internal cache memory.  Select Both if you want to use both the internal and external cache memories.  Select Disabled to disregard the internal and external cache features.

## System BIOS Shadow Cacheable

The default setting for this parameter is Enabled.  This enhances the system performance.  Disabling the parameter prevents the system BIOS from being cached.

## Adapter ROM Control

### C000, 32 K

This address is for shadowing video ROMs.  Select Shadow to assign the address for shadowing expansion video card with ROM.  Select Cache to assign them for cache.  The default setting is Cache.

### C800 ~ DC00, 16 K

These addresses are for shadowing other expansion card ROMs.  The default setting for these areas is Disabled.  Set the addresses to Shadow if you want to use them for shadowing expansion cards with ROM.  Set the addresses to Cache to assign them for cache.

|  | The F000 and E000 addresses are exclusively shadowed for BIOS. |
| --- | --- |

## IDE Control

### IDE Auto-detect

There are cases wherein the HDD parameters that you entered and those detected by the auto-detection function are mismatched.  This causes the system not to boot.  If this happens, we recommend that you set this parameter to Disabled to bypass the auto-detection function.  The default setting is Enabled.

### Block Mode

This function enhances disk performance depending on the hard disk in use.  This parameter is normally set to Auto.  This setting allows data transfer in block (multiple sectors) by increasing the data transfer rate.  The other selections for this parameter are 2 S/B, 4 S/B, 8 S/B, 16 S/B, 32 S/B, 64 S/B and Disabled.  Disable the parameter if your hard disk does not support this feature.

### Onboard IDE Present

The settings for this function are Auto, Enabled and Disabled.  Select Enabled if you have an onboard PCI IDE.  Select Auto to automatically detect the presence of PCI IDE.  Select Disabled to disregard the PCI-IDE function.

### PIO Mode

This parameter lets you set the PIO mode that your onboard PCI IDE supports.  The selections are from Mode 0 to Mode 4, Auto and Disabled.  Set this parameter either by entering the PIO mode manually or selecting Auto to automatically detect the supported PIO mode.  Disable the parameter to bypass the feature.

### 32-bit Mode

Enabling this function improves the hard disk performance by increasing the data transfer rate from 16-bit to 32-bit.  The data transfer rate is auto-detected by BIOS.

### Primary 1st LBA Mode

This enhanced IDE feature allows you to use a hard disk with a capacity higher than 528 MB.  This is made possible through the Logical Block Address (LBA) mode translation.  This parameter affects the primary IDE hard disk drive connected to the IDE 1 connector.  The default setting is Disabled.

### Primary 2nd LBA Mode

This enhanced IDE feature allows you to use a hard disk with a capacity higher than 528 MB.  This is made possible through the Logical Block Address (LBA) mode translation.  This parameter affects the secondary IDE hard disk drive connected to the IDE 1 connector.  The default setting is Disabled.

### Secondary 1st LBA Mode

This enhanced IDE feature allows you to use a hard disk with a capacity higher than 528 MB.  This is made possible through the Logical Block Address (LBA) mode translation.  This parameter affects the primary IDE hard disk drive connected to the IDE 2 connector.  The default setting is Disabled.

### Secondary 2nd LBA Mode

This enhanced IDE feature allows you to use a hard disk with a capacity higher than 528 MB.  This is made possible through the Logical Block Address (LBA) mode translation.  This parameter affects the secondary IDE hard disk drive connected to the IDE 2 connector.  The default setting is Disabled.

## Secondary Drives Present

This parameter lets you install up to two IDE hard disks in the secondary channel.  Select None if you do not have any.

## Chipset Setup Mode

This function allows you to change the Chipset Setup DRAM control parameters according to the end-user type.  The available settings are End-user and Engineer.  We recommend that you select End-user.  See the following section for more details on Chipset Features Setup.

# Chipset Features Setup

The Chipset Features Setup controls the board's chipset settings.  The controls for this menu are the same as for the previous screen.

The Chipset Setup DRAM control parameters differ depending on the Chipset Setup Mode setting in the Advanced CMOS Setup.  This screen appears if you select the Chipset option from the Setup menu and if the Chipset Setup Mode parameter setting is End-user.

The following screen appears if your Chipset Setup Mode parameter setting is Engineer.  Take note of the DRAM control parameters.

Both screens do not show all the parameters of the Chipset Setup menu.  Use w or y to highlight the desired parameter.  Press } to view the rest of the parameters.  The following screens appear regardless of the end-user type or the Chipset Setup Mode parameter setting:

|  | This manual describes only the End-user setting parameters. |
| --- | --- |

## DRAM Control

### Speed

This DRAM control parameter lets you set the DRAM speed.  The speed settings are 60 ns and 70 ns.  The default setting is 70 ns.

### Memory Hole

This option lets you assign the system memory area to avoid memory conflicts.  The settings are 512 ~ 640 K, 15 ~ 16 M and Disabled.

## ISA Control

### 8-bit I/O Recovery Time

This parameter allows you to set the response time of the 8-bit I/O connected to your system.  The range is from 1~7 SYSCLK.  The default setting is 4 SYSCLK.

### 16-bit I/O Recovery Time

This parameter allows you to set the response time of the 16-bit I/O connected to your system.  The range is from 1~4 SYSCLK.  The default setting is 1 SYSCLK.

## PCI Control

### VGA Palette Snooping

PCI devices support the “palette snooping” technique that enables the device to control access to their palette registers.

Set this parameter to Enabled to activate the palette snooping function in the PCI VGA devices installed in your system.

### PCI-IDE Card Selection

This parameter allows you to select the PCI-IDE card that you want to enable.  The board supports a maximum of four PCI-IDE cards.  The available selections are Slot 1, Slot 2, Slot 3, Slot 4, and Absent.  Select Absent if you do not have a PCI card installed.

### PCI Primary IDE INT# Line

This parameter lets you assign an INT for the IDE device connected to your primary IDE connector.  The settings are INT A, INT B, INT C, INT D, Absent and Not Used.  If you do not have a PCI-IDE card installed in your system and your PCI-IDE Card Selection parameter setting is Absent, this parameter becomes non-configurable.

### PCI Secondary IDE INT# Line

This parameter lets you assign an INT for the IDE device connected to your secondary IDE connector.  The settings are INT A, INT B, INT C, INT D, Absent and Not Used.  If you do not have a PCI-IDE card installed in your system and your PCI-IDE Card Selection parameter setting is Absent, this parameter becomes non-configurable.

## IRQ Allocated

### IRQ 3, 4, 5, 7, 9, 10, 11, 14, 15 is for...

These lines allow you to assign the available IRQs to either ISA or PCI/PnP devices.

# Power Management Setup

To take advantage of the power management features, select Power Management from the Setup menu.  To select, highlight Power Mgmt and press e or double-click on the Power Management icon.  The following screen appears:

The screen above does not show all the parameters of the Power Management Setup menu.  Use w or y to highlight the desired parameter.  Press } to view the remaining parameters.  The following screens appear:

## Advanced Power Management

Set this parameter to Enabled to take advantage of the power-saving feature.  Disable the parameter to bypass the feature.

## Full-on to Standby Timeout

This function lets you determine when to put the system into standby mode.  In standby mode, the CPU clock slows down and the VGA suspends the video signal.  Any events detected returns the system to full power.  The settings range from 1~255 Min. and Disabled.

## Standby to Suspend Timeout

This function lets you specify when to put the system into suspend mode.  In suspend mode. the CPU clock stops, the IDE hard disk spins down and the VGA suspends video signal.  This mode conserves the most power.  Any events detected returns the system to full power.  The settings range from 1~255 Min. and Disabled.

## IDE Drive Power Down in

This option allows you to specify the mode when to "spin down" your IDE hard disk.  The disk returns to full speed once the system resumes to normal mode.  The settings are Standby, Suspend and Disabled.

## Video Power Down in

This option allows you to set the mode when to power down your video monitor.  The video monitor returns to full power once the system returns to normal mode.  The settings are Standby, Suspend and Disabled.

## VGA Power Down Mode

This option lets you choose the VGA power down mode.  The settings are Standby, Suspend and Off.

## Slow Clock Ratio

When the system enters the standby mode, the CPU clock starts to slow down.  This parameter lets you set the “slow-down” clock ratio.  The settings are 1:2, 1:4, 1:8, 1:16, 1:32, 1:64, and 1:128.

## Break Event (IRQ 3~7, 9~15)

Enabling these parameters allows your system to monitor the IRQ activities.  Any activity detected resets the power-management timers and returns the system to normal mode.

|  | You must enable at least one IRQ activity. Otherwise, the system stays in suspend mode. Under Windows 95, do not disable the parameter IRQ 12. Otherwise, the system disregards any mouse or keyboard activity and stays in power-saving mode. |
| --- | --- |

# Peripheral Setup

This screen appears if you select Peripherals or double-click on the Peripheral Setup icon from the Setup menu.  The Peripheral Setup screen allows you to set up your system peripherals.

## Programming Mode

The settings for this option are Auto and Manual.  The Manual setting allows you to set up the screen items manually.  The Auto setting sets up all the items automatically except for the Parallel Port Mode parameter.

## Onboard FDC

Enabling this function allows you to use the onboard floppy disk controller (FDC).  The default setting is Enabled.

## Serial Port 1

This parameter allows you to set the base address of serial port 1.  The available settings are 3F8H, 2F8H, 3E8H, 2E8H and Disabled.

## Serial Port 2

This parameter allows you to set the base address of serial port 2.  The available settings are 3F8H, 2F8H, 3E8H, 2E8H and Disabled.

## Parallel Port

This parameter allows you to set the base address of the parallel port.  The available settings are 3BCH, 378H, 278H and Disabled.

## IRQ Active

This option specifies if the parallel and serial port IRQs are active high or active low.  The settings are High and Low.

## Parallel Port Mode

This option lets you set the parallel port mode.  The settings are Normal or Extended.

# Utility Setup

## IDE Setup

This function allows your system to automatically configure your IDE hard disk(s).  This screen appears if you select IDE Setup.

After a few seconds, the screen below appears showing your disk(s) parameters.  Select Yes to accept the values.

## Color Set

This pop-up window appears if you select Color Set from the Utility Setup menu.

Color Set lets you specify the color of your windows background.  The selections are LCD, Army, Pastel, and Sky.

# Default Setup

Select this option to automatically set your system configuration parameters.  To select, highlight Default and press e.

## Original

This option loads the values that you saved before shutting off the system.  The following prompt appears if you choose Original from the Default Setup menu.  Select Yes to load the original values.

## Optimal

Choose this option and the BIOS configures the system using the best-case values to optimize system performance.  However, these values may not be applicable to your system.  If your system does not boot after choosing this setting, reconfigure it using the Fail-safe settings.  Refer to the following section.

The screen below appears if you choose Optimal from the Default Setup menu.  Select Yes to load the optimum values.

## Fail-safe

Choose this option and the BIOS automatically configures the system using the most stable settings.  These settings are not necessarily the best settings for system performance, but they are safe and stable enough to guarantee that your system will boot.  This is useful if you are having problems with your current system configuration and need to determine the cause.

A prompt appears if you choose Fail-safe from the Default Setup menu.  Select Yes to load the fail-safe values.

# Security Setup

## Password

The system password prevents unauthorized use of your computer.  If you enabled the password feature, it is impossible to boot the computer without entering the password.

To set a password, highlight Password or simply double-click the Password icon.  The following screen appears:

Your password can consist of up to six characters.  The password does not appear on the screen.  WinBIOS prompts you to retype the password.  The following screen appears.

If you forget your password, you must clear the CMOS RAM and reconfigure the system.

To disable the password, press e when prompted for your password.  Press e again when prompted to retype the password.

## Anti-virus

Set this parameter to Enabled to protect the boot sector and partition table of your hard disk from virus intrusion.  Set it to Disabled to bypass the feature.

A prompt appears when you select Anti-virus from the Security Setup menu:

Select Enabled and the screen below appears:

# Exit Setup

To exit Setup, you can either double-click on the Control menu box or simply press ^.  A dialog box appears on the screen.

If you select Save Changes and Exit, BIOS automatically saves all CMOS values before leaving Setup.  Select Do Not Save Changes and Exit to exit Setup without saving the CMOS values.  Select Continue to return to Setup if you want to reconfigure your system.