# INTRODUCTION

## SYSTEM OVERVIEW

The【PT-5IVH】Pentium PCI Local Bus system board is designed based on the Intel 82430VX PCIset system chipset and Winbond I/O chipset, which built-in two channel PIO and Bus Master Enhanced PCI IDE ports, one Floppy Disk control port, two high speed Serial ports (UARTs) and one multimode Parallel port and also supports IR and USB ports. It is designed to fit a high performance, Pentium 75 MHz to 233 MHz based solution for high-end and true GREEN-PC computer systems.

This system board supports the Peripheral Component Interconnect (PCI) Local Bus standard (PCI Specification Rev. 2.1 compliant). It not only breaks through the I/O bottlenecks of the traditional ISA system board, but also provides the performance needs for networking and multi-user environments.

## FEATURES

The PT-5IVH system board contains the following features:

□ 	Pentium P54C/P54CS/P54CTB/P55C, Cyrix 6x86 / 6x86L & AMD K5 based,

PC/AT compatible system board with ISA Bus and PCI Local Bus.

□ 	Supports the most part of "586" level CPUs designed by Intel, Cyrix and AMD.

□ 	Built-in 2 set voltage regulator circuits to support multi-specification CPU I/O

voltage (+3.3V DC and +3.52V DC) and CPU CORE voltage (+2.8V DC).

□ 	DRAM Memory	:	Supports fast page mode (FPM), Extended Data Out (EDO) and

SDRAM (synchronous) memory.

□ 	Cache Memory	:	Supports pipelined burst SRAMs. Cache size up to 512KB.

□ 	BIOS	:	Supports Plug and Play BIOS.

□ 	IDE ports	:	Supports two channel PIO and Bus Master Enhanced PCI IDE

ports, up to Mode 4 timing, and up to 22 MBytes/s transfer rates.

□ 	I/O ports	:	Supports two high speed serial ports (UARTs), One multimode

parallel port for standard (SPP), enhanced (EPP) and high speed

(ECP) modes. One Floppy Disk Control port.

□ 	IR Port 	:	Supports IR (Infrared Rays) functions. Both HPSIR and ASKIR

are supported.

□ 	USB Ports	:	Supports two Universal Serial Bus (USB) ports.

□ 	Software	:	MS-DOS, WindowsNT, OS2, XENIX, UNIX, NOVELL, CAD

compatibility		/ CAM, Windows, Windows 95..etc. full tested.

# SPECIFICATIONS

□	Chipset

Intel 82437VX, 82438VX, 82371SB and Winbond W83977F.

□	CPU

Intel		:	Pentium processor and OverDrive processor (P54C / P54CS / P54CTB /

P55C) 75 / 90 / 100 / 120 / 133 / 150 / 166 / 180 / 200 / 233 MHz.

Cyrix	:	6x86 / 6x86L - P120+ / P133+ / P150+ / P166+

AMD	:	K5-PR75 / PR90 / PR100 / PR120 / PR133 / PR150 / PR166.

□	CPU  VCC

(1).	CPU I/O voltage	:	+3.3V DC and +3.52V DC.

(2).	CPU CORE voltage	:	+2.8V DC.

□	System  Bus  Clock

50 / 55 / 60 / 66.6 MHz adjustable.

□	Memory

DRAM	:	Two banks, each bank could be single or double sided, 8MB up to

128 MB. Supports fast page mode (FPM) , Extended Data Out (EDO)

and SDRAM memory (Using  72-pin SIMM module x 4, and 168-pin

DIMM module x 2). DIMMs' operating voltage supports +5V DC or

+3.3V DC adjustable. 5 Qword deep buffer for posted write, and

Dword and Burst merging.

SRAM		:	256KB or 512KB pipelined burst SRAM on board , supports

Direct-Mapped write back cache memory.

□	BIOS

AWARD System BIOS. 128KBx8 Flash ROM (for Plug & Play BIOS).

□	Expansion  Slots

PCI Slots	:	32-bit x 4 (All Master/Slave, PCI 2.1 Compliant).

ISA Slots	:	16-bit x 4 (One slot PCI/ISA shared).

□	IDE  Ports

Two channel PIO and Bus Master PCI IDE ports, maximum could be connected to

4 IDE Hard Disk and ATAPI CD-ROM device. Supports up to Mode 4 timing, and

up to 22 MBytes/s transfer rates.

□	Super  I/O  Ports

1. Two high speed NS16C550 compatible serial ports (UARTs).

2. One SPP/EPP/ECP mode Bi-directional parallel port.

3. One Floppy Disk Control port.

□	IR Port

One HPSIR and ASKIR compatible Infrared port.

□	Mouse and Keyboard

Supports PS/2 Mouse connector, PS/2 Keyboard connector (optional) and AT

Keyboard connector.

□	USB  Ports

Two Universal Serial Bus (USB) ports.

□	DIMENSION

Width & Length	:	220 mm x 255 mm.

Height	:	3/4 inches with components mounted, but without expansion

boards and cables.

PCB Thickness	:	4 layers, 0.05 inches normal.

Weight	:	20 ounces.

□	ENVIRONMENT

Operating Temperature	:	10℃ to 40℃. (50℉ to 104℉)

Required Airflow	:	50 linear feet per minute across CPU.

Storage Temperature	:	- 40℃ to 70℃. (- 40℉ to 158℉)

Humidity	:	0 to 90% noncondensing.

Altitude	:	0 to 10,000 feet.

# SYSTEM  BOARD  LAYOUT

## PT-5IVH  VER. 2.x

Explanation	:	All connectors, jumpers and components which marks by a black point on

the corner means the pin-1 side of the connector, jumper and component.

# HARDWARE  SETUP

## UNPACKING

The system board package should contain the following parts :

‧	The PT-5IVH system board.

‧	OPERATION MANUAL.

‧	Cable set for IDE and I/O device.

## HARDWARE  CONFIGURATION

Before the system board is ready to operate, the hardware must be configured to allow for various functions within the system. To configure the PT-5IVH system board is a simple task, only a few jumpers, connectors, cables and sockets needs to be selected and installed. (For the detailed locations of each component please refer to the "system board layout figure" which appears in page 3-1.)

### DRAM  INSTALLATION

The PT-5IVH system board will support two banks main memory (bank0 and bank 1) on board, (using four 72-Pin SIMM socket, SIMM 1 - 4 and two 168-pin DIMM socket, DIMM 1-2) each bank could be single-sided or double-sided, 8MB up to 128 MB of local memory can be attained. It supports standard fast page mode (FPM), Extended Data Out (EDO) and synchronous (SDRAM) memory.

The usable DRAM modules are :	(Note : S = Single-sided , D = Double-sided)

(1)	FPM and EDO memory	:	1MBx32(36)-S (  4MB),	1MBx32(36)-D (  8MB),

2MBx32(36)-S (  8MB),	2MBx32(36)-D (16MB),

4MBx32(36)-S (16MB),	4MBx32(36)-D (32MB).

(2)	SDRAM memory	:	1MBx64-S (8MB),	2MBx64-D (16MB),

4MBx64-S (32MB).

The speed of FPM DRAMs must be used 70ns or faster than 70ns, the speed of EDO DRAMs and SDRAMs must be used 60ns or faster than 60ns.

SIMMs' and DIMM's operating voltage :

(1)	SIMM1 - SIMM4	:	+ 5V DC

(2)	DIMM1 - DIMM2	:	+3.3V DC or +5V DC

(factory default at +3.3V DC, adjusted by “purchase order”)

DRAM bank can be populated in any order (bank 0 does not have to be populated before bank 1). Within any given bank, the SIMMs (DIMMs) must be the same size. Among the two banks, SIMM (DIMM) densities can be mixed in any order. EDO, FPM and SDRAMs can be mixed between SIMMs (DIMM), a given SIMM (DIMM) must contain only one type of DRAM. When DRAM types are mixed each SIMM (DIMM) runs optimized for that particular type of DRAM.

There is no jumper needed for DRAM configuration, DRAMs' type and size will be detected by system BIOS automatically.

The following table provides the possable combinations for DRAM memory installation.

| Bank 0 |  | Bank 1 |  |
| --- | --- | --- | --- |
| SIMM1 and SIMM2 | DIMM1 | SIMM3 and SIMM4 | DIMM2 |
| Single-sided | Empty | Single-sided | Empty |
| Double-sided | Empty | Double-sided | Empty |
| Empty | Single-sided | Empty | Single-sided |
| Empty | Double-sided | Empty | Double-sided |

Note	:	We strong recommend, Don't install 5V SIMM modules and 3.3V DIMM

modules at the same time.

### L2  CACHE  MEMORY  INSTALLATION

The PT-5IVH system board will support 256KB or 512KB cache memory on board, using synchronous pipeline-burst SRAMs. Both Write Back and Write Through cache update policy are supported.

The following table lists the detalied combination about cache memory installation.

| Cache Size | Data SRAMs (U3, U4) | Tag SRAMs (U10) |
| --- | --- | --- |
| 256 KB | 32KB x 32 | 8KB or 16KB or 32KB x 8 |
| 512 KB | 64KB x 32 | 16KB or 32KB x 8 |

### CONNECTORS

A connector is two or more pins that are used make connections to the system standard accessories (such as power, mouse, printer,...etc.) The following is a list of connectors on board, as well as descriptions of each individual connector.

(A)	BAT1	Non-Rechargeable  battery  (Using 3 Vlots Lithium battery : CR2032)

Pin #	Assignment

＋	Battery Positive

－	Ground

(B)	CN1	PS/2  Mouse  converted  connector

Pin #	Assignment	Pin #	Assignment	Pin #	Assignment

1	Mouse Data		3	Ground		5	Mouse Clock

2	No Connection		4	+5V DC

(C)	CN2	PS/2  Keyboard  connector  (optional)

Pin #	Assignment	Pin #	Assignment	Pin #	Assignment

1	Keyboard Data		3	Ground		5	Keyboard Clock

2	No Connection		4	+5V DC		6	No Connection

(D)	CN3	AT  Keyboard  connector

Pin #	Assignment	Pin #	Assignment	Pin #	Assignment

1	Keyboard Clock		3	No Connection		5	+5V DC

2	Keyboard Data		4	Ground

(E) CN4 	Power  connector

Pin #	Assignment	Pin #	Assignment	Pin #	Assignment

1	Power Good		5	Ground		9	-5V DC

2	+5V DC		6	Ground		10	+5V DC

3	+12V DC		7	Ground		11	+5V DC

4	-12V DC		8	Ground		12	+5V DC

(F)	CN5	Floppy  Disk  Control  Port  connector  (Using IRQ6, DMA channel 2)

(G)	CN6	IDE  1  connector  (Primary IDE Port, using IRQ14)

(H)	CN7	IDE  2  connector  (Secondary IDE Port, using MIRQ0)

(I)	CN8	COM  B  (Serial Port 2)  connector

COM1/2/3/4, selected by BIOS setup, using IRQ3 or 4

|  | 1 | 2 |  | Pin # | Assignment | Pin # | Assignment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ○ | ○ |  | 1 | DCD (Data Carrier Detect) | 2 | RD (Received Data) |
|  | ○ | ○ |  | 3 | TD (Transmit Data) | 4 | DTR (Data Terminal Ready) |
|  | ○ | ○ |  | 5 | Ground | 6 | DSR (Data Set Ready) |
|  | ○ | ○ |  | 7 | RTS (Request To Send) | 8 | CTS (Clear To Send) |
|  | ○ | ○ |  | 9 | RI (Ring Indicator) | 10 | NC (No Connection) |
|  | 9 | 10 |  |  |  |  |  |

(J)	CN9	Parallel  Port  connector

(Supports SPP/EPP/ECP mode, selected by BIOS setup,using IRQ7 or

IRQ5, ECP using DMA channel 3 or 1)

|  | 1 | 14 |  | Pin # | Assignment | Pin # | Assignment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ○ | ○ |  | 1 | STROBE- | 14 | AUTO FEED- |
|  | ○ | ○ |  | 2 | Data Bit 0 | 15 | ERROR- |
|  | ○ | ○ |  | 3 | Data Bit 1 | 16 | INIT- |
|  | ○ | ○ |  | 4 | Data Bit 2 | 17 | SLCT IN- |
|  | ○ | ○ |  | 5 | Data Bit 3 | 18 | Ground |
|  | ○ | ○ |  | 6 | Data Bit 4 | 19 | Ground |
|  | ○ | ○ |  | 7 | Data Bit 5 | 20 | Ground |
|  | ○ | ○ |  | 8 | Data Bit 6 | 21 | Ground |
|  | ○ | ○ |  | 9 | Data Bit 7 | 22 | Ground |
|  | ○ | ○ |  | 10 | ACK- | 23 | Ground |
|  | ○ | ○ |  | 11 | BUSY | 24 | Ground |
|  | ○ | ○ |  | 12 | PE | 25 | Ground |
|  | ○ | ○ |  | 13 | SLCT | 26 | N.C. (No Connection) |
|  | 13 | 26 |  |  |  |  |  |

(K)	CN10	COM  A  (Serial Port 1)  connector

COM1/2/3/4, selected by BIOS setup, using IRQ4 or 3

|  | 1 | 2 |  | Pin # | Assignment | Pin # | Assignment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ○ | ○ |  | 1 | DCD (Data Carrier Detect) | 2 | RD (Received Data) |
|  | ○ | ○ |  | 3 | TD (Transmit Data) | 4 | DTR (Data Terminal Ready) |
|  | ○ | ○ |  | 5 | Ground | 6 | DSR (Data Set Ready) |
|  | ○ | ○ |  | 7 | RTS (Request To Send) | 8 | CTS (Clear To Send) |
|  | ○ | ○ |  | 9 | RI (Ring Indicator) | 10 | NC (No Connection) |
|  | 9 | 10 |  |  |  |  |  |

(L)	CN11	IR  (Infrared Rays)  transmission  connector

Pin #	Assignment	Pin #	Assignment	Pin #	Assignment

1	+5V DC		3	IR Receive		5	IR Transmit

2	No Connection		4	Ground

(M)  CN12	Cooling fan power connector

Pin #	Assignment

1	+12V DC

2	Ground

(N)	CN13	USB  1  (Universal Serial Bus port1)  connector

(O)	CN14	USB  2  (Universal Serial Bus port2)  connector

(P)	RS	Reset  Button  connector

Pin #	Assignment	Pin1&2	Function

1	Reset Control	Open		No action

2	Ground	Short		System Reset

(Q)	HL	IDE  HDD  LED  connector

Pin #	Assignment

1	Pullup (+5V DC)

2	Signal Pin

(R)	TL	Turbo  LED  connector

Pin #	Assignment

1	Pullup (+5V DC)

2	Signal Pin

(S)	SS	External  SMI  button  connector

Pin #	Assignment	Pin1&2	Function

1	SMI Control	Open		For normal operation

2	Ground	Short		To get into Suspend mode

(T)	TS	Turbo  Switch  connector (This function is reserved)

Pin #	Assignment	Pin1&2	Function

1	Turbo Control	Open		Turbo

2	Ground	Short		Normal

(U)	SPEAKER	Speaker  connector

Pin #	Assignment	Pin #	Assignmen

1	+5V DC		4	Speaker Data Signal

2	No Connection		5	No Connection

3	No Connection

(V)	KEY LOCK	Front  Panal  Power  LED  &  Key-Lock  connector

Pin #	Assignment

1	Pullup (+5V DC for Power LED)

2	No Connection

3	Ground

4	Keyboard Lock

5	Ground

Explanation : ( For next section : JUMPERS )

### JUMPERS

A jumper is two, three or more pins which may or may not be covered by a plastic

connector plug (mini-jumper). A jumper is used to select different system options. Please

make sure all jumpers at correct position before this system board used.

(A)	JP4, JP5, JP6, JP7, JP8	CPU  type  selection

(1)	INTEL  Pentium  CPUs

|  | CPU Type | JP4 | JP5 | JP6 | JP7 | JP8 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 80502-75 | open | open | 1-2 | 1-2 | 1-2 |  |
|  | 80502-90 | open | open | 1-2 | 1-2 | 2-3 |  |
|  | 80502-100 | open | open | 1-2 | 2-3 | 1-2 |  |
|  | 80502-120 | open | short | 1-2 | 1-2 | 2-3 |  |
|  | 80502-133 | open | short | 1-2 | 2-3 | 1-2 |  |
|  | 80502-150 | short | short | 1-2 | 1-2 | 2-3 |  |
|  | 80502-166 / 80503-166 | short | short | 1-2 | 2-3 | 1-2 |  |
|  | 80502-180 | short | open | 1-2 | 1-2 | 2-3 |  |
|  | 80502-200 / 80503-200 | short | open | 1-2 | 2-3 | 1-2 |  |
|  | 80503-233 | open | open | 1-2 | 2-3 | 1-2 |  |

(2)	Cyrix  6x86 / 6x86L / 6x86MX  CPUs

|  | CPU Type | JP4 | JP5 | JP6 | JP7 | JP8 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6x86 / 6x86L - P120+ | open | short | 1-2 | 1-2 | 1-2 |  |
|  | 6x86 / 6x86L - P133+ | open | short | 2-3 | 1-2 | 1-2 |  |
|  | 6x86 / 6x86L - P150+ | open | short | 1-2 | 1-2 | 2-3 |  |
|  | 6x86 / 6x86L - P166+ | open | short | 1-2 | 2-3 | 1-2 |  |
|  | 6x86 / 6x86L - P200+ | open | short | 2-3 | 1-2 | 2-3 | * |
|  | 6x86 / 6x86L - P200+ | open | open | 1-2 | 1-2 | 1-2 | ** |
|  | 6x86MX-PR166 | short | short | 1-2 | 1-2 | 2-3 |  |
|  | 6x86MX-PR200 | short | short | 1-2 | 2-3 | 1-2 |  |
|  | 6x86MX-PR233 | short | open | 1-2 | 2-3 | 1-2 |  |
|  | 6x86MX-PR266 | open | open | 1-2 | 2-3 | 1-2 |  |

Remark	*	: 75Mhz x 2, Better performance,but more peripheral compatible problem.

**	: 50Mhz x 3, (Maker suggest this jumple setting).

(3)	AMD  K5 / K6  CPUs

|  | CPU Type | JP4 | JP5 | JP6 | JP7 | JP8 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | AMD-K5-PR75 | open | open | 1-2 | 1-2 | 1-2 |  |
|  | AMD-K5-PR90 | open | open | 1-2 | 1-2 | 2-3 |  |
|  | AMD-K5-PR100 | open | open | 1-2 | 2-3 | 1-2 |  |
|  | AMD-K5-PR120 | open | open | 1-2 | 1-2 | 2-3 |  |
|  | AMD-K5-PR133 | open | open | 1-2 | 2-3 | 1-2 |  |
|  | AMD-K5-PR150 | short | short | 1-2 | 1-2 | 2-3 |  |
|  | AMD-K5-PR166 | short | short | 1-2 | 2-3 | 1-2 |  |
|  | AMD-K6-PR166 | short | short | 1-2 | 2-3 | 1-2 |  |
|  | AMD-K6-PR200 | short | open | 1-2 | 2-3 | 1-2 |  |

(B)	JP10, JP11, JP12	CPU  operating  voltage  selection

Note	:	For the detailed CPU-VCC requirement, please inquire of your CPU

supplier.

|  | CPU VCC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | I/O | CORE | JP10 |  | JP11 |  | JP12 |  | Remark |  |
|  | voltage | voltage |  |  |  |  |  |  |  |  |
|  | + 3.52 V |  | 1-2 |  | 1-2 |  | 1-2 |  | * |  |
|  | + 3.3 V | + 2.8 V | 2-3 |  | 2-3 |  | 2-3 |  | ** |  |

Remark :

*	:	For the single-VCC required CPUs. (e.g. Intel P54C, Cyrix 6x86,

AMD-K5-xxAB?,AMD-K5-xxAC?, AMD-K5-xxAF? ... etc.)

**	:	For the dual-VCC required CPUs. (e.g. Intel P55C, Cyrix 6x86L,

Cyrix 6x86MX, AMD-K5-xxAH? ,AMD-K5-xxAJ? ,AMD-K6... etc.)

(C)	JP9	Clear  CMOS  switch

Pin #	Function

1-2	Normal operation

2-3	Clear  CMOS	(Note : Don't  forget to turn this jumper return

1-2 	within 3 to 5 seconds)

(D)	JP13	ROM  BIOS  selection

Pin #	Function

1-2	for +5V FLASH ROM

2-3	for +12V FLASH ROM

(E)	JP2,JP3	DIMM  voltage  selection

Pin #	Function

1-2	+5V DC

2-3	+3.3V DC	(Factory default)

# AWARD  BIOS  SETUP

## GETTING  STARTED

When the system is first powered on or reset, the BIOS will enter the Power-On Self Test routines ( POST : Display a copyright message on the screen followed by a diagnostics and initialization procedure.) (If an EGA or VGA card is installed, the copyright message of the video card maybe displayed on the screen first.) The BIOS will indicate any error or malfunction by a series of beeps or display the error message on screen.

Normally, the simulate figure 5-1 will display on the screen when the system is powered

on.

Fig. 5-1  Initial Power-On screen.

After the POST routines are completed, the following message appears :

" Press  DEL  to enter SETUP "

To execute the Award BIOS Setup program, press DEL key. The simulate screen in figure 5-2 MAIN MENU will be displayed at this time.

## MAIN  MENU

ROM  PCI / ISA  BIOS  (2A59GXXX)

CMOS  SETUP  UTILITY

AWARD  SOFTWARE,  INC.

|  | STANDARD CMOS SETUP |  | SUPERVISOR PASSWORD |
| --- | --- | --- | --- |
| BIOS FEATURES SETUP |  |  | USER PASSWORD |
| CHIPSET FEATURES SETUP |  |  | IDE HDD AUTO DETECTION |
| POWER MANAGEMENT SETUP |  |  | HDD LOW LEVEL FORMAT |
| PNP/PCI CONFIGURATION |  |  | SAVE & EXIT SETUP |
| INTEGRATED PERIPHERALS |  |  | EXIT WITHOUT SAVING |
| LOAD SETUP DEFAULTS |  |  |  |
| ESC : Quit |  |  | ↑↓→← : Select Item |
| F10 : Save & Exit Setup |  |  | (Shift)F2 : Change Color |
| Time, Date, Hard Disk Type ... |  |  |  |

Fig. 5-2 CMOS SETUP MAIN MENU screen.

## CONTROL  KEYS

Listed below is an explanation of the keys displayed at the bottom of the screens accessed through the BIOS SETUP program :

Arrow Keys	:	Use the arrow keys to move the cursor to the desired item.

Enter	:	To Select the desired item.

F1	:	Display the help screen for the selected feature.

(Shift)F2	:	To change the screen color, total 16 colors.

ESC	:	Exit to the previous screen.

PgUp(-)/PgDn(+)	:	To modify the default value of the options for the highlighted

feature.

F5	:	Retrieves the previous CMOS values from CMOS, only for the

current option page setup menu.

F6	:	Loads the BIOS default values from BIOS default table, only for

the current option page setup menu.

F7	:	Loads the SETUP default values from BIOS default table, only for

the current option page setup menu.

F10	:	Save all changes made to CMOS RAM, only for the MAIN MENU.

The following pages will show the simulate screens of CMOS SETUP, each figure contains the setup items and the default settings of them. Below each figure may or may not be contained a lists of function description for commonly used settings. For the other settings' function description you needed, please feel free to contact with your supplier.

## STANDARD  CMOS  SETUP

ROM  PCI / ISA  BIOS  (2A59GXXX)

STANDARD CMOS  SETUP

AWARD  SOFTWARE,  INC.

| Date (mm : dd : yy) : Thu, |  |  |  |  | Feb | 27 1997 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time (hh : mm : ss) : 11 : 33 : 55 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | HARD DISKS | TYPE |  | SIZE |  |  | CYLS |  | HEAD | PRECOMP |  |  | LANDZ |  | SECTOR |  |  |  | MODE |  |  |  |
| Primary Master |  | Auto |  | 0 |  |  | 0 |  | 0 | 0 |  |  |  | 0 |  |  | 0 |  |  | Auto |  |  |
| Primary Slave |  | None |  | 0 |  |  | 0 |  | 0 | 0 |  |  |  | 0 |  |  | 0 |  |  | ------ |  |  |
| Secondary Master |  | Auto |  | 0 |  |  | 0 |  | 0 | 0 |  |  |  | 0 |  |  | 0 |  |  | Auto |  |  |
| Secondary Slave |  | None |  | 0 |  |  | 0 |  | 0 | 0 |  |  |  | 0 |  |  | 0 |  |  | ------ |  |  |
| Drive A : 1.44M, 3.5 in. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Drive B : None |  |  |  |  |  |  |  | Base Memory : |  |  |  |  |  |  |  | 640 K |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Extended Memory : |  |  |  |  |  |  |  | xxxxxx K |  |  |  |  |  |  |
| Video : EGA/VGA |  |  |  |  |  |  |  | Other Memory : |  |  |  |  |  |  |  | xxxxxx K |  |  |  |  |  |  |
| Halt On : All Errors |  |  |  |  |  |  |  |  |  |  | Total Memory : |  |  |  |  |  |  | xxxxxx K |  |  |  |  |
| ESC : Quit |  |  | ↑↓→← : Select Item |  |  |  |  |  |  |  |  | PU/PD/+/- : Modify |  |  |  |  |  |  |  |  |  |  |
| F1 : Help |  |  | (Shift)F2 : Change Color |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Fig. 5-3 STANDARD CMOS SETUP screen.

MODE :

For IDE hard disks, this BIOS provides three modes to support both normal size IDE hard disks and also disks size larger the 528MB:

‧NORMAL	:	For IDE hard disks size smaller then 528MB.

‧LBA	:	For IDE hard disks size larger then 528MB and up to 8.4GB (Giga Bytes)

that use Logic Block Addressing (LBA) mode.

‧Large	:	For IDE hard disks size larger then 528MB that do not use LBA mode.

Large mode is a new specifition which may not be fully supported by all

operation systems.  Now it can only be used with the MS-DOS and is

uncommon.

Note	:	Some OSes (like SCO-UNIX) must use "NORMAL" for installation.

## BIOS  FEATURES  SETUP

ROM  PCI / ISA  BIOS  (2A59GXXX)

BIOS  FEATURES  SETUP

AWARD  SOFTWARE,  INC.

| Virus Warning | : | Disabled |  | Video BIOS Shadow |  | : Enabled |
| --- | --- | --- | --- | --- | --- | --- |
| CPU Internal Cache | : Enabled |  |  | C8000-CBFFF Shadow |  | : Disabled |
| External Cache | : Enabled |  |  | CC000-CFFFF Shadow |  | : Disabled |
| Quick Power On Self Test | : Enabled |  |  | D0000-D3FFF Shadow |  | : Disabled |
| Boot Sequence | : A, C |  |  | D4000-D7FFF Shadow |  | : Disabled |
| Swap Floppy Drive | : Disabled |  |  | D8000-DBFFF Shadow |  | : Disabled |
| Boot Up Floppy Seek | : Enabled |  |  | DC000-DFFFF Shadow |  | : Disabled |
| Boot Up NumLock Status | : On |  |  |  |  |  |
| Boot Up System Speed | : High |  |  |  |  |  |
| Gate A20 Option | : Fast |  |  |  |  |  |
| Typematic Rate Setting | : Disabled |  |  |  |  |  |
| Typematic Rate (Chars/Sec) | : 6 |  |  |  |  |  |
| Typematic Delay (Msec) | : 250 |  |  |  |  |  |
|  |  |  |  | ESC : Quit | ↑↓→← : Select Item |  |
| Security Option | : Setup |  |  | F1 : Help | PU/PD/+/- : Modify |  |
| PCI/VGA Palette Snoop | : Disabled |  |  | F5 : Old Values | (Shift)F2 : Color |  |
| OS Select For DRAM > 64MB | : Non-OS2 |  |  | F7 : Load Setup Defaults |  |  |
| PS/2 mouse function control | : Enabled |  |  |  |  |  |

Fig. 5-4 BIOS FEATURES SETUP screen.

Virus  Warning :

This feature flashes on the screen. During and after the system boots up, any attempt to

write to the boot sector or partition table of the hard disk drive will halt the system and

an error message will appear, in the mean time, you can run anti-virus program to locate

the problem. Default values is "Disabled"

Enabled	:	Activate automatically when the system boots up causing a warning message

to appear when anything attempts to access the boot sector or hard disk

partition table.

Disabled	:	No warning message to appear when anything attempts to access the boot

sector or hard disk partition table.

CPU  Internal  Cache :

This option enables CPU's internal (L1) cache memory. If you want to use the internal

(L1) cache memory and external (L2) cache memory, this option must be enabled.

External  Cache :

This option enables L2 (secondary) external cache memory. If none external cache

memory on board you must set this option to "disabled", otherwise, you can select

enabled or disabled.

## CHIPSET  FEATURES  SETUP

ROM  PCI / ISA  BIOS  (2A59GXXX)

CHIPSET  FEATURES  SETUP

AWARD  SOFTWARE,  INC.

| Auto Configuration | : | Enabled |  | Delayed Transaction |  |  | : Disabled |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DRAM Timing | : 60 ns |  |  |  |  |  |  |
| DRAM RAS# Precharge Time | : 3 |  |  |  |  |  |  |
| DRAM R/W Leadoff Timing | : 6 |  |  |  |  |  |  |
| Fast RAS To CAS Delay | : 3 |  |  |  |  |  |  |
| DRAM Read Burst (EDO/FP) | : x222/x333 |  |  |  |  |  |  |
| DRAM Write Burst Timing | : x222 |  |  |  |  |  |  |
| Fast MA to RAS# Delay CLK | : 1 |  |  |  |  |  |  |
| Fast EDO Path Select | : Disabled |  |  |  |  |  |  |
| Refresh RAS# Assertion | : 4 Clks |  |  |  |  |  |  |
| ISA Bus Clock | : PCICLK/4 |  |  |  |  |  |  |
| System BIOS Cacheable | : Disabled |  |  |  |  |  |  |
| Video BIOS Cacheable | : Disabled |  |  |  |  |  |  |
| 8 Bit I/O Recovery Time | : 1 |  |  | ESC : Quit | ↑↓→← : Select Item |  |  |
| 16 Bit I/O Recovery Time | : 1 |  |  | F1 : Help | PU/PD/+/- : Modify |  |  |
| Memory Hole At 15M-16M | : Disabled |  |  | F5 : Old Values | (Shift)F2 : Color |  |  |
| Peer Concurrency | : Enabled |  |  | F7 : Load Setup Defaults |  |  |  |
| Passive Released | : Enabled |  |  |  |  |  |  |

Fig. 5-5 CHIPSET FEATURES SETUP screen.

WARNING	:	The CHIPSET FEATURES SETUP in this screen are provided so that

technical professionals can modify the Chipset to suit their requirement.

If you are not a technical engineer, do not use this program !

Auto  Configuration :

When "Enabled", this parameter automatically enters and locks some of the optimum values for the chipset and CPU. Otherwise, this parameter allows the values of these parameters could be changed.

DRAM  Timing :

When "Auto Configuration" is "Enabled", this parameter provides two suit of the optimum values for the chipset and CPU, depends on the DRAMs' speed, you can select

"70 ns" or "60 ns", but the first value maybe caused your system more stable.

## POWER  MANAGEMENT  SETUP

ROM  PCI / ISA  BIOS  (2A59GXXX)

POWER  MANAGEMENT  SETUP

AWARD  SOFTWARE,  INC.

| Power Management | : | User Define |  | ** Power Down & Resume Events ** |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PM Control by APM | : Yes |  |  | IRQ 3 (COM 2) |  |  | : ON |
| Video Off Method | : Blank Screen |  |  | IRQ 4 (COM 1) |  |  | : ON |
| MODEM Use IRQ | : 3 |  |  | IRQ 5 (LPT 2) |  |  | : ON |
|  |  |  |  | IRQ 6 (Floppy Disk) |  |  | : ON |
| Doze Mode | : Disable |  |  | IRQ 7 (LPT 1) |  |  | : ON |
| Standby Mode | : Disable |  |  | IRQ 8 (RTC Alarm) |  |  | : OFF |
| Suspend Mode | : Disable |  |  | IRQ 9 (IRQ2 Redir) |  |  | : ON |
| HDD Power Down | : Disable |  |  | IRQ 10 (Reserved) |  |  | : ON |
|  |  |  |  | IRQ 11 (Reserved) |  |  | : ON |
| ** Wake Up Events In Doze & Standby ** |  |  |  | IRQ 12 (PS/2 Mouse) |  | : ON |  |
| IRQ3 (Wake-Up Event) | : ON |  |  | IRQ 13 (Coprocessor) |  |  | : ON |
| IRQ4 (Wake-Up Event) | : ON |  |  | IRQ 14 (Hard Disk) |  |  | : ON |
| IRQ8 (Wake-Up Event) | : OFF |  |  | IRQ 15 (Reserved) |  |  | : OFF |
| IRQ12 (Wake-Up Event) | : ON |  |  |  |  |  |  |
|  |  |  |  | ESC : Quit | ↑↓→← : Select Item |  |  |
|  |  |  |  | F1 : Help | PU/PD/+/- : Modify |  |  |
|  |  |  |  | F5 : Old Values | (Shift)F2 : Color |  |  |
|  |  |  |  | F7 : Load Setup Defaults |  |  |  |

Fig. 5-6 POWER MANAGEMENT SETUP screen.

WARNING	:	The POWER MANAGEMENT SETUP in this screen are provided so that

technical professionals can modify the Chipset to suit their requirement.

If you are not a technical engineer, do not use this program !

Power  Management :

This setting controls the Power Management functions. "User Define" allows the values

of all parameters could be modified. "Min Saving" and "Max Saving" fixed the values of

four parameters, including "Doze Mode", "Standby Mode", "Suspend Mode" and "HDD

Power Down". "Disable" disabled all Power Management functions. Default is "User

Define".

## PNP/PCI  CONFIGURATION

ROM  PCI / ISA  BIOS  (2A59GXXX)

PNP/PCI  CONFIGURATION

AWARD  SOFTWARE,  INC.

| Resources Controlled By | : | Manual |  | PCI IRQ Actived By |  | : Level |
| --- | --- | --- | --- | --- | --- | --- |
| Reset Configuration Data | : Disabled |  |  | PCI IDE IRQ Map To |  | : PCI-AUTO |
|  |  |  |  | Primary IDE INT# |  | : A |
| IRQ-3 assigned to | : Legacy ISA |  |  | Secondary IDE INT# |  | : B |
| IRQ-4 assigned to | : Legacy ISA |  |  |  |  |  |
| IRQ-5 assigned to | : PCI/ISA PnP |  |  | Used MEM base addr |  | : N/A |
| IRQ-7 assigned to | : Legacy ISA |  |  |  |  |  |
| IRQ-9 assigned to | : PCI/ISA PnP |  |  |  |  |  |
| IRQ-10 assigned to | : PCI/ISA PnP |  |  |  |  |  |
| IRQ-11 assigned to | : PCI/ISA PnP |  |  |  |  |  |
| IRQ-12 assigned to | : Legacy ISA |  |  |  |  |  |
| IRQ-14 assigned to | : Legacy ISA |  |  |  |  |  |
| IRQ-15 assigned to | : Legacy ISA |  |  |  |  |  |
| DMA-0 assigned to | : PCI/ISA PnP |  |  |  |  |  |
| DMA-1 assigned to | : PCI/ISA PnP |  |  |  |  |  |
| DMA-3 assigned to | : PCI/ISA PnP |  |  | ESC : Quit | ↑↓→← : Select Item |  |
| DMA-5 assigned to | : PCI/ISA PnP |  |  | F1 : Help | PU/PD/+/- : Modify |  |
| DMA-6 assigned to | : PCI/ISA PnP |  |  | F5 : Old Values | (Shift)F2 : Color |  |
| DMA-7 assigned to | : PCI/ISA PnP |  |  | F7 : Load Setup Defaults |  |  |

Fig. 5-7 PNP/PCI CONFIGURATION setup screen.

WARNING	:	The PNP/PCI CONFIGURATION in this screen are provided so that

technical professionals can modify the Resources Configuration to suit

their requirement. If you are not a technical engineer, do not use this

program !

Resources  Controlled  by :

Manual	:	The system BIOS will not reference the ESCD for IRQ & DMA informations.

Instead, it will reference the items in this setup menu for assigning IRQ &

DMA, but for I/O and Memory space the system BIOS still refer to the ESCD.

Atuo	:	The system BIOS will reference the ESCD for all legacy informations.

Reset  Configuration  Data :

Disabled	:	The system BIOS will do nothing.

Enabled	:	The system BIOS will clear/reset the ESCD during "POST". After clearing the

ESCD, the system BIOS will then change this item's value back to "Disable",

otherwise, the ESCD will become useless.

IRQ# / DMA#  assigned  to :

Legacy	:	The system BIOS will skip never assign this specified IRQ/DMA resource to

ISA		PCI or ISA PnP devices.

PCI/ISA	:	All items set to this value will make the specified IRQ/DMA have a chance to

PnP		be assigned to PCI or ISA PnP devices.

PCI  IRQ  Actived  By :

This option tells the system board chipset the IRQ signals input is Level or Edge trigger.

PCI  IDE  IRQ  Map  To : (for off-board PCI ICE cards)

PCI-AUTO	:	The BIOS will scan for PCI IDE devices and determine the location of the

PCI IDE device, then assign IRQ 14 for primary IDE INT#, and assign

IRQ 15 for secondary IDE INT#.

PCI-SLOT1	:	For the specified slot, the BIOS will assign IRQ 14 for primary IDE

to		INT#, and assign IRQ 15 for secondary IDE INT#.

PCI-SLOT4

ISA		:	The BIOS will not assign any IRQs even if PCI IDE card is found.

Because some IDE cards connect the IRQ 14 and 15 directly from ISA slot

thru a card. (This card is called Legacy Header)

Note	:	No matter the item "Resources Controlled By" is set to "Manual" or "Auto", the

system BIOS assign IRQs to PCI devices from high to low. For ISA PnP devices,

the sequence is from low to high. IRQ 12 is always the last one available for

PCI/PnP due to IRQ 12 is always reserves for the PS/2 mouse.

Explanation  for  proper  nouns :

PnP  device :

‧	Device that has Plug & Play compatibility. That means it can request for DMA, IRQ,

I/O and Memory from the PnP BIOS and all these requests can be relocatable. In

other words, these devices does not utilized any fixed resources.

‧	All PCI devices and all ISA PnP devices are PnP devices.

Legacy  device :

‧	A legacy device is a device that all its resources are fixed by hardware (or selected

by jumpers).

‧	All ISA Non-PnP devices are legacy device.

Extended  System  Configuration  Data (ESCD) :

‧	A media between the user and the system BIOS for passing the legacy devices

informations. These informations are stored in the onboard NVRAM (flash ROM).

## INTEGRATED  PERIPHERALS

ROM  PCI / ISA  BIOS  (2A59GXXX)

INTEGRATED  PERIPHERALS

AWARD  SOFTWARE,  INC.

| IDE HDD Block Mode | : | Enabled |  | Onboard Parallel Port |  | : 378 / IRQ7 |
| --- | --- | --- | --- | --- | --- | --- |
| IDE Primary Master PIO | : Auto |  |  | Parallel Port Mode |  | : SPP |
| IDE Primary Slave PIO | : Auto |  |  |  |  |  |
| IDE Secondary Master PIO | : Auto |  |  |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |  |  |
| On-Chip Primary PCI IDE | : Enabled |  |  |  |  |  |
| On-Chip Secondary PCI IDE | : Enabled |  |  |  |  |  |
| PCI Slot IDE 2nd Channel | : Enabled |  |  |  |  |  |
| USB Controller | : Disabled |  |  |  |  |  |
| Onboard FDD Controller | : Enabled |  |  |  |  |  |
| Onboard Serial Port 1 | : 3F8 / IRQ4 |  |  |  |  |  |
| Onboard Serial Port 2 | : 2F8 / IRQ3 |  |  | ESC : Quit | ↑↓→← : Select Item |  |
| Onboard IR Controller | : Disabled |  |  | F1 : Help | PU/PD/+/- : Modify |  |
|  |  |  |  | F5 : Old Values | (Shift)F2 : Color |  |
|  |  |  |  | F7 : Load Setup Defaults |  |  |

Fig. 5-8  INTEGRATED PERIPHERALS setup screen.

WARNING	:	The INTEGRATED PERIPHERALS in this screen are provided so that

technical professionals can modify the Chipset  to suit their requirement.

If you are not a technical engineer, do not use this program !

Onboard  IR  Controller :

This setting determines the IR port (CN 11) function mode. Supports both HPSIR and ASKIR.

Onboard  Parallel  Mode :

This setting determines the onboard parallel port (CN 9) transmission mode. Supports either SPP, EPP, ECP or ECP+EPP.

## LOAD  SETUP  DEFAULTS

This option loads the SETUP default values from BIOS default table. By pressing "Enter" key, while "LOAD SETUP DEFAULTS" is highlighted, then presses "Y" and "Enter" key. the SETUP default values will be loaded. The SETUP default settings are the best-case values that should optimize system performance and increase system stability. If CMOS RAM is corrupted, the SETUP DEFAULTS settings are loaded automatically.

## SUPERVISOR  PASSWORD  /  USER  PASSWORD

Type the Password and press "Enter",then repeat. Enters up to eight alphanumeric characters. By pressing "Enter" key twice, without any alphanumeric character enters, the PASSWORD will be disabled.

## IDE  HDD  AUTO  DETECTION

By pressing "Enter" key, while "IDE HDD AUTO DETECTION" is highlighted causes the system to attempt to detect the type of hard disk. If successful, then presses "Y" (or 1, 2, ...) and "Enter" key, it fills in the remaining fields on this menu and the correlated fields in the STANDARD CMOS SETUP menu.

## HDD  LOW  LEVEL  FORMAT

This option provides an utility program for IDE HDD Low Level Format. Performing the Hard Disk Format will destory any data on the Hard Disk. Back up the Hard Disk(s) before actually performing of these routines.

Note : These routines are not valid for a SCSI Hard Disk Drive.

## SAVE  &  EXIT  SETUP

This option saves all setup values to CMOS RAM & EXIT SETUP routine, by moving the cursor to "SAVE & EXIT SETUP" and pressing "Enter" key, then types "Y" and "Enter" key, the values will be saved, the setup program will be terminated and the system will be reboot.

## EXIT  WITHOUT  SAVING

This option exites setup routine without saving any changed values to CMOS RAM, by moving the cursor to "EXIT WITHOUT SAVING" and pressing "Enter" key, then types "Y" and "Enter" key, the setup program will be terminated and the system will be reboot.