Standard Microsystems Corporation

Austin Design Center

11000 North Mopac Expressway

Stonelake Bldg. 6 Suite 500

Austin, Texas 78759

Release Notes for

SMSC USB FDC 0.03 (BETA2)

August 10, 1999

Table Of Contents

Table Of Contents	2

1.0 Changes from FDC 0.01 (ALPHA)	3

2.0 Files Included in this Distribution	3

3.0 W98 Driver Removal	4

3.1 First time install for YE-Data:	4

3.2 Removal of YE-Data Driver	4

3.3 Removal of PhoenixKK Driver	4

4.0 ISA RAM requirements	5

5.0 Jumper list for EVB6131rev A	5

6.0 Jumper list for EVB6131rev B	5

7.0 Reset button	6

8.0 Outstanding issues	6

8.1 W98, YE-Data driver ver 1.11	6

8.2 W98, Phoenix-KK driver ver 1.0.0.0	6

8.3 Mac-OS 8.6, Macintosh Mass Storage Extension ver 1.3	6

# Changes from FDC 0.01 (ALPHA)

Note:  “FDC 0.02 (BETA1)” was never released outside of SMSC.

- The format command implementation was modified to work around an intermittent problem with the DMA controller. This allows unformatted disks to be formatted.  It also allows 1.25MB disks to be formatted as 1.44MB.

- Enhanced formatting routine to use relative seek if seek fails to improve formatting of corrupt disks.

- Fixed media capacity reporting to allow Macintosh to  format 720K disks.

- Changed reporting of unformatted media to persist until a format command is issued or the media is ejected.  This helps W98 to recognize that the drive contains blank media.

- Monitor floppy 36 pin cable pin 4 HDO (High Density Out).  This allows the firmware to detect 2DD or 2HD for completely blank disk during format.  Report format capacities based on HDO pin.

- Media detect routine enhanced to check first and last track for matching formats (detects a disk corrupted by ejecting during format).  Reads first 64 bytes of boot sector and verifies good data and no dma stall.

- Fixed intermittent problem in USB data pump that caused packets to be dropped under specific timing conditions.

- Do not report “unformatted” in response to “test-unit-ready” as this confuses Macintosh.

# Files Included in this distribution

YEDATA.ZIP – The YE-Data driver.

PHOENIXKK.ZIP – The Phoenix-KK driver.

FDC0003.HEX  - The ROM file in intel hex format.  Load at address 0x0000.

(Use AT27LV512 flash.)

ROMSET.EXE – The ROM file resource editor.  Used to alter VID/PID, strings, etc.  Run this program, then click “Load HEX file”, and choose FDC0003.HEX.  The modifiable information will be displayed.  After editing the data, press “Change data”.  At this point, exit the application.  To see the modified data, run it again and reload the HEX file.  (The application will not display the correct data if you simply reload the HEX file without exiting and rerunning it.  We’ll fix that.)

# W98 Driver Removal

## First time install for YE-Data:

Reboot your PC.  Do not plug in the FDC yet.  Insert the YE-Data driver disk into an internal drive.  Read the README.TXT file on the driver disk.  Run the SETUP.EXE.  Reboot again.  Plug in the FDC.

## Removal of YE-Data Driver

- Unplug the USB cable connecting the USB floppy drive to the PC.

- Delete these files:

- \windows\system\iosubsys\usbfdvsd.vxd

- \windows\system32\drivers\usbfdd.sys

- \windows\inf\other\yedusbfd.inf  (sometimes w98 puts this into \windows\inf\)

- Run "regedit" and delete the following keys:

- hkey_local_machine\enum\usb\VID_0424B&PID_0FDC

- hkey_local_machine\system\currentcontrolset\services\class\usb\ (any which say “bad device” or USBMASS with SMSC listed).

## Removal of PhoenixKK Driver

- Unplug the USB cable connecting the USB floppy drive to the PC.

- Delete these files:

- \windows\system\iosubsys\pusbfd2.vxd

- \windows\system32\drivers\pusbfd1.sys

- \windows\inf\other\PhoenixTechnologiesKK PUSBFD.INF  (sometimes w98 puts this into \windows\inf\)

- Run "regedit" and delete the following keys:

- hkey_local_machine\enum\usb\VID_0424B&PID_0FDC

- hkey_local_machine\system\currentcontrolset\services\class\usb\ (any which say “bad device” or USBMASS with SMSC listed).

After you delete the drivers, you can re-install one of them.  Once the driver has been installed, Windows will not ask for the driver disk again.  It only needs to be installed once.  After that it remembers.

# ISA RAM requirements

This firmware does not require the use of any external RAM.  If the RAM is present on the board it will not be used and remains disabled to avoid unnecessary current consumption.

# Jumper list for EVB6131rev A (self powered)

| jp1 | on |
| --- | --- |
| jp2 | on |
| jp3 | 2&3 |
| jp4 | 2&3 |
| jp5 | on |
| jp6 | vdd_at |
| jp7 | off |
| jp8 | on |
| jp9 | off |
| jp10 | off |
| jp11 | off |
| jp12 | off |
| jp13 | 1&2 |

# Jumper list for EVB6131rev B (self powered)

| jp1 | on |
| --- | --- |
| jp2 | on |
| jp3 | 2&3 |
| jp4 | 2&3 |
| jp5 | on |
| jp6 | vdd_at |
| jp7 | off |
| jp8 | on |
| jp9 | off |
| jp10 | off |
| jp11 | off |
| jp12 | off |
| jp13 | 1&2 |
| jp14 | osc |
| jp15 | on |
| gpio | off |
| extrst | off |

# HDO pin modification

Connect HDO pin 4 of the floppy cable to pin 24 (/OPTEN) of the USB97CFDC.  Include a pull-up resistor.  This allows the firmware to monitor the pin.

Note: A high level on this pin indicates 2DD.  A low level indicates 2HD.  If the disk drive used does not support the HDO pin functionality then pin 24 of the USB97CFDC can be tied low with a pull-down resistor.  This will enable 2HD to be formatted as 1.44MB, but 2DD cannot be formatted.  If pin 24 is tied high with a pull-up resistor, then 2DD can be formatted as 720K, but 2HD cannot be formatted.

# Reset button

The reset button on the EVB6131 does not cause the device to detach from the USB and re-enumerate.  The reset button only causes the device to reset.  If the reset button is pressed, the USB cable must be disconnected and then reconnected to cause re-enumeration.

# Outstanding issues

The following is s list of known issues at the time that BETA2 was submitted for testing at SMSC.  Any issues uncovered during the tests will be indicated in the test suite results documents.  Those documents will be made available upon completion of the testing.

## W98, Phoenix-KK driver ver 1.0.0.0

- Phoenix driver crashes system if diskcopy is requested from a DOS Box.

## Mac-OS 8.6, Macintosh Mass Storage Extension ver 1.3.5

- Attach FDC to USB.  Insert a blank unformatted disk.  Host offers dialog to format disk.  Choose “format”, and the format runs until the final track is formatted, then the host hangs.  Detach FDC from USB and re-attach.  Host asks to format again.  Choose “format” and the format completes successfully.  (Detailed examination of the data transfer during the failure reveals that the host issues format commands for all tracks, then issues some read commands, then stops sending any data or commands to the device). THIS IS AN APPLE DRIVER ISSUE, they are investigating.

- Sometimes a blank or corrupt disk is inserted into the drive and the host does not recognize it and asks if it should be formatted. THIS IS AN APPLE DRIVER ISSUE, they are investigating.

- 1.2MB geometry is not recognized. THIS IS AN APPLE DRIVER ISSUE, they are investigating.