AIO4045

USER’S MANUAL

TABLE OF CONTENTS

CHAPTER 1 MOTHERBOARD	1

1.1 SYSTEM FEATURES	1

1.2 SYSTEM SPECIFICATIONS	1

1.3 AIO4045 BOARD LAYOUT	2

1.4 DRAM INSTALLATION	3

1.5 SRAM INSTALLATION	4

1.6 CPU INSTALLATION	5

1.7 AWARD BIOS SYSTEM CONFIGURATION SETUP	10

1.8 ALTERNATIVE SYSTEM SPEED	14

CHAPTER 2 IDE	15

2.1 SOFTWARE INSTALLATION	16

- DOS DRIVER	16

- WINDOWS 3.1 DRIVER	18

- NETWARE DRIVER	18

- OS/2	19

CHAPTER 3 VGA	20

3.1 INTRODUCTION	21

3.2 FEATURES	21

3.3 HARDWARE INSTALLATION	22

3.4 SOFTWARE INSTALLATION	24

CHAPTER 4 LAN	20

4.1 OVERVIEW	21

4.2 FEATURES	21

4.3 SOFTWARE SETUP	22

APPENDIX A AWARD POST CODE	57

APPENDIX B BEEF CODE	61

RMA FORM

CHAPTER  1 MOTHERBOARD

1.1 System Features

q Support  INTEL 486SX, 486DX, 486DX2, P24T, P24D, S-SERIAL.

AMD 486DX, 486DX2.

Cyrix M6, M7.

q Support L1 write back CPU.

1.2 System Specifications

CPU Clock : 	25/ 33/ 40 MHz

Memory :	1MB to 128MB

SRAM Size : 	128KB/256KB/512KB

VGA:	Avance Logic VL-Bus VGA

I/O :	2 serial port, 1 parallel port, 2 floppy

HDD:	VL-Bus IDE

Expand I/O slot:	5 ISA slots, 1 VL-Bus slot

Dimension :	27.0cm ´ 21.3cm

Board design:	4-Layer implementation

1.3 AIO4045 Board Layout

VGA

PARALLEL

COM1

COM2

BNC

K/B

J10

W83757F

BT1

W83758F

JP66

JP72

JP70

JP71

JP73,JP69-JP67

J9

JP54

JP61

JP60

JP65

BIOS

K/B

BIOS

W89C904AXF

JP59

JP58

JP57

PS

W83C492F

ALG2228

JP56

W83C759F

JP55

JP17

JP53

FDD

JP49

JP47

HDD

486 CPU

JP37

JP36

JP35

JP43

JP46

JP45

JP44

JP48

JP32

U15

JP39

JP52

W83C491F

U14

U10

JP16

JP28

JP29

J5

U13

U9

JP14

JP30, JP31

JP26

JP34

JP33

SIM4

SIM3

J3

J2

JP4

JP5

JP13

SIM2

SIM1

J1

JP1-JP3

JP6

1.4 DRAM INSTALLATION

The AIO4045 main board  can support expanded memory from 1MB to 128MB.  Either 1MB, 2MB, 4MB, 8MB, 12MB, 16MB, 24MB, 32MB, 64MB, 128MB SIM DRAM can be  used on the AIO4045 motherboard.

n The board layout below shows the locations of the DRAM memory 	banks :

SIM4

SIM3

SIM2

SIM1

n DRAM Configuration

| SIM1 | SIM2 | SIM3 | SIM4 | TOTAL MEMORY |
| --- | --- | --- | --- | --- |
| 1MB*1 | NONE | NONE | NONE | 1MB |
| 1MB*1 | 1MB*1 | NONE | NONE | 2MB |
| 4MB*1 | NONE | NONE | NONE | 4MB |
| 4MB*1 | 4MB*1 | NONE | NONE | 8MB |
| 4MB*1 | 4MB*1 | 4MB*1 | NONE | 12MB |
| 4MB*1 | 4MB*1 | 4MB*1 | 4MB*1 | 16MB |
| 4MB*1 | NONE | 4MB*2 | NONE | 12MB |
| 4MB*2 | NONE | NONE | NONE | 8MB |
| 4MB*2 | 4MB*2 | NONE | NONE | 16MB |
| 4MB*2 | 4MB*2 | 4MB*2 | NONE | 24MB |
| 4MB*2 | 4MB*2 | 4MB*2 | 4MB*2 | 32MB |
| 16MB*1 | NONE | NONE | NONE | 16MB |
| 16MB*1 | 16MB*1 | NONE | NONE | 32MB |
| 16MB*1 | 16MB*1 | 16MB*1 | 16MB*1 | 64MB |
| 16MB*2 | NONE | NONE | NONE | 32MB |
| 16MB*2 | 16MB*2 | 16MB*2 | 16MB*2 | 128MB |

1.5 SRAM INSTALLATION

U13

JP36

JP35

486 CPU

U15

U14

W83C491F

U10

JP34

JP33

U9

CACHE CONFIGURATION SIZE

| 128K |  |  | 256K |  |  |  | 512K |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TAG RAM (U13) | DATA RAM (U9,U10,U14,U15) |  | TAG RAM (U13) | DATA RAM (U9,U10,U14,U15) |  | TAG RAM (U13) |  | DATA RAM (U9,U10,U14,U15) |
| 8K*8 | 32K*8 |  | 32K*8 | 64K*8 |  | 32K*8 |  | 128K*8 |
| JP34 JP34 JP33 JP33 JP35 JP35 JP36 JP36 1 1 1 1 1 1 1 |  | JP34 JP33 JP35 JP36 1 1 |  |  |  |  |  |  |

1.6 CPU INSTALLATION

JP37

486 CPU

JP46

JP45

JP44

JP32

W83C491F

J5

J4

J3

J2

J1

JP16

JP28

JP29

JP14

JP26

JP5

JP4

JP1-JP3

CPU CLOCK

| 20 MHz | 25 MHz | 33 MHz | 40 MHz |
| --- | --- | --- | --- |
| JP1, JP2, JP3 JP1, JP2, JP3 JP1, JP2, JP3 JP1, JP2, JP3 |  |  |  |

J1: POWER LED & K/B LOCK

J2: SPEAKER (3-4 SHORT)

- INTEL/AMD CPU TYPE SELECT

| 486SX | 20 19 2 1 JP14 JP5 3 2 1 |
| --- | --- |
| 20 19 2 1 JP14 JP5 3 2 1 JP16 9 8 7 6 5 4 3 2 1 JP32 JP44 1 9 10 JP37 486DX/DX2 |  |
| JP14 JP26 6 2 JP16 9 8 7 6 5 4 3 2 1 JP28 JP29 5 1 JP32 JP44 P24D | 20 19 2 1 1 7 8 10 JP37 |
| 9 8 7 6 5 4 3 2 1 JP28 JP32 JP29 JP26 1 2 5 6 JP44 P24T | 20 19 2 1 JP14 JP16 |
| JP5 9 8 7 6 5 4 3 2 1 JP28 JP44 JP32 JP59 486DX/DX2 enhance CPU | 20 19 2 1 JP14 3 2 1 JP16 1 9 10 JP37 |

- CYRIX CPU TYPE SELECT

| 9 8 7 6 5 4 3 2 1 M7 | 20 19 2 1 JP14 JP5 3 2 1 JP16 JP59 JP32 JP28 JP61 JP44 JP29 10 9 8 7 6 5 4 3 2 1 JP37 |
| --- | --- |
| 20 19 2 1 JP14 JP5 3 2 1 JP16 9 8 7 6 5 4 3 2 1 JP59 JP32 JP28 JP61 JP44 JP29 10 9 8 7 6 5 4 3 2 1 JP37 M6 |  |

CPU POWER VOLTAGE

| 3.3V FOR DX4 | 5V FOR NORMAL |
| --- | --- |
| JP17 JP31 JP17 JP |  |

OTHER JUMPER DESCRIPTION

| JUMPER | DESCRIPTION |
| --- | --- |
| SUSPEND RESUME JP45 |  |
| CMOS CLEAR JP66 |  |

CONNECTOR DESCRIPTION

| CONNECTOR | PIN OUT | SIGNAL NAME |
| --- | --- | --- |
| J1: KEY LOCK | 1 2 3 4 5 | LED POWER NOT USED GROUND KEYBOARD INHIBITOR GROUND |
| TURBO SPEED NORMAL SPEED J3: TURBO/SW | 1 2 | GROUND SELECT PIN |
| J4: LED | 1 2 | +ANODE -CATHODE |
| J5: RESET | 1 2 | GROUND RESET IN |
| KB: KEYBOARD CONNECTOR | 1 2 3 4 5 | KEYBOARD CLOCK KEYBOARD DATA SPACE GROUND +5V |
| PS: POWER CONNECTOR | 1 2 3 4 5,6,7,8 9 10,11,12 | POWER GOOD +5V DC +12V DC -12V DC GROUND -5V DC +5V DC |

1.7 AWARD BIOS SYSTEM CONFIGURATION 	SETUP

This section will explain  how to set up the system configuration (CMOS)  under the AWARD BIOS. The SETUP program is contained in the system's Read-Only-Memory rather than on a diskette.  To enter SETUP, press the <Del> key when the system is booting up.  The following menu appears:

Please enter " STANDARD CMOS SETUP " to enter the next screen.

ROM ISA BIOS (2C4J6000)

CMOS SETUP UTILITY

AWARD SOFTWARE,INC.

| STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | PASSWORD SETTING IDE HDD AUTO DETECTION SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| Esc : Quit F10 : Save & Exit Setup | ¯®¬ : Select Item (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

ROM ISA BIOS (2C4J6000)

STANDARD CMOS SETUP

AWARD SOFTWARE,INC.

| Date (mm:dd:yy) Time (hh:mm:ss) | : Mon, Nov 14 1994 : 11 : 26 : 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CYLS. HEADS PRECOMP LANDZONE SECTORS MODE |  |  |  |
| Drive C : User (365Mb) Drive D : None ( 0Mb) |  |  | 929 16 0 929 48 NORMAL 0 0 0 0 0 ---------- |  |  |
| Drive A : 1.44M , 3.5 in. Drive B : 1.2M, 5.25 in. Video : EGA/VGA Halt On : ALL Errors |  |  |  | Base Memory : 640K Extended Memory : 15360K Expanded Memory : 0K Other Memory : 384K Total Memory : 16384K |  |
| Esc : Quit F1 : Help |  | ¯®¬ : Select Item (Shift)F2 :Change Color |  |  | PU/PD/+/- : Modify F3 : Toggle Calender |

ROM ISA BIOS (2C4J6000)

CMOS SETUP UTILITY

AWARD SOFTWARE,INC.

| STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | PASSWORD SETTING IDE HDD AUTO DETECTION SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| Esc : Quit F10 : Save & Exit Setup | ¯®¬ : Select Item (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

ROM ISA BIOS (2C4J6000)

BIOS FEATURES SETUP

AWARD SOFTWARE,INC.

| Virus Warning CPU Internal Cache External Cache Quick Power On Self Test Boot Sequence Swap Floppy Drive Boot Up Floppy Seek Boot Up NumLock Status Boot Up System Speed IDE HDD Block Mode Gate A20 Option | : Disabled : Enabled : Enabled : Disabled : A, C : Disabled : Enabled : On : High : Disabled : Fast | Video BIOS Shadow C8000-CBFFF Shadow CC000-CFFFF Shadow D0000-D3FFF Shadow D4000-D7FFF Shadow D8000-DBFFF Shadow DC000-DFFFF Shadow E0000-E3FFF Shadow E4000-E7FFF Shadow E8000-EBFFF Shadow EC000-EFFFF Shadow | : Enabled : Disabled : Disabled : Disabled : Disabled : Disabled : Disabled : Disabled : Disabled : Disabled : Disabled |
| --- | --- | --- | --- |
| Memory Parity Check Typematic Rate Setting Typematic Rate (Chars/Sec) Typematic Delay (Msec) Security Option | : Enabled : Disabled : 6 : 250 : Setup | Esc : Quit ¯®¬ : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Default F7 : Load Setup Default |  |

ROM ISA BIOS (2C4J6000)

CMOS SETUP UTILITY

AWARD SOFTWARE,INC.

| STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | PASSWORD SETTING IDE HDD AUTO DETECTION SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| Esc : Quit F10 : Save & Exit Setup | ¯®¬ : Select Item (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

ROM ISA BIOS (2C4J6000)

CHIPSET FEATURES SETUP

AWARD SOFTWARE,INC.

| Auto Configuration AT Bus Clock Extended ALE Internal Cache Burst WB Internal Cache WB/WT External Cache WB/WT Keyboard A20 Emulation Keyboard RC Emulation DRAM Wait State DRAM Decouple Refresh DRAM Slow Refresh | : Enable : CLKIN/5 : Disable : Enable : Write Back : Write Back : Disable : Disable : 40Mhz, 1WS : Disable : Disable |  |  |
| --- | --- | --- | --- |
|  |  | Esc : Quit ¯®¬ : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Default F7 : Load Setup Default |  |

ROM ISA BIOS (2C4J6000)

CMOS SETUP UTILITY

AWARD SOFTWARE,INC.

| STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | PASSWORD SETTING IDE HDD AUTO DETECTION SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| Esc : Quit F10 : Save & Exit Setup | ¯®¬ : Select Item (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

ROM ISA BIOS (2C4J6000)

POWER MANAGEMENT SETUP

AWARD SOFTWARE,INC.

| Power Management PM Control by APM Doze Timer Global Standby Timer Suspend Timer HDD Standby Timer Video Off Option Video Off Method Global Standby General I/O Suspend General I/O *** Activity Detect *** Video activity detect Hard Disk activity detect | : Max Saving : No : 1 Sec : 1 Min : 4 Min : Disable : Susp, Stby -> Off : V/H SYNC+Blank : Enable : Enable : Disable : Enable | VL Bus Master detect *** Wakeup Event *** IRQ 1&3 as Wakeup Event IRQ 4&5 as Wakeup Event IRQ 6&7 as Wakeup Event IRQ 8&9 as Wakeup Event IRQ A&B as Wakeup Event IRQ C&D as Wakeup Event IRQ E&F as Wakeup Event IRQ 0&1 as Wakeup Event IRQ 2&3 as Wakeup Event IRQ 4&5 as Wakeup Event IRQ 6&7 as Wakeup Event | : Disable : IRQ 1&3 : IRQ 4&5 : IRQ 6&7 : IRQ 8 : IRQ B : IRQ D : IRQ F : Disable : Disable : Disable : Disable |
| --- | --- | --- | --- |
| Floppy Disk activity detect Keyboard activity detect Serial Port activity detect Parallel Port activity detect VL Bus activity detect | : Enable : Enable : Disable : Disable : Disable | Esc : Quit ¯®¬ : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Default F7 : Load Setup Default |  |

ROM ISA BIOS (2C4J6000)

CMOS SETUP UTILITY

AWARD SOFTWARE,INC.

| STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | PASSWORD SETTING IDE HDD AUTO DETECTION SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| Esc : Quit F10 : Save & Exit Setup | ¯®¬ : Select Item (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

ROM ISA BIOS (2C4J6000)

CMOS SETUP UTILITY

AWARD SOFTWARE,INC.

| Enter Password: STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULT | PASSWORD SETTING IDE HDD AUTO DETECTION SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| Esc : Quit F10 : Save & Exit Setup | ¯®¬ : Select Item (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

ROM ISA BIOS (2C4J6000)

CMOS SETUP UTILITY

AWARD SOFTWARE,INC

| STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | PASSWORD SETTING IDE HDD AUTO DETECTION SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| Esc : Quit F10 : Save & Exit Setup | ¯®¬ : Select Item (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

ROM ISA BIOS (2C4J6000)

CMOS SETUP UTILITY

AWARD SOFTWARE,INC.

| SAVE to CMOS and EXIT (Y/N) : Y STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT LOAD BIOS DEFAULTS LOAD SETUP DEFAULT | PASSWORD SETTING IDE HDD AUTO DETECTION SAVE & EXIT SETUP |
| --- | --- |
| Esc : Quit F10 : Save & Exit Setup | ¯®¬ : Select Item (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

1.8 ALTERNATIVE SYSTEM SPEED

After booting the system, hold down <Ctrl> and <Alt> keys, then press <-> to select low-speed, press <+> to select high-speed.

Please review the following configuration :

| HOLD DOWN <Ctrl> AND <Alt> KEYS, PRESS <+> HOLD DOWN <Ctrl> AND <Alt> KEYS, PRESS <-> | CPU SPEED NORMAL Þ TURBO CPU SPEED TURBO Þ NORMAL |
| --- | --- |

CHAPTER  2  IDE

JP52

W83C759F

5                1

10              6

15            11

JP53

| 1 6 11 1 6 11 IDE MODE (JP53) | ENABLE DISABLE |  |
| --- | --- | --- |
| CPU SPEED (JP53) | 1 6 11 1 6 11 <=33MHz >33MHz |  |
| 1 6 11 1 6 11 1 6 11 1 6 11 MODE2 MODE3 HDD MODE (JP53) | MODE0 MODE1 |  |
| ENABLE DISABLE I/OCHECK READY (JP52) |  |  |

2.1 SPECIFICATIONS

q  Compatible with HOST VESA bus standard

q  I/O address 1F0 to 1F7

q  Floppy Disk Controller

l  Support up to two 3.5” or 5.25” floppy disk drives

l  360K/720K/1.2M/1.44M

q  Serial port and Parallel port

l  Two serial ports, COM1/COM2/COM3/COM4 selectable one 	     parallel port, 378H/3BCH/278H selectable

2.2 SOFTWARE  INSTALLATION

1. DOS Driver

WBIDE.EXE [Options]

(A) The WBIDE.EXE is two in one exceptional program:

A DOS Device Driver and a Configuration Utility.

1) A Device Driver on DOS.

To edit the line -

DEVICE=[d:\path\]WBIDE.EXE [/S] [/L] [/C]

into the CONFIG.SYS file, and reboot the DOS.

[Note]: The WBIDE.EXE driver will detect automatically for 		the IDE disk configuration data : Xfer-bit & best		multiple sector. Those parameters are used only when 		a trouble was encounted.

The parameters, [/S] [/L] and [/C] are optional.

[/S] parameter is selected to indicate that the WBIDE.EXE does 		not use the “ multiple sector read/write” functions to access 		all IDE disks.

[/L] parameter is selected to indicate hat the WBIDE.EXE does 		use 16-bit data transfer between IDE disks and CPU.

[/C] parameter is valid only when WBIDE BIOS was existed. It 		is selected to indicate that the WBIDE.EXE will replace 		WBIDE BIOS functions. If WBIDE BIOS was present, the 		WBIDE.EXE will not work unless the parameter [/C] was 		specified.

* Normally setting as follows :

DEVICE=C:\path\WBIDE.EXE

2) A Configuration Utility on DOS.

To run the WBIDE.EXE directly on the DOS prompt-

d:\> [d:\path\]WBIDE.EXE <enter>

(B) Functional Description:

1) WBIDE DOS Device Driver supports up to four IDE disks. They 	are 1FXH I/O port address and 17XH I/O port address.

The IDE disks, where are attached into 17XH port connector, do 	not specify CYL/HD/SECT parameters in ROM BIOS SETUP.

The WBIDE.EXE Device Driver will ask you to correct 	CYL/HD/SECT value to match the pre-setting in ROM BIOS 	SETUP, when the IDE disk was first time attached to WBIDE 	Device Driver on 17XH connector.

2) WBIDE Configuration Utility will show you the current configur- 	ation of IDE disks, and easy to modify the CYL/HD/SECT 	configuration.

[Note]: The CYL/HD/ECT setup of WBIDE Device Driver 		was set only one time, when the IDE disk was first 		time attached. If a wrong CYL/HD/SECT value is 		set to WBIDE Device Driver, you need to execute 		WBIDE Configuration Utility on DOS to correct the 		value and reboot.

2. Windows 3.1 Driver

There are six files in subdirectory WINDRV:

SETUP.EXE

SETUPKIT.DLL

VBRUN200.DLL

VER.DLL

WIDE310.386

README.TXT

You should install Windows 3.1 first. And then execute SETUP.EXE in subdirectory WINDRV on DOS prompt, it will install Windows 3.1 driver WIDE310.386 automatically.

-OR-

You can refer README.TXT in subdirectory WINDRV to install Windows 3.1 driver WIDE310.386 by yourself.

[NOTE] Before you execute Windows 3.1 with WIDE310.386 Driver, 	  the DOS device driver - WBIDE.EXE must be run first.

3. Netware driver

(A) Netware V2.15 and V2.2

Label this diskette as DSK_DRV_003 and follow the installation procedure of Netware.

(B) Netware V3.1 and V3.11

At server prompt

load NWIDE310	for V3.1

or	load NWIDE311	for V3.11

then select the proper I/O port and interrupt number.

Options supported

/CS force master IDE drive to be single sector access

/DS force slave IDE drive to be single sector access

(C) Netware 4.00

Follow the Netware 4.00 installation procedure

4.  OS/2

Refer to the OS2IDE2X.DOC in OS2 subdirectory for OS2 driver installation

CHAPTER  3  VGA

Trademarks

AutoCAD and AutoShade are trademarks of  Autodesk Inc.

IBMÒ, OS/2, VGA and PS/2 are trademarks of International Business

Machines Corporation.

Lotus, 1-2-3 and Symphony are trademarks of Lotus Development

Corporation.

Windows,  Word,  MS-DOS and Microsoft are trademarks of Microsoft

Corporation.

WordPerfect is a trademark of WordPerfect Corporation.

VESA is a trademark of Video Electronics Standards Association.

MultiSync is a trademark of Nippon Electric Company(NEC).

ImgDAC, photoDAC, and GUI-ULTRA are trademarks of Avance

Logic Inc.

All other trade names are trademarks of their original Vendors.

3.1  INTRODUCTION

AIO-4045 is an all purpose single-chip graphics controller based adapter which excels in GUI, CAD and DOS application environments.  It delivers high speed, high resolution display with true color support at 16.7 million colors.  It is a cost effective and ideal graphics solution for desktop PC users.  To take full advantage of  the AIO-4045 hardware functions, it comes with a complete set of software drivers such as for MS Windows 3.1, AutoCAD, and more.

AIO-4045 is fully backward compatible with the IBM and Hercules video standards (i.e. IBM MDA, CGA, EGA, VGA and Hercules).  It protects users of their early software investment.  AIO-4045 is designed to work with VESA VL-BUS platforms with high speed CPU support.  AIO-4045 is the graphics solution which offers you quality, performance and advanced features all in one card.

3.2 FEATURES

* 	VESA Revision 2.0 Compliance.

*	Full 32-bit , GUI Acceleration.

* 	Green PC support, VESA DPMS Power Saving.

* 	24-bit DAC included ( Support 16.7M True Colors ).

* 	Displays up to 1280´1024 resolution at 16 colors.

JP57:  IRQ9

Enable

Disable (default)

3.3 SOFTWARE DRIVERS

Utility Programs

1	Insert AIO-4045 driver disk 1 into the floppy drive A and type:

A:\INSTALL	[Enter]

2	The INSTALL program will display a list of options.  Select the option for installing Utility Programs and press [Enter].

Note: MS Windows 3.1 has its own install program.  Details refer to "Installing MS Windows 3.1".

3	Provide the correct drive path where you would like to install the AIO-4045 utility  programs.

4	The   INSTALL   program   will   automatically   copy  all  available

AIO-4045 utility programs into the specified drive path.

1.   Using VMODE.EXE

VMODE is a menu-driven program to help you select the AIO-4045 video modes which are available for display.  With a few simple keystrokes, you can switch the display into one of the many extended high resolution modes listed.

VMODE can be run directly from the DOS command line or from a menu.

The following table provides the detailed instructions on how you can work with VMODE.EXE:

| VMODE COMMAND LINE | DESCRIPTION |
| --- | --- |
| VMODE | Window-screen VMODE |
| VMODE? | VMODE Help screen |
| VMODE Vn{n=Oto 13 (hex)} | Switch to VGA modes |
| VMODE n {n=valid hex number} | Set video modes with a valid mode number. See Video mode table for detail. |

2.   MS Windows

If you have never installed MS Windows before, please refer to MS Windows User's Manual for detailed installation procedures. When  it comes to the section where you need to select Display options, choose VGA for your display. After you have completed the MS Windows installation, go through the following steps to install high resolution drivers for your AIO-4045 VGA. Or, if you have already installed MS Windows with the option of another VGA card, do the following steps to re-configure MS Windows to support the AIO-4045 VGA.

1	Enter MS Windows directory and run MS Windows SETUP program by typing "SETUP".

2	A display with the current settings will appear on the screen.  Use the arrow key to highlight the Display Settings and press [Enter].

3	A list of the display options will appear on the screen.  Select the "VGA" option and press [Enter].

4 	Follow the remaining instructions to complete Windows SETUP.

Exit Windows SETUP and start Windows by typing "Win".

5	Under Windows Program Manager, go to heading option "File" and select "RUN".

6	There will be a Windows menu asking for command file.  Insert the AIO-4045 Driver  Disk  2  into your floppy drive A and enter "A:\WIN31UL".  Press [Enter]. Warning: Do not copy the files to your hard disk (installation errors will result).  The software will do this automatically.

7	Windows should start copying all necessary files for you to display high resolution using AIO-4045.

8	Once all the files have been down loaded from the AIO-4045  Software Diskette, there should be a small Windows menu with heading "ALG2228 Resolutions...".  Select the desired resolution and press [Enter].

9	Restart Windows if you want to change the resolution, otherwise, cancel the option.

Note: It is necessary to restart Windows if you select a different display resolution than the one you are currently using.

10	Should you select a different display resolution in the future, simply select the new resolution by double-clicking on the Avance Logic logo icon and restart Windows.

3.   OS/2 Ver. 2.1

NOTE!

0S/2 Ver 2.1 and windows NT drivers are available upon 	request.  Please contact your vendor for details.

To successfully install OS/2 driver, please make sure you have the     AIO-4045  OS/2 2.1 Display Driver Disk 3.

For first time installation of  AIO-4045 OS/2 Drivers Disk 3, please follow steps 1 to 11.  On subsequent  installation of drivers, you may skip steps 1-4.

The OS/2 installation procedure is as follows:

1	Under OS/2 desktop select icon: OS/2 System and then icon: Command Prompts.

2	Select OS/2 or DOS Window (either Desktop or FullScreen).

3	Insert the driver diskette into the floppy drive. (e.g. drive A:)

4	Run INSTALL.BAT or INSTALL.CMD.  (e.g. if the floppy drive is A: and OS/2 is installed on C: then type “A:\install a: c:”). For help on INSTALL.BAT, just type “A:\install”. This will load necessary files needed for OS/2 installation on the hard disk.

Steps 1-4 are required only for the first time installation of Avance Logic drivers. On subsequent installation of drivers you may skip the steps 1-4.

5	On OS/2 window (under Command Prompt) run DSPINSTL.

6 	Select  “Primary Display” and click “OK”.

7	Select “Avance Logic, Inc.” on the driver selection menu.

8 	Select “Install Using Defaults for Monitor Type” on the menu “Monitor Configuration/Selection Utility”.

9 	User will be prompted for Video Refresh Rate Selection Menu.  If you are not sure about the monitor type, select “NO”.  Selecting wrong refresh rate not supported by monitor could hang OS/2 system and you may be required to load the OS/2 system all over again.  If you select “YES”, utility: AScanOS2 is invoked which will present the available scan rates.  If you are still not sure about which scan rates to choose, select the lowest scan rates in each of the four columns.  For information on AScanOS2 read Help provided in the program.

10	Select desired display resolution.

11	Use Disk: Display Driver Diskette when prompted.

NOTE

- 	Please note that the volume label of “Display Driver Diskette” is “DISP 1”.  If you make copies of this disk, make sure you give correct volume label.  (Use DOS command “LABEL” to change the volume label of a disk.)

- 	Close all Windows Seamless and FullScreen applications before you use DSPINSTL.

-	Under 0S/2  desktop, if you are running Windows Seamless session it is possible that the shades of colors may change in the 0S/2 desktop.  This may happen because of palette change and it is quite O.K.  To refresh the screen click  mouse on 0S/2  desktop with Button 2 (just like you do the system shutdown) and choose menu item "Refresh Now".  This will  refresh the 0S/2 desktop with correct color shades.  However, this will not correct any palette problem for applications running under Windows Seamless.

- 	On screen resolution of 1024x768x256 colors, under Windows session if you open “Color” (“Control Panel”) you may not get correct “Custom Color Selector”.  This has been observed in video boards by other manufacturers.  We are contacting IBM in regards to this.

- 	Once you have successfully installed Avance Logic Inc. SVGA drivers, you can easily change Video Refresh rate (without changing screen resolution) by performing steps 5 thru 9.  Select “CANCEL” when prompted at step 10 and onward still you exit from DSPINSTL.  Do system shutdown immediately.

DE-INSTALLING DISPLAY DRIVERS:

- 	If you want to remove Avance Logic, Inc. display drivers, follow steps 5-11 and select “Super VGA (SVGA)” instead of “Avance Logic, Inc.” when prompted at step 7.  Use IBM’s OS/2 original disks during this installation.  During this installation you may get a message saying “... File BVHSVGA is newer than source file ...”.  Press O.K. to this dialogue box.  It is not necessary to perform shutdown at this point.

Open either DOS or OS/2 window.  Insert Avance Logic’s driver diskette in the floppy drive and run “DEINSTAL”.  For instance, if OS/2 has been installed on drive C then type: “A:\deinstal c”. This will restore the original files changed during installation of Avance Logic drivers.  For help on “DEINSTAL” just type “A:\deinstal”.”.

Do system shutdown for the changes to take effect.

NOTE

If your system has not been installed for "Dual Boot" feature, then step 1 will not work for you.  You should then re-boot system with DOS from the floppy drive and continue on with steps 2 to 10.

4.   Windows NT

1	Enter Windows NT.

2	Select Windows NT SETUP, click on the option "Change System Settings".

3	Go to the option "Display" and select "Others".  Insert ALG 2228 windows NT drivers disk into A drive.

4	Select the display resolution of your choice then reboot your system.  You have now installed high resolution Windows NT driver.

5.  Memory  Address  Segments

| ITEM | SPECIFICATION |
| --- | --- |
| RAM | A0000h-BFFFFh |
| ROM | C0000h-C7FFFFh |
| I/O ADDRESS | 3B0h--3DFh (IBM standard) |
| DMA | No DMA channels exist |
| Interrupt | interrupt 9 is used to indicate vertical retrace in some modes (software programmable) |

6. Configure AutoCAD Release 11 for High Resolution Display Driver

To install high resolution display driver successfully, please first make sure that your ACAD R11 runs with the VGA display first.

1	Insert command "SET DSPADI=DSPEGA41" onto the AUTOEXEC.BAT file.  Re-boot system to activate the new AUTOEXEC.BAT file.

2	Insert the ALG2228 driver disk 2 and type "Install".  Select "AutoCAD/R11".

3	Start ACAD.

4	Select option 5, "Configure AutoCAD from the Main Menu".

5	Select Option 3, "Configure Video Display and select device number.  For Example, "ADI P386 v4.0/4.1 display".

6	Select display mode.

7	Finish the rest of the setups and save all changes.

7. Configure AutoCAD Release 12 for High Resolution Display Driver

To install high resolution display driver successful, please first make sure that your ACAD R12 runs with the VGA display first.

1     Insert the ALG 2228 driver disk 2 and type “ Install”. It will copy all    drivers into the “DRV” directory of your ACAD R12 directory.

2	Begin AutoCAD by entering "ACAD -R".  Select option 3 "Configured Video Display".

3	Answer "Yes" when prompted "Do you want to select....." message to display the available video options for AutoCAD.

4	Select "Avance VGA P386 ADI V4.2 32-bit Display List, by Avance Logic".

5	Select the display resolution you desire.

6	Follow the directions provided by the ACAD program to complete the configuration.

7	Save new changes and you have completed the installation.

8. VersaCAD/5.X

The AIO-4045 VGA supports the following high resolution software drivers for VersaCAD/5.X:

AC16.COM for 16-color modes, and

--	800x600 	16-color

--	1024x768	16-color

--	1280x1024	16-color (1MB DRAM)

AC256.COM for 256-color modes, and

--	640x480	256-color

--	800x600	256-color

--	1024x768	256-color (1MB DRAM)

"--": Please refer to the screen display for the selection number that is associated with the display resolution.

1	Insert the AIO-4045 Driver  Disk 1  into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

2	Select the option for Driver Installation and select the option for VersaCAD.

3      Provide the correct drive path for the VersaCAD directory and press

[Enter]. The INSTALL program will automatically copy all            available driver files into the specified directory.

4	Go to your VersaCAD directory and run ENVIRO program by typing "ENVIRO".

5	Press a series of N's to bring up the Screen Device Menu.

6	Press keystroke [S] to select the desired display resolution for your VersaCAD.

7	Once you have selected the display driver, type "X" to exit.

8	While inside the VersaCAD directory, type AC16 or AC256 to load high resolution VersaCAD drivers.

9	You have now completed the installation.  Start VersaCAD as usual and you will be able to work with the high resolution display that you have just configured.

Installing High Resolution Protected Mode ADI Display List Drivers for AutoCAD R11, AutoShade V2.0

Unlike Real Mode ADI Drivers, AVANCE LOGIC GRAPHICS Protected Mode ADI Drivers are not TSR programs and will not occupy your system memory. AutoCAD Release 11 and AutoShade 2.0 will automatically load these protected mode ADI drivers into protected mode memory space (high memory) not only at run time but also at the configuration time.

Setting Proper ADI Environment for AutoCAD Protected Mode Display List Drivers

To allow successful installation, first you will need to setup your ADI environment properly by using the following steps:

1 	Insert the  AIO-4045 Driver Disk 2  into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

2	Select the option for Driver Installation and select "PADI386".

3	Provide the correct drive path for the AutoCAD directory and press [Enter].  The INSTALL program will automatically copy all available driver files into the specified directory.

4	Use DOS editor or any other editor to place the following commands to be the first two line commands of your AUTOEXEC.BAT file:

SET DSPADI=drv:\path\filename

SET RDPADI=drv:\path\filename

NOTE

"DSPADI": The environment name recognized by AutoCAD.

"RDPADI": The environment name recognized by AutoShade.

"drv:\path"  Enter  the  drive  ID  and  path  name  where   you have       stored your AVANCE LOGIC GRAPH ICS high resolution protected mode ADI display List drivers.

"filename":	Select the following filenames:

RCPVGA41.EXP (w/o Display List)

DSPEGA41.EXP (with Display List)

SDSP.EXP (Display List with Linear Address)

5	Reboot your system to activate the new AUTOEXEC.BAT file.

NOTE!

Do not install Display List with Linear Address driver(SDSP.EXP) if you have a system which contains 16 MB of the memory.  Due to the limitation on the ADI4.1 and AT bus, installing SDSP.EXP will create memory conflicts between your system and the video memory.

9. Configure AutoShade Version 2.0 for High  Resolution Protected Mode ADI Display List Driver

Before  configuring   AutoShade  Version  2.0,  set  environment  variables

RDPADI  as  described in the section Setting Proper ADI Environment; then do the following steps:

1	At the DOS prompt, type the following command:

SHADE -r	[Enter]

This command allows AutoShade to run the configuration screen first.

2 	You will now be prompted to select the pointing device:

Select pointing device:

1. Autodesk Device Interface pointer.

2. P386 Autodesk Device Interface pointer.

.

.

Pointer selection:

Enter the option number to choose the appropriate pointing device.

3	Select the display device driver:

Select display device:

1. Autodesk Device Interface display driver.

2. P386 Autodesk Device Interface display driver.

.

.

Display selection:

Note!

Select option 1:

If you have installed AVANCE LOGIC GRAPHICS ADI 4.0 Display List driver, follow the installation procedures under Installing High Resolution Real Mode ADI Display List Drivers for AutoCAD Release 9 and 10.

If you have installed AVANCE LOGIC GRAPHICS ADI 4.1 Display List driver, follow the installation procedures under Installing High Resolution Real Mode ADI Display List Driver for AutoCAD Release 11.

Select option 2:

If you have installed AVANCE LOGIC GRAPHICS Protected Mode ADI Display List driver, follow the installation procedures under Installing High Resolution Protected Mode ADI Display List Drivers for AutoCAD R11, AutoShade V2.0.

NOTE!

The dual screen feature will not be supported if you are using the Protected Mode ADI Display List driver.

4	Select display resolution by entering the option number.

5	Select the rendering device:

Select rendering display device:

1. Autodesk Device Interface rendering driver.

2. P386 Autodesk Device Interface rendering driver.

.

.

Rendering selection:

Select option 2, then press [Enter].

6	Select the Rendering resolution:

Select rendering resolution:

1. 640 x 400 x 256 color

2. 640 x 480 x 256 color

.

.

Select the resolution you wish to use and press [Enter].

7	You will now be prompted with the following message:

"Choose Breshenham polygon fill from scan conversion polygon fill. Select Breshenham polygon fill algorithm [Y/N]? <Y>"

Answer Y or N depending on your application.

8	When you are prompted "Does the display and AutoShade rendering devices share a single screen? (default = NO):"

9   	When you are prompted with the message "Does the display and Autodesk RenderMan rendering devices share a single screen? (default = NO):"

Type Y and press [Enter]

10	When you are prompted "Does FLIP SCREEN require a redraw? (default = NO):"

Type Y and press [Enter].

11 	Select Rendering hard copy device:

Select Rendering hard copy device:

1. Autodesk Device Interface Rendering driver.

2. P386 Autodesk Device Interface Rendering driver.

Rendering hard copy selection:

If you have any rendering hard copy devices installed in your system, select the option number associated with the device that you are using. Otherwise, select the number associated with item "none".

You have now completed the configuration of AVANCE LOGIC GRAPHICS Protected Mode ADI driver for AutoShade.

10.  CADKEY/3

The AIO-4045 VGA Card supports the following high resolution software drivers for CADKEY/3:

AC16.EXE for 16-color modes, and

--	640x480	16-color

--	800x600 	16-color

--	1024x768	16-color

--	1280x1024	16-color (1MB DRAM)

AC256.EXE for 256-color modes, and

--	640x480	256-color

--	800x600	256-color

--	1024x768	256-color (1MB DRAM)

--	1280x1024	16-color (1MB DRAM)

"--": Please refer to the screen display for the selection number that is associated with the display resolution.

1 	Insert the AIO-4045 Driver Disk 1  into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

2	Select the option for Driver Installation and select the option for CADKEY.

3	Provide the correct drive path for the CADKEY directory and press [Enter].  The INSTALL program will automatically copy all available driver files into the specified directory.

4	Go to your CADKEY directory and list out the contents of the ALCK3.DAT file on screen by entering:

Type ALCK3.DAT		[Enter]

NOTE!

Use [Ctrl]-[S] to pause for reading and press [Enter] to continue (You may want to print out the contents of the ALCK3.DAT).

5	Use line editor to append the entire contents of the ALCK3.DAT into the filename "GRDEV.DAT".

6	Inside the CADKEY directory, type "CONFIG" to re-configure

ADKEY.

7	Select option (1) "Set graphics options" from the CADKEY Main menu.

8	Select option (1) "Select graphics device" from the Set Graphics options.  You should see the following display:

*	ALGXXXX  16-color display

*	ALGXXXX 256-color display

9	Choose option (1) "Graphics Device Mode Selection" to select the desired display resolution.

10	At the Display Monitor Selection choose the option for your monitor.

11	You will now be prompted "Enter the X & Y size for the CADKEY graphics cursor."  Simply press [Enter] to select the default configuration.

12	Again, you will be prompted "Enter the size for the CADKEY graphics marker".  Simply press [Enter] again to select the default configuration.

13	Continue with the rest of the installation procedure until you  are able to select option (9) "Exit and save changes".

14	While you are in the CADKEY directory, type "AC16" or "AC256" to load the CADKEY drivers.

15	You have now completed the installation.  Start CADKEY by typing "CKGR" and you will be able to work with the high resolution display that you have just configured.

11.   EASYCAD/2.72

The AIO-4045 VGA supports the following high resolution software drivers for EASYCAD/2:

Driver name		Resolution

DSA800.BIN	--	800x600/16 color

DSA1024.BIN --	1024x768/16 color

DSA1280.BIN --	1280x1024/16 color (1MB DRAM)

1	Insert the DSP-2228 Driver Disk 1 into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

2	Select the option for Driver Installation and select the option for EASYCAD.

3	Provide the correct drive path for the EASYCAD directory and press [Enter].  The INSTALL program will automatically copy all available driver files into the specified directory.

4	Select the driver name with the resolution you would like to display and copy the driver to filename DSDRV.BIN.  For example, select DSA1024.BIN for your 1024 x 768 display and type "COPY DSA1024.BIN DSDRV.BIN" at the DOS prompt.

5	Run the program "ADJUST.EXE" to reconfigure your EASYCAD  environment and select option 2 for display driver.  Follow through the instructions on the screen to complete the configuration.

6	Save the new configurations and a display message "INSTALLATION COMPLETED SUCCESSFULLY" will appear on screen.

7	Start EASYCAD by typing "ECAD".

12.  Framework III

1	Insert the AIO-4045 Driver  Disk  1 into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

2	Select the option for Driver Installation and select the option for Framework III.

3	Provide the correct drive path for the Framework III directory and press [Enter].  The INSTALL program will automatically copy all available driver files into the specified directory.

4	Enter the Framework III directory and type "SETUP" at the DOS prompt.

5 	When the SETUP screen appears, choose option 2, "All other uses of the setup program".

6 	Choose option 2 for running Framework III from your hard disk drive.

7 	Select option 2 again for CONFIGURATION.

8 	Choose option 1 to select the "PRIMARY HARDWARE" configurations.

9 	Enter number 1 to indicate that you wish to select the Screen Driver.  Enter number 7 for "I want to enter my own driver file name" and enter the name of the desired driver from the following list.

Screen 	Format	Zoom Screen	Filename

80Cx30R 	Text	640Hx480V	AF240.SC   80Cx43R  	Text	640Hx480V	AF250.SC 80Cx60R  	Text	640Hx480V	AF260.SC  132Cx25R 	Text	640Hx480V	AF200.SC 132Cx43R 	Text	640Hx480V	AF220.SC 132Cx60R 	Text	640Hx480V	AF230.SC 80Cx30R  	Text	800Hx600V	AF241.SC 80Cx43R  	Text	800Hx600V	AF251.SC 80Cx60R  	Text	800Hx600V	AF261.SC 132Cx25R 	Text	800Hx600V	AF201.SC

132Cx30R 	Text	800Hx600V	AF211.SC

132Cx43R 	Text	800Hx600V	AF221.SC

132Cx60R 	Text	800Hx600V	AF231.SC

640Hx480V 	Graphics	640Hx480V	AF12.SC 800Hx600V	Graphics	800Hx600V	AF2B.SC

10	After you have entered the driver name, type "M" to return to the Main menu.

11	From the Main Menu, type "7" to save all your settings.

12	Save the new configuration. Choose option "2" for running Framework III from the hard disk drive. Once you have made the choice, strike any key to exit to DOS.

i.e. : To   configure 132x60-column  display with 800x600  resolution for   the  "Zoom Screen"  function,   choose   the   driver  named AF1E1.SC

13.  LOTUS/123 and Symphony

Installing extended display drivers for Lotus/123 Release 2.x, and Symphony Release 1.x.

1	Insert the AIO-4045 Driver Disk 1 into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

2	Select the option for Driver Installation and select the option for Lotus 123 and Symphony.

3	Provide the correct drive path for the Lotus and Symphony directory and press [Enter].  The INSTALL program will automatically copy all available driver files into the specified directory.

4 	Change to your Lotus/123 directory and delete old SINGLE.LIB by typing:

Del SINGLE.LIB	[Enter]

5  	Type "Lotus" to open the Lotus Main menu.

6 	Select "Install" from the Lotus Main menu.

7 	Select "Advanced options" from the Install menu.

8 	Select "Add new driver to library" from the Advanced Options menu.

9 	Select "Modify current driver set" from the menu.

10  	Select either Text or Graphics display.

11	Select the following display resolutions:

ALGXXXX ( 132x25 ) Rel.2	<--	Text

ALGXXXX ( 132x30 ) Rel.2.	<-- 	Text

ALGXXXX ( 132x43 ) Rel.2.	<-- 	Text

ALGXXXX ( 132x60 ) Rel.2.	<-- 	Text

ALGXXXX ( 640x480/16 ) Rel.2.	<-- 	Graphics

ALGXXXX ( 800x600/16 ) Rel.2.	<-- 	Graphics

12 	Choose "Save Changes" to keep the changes you have made and  exit the Lotus/123 installation program.

13	Use the same procedures to setup Lotus/Symphony.

14.  PCAD

1 	Insert the AIO-4045 Driver Disk1  into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

2	Select the option for Driver Installation and select the option for PCAD.

3      Provide the correct drive path for the PCAD directory and press  [Enter].  The INSTALL program will automatically copy all available driver files into the specified directory.

4	Available display drivers for PCAD version 4.0.

800x600/16  	:	AP800.DRV

1024x768/16 	: 	AP1024.DRV

1280x1024/16	: 	AP1280.DRV (required 1MB)

Available display drivers for PCAD version 4.5.

800x600/16  	: 	AP800.DRV

1024x768/16 	: 	AP1024.DRV

1280x1024/16	: 	AP1280.DRV (required 1MB)

Use line editor to modify the "display" line in the PCADDRV.SYS file to the selected display driver name.

For example, PCADDRV.SYS can be modified to select 800x600 16-color driver as:

DISPLAY \ PCAD \ AP800.DRV

5	Reboot your system and you are able to run PCAD in high resolution display.

15.  Ventura Publisher Rel. 2

1	Insert the AIO-4045 Driver Disk  1 into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

2	Select the option for Driver Installation and select the option for Ventura Publisher.

3	Provide the correct drive path for the VP directory and press [Enter].  The INSTALL program will automatically copy all available driver files into the specified directory.

4 	Start VP installation program by typing:

A:\VPDRV2_0		[Enter]

5  	Select high resolution display mode a by choosing option A, B, C, etc.:

A. ALGXXXX 800x600/16 colors display

B. ALGXXXX 1024x768/4 colors display

C. ALGXXXX 1024x768/16 colors display

D. ALGXXXX 1280x1024/16 colors display (1MB)

6.  	Complete the installation procedures by following the screen instructions and responding to the remaining questions.

16.  WordPerfect

1	Configure WordPerfect/4.2 for extended text display:

1)	Enter the directory where you have installed the ALG 2228 Driver Disk 1 utility programs.  Run VMODE.EXE program to select the extended text mode that you wish to use (i.e. 132 column mode).  For convenience, you may copy "VMODE.EXE" program to your WP or root directory.

2)  	Go to the directory where you have stored WordPerfect or insert the WordPerfect program diskette into floppy drive A if you are using a floppy system and type:

WP/S		[Enter]

3)  	Select option 3, "Set Screen and Beep options".

4)  	Enter the numbers for the column and row to match the one you have selected with the VMODE.EXE program.

5)  	Choose option 0 to accept the new configuration and enter WordPerfect.

NOTE!

When you exit from WordPerfect, type the command line "VMODE 3" and press [Enter] to return to the standard 80x25 VGA text display.

Each time you run WordPerfect for extended text mode display, you will need to type "VMODE" and select the option that matches the columns and rows that you had set previously in your WordPerfect setup (WP/S).

Once you have configured WordPerfect for a particular extended text mode, you need not repeat steps 1 through 5 unless you want to change to a different display mode.

2	Configure WordPerfect/5.0 for Extended Graphics display:

1) 	Insert the AIO-4045 Driver Disk 1  into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

2)	Select the option for Driver Installation and select the option for WordPerfect/5.0.

3)	Provide the correct drive path for the WP50 directory and press [Enter].  The INSTALL program will automatically copy all available driver files into the specified directory.

4)  	Start WordPerfect program by typing "WP".

5)  	Setup new display by pressing keystroke [SHIFT-F1].

6)  	Select option "(2) Display".

7)  	Select new Screen driver. Select option "(5) Graphics Screen Type".

Ext. 16 colors 	- 	-  .  .  .  .  .  .

- 1024x768

- 1280x1024

8)  	Save the new configuration.

3	Configure WordPerfect/5.1 for extended text/graphic display:

1) 	Insert the AIO-4045 Driver Disk 1  into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

2)	Select the option for Driver Installation and select the option for WordPerfect/5.1.

3)	Provide the correct drive path for the WP51 directory and press [Enter].  The INSTALL program will automatically copy all available driver files into the specified directory.

4)  	Start WordPerfect program by typing "WP".

5)	Setup new display by pressing keystroke "SHIFT-F1".

6)  	Select option "(2) Display".

7)  	Select new Screen driver. Select option "(2) Graphics Screen Type".

Ext. 16 colors	-  .  .  .  .  .  .

- 1024x768

- 1280x1024	(required 1MB)

Ext. 256 colors 	-  .  .  .  .  .  .

-  640x480

-  800x600

- 1024x768	(require 1MB)

Select (3) Text Screen Type.	132 column TEXT 	-  .  .  .  .  .

-  132x30

-  132x43

-  132x60

-  .  .  .  .  .

8)  	Save the new configuration.

17.	WordStar Release 3

1	Check your DOS system disk or directory to see if you have the  file named "DEBUG.COM" or "DEBUG.EXE".

2	Make a backup copy of the WS.COM file from your WordStar diskette or the WordStar directory on your hard drive, and give it a new filename "WS132.COM".

3	Insert the AIO-4045 Driver Disk 1 into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

4	Select the option for Driver Installation and select the option for WordStar .

5	Provide the correct drive path for the WS3 directory and press [Enter].  The INSTALL program will automatically copy all available driver files into the specified directory.

6 	Go to WordStar directory and modify WordStar to work with the 132 column text mode (i.e. 132x25) by typing:

MAKEWS  13225  WS132.COM 	[Enter]

7 	Run VMODE.EXE to select the video mode you wish to display. (i.e. Select the 132x25 mode)

8  	Go to your WordStar directory and run WordStar Release 3 at 132x25 text display by typing:

WS132 			 [Enter]

18.	 WordStar Release 4

1 	Insert the AIO-4045 Driver Disk 1 into your floppy drive A.  At the DOS prompt, simply type:

A:\INSTALL	[Enter]

2	Select the option for Driver Installation and select the option for WordStar 4.

3	Provide the correct drive path for the WS4 directory and press [Enter].  The INSTALL program will automatically copy all available driver files into the specified directory.

4	Go to your WordStar Professional Release 4 directory and start WordStar’s installation program by typing:

WSCHANGE

5  	Type "WS.EXE" as the filename of your WS program file, and type "WS132.EXE" as the filename for new changes.

6  	Select option A for "Console options".

7 	Select option A for "Monitor options".

8 	Choose option C for "Screen Sizing".

9	Choose option A for "Height" at the Screen Sizing Menu and   enter a desired row number for the screen format  (i.e. 25, 30, 43 or 60).

10 	Choose option B for “width” at the Screen Sizing Menu and enter a desired column number (i.e. 132 or 80) for the screen size.

11  	Once you have selected the screen format, return to the Main Installation Menu by typing a series of "X"s.

12 	Run VMODE.EXE to select the extended video mode (be sure that the mode you selected is the same as the one you have chosen earlier in the WordStar configuration).

13 	Start WordStar Release 4 by typing:

WS132		[Enter]

You won’t need to repeat this installation procedure unless you want to configure WordStar Release 4 with a different display mode.

NOTE

Do the following steps whenever  you start WordStar Release 4:

1)	Run VMODE.EXE to  select the extended display mode (same as 	the one you have previously saved in the WordStar Release 4 	configuration.)

2)	Type "WS" or "WS123" to start WordStar.

3)    When you exit WordStar, type "VMODE 3" return to the 	 standard VGA display mode.

CHAPTER  4 LAN

4.1  OVERVIEW

The Jumperless LAN, referred to as W89C904AX Series, are designed in one type of boards as described below:

INET2000B:	Equipped with BNC ports, which  allow connecting  to      thin coaxial cables.

Featuring a high data transmission rate of 10 mega bits per second, the Cards conform to the IEEE 802.3 standard protocol.

The Jumperless LAN use the software program named LANSTART.EXE to set up the hardware parameters (such as IRQ, I/O address...etc.) or the configurations, and to test/diagnose the condition of the boards.

Designed with menu-driven screens, the LANSTART.EXE software program is user-friendly and easy to use.  The cards can be installed directly without running the software when users use the default settings of the cards.  In other words, to configure the cards is necessary only when non-default parameters are required.  Notice that if multiple cards are to be used, you should install and configure one at a time.

The LAN series are fully hardware and software compatible with Novell's NE2000, which can be used in a file server or a workstation under Novell's Netware operating system.

The LAN series also support various network system, such as Artisoft's LANtastic, Microsoft's Windows for Workgroups, IBM OS/2 etc.

4.2  FEATURES

¯	16 Bit Ethernet adapter.

¯	Novell NE2000 Ethernet AT hardware compatible board.

¯	Compliant with IEEE 802.3 Ethernet Standard.

¯	The board with 10Base-2 BNC Interface.

¯	Supports Netware, TCP/IP, NDIS, ODI and NetBIOS etc.

¯	Supports 16K/64K multi-packet RAM buffer.

¯	IEEE registered NODE ID.

¯    No jumpers or switches, Software configuration for:

¡X IRQ Lines

¡X I/O base address

¡X Boot ROM size and address

¡X Cable type 10Base-2

¯	A remote  reset  Boot  ROM  chip  is  available   that  allows  a  	diskless workstation to  boot  from   the   file server or ChipAway 	AntiVirus chip, that detects the hard disk virus.

NOTE :   If  you want to create a  new or modify the partition  for hard 	 	disk, disable the Boot ROM function first.

4.3  SOFTWARE SETUP

1. CONFIGURATION

The LAN is jumperless. Settings are controlled entirely through menu choices and entries in the SETUP program, supplied on the diskette.  The LAN card must be installed when the setup program is run.  But, network drivers should not be installed.  To start the program, place the diskette in your floppy drive, for example, drive A.  Then type:

A:>LANSTART     Ê(Enter)

The main menu will appear:

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

‘Enter’ key = OK ‘Enter’ key = CANCEL (a) There is 1 Winbond W89C904AX only. I/O base=300h, IRQ=3, IO Mapping.

LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Press any key to continue... get id number ... PASS DWID setting ... PASS Initialize NIC page 0 register contains ... OK Initialize NIC page 1 register contains ... OK check ring buffer : address 0x4000 - 0x7FFF ... PASS Allocate pc memory and get tested data ... PASS

LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

2. CONFIGURING THE CARD

Press <Enter> to open the “Configurate the W89C904AX” when this option is high-lighted in the main menu of the software. Refer to the figure below:

”

¡¹ Configurate the W89C904AX ¡¹

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Configure the W89C904AX Show current configuration of W89C904AX set configuration A set configuraiton B set configuration C write new configuration to EEPROM Update new configuraioin to W89C904AX

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

Press <enter> to open the current configuration of W89C904AX when the “Show current configuration of W89C904AX” option is high-lighted in the sub-menu of the “Configurate the W89C904AX” of the software.

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Configuration A Configuratioin C IOBASE : 300H BPBASE : No BOOT INT : 000 IRQ3 COMP : Compatible mode FAST : Normal Remote DMA INTM : Coded mode CLKSEL : Clock from 20MHz SOFTTEN : Software Enable Configuration B Thick : Thin Ethernet GDLINK : Enable GDLINK EIO16 : Normal IO 16 ECHRDY : Normal IOCHRDY BPWE : Boot PROM Write Enable

Press any key to exit.... LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

¡¹ OPEN THE SET CONFIGURATION A:

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Configure the W89C904AX S S S S w Update new configuraioin to W89C904AX Set Configuration A IOBASE: INT : 000 IRQ3 FAST : Normal Remote DMA 300H

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

¡¹ IOBASE : I/O ADDRESS

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Configure the W89C904AX S S S S w Update new configuraioin to W89C904AX I/O Address HOME-é IOBASE= Soft 240H 280H 2C0H 320H 340H 360H 300H 300H “Enter” key = OK “Esc” key = CANCEL END

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

¡¹ INT : Interrupt Request (IRQ)

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Configure the W89C904AX S S S S w Update new configuraioin to W89C904AX Interrupt Line Used HOME-é INT = 000 IRQ3 001 IRQ4 010 IRQ5 011 IRQ9 100 IRQ10 101 IRQ11 110 IRQ12 111 IRQ15 000 IRQ3 “Enter” key = OK “Esc” key = CANCEL END

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Configure the W89C904AX S S S S w Update new configuraioin to W89C904AX Fast Read HOME-é FAST = Fast Remote DMA Normal Remote DMA “Enter” key = OK “Esc” key = CANCEL END Normal Romote DMA

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Configure the W89C904AX S S S S w U Set Configuration B Thick : GDLINK : Enable GDLINK EIO16 : Normal IO16 ECHRDY : Normal IOCHRDY BPWE : Boot PROM Write Enable Thin Ethernet

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Configure the W89C904AX S S S S w Update new configuraioin to W89C904AX Physical Layer Interface HOME-é Thick = TPI (10 BASE T) Thick Ethernet TPI (No. spec) Thin Ethernet “Enter” key = OK “Esc” key = CANCEL END Thin Ethernet

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

Boot ROM Address

The “disable” and a list of Boot ROM addresses will be displayed when this option is selected. The default Boot ROM setting is “disable” which indicates the remote boot function is disabled. You are able to enable the remote boot by selecting one of  Boot ROM addresses.

Note

Each time when the setup software is run, the Boot ROM setting is always “disabled” whether the Boot ROM is installed or not; even if the Boot ROM setting has been “enabled” last time.

Therefore, each time when the software is run, the Boot ROM address should be reselected if you need the remote boot function.

For the list of boot ROM address, refer to the table on the next page.

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Configure the W89C904AX S S S S w U Set Configuration C BPBASE : COMP : Compatible mode INTM : Coded mode CLKSEL : Clock from 20MHz SOFTTEN : Software Enable No BOOT

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

Note:

There are no mapping limitations between Boot ROM address and I/O base address. Yet you should be careful not to let the conflict happen between your your Shared RAM address and Boot ROM address:

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Configure the W89C904AX S S S S w Update new configuraioin to W89C904AX Physical Layer Interface HOME-é BPBASE = C8000H 8K/16K CC000H 8K/16K D0000H 8K/16K D4000H 8K/16K D8000H 8K/16K DC000H 8K/16K C8000H 32/32K No BOOT “Enter” key = OK “Esc” key = CANCEL END No BOOT

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

Save the Settings

In the “Configurate the W89C904AX”, select the “Update new configuration to W89C904AX” option allowing you to save the settings.

3. Diagnosing the Card

Select the “Diagnose” option in the main menu and press <Enter> to open the “Diagnose”.  You are allowed to do the tests from the Diagnose. To perform one of tests, press <Enter> after the option is high-lighted.

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Diagnose Network functional test Auto functional test Perform functional tests

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

Internal Test:

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Diagnose S S S S w Update new configuraioin to W89C904AX Network functional test register read/write test : PASS remote DMA read/write test : PASS send packet test : PASS loopback mode 1 test : PASS loopback mode 2 test : PASS

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions C D T A R Access physical address physical address = 00 c0 4c 3 50 check ring buffer : address 0x4000 - 0x7FFF... PASS press <ESC> to continue...

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

Exit the Software

By pressing the <Esc>  key allows you to leave the software and return to the Main Menu as the table below:

Winbond W89C904AX LANSTART (Setup, Diagnostics and Tx/Rx) Ver 1.17

Main Functions Diagnostics Transmit/receive p Access physical ad Read W89C904AX Press “X” to confirm exit. Press “C” to continue Configure the W89C904AX

F1: Help F10: Exit á: up â: down <Enter>: execute <Esc>: End testing LANSTART is a copy righted s/w by Winbond Elec. Corp, lisence to Winbond

By pressing <C> button to continue or by pressing <X> button to confirm exit and return to the DOS prompt.

APPENDIX A  AWARD POST CODE

When the system is powered on, the BIOS will perform diagnostics and initialize all system components, including the video system.  All errors found by the BIOS will be put in I/O port 80H. (By installing the post code display card, the post codes can be displayed.)

| POST | Name | Description |
| --- | --- | --- |
| C0 | Turn Off Chipset Cache | OEM Specific-Cache controller. |
| 1 | Processor Test 1 | Processor Status (1FLAGS) Verification. Tests the following processor status flags carry, zero, sign, overflow. The BIOS will set each of these flags, verify they are set, then turn each flag off and verify it is off. |
| 2 | Processor Test 2 | Read/Write/Verify all CPU registers except SS, SP, and BP with data pattern FF and 00. |
| 3 | Initialize Chips | Disable NMI, PIE, AIE, UEI, SQWV. Disable video, parity checking, DMA. Reset math coprocessor. Clear all page registers, CMOS shutdown byte. Initialize timer 0, 1, and 2, including set EISA timer to a known state. Initialize DMA Controllers 0 and 1. Initialize interrupt controllers 0 and 1. Initialize EISA extended registers. |
| 4 | Test Memory Refresh Toggle | RAM must be periodically refreshed inorder to keep the memory from decaying. This function assures that the memory refresh function is working properly. |
| 5 | Blank video, Initialize keyboard | Keyboard controller initialization. |
| 6 | Reserved |  |
| 7 | Test CMOS Interface and Battery Status | Verifies CMOS is working correctly, detects bad battery. |
| BE | Chipset Default Initialization | Program chipset registers with power on BIOS defaults. |
| C1 | Memory presence test | OEM Specific-Test to size on-board memory. |
| C5 | Early Shadow | OEM Specific-Early Shadow enable for fast boot. |
| C6 | Cache presence test | External cache size detection. |
| 8 | Setup low memory | Early chip set initialization. Memory presence test. OEM chip set routines. Clear low 64K of memory. Test first 64K memory. |
| 9 | Early Cache Initialization | Cyrix CPU initialization. Cache initialization. |
| A | Setup Interrupt Vector Table | Initialization first 120 interrupt vectors with SPURIOUS_INT_HDLR and initialize INT 00h-1Fh according to INT_TBL. |
| B | Test CMOS RAM Checksum | Test CMOS RAM Checksum, if bad, or insert key pressed, load defaults. |
| C | Initialize keyboard | Detect type of keyboard controller (optional) Set NUM_LOCK status. |
| D | Initialize Video Interface | Detect CPU clock. Read CMOS location 14h to find out type of video in use. Detect and Initialize Video Adapter. |
| E | Test Video Memory | Test video memory, write sign-on message to screen. Setup shadow RAM - Enable shadow according to Setup. |
| F | Test DMA Controller 0 | BIOS checksum test. Keyboard detect and initialization. |
| 10 | Test DMA Controller 1 |  |
| 11 | Test DMA Page Registers | Test DMA Page Registers. |
| 12-13 | Reserved |  |
| 14 | Test Timer Counter 2 | Test 8254 Timer 0 Counter 2. |
| 15 | Test 8259-1 Mask Bits | Verify 8259 Channel 1 masked interrupts by alternately turning off and on the interrupt lines. |
| 16 | Test 8259-2 Mask Bits | Verify 8259 Channel 2 masked interrupts by alternately turning off and on the interrupt lines. |
| 17 | Test Stuck 8259's Interrupt Bits | Turn off interrupts then verify no interrupt mask register is on. |
| 18 | Test 8259 Interrupt Functionality | Force an interrupt and verify the interrupt occurred. |
| 19 | Test Stuck NMI Bits (Parity/IO Check) | Verify NMI can be cleared. |
| 1A |  | Display CPU clock. |
| 1B-1E | Reserved |  |
| 1F | Set EISA Mode | If EISA non-volatile memory checksum is good, execute EISA initialization. If not, execute ISA tests an clear EISA mode flag. Test EISA Configuration Memory Integrity (checksum & communication interface). |
| 20 | Enable Slot 0 | Initialization slot 0 (System Board). |
| 21-2F | Enable Slts 1-15 | Initialize slots 1 through 15. |
| 30 | Size Base and Extended Memory | Size base memory from 256K to 640K and extended memory above 1MB |
| 31 | Test Base and Extended Memory | Test base memory from 256K to 640K and extended memory above 1MB using various patterns. NOTE: This will be skipped in EISA mode and can be "skipped" with ESC key in ISA mode. |
| 32 | Test EISA Extended Memory | If EISA Mode flag is set then test EISA memory found in slots initialization. NOTE: This will be skipped in ISA mode and can be "skipped" with ESC key in EISA mode. |
| 33-3B | Reserved |  |
| 3C | Setup Enabled |  |
| 3D | Initialize & Install Mouse | Detect if mouse is present, initialize mouse, install interrupt vectors. |
| 3E | Setup Cache Controller | Initialize cache controller. |
| 3F | Reserved |  |
| BF | Chipset Initialization | Program chipset registers with Setup values. |
| 40 |  | Display virus protest disable or enable. |
| 41 | Initialize Floppy Drive & Controller | Initialize floppy disk drive controller and a drives. |
| 42 | Initialize Hard Drive & Controller | Initialize hard drive controller and any drives. |
| 43 | Detect & Initialize Serial/Parallel Ports | Initialize any serial and parallel ports (also game port). |
| 44 | Reserved |  |
| 45 | Detect & Initialize Math Coprocessor | Initialize math coprocessor. |
| 46 | Reserved |  |
| 47 | Reserved |  |
| 48-4D | Reserved |  |
| 4E | Manufacturing POST Loop or Display Messages | Reboot if Manufacturing POST Loop pin is set. Otherwise display any messages (i.e., any non-fatal errors that were detected during POST) and enter Setup. |
| 4F | Security Check | Ask password security (optional). |
| 50 | Write CMOS | Write all CMOS values back to RAM and clear screen. |
| 51 | Pre-boot Enable | Enable parity checker. Enable NMI, Enable cache before boot. |
| 52 | Initialize Option ROMs | Initialize any option ROMs present from C8000h to EFFFFh. NOTE: When FSCAN option is enabled, will initialize from C8000h to F7FFFh. |
| 53 | Initialize Time Value | Initialize time value in 40h: BIOS area. |
| 60 | Setup Virus Protect | Setup virus protect according to Setup. |
| 61 | Set Boot Speed | Set system speed for boot. |
| 62 | Setup NumLock | Setup NumLock status according to Setup. |
| 63 | Boot Attempt | Set low stack. Boot via INT 19h. |
| B0 | Spurious | If interrupt occurs in protected mode. |
| B1 | Unclaimed NMI | If unmasked NMI occurs, display. Press F1 to disable NMI, F2 reboot. |
| E1-EF | Setup Pages | E1-Page 1,E2-Page 2, etc. |
| FF | Boot |  |

APPENDIX  B  BEEP CODE

During the POST (Power On Self Test) routines, which will be performed each time when the system is powered on, errors may occur.

Non-fatal errors are those which, in most cases, allow the system to continue the boot-up process. The error messages normally appear on the screen.  See Appendix B for BIOS Error Messages.

Fatal errors are those which will not allow the system to continue the boot-up procedure.  If a fatal error occurs, you should consult with your system manufacturer for possible repairs.

These fatal errors are usually communicated through a series of audible beeps.  The numbers on the fatal error list below correspond to the number of beeps for the corresponding error.  All errors listed, with the exception of #8, are fatal errors.

No. of Beeps

1.	Refresh Failure- The memory refresh circuitry of the motherboard is faulty.

2. 	Parity Error- A  parity error  was  detected  in  the  base memory ( the first block of 64KB )  of  the system.

3.	Base 64KB Memory Failure- A memory failure occurred within the first 64KB of memory.

4.	Timer Not Operational- Timer #1 on the system board has failed to function  	properly.

5. 	Processor Error- The CPU on the system board has generated an error.

6. 	8042-Gate A20 Failure- The  keyboard  controller  (8042)  contains  the Gate A20 switch    which allows the  CPU  to  operate   in  virtual  mode.  This  error  message means that  the BIOS is not able to switch the CPU into protected mode.

7. 	Processor Exception Interrupt Error- The  CPU  on    the   motherboard   has   generated   an exception interrupt.

8.  	Display Memory Read/Write Error- The system video adapter is either missing or its memory is faulty. PLEASE NOTE: This is not a fatal error.

9.	ROM Checksum Error- The  ROM  checksum   value   does   not   match  the value  encoded in  the BIOS.

10.	CMOS Shutdown Register Read/Write Error-    The  shutdown  register  for  the  CMOS  memory has failed.

RMA FORM

When M/B can't work well, please fill this form to describe related situations. If the space is not enough to use, you can attach separate paper.

MODEL:                                                               MODEL NO.:

HARDWARE:

CPU: Brand _____________, Model ___________, Speed _____________ MHz

CO-PROCESSOR: Brand___________, Model __________, Speed ________MHz

SIMM: Brand________, Speed________ ns, Q'ty _______ pcs, Total ________ MB

CACHE: Brand ____________, Speed ___________ns, Total ____________K

TAG RAM: Brand ____________, Speed ___________ns

BIOS DATE CODE: ___________

SYSTEM SPEED RUNNING ___________ MHz

VIDEO  CARD: Chip__________, RAM_________, Masker_________VGA Mode

OTHER ADD-ON CARDS:

SOFTWARE:

OPERATION SYSTEM _________________ VERSION _______________

SOFTWARE PROGRAM ________________________________________

BIOS SETUP: DRAM wait _______________ CACHE wait _______________

If you change BIOS SETUP, please describe the changes:

<A> ERROR

¨  HANG UP         ¨  NO  SCREEN       ¨  FLOPPY R/W ERROR

¨  HARD DISK R/W ERROR      ¨  PARITY MEMORY ERROR

¨  OTHER ______________________________________________

<B> ERROR MESSAGES ON YOUR SCREEN (PLEASE SHOW US THE 	WHOLE SENTENCE)

<C> PROBLEM DESCRIPTION