AIO8554

SYSTEM SECTION

TABLE OF CONTENTS

CHAPTER 1 INTRODUCTION	1

1.1   Overview	1

1.2   System Features	1

1.2.1 VGA FEATURES	2

1.3   System Specification	2

1.4   SYSTEM PERFORMANCE	3

1.5   AIO8554 BOARD LAYOUT	4

CHAPTER 2 INSTALLATION	5

2.1   DRAM Installation	5

2.2   sram installation	6

2.3   SYSTEM BUS frequency installation 	7

2.4   CPU FREQUENCY INSTALLATION	8

2.5   other jumper & connector installation	9

CHAPTER 3 SYSTEM BIOS SETUP	12

APPENDIX 		35

RMA FORM

chapter 1

INTRODUCTION

1.1	Overview

The AIO8554 is complemented by a 512KB second level Write-Back cache providing workstation level computing performance, and SIMM sockets support up to 128MB of DRAM.

The AIO8554 motherboard offers outstanding I/O capabilities. One EISA expanded slot for ISA/PCI Local Bus slots provide a high bandwidth data path for data-movement intensive function such as Graphics.

The AIO8554 motherboard provides the foundation for cost effective, high performance, highly expandable platforms which deliver the latest in CPU and I/O technologies.

1.2	System Features

q	Supports INTEL PENTIUM  75/90/100/120/133 MHz

q	Supports H/W GREEN/WAKE UP Switch

q	Supports EDO DRAM

q	Supports L1/L2 Write Back/Write Through Cache Feature

q	Supports EISA Expanded Slot for ISA/PCI Bus

q	Supports 256KB/512KB Cache Size

q	Supports 72pin SIM MODULES

q	Supports SMI/SMM/PMU/APM Power Controllers

q	Enchanced PCI IDE on Board (Two Channels)

q	Supports 2S1P, Floppy on Board

q	BIOS has been Hardware Integrated with Enhanced IDE Driver for Best Hard Disk Performance

q	Supports Parallel Port EPP/ECP Mode

1.2.1	VGA Features

q	High performance acceleration

q	Screen refresh rates up to 75 Hz

q	Full PCI compatibility

q   Accelerated modes include 1024 x 768 x 64K colors and 1600 x 1200 x 256  colors (If 2M of memory is available)

q  Full software support, including enhanced drivers for WindowsTM 3.1,    AutoCADâ applications, MicroStationâ 4.0 and MicroStation 5.0

q Galileo Windows utility, which provides the capability of changing resolutions, color depth and refresh rates within Windows

1.3	System Specifications

Processor :	INTEL PENTIUM 75/90/100/120/133 MHz

CPU Clock :	50/60/66 MHz CPU

Memory :	Up to 128MB

Memory Configuration :	1MB/2MB/4MB/8MB/16MB/32MB/64MB/128MB

SRAM Configuration :	256KB/512KB

BIOS Subsystem :	AMI BIOS

I/O Subsystem No. Slot :	One EISA Slot

Dimension :	21cm x 33cm, Baby AT Size

Additional Features

Miscellaneous Connectors :	Reset Button, Internal Battery, Turbo SW

Board Design :	Four-layer Implementation for Low Noise Operation

1.4	System Performance

| SOFTWARE CPU TYPE | LANDMARK 2.0 | POWER METER V1.8 MIPS | NORTON V8.0 CPU SPEED |
| --- | --- | --- | --- |
| INTEL PENTIUM 75 | 432.90 MHz | 36.9 MIPS | 237.7 |
| INTEL PENTIUM 90 | 519.49 MHz | 44.4 MIPS | 285.2 |
| INTEL PENTIUM 100 | 574.59 MHz | 49 MIPS | 315.5 |
| INTEL PENTIUM 120 | 692.66 MHz | 58 MIPS | 380.3 |
| INTEL PENTIUM 133 | 766.13 MHz | 64.6 MIPS | 420.6 |

1.5   AIO8554 BOARD LAYOUT

1

1326

25

PRINTER

COM1

82C930

W83787

8042

82558

TDA1517

14

5                                  1

9                                  6

9                                6

5                               1

W83768

KB1

27010

27512

COM2

KB3

KEYLOCK

J21

J20

SPEAKER

J19

TB-LED

TB-SW

RESET

J18

J17

JP22

JP18

JP13

JP14

JP15

J6

JP11

J10

J11

J14

J15

J16

JP27

JP30

JP29

82556

32KX36

32KX36

1ST IDE

2ST IDE

J13

JP38

SM1

SM2

SM3

SM4

82557

TRI064

BQ3285

PS1

FDC

256X16                     256X16                     256X16                     256X16

VGA

TAG

32KX32

32KX32

32KX32

32KX32

14   1

26 13

1      2

33    34

2

1

2     1

26  25

1        2

1               5

1           4

1  1

3  3

1     3

1

4

U10          U9               U8          U7               U6          U5              U4          U3

U14                           U13                            U12                           U11

U22       U21            U20    U19                                                   U16         U15

U18                      U17

+

J4

BT2

1          4

1          4

1          4

40

39

40

39

2

1

26  25

JP31

1      3

JP24

PENTIUM CPU SOCKET

é

BANK0

ë

ù

BANK1

û

é

BANK1

ë

JP40

FEATURE CONNECTOR

ù

BANK0

û

J8

J9

J7

WAVE TABLE CONNECTOR

MIDI CONNECTOR

JP12

J3

J2

J1

U54

U58

U60

U63

U41

+   -

CHAPTER  2

INSTALLATION

Before the system is ready to operate, the hardware must be set up for various functions of the system.  To set up the AIO8554 motherboard is a simple task.  The  user only has to set a few jumpers, connectors and sockets.

2.1	DRAM INSTALLATION

The AIO8554 motherboard  can support expanded memory from 1MB to 128MB.  Either 1MB, 2MB, 4MB, 8MB,16MB, 32MB, SIM Modules can be  used on the AIO8554 motherboard.

n The board layout below shows the locations of the DRAM memory banks :

SM1

SM2

SM3

SM4

SM1      SM2      SM3      SM4

SM1 + SM4

SM2 + SM3

TOTAL

+                              =

Each group includes two SIMMs, each SIMM size can be 1,2,4,8,16,32MB,

please install the same DRAM size in one group.

2.2	SRAM INSTALLATION

JP18

1ST IDE

82557

2

1

40

39

J8

82558

2ST IDE

TAG

40

39

2

1

J9

U41

J10

J11

J13

JP22

JP27

26  25

JP24

82C930

TDA1517

JP31

JP30

32KX32

1      3

PENTIUM CPU SOCKET

WAVE TABLE CONNECTOR

U54

JP29

2     1

é

BANK1

ë

32KX36

32KX32

ù

BANK0

û

U58

32KX32

U60

32KX36

U63

32KX32

CACHE CONFIGURATION SIZE

| 256K ¶ |  | 512K |  |
| --- | --- | --- | --- |
| TAG RAM | DATA RAM | TAG RAM | DATA RAM |
| U41 8Kx8,16Kx8,32Kx8 | U58,U63 32Kx32 | U41 16Kx8, 32Kx8 | U54,U58,U60,U63 32Kx32 |
| JP22 JP31 l l l l 1 2 3 JP22 JP31 l l 1 2 3 |  |  |  |

¶ Default Setting

2.3	SYSTEM BUS FREQUENCY INSTALLATION

82C930

TDA1517

KEYLOCK

J21

J20

SPEAKER

J19

TB-LED

TB-SW

RESET

J18

J17

JP22

J14

J15

J16

JP27

JP30

JP29

82556

32KX36

32KX36

J13

SM1

SM2

SM3

SM4

32KX32

32KX32

32KX32

32KX32

2     1

26  25

1        2

1               5

1           4

1          4

1          4

1          4

26  25

JP31

1      3

JP24

PENTIUM CPU SOCKET

é

BANK0

ë

ù

BANK1

û

é

BANK1

ë

JP40

ù

BANK0

û

WAVE TABLE CONNECTOR

MIDI CONNECTOR

U63

U60

U58

U54

JP38

+   -

| 50MHz | 60MHz ¶ | 66MHz |
| --- | --- | --- |
| l l JP24 l l JP27 JP27 JP24 l l JP24 JP27 |  |  |

¶ Default Setting

CPU TYPE REFERENCE SETTING

| CPU TYPE | 75MHz | 90MHz | 100MHz | 120MHz | 133MHz |
| --- | --- | --- | --- | --- | --- |
| FREQUENCY | 50MHz | 60MHz | 66MHz | 60MHz | 66MHz |
| TIMES | 1.5 | 1.5 | 1.5 | 2 | 2 |

2.4	CPU FREQUENCY INSTALLATION

82C930

TDA1517

KEYLOCK

J21

J20

SPEAKER

J19

TB-LED

TB-SW

RESET

J18

J17

JP22

J14

J15

J16

JP27

JP30

JP29

82556

32KX36

32KX36

J13

SM1

SM2

SM3

SM4

32KX32

32KX32

32KX32

32KX32

2     1

26  25

1        2

1               5

1           4

1          4

1          4

1          4

26  25

JP31

1      3

JP24

PENTIUM CPU SOCKET

é

BANK0

ë

ù

BANK1

û

é

BANK1

ë

JP40

ù

BANK0

û

WAVE TABLE CONNECTOR

MIDI CONNECTOR

U63

U60

U58

U54

JP38

+   -

CPU FREQ. TIMES

| 1.5 ¶ | 3 | 2 | 2.5 |
| --- | --- | --- | --- |
| l l JP29 JP29 JP29 JP29 JP30 JP30 JP30 JP30 l l l l l l |  |  |  |

¶ Default Setting

2.5	OTHER JUMPER SETTING

25

14

9                                  6

9                                6

PRINTER

COM1

KB1

COM2

KB3

VGA

J4

J1

1

1326

5                                  1

5                               1

J3

J2

1      2

U10          U9               U8          U7               U6          U5              U4          U3

W83787

W83768

PS1

FDC

1

256X16                     256X16                     256X16                     256X16

4

U14                           U13                            U12                           U11

+

J6

33    34

U18                      U17

JP14

JP11

BT2

JP13

1  1

1     3

U22       U21            U20    U19                                                   U16         U15

J7

14   1

3  3

JP12

8042

JP15

TRI064

FEATURE CONNECTOR

27010

BQ3285

27512

26 13

JP18

1ST IDE

2

40

J8

82557

TAG

1

39

82558

2ST IDE

40

2

J9

39

1

U41

J10

J11

J13

JP22

26  25

JP24

JP27

82C930

TDA1517

JP31

1      3

JP30

PENTIUM CPU SOCKET

WAVE TABLE CONNECTOR

U63

U60

U58

U54

JP29

32KX32

2     1

é

BANK1

ë

32KX36

32KX32

ù

BANK0

û

32KX32

32KX36

JP38

32KX32

1        2

MIDI CONNECTOR

SM1

ù

BANK1

û

SM2

é

BANK0

ë

82556

SM3

J14

SM4

KEYLOCK

J21

J20

SPEAKER

J19

TB-LED

TB-SW

RESET

J18

J17

J15

J16

1               5

1           4

1          4

1          4

1          4

26  25

JP40

+   -

CONNECTOR DESCRIPTION

| CONNECTOR | PIN OUT | SIGNAL NAME |
| --- | --- | --- |
| JP13 | 1-2 2-3 | DMA 1 DMA 3 |
| JP14 | 1-2 2-3 | DMA 1 DMA 3 |
| JP15 | Open For EPP/ECP Mode Close For Standar mode |  |
| 1 3 JP11 |  | 1-2 Discharge CMOS (When use Battary) ¶ 2-3 Normal operation |
| JP18 | GREEN SWITCH Touch one time GREEN next time WAKE UP. |  |
| J21 : KEY LOCK | 1 2 3 4 5 | Power LED Not Used Ground Keyboard Inhibitor Ground |
| J20 : SPEAKER | 1 2 3 4 | +5V DC Data Out Data Out Data Out |
| J17 : RESET | 1 2 | Ground Reset In |
| J18 : TB-LED | 1 2 | +Anode -Cathode |
| l l Turbo Speed Normal Speed * J19 : TURBO SWITCH |  |  |
| KB1: KEYBOARD CONNECTOR | 1 2 3 4 5 | Keyboard Clock Keyboard-Data Space Ground +5V |
| P1 & P2 : POWER CONNECTOR | 1 2 3 4 5,6,7,8 9 10,11,12 | Power Good +5V DC +12V DC -12V DC Ground -5V DC +5V DC |

CONNECTOR DESCRIPTION

| CONNECTOR | PIN OUT | SIGNAL NAME |
| --- | --- | --- |
| JP12 | CLOSE for CLEAR CMOS When Use 12887A |  |
| JP40 | Normal Open , Close For Feature Function |  |
| - + J10 | Secondary IDE LED |  |
| - + J11 | Primary IDE LED |  |
| J1 | COM1 Connector |  |
| J2 | COM2 Connector |  |
| J3 | LPT1 Connector |  |
| J4 | VGA Connector |  |
| J6 : EXTERNAL BATTERY CONNECTOR | 1 2 3 4 | +3.6V NC Ground Ground |
| J14 : PANA CD AUDIO IN | 1 2 3 4 | Ground Right Ground Left |
| J15 : SONY CD AUDIO IN | 1 2 3 4 | Left Ground Ground Right |
| J16: MITUSMI CD AUDIO IN | 1 2 3 4 | Right Ground Left Ground |

CHAPTER 3 SYSTEM BIOS SETUP

WinBIOS Setup can be accessed via keyboard, mouse, or pen. The mouse click functions are:

- single click to change or select both global and current fields and

- double click to perform an operation in the selected field.

Using the keyboard with WinBIOS Setup

WinBIOS Setup has a built-in keyboard driver that uses simple keystroke combinations:

| Keystroke | Function |
| --- | --- |
| <Tab> | Move to the next window or field. |
| à, ß, á, â | Move to the next field to the right, left, above, or below. |
| <Enter> | Select in the current field. |
| + | Increments a value. |
| - | Decrements a value. |
| <Esc> | Closes the current operation and return to previous level. |
| <PgUp> | Returns to the previous page. |
| <PgDn> | Advances to the next page. |
| <Home> | Returns to the beginning of the text. |
| <End> | Advances to the end of the text. |
| <Alt>, <H> | Access a help window. |
| <Alt><Spacebar> | Exit WinBIOS Setup. |
| Alphabetic Keys | A to Z are used in the Virtual Keyboard, and are not case sensitive. |
| Numeric Keys | 0 to 9 are used in the Virtual Keyboard and Numeric Keypad. |

The hardware features and options of the AIO8554 are on-site selectable for maximum flexibility. You will need to configure these options through the built-in Setup Utility prior to using AIO8554 for the first time. This setup Utility is a multi-screen,menu driven program and is contained within the BIOS EPROM.

The following sections show the procedures that you may need to configure the AIO8554 :

1.	Press <DEL> while turning on or rebooting the system to invoke Setup Utility program.

2.	The Main Menu will be shown as follows:

Figure 1

3.	The functions are grouped into four categories which are Setup, Utility, Security and Default.

4.	By using <TAB> key or mouse cursor to select the function group.

5.	Use arrow keys or mouse to select the function icon within the group. Then press <Enter> key to invoke the setup fucntion.

6.	Use <ESC> key to go back to the previous screen.

3.1	SYSTEM SETUP

There are five icons in the Setup Group.

Selecting Standard icon displays the following menu:

Figure 2

Selecting Date/Time icon displays the following  menu:

Figure 3

After entering correct date and time, press <ESC> to go back to the previous menu.

Using arrow keys or mouse to select the correct specification of floppy drive. Press <ESC> key to go back to the previous menu.

Figure 4

Selecting Floppy A/B icon displays the following menu:

Figure 5

Selecting Master/Slave Disk icon displays the following menu:

Figure 6

Use arrow keys or mouse to select or enter the Hard Disk specifications. Press <ESC> to go back to the previous menu.

Selecting Slave Disk icon displays the following menu:

Figure 7

Selecting Advanced icon displays the following menu:

Use arrow keys to select the desired entries and make changes. Press <Esc> key to go back to the previous menu.

Figure 8

Figure 9

Figure 10

Figure 11