EXP8051

ALI 1449+1451

USER’S MANUAL

TABLE OF CONTENTS

CHAPTER 1 INTRODUCTION	1

1.1   Overview	1

1.2   System Features	1

1.3   System Specification	2

1.4   System Performance	2

1.5   EXP8051 Board Layout	3

CHAPTER 2 INSTALLATION	4

2.1   DRAM Installation	4

2.2   SRAM Installation	5

2.3   CPU Installation	6

2.4   Other Jumper and Connector Installation	7

CHAPTER 3 SYSTEM BIOS SETUP	10

3.1   System Setup	11

3.2   Fixed Disk Setup	12

3.3   Advanced System Setup	13

3.3.1   Memory Cache	15

3.3.2   Memory Shadow	16

3.3.3   Advanced Chipset Control	18

3.3.4   PCI Devices	20

3.4   Boot Option	23

3.5   Security and Anti - Virus	24

3.6   Green PC Features	26

3.7   Load ROM Default Values	28

3.8   Load Values from CmoS	29

3.9   Save Values to CmoS	30

RMA FORM

CHAPTER  1

INTRODUCTION

1.1 Overview

The EXP8051 is complemented by a 1MB second level Write-Back cache providing workstation level computing performance , and SIMM sockets support up to 128MB of DRAM.

The EXP8051 motherboard offers outstanding I/O capabilities .Four PCI Local Bus slots provide a high bandwidth data path for data-movement intensive function such as Graphics . Four ISA slots complete the I/O mix .

The EXP8051 Baby-AT motherboard provides the foundation for cost effective , high performance , highly expandable platforms.

1.2 System Features

q Supports INTEL PENTIUM

q Supports 4 MASTER 32-bit PCI Bus

q Supports L1/L2 Write Back/ Write Through Cache Feature

q Supports 256KB/512KB/1MB Cache Size

q Supports 72pin SIM MODULES

1.3 System Specifications

Processor :	INTEL PENTIUM

CPU Clock : 	60/66 MHz CPU

Memory :	2MB to 128MB

SRAM Configuration : 	256KB/512KB/1MB

BIOS Subsystem :	PHOENIX/AWARD BIOS

Additional BIOS Feature :	Set Program Resides in ROM

I/O Subsystem No. Slot :	Four 16-bit ISA Bus & Four PCI 32-bit Bus

Additional Features

Miscellaneous Connectors :	Reset Button, Internal Battery

Board Design :	4-layer Implementation for Low Noise Operation

1.4 System Performance

| SOFTWARE CPU TYPE | LANDMARK V2.0 | POWER METER V1.8 MIPS | NORTON V8.0 CPU SPEED |
| --- | --- | --- | --- |
| 60MHz | 346.43MHz | 28.4MIPS | 190.1 |
| 66MHz | 383.20MHz | 32 MIPS | 210.3 |

1.5 EXP8051 Board Layout

1

4

JP5

JP4

1

3

JP3

+           -

P1

· · · · · · · · · ·

P2

1

SIMM1

SIMM2

SIMM3

SIMM4

PC 3

PC 2

PC4

PC 1

BIOS

JP1

JP2

JP6

U20

U19

U18

U17

1    2

3    4

JP11

JP10

JP  9

JP  8

JP  7

3       1

U21

U22

U24

U23

U25

PENTIUM

1   2

JP12

9 10

1

4

1

5

JP14

JP16

JP18

JP13

JP15

JP17

CHAPTER  2

INSTALLATION

Before the system is ready to operate, the hardware must be set up for various functions of  the system.  To set up the EXP8051 main board is a simple task.  The  user only has to set a few jumpers, connectors and sockets.

2.1 DRAM Installation

n The board layout below shows the locations of the DRAM memory banks :

SIMM1

SIMM3

SIMM4

SIMM2

2        SIMM1+SIMM2  +  SIMM3+SIMM4  =  TOTAL

Each group includes two SIMMs, each SIMM size can be 1,2,4,8,16,32MB,

please install the same DRAM size in one group.

JP13

1

5

JP18

JP17

JP16

JP15

JP14

9 10

JP12

1   2

JP11

JP10

JP  9

JP  8

JP  7

3       1

JP6

1    2

3    4

U23

U25

U24

U22

U21

U20

U19

U18

U17

1

4

PENTIUM

2.2 SRAM INSTALLATION

CACHE SIZE CONFIGURATION

| * 256K | 512K | 1M |
| --- | --- | --- |
| DATA RAM TAG RAM TAG RAM DATA RAM TAG RAM DATA RAM |  |  |
| U25 32KX8 U17~U24 128KX8 U17~U24 64KX8 U25 32KX8 U17~U24 32KX8 U25 8KX8 |  |  |
| JP15 ·· 1 2 5 6 5 6 JP9 JP10 3 2 1 JP8 3 2 1 JP7 3 2 1 3 2 1 JP9 JP10 3 2 1 JP8 3 2 1 JP7 3 2 1 3 2 1 JP9 JP10 3 2 1 JP8 3 2 1 JP7 3 2 1 3 2 1 JP15 ·· 1 2 ·· ·· ·· ·· ·· ·· ·· ·· ·· ·· JP15 ·· 1 2 5 6 |  |  |

* : Default Setting

2.3 CPU INSTALLATION

JP15

JP17

JP13

1

5

JP18

JP16

JP14

9 10

JP12

1   2

JP11

JP10

JP  9

JP  8

JP  7

3       1

JP6

1    2

3    4

U23

U25

U24

U22

U21

U20

U19

U18

U17

1

4

PENTIUM

CPU FREQUENCY SETTING

| * PENTIUM 50.0 MHz | PENTIUM 60.0 MHz | PENTIUM 66.6 MHz |
| --- | --- | --- |
| JP15 ·· 1 2 5 6 5 6 JP15 1 2 ·· JP11 1 2 3 4 JP11 1 2 3 4 JP11 1 2 3 4 ·· ·· JP15 ·· 1 2 5 6 |  |  |

JP15

··

1        2

5        6

··

* : Default Setting

2.4 OTHER JUMPER & CONNECTOR SETTING

+           -

JP6

PC 1

PC4

PC 2

PC 3

BIOS

JP1

JP2

JP3

1

3

JP4

JP5

1

4

· · · · · · · · · ·

P2

P1

SIMM1

SIMM2

SIMM3

SIMM4

JP13

1

5

JP18

JP16

JP14

9 10

JP12

1   2

1

4

PENTIUM

JP17

JP15

OTHER JUMPER DESCRIPTION

| JUMPER | DESCRIPTION |  |
| --- | --- | --- |
| NORMAL JP 1 |  |  |
| ·· BUILT IN BATTERY USE EXTERNAL BATTERY * JP 2 |  |  |
| 1 2 3 ·· 1 2 3 ·· LM 393 POWER SUPPLY * JP 3 |  |  |
| 1 2 3 ·· ·· FLASH ROM NORMAL ROM * 1 2 3 JP 4 |  |  |
| NORMAL* DELAY ADS# ·· JP 6 |  |  |
| JP13 | EXTERNAL SUSPEND SWITCH |  |

* : Default Setting

CONNECTOR DESCRIPTION

| CONNECTOR | PIN OUT |  | SIGNAL NAME |
| --- | --- | --- | --- |
| JP18 : KEY LOCK | 1 2 3 4 5 |  | LED POWER NOT USED GROUND KEYBOARD INHIBITOR GROUND |
| JP17 : SPEAKER | 1 2 3 4 |  | + 5V DATA OUT DATA OUT DATA OUT |
| JP16: RESET | 1 2 |  | GROUND RESET IN |
| P1: KEYBOARD CONNECTOR | 1 2 3 4 5 |  | KEYBOARD CLOCK KEYBOARD DATA SPACE GROUND + 5V DC |
| P2 : POWER CONNECTOR | 1 2 3 4 5,6,7,8 9 10,11,12 |  | POWER GOOD + 5V DC +12V DC - 12V DC GROUND - 5V DC + 5V DC |
| JP12: PERIPHERAL POWER CONTROL CONNECTOR | 1,2 3,4 5,6 7,8 9,10 | FOR V-SYNC FOR H-SYNC FOR MONITOR POWER FOR POWER FAN RESERVED |  |
| JP 5 | EXTERNAL BATTERY 1-2 OR 1- 4 ( PIN 1 is GROUND ) |  |  |

CHAPTER  3

SYSTEM BIOS SETUP

PHOENIX BIOS SYSTEM CONFIGURATION SETUP

Use the EXP8051 to record changes in your hardware and to control it's special features.  The setup program uses a number of menus in which you can specify changes to your hardware and turn the special features on or off.

1.  To start the EXP8051  setup program, take the following steps:

Press <DEL> to enter setup while turning on or rebooting your system.

.

The Phoenix BIOS for EXP8051 displays this message:

2.  The Main Menu which looks like this appears:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

3.	Select an option by moving the highlight with your up-and-down arrow keys and press. <Enter>.

4.	After making your changes, select Save Values to CMOS to make them operative. Press <Esc> to exit the setup program. The following pages describe each one of these selections.

3.1 System Setup

Selecting "System Setup" on the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

SYSTEM SETUP

System Time:	[14:29:46]
System Date:	[10/19/1994]
 
Video System:	[EGA / VGA]

System Memory:	[640 KB]
Extended Memory:	[7MB]

Diskette Drive A:	[1.44 MB, 3.5"]                  Diskette Drive B:	[Not Installed]

Keyboard:	[Not Installed]

¯  Move	ESC  Exit	PgUp  Previous Value	F5  Previous Configuration
	F1  Help	PgDn  Next Value	F6  Default Configuration

3.2  Fixed Disk Setup

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting "Fixed Disk Setup" on the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

Fixed Disk Setup

IDE Adapter 0 Master (C: 541MB)

IDE Adapter 0 Slave (None)

Large Disk Access Mode: [DOS]

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting either option and pressing <Enter> displays a menu like this:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

IDE Adapter 0 Master (C: 541Mb)

Autotype Fixed Disk: [Press Enter]

Type: [User] 541 MB

Cylinders: [ 1120]

Heads: [ 16]

Sectors / Track: [ 59]

Write Precomp: [None]

Multi-Sector Transfers: [Disabled]

LBA Mode Control: [Disabled]

32 Bit I / O: [Disabled]

Transfer Mode: [Standard]

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

3.3 Advanced System Setup

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting “Advanced System Setup” from the Main Menu displays the following menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED SYSTEM SETUP

Warning!

Items on this menu, if set incorrectly, could cause your system to malfunction.

· Memory Cache

· Memory Shadow

· Advanced Chipset Control

· PCI Devices

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Choose an option and press <Enter>. See the following pages for a description of each feature and its options.

3.3.1 Memory Cache

Selecting "Memory Cache" from the Advanced Setup menu displays a menu like the one shown here. The actual features displayed depend on the capabilities of your system's hardware.

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MEMORY CACHE

External Cache: [Enabled]

External Cache Method: [Write-Back]

External Cache Write Timing: [Normal]

External Cache Read Timing: [Normal]

Cache Hit Sampling Time: [Normal]

PCI features

PCI Master Cycles: [Normal]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

3.3.2 Memory Shadow

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED SYSTEM SETUP

Warning!

Items on this menu, if set incorrectly, could cause your system to malfunction.

· Memory Cache

· Memory Shadow

· Advanced Chipset Control

· PCI Devices

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting "Memory Shadow" from the Advanced Setup menu displays a menu like the one shown here. The actual features displayed depend on the capabilities of your system's hardware.

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MEMORY SHADOW

System Shadow: [Enabled]

Video Shadow: [Enabled]

Shadow Memory Regions:

C800 - CBFF: [Disabled]

CC00 - CFFF: [Disabled]

D000 - D3FF: [Disabled]

D400 - D7FF: [Disabled]

D800 - DBFF: [Disabled]

DC00 - DFFF: [Disabled]

E000 - E3FF: [Disabled]

E400 - E7FF: [Disabled]

E800 - EBFF: [Disabled]

EC00 - EFFF: [Disabled]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

3.3.3 Advanced Chipset Control

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED SYSTEM SETUP

Warning!

Items on this menu, if set incorrectly, could cause your system to malfunction.

· Memory Cache

· Memory Shadow

· Advanced Chipset Control

· PCI Devices

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting "Advanced Chipset Control " from the Advanced Setup menu displays a menu like the one shown here.

Technicians use this menu to change the values in the chipset registers and optimize your system's performance.

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED CHIPSET CONTROL

DRAM Timing: [Normal]

DRAM Refresh Period: [2048 us]

DRAM Hidden Refresh: [Disabled]

CPU Piped Address: [Disabled]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED CHIPSET CONTROL

PCI FEATURES

CPU/PCI Write Buffer: [Enabled]

CPU/PCI Write-Byte-Merge: [Enabled]

CPU/PCI Write-Fast: [Disabled]

CPU/PCI Write-Burst: [Disabled]

CPU/PCI Read Buffer: [Disabled]

PCI Read-Line Command: [Disabled]

CPU/PCI Burst Read: [Disabled]

PCI/CPU Write Buffer: [Enabled]

PCI/CPU Read Buffer: [Disabled]

PCI Cycle Decoding: [Normal]

PCI/CPU Wait State: [1 ws]

Burst on Non-Bufferable Cycle: [Disabled]

Flash PCI/CPU Buffer: [Enabled]

BRIDGE FEATURES

DMA Line Buffer: [Disabled]

ISA Line Buffer: [Disabled]

PCI Posted Write Buffer: [Disabled]

Arbiter Bus Lock: [Disabled]

Arbiter Bus Park: [Enabled]

DMA Write Cycle Timing: [Normal]

ISA Clock Select: [CLK2/8]

I/O Recovery: [Disabled]

I/O Recovery Period: [1 us]

ISA I/O Cmd Wait Count: [Normal]

ISA Memory Cmd Wait Count: [Normal]

Refresh Period: [15 us]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

NOTE: The contents of this menu depends on the chipset installed on your motherboard, and chipsets vary widely. Consult your dealer or the <F1> help screens before changing the items on this menu. Incorrect settings can cause your system to malfunction.

3.3.4 PCI Devices

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED SYSTEM SETUP

Warning!

Items on this menu, if set incorrectly, could cause your system to malfunction.

· Memory Cache

· Memory Shadow

· Advanced Chipset Control

· PCI Devices

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

PCI DEVICES

Base I/O Address: [3000]

Base Memory Address: [008000000]

Multimedia Mode: [Disabled]

Parity: [Disabled]

PCI Interrupt INTB#: [None]

PCI Interrupt INTC#: [None]

PCI Interrupt INTD#: [None]

PCI INT-1 Level/Edge Selection: [Edge]

PCI INT-2 Level/Edge Selection: [Edge]

PCI INT-3 Level/Edge Selection: [Edge]

PCI INT-4 Level/Edge Selection: [Edge]

PCI INT-5 Level/Edge Selection: [Edge]

PCI INT-6 (INTB#) Level/Edge Selection: [Edge]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

.

PCI DEVICES

PCI INT-7 (INTC#) Level/Edge Selection: [Edge]

PCI INT-8 (INTD#) Level/Edge Selection: [Edge]

PCI Device, Slot #1:

Enable Device: [Disabled]

Enable Master: [Disabled]

Use Default Latency Timer Value: [Yes]

Latency Timer Value: [0040]

IRQ Line: [None]

PCI Device, Slot #2:

Enable Device: [Disabled]

Enable Master: [Disabled]

Use Default Latency Timer Value: [Yes]

Latency Timer Value: [0040]

IRQ Line: [None]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

PCI DEVICES

PCI Device, Slot #3:

Enable Device: [Disabled]

Enable Master: [Disabled]

Use Default Latency Timer Value: [Yes]

Latency Timer Value: [0040]

IRQ Line: [None]

PCI Device, Slot #4:

Enable Device: [Disabled]

Enable Master: [Disabled]

Use Default Latency Timer Value: [Yes]

Latency Timer Value: [0040]

IRQ Line: [None]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

NOTE: The number of  PCI device  slots depends on  motherboard  specifications. 		 When  you  install  PCI IDE card on any  PCI slot, you  must set that slot¢s

IRQ line [14] and [Enabled Device] [Enable Master] items  must set Enable.

3.4 Boot Options

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting "Boot Options" from the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

BOOT OPTIONS

Boot Sequence: [A:then C:]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

3.5 Security and Anti-Virus

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting "Security and Anti-Virus" from the Main Menu displays a menu like this:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

SECURITY AND ANTI-VIRUS

Supervisor Password is: Disabled

User Password is: Disabled

Set Supervisor Password: [Press Enter]

Set User Password: [Press Enter]

Password on Boot: [Disabled]

Diskette Access: [Supervisor]

Fixed Disk Boot Sector: [Normal]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

"Supervisor Password is" shows whether supervisor password is set or not. "User Password is" shows whether user password is set or not. Setting Supervisor Password requires a password on entering Setup. The passwords are not case sensitive. Pressing <Enter> at either Set Supervisor Password or Set User Password displays a dialog box like this:

SET SUPERVISOR PASSWORD

Enter New Password: [ ]

Re-enter New Password: [ ]

Enter Accept

SET USER PASSWORD

Enter New Password: [ ]

Re-enter New Password: [ ]

Enter Accept

|  | NOTICE |  |
| --- | --- | --- |
|  | Changes have been saved. |  |
|  | [Continue] |  |

3.6 Green PC Features

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting " Green PC Features " on the main menu displays an information window like this:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

GREEN PC FEATURES

Power Saving Mode: [Enabled]

System Doze Timer: [10 sec]

System Standby Timer: [10 sec]

System Suspend Timer: [10 sec]

Wakeup System During Suspend: [Disabled]

Wakeup Time at (hour): [0 hr]

Wakeup Time at (minute): [0 min]

Hard Disk Standby Timer: [Disabled]

Video Standby Timer: [Disabled]

VGA With Power Down Feature: [None]

Advanced Power Management Setup

System Enters Power Down/Wakeup Modes By:

VGA Access Event: [Off]

IRQ 1 (Keyboard): [On]

IRQ 3 (COM2): [On]

IRQ 4 (COM1): [On]

IRQ 5 (Alt Printer): [On]

IRQ 6 (Diskette): [On]

IRQ 7 (Printer): [On]

IRQ 9 (IRQ2 Redir): [On]

IRQ10 (Reserved): [On]

IRQ11 (Reserved): [On]

IRQ12 (PS/2 Mouse): [On]

IRQ14 (Fixed Disk): [On]

IRQ15 (Reserved): [On]

DMA1 (LAN Card) : [On]

DMA3 (LAN Card) : [On]

LPT IO Address: [On]

PCI Master Event: [Off]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

3.7  Load ROM  Default Values

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting " Load ROM Default Values " on the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

NOTICE

Default values have been loaded.

[Continue]

3.8 Load Values from CMOS

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting " Load Values from CMOS " on the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

NOTICE

Previous values have been loaded.

[Continue]

3.9  Save Values to CMOS

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting " Save Values to CMOS " on the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

NOTICE

Changes have been saved.

[Continue]

RMA FORM

When the motherboard can not work well, please fill this form to describe related situations. If the space is not enough to use, you can attach separate paper.

MODEL :                                                               MODEL NO :

HARDWARE

CPU: Brand ______________, Model _____________, Speed ______________ MHz

CO-PROCESSOR: Brand ____________, Model ____________, Speed __________ MHz

SIMM: Brand _________, Speed _________ ns, Q'ty ________ pcs, Total _________ MB

CACHE: Brand ____________, Speed ___________ns, Total ____________K

TAG RAM: Brand ____________, Speed ___________ns

BIOS DATE CODE: ___________

SYSTEM SPEED RUNNING ___________ MHz

VIDEO  CARD: Chip ___________, RAM ___________, Masker ___________

VGA Mode                         , Bus                         (ISA ,VESA or PCI)

OTHER ADD-ON CARDS :

SOFTWARE

OPERATION SYSTEM _________________ VERSION _______________

SOFTWARE PROGRAM ________________________________________

BIOS SETUP: DRAM wait _______________ CACHE wait _______________

If you change BIOS SETUP, please describe the changes:

<A> ERROR

¨  HANG UP         ¨  NO  SCREEN       ¨  FLOPPY R/W ERROR

¨  HARD DISK R/W ERROR      ¨  PARITY MEMORY ERROR

¨  OTHER ______________________________________________

<B> ERROR MESSAGES ON YOUR SCREEN (PLEASE SHOW US THE WHOLE SENTENCE)

<C> PROBLEM DESCRIPTION