EXP8059P

(PHOENIX BIOS)

P54C 90/100MHZ

ALI M1449/M1461

PCI HOST

USER’S MANUAL

TABLE OF CONTENTS

CHAPTER 1 INTRODUCTION	1

1.1   Overview	1

1.2   System Features	1

1.3   System Specification	2

1.4   System Performance	2

1.5   EXP8059 Board Layout	3

CHAPTER 2 INSTALLATION	4

2.1   DRAM Installation	4

2.2   SRAM Installation	5

2.3   CPU Installation	6

2.4   Other Jumper and Connector Installation	7

CHAPTER 3 SYSTEM BIOS SETUP	10

3.1   System Setup	11

3.2   Fixed Disk Setup	12

3.3   Advanced System Setup	13

3.3.1   Memory Cache	15

3.3.2   Memory Shadow	16

3.3.3   Advanced Chipset Control	18

3.3.4   PCI Devices	20

3.4   Boot Option	23

3.5   Security and Anti-Virus	24

3.6   Green PC Features	26

3.7   Load ROM Default Values	28

3.8   Load Values from CMOS	29

3.9  save  Values to CMOS	30

RMA FORM

CHAPTER  1

INTRODUCTION

1.1   Overview

The EXP8059 is complemented by a 1M second-level Write-Back cache providing workstation level computing performance, and SIMM sockets support up to 128MB of DRAM.

The EXP8059 motherboard offers outstanding I/O capabilities.  Four PCI Local Bus slots provide a high bandwidth data path for data-movement intensive function such as Graphics.  Three ISA slots complete the I/O mix.

The EXP8059 Baby-AT motherboard provides the foundation for cost effective, high performance, highly expandable platforms which deliver the latest EXP8059 in CPU and I/O technologies.

1.2   System Features

q Supports INTEL P54C

q Supports 4  MASTER 32-bit PCI Bus

q Supports L1/L2 Write Back/Write Through Cache Feature

q Supports 256KB/ 512KB /1M Cache Size

q Supports 72pin SIM MODULES

1.3   System Specifications

Processor :	INTEL P54C

CPU Clock : 	90/100 MHz CPU

Memory :	2MB to 128MB

SRAM Configuration : 	256K/512K/1M

BIOS Subsystem :	PHOENIX/AWARD BIOS

Additional BIOS Feature :	Set Program Resides in ROM

I/O Subsystem No. Slot :	Three 16-bit ISA Bus

Four PCI 32-bit Bus

Dimension :	Baby AT Size

Additional Features

Miscellaneous Connectors :	Reset Button, Internal Battery

Board Design :	4-layer Implementation for Low Noise Operation

1.4   System Performance

| SOFTWARE CPU TYPE | LANDMARK V2.0 | POWER METER V1.8 MIPS | NORTON V8.0 CPU SPEED |
| --- | --- | --- | --- |
| 90MHz | 519.36MHz | 40.60MIPS | 285.20 |
| 100MHz | 574.47MHz | 45.10MIPS | 315.40 |

1.5   EXP8059 Board Layout

KB1

146818

8042

JP26

1

1

JP3

JP1

JP2

1

27010

M1449

M1461

U26      U25     U24     U23

2

9

10

JP24

1

JP11

JP12

JP13

KEYLOCK

SPK

RESET

JP15

JP14

PENTIUM

90/100 (P54C)

JP4

JP5

JP6

3

JP7

JP9

JP8

1

2

3

3

4

1

J10

EXT SW

TUB SW

TUB LED

JP23

4

3

1

JP18

1

3

3

BT1

KB1

P1

P2

P3

JP16

1

3

PS1

J9

J10

1

1

1

1

J8

W83758P

1

J7

W83757

1

1

J5

JP23

PCI 4

PCI 3

PCI 2

PCI 1

SIMM1

SIMM2

SIMM3

SIMM4

SIMM6

SIMM5

U22     U21     U20      U19       U18

JP17

JP17

CHAPTER  2

INSTALLATION

Before the system is ready to operate, the hardware must be set up for various functions of  the system.  To set up the EXP8059 motherboard is a simple task.  The  user only has to set a few jumpers, connectors and sockets.

2.1   DRAM INSTALLATION

The EXP8059 motherboard can support expanded memory from 2MB to 128MB.

n The board layout below shows the locations of the DRAM memory banks :

SIMM1

SIMM2

SIMM3

SIMM4

SIMM6

SIMM5

DRAM INSTALLATION

SIMM1+SIMM2  +  SIMM3+SIMM4  +  SIMM5+SIMM6   =   TOTAL

Each group includes two SIMMs each SIMM size can be 1, 2, 4, 8, 16, 32MB, please install the same DRAM size in one group.

2.2   SRAM INSTALLATION

U26      U25     U24     U23

1

J11

JP12

JP13

KEYLOCK

SPK

RESET

JP15

JP14

PENTIUM

90/100 (P54C)

JP4

JP5

JP6

3

JP7

JP9

JP8

1

2

3

3

4

1

J10

EXT SW

TUB SW

TUB LED

JP23

JP17

1

JP18

1

3

3

U22     U21     U20      U19       U18

CACHE SIZE CONFIGURATION

| 256K ¨ |  |  | 512K |  | 1M |  |
| --- | --- | --- | --- | --- | --- | --- |
| TAG RAM | DATA RAM | TAG RAM |  | DATA RAM | TAG RAM | DATA RAM |
| U22 8KX8 | U18-U21 U23-U26 32KX8 | U22 32KX8 |  | U18-U21 U23-U26 64KX8 | U22 32KX8 | U18-U21 U23-U26 128KX8 |
| JP18 JP17 3 2 1 3 2 1 l l l l JP18 JP17 3 2 1 3 2 1 l l l l JP18 JP17 3 2 1 3 2 1 l l l l |  |  |  |  |  |  |

¨ : Default Setting

1

JP12

JP13

KEYLOCK

SPK

RESET

JP15

JP14

PENTIUM

90/100 (P54C)

JP4

JP5

JP6

3

JP7

JP9

JP8

1

2

3

3

4

1

J10

EXT SW

TUB SW

TUB LED

JP23

2.3   CPU INSTALLATION

JP11

CPU FREQUENCY SETTING

| P54C 75 MHz | P54C 90 MHz¨ | P54C 100 MHz |
| --- | --- | --- |
| JP9 3 4 1 2 l l l l JP9 3 4 1 2 l l JP9 3 4 1 2 |  |  |

¨ : Default Setting

2.4   OTHER JUMPER & CONNECTOR INSTALLATION

146818

8042

JP26

1

1

JP3

JP1

JP2

1

27010

M1449

M1461

2

9

10

JP24

1

JP11

JP12

JP13

KEYLOCK

SPK

RESET

JP15

JP14

PENTIUM

90/100 (P54C)

JP4

JP5

JP6

3

JP7

JP9

JP8

1

2

3

3

4

1

J10

EXT SW

TUB SW

TUB LED

JP23

4

3

BT1

KB1

P1

P2

P3

JP16

1

3

PCI 4      PCI 3      PCI 2      PCI 1

OTHER JUMPER DESCRIPTION

| JUMPER | DESCRIPTION |  |
| --- | --- | --- |
| NORMAL JP1 |  |  |
| l l BUILD IN BATTERY RTC NORMAL RTC ¨ JP2 |  |  |
| l l l l 3 2 1 POWER GOOD COMES FROM LM393 3 2 1 POWER SUPPLY ¨ l l JP3 |  |  |
| l l DELAY ADS# NORMAL ¨ l l 3 1 3 1 JP4 |  |  |
| NORMAL ROM ¨ 3 2 1 FLASH ROM l l l l l l 3 2 1 JP8 |  |  |

¨ : Default Setting

CONNECTOR DESCRIPTION

| CONNECTOR |  | PIN OUT | SIGNAL NAME |  |
| --- | --- | --- | --- | --- |
| JP15 : KEY LOCK |  | 1 2 3 4 5 | LED POWER NOT USED GROUND KEYBOARD INHIBITOR GROUND |  |
| JP14 : SPEAKER |  | 1 2 3 4 | DATA OUT NOT USED GROUND +5V DC |  |
| JP11 : RESET |  | 1 2 | GROUND RESET IN |  |
| P1: KEYBOARD CONNECTOR |  | 1 2 3 4 5 | KEYBOARD CLOCK KEYBOARD DATA SPACE GROUND +5V |  |
| P2 : POWER CONNECTOR |  | 1 2 3 4 5,6,7,8 9 10,11,12 | POWER GOOD +5V DC +12V DC -12V DC GROUND -5V DC +5V DC |  |
| J24 | 1 2 3 4 2-3 : N.C. , 1-4 : EXTERNAL BATTERY |  |  |  |
| JP7 | CLOCK GENERATOR PWR DOWN GRADE |  |  |  |
| JP16 | 2-3 NORMAL |  |  |  |
| JP26 | EXTERNAL POWER SAVING CONNECTOR 1-2 FOR VGA V-SYNC 3-4 FOR VGA H-SYNC 5-6 FOR MONITOR POWER 7-8 FOR POWER FAN 9-10 RESERVED |  |  |  |

CHAPTER  3

SYSTEM BIOS SETUP

PHOENIX  BIOS  SYSTEM  CONFIGURATION SETUP

Use the EXP8059 to record changes in your hardware and to control it's special features.  The setup program uses a number of menus in which you can specify changes to your hardware and turn the special features on or off.

1.  To start the EXP8059  setup program, take the following steps:

Press <DEL> to enter setup while turning on or rebooting your system.

The Phoenix BIOS for EXP8059 displays this message:

2.  The Main Menu which looks like this appears:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

3.	Select an option by moving the highlight with your up-and-down arrow keys and press. <Enter>.

4.	After making your changes, select Save Values to CMOS to make them operative. Press <Esc> to exit the setup program. The following pages describe each one of these selections.

3.1   System Setup

Selecting "System Setup" on the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

SYSTEM SETUP

System Time:	[14:29:46]
System Date:	[10/19/1994]
 
Video System:	[EGA / VGA]

System Memory:	640 KB
Extended Memory:	    7 MB

Diskette Drive A:	[1.44 MB, 3.5"]                  Diskette Drive B:	[Not Installed]

Keyboard:	[Not Installed]

¯  Move	ESC  Exit	PgUp  Previous Value	F5  Previous Configuration
	F1  Help	PgDn  Next Value	F6  Default Configuration

3.2   Fixed Disk Setup

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting "Fixed Disk Setup" on the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

Fixed Disk Setup

IDE Adapter 0 Master (C: 541MB)

IDE Adapter 0 Slave (None)

Large Disk Access Mode: [DOS]

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting either option and pressing <Enter> displays a menu like this:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

IDE Adapter 0 Master (C: 541Mb)

Autotype Fixed Disk: [Press Enter]

Type: [User] 541 MB

Cylinders: [ 1120]

Heads: [ 16]

Sectors / Track: [ 59]

Write Precomp: [None]

Multi-Sector Transfers: [Disabled]

LBA Mode Control: [Disabled]

32 Bit I / O: [Disabled]

Transfer Mode: [Standard]

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

3.3   Advanced System Setup

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting “Advanced System Setup” from the Main Menu displays the following menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED SYSTEM SETUP

Warning!

Items on this menu, if set incorrectly, could cause your system to malfunction.

· Memory Cache

· Memory Shadow

· Advanced Chipset Control

· PCI Devices

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Choose an option and press <Enter>. See the following pages for a description of each feature and its options.

3.3.1   Memory Cache

Selecting "Memory Cache" from the Advanced Setup menu displays a menu like the one shown here. The actual features displayed depend on the capabilities of your system's hardware.

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MEMORY CACHE

External Cache: [Enabled]

External Cache Method: [Write-Back]

External Cache Write Timing: [Normal]

External Cache Read Timing: [Normal]

Cache Hit Sampling Time: [Normal]

PCI features

PCI Master Cycles: [Normal]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

3.3.2   Memory Shadow

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED SYSTEM SETUP

Warning!

Items on this menu, if set incorrectly, could cause your system to malfunction.

· Memory Cache

· Memory Shadow

· Advanced Chipset Control

· PCI Devices

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting "Memory Shadow" from the Advanced Setup menu displays a menu like the one shown here. The actual features displayed depend on the capabilities of your system's hardware.

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MEMORY SHADOW

System Shadow: [Enabled]

Video Shadow: [Enabled]

Shadow Memory Regions:

C800 - CBFF: [Disabled]

CC00 - CFFF: [Disabled]

D000 - D3FF: [Disabled]

D400 - D7FF: [Disabled]

D800 - DBFF: [Disabled]

DC00 - DFFF: [Disabled]

E000 - E3FF: [Disabled]

E400 - E7FF: [Disabled]

E800 - EBFF: [Disabled]

EC00 - EFFF: [Disabled]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

3.3.3   Advanced Chipset Control

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED SYSTEM SETUP

Warning!

Items on this menu, if set incorrectly, could cause your system to malfunction.

· Memory Cache

· Memory Shadow

· Advanced Chipset Control

· PCI Devices

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting "Advanced Chipset Control " from the Advanced Setup menu displays a menu like the one shown here.

Technicians use this menu to change the values in the chipset registers and optimize your system's performance.

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED CHIPSET CONTROL

DRAM Timing: [Normal]

DRAM Refresh Period: [2048 us]

DRAM Hidden Refresh: [Disabled]

CPU Piped Address: [Disabled]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED CHIPSET CONTROL

PCI FEATURES

CPU/PCI Write Buffer: [Enabled]

CPU/PCI Write-Byte-Merge: [Enabled]

CPU/PCI Write-Fast: [Disabled]

CPU/PCI Write-Burst: [Disabled]

CPU/PCI Read Buffer: [Disabled]

PCI Read-Line Command: [Disabled]

CPU/PCI Burst Read: [Disabled]

PCI/CPU Write Buffer: [Enabled]

PCI/CPU Read Buffer: [Disabled]

PCI Cycle Decoding: [Normal]

PCI/CPU Wait State: [1 ws]

Burst on Non-Bufferable Cycle: [Disabled]

Flash PCI/CPU Buffer: [Enabled]

BRIDGE FEATURES

DMA Line Buffer: [Disabled]

ISA Line Buffer: [Disabled]

PCI Posted Write Buffer: [Disabled]

Arbiter Bus Lock: [Disabled]

Arbiter Bus Park: [Enabled]

DMA Write Cycle Timing: [Normal]

ISA Clock Select: [CLK2/8]

I/O Recovery: [Disabled]

I/O Recovery Period: [1 us]

ISA I/O Cmd Wait Count: [Normal]

ISA Memory Cmd Wait Count: [Normal]

Refresh Period: [15 us]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

NOTE: 	The contents of this menu depends on the chipset installed on your motherboard, and chipsets vary widely. Consult your dealer or the <F1> help screens before changing the items on this menu. Incorrect settings can cause your system to malfunction.

3.3.4   PCI Devices

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

ADVANCED SYSTEM SETUP

Warning!

Items on this menu, if set incorrectly, could cause your system to malfunction.

· Memory Cache

· Memory Shadow

· Advanced Chipset Control

· PCI Devices

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

PCI DEVICES

Base I/O Address: [3000]

Base Memory Address: [008000000]

Multimedia Mode: [Disabled]

Parity: [Disabled]

PCI Interrupt INTB#: [None]

PCI Interrupt INTC#: [None]

PCI Interrupt INTD#: [None]

PCI INT-1 Level/Edge Selection: [Edge]

PCI INT-2 Level/Edge Selection: [Edge]

PCI INT-3 Level/Edge Selection: [Edge]

PCI INT-4 Level/Edge Selection: [Edge]

PCI INT-5 Level/Edge Selection: [Edge]

PCI INT-6 (INTB#) Level/Edge Selection: [Edge]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

.

PCI DEVICES

PCI INT-7 (INTC#) Level/Edge Selection: [Edge]

PCI INT-8 (INTD#) Level/Edge Selection: [Edge]

PCI Device, Slot #1:

Enable Device: [Disabled]

Enable Master: [Disabled]

Use Default Latency Timer Value: [Yes]

Latency Timer Value: [0040]

IRQ Line: [None]

PCI Device, Slot #2:

Enable Device: [Disabled]

Enable Master: [Disabled]

Use Default Latency Timer Value: [Yes]

Latency Timer Value: [0040]

IRQ Line: [None]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

PCI DEVICES

PCI Device, Slot #3:

Enable Device: [Disabled]

Enable Master: [Disabled]

Use Default Latency Timer Value: [Yes]

Latency Timer Value: [0040]

IRQ Line: [None]

PCI Device, Slot #4:

Enable Device: [Disabled]

Enable Master: [Disabled]

Use Default Latency Timer Value: [Yes]

Latency Timer Value: [0040]

IRQ Line: [None]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

NOTE: 	The number of PCI device slots depends on motherboard specifications. When you install PCI IDE card on any PCI slot, you must set that slot¢s IRQ line [14] and [Enabled Device] [Enable Master] items must set Enable.

3.4   Boot Options

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting "Boot Options" from the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

BOOT OPTIONS

Boot Sequence: [A:then C:]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

3.5   Security and Anti-Virus

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting "Security and Anti-Virus" from the Main Menu displays a menu like this:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

SECURITY AND ANTI-VIRUS

Supervisor Password is: Disabled

User Password is: Disabled

Set Supervisor Password: [Press Enter]

Set User Password: [Press Enter]

Password on Boot: [Disabled]

Diskette Access: [Supervisor]

Fixed Disk Boot Sector: [Normal]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

"Supervisor Password is" shows whether supervisor password is set or not. "User Password is" shows whether user password is set or not. Setting Supervisor Password requires a password on entering Setup. The passwords are not case sensitive. Pressing <Enter> at either Set Supervisor Password or Set User Password displays a dialog box like this:

SET SUPERVISOR PASSWORD

Enter New Password: [ ]

Re-enter New Password: [ ]

Enter Accept

SET USER PASSWORD

Enter New Password: [ ]

Re-enter New Password: [ ]

Enter Accept

|  | NOTICE |  |
| --- | --- | --- |
|  | Changes have been saved. |  |
|  | [Continue] |  |

3.6   Green PC Features

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting " Green PC Features " on the main menu displays an information window like this:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

GREEN PC FEATURES

Power Saving Mode: [Enabled]

System Doze Timer: [10 sec]

System Standby Timer: [10 sec]

System Suspend Timer: [10 sec]

Wakeup System During Suspend: [Disabled]

Wakeup Time at (hour): [0 hr]

Wakeup Time at (minute): [0 min]

Hard Disk Standby Timer: [Disabled]

Video Standby Timer: [Disabled]

VGA With Power Down Feature: [None]

Advanced Power Management Setup

System Enters Power Down/Wakeup Modes By:

VGA Access Event: [Off]

IRQ 1 (Keyboard): [On]

IRQ 3 (COM2): [On]

IRQ 4 (COM1): [On]

IRQ 5 (Alt Printer): [On]

IRQ 6 (Diskette): [On]

IRQ 7 (Printer): [On]

IRQ 9 (IRQ2 Redir): [On]

IRQ10 (Reserved): [On]

IRQ11 (Reserved): [On]

IRQ12 (PS/2 Mouse): [On]

IRQ14 (Fixed Disk): [On]

IRQ15 (Reserved): [On]

DMA1 (LAN Card) : [On]

DMA3 (LAN Card) : [On]

LPT IO Address: [On]

PCI Master Event: [Off]

¯ Move ESC Exit PgUp Previous Value F5 Previous Configuration

F1 Help PgDn Next Value F6 Default Configuration

3.7   Load ROM  Default Values

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting " Load ROM Default Values " on the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

NOTICE

Default values have been loaded.

[Continue]

3.8   Load Values from CMOS

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting " Load Values from CMOS " on the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

NOTICE

Previous values have been loaded.

[Continue]

3.9   Save Values to CMOS

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

MAIN MENU

· System Setup

· Fixed Disk Setup

· Advanced System Setup

· Boot Options

· Security and Anti-Virus

· Green PC Features

Load ROM Default Values

Load Values from CMOS

Save Values to CMOS

¯ Move Enter Select

F1 Help ESC Exit F10 Save & Exit

Selecting " Save Values to CMOS " on the Main Menu displays this menu:

Phoenix BIOS Setup - Copyright 1985-94 Phoenix Technologies Ltd.

NOTICE

Changes have been saved.

[Continue]

RMA FORM

When the motherboard can not work well, please fill up this form to describe related situations. If the space is not enough to use, you can attach separate paper.

MODEL:	MODEL NO:

HARDWARE

CPU:   Brand ______________, Model _____________, Speed ______________ MHz

CO-PROCESSOR:   Brand ____________, Model ____________, Speed __________ MHz

SIMM:   Brand _________, Speed _________ ns, Q'ty ________ pcs, Total _________ MB

CACHE:   Brand ____________, Speed ___________ns, Total ____________K

TAG RAM:   Brand ____________, Speed ___________ns

BIOS DATE CODE: ___________

SYSTEM SPEED RUNNING ___________ MHz

VIDEO  CARD:    Chip ___________, RAM ___________, VGA Mode ___________

Bus___________(ISA, VESA or PCI)

OTHER ADD-ON CARDS:

SOFTWARE

OPERATING SYSTEM _______________________	VERSION _____________________

SOFTWARE PROGRAM __________________________________________________

BIOS SETUP: DRAM Wait State_________________ CACHE Wait State_______________

If you change BIOS SETUP, please describe the changes:

<A> ERROR

¨  HANG UP	¨  NO  SCREEN	¨  FLOPPY R/W ERROR

¨  HARD DISK R/W ERROR	¨  PARITY MEMORY ERROR

¨  OTHER ____________________________________________________

<B> ERROR MESSAGES ON YOUR SCREEN (PLEASE SHOW US THE WHOLE SENTENCE)

<C> PROBLEM DESCRIPTION