451-009700300

EXP8551

TABLE OF CONTENTS

CHAPTER  1  INTRODUCTION	1

1.1 OVERVIEW	1

1.2 SYSTEM FEATURES	1

1.3 SYSTEM SPECIFICATION	2

1.4 SYSTEM PERFORMANCE	2

1.5 EXP8551 BOARD LAYOUT	3

CHAPTER  2  INSTALLATION	4

2.1 DRAM INSTALLATION	4

2.2 SRAM INSTALLATION	6

2.3 CPU INSTALLATION	7

2.4 CPU TYPE REFERENCY SETTING	9

2.5 IDE SELECT	10

2.6 FLASH ROM INSTALLATION	11

2.7 EPP MODE SETTING	12

2.8 OTHER JUMPER & CONNECTOR INSTALLATION	13

CHAPTER  3  SYSTEM  BIOS SETUP	16

3.1 SYSTEM SETUP	18

3.2 SECURITY SETUP	24

3.3 DEFAULT SETUP	27

RMA FORM

CHAPTER 1 INTRODUCTION

1.1   OVERVIEW

The EXP8551 is complemented by a 512K second-level Write-Back cache providing workstation level computing performance, and SIMM sockets support up to 128MB of DRAM.

The EXP8551  motherboard offers outstanding I/O capabilities.  Four PCI Local Bus slots provide a high bandwidth data path for data-movement intensive function such as Graphics. Four ISA slots complete the I/O mix.

The EXP8551 motherboard provides the foundation for cost effective, high  performance, highly expandable platforms which deliver the latest EXP8551  in CPU and I/O technologies.

1.2   SYSTEM FEATURES

q Supports INTEL PENTIUM 75/90/100/120/133/150/166/200 MHZ CPU,

CYRIX 6x86-120+(100MHZ)/6x86-150+(120MHZ)/6x86-166+(133MHZ) CPU,

AMD 5K86-P75/5K86-P90 CPU

q Supports 4 MASTER 32-bit PCI Bus

q Supports L1/L2 Write Back/Write Through Cache Feature

q Supports 256KB/512KB Cache Size

q Supports 72pin SIM MODULES

q Supports 2 Serial 1 Parallel 1 FDC on board

q Supports 2 Channels PCI IDE on board

1.3   SYSTEM SPECIFICATIONS

Processor:	INTEL PENTIUM 75/90/100/120/133/150/166/200 MHZ CPU

CYRIX 6x86-120+(100MHZ)/6x86-150+(120MHZ)/

6x86-166+(133MHZ) CPU

AMD 5K86-P75/5K86-P90 CPU

CPU Clock:	50/60/66 MHZ CPU

Memory:	2MB to 128MB

SRAM Configuration:	256K/512K

BIOS Subsystem:	AMI BIOS

Additional BIOS Feature:	Set Program Resides in ROM

I/O Subsystem No. Slot:	Four 16-bit ISA Bus

Four PCI 32-bit Bus

Dimension:	29x22 cm

Additional Features

Miscellaneous Connectors: 	Reset Button, Internal Battery

Board Design: 	4-layer Implementation for Low Noise Operation

1.4   SYSTEM  PERFORMANCE

| SOFTWARE CPU TYPE | LANDMARK V2.0 | POWER METER V1.8 MIPS | NORTON V8.0 CPU SPEED |
| --- | --- | --- | --- |
| PENTIUM 75 | 432.86 MHZ | 36.5 MIPS | 237.7 |
| PENTIUM 90 | 519.45 MHZ | 44.4 MIPS | 285.2 |
| PENTIUM 100 | 574.55 MHZ | 49.0 MIPS | 315.4 |
| PENTIUM 120 | 692.62 MHZ | 58.0 MIPS | 380.2 |
| PENTIUM 133 | 766.1 MHZ | 64.6 MIPS | 420.6 |
| PENTIUM 150 | 865.85 MHZ | 69.4 MIPS | 475.4 |
| PENTIUM 166 | 957.65 MHZ | 81.3 MIPS | 525.8 |
| CYRIX 6x86-120+ (100MHZ) | 1209.93 MHZ | 66.1 MIPS | 680.2 |
| CYRIX 6x86-150+ (120MHZ) | 1448.60 MHZ | 81.3 MIPS | 813.9 |
| CYRIX 6x86-166+ (133MHZ) | 1602.13 MHZ | 88.9 MIPS | 900.7 |
| AMD 5K86-P75 | 671.71 MHZ | 38.4 MIPS | 297 |
| AMD 5K86-P90 | 806.09 MHZ | 47.4 MIPS | 356.5 |

1.5   EXP8551 BOARD LAYOUT

82371

1               5

1           4

3

1

JP12

KEYLOCK          SPKER       RESET  TB-LED

JP9

JP10

CPU  SOCKET

TAG

PCI2

PCI3

1  1

3  3

JP27

JP28

1  1

3  3

JP21

JP22

JP19

JP20

1  1  1

3  3  3

3

1

JP11

JP5

J1              J2          J3      J4

W83768

KB1

KB2

W83787

COM2 COM1

PS1

IDE2   IDE1

FDC

SIM1   SIM2   SIM3   SIM4

82438                82438

SYN.SRAM

32KX32

SYN.SRAM

32KX32

VRM

8042

DS12887

BIOS

U40      U41       U42       U43

PCI4

PCI1

J5

JP14

JP13

U22

JP15

JP16

JP17

JP9

3

1

JP8

PRN

82371

82437

+  -

-

HDD LED

+

3

1

JP31

BANK0   BANK1   BANK0   BANK1

JP23

2     1    2     1    2    1

14   1

26 13

10   9

10   9

2     1  2     1

34 33

40  39  40  39

4

1

JP30

JP18

U36       U37      U38       U39

CHAPTER 2 INSTALLATION

Before the system is ready to operate, the hardware must be set up for various functions of the system.  To set up the EXP8551 motherboard is a simple task.  The user only has to set a few jumpers, connectors and sockets.

2.1   DRAM INSTALLATION

The EXP8551 motherboard can support expanded memory from 2MB to 128MB.

nThe board layout below shows the locations of the DRAM memory banks:

BANK0 BANK1  BANK0   BANK1

SIM1   SIM2   SIM3   SIM4

DRAM  INSTALLATION

SIM1+SIM3

SIM2+SIM4

TOTAL

+                    =

Each group includes two SIMs each SIM size can be 1, 2, 4, 8,16, 32MB, please install the same DRAM size in one group.

n  TABLE  (SIMM)

| BANK 0 |  | BANK 1 |  |  |
| --- | --- | --- | --- | --- |
| SIMM1 | SIMM3 | SIMM2 | SIMM4 | TOTAL MEMORY |
| 4MB | 4MB | None | None | 8MB |
| 4MB | 4MB | 4MB | 4MB | 16MB |
| 8MB | 8MB | None | None | 16MB |
| 4MB | 4MB | 8MB | 8MB | 24MB |
| 8MB | 8MB | 4MB | 4MB | 24MB |
| 8MB | 8MB | 8MB | 8MB | 32MB |
| 16MB | 16MB | None | None | 32MB |
| 4MB | 4MB | 16MB | 16MB | 40MB |
| 16MB | 16MB | 4MB | 4MB | 40MB |
| 8MB | 8MB | 16MB | 16MB | 48MB |
| 16MB | 16MB | 8MB | 8MB | 48MB |
| 16MB | 16MB | 16MB | 16MB | 64MB |
| 32MB | 32MB | None | None | 64MB |
| 4MB | 4MB | 32MB | 32MB | 72MB |
| 32MB | 32MB | 4MB | 4MB | 72MB |
| 8MB | 8MB | 32MB | 32MB | 80MB |
| 32MB | 32MB | 8MB | 8MB | 80MB |
| 16MB | 16MB | 32MB | 32MB | 96MB |
| 32MB | 32MB | 16MB | 16MB | 96MB |
| 32MB | 32MB | 32MB | 32MB | 128MB |

2.2   SRAM  INSTALLATION

U22

JP16

4

1

3

1

JP15

JP17

JP18

TAG

BIOS

JP30

JP10

1  1  1

3  3  3

JP11

82438                82438

DS12887

JP13

82437

BANK1

BANK0

JP12

JP14

3

1

3

1

JP9

SYN.SRAM

32KX32

JP8

CPU  SOCKET

8042

U40      U41       U42       U43

JP31

SYN.SRAM

32KX32

U36       U37      U38       U39

JP5

J1              J2          J3      J4

VRM

1               5

1           4

+  -

KEYLOCK          SPKER       RESET  TB-LED

CACHE SIZE CONFIGURATION

| §256K |  |  | 512K |  |
| --- | --- | --- | --- | --- |
| TAG RAM | DATA RAM | TAG RAM |  | DATA RAM |
| U22 8KX8,16KX8, 32KX8 | U36-U43 32KX8 | U22 16KX8, 32KX8 |  | U36-U43 64KX8 |
| JP12 JP12 JP13 JP14 JP13 JP14 · · 3 2 1 3 2 1 3 2 1 3 2 1 · · · · · · · · 3 2 1 · · 3 2 1 JP14 3 2 1 · · |  |  |  |  |

§ Default setting

2.3   CPU INSTALLATION

U22

JP16

4

1

JP18

3

1

TAG

JP15

JP17

JP30

JP10

BIOS

JP13

1  1  1

3  3  3

JP11

82438                82438

DS12887

82437

JP12

JP14

3

1

3

1

JP9

SYN.SRAM

32KX32

JP8

BANK1

BANK0

CPU  SOCKET

8042

U40      U41       U42       U43

JP31

SYN.SRAM

32KX32

JP5

U36       U37      U38       U39

J1              J2          J3      J4

VRM

1               5

1           4

KEYLOCK          SPKER       RESET  TB-LED

+  -

AMD 5K86/INTEL PENTIUM FREQUENCY SETTING

| 50MHZ | §60MHZ | 66MHZ |
| --- | --- | --- |
| · · · JP16 JP16 JP17 · · · JP16 JP17 JP17 · JP16 JP17 · · · |  |  |

CPU FREQUENCY TIMES

| · §1.5 | 2 | 2.5 | 3 |
| --- | --- | --- | --- |
| · · · · · · · JP31 JP5 JP5 JP31 · · JP31 JP5 JP31 JP5 JP31 |  | · · JP17 |  |

§ Default Setting

3

1

U22

JP15

JP16

JP17

4

1

JP18

JP10

TAG

BIOS

JP13

JP30

1  1  1

3  3  3

JP11

82438                82438

DS12887

82437

JP12

JP14

JP9

3

1

3

1

JP8

SYN.SRAM

32KX32

8042

U40      U41       U42       U43

CPU  SOCKET

SYN.SRAM

32KX32

JP31

JP5

VRM

U36       U37      U38       U39

1               5

1           4

J1              J2          J3      J4

+  -

KEYLOCK          SPKER       RESET  TB-LED

CYRIX 6x86 CPU FREQUENCY SETTING

| CPU TYPE | FREQUENCY | JUMPER SETTING |
| --- | --- | --- |
| · · JP5 JP31 6x86-120+ | 50MHZ |  |
| · · · · JP31 JP5 JP31 6x86-150+ | 60MHZ |  |
| · · · · JP31 JP5 JP31 6x86-166+ | 66MHZ |  |

NOTE: FREQUENCY mean CPU FREQUENCY SETTING (on Page 6).

So you must set JP16, JP17 Jumper.

2.4  CPU TYPE REFERENCE SETTING

| CPU TYPE | FREQUENCY | TIMES |
| --- | --- | --- |
| PENTIUM 75 | 50MHZ | 1.5 |
| PENTIUM 90 | 60MHZ | 1.5 |
| PENTIUM 100 | 66MHZ | 1.5 |
| PENTIUM 120 | 60MHZ | 2 |
| PENTIUM 133 | 66MHZ | 2 |
| PENTIUM 150 | 60MHZ | 2.5 |
| PENTIUM 166 | 66MHZ | 2.5 |
| PENTIUM 200 | 66MHZ | 3 |
| CYRIX 6x86-120+ (100MHZ) | 50MHZ | 2 |
| CYRIX 6x86-150+ (120MHZ) | 60MHZ | 2 |
| CYRIX 6x86-166+ (133MHZ) | 66MHZ | 2 |
| AMD 5K86-P75 | 50MHZ | 1.5 |
| AMD 5K86-P90 | 60MHZ | 1.5 |

2.5   IDE SELECT

82371

PCI2

PCI3

1  1

3  3

JP27

1  1

3  3

JP21

JP22

JP19

JP20

W83768

KB1

KB2

W83787

COM2 COM1

PS1

IDE2   IDE1

FDC

PCI4

PCI1

PRN

-

HDD LED

+

1           4

J6

I.R. CONNECTOR

JP28

JP23

10   9  10   9

14  15

J5

2    1   2     1

2      1

40  39  40  39

34  33

2     1    2     1   2     1

SIM1   SIM2   SIM3   SIM4

BANK0   BANK1  BANK0   BANK1

| DEFAULT | FEATURE FUNCTION |
| --- | --- |
| JP21 JP21 JP22 JP22 · · 3 2 1 3 2 1 3 2 1 · · · · · 3 2 1 · |  |

2.6   FLASH ROM INSTALLATION

3

1

U22

JP16

JP17

4

1

JP18

TAG

BIOS

JP15

JP30

JP10

DS12887

JP13

82437

JP12

1  1  1

3  3  3

JP11

82438                82438

JP14

3

1

3

1

JP9

SYN.SRAM

32KX32

8042

U40      U41       U42       U43

JP8

CPU  SOCKET

JP31

SYN.SRAM

32KX32

JP5

VRM

U36       U37      U38       U39

1               5

1           4

J1              J2          J3      J4

+  -

KEYLOCK          SPKER       RESET  TB-LED

| EPROM | §5V FLASH ROM | 12V FLASH ROM |
| --- | --- | --- |
| JP9 3 2 1 3 2 1 · · · · JP8 JP8 JP8 JP9 · · 3 2 1 3 2 1 3 2 1 · · · · · · 3 2 1 3 2 1 · · JP8 JP8 3 2 1 3 2 1 · · · · JP8 3 2 1 · · JP9 · · 3 2 1 |  |  |

§ Default Setting

3

2

1

2.7  EPP MODE SETTING

82371

PCI2

PCI3

1  1

3  3

JP21

JP22

JP19

JP20

W83768

KB1

KB2

W83787

COM2 COM1

PS1

IDE2   IDE1

FDC

PCI4

PCI1

PRN

-

HDD LED

+

1  1

3  3

JP27

JP23

JP28

26  13

10   9  10   9

J5

2    1    2    1

14   1

40  39  40  39

34  33

2    1    2     1    2     1

SIM1   SIM2   SIM3   SIM4

BANK0  BANK1 BANK0   BANK1

DMA CHANNEL CONFIGURATION

| DMA1 | DMA3 |
| --- | --- |
| JP27 JP27 JP28 JP28 · · 3 2 1 3 2 1 3 2 1 · · · · · 3 2 1 · |  |

2.8   OTHER JUMPER & CONNECTOR INSTALLATION

KB1

KB2

JP23

W83787

W83768

PS1

26 13

10   9  10    9

J5

COM2 COM1

PRN

2      1  2     1

14   1

40  39  40  39

34  33

IDE2   IDE1

FDC

PCI2

PCI3

PCI4

PCI1

2     1   2     1    2     1

82371

JP19

JP20

-

HDD LED

+

3

1

U22

JP15

JP16

JP17

4

1

JP18

BIOS

TAG

JP30

JP10

DS12887

JP11

82438                82438

82437

1               5

1           4

KEYLOCK          SPKER       RESET  TB-LED

CPU SOCKET

J1              J2          J3      J4

SYN.SRAM

32KX32

SYN.SRAM

32KX32

VRM

8042

+  -

OTHER JUMPER DESCRIPTION

| JUMPER | DESCRIPTION |  |
| --- | --- | --- |
| · · JP10 | INTERNAL CACHE WRITE BACK § | INTERNAL CACHE WRITE THROUGH |
| · · JP11 | CPU PIPELINE DISABLE | CPU PIPELINE ENABLE § |
| · · 3 2 1 JP15 · · 3 2 1 | AT BUS CLOCK EQUAL PCICLK/3 ( FOR FREQUENCY 50MHZ ) | AT BUS CLOCK PCICLK/4 § ( FOR FREQUENCY 60, 66MHZ) |
| · · JP18 | NORMAL | CLEAR CMOS |
| · · JP19 | NORMAL MODE | SMI MODE |
| · · JP23 | FOR PARALLEL PORT EPP MODE | FOR STANDARD PARALLEL PORT § |

§ Default Setting

CONNECTOR DESCRIPTION

| CONNECTOR | PIN OUT | SIGNAL NAME |
| --- | --- | --- |
| J1 KEY LOCK | 1 2 3 4 5 | LED POWER NOT USED GROUND KEYBOARD INHIBITOR GROUND |
| J2 SPEAKER | 1 2 3 4 | DATA OUT NOT USED GROUND +5V DC |
| J3 RESET | 1 2 | GROUND RESET IN |
| J4 TB-LED | 1 2 | +ANODE -CATHODE |
| JP20 HDD LED | 1 2 | +ANODE -CATHODE |
| JP30 CPU FAN CONNECTOR | 1 2 3 4 | GROUND GROUND GROUND +12V |

CHAPTER 3 SYSTEM BIOS SETUP

WinBIOS Setup can be accessed via keyboard, mouse, or pen.  The mouse click functions are:

·   single click change or select both global and current fields and

·   double click to perform an operation in the selected field.

Using the keyboard with WinBIOS Setup

WinBIOS Setup has a built-in keyboard driver that uses simple keystroke combinations:

| KEYSTROKE | FUNCTION |
| --- | --- |
| <Tab> | Move to the next window or field. |
| ®,¬,,¯ | Move to the next field to the right, left, above, or below. |
| <Enter> | Select in the current field. |
| + | Increments a value. |
| - | Decrements a value. |
| <Esc> | Closes the current operation and return to previous level. |
| <PgUp> | Returns to the previous page. |
| <PgDn> | Advances to the next page. |
| <Home> | Returns to the beginning of the text. |
| <End> | Advances to the end of the text. |
| <Alt>,<H> | Access a help window. |
| <Alt>,<Spacebar> | Exit WinBIOS Setup. |
| Alphabetic keys | A to Z are used in the Virtual Keyboard, and are not case sensitive. |
| Numeric Keys | 0 to 9 are used in the Virtual Keyboard and Numeric Keypad. |

The hardware features and options of the EXP8551 are on-site selectable for maximum flexibility.  You will need to configure these options through the built-in Setup Utility prior to using EXP8551 for the first time.  This setup Utility is a multi-screen, menu driven program and is contained within the BIOS EPROM.

The  following  sections  show  the  procedures  that  you  may  need  to  configure  the EXP8551:

1.   Press<DEL>while turning on or rebooting the system to invoke Setup Utility

program.

2.   The main menu will be shown as follows:

Figure 1

3.   The functions are grouped into four categories which are Setup, Utility, Security and

Default.

4.   By using <TAB>key or mouse cursor to select the function group.

5.   Use arrow keys or mouse to select  the  function icon  within  the group.  Then press

<Enter>key to invoke the setup function.

6.   Use<Esc>key to go back to the previous screen.

3.1    SYSTEM  SETUP

3.1.1	Advanced Setup

Optimal Setting

You can load the optimal default settings for the WINBIOS by pressing the “END” key upon power-up. The Optimal default settings are best-case values that should optimize system performance. The Optimal settings will automatically detect and load  the parameter of  hard disk type.

### Advanced Setup

Advanced Setup options are displayed by choosing the Advanced icon from the WINBIOS Setup main menu. All Advanced Setup options are described in this section.

Quick Boot

Set this option to Enabled to instruct AMIBIOS to boot quickly when the computer is powered on. This option replaces the old Above 1 MB Memory Test Advanced Setup option. The settings are:

| Setting | Description |
| --- | --- |
| Disabled | AMIBIOS test all system memory. AMIBIOS waits up to 40 seconds for a READY signal from the IDE hard disk drive. AMIBIOS waits for .5 seconds after sending a RESET signal to the IDE drive to allow the IDE drive time to get ready again. AMIBIOS checks for a <Del> key press and runs WINBIOS Setup if the key has been pressed. |
| Enabled | AMIBIOS does not test system memory above 1 MB. AMIBIOS does not wait up to 40 seconds for a READY signal from the IDE hard disk drive. If a READY signal is not received immediately from the IDE drive, AMIBIOS does not configure that drive. AMIBIOS does not wait for .5 seconds after sending a RESET signal to the IDE drive to allow the IDE drive time to get ready again. You cannot run WINBIOS Setup at system boot, because there is no delay for the Hit <Del> to run Setup message. |

The Optimal and Fail-Safe default settings are Enabled.

OS/2 Compatible Mode

Set this option to Enabled to permit AMIBIOS to run with IBM OS/2. The settpings are Enabled or Disabled. The default settings are Disabled.

System BIOS Cacheable

When this option is set to Enabled, the contents of the F0000h system memory segment can be read from or written to L2 secondary cache memory. The contents of the F0000h memory segment are always copied from the BIOS ROM to system RAM for faster execution.

The settings are Enabled or Disabled. The Optimal default setting is Enabled. The Fail-Safe default setting is Disabled.

### 3.1.2	Power Management Setup

Power Management Setup options are displayed by choosing the Power Mgmt icon from the WINBIOS Setup main menu. All Power Management Setup options are described in this section.

Power Management/APM

Set this option to Enabled to enable the power management and APM Advanced Power Management) features. The settings are Enabled or Disabled. The default settings are Disabled.

Instant On Timeout

Set this option to Enabled to allow the computer to go to full power on mode when leaving a power-conserving state. This option is only available if supported by the computer hardware. AMIBIOS uses the RTC Alarm function to wake the computer at a prespecified time. The settings are Enabled or Disabled. The default settings are Disabled.

Green PC Monitor Power State

This option specifies the power management state that the Green PC-compliant video monitor enters after the specified period of display inactivity has expired. The settings are Disabled, Off, Standby, or Suspend. The default settings are Disabled.

Video Power Down Mode

This option specifies the power management state that the video subsystem enters after the specified period of display inactivity has expired. The settings are Disabled, Standby, or Suspend. The default settings are Disabled.

Hard Disk Power Down Mode

This option specifies the power management state that the hard disk drive enters after the specified period of display inactivity has expired. The settings are Disabled, Standby, or Suspend. The default settings are Disabled.

Hard Disk Time out (Min)

This option specifies the length of a period of hard disk inactivity. When this period expires, the hard disk drive enters the power-conserving mode specified in the Hard Disk Power Down Mode

option described on the previous page. The settings are Disabled, 1 Min (minutes), and all one minute intervals up to and including 15 Min. The default settings are Disabled.

Standby Time out

This option specifies the length of the period of system inactivity when the computer is in Full-On mode before the computer is placed in Standby mode. In Standby mode, some power use is curtailed. The settings are Disabled, 1 Min, 2 Min, and all one minute intervals up to and including 15 Min. The default settings are Disabled.

Suspend  Time out

This option specifies the length of the period of system inactivity when the computer is already in Standby mode before the computer is placed in Suspend mode. In Suspend mode, nearly all power use is curtailed. The settings are Disabled, 1 Min, 2 Min, and all one minute intervals up to and including 15 Min. The default settings are Disabled.

Slow Clock Ratio

This option specifies the speed at which the system clock runs in power saving modes. The settings are expressed as a ratio between the normal clock speed and the power down clock speed. The settings are 1:1, 1:2 (half as fast as normal), 1:4 (the normal clock speed), 1:8, 1:16, 1:32, 1:64, or 1:128. The default setting is 1:1.

Display Activity

This option specifies if AMIBIOS is to monitor activity on the display monitor for power conservation purposes. When this options set to Monitor and there is no display activity for the length of time specified in the value in the Full-On to Standby Timeout (Min) option, the computer enters a power saving state. The settings are Monitor or Ignore. The default settings are Ignore.

IRQ XX

These options enable event monitoring. When the computer is in a power saving mode, activity on the named interrupt request line is monitored by AMIBIOS. When any activity occurs, the computer enters Full On mode.

Each of these options can be set to Monitor or Ignore. The default setting for all options is Ignore.

### 3.1.3    PCI/PnP Setup

PCI/PnP Setup options are displayed by choosing the PCI/PnP Setup icon from the WINBIOS Setup main menu. All PCI/PnP Setup options are described in this section

Plug and Play Aware O/S

Set this option to Yes if the operating system installed in the computer is Plug and Play-aware. AMIBIOS only detects and enables PnP ISA adapter cards that are required for system boot. The Windows 95 operating system detects and enables all other PnP-aware adapter cards. Windows 95 is PnP-aware. Set this option to No if the operating system (such as DOS, OS/2, Windows 3.x) does not use PnP. You must set this option correctly or PnP-aware adapter cards installed in your computer will not be configured properly. The settings are No or Yes. The Optimal and Fail-Safe default settings are No.

PCI Burst Mode

Set this option to Enabled to enable PCI burst mode. The settings are Disabled or Enabled. The Optimal default setting is Enabled. The Fail-Safe default setting is Disabled.

PCI Latency Timer (PCI Clocks)

This option sets latency of all PCI devices on the PCI bus. The settings are in units equal to PCI clocks. The settings are 32, 64, 96, 128, 160, 192, 224, or 248. The Optimal and Fail-Safe default settings are 64.

PCI VGA Palette Snoop

This option must be set to Enabled if any ISA adapter card installed in the computer requires VGA palette snooping. The settings are Disabled or Enabled. The Optimal and Fail-Safe default settings are Disabled.

PCI IDE BusMaster

Set this option to Enabled to specify that the IDE controller on the PCI local bus has bus mastering capability. The settings are Disabled or Enabled. The Optimal and Fail-Safe default settings are Disabled.

Offboard PCI IDE Card

This option specifies if an offboard PCI IDE controller adapter card is used in the computer. You must also specify the PCI expansion slot on the motherboard where the offboard PCI IDE controller card is installed. If an offboard PCI IDE controller is used, the onboard IDE controller on the motherboard is automatically disabled. The settings are Disabled, Auto, Slot1, Slot2, Slot3, or Slot4.

If Auto is selected, AMIBIOS automatically determines the correct setting for this option. The Optimal and Fail-Safe default settings are Auto.

In the AMIBIOS for the Intel Triton chipset, this option forces IRQ 14 and 15 to a PCI slot on the PCI local bus. This is necessary to support non-compliant PCI IDE adapter cards.

Offboard PCI IDE Primary IRQ

This option specifies the PCI interrupt used by the primary IDE channel on the offboard PCI IDE controller. The settings are Disabled, INTA, INTB, INTC, or INTD. The Optimal and Fail-Safe default settings are Disabled.

Offboard PCI IDE Secondary IRQ

This option specifies the PCI interrupt used by the secondary IDE channel on the offboard PCI IDE controller. The settings are Disabled, INTA, INTB, INTC, or INTD. The Optimal and Fail-Safe default settings are Disabled.

PCI/PnP Setup

These options specify the bus that the named interrupt request lines (IRQs) are used on. These options allow you to specify IRQs for use by legacy ISA adapter cards.

These options determine if AMIBIOS should remove an IRQ from the pool of available IRQs passed to BIOS configurable devices. The available IRQ pool is determined by reading the ESCD NVRAM. If more IRQs must be removed from the pool, the end user can use these PCI/PnP Setup options to remove the IRQ by assigning the option to the  ISA/EISA setting. Onboard I/O is configurable by AMIBIOS. The IRQs used by onboard I/O are configured as PCI/PnP.

The settings are PCI/PnP or ISA/EISA. The Optimal and Fail-Safe default settings are PCI/PnP.

Reserved Memory Size

This option specifies the size of the memory area reserved for legacy ISA adapter cards.

The settings are Disabled, 16K, 32K, or 64K. The Optimal and Fail-Safe default settings are Disabled.

Reserved Memory Address

This option specifies the beginning address (in hex) of the reserved memory area. The specified ROM memory area is reserved for use by 	legacy ISA adapter cards.

The settings are C0000, C4000, C8000, CC000, D0000, D4000, D8000, or DC000. The Optimal and Fail-Safe default settings are C0000.

3.2    Security Setup

WinBIOS Setup has an optional password feature.  The system can be configured so that all users must enter a password every time the system boots or when WinBIOS Setup is executed.  The following screen appears when you select the password icon.

Figure 2

You can enter a password by:

·   typing the password on the keyboard,

·   selecting each letter via the mouse, or

·   selecting each letter via the pan stylus.

Pen access must be customized for each specific hardware platform.

The password check option is enabled in Advanced Setup by choosing either Always ( the

password prompt appears every time the system is powered on ) or Setup ( the password

prompt appears only when WinBIOS Setup is run).  The password is stored in CMOS RAM

CMOS RAM.  The system asks for a password.

Enter a 1-6 character password.  The password does not appear on the screen when typed. WinBIOS  will ask you  to retype the password .  Make sure you  write it  down.  If you forget it, you must drain CMOS RAM and  reconfigure the system.  WinBIOS  will then display the following:

Figure 3

Select the Password icon from the Security section of the WinBIOS Setup  main menu.  Enter the password and press <Enter>.  The screen does not display the characters entered.  After the new password is entered, retype the new password as prompted and press<Enter>.

If the password confirmation is incorrect,  an error message appears.  If the new password is entered without error,  press<Esc>to return to the WinBIOS Setup Main Menu. The password is stored in CMOS RAM after WinBIOS Setup completes.  The next time the system boots,  you are prompted for the  password if the password function is present and is enabled.

Remember the Password

Keep a record of the new password when the password is changed.  If you forget the password and you computer has an American Megatrends motherboard, remove the  computer cover, set switch 1-2 ( the DIAG switch ) to ON, power on the computer.  WinBIOS erases the password.

When this icon is selected from the Security section of the WinBIOS Setup main menu, WinBIOS issues a warning when any program (or virus) issues a Disk Format command or attempts to write to the boot sector of the hard disk drive.  The following screen appears when you select the Anti-Virus icon:

Figure 4

The setting are Enabled or Disabled.  If enabled, the following appears when a write is attempted to the boot sector.  You may have to type N several times to prevent the boot sector write.

3.3    Default Setup

The icons in this section permit you to select a group of settings for all WinBIOS Setup options.  Not only can you use these icons to quickly set system configuration parameters, you can choose a group of settings that have a better chance of working when the system is having configuration-related problems.

Original

Choose the Original icon to return to the system configuration values present in WinBIOS Setup when you first began this WinBIOS Setup session.

Optimal You can load the optimal default settings for the WinBIOS Setup options by selecting the Optimal icon. The Optimal default settings are best-case values that should optimize system performance. If CMOS RAM is corrupted, the Optimal settings are loaded automatically.

Fail-Safe

You can load the Fail-Safe WinBIOS Setup option settings by selecting the Fail-Safe icon from the Default section of the WinBIOS Setup main menu.

The Fail-Safe settings provide far from optimal system performance, but are the most stable  settings.  Use this option as a diagnostic aid if the system is behaving erratically.

Figure 5

Figure 6

WINBIOS BEEP CODES

| BEEPS | ERROR MESSAGE | DESCRIPTION |
| --- | --- | --- |
| 1 | Refresh Failure | The memory refresh circuitry is faulty. |
| 2 | Parity error | Parity error in the base memory (the first 64 KB block) of memory. |
| 3 | Base 64 KB Memory Failure | Memory failure in first 64 KB. |
| 4 | Timer Not Operational | A memory failure in the first 64 KB of memory, or Timer is not functioning. |
| 5 | Processor error | The CPU generated an error. |
| 6 | 8042-Gate A20 Failure | Cannot switch to protected mode. |
| 7 | Processor exception interrupt Error | The CPU on the CPU Card generated an exception interrupt. |
| 8 | Display Memory Read/Write Error | The system video adapter is either missing or its memory is faulty. This is not a fatal error. |
| 9 | ROM Checksum Error | The ROM checksum value does not match the value encoded in WinBIOS. |
| 10 | CMOS Shutdown Register Read/Write Error | The shutdown register for CMOS RAM has failed. |
| 11 | Cache memory bad-do not enable cache | The cache memory test failed. Cache memory is disabled. Do not press<Ctrl><Alt><+> to enable cache memory. |

What to Do If the Computer Beeps

Here is what you need to do if your computer has a WinBIOS and it starts beeping:

| IF THE SYSTEM BEEPS...... | THEM... |
| --- | --- |
| 1, 2, or 3 times... | Reseat the memory SIMMs or DIPs. If the system still beeps, replace the memory. |
| 6 times... | Reseat the keyboard controller chip. If it still beeps, replace the keyboard controller. If it still beeps, try a different keyboard, or replace the keyboard fuse, if the keyboard has one. |
| 8 times... | There is a memory error on the video adapter. Replace the video adapter , or the RAM on the video adapter. |
| 9 times... | The BIOS ROM chip is bad. The system probably needs a new BIOS ROM chip. |
| 11 times... | Reseat the cache memory on the motherboard. If it still beeps, replace the cache memory. |
| 4, 5, 7 or 10 times... | The motherboard must be replaced. |

RMA  FORM

When the motherboard can not work well, please fill up this form to describe related

situations.  If the space is not enough to use, you can attach separate paper.

MODEL:                                                        MODEL NO:

HARDWARE

CPU :  Brand                               , Model                              , Speed                                    MHz

CD-PROCESSOR:  Brand                               , Model                               , Speed                            MHz

SIMM: Brand                         , Speed                           ns, Q’ty                        pcs,Total                   MB

CACHE: Brand                               , Speed                                ns,Total                              K

TAG RAM: Brand                              , Speed                              ns

BIOS DATA CODE:

SYSTEM SPEED RUNNING                                 MHz

VIDEO CARD:   Chip                            , RAM                            , VGA Mode

Bus                                     (ISA,VESA or PCI)

OTHER ADD-ON CARDS:

SOFTWARE

OPERATING SYSTEM                                                 VERSION

SOFTWARE PROGRAM

BIOS SETUP: DRAM Wait State                                     CACHE Wait State

If you change BIOS SETUP, please describe the changes:

<A>ERROR

¨HANG UP                             ¨NO SCREEN                             ¨FLOPPY R/W ERROR

¨HARD DISK R/W ERROR      ¨PARITY MEMORY ERROR

¨OTHER

<B>ERROR MESSAGES ON YOUR SCREEN (PLEASE SHOW US THE WHOLE  SENTENCE)

<C>PROBLEM DESCRIPTION