451-011400100

REVISION:D

EXP8561

TABLE OF CONTENTS

CHAPTER  1  INTRODUCTION	1

1.1   OVERVIEW	1

1.2   SYSTEM FEATURES	1

1.3   SYSTEM SPECIFICATION	2

1.4   SYSTEM PERFORMANCE	2

1.5   EXP8561 BOARD LAYOUT	3

CHAPTER  2  INSTALLATION	4

2.1   DRAM INSTALLATION	4

2.2   SRAM INSTALLATION	6

2.3   CPU TYPE JUMPER SETTING	7

2.4   CPU I/O & CPU CORE VOLTAGE SELECT	8

2.5   FLASH ROM INSTALLATION	9

2.6   EPP MODE SETTING	10

2.7   OTHER JUMPER & CONNECTOR INSTALLATION	11

CHAPTER  3  SYSTEM  BIOS SETUP	13

3.1    ENTERING SETUP	13

3.2    CONTROL KEYS	13

3.3    GETTING HELP	14

3.4    THE MAIN MENU	14

3.5    STANDARD CMOS SETUP MENU	16

3.6    BIOS FEATURES SETUP	19

3.7    CHIPSET FEATURES SETUP	22

3.8    POWER MANAGEMENT	23

3.9    PNP/PCI CONFIGURATION SETUP	25

3.10  INTEGRATED PERIPHERALS	26

3.11  PASSWORD SETTING	27

3.12  IDE HDD AUTO DETECTION	28

RMA FORM

CHAPTER   1

INTRODUCTION

1.1	OVERVIEW

The EXP8561 is complemented by a 512K second-level Write-Back cache providing workstation level computing performance, and SIMM sockets support up to 128MB of DRAM.

The EXP8561  motherboard offers outstanding I/O capabilities.  Four PCI Local Bus slots provide a high bandwidth data path for data-movement intensive function such as Graphics. Four ISA slots complete the I/O mix.

The EXP8561 motherboard provides the foundation for cost effective, high  performance, highly expandable platforms which deliver the latest EXP8561  in CPU and I/O technologies.

1.2	SYSTEM FEATURES

q  Supports INTEL PENTIUM 75/90/100/120/133/150/166/200 MHZ CPU,

CYRIX 6x86-120+(100MHZ)/ 6x86-133+(110MHZ)/6x86-150+(120MHZ)/6x86-166+(133MHZ)

AMD 5K86-P75/5K86-P90/5K86-P100 CPU

q  Supports 4 MASTER 32-bit PCI Bus

q  Supports L1/L2 Write Back/Write Through Cache Feature

q  Supports 256KB/512KB Cache Size

q  Supports 72 pin SIMM MODULES

q  Supports 2 Serial 1 Parallel 1 FDC on board

q  Supports 2 Channels PCI IDE on board

q  Supports MESI Protocol (Modified Exclusive Shared Invalid) to maintain the data coherence for       L2 Cache to optimize CPU bus utilization.

1.3	SYSTEM SPECIFICATIONS

Processor:	INTEL PENTIUM 75/90/100/120/133/150/166/200 MHZ CPU

CYRIX 6x86-120+(100MHZ)/6x86-133+(110MHZ)/

6x86-150+(120MHZ)/ 6x86-166+(133MHZ) CPU

AMD 5K86-P75/5K86-P90/5K86-P100 CPU

CPU Clock:	50/55/60/66 MHZ CPU

Memory:	2MB to 128MB

SRAM Configuration:	256K/512K

BIOS Subsystem:	AWARD BIOS

Additional BIOS Feature:	Set Program Resides in ROM

I/O Subsystem No. Slot:	Four 16-bit ISA Bus

Four 32-bit PCI Bus

Dimension:	26x22 cm

Additional Features

Miscellaneous Connectors: 	Reset Button, Internal Battery

Board Design: 	4-layer Implementation for Low Noise Operation

1.4	SYSTEM  PERFORMANCE

| SOFTWARE CPU TYPE | LANDMARK V2.0 | POWER METER V1.8 MIPS | NORTON V8.0 CPU SPEED |
| --- | --- | --- | --- |
| PENTIUM 75 | 433.85 MHz | 36.9 MIPS | 238.2 |
| PENTIUM 90 | 526.85 MHz | 45.1 MIPS | 289.2 |
| PENTIUM 100 | 578.51 MHz | 49.9 MIPS | 317.6 |
| PENTIUM 120 | 702.43 MHz | 61.8 MIPS | 385.6 |
| PENTIUM 133 | 771.33 MHz | 67.7 MIPS | 423.5 |
| PENTIUM 150 | 878.06 MHz | 76.9 MIPS | 482.1 |
| PENTIUM 166 | 964.19 MHz | 83.6 MIPS | 529.3 |
| PENTIUM 200 | 1157.04 MHz | 94.8 MIPS | 635.2 |
| CYRIX 6x86-120+ (100MHZ) | 1209.95 MHz | 67.7 MIPS | 680.3 |
| CYRIX 6x86-133+ (110MHZ) | 1353.98 MHz | 74.8 MIPS | 761.2 |
| CYRIX 6x86-150+ (120MHZ) | 1469.25 MHz | 81.3 MIPS | 826 |
| CYRIX 6x86-166+ (133MHZ) | 1613.31 MHz | 84.3 MIPS | 907 |
| AMD 5K86-P75 | 673.26 MHz | 39.0 MIPS | 297.7 |
| AMD 5K86-P90 | 817.55 MHz | 47.4 MIPS | 361.5 |
| AMD 5K86-P100 | 892.56 MHz | 56.9 MIPS | 394.7 |

1.5	EXP8561 BOARD LAYOUT

JP9

82371

3

1

KEYBOARD BIOS

PS/2 MOUSE CONNECTOR

KEYBOARD CONNECTOR

J13

1           4

1           4

1     3

W83768

W83787

POWER CONNECTOR

JP4

1           4

J14

JP5

1    2

1     2  1     2

1     3

COM2 COM1

J4

PRN

9   10  9   10

25 26

1    2

1    2    1    2

IDE2   IDE1

FDC

J7

J6

J5

J8

U5

33 34   39 40    39 40

82371SB

SIM4   SIM3   SIM2    SIM1

BANK1   BANK1   BANK0   BANK0

J12

JP7

JP8

U10

U18

U17

DS12887

27010

82439HX

JP6

JP9

3

1

JP14

1      3

JP10

JP12

TAG

JPA1

JP13

J16

JP15

CPU  SOCKET

JPA2

PBSRAM

32KX32

PBSRAM

32KX32

3

1

JP19

3

1

JP18

U21

U19

U23

J19               J18            JP23 JP22   J15

JP21

J20

1               5

1           4

1          4

CHAPTER   2

INSTALLATION

Before the system is ready to operate, the hardware must be set up for various functions of the system.  To set up the EXP8561 motherboard is a simple task.  The user only has to set a few jumpers, connectors and sockets.

2.1	DRAM INSTALLATION

The EXP8561 motherboard can support expanded memory from 2MB to 128MB.

nThe board layout below shows the locations of the DRAM memory banks:

BANK1 BANK1  BANK0  BANK0

SIMM4  SIMM3 SIMM2  SIMM1

DRAM  INSTALLATION

SIMM1+SIMM2

SIMM3+SIMM4

TOTAL

+                          =

Each group includes two SIMMs each SIMM size can be 1, 2, 4, 8,16, 32MB, please install the same DRAM size in one group.

n  TABLE   (SIMM)

| BANK 0 |  | BANK 1 |  |  |
| --- | --- | --- | --- | --- |
| SIMM1 | SIMM2 | SIMM3 | SIMM4 | TOTAL MEMORY |
| 4MB | 4MB | None | None | 8MB |
| 4MB | 4MB | 4MB | 4MB | 16MB |
| 8MB | 8MB | None | None | 16MB |
| 4MB | 4MB | 8MB | 8MB | 24MB |
| 8MB | 8MB | 4MB | 4MB | 24MB |
| 8MB | 8MB | 8MB | 8MB | 32MB |
| 16MB | 16MB | None | None | 32MB |
| 4MB | 4MB | 16MB | 16MB | 40MB |
| 16MB | 16MB | 4MB | 4MB | 40MB |
| 8MB | 8MB | 16MB | 16MB | 48MB |
| 16MB | 16MB | 8MB | 8MB | 48MB |
| 16MB | 16MB | 16MB | 16MB | 64MB |
| 32MB | 32MB | None | None | 64MB |
| 4MB | 4MB | 32MB | 32MB | 72MB |
| 32MB | 32MB | 4MB | 4MB | 72MB |
| 8MB | 8MB | 32MB | 32MB | 80MB |
| 32MB | 32MB | 8MB | 8MB | 80MB |
| 16MB | 16MB | 32MB | 32MB | 96MB |
| 32MB | 32MB | 16MB | 16MB | 96MB |
| 32MB | 32MB | 32MB | 32MB | 128MB |

2.2	SRAM  INSTALLATION

J12

JP8

U17

JP7

U10

U18

27010

JP6

DS12887

82439HX

BANK1

BANK0

JP9

JP14

1      3

JP10

JP12

TAG

3

1

JPA1

JP13

J16

JP15

CPU  SOCKET

JPA2

3

1

PBSRAM

32KX32

JP19

3

1

PBSRAM

32KX32

U21

U19

U23

JP18

JP21

J20

1               5

J19               J18            JP23 JP22   J15

1           4

1          4

CACHE SIZE CONFIGURATION

| §256K |  | 512K |  |
| --- | --- | --- | --- |
| TAG RAM | DATA RAM | TAG RAM | DATA RAM |
| U23 8KX8,16KX8, 32KX8 | U19,U21 32KX32 | U23 16KX8, 32KX8 | U19, U21,J12 32KX32 |
| JP15 · · JP15 JP21 3 2 1 JP19 · · 3 2 1 JP21 · · JP19 J16 J16 · · |  |  |  |

§ Default setting

2.3	CPU TYPE JUMPER SETTING

| CPU TYPE | JUMPER SETTING |
| --- | --- |
| JP9 JP10 · · JP23 JP22 · · AMD 5K86-P75/PENTIUM 75MHZ |  |
| JP23 JP22 · · JP10 JP9 AMD 5K86-P90/PENTIUM 90MHZ |  |
| JP23 JP22 · · JP10 JP9 AMD 5K86-P100/PENTIUM 100MHZ |  |
| · · JP23 JP22 · · JP10 JP9 · · JP10 JP9 INTEL PENTIUM 120MHZ |  |
| · · JP23 JP22 · · JP10 JP9 INTEL PENTIUM 133MHZ |  |
| · · · · JP23 JP22 INTEL PENTIUM 150MHZ |  |
| · · · · JP23 JP22 · · JP10 JP9 · · JP10 JP9 INTEL PENTIUM 166MHZ |  |
| · · JP23 JP22 · · · · JP10 JP9 INTEL PENTIUM 200MHZ |  |
| · · JP23 JP22 CYRIX 6x86-120+(100MHZ) |  |
| · · JP23 JP22 JP10 JP9 · · JP10 JP9 CYRIX 6x86-133+(110MHZ) |  |
| · · JP23 JP22 CYRIX 6x86-150+(120MHZ) |  |
| · · JP23 JP22 JP9 JP10 · · 3 2 1 · · JPA2 JPA1 JP18 3 2 1 · · CYRIX 6x86-166+(133MHZ) |  |
| · · JP23 JP22 · · JP9 JP10 · · INTEL PENTIUM MMX 166MHZ |  |
| JP23 · · JP22 JP9 · · JP10 3 2 1 · · JPA2 JPA1 JP18 3 2 1 · · INTEL PENTIUM MMX 200MHZ |  |
| JP23 JP22 · · JP9 JP10 · · 3 2 1 · · JPA2 JPA1 JP18 3 2 1 · · CYRIX 6x86-166+(133MHZ) (Dual Voltage) |  |

2.4	CPU I/O & CPU CORE VOLTAGE SELECT

J12

JP8

U18

U17

JP7

27010

U10

DS12887

82439HX

JP6

JP14

1      3

JP9

3

1

JP10

JP12

TAG

JPA1

JPA2

JP13

J16

JP15

CPU  SOCKET

PB SRAM

32KX32

3

1

PB SRAM

32KX32

3

1

JP19

JP18

U21

U19

U23

JP21

J19               J18            JP23 JP22   J15

J20

1               5

1           4

1          4

CPU I/O VOLTAGE SELECT

The jumper setting is only for Dual Voltage CPU (INTEL P55C etc..)

| 3.3V | 3.45V | §Normal CPU (Single Voltage) |
| --- | --- | --- |
| 3 2 1 · · JP18 3 2 1 · · JP18 3 2 1 JP18 |  |  |

CPU CORE VOLTAGE SELECT

| 2.9V | §3.3V | 3.45V~3.6V |
| --- | --- | --- |
| JPA1 JPA2 · · 3 2 1 3 2 1 · · JPA2 JPA1 3 2 1 · · JPA1 JPA2 |  |  |

§ Default setting

2.5	FLASH ROM INSTALLATION

J12

JP8

U17

U10

U18

JP7

27010

JP6

DS12887

82439HX

JP14

1      3

JP9

JP10

3

1

JP12

TAG

JPA1

JP13

J16

JP15

CPU  SOCKET

JPA2

3

1

PBSRAM

32KX32

3

1

PBSRAM

32KX32

U21

U19

U23

JP19

JP18

JP21

J20

1               5

J19               J18            JP23 JP22   J15

1           4

1          4

| §5V FLASH ROM | 12V FLASH ROM |
| --- | --- |
| JP8 JP8 3 2 1 3 2 1 · · · · JP14 1 2 3 · · JP14 1 2 3 · · |  |

§ Default Setting

3

2

1

2.6	EPP MODE SETTING

KEYBOARD BIOS

PS/2 MOUSE CONNECTOR

KEYBOARD CONNECTOR

J13

1           4

W83768

W83787

1           4

1           4

J14

1     3

POWER CONNECTOR

JP4

JP5

1    2

1     2  1     2

1     3

J4

9   10  9   10

COM2 COM1

PRN

25 26

1    2    1    2

1    2

IDE2   IDE1

FDC

J7

J6

J5

J8

33 34   39 40    39 40

U5

82371SB

SIM4   SIM3   SIM2    SIM1

BANK1   BANK1   BANK0   BANK0

DMA CHANNEL CONFIGURATION (FOR EPP DEVICE)

| DMA1 | DMA3 |
| --- | --- |
| · · · · 1 2 3 JP4 JP5 JP4 JP5 · · · · 1 2 3 |  |

2.7	OTHER JUMPER & CONNECTOR INSTALLATION

PS/2 MOUSE CONNECTOR

KEYBOARD CONNECTOR

1           4

KEYBOARD BIOS

J13

1           4

W83768

W83787

POWER CONNECTOR

1           4

J14

1     2  1     2

1    2

J4

9   10  9   10

COM2 COM1

PRN

25 26

1    2    1    2

1    2

IDE2   IDE1

FDC

J7

J6

J5

J8

33 34   39 40    39 40

U5

82371SB

JP8

U17

JP7

U10

U18

27010

JP6

DS12887

82439HX

TAG

CPU  SOCKET

U23

J20

1               5

J19               J18                                J15

1           4

1          4

OTHER JUMPER DESCRIPTION

| JUMPER | DESCRIPTION |  |
| --- | --- | --- |
| · · JP6 | NORMAL MODE | SMI MODE |
| · · JP7 | AT BUS CLOCK EQUAL PCICLK/3 (FOR FREQUENCY 50MHZ) | AT BUS CLOCK PCICLK/4 § ( FOR FREQUENCY 60, 66MHZ) |
| · · JP8 | NORMAL | CLEAR CMOS |

§ Default Setting

CONNECTOR DESCRIPTION

| CONNECTOR | PIN OUT | SIGNAL NAME |
| --- | --- | --- |
| J13, J14 USB CONNECTOR | 1 2 3 4 | +5V DC DATA OUT DATA OUT GROUND |
| J15 RESET | 1 2 | GROUND RESET IN |
| J18 KEYLOCK CONNECTOR | 1 2 3 4 5 | VCC NC GROUND KEYBOARD INHIBITOR GROUND |
| J19 SPEAKER CONNECTOR | 1 2 3 4 | DATA LINE NC GROUND +5V DC |
| J20 IDE_LED | 1 2 3 4 | +5V DC DATA OUT DATA OUT +5V DC |
| PS/2 MOUSE CONNECTOR | 1 2 3 4 | DATA DATA GROUND VCC |

CHAPTER   3

SYSTEM BIOS SETUP

The section will explain how to set up the system configuration (CMOS) under the AWARD BIOS.  The SETUP program is contained in the system’s Read-Only-Memory rather than on a diskette.

3.1	Entering Setup

Power on the computer and press <Del> immediately will allow you to enter Setup.  The other way to enter Setup is to power on the computer, when the below message appears briefly at the bottom of the screen during the (Power On Self Test), press <Del> key or simultaneously press <Ctrl>, <Alt>, and <Esc> keys.

TO ENTER SETUP BEFORE BOOT PRESS <CTRL-ALT-ESC> OR <DEL> KEY

If the message disappears before you respond and you still wish to enter Setup, restart the system to try again by turning it OFF then ON or pressing the "RESET" button on the system case.  You may also restart by simultaneously pressing <Ctrl>, <Alt>, and <Delete> keys.  If you do not press the keys at the correct time and the system does not boot, an error message will be displayed and you will again be asked to,

PRESS <F1> TO CONTINUE, <CTRL-ALT-ESC> OR <DEL> TO ENTER SETUP

3.2	Control Keys

| Up arrow | Move to previous item |
| --- | --- |
| Down arrow | Move to next item |
| Left arrow | Move to the item in the left hand |
| Right arrow | Move to the item in the right hand |
| Esc key | Main Menu -- Quit and not save changes into CMOS Status Page Setup Menu and Option Page Setup Menu -- Exit current page and return to Main Menu |
| PgUp / “+” key | Increase the numeric value or make changes |
| PgDn / “-“ key | Decrease the numeric value or make changes |
| F1 key | General help, only for Status Page Setup Menu and Option Page Setup Menu |
| (Shift)F2 key | Change color from total 16 colors. F2 to select color forward, (Shift) F2 to select color backward |
| F3 key | Reserved |
| F4 key | Reserved |
| F8 key | Reserved |
| F9 key | Reserved |
| F10 key | Save all the CMOS changes, only for Main Menu |

3.3	Getting Help

- Main Menu

The on-line description of the highlighted setup function is displayed at the bottom of the screen.

Status Page Setup Menu/Option Page Setup Menu

Press F1 to pop up a small help window that describes the appropriate keys to use and the possible selections for the highlighted item.  To exit the Help Window press <F1> or <Esc>.

3.4	The Main Menu

Once you enter AWARD BIOS CMOS Setup Utility, the Main Menu will appear on the screen.  The Main Menu allows you to select from ten setup functions and two exit choices.  Use arrow keys to select among the items and press <Enter> to accept or enter the sub-menu.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | HDD LOW LEVEL FORMAT |
| LOAD BIOS DEFAULTS | SAVE & EXIT SETUP |
| LOAD SETUP DEFAULTS | EXIT WITHOUT SAVING |
| Esc : Quit | ¯ ® ¬ : Select Item |
| F10 : Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

- Standard CMOS Setup

This setup page includes all the items in a standard compatible BIOS.

- BIOS Features Setup

This setup page includes all the items of Award special enhanced features.

- Chipset Features Setup

This setup page includes all the items of chipset special features.

- Power Management Setup

This category determines how much power consumption for system after selecting belo items.  Default value is Disable.

- PNP/PCI Configuration Setup

This category specifies the balue (in units of PCI bus clocks) of the latency timer for this PCI bus master and the IRQ level for PCI device.

- Load  BIOS Defaults

BIOS defaults indicates the most appropriate value of the system parameter which the system would be in minimum performance.  The OEM manufacturer may change the defaults through MODBIN before the binary image burn into the ROM.

- Load Setup Defaults

Chipset defaults indicates the values required by the system for the maximum performance.  The OEM manufacturer may change to defaults through MODBIN before the binary image burn into the ROM.

- User  Password

Change, set, or disable password.  It allows you to limit access to the system and Setup, or just to Setup.

- IDE HDD Auto Detection

Automatically configure hard disk parameters.

- Save & Exit Setup

Save CMOS value changes to CMOS and exit setup.

- Exit Without Save

Abandon all CMOS value changes and exit setup.

3.5	Standard CMOS Setup Menu

The items in Standard CMOS Setup Menu are divided into 10 categories.  Each category includes no, one or more than one setup items.  Use the arrow keys to highlight the item and then use the <PgUp> or <PgDn> keys to select the value you want in each item.

ROM PCI/ISA BIOS

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| Date (mm:dd:yy) : Wed, Jun 25 1996 |  |  |  |
| --- | --- | --- | --- |
| Time (hh:mm:ss) : 10:21:21 |  |  |  |
| HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDE SECTOR MODE |  |  |  |
| Primary Master : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Primary Slave : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Secondary Master : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Secondary Slave : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Drive A | : 1.44M, 3.5 in. |  |  |
| Drive B | : None |  |  |
|  |  | Base Memory | : 640K |
| Video | : EGA/VGA | Extended Memory | : 15360K |
| Halt On | : All Errors | Other Memory | : 384K |
|  |  | Total Memory | : 16384K |
| ESC: Quit ¯®¬: Select Item PU / PD /+ /- : Modify |  |  |  |
| F1 : Help (Shift)F2: Change Color |  |  |  |

- Date

The date format is <day>, <date> <month> <year>.  Press <F3> to show the calendar.

| day | The day of week, from Sun to Sat, determined by the BIOS, is read only |
| --- | --- |
| date | The date, from 1 to 31 (or the maximum allowed in the month), can key in the numerical / function key |
| month | The month, Jan through Dec. |
| year | The year, depend on the year of BIOS |

- Time

The time format is <hour> <minute> <second>. which accepts both function key or numerical key The time is calculated based on the 24-hour military-time clock.  For example, 1 p.m. is 13:00:00.

- Drive C Type/Drive D Type

The categories identify the types of hard disk drive C or drive D that have been installed in the computer.  There are 45 predefined types and 2 user definable types are for Normal BIOS..  Type 1 to Type 45 are predefined.  Type User is user-definable.

- Primary Master/Primary Slave/Secondary Master/Secondary Slave

The categories identify the types of 2 channels that have been installed in the computer.  There are 45 predefined types and 4 user definable types are for Enhanced IDE BIOS.  Type 1 to Type 45 are predefined.  Type User is user-definable.

Press PgUp/<+> or PgDn/<-> to select a numbered hard disk type or type the number and press <Enter>.    Note that the specifications of your drive must match with the drive table.  The hard disk will not work properly if you enter improper information for this category.  If your hard disk drive type is not matched or listed, you can use Type User to define your own drive type manually.

If you select Type User, related information is asked to be entered to the following items.  Enter the information directly from the keyboard and press <Enter>.  This information should be provided in the documentation from your hard disk vendor or the system manufacturer.

If the controller of HDD interface is ESDI, the selection shall be “Type 1”.

If the controller of HDD interface is SCSI, the selection shall be “None”.

If the controller of HDD interface is CD-ROM, the selection shall be “None”.

| CYLS. | Number of Cylinders |
| --- | --- |
| HEADS | Nnumber of Heads |
| PRECOMP | Write Precom |
| LANDZONE | Landing Zone |
| SECTORS | Nnumber of Sectors |
| MODE | HDD Aaccess Mode |

If a hard disk has not been installed select NONE and press <Enter>.

- Drive A Type/Drive B Type

The category identifies the types of floppy disk drive A or drive B that have been installed in the computer.

| None | No floppy drive installed |
| --- | --- |
| 360K, 5-1/4 inch | 5-1/4 inch PC-type standard drive; 360 kilobyte capacity |
| 1.2M, 5-1/4 inch | 5-1/4 inch AT-type high-density drive; 1.2 megabyte capacity |
| 720K, 3-1/2 inch | 3-1/2 inch double-sided drive; 720 kilobyte capacity |
| 1.44M, 3-1/2 inch | 3-1/2 inch double-sided drive; 1.44 megabyte capacity |
| 2.88M, 3-1/2 inch | 3-1/2 inch double-sided drive; 2.88 megabyte capacity |

- Video

The category selects the type of adapter used for the primary system monitor that must match your video display card and monitor.  Although secondary monitors are supported, you do not have to select the type in Setup.

You have two ways to boot up the system:

- When VGA as primary and monochrome as secondary, the selection of the video tape is  “VGA Mode”.

- When monochrome as primary and VGA as secondary, the selection of the video type is           “Monochrome mode”.

| EGA/VGA | Enhanced Graphics Adapter/video Graphics Array. For EGA, VGA, SEGA, or PGA monitor adapters. |
| --- | --- |
| CGA 40 | Color Graphics Adapter, power up in 40 column mode |
| CGA 80 | Color Graphics Adapter, power up in 80 column mode |
| MONO | Monochrome adapter, includes high resolution monochrome adapters |

- Error Halt

The category determines whether the computer will stop if an error is detected during power up.

| No errors | Whenever the BIOS detects a non-fatal error the system will be stopped and you will be prompted. |
| --- | --- |
| All errors | The system boot will not be stopped for any error that may be detected. |
| All,But Keyboard | The system boot will not stop for a keyboard error; it will stop for all other errors. |
| All, But Diskette | The system boot will not stop for a disk error; it will stop for all other errors. |
| All, But Disk/Key | The system boot will not stop for a keyboard or disk error; it will stop for all other errors. |

- Memory

The category is display-only which is determined by POST (Power On Self Test) of the BIOS.

Base Memory

The POST of the BIOS will determine the amount of base (or conventional) memory installed in the system.  The value of the base memory is typically 512K for systems with 512K memory installed on the motherboard, or 640K for systems with 640K or more memory installed on the motherboard.

Extended Memory

The BIOS determines how much extended memory is present during the POST.  This is the amount of memory located above 1MB in the CPU's memory address map.

Other Memory

This refers to the memory located in the 640K to 1024K address space.  This is memory that can be used for different applications.  DOS uses this area to load device drivers to keep as much base memory free for application programs.  Most use for this area is Shadow RAM.

- Total Memory

System total memory is the sum of basic memory, extended memory, and other memory.

3.6	BIOS Features Setup Menu

ROM PCI/ISA BIOS

BIOS FEATURES SETUP

AWARD SOFTWARE, INC.

| Virus Warning CPU Internal Cache External Cache Quick Power On Self Test Boot Sequence Swap Floppy Drive Boot Up Floppy Seek Boot Up NumLock Status Boot Up System Speed Gate A20 Option Typematic Rate Setting Typematic Rate (Chars/Sec) Typematic Delay (Msec) Security Option PCI/VGA Palette Snoop OS Select For DRAM > 64MB | : Disabled : Enabled : Enabled : Disabled : A ,C : Disabled : Enabled : On : High : Fast : Disabled : 6 : 250 : Setup : Disabled : Non-OS2 | Video BIOS Shadow C8000-CBFFF Shadow CC000-CFFFF Shadow D0000-D3FFF Shadow D4000-D7FFF Shadow D8000-DBFFF Shadow DC000-DFFFF Shadow | : Enabled : Disabled : Disabled : Disabled : Disabled : Disabled : Disabled |
| --- | --- | --- | --- |
|  |  | ESC : Quit ¯ ® ¬ : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |  |

- Virus Warning

This category flashes on the screen.   During and after the system boots up, any attempt to write to the boot sector or partition table of the hard disk drive will halt the system and the following error message will appear, in the mean time, you can run an anti-virus program to locate the problem.

! WARNING ! Disk boot sector is to be modified Type "Y" to accept write or "N" to abort write Award Software, Inc.

| Enabled | Activates automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector or hard disk partition table. |
| --- | --- |
| Disabled | No warning message to appear when anything attempts to access the boot sector or hard disk partition table. |

Note: This function is available only for DOS and other OSes that do not trap INT13.

- CPU Internal Cache/External Cache

These two categories speed up memory access.  However, it depends on CPU/chipset design.  The default value is Enable.  If your CPU without Internal Cache then this item “CPU Internal Cache” will not be show.

| Enabled | Enable cache |
| --- | --- |
| Disabled | Disable cache |

- Quick Power On Self Test

This category speeds up Power On Self Test (POST) after you power on the computer.  If it is set to Enable, BIOS will shorten or skip some check items during POST.

| Enabled | Enable quick POST |
| --- | --- |
| Disabled | Normal POST |

- Boot Sequence

This category determines which drive computer searches first for the disk operating system (i.e., DOS).  Default value is A,C.

| C,A | System will first search for hard disk drive then floppy disk drive. |
| --- | --- |
| A,C | System will first search for floppy disk drive then hard disk drive. |

Note: This function is only available for IDE type

For SCSI type is always boot from A.

- Boot Up Floppy Seek

During POST, BIOS will determine if the floppy disk drive installed is 40 or 80 tracks.  360K type is 40 tracks while 760K, 1.2M and 1.44M are all 80 tracks.

| Enabled | BIOS searches for floppy disk drive to determine if it is 40 or 80 tracks. Note that BIOS can not tell from 720K, 1.2M or 1.44M drive type as they are all 80 tracks. |
| --- | --- |
| Disabled | BIOS will not search for the type of floppy disk drive by track number. Note that there will not be any warning message if the drive installed is 360K. |

- Boot Up NumLock Status

The default value is On.

| On | Keypad is number keys |
| --- | --- |
| Off | Keypad is arrow keys |

- Boot Up System Speed

It selects the default system speed - the speed that the system will run at immediately after power up.

| High | Set the speed to high |
| --- | --- |
| Low | Set the speed to low |

- IDE HDD Block Mode

| Enabled | Enable IDE HDD Block Mode. The BIOS will detect the block size of the HDD and send block command automatically. |
| --- | --- |
| Disabled | Disable IDE HDD Block Mode |

- Gate A20 Option

| Normal | The A20 signal is controlled by keyboard controller or chipset hardware. |
| --- | --- |
| Fast | Default : Fast. The A20 signal is controlled by Port 92 or chipset specific method. |

- Typematic Rate Setting

This determines the typematic rate.

| Enabled | Enable typematic rate and typematic delay programming |
| --- | --- |
| Disabled | Disable typematic rate and typematic delay programming. The system BIOS will use default value of this 2 items and the default is controlled by keyboard. |

- Typematic Rate (Chars/Sec)

| 6 | 6 characters per second |
| --- | --- |
| 8 | 8 characters per second |
| 10 | 10 characters per second |
| 12 | 12 characters per second |
| 15 | 15 characters per second |
| 20 | 20 characters per second |
| 24 | 24 characters per second |
| 30 | 30 characters per second |

- Typematic Delay (Msec)

When holding a key, the time between the first and second character displayed.

| 250 | 250 msec |
| --- | --- |
| 500 | 500 msec |
| 750 | 750 msec |
| 1000 | 1000 msec |

- Security Option

This category allows you to limit access to the system and Setup, or just to Setup.

| System | The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt. |
| --- | --- |
| Setup | The system will boot, but access to Setup will be denied if the correct password is not entered at the prompt. |

Note:	To disable security, select PASSWORD SETTING at Main Menu and then you will be asked to enter password.  Do not type anything and just press <Enter>, it will disable security.  Once the security is disabled, the system will boot and you can enter Setup freely.

- System BIOS Shadow

It determines whether system BIOS will be copied to RAM or the system BIOS is always shadow to support LBA HDD.

| Enabled | System shadow is enabled |
| --- | --- |
| Disabled | System shadow is disabled |

- Video BIOS Shadow

It determines whether video BIOS will be copied to RAM, however, it is optional from chipset design.  Video Shadow will increase the video speed.

| Enabled | Video shadow is enabled |
| --- | --- |
| Disabled | Video shadow is disabled |

3.7   Chipset Features Setup

ROM PCI/ISA BIOS

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Auto Configuration | : Enabled | Memory Parity/ECC Check |  |  | : Auto |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAM Timing | : 70 ns | Single Bit Error Report |  |  | : Enabled |  |  |  |
| DRAM RAS# Precharge Time | : 4 | L2 Cache Cacheable Size |  |  | : 64MB |  |  |  |
| DRAM R/W Leadoff Timing | : 7/6 | Chipset NA# Asserted |  |  | : Enabled |  |  |  |
| Fast RAS# To CAS# Delay | : 3 | Pipline Cache Timing |  |  | : Faster |  |  |  |
| DRAM Read Burst (EDO/FPM) | : x333/x444 |  |  |  |  |  |  |  |
| DRAM Write Burst Timing | : x333 |  |  |  |  |  |  |  |
| Trubo Read Leadoff | : Disabled |  |  |  |  |  |  |  |
| DRAM Speculative Leadoff | : Disabled |  |  |  |  |  |  |  |
| Turn-Around Insertion | : Disabled |  |  |  |  |  |  |  |
| ISA Clock | : PCICLK/3 |  |  |  |  |  |  |  |
| System BIOS Cacheable | : Disabled |  |  |  |  |  |  |  |
| Video BIOS Cacheable | : Disabled |  |  |  |  |  |  |  |
| 8 Bit I/O Recovery Time | : 1 |  |  |  |  |  |  |  |
| 16 Bit I/O Recovery Time | : 1 | ESC | : Quit | ¯®¬ |  |  | : Select |  |
| Memory Hole At 15M-16M | : Disabled | F1 | : Help | PU/PD/+/- |  |  | : Modify |  |
| Peer Concurrency | : Enabled | F5 | : Old Values | (Shift)F2 |  |  | : Color |  |
| Chipset Special Features | : Enabled | F6 | : Load BIOS Defaults |  |  |  |  |  |
| DRAM ECC/PARITY Select | : Parity | F7 | : Load Setup Defaults |  |  |  |  |  |

3.8	Power Management Setup

The Power management setup will appear on your screen like this:

ROM PCI/ISA BIOS

POWER MANAGEMENT SETUP

AWARD SOFTWARE, INC.

| Power Management | : Disable | ** Power Down & Resume Events ** |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PM Control by APM | : Yes | IRQ3 |  | (COM2) |  | : ON |  |  |  |
| Video Off Method | : V/H SYNC+ Blank | IRQ4 |  | (COM1) |  | : ON |  |  |  |
| MODEM Use IRQ | : 3 | IRQ5 |  | (LPT2) |  | : ON |  |  |  |
|  |  | IRQ6 |  | (Floppy Disk) |  | : ON |  |  |  |
| Doze Mode | : Disable | IRQ7 |  | (LPT1) |  | : ON |  |  |  |
|  |  | IRQ8 |  | (RTC Alarm) |  | : OFF |  |  |  |
| Suspend Mode | : Disable | IRQ9 |  | (IRQ2 Redir) |  | : ON |  |  |  |
| HDD Power Down | : Disable | IRQ10 |  | (Reserved) |  | : ON |  |  |  |
|  |  | IRQ11 |  | (Reserved) |  | : ON |  |  |  |
| ** Wake Up Events In Doze & Standby ** |  | IRQ12 |  | (PS/2 Mouse) |  | : ON |  |  |  |
| IRQ3 (Wake-up Event) | : ON | IRQ13 |  | (Coprocessor) |  | : ON |  |  |  |
| IRQ4 (Wake-up Event) | : ON | IRQ14 |  | (Hard Disk) |  | : ON |  |  |  |
| IRQ8 (Wake-up Event) | : OFF | IRQ15 |  | (Reserved) |  | : ON |  |  |  |
| IRQ12 (Wake-up Event) | : ON |  |  |  |  |  |  |  |  |
|  |  | ESC | : Quit |  | ¯®¬ |  |  | : Select |  |
|  |  | F1 | : Help |  | PU / PD / + / - |  |  | : Modify |  |
|  |  | F5 | : Old Values |  | (Shift)F2 |  |  | : Color |  |
|  |  | F6 | : Load BIOS Defaults |  |  |  |  |  |  |
|  |  | F7 | : Load Setup Defaults |  |  |  |  |  |  |

- Power Management

This category determines how much power consumption for system after selecting below items.  Default value is Disable.  The following pages tell you the options of each item & describe the meanings of each options.

| Item | Options | Descriptions |
| --- | --- | --- |
| A. Power Management | 1. Disable | Global Power Management will be disabled |
|  | 2. User Define | Users can configure their own power management |
|  | 3. Min Saving | Pre-defined timer values are used such that all timers are in their MAX value |
|  | 4. Max Saving | Pre-defined timer values are used such that all timers MIN value |
| B. PM Control by APM | 1. No | System BIOS will ignore APM when power managing the system |
|  | 2. Yes | System BIOS will wait for APM’s prompt before it enter any PM mode e.g. DOZE, STANDBY or SUSPEND Note: If APM is installed, & if there is a task running, even the timer is time out, the APM will not prompt the BIOS to put the system into any power saving mode! |
|  |  | Note: - if APM is no installed, this option has no effect |
| C. Video Off Option | 1. Always On | System BIOS will never turn off the screen |
|  | 2. Suspend -> Off | Screen off when system is in SUSPEND mode |
|  | 3. Susp, Stby -> Off | Screen off when system is in STANDBY or SUSPEND mode |
|  | 4. All Modes -> Off | Screen off when system is in DOZE, STANDBY or SUSPEND mode |
|  |  | Note: The M/B markers are recommended to fix this item to (2) or (3) & hidden it by using MODBIN Utility |

3.9	PNP/PCI Configuration Setup

You can manually configurate the PCI Device’s IRQ. The following pages tell you the options of each item & describe the meanings of each options.

ROM PCI/ISA BIOS

PNP/PCI CONFIGURATION SETUP

AWARD SOFTWARE, INC.

| Resources Controlled By |  | : Manual | PCI IRQ Actived BY |  |  | : Level |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset Configuration Data |  | : Disabled | PCI IDE IRQ Map To |  |  | : PCI-AUTO |  |  |
|  |  |  | Primary IDE INT# |  |  | : A |  |  |
| IRQ-3 | assigned to | : Legacy ISA | Secondary IDE INT# |  |  | : B |  |  |
| IRQ-4 | assigned to | : Legacy ISA |  |  |  |  |  |  |
| IRQ-5 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-7 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-9 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-10 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-11 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-12 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-14 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-15 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| DMA-1 | assigned to | : PCI/ISA PnP | ESC | : Quit | ¯®¬ |  |  | : Select Item |
| DMA-3 | assigned to | : PCI/ISA PnP | F1 | : Help | PU/PD/+/ - |  |  | : Modify |
| DMA-5 | assigned to | : PCI/ISA PnP | F5 | : Old Values | (Shift)F2 |  |  | : Color |
| DMA-7 | assigned to | : PCI/ISA PnP | F6 | : Load BIOS Defaults |  |  |  |  |
|  |  |  | F7 | : Load Setup Defaults |  |  |  |  |

3.10	INTEGRATED PERIPHERALS

ROM PCI/ISA BIOS

INTEGRATED PERIPHERALS

AWARD SOFTWARE, INC.

| IDE HDD Block MODE | : Enabled |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PCI Slot IDE 2nd Channel | : Enabled |  |  |  |  |  |  |
| On-Chip Primary PCI IDE | : Enabled |  |  |  |  |  |  |
| On-Chip Secondary PCI IDE | : Enabled |  |  |  |  |  |  |
| IDE Primary Master PIO | : Auto |  |  |  |  |  |  |
| IDE Primary Slave PIO | : Auto |  |  |  |  |  |  |
| IDE Secondary Master PIO | : Auto |  |  |  |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |  |  |  |
| USB Controller | : Enabled |  |  |  |  |  |  |
| Onboard FDD Controller | : Enabled |  |  |  |  |  |  |
| Onboard Serial Port 1 | : COM1/3F8 |  |  |  |  |  |  |
| Onboard Serial Port 2 | : COM2/2F8 |  |  |  |  |  |  |
| Onboard Parallel Port | : 378H |  |  |  |  |  |  |
| Onboard Parallel Mode | : ECP |  |  |  |  |  |  |
| Onboard Game Port | : Enabled | ESC | : Quit | ¯®¬ |  |  | : Select |
| Serial Port 1 MIDI | : Disabled | F1 | : Help | PU/PD/+/- |  |  | : Modify |
| Serial Port 2 MIDI | : Disabled | F5 | : Old Values | (Shift)F2 |  |  | : Color |
|  |  | F6 | : Load BIOS Defaults |  |  |  |  |
|  |  | F7 | : Load Setup Defaults |  |  |  |  |

3.11	Password Setting

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP |  |  |  | INTEGRATED PERIPHERALS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BIOS FEATURES SETUP |  |  |  | SUPERVISOR PASSWORD |  |  |
| CHIPSET FEATURES SETUP |  |  |  | USER PASSWORD |  |  |
| POWER MANAGEMENT SETUP |  |  |  | IDE HDD AUTO DETECTION |  |  |
| PNP/PCI CONFIRURATION |  |  |  | HDD LOW LEVEL FORMAT |  |  |
| LOAD BIOS DEFAULT |  | Enter Password: |  |  |  | T SETUP |
| LOAD SETUP DEFAULTS |  |  | EXIT WITHOUT SAVING |  |  |  |
| Esc | : Quit |  |  | ¯®¬ | : Select Item |  |
| F10 | : Save & Exit Setup |  |  | (Shift)F2 | : Change Color |  |
| Change /set /Disable Password |  |  |  |  |  |  |

When you select this function, the following message will appear at the center of the screen to assist you in creating a password.

ENTER PASSWORD:

Type the password, up to eight characters, and press <Enter>.  The password typed now will clear any previously entered password from CMOS memory.  You will be asked to confirm the password.  Type the password again and press <Enter>.  You may also press <Esc> to abort the selection and not enter a password.

To disable password, just press <Enter> when you are prompted to enter password.  A message will confirm the password being disabled.  Once the password is disabled, the system will boot and you can enter Setup freely.

PASSWORD DISABLED.

If you select System at Security Option of BIOS Features Setup Menu, you will be prompted for the password every time the system is rebooted or any time you try to enter Setup.  If you select Setup at Security Option of BIOS Features Setup Menu, you will be prompted only when you try to enter Setup.

3.12	IDE HDD Auto Detection

The Enhance IDE features was included in all Award BIOS.  Below is a brief description of this feature.

1. Setup Changes

<I> Auto-detection

- BIOS setup will display all possible modes that supported by the HDD including NORMAL, LBA & LARGE.

- If HDD does not support LBA modes, no ‘LBA’option will be shown.

- If no of cylinders is less than or equal to 1024, no ‘LARGE’ option will be show.

- Users can select a mode which is appropriate for them.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| Select Primary Slave Option (N = Skip ) : N OPTIONS SIZE CYLS HEADS PRECOMP LANDZ SECTOR MODE 1(Y) 0 0 0 0 0 0 NORMAL Note: Some OSes (like SCO-UNIX) must use “NORMAL “ for installation HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE Primary Master : 1282 621 64 0 2483 63 LBA Primary Slave : |  |  |
| --- | --- | --- |
| ESC: Skip |  |  |

RMA  FORM

When the motherboard can not work well, please fill up this form to describe related

situations.  If the space is not enough to use, you can attach separate paper.

MODEL:                                                        MODEL NO:

HARDWARE

CPU :  Brand                               , Model                              , Speed                                    MHz

CD-PROCESSOR:  Brand                               , Model                               , Speed                            MHz

SIMM: Brand                         , Speed                           ns, Q’ty                        pcs,Total                   MB

CACHE: Brand                               , Speed                                ns,Total                              K

TAG RAM: Brand                              , Speed                              ns

BIOS DATA CODE:

SYSTEM SPEED RUNNING                                 MHz

VIDEO CARD:   Chip                            , RAM                            , VGA Mode

Bus                                     (ISA,VESA or PCI)

OTHER ADD-ON CARDS:

SOFTWARE

OPERATING SYSTEM                                                 VERSION

SOFTWARE PROGRAM

BIOS SETUP: DRAM Wait State                                     CACHE Wait State

If you change BIOS SETUP, please describe the changes:

<A>ERROR

¨HANG UP                             ¨NO SCREEN                             ¨FLOPPY R/W ERROR

¨HARD DISK R/W ERROR      ¨PARITY MEMORY ERROR

¨OTHER

<B>ERROR MESSAGES ON YOUR SCREEN (PLEASE SHOW US THE WHOLE  SENTENCE)

<C>PROBLEM DESCRIPTION