451-011200400

REVISION:D

EXP8661

TABLE OF CONTENTS

CHAPTER  1  INTRODUCTION	1

1.1 OVERVIEW	1

1.2 SYSTEM FEATURES	1

1.3 SYSTEM SPECIFICATION	2

1.4 SYSTEM PERFORMANCE	2

1.5 EXP8661 BOARD LAYOUT	3

CHAPTER  2  INSTALLATION	4

2.1 DRAM INSTALLATION	4

2.2 SRAM INSTALLATION	6

2.3 CPU TYPE JUMPER SETTING	7

2.4 CPU I/O & CPU CORE VOLTAGE SELECT	8

2.5 FLASH ROM INSTALLATION	9

2.6 EPP MODE SETTING	10

2.7 OTHER JUMPER & CONNECTOR INSTALLATION	11

CHAPTER  3  SYSTEM BIOS SETUP	14

3.1 SYSTEM SETUP	16

3.2 SECURITY SETUP	22

3.3 DEFAULT SETUP	25

CHAPTER  4  SYSTEM AWARD BIOS SETUP	29

4.1    ENTERING SETUP	29

4.2    CONTROL KEYS	29

4.3    GETTING HELP	30

4.4    THE MAIN MENU	30

4.5    STANDARD CMOS SETUP MENU	32

4.6    BIOS FEATURES SETUP	35

4.7    CHIPSET FEATURES SETUP	38

4.8    POWER MANAGEMENT	39

4.9    PNP/PCI CONFIGURATION SETUP	41

4.10  INTEGRATED PERIPHERALS	42

4.11  PASSWORD SETTING	43

4.12  IDE HDD AUTO DETECTION	44

RMA FORM

CHAPTER   1

INTRODUCTION

1.1    OVERVIEW

The EXP8661 motherboard is complemented by a maximum 512K L2 Write-Back cache, providing workstation level computing performance. SIMM and DIMM sockets support up to 128MB of DRAM.

The EXP8661 motherboard offers the outstanding I/O capabilities.  Three PCI Local Bus slots provide a high bandwidth data path for data-movement intensive functions such as graphics.  Four ISA slots complete the I/O mix.

The EXP8661 motherboard provides the foundation for cost effective, high  performance, and highly expandable platforms which deliver the latest CPU and I/O technologies.

1.2    SYSTEM FEATURES

The EXP8661 motherboard supports the following features:

q  INTEL PENTIUM 75/90/100/120/133/150/166/200 MHz CPU,

CYRIX 6x86-120+(100MHz)/ 6x86-133+(110MHz)/6x86-150+(120MHz)/6x86-166+(133MHz)

AMD 5K86-P75/5K86-P90/5K86-P100/K5-PR120(90MHz)/K5-PR133(100MHz)/K5-PR166 CPU

q  3 MASTER 32-bit PCI Bus

q  L1/L2 Write Back/Write Through Cache

q  256KB/512KB cache cize

q  72-pin SIMM Modules and 168-pin DIMM Module

q  2 Serial/1 Parallel/1 FDC on board

q  2-CHANNEL PCI IDE on board

q  MESI (Modified Exclusive Shared Invalid) protocol to maintain the data coherence for L2 Cache to optimize CPU bus.

1.3    SYSTEM SPECIFICATIONS

Processor:	INTEL PENTIUM 75/90/100/120/133/150/166/200 MHz CPU

CYRIX 6x86-120+(100MHz)/6x86-133+(110MHz)/

6x86-150+(120MHz)/ 6x86-166+(133MHz) CPU; AMD 5K86-P75/

5K86-P90/5K86-P100/K5-PR120/K5-PR133/K5-PR166 CPU

CPU Clock Speed:	50/55/60/66 MHz

Memory:	8MB to 128MB

SRAM:	256K/512K

BIOS type:	AMI/AWARD BIOS

Additional BIOS Feature:	Set Program Resides in ROM

Slot type:	Four 16-bit ISA Bus

Three 32-bit PCI Bus

Dimension:	28x22 cm

Additional Features

Miscellaneous Connectors: 	Reset button, Suspend button

Board Design: 	4-layer Implementation for Low noise operation

1.4    SYSTEM  PERFORMANCE

| SOFTWARE CPU TYPE | LANDMARK V2.0 | POWER METER V1.8 MIPS | NORTON V8.0 CPU SPEED |
| --- | --- | --- | --- |
| PENTIUM 75 | 433.85 MHz | 36.9 MIPS | 238.2 |
| PENTIUM 90 | 526.85 MHz | 45.1 MIPS | 289.2 |
| PENTIUM 100 | 578.51 MHz | 49.9 MIPS | 317.6 |
| PENTIUM 120 | 702.43 MHz | 61.8 MIPS | 385.6 |
| PENTIUM 133 | 771.33 MHz | 67.7 MIPS | 423.5 |
| PENTIUM 150 | 878.06 MHz | 76.9 MIPS | 482.1 |
| PENTIUM 166 | 964.19 MHz | 83.6 MIPS | 529.3 |
| PENTIUM 200 | 1157.04 MHz | 94.8 MIPS | 635.2 |
| CYRIX 6x86-120+ (100MHZ) | 1209.95 MHz | 67.7 MIPS | 680.3 |
| CYRIX 6x86-133+ (110MHZ) | 1353.98 MHz | 74.8 MIPS | 761.2 |
| CYRIX 6x86-150+ (120MHZ) | 1469.25 MHz | 81.3 MIPS | 826 |
| CYRIX 6x86-166+ (133MHZ) | 1613.31 MHz | 84.3 MIPS | 907 |
| AMD 5K86-P75 | 673.26 MHz | 39.0 MIPS | 297.7 |
| AMD 5K86-P90 | 817.55 MHz | 47.4 MIPS | 361.5 |
| AMD 5K86-P100 | 897.68 MHz | 53.7 MIPS | 397 |
| AMD K5-PR120 (90MHz) | 817.54 MHz | 59.2 MIPS | 394.3 |
| AMD K5-PR133 (100MHz) | 897.72 MHz | 64.6 MIPS | 433 |
| AMD K5-PR166 | 1047.35 MHz | 76.9 MIPS | 505.1 |

USB CONNECTOR

1.5    EXP8661 BOARD LAYOUT

JP9

82371

3

1

82371SB

1               5

JP14

IDE_LED

JP16

JP10

CPU  SOCKET

TAG

PCI3

PCI2

1     3

JP22

J26

W83768

IDE1   IDE2

FDC

SIMM1 SIMM2  SIMM3  SIMM4

27010

U11

PCI1

J13

JP12

JP6

JP7

JP18

3     1

1      3

JPA1

PRN

JP19

BANK0   BANK0  BANK1  BANK1

39  40  39  40 1 1

33 34

25 26

1    2

1    2

1     2    1    2

JPA2

JP3

3

1

JP4

JP21

1           4

JP2

U25

U24

U26

J22

DS12887

KEYBOARD BIOS

1

4

MS1

J3

1   1

4   4

J2

U9

JP5

JP8

JP9

1           4

RESET  SPEAKER      KEYLOCK

JP23     J25           J24

9   10   9  10

1     2   1    2

J18    J12

COM1 COM2

82438VX

82438VX

82437VX

PB SRAM

PB SRAM

POWER

CONNECTOR

U6

1      3

JP15

1      3

1      3

KEYBOARD

JP13

BANK0

DIM1

U12

J21

IR CONNECTOR

1  3  5  7  9

J1

CHAPTER   2

INSTALLATION

Before the system is ready to operate, the hardware must be set up for various functions of the system.  To set up the EXP8661 motherboard is a simple task.  The user only has to set a few jumpers, connectors and sockets.

2.1    DRAM INSTALLATION

The EXP8661 motherboard can support expanded memory from 2MB to 128MB.

nThe board layout below shows the locations of the DRAM memory banks:

BANK0

BANK0 BANK0  BANK1   BANK1

SIMM1  SIMM2  SIMM3  SIMM4

DIM1

DRAM  INSTALLATION

SIMM1+SIMM2

SIMM3+SIMM4

TOTAL

DIM1

+                         or         =

Each group includes two SIMMs each SIMM size can be 1, 2, 4, 8,16, 32MB, please install the same DRAM size in one group.

n  TABLE 1 (SIMM)

| BANK 0 |  | BANK 1 |  |  |
| --- | --- | --- | --- | --- |
| SIMM1 | SIMM2 | SIMM3 | SIMM4 | TOTAL MEMORY |
| 4MB | 4MB | None | None | 8MB |
| 4MB | 4MB | 4MB | 4MB | 16MB |
| 8MB | 8MB | None | None | 16MB |
| 4MB | 4MB | 8MB | 8MB | 24MB |
| 8MB | 8MB | 4MB | 4MB | 24MB |
| 8MB | 8MB | 8MB | 8MB | 32MB |
| 16MB | 16MB | None | None | 32MB |
| 4MB | 4MB | 16MB | 16MB | 40MB |
| 16MB | 16MB | 4MB | 4MB | 40MB |
| 8MB | 8MB | 16MB | 16MB | 48MB |
| 16MB | 16MB | 8MB | 8MB | 48MB |
| 16MB | 16MB | 16MB | 16MB | 64MB |
| 32MB | 32MB | None | None | 64MB |
| 4MB | 4MB | 32MB | 32MB | 72MB |
| 32MB | 32MB | 4MB | 4MB | 72MB |
| 8MB | 8MB | 32MB | 32MB | 80MB |
| 32MB | 32MB | 8MB | 8MB | 80MB |
| 16MB | 16MB | 32MB | 32MB | 96MB |
| 32MB | 32MB | 16MB | 16MB | 96MB |
| 32MB | 32MB | 32MB | 32MB | 128MB |

n  TABLE 2  (DIMM)

| DIM1 (BANK0) | TOTAL MEMORY |
| --- | --- |
| 8MB | 8MB |
| 16MB | 16MB |

Note :DIMM and SIMM can’t install together.

2.2    SRAM  INSTALLATION

U9

J22

27010

U11

JP5

82438VX

82438VX

JP6

JP7

DS12887

JP8

U12

JP9

BANK1

BANK0

JP10

1     3

JP12

82437VX

TAG

1      3

JPA1

JP15

CPU  SOCKET

JPA2

1      3

JP13

U25

U24

U26

JP16

PB SRAM

PB SRAM

JP14

JP22

3     1

3

1

JP21

IDE_LED

J26

JP19

1           4

JP23     J25           J24

1               5

JP18

1           4

RESET  SPEAKER      KEYLOCK

CACHE SIZE CONFIGURATION

| NON CACHE |  | §256K |  |
| --- | --- | --- | --- |
|  |  | TAG RAM | DATA RAM |
|  |  | U26 8KX8,16KX8, 32KX8 | U24,U25 32KX32 |
| 3 2 1 · · JP21 JP15 1 2 3 JP13 JP13 1 2 3 JP15 JP21 3 2 1 · · |  |  |  |

| 512K |  | 512K (on Board) |  |
| --- | --- | --- | --- |
| TAG RAM | DATA RAM | TAG RAM | DATA RAM |
| U26 16KX8, 32KX8 | U24, U25, J22 32KX32 | U26 16KX8, 32KX8 | U24, U25 64KX32 |
| 3 2 1 · · JP21 JP15 · · 1 2 3 JP13 · · 3 2 1 · · JP21 JP15 · · 1 2 3 JP13 |  |  |  |

§ Default setting

2.3    CPU TYPE JUMPER SETTING

| CPU TYPE | JUMPER SETTING |
| --- | --- |
| JP19 JP18 JP7 · · · · JP6 JP5 AMD 5K86-P75/PENTIUM 75MHz |  |
| JP19 JP18 JP7 · · JP6 JP5 AMD 5K86-P90/AMD K5-PR120/ PENTIUM 90MHz |  |
| JP19 JP18 JP7 JP6 JP5 AMD 5K86-P100/AMD K5-PR133/ PENTIUM 100MHz |  |
| JP19 JP18 JP7 · · · · JP6 JP5 INTEL PENTIUM 120MHz |  |
| JP19 JP18 JP7 · · JP6 JP5 INTEL PENTIUM 133MHz |  |
| JP19 JP18 JP7 · · · · · · JP6 JP5 INTEL PENTIUM 150MHz |  |
| AMD K5-PR1 JP19 JP18 JP7 · · · · JP6 JP5 66/PENTIUM 166MHz |  |
| JP19 JP18 JP7 · · JP6 JP5 INTEL PENTIUM 200MHz |  |
| JP19 JP18 JP7 · · · · · · JP6 JP5 CYRIX 6x86-120+(100MHz) |  |
| JP19 JP18 JP7 JP6 JP5 · · · · CYRIX 6x86-133+(110MHz) |  |
| JP19 JP18 JP7 · · · · JP6 JP5 CYRIX 6x86-150+(120MHz) |  |
| JP19 JP18 JP7 · · JP6 JP5 CYRIX 6x86-166+(133MHz) |  |

2.4    CPU I/O & CPU CORE VOLTAGE SELECT

J22

U9

U11

82438VX

82438VX

27010

JP6

JP5

JP7

DS12887

JP8

U12

JP9

JP10

1     3

JP12

82437VX

TAG

JPA1

1      3

1      3

JP15

CPU  SOCKET

JPA2

JP13

U25

U24

U26

JP14

JP16

PB SRAM

PB SRAM

JP22

3     1

3

1

JP21

1               5

IDE_LED

J26

JP18

JP19

1           4

1           4

JP23     J25           J24

RESET  SPEAKER      KEYLOCK

CPU I/O VOLTAGE SELECT

The jumper setting is only for Dual Voltage CPU (INTEL P55C etc..)

| 3.3V | 3.45V | §Normal CPU (Single Voltage) |
| --- | --- | --- |
| · · 3 2 1 JP22 · · 3 2 1 JP22 3 2 1 JP22 |  |  |

CPU CORE VOLTAGE SELECT

| 2.9V | §3.3V | 3.45V~3.6V |
| --- | --- | --- |
| JPA1 · · JPA2 1 2 3 JPA1 JPA2 · · 1 2 3 JPA2 JPA1 · · 1 2 3 |  |  |

§ Default setting

2.5    FLASH ROM INSTALLATION

J22

U9

U11

JP5

82438VX

82438VX

27010

JP6

JP7

DS12887

JP8

U12

JP9

JP10

1     3

JP12

82437VX

TAG

JPA1

CPU  SOCKET

1      3

1      3

JP15

JPA2

JP13

U25

U24

U26

JP14

JP16

JP22

PB SRAM

PB SRAM

IDE_LED

3     1

JP19

3

1

JP21

1               5

J26

JP18

1           4

1           4

RESET  SPEAKER      KEYLOCK

JP23     J25           J24

| §5V FLASH ROM | 12V FLASH ROM |
| --- | --- |
| JP8 JP8 3 2 1 3 2 1 · · · · · · 1 2 3 JP10 · · 1 2 3 JP10 |  |

§ Default Setting

3

2

1

2.6    EPP MODE SETTING

USB CONNECTOR

1

4

1   1

4   4

KEYBOARD BIOS

MS1

J3

J2

KEYBOARD

IR CONNECTOR

1  3  5  7  9

J1

JP4

1     2   1    2

COM1 COM2

BANK0

W83768

BANK0   BANK0  BANK1  BANK1

JP3

J18    J12

1      3

1      3

1    2

JP2

9   10   9  10

J13

PRN

POWER

CONNECTOR

25 26

1    2

1     2    1    2

FDC

J21

PCI3

PCI2

IDE1   IDE2

PCI1

33 34

U6

82371SB

SIMM1 SIMM2  SIMM3  SIMM4

39  40  39  40 1 1

DIM1

DMA CHANNEL CONFIGURATION (FOR EPP DEVICE)

| DMA1 | DMA3 |
| --- | --- |
| JP2 JP3 1 2 3 · · · · JP3 JP2 · · · · 1 2 3 |  |

2.7    OTHER JUMPER & CONNECTOR INSTALLATION

USB CONNECTOR

1

4

MS1

1   1

4   4

J2

KEYBOARD

1  3  5  7  9

J1

KEYBOARD BIOS

J3

IR CONNECTOR

JP4

1     2   1    2

COM1 COM2

W83768

J18    J12

9   10   9  10

1    2

J13

PRN

POWER

CONNECTOR

25 26

1    2

1     2    1    2

FDC

J21

PCI3

PCI2

IDE1   IDE2

PCI1

33 34

U6

82371SB

39  40  39  40 1 1

U9

DIM1

U11

82438VX

82438VX

27010

DS12887

JP8

U12

JP9

JP12

1               5

JP14

IDE_LED

JP16

CPU  SOCKET

TAG

J26

1           4

U26

1           4

RESET  SPEAKER      KEYLOCK

JP23     J25           J24

82437VX

OTHER JUMPER DESCRIPTION

| JUMPER | DESCRIPTION |  |
| --- | --- | --- |
| · · JP4 | FOR PARALLEL PORT EPP MODE | FOR STANDARD PARALLEL PORT § |
| · · JP8 | AT BUS CLOCK EQUAL PCICLK/3 (FOR FREQUENCY 50MHZ) | AT BUS CLOCK PCICLK/4 § ( FOR FREQUENCY 60, 66MHZ) |
| · · JP9 | NORMAL MODE | SMI MODE |
| · · JP12 | NORMAL | CLEAR CMOS |

§ Default Setting

CONNECTOR DESCRIPTION

| CONNECTOR | PIN OUT | SIGNAL NAME |
| --- | --- | --- |
| J1 IR CONNECTOR | 1 2 3 4 5 | VCC NC IRRX GROUND IRTX |
| J2, J3 USB CONNECTOR | 1 2 3 4 | +5V DC DATA OUT DATA OUT GROUND |
| JP23 RESET | 1 2 | GROUND RESET IN |
| J24 KEYLOCK CONNECTOR | 1 2 3 4 5 | VCC NC GROUND KEYBOARD INHIBITOR GROUND |
| J25 SPEAKER CONNECTOR | 1 2 3 4 | DATA LINE NC GROUND +5V DC |
| J26 IDE_LED | 1 2 3 4 | +5V DC DATA OUT DATA OUT +5V DC |
| MS1 PS/2 MOUSE CONNECTOR | 1 2 3 4 | DATA DATA GROUND VCC |

%  JP9: EXTERNAL POWER SAVING CONNECTOR.

CHAPTER   3

SYSTEM AMI BIOS SETUP

WinBIOS Setup can be accessed via keyboard, mouse, or pen.  The mouse click functions are:

·   single click change or select both global and current fields and

·   double click to perform an operation in the selected field.

Using the keyboard with WinBIOS Setup

WinBIOS Setup has a built-in keyboard driver that uses simple keystroke combinations:

| KEYSTROKE | FUNCTION |
| --- | --- |
| <Tab> | Move to the next window or field. |
| ®,¬,,¯ | Move to the next field to the right, left, above, or below. |
| <Enter> | Select in the current field. |
| + | Increments a value. |
| - | Decrements a value. |
| <Esc> | Closes the current operation and return to previous level. |
| <PgUp> | Returns to the previous page. |
| <PgDn> | Advances to the next page. |
| <Home> | Returns to the beginning of the text. |
| <End> | Advances to the end of the text. |
| <Alt>,<H> | Access a help window. |
| <Alt>,<Spacebar> | Exit WinBIOS Setup. |
| Alphabetic keys | A to Z are used in the Virtual Keyboard, and are not case sensitive. |
| Numeric Keys | 0 to 9 are used in the Virtual Keyboard and Numeric Keypad. |

The hardware features and options of the EXP8661 are on-site selectable for maximum flexibility.  You will need to configure these options through the built-in Setup Utility prior to using EXP8661 for the first time.  This setup Utility is a multi-screen, menu driven program and is contained within the BIOS EPROM.

The  following  sections  show  the  procedures  that  you  may  need  to  configure  the  EXP8661:

1.   Press<DEL>while turning on or rebooting the system to invoke Setup Utility

program.

2.   The main menu will be shown as follows:

Figure 1

3.   The functions are grouped into four categories which are Setup, Utility, Security and

Default.

4.   By using <TAB>key or mouse cursor to select the function group.

5.   Use arrow keys or mouse to select  the  function icon  within  the group.  Then press

<Enter>key to invoke the setup function.

6.   Use<Esc>key to go back to the previous screen.

3.1    SYSTEM  SETUP

3.1.1	Advanced Setup

Optimal Setting

You can load the optimal default settings for the WINBIOS by pressing the “END” key upon power-up. The Optimal default settings are best-case values that should optimize system performance. The Optimal settings will automatically detect and load  the parameter of  hard disk type.

### Advanced Setup

Advanced Setup options are displayed by choosing the Advanced icon from the WINBIOS Setup main menu. All Advanced Setup options are described in this section.

Quick Boot

Set this option to Enabled to instruct AMIBIOS to boot quickly when the computer is powered on. This option replaces the old Above 1 MB Memory Test Advanced Setup option. The settings are:

| Setting | Description |
| --- | --- |
| Disabled | AMIBIOS test all system memory. AMIBIOS waits up to 40 seconds for a READY signal from the IDE hard disk drive. AMIBIOS waits for .5 seconds after sending a RESET signal to the IDE drive to allow the IDE drive time to get ready again. AMIBIOS checks for a <Del> key press and runs WINBIOS Setup if the key has been pressed. |
| Enabled | AMIBIOS does not test system memory above 1 MB. AMIBIOS does not wait up to 40 seconds for a READY signal from the IDE hard disk drive. If a READY signal is not received immediately from the IDE drive, AMIBIOS does not configure that drive. AMIBIOS does not wait for .5 seconds after sending a RESET signal to the IDE drive to allow the IDE drive time to get ready again. You cannot run WINBIOS Setup at system boot, because there is no delay for the Hit <Del> to run Setup message. |

The Optimal and Fail-Safe default settings are Enabled.

OS/2 Compatible Mode

Set this option to Enabled to permit AMIBIOS to run with IBM OS/2. The settpings are Enabled or Disabled. The default settings are Disabled.

System BIOS Cacheable

When this option is set to Enabled, the contents of the F0000h system memory segment can be read from or written to L2 secondary cache memory. The contents of the F0000h memory segment are always copied from the BIOS ROM to system RAM for faster execution.

The settings are Enabled or Disabled. The Optimal default setting is Enabled. The Fail-Safe default setting is Disabled.

### 3.1.2    Power Management Setup

Power Management Setup options are displayed by choosing the Power Mgmt icon from the WINBIOS Setup main menu. All Power Management Setup options are described in this section.

Power Management/APM

Set this option to Enabled to enable the power management and APM Advanced Power Management) features. The settings are Enabled or Disabled. The default settings are Disabled.

Instant On Timeout

Set this option to Enabled to allow the computer to go to full power on mode when leaving a power-conserving state. This option is only available if supported by the computer hardware. AMIBIOS uses the RTC Alarm function to wake the computer at a prespecified time. The settings are Enabled or Disabled. The default settings are Disabled.

Green PC Monitor Power State

This option specifies the power management state that the Green PC-compliant video monitor enters after the specified period of display inactivity has expired. The settings are Disabled, Off, Standby, or Suspend. The default settings are Disabled.

Video Power Down Mode

This option specifies the power management state that the video subsystem enters after the specified period of display inactivity has expired. The settings are Disabled, Standby, or Suspend. The default settings are Disabled.

Hard Disk Power Down Mode

This option specifies the power management state that the hard disk drive enters after the specified period of display inactivity has expired. The settings are Disabled, Standby, or Suspend. The default settings are Disabled.

Hard Disk Time out (Min)

This option specifies the length of a period of hard disk inactivity. When this period expires, the hard disk drive enters the power-conserving mode specified in the Hard Disk Power Down Mode

option described on the previous page. The settings are Disabled, 1 Min (minutes), and all one minute intervals up to and including 15 Min. The default settings are Disabled.

Standby Time out

This option specifies the length of the period of system inactivity when the computer is in Full-On mode before the computer is placed in Standby mode. In Standby mode, some power use is curtailed. The settings are Disabled, 1 Min, 2 Min, and all one minute intervals up to and including 15 Min. The default settings are Disabled.

Suspend  Time out

This option specifies the length of the period of system inactivity when the computer is already in Standby mode before the computer is placed in Suspend mode. In Suspend mode, nearly all power use is curtailed. The settings are Disabled, 1 Min, 2 Min, and all one minute intervals up to and including 15 Min. The default settings are Disabled.

Slow Clock Ratio

This option specifies the speed at which the system clock runs in power saving modes. The settings are expressed as a ratio between the normal clock speed and the power down clock speed. The settings are 1:1, 1:2 (half as fast as normal), 1:4 (the normal clock speed), 1:8, 1:16, 1:32, 1:64, or 1:128. The default setting is 1:1.

Display Activity

This option specifies if AMIBIOS is to monitor activity on the display monitor for power conservation purposes. When this options set to Monitor and there is no display activity for the length of time specified in the value in the Full-On to Standby Timeout (Min) option, the computer enters a power saving state. The settings are Monitor or Ignore. The default settings are Ignore.

IRQ XX

These options enable event monitoring. When the computer is in a power saving mode, activity on the named interrupt request line is monitored by AMIBIOS. When any activity occurs, the computer enters Full On mode.

Each of these options can be set to Monitor or Ignore. The default setting for all options is Ignore.

### 3.1.3    PCI/PnP Setup

PCI/PnP Setup options are displayed by choosing the PCI/PnP Setup icon from the WINBIOS Setup main menu. All PCI/PnP Setup options are described in this section

Plug and Play Aware O/S

Set this option to Yes if the operating system installed in the computer is Plug and Play-aware. AMIBIOS only detects and enables PnP ISA adapter cards that are required for system boot. The Windows 95 operating system detects and enables all other PnP-aware adapter cards. Windows 95 is PnP-aware. Set this option to No if the operating system (such as DOS, OS/2, Windows 3.x) does not use PnP. You must set this option correctly or PnP-aware adapter cards installed in your computer will not be configured properly. The settings are No or Yes. The Optimal and Fail-Safe default settings are No.

PCI Burst Mode

Set this option to Enabled to enable PCI burst mode. The settings are Disabled or Enabled. The Optimal default setting is Enabled. The Fail-Safe default setting is Disabled.

PCI Latency Timer (PCI Clocks)

This option sets latency of all PCI devices on the PCI bus. The settings are in units equal to PCI clocks. The settings are 32, 64, 96, 128, 160, 192, 224, or 248. The Optimal and Fail-Safe default settings are 64.

PCI VGA Palette Snoop

This option must be set to Enabled if any ISA adapter card installed in the computer requires VGA palette snooping. The settings are Disabled or Enabled. The Optimal and Fail-Safe default settings are Disabled.

PCI IDE BusMaster

Set this option to Enabled to specify that the IDE controller on the PCI local bus has bus mastering capability. The settings are Disabled or Enabled. The Optimal and Fail-Safe default settings are Disabled.

Offboard PCI IDE Card

This option specifies if an offboard PCI IDE controller adapter card is used in the computer. You must also specify the PCI expansion slot on the motherboard where the offboard PCI IDE controller card is installed. If an offboard PCI IDE controller is used, the onboard IDE controller on the motherboard is automatically disabled. The settings are Disabled, Auto, Slot1, Slot2, Slot3, or Slot4.

If Auto is selected, AMIBIOS automatically determines the correct setting for this option. The Optimal and Fail-Safe default settings are Auto.

In the AMIBIOS for the Intel Triton chipset, this option forces IRQ 14 and 15 to a PCI slot on the PCI local bus. This is necessary to support non-compliant PCI IDE adapter cards.

Offboard PCI IDE Primary IRQ

This option specifies the PCI interrupt used by the primary IDE channel on the offboard PCI IDE controller. The settings are Disabled, INTA, INTB, INTC, or INTD. The Optimal and Fail-Safe default settings are Disabled.

Offboard PCI IDE Secondary IRQ

This option specifies the PCI interrupt used by the secondary IDE channel on the offboard PCI IDE controller. The settings are Disabled, INTA, INTB, INTC, or INTD. The Optimal and Fail-Safe default settings are Disabled.

PCI/PnP Setup

These options specify the bus that the named interrupt request lines (IRQs) are used on. These options allow you to specify IRQs for use by legacy ISA adapter cards.

These options determine if AMIBIOS should remove an IRQ from the pool of available IRQs passed to BIOS configurable devices. The available IRQ pool is determined by reading the ESCD NVRAM. If more IRQs must be removed from the pool, the end user can use these PCI/PnP Setup options to remove the IRQ by assigning the option to the  ISA/EISA setting. Onboard I/O is configurable by AMIBIOS. The IRQs used by onboard I/O are configured as PCI/PnP.

The settings are PCI/PnP or ISA/EISA. The Optimal and Fail-Safe default settings are PCI/PnP.

Reserved Memory Size

This option specifies the size of the memory area reserved for legacy ISA adapter cards.

The settings are Disabled, 16K, 32K, or 64K. The Optimal and Fail-Safe default settings are Disabled.

Reserved Memory Address

This option specifies the beginning address (in hex) of the reserved memory area. The specified ROM memory area is reserved for use by 	legacy ISA adapter cards.

The settings are C0000, C4000, C8000, CC000, D0000, D4000, D8000, or DC000. The Optimal and Fail-Safe default settings are C0000.

3.2    Security Setup

WinBIOS Setup has an optional password feature.  The system can be configured so that all users must enter a password every time the system boots or when WinBIOS Setup is executed.  The following screen appears when you select the password icon.

Figure 2

You can enter a password by:

·   typing the password on the keyboard,

·   selecting each letter via the mouse, or

·   selecting each letter via the pan stylus.

Pen access must be customized for each specific hardware platform.

The password check option is enabled in Advanced Setup by choosing either Always ( the

password prompt appears every time the system is powered on ) or Setup ( the password

prompt appears only when WinBIOS Setup is run).  The password is stored in CMOS RAM

CMOS RAM.  The system asks for a password.

Enter a 1-6 character password.  The password does not appear on the screen when typed. WinBIOS  will ask you  to retype the password .  Make sure you  write it  down.  If you forget it, you must drain CMOS RAM and  reconfigure the system.  WinBIOS  will then display the following:

Figure 3

Select the Password icon from the Security section of the WinBIOS Setup  main menu.  Enter the password and press <Enter>.  The screen does not display the characters entered.  After the new password is entered, retype the new password as prompted and press<Enter>.

If the password confirmation is incorrect,  an error message appears.  If the new password is entered without error,  press<Esc>to return to the WinBIOS Setup Main Menu. The password is stored in CMOS RAM after WinBIOS Setup completes.  The next time the system boots,  you are prompted for the  password if the password function is present and is enabled.

Remember the Password

Keep a record of the new password when the password is changed.  If you forget the password and you computer has an American Megatrends motherboard, remove the  computer cover, set switch 1-2 ( the DIAG switch ) to ON, power on the computer.  WinBIOS erases the password.

When this icon is selected from the Security section of the WinBIOS Setup main menu, WinBIOS issues a warning when any program (or virus) issues a Disk Format command or attempts to write to the boot sector of the hard disk drive.  The following screen appears when you select the Anti-Virus icon:

Figure 4

The setting are Enabled or Disabled.  If enabled, the following appears when a write is attempted to the boot sector.  You may have to type N several times to prevent the boot sector write.

3.3    Default Setup

The icons in this section permit you to select a group of settings for all WinBIOS Setup options.  Not only can you use these icons to quickly set system configuration parameters, you can choose a group of settings that have a better chance of working when the system is having configuration-related problems.

Original

Choose the Original icon to return to the system configuration values present in WinBIOS Setup when you first began this WinBIOS Setup session.

Optimal You can load the optimal default settings for the WinBIOS Setup options by selecting the Optimal icon. The Optimal default settings are best-case values that should optimize system performance. If CMOS RAM is corrupted, the Optimal settings are loaded automatically.

Fail-Safe

You can load the Fail-Safe WinBIOS Setup option settings by selecting the Fail-Safe icon from the Default section of the WinBIOS Setup main menu.

The Fail-Safe settings provide far from optimal system performance, but are the most stable  settings.  Use this option as a diagnostic aid if the system is behaving erratically.

Figure 5

Figure 6

WINBIOS BEEP CODES

| BEEPS | ERROR MESSAGE | DESCRIPTION |
| --- | --- | --- |
| 1 | Refresh Failure | The memory refresh circuitry is faulty. |
| 2 | Parity Error | Parity error in the base memory (the first 64 KB block) of memory. |
| 3 | Base 64 KB Memory Failure | Memory failure in first 64 KB. |
| 4 | Timer Not Operational | A memory failure in the first 64 KB of memory, or Timer 1 is not functioning. |
| 5 | Processor Error | The CPU generated an error. |
| 6 | 8042-Gate A20 Failure | Cannot switch to protected mode. |
| 7 | Processor Exception Interrupt Error | The CPU on the CPU Card generated an exception interrupt. |
| 8 | Display Memory Read/Write Error | The system video adapter is either missing or its memory is faulty. This is not a fatal error. |
| 9 | ROM Checksum Error | The ROM checksum value does not match the value encoded in WinBIOS. |
| 10 | CMOS Shutdown Register Read/Write Error | The shutdown register for CMOS RAM has failed. |
| 11 | Cache Memory Bad Do Not Enable Cache | The cache memory test failed. Cache memory is disabled. Do not press<Ctrl><Alt><Shift> <+>to enable cache memory. |

What to Do If the Computer Beeps

Here is what you need to do if your computer has a WinBIOS and it starts beeping:

| IF THE SYSTEM BEEPS...... | THEM... |
| --- | --- |
| 1, 2, or 3 times... | Reseat the memory SIMMs or DIPs. If the system still beeps, replace the memory. |
| 6 times... | Reseat the keyboard controller chip. If it still beeps, replace the keyboard controller. If it still beeps, try a different keyboard, or replace the keyboard fuse, if the keyboard has one. |
| 8 times... | There is a memory error on the video adapter. Replace the video adapter , or the RAM on the video adapter. |
| 9 times... | The BIOS ROM chip is bad. The system probably needs a new BIOS ROM chip. |
| 11 times... | Reseat the cache memory on the motherboard. If it still beeps, replace the cache memory. |
| 4, 5, 7 or 10 times... | The motherboard must be replaced. |

CHAPTER   4

SYSTEM AWARD BIOS SETUP

The section will explain how to set up the system configuration (CMOS) underr the AWARD BIOS.  The SETUP program is contained in the system’s Read-Only-Memory rather than on a diskette.

4.1   Entering Setup

Power on the computer and press <Del> immediately will allow you to enter Setup.  The other way to enter Setup is to power on the computer, when the below message appears briefly at the bottom of the screen during the (Power On Self Test), press <Del> key or simultaneously press <Ctrl>, <Alt>, and <Esc> keys.

TO ENTER SETUP BEFORE BOOT PRESS <CTRL-ALT-ESC> OR <DEL> KEY

If the message disappears before you respond and you still wish to enter Setup, restart the system to try again by turning it OFF then ON or pressing the "RESET" button on the system case.  You may also restart by simultaneously pressing <Ctrl>, <Alt>, and <Delete> keys.  If you do not press the keys at the correct time and the system does not boot, an error message will be displayed and you will again be asked to,

PRESS <F1> TO CONTINUE, <CTRL-ALT-ESC> OR <DEL> TO ENTER SETUP

4.2   Control Keys

| Up arrow | Move to previous item |
| --- | --- |
| Down arrow | Move to next item |
| Left arrow | Move to the item in the left hand |
| Right arrow | Move to the item in the right hand |
| Esc key | Main Menu -- Quit and not save changes into CMOS Status Page Setup Menu and Option Page Setup Menu -- Exit current page and return to Main Menu |
| PgUp / “+” key | Increase the numeric value or make changes |
| PgDn / “-“ key | Decrease the numeric value or make changes |
| F1 key | General help, only for Status Page Setup Menu and Option Page Setup Menu |
| (Shift)F2 key | Change color from total 16 colors. F2 to select color forward, (Shift) F2 to select color backward |
| F3 key | Reserved |
| F4 key | Reserved |
| F8 key | Reserved |
| F9 key | Reserved |
| F10 key | Save all the CMOS changes, only for Main Menu |

4.3   Getting Help

- Main Menu

The on-line description of the highlighted setup function is displayed at the bottom of the screen.

Status Page Setup Menu/Option Page Setup Menu

Press F1 to pop up a small help window that describes the appropriate keys to use and the possible selections for the highlighted item.  To exit the Help Window press <F1> or <Esc>.

4.4   The Main Menu

Once you enter AWARD BIOS CMOS Setup Utility, the Main Menu will appear on the screen.  The Main Menu allows you to select from ten setup functions and two exit choices.  Use arrow keys to select among the items and press <Enter> to accept or enter the sub-menu.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | HDD LOW LEVEL FORMAT |
| LOAD BIOS DEFAULTS | SAVE & EXIT SETUP |
| LOAD SETUP DEFAULTS | EXIT WITHOUT SAVING |
| Esc : Quit | ¯ ® ¬ : Select Item |
| F10 : Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

- Standard CMOS Setup

This setup page includes all the items in a standard compatible BIOS.

- BIOS Features Setup

This setup page includes all the items of Award special enhanced features.

- Chipset Features Setup

This setup page includes all the items of chipset special features.

- Power Management Setup

This category determines how much power consumption for system after selecting belo items.  Default value is Disable.

- PNP/PCI Configuration Setup

This category specifies the balue (in units of PCI bus clocks) of the latency timer for this PCI bus master and the IRQ level for PCI device.

- Load  BIOS Defaults

BIOS defaults indicates the most appropriate value of the system parameter which the system would be in minimum performance.  The OEM manufacturer may change the defaults through MODBIN before the binary image burn into the ROM.

- Load Setup Defaults

Chipset defaults indicates the values required by the system for the maximum performance.  The OEM manufacturer may change to defaults through MODBIN before the binary image burn into the ROM.

- User  Password

Change, set, or disable password.  It allows you to limit access to the system and Setup, or just to Setup.

- IDE HDD Auto Detection

Automatically configure hard disk parameters.

- Save & Exit Setup

Save CMOS value changes to CMOS and exit setup.

- Exit Without Save

Abandon all CMOS value changes and exit setup.

4.5   Standard CMOS Setup Menu

The items in Standard CMOS Setup Menu are divided into 10 categories.  Each category includes no, one or more than one setup items.  Use the arrow keys to highlight the item and then use the <PgUp> or <PgDn> keys to select the value you want in each item.

ROM PCI/ISA BIOS

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| Date (mm:dd:yy) : Tue, July 23 1996 |  |  |  |
| --- | --- | --- | --- |
| Time (hh:mm:ss) : 10:21:21 |  |  |  |
| HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDE SECTOR MODE |  |  |  |
| Primary Master : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Primary Slave : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Secondary Master : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Secondary Slave : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Drive A | : 1.44M, 3.5 in. |  |  |
| Drive B | : None |  |  |
|  |  | Base Memory | : 640K |
| Video | : EGA/VGA | Extended Memory | : 15360K |
| Halt On | : All Errors | Other Memory | : 384K |
|  |  | Total Memory | : 16384K |
| ESC: Quit ¯®¬: Select Item PU / PD /+ /- : Modify |  |  |  |
| F1 : Help (Shift)F2: Change Color |  |  |  |

- Date

The date format is <day>, <date> <month> <year>.  Press <F3> to show the calendar.

| day | The day of week, from Sun to Sat, determined by the BIOS, is read only |
| --- | --- |
| date | The date, from 1 to 31 (or the maximum allowed in the month), can key in the numerical / function key |
| month | The month, Jan through Dec. |
| year | The year, depend on the year of BIOS |

- Time

The time format is <hour> <minute> <second>. which accepts both function key or numerical key The time is calculated based on the 24-hour military-time clock.  For example, 1 p.m. is 13:00:00.

- Drive C Type/Drive D Type

The categories identify the types of hard disk drive C or drive D that have been installed in the computer.  There are 45 predefined types and 2 user definable types are for Normal BIOS..  Type 1 to Type 45 are predefined.  Type User is user-definable.

- Primary Master/Primary Slave/Secondary Master/Secondary Slave

The categories identify the types of 2 channels that have been installed in the computer.  There are 45 predefined types and 4 user definable types are for Enhanced IDE BIOS.  Type 1 to Type 45 are predefined.  Type User is user-definable.

Press PgUp/<+> or PgDn/<-> to select a numbered hard disk type or type the number and press <Enter>.    Note that the specifications of your drive must match with the drive table.  The hard disk will not work properly if you enter improper information for this category.  If your hard disk drive type is not matched or listed, you can use Type User to define your own drive type manually.

If you select Type User, related information is asked to be entered to the following items.  Enter the information directly from the keyboard and press <Enter>.  This information should be provided in the documentation from your hard disk vendor or the system manufacturer.

If the controller of HDD interface is ESDI, the selection shall be “Type 1”.

If the controller of HDD interface is SCSI, the selection shall be “None”.

If the controller of HDD interface is CD-ROM, the selection shall be “None”.

| CYLS. | Number of Cylinders |
| --- | --- |
| HEADS | Nnumber of Heads |
| PRECOMP | Write Precom |
| LANDZONE | Landing Zone |
| SECTORS | Nnumber of Sectors |
| MODE | HDD Aaccess Mode |

If a hard disk has not been installed select NONE and press <Enter>.

- Drive A Type/Drive B Type

The category identifies the types of floppy disk drive A or drive B that have been installed in the computer.

| None | No floppy drive installed |
| --- | --- |
| 360K, 5-1/4 inch | 5-1/4 inch PC-type standard drive; 360 kilobyte capacity |
| 1.2M, 5-1/4 inch | 5-1/4 inch AT-type high-density drive; 1.2 megabyte capacity |
| 720K, 3-1/2 inch | 3-1/2 inch double-sided drive; 720 kilobyte capacity |
| 1.44M, 3-1/2 inch | 3-1/2 inch double-sided drive; 1.44 megabyte capacity |
| 2.88M, 3-1/2 inch | 3-1/2 inch double-sided drive; 2.88 megabyte capacity |

- Video

The category selects the type of adapter used for the primary system monitor that must match your video display card and monitor.  Although secondary monitors are supported, you do not have to select the type in Setup.

You have two ways to boot up the system:

- When VGA as primary and monochrome as secondary, the selection of the video tape is  “VGA Mode”.

- When monochrome as primary and VGA as secondary, the selection of the video type is           “Monochrome mode”.

| EGA/VGA | Enhanced Graphics Adapter/video Graphics Array. For EGA, VGA, SEGA, or PGA monitor adapters. |
| --- | --- |
| CGA 40 | Color Graphics Adapter, power up in 40 column mode |
| CGA 80 | Color Graphics Adapter, power up in 80 column mode |
| MONO | Monochrome adapter, includes high resolution monochrome adapters |

- Error Halt

The category determines whether the computer will stop if an error is detected during power up.

| No errors | Whenever the BIOS detects a non-fatal error the system will be stopped and you will be prompted. |
| --- | --- |
| All errors | The system boot will not be stopped for any error that may be detected. |
| All, But Keyboard | The system boot will not stop for a keyboard error; it will stop for all other errors. |
| All, But Diskette | The system boot will not stop for a disk error; it will stop for all other errors. |
| All, But Disk/Key | The system boot will not stop for a keyboard or disk error; it will stop for all other errors. |

- Memory

The category is display-only which is determined by POST (Power On Self Test) of the BIOS.

Base Memory

The POST of the BIOS will determine the amount of base (or conventional) memory installed in the system.  The value of the base memory is typically 512K for systems with 512K memory installed on the motherboard, or 640K for systems with 640K or more memory installed on the motherboard.

Extended Memory

The BIOS determines how much extended memory is present during the POST.  This is the amount of memory located above 1MB in the CPU's memory address map.

Other Memory

This refers to the memory located in the 640K to 1024K address space.  This is memory that can be used for different applications.  DOS uses this area to load device drivers to keep as much base memory free for application programs.  Most use for this area is Shadow RAM.

- Total Memory

System total memory is the sum of basic memory, extended memory, and other memory.

4.6   BIOS Features Setup Menu

ROM PCI/ISA BIOS

BIOS FEATURES SETUP

AWARD SOFTWARE, INC.

| Virus Warning CPU Internal Cache External Cache Quick Power On Self Test Boot Sequence Swap Floppy Drive Boot Up Floppy Seek Boot Up NumLock Status Boot Up System Speed Gate A20 Option Typematic Rate Setting Typematic Rate (Chars/Sec) Typematic Delay (Msec) Security Option PCI/VGA Palette Snoop OS Select For DRAM > 64MB | : Disabled : Enabled : Enabled : Disabled : A ,C : Disabled : Enabled : On : High : Fast : Disabled : 6 : 250 : Setup : Disabled : Non-OS2 | Video BIOS Shadow C8000-CBFFF Shadow CC000-CFFFF Shadow D0000-D3FFF Shadow D4000-D7FFF Shadow D8000-DBFFF Shadow DC000-DFFFF Shadow | : Enabled : Disabled : Disabled : Disabled : Disabled : Disabled : Disabled |
| --- | --- | --- | --- |
|  |  | ESC : Quit ¯ ® ¬ : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |  |

- Virus Warning

This category flashes on the screen.   During and after the system boots up, any attempt to write to the boot sector or partition table of the hard disk drive will halt the system and the following error message will appear, in the mean time, you can run an anti-virus program to locate the problem.

! WARNING ! Disk boot sector is to be modified Type "Y" to accept write or "N" to abort write Award Software, Inc.

| Enabled | Activates automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector or hard disk partition table. |
| --- | --- |
| Disabled | No warning message to appear when anything attempts to access the boot sector or hard disk partition table. |

Note: This function is available only for DOS and other OSes that do not trap INT13.

- CPU Internal Cache/External Cache

These two categories speed up memory access.  However, it depends on CPU/chipset design.  The default value is Enable.  If your CPU without Internal Cache then this item “CPU Internal Cache” will not be show.

| Enabled | Enable cache |
| --- | --- |
| Disabled | Disable cache |

- Quick Power On Self Test

This category speeds up Power On Self Test (POST) after you power on the computer.  If it is set to Enable, BIOS will shorten or skip some check items during POST.

| Enabled | Enable quick POST |
| --- | --- |
| Disabled | Normal POST |

- Boot Sequence

This category determines which drive computer searches first for the disk operating system (i.e., DOS).  Default value is A,C.

| C,A | System will first search for hard disk drive then floppy disk drive. |
| --- | --- |
| A,C | System will first search for floppy disk drive then hard disk drive. |

Note: This function is only available for IDE type

For SCSI type is always boot from A.

- Boot Up Floppy Seek

During POST, BIOS will determine if the floppy disk drive installed is 40 or 80 tracks.  360K type is 40 tracks while 760K, 1.2M and 1.44M are all 80 tracks.

| Enabled | BIOS searches for floppy disk drive to determine if it is 40 or 80 tracks. Note that BIOS can not tell from 720K, 1.2M or 1.44M drive type as they are all 80 tracks. |
| --- | --- |
| Disabled | BIOS will not search for the type of floppy disk drive by track number. Note that there will not be any warning message if the drive installed is 360K. |

- Boot Up NumLock Status

The default value is On.

| On | Keypad is number keys |
| --- | --- |
| Off | Keypad is arrow keys |

- Boot Up System Speed

It selects the default system speed - the speed that the system will run at immediately after power up.

| High | Set the speed to high |
| --- | --- |
| Low | Set the speed to low |

- IDE HDD Block Mode

| Enabled | Enable IDE HDD Block Mode. The BIOS will detect the block size of the HDD and send block command automatically. |
| --- | --- |
| Disabled | Disable IDE HDD Block Mode |

- Gate A20 Option

| Normal | The A20 signal is controlled by keyboard controller or chipset hardware. |
| --- | --- |
| Fast | Default : Fast. The A20 signal is controlled by Port 92 or chipset specific method. |

- Typematic Rate Setting

This determines the typematic rate.

| Enabled | Enable typematic rate and typematic delay programming |
| --- | --- |
| Disabled | Disable typematic rate and typematic delay programming. The system BIOS will use default value of this 2 items and the default is controlled by keyboard. |

- Typematic Rate (Chars/Sec)

| 6 | 6 characters per second |
| --- | --- |
| 8 | 8 characters per second |
| 10 | 10 characters per second |
| 12 | 12 characters per second |
| 15 | 15 characters per second |
| 20 | 20 characters per second |
| 24 | 24 characters per second |
| 30 | 30 characters per second |

- Typematic Delay (Msec)

When holding a key, the time between the first and second character displayed.

| 250 | 250 msec |
| --- | --- |
| 500 | 500 msec |
| 750 | 750 msec |
| 1000 | 1000 msec |

- Security Option

This category allows you to limit access to the system and Setup, or just to Setup.

| System | The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt. |
| --- | --- |
| Setup | The system will boot, but access to Setup will be denied if the correct password is not entered at the prompt. |

Note:	To disable security, select PASSWORD SETTING at Main Menu and then you will be asked to enter password.  Do not type anything and just press <Enter>, it will disable security.  Once the security is disabled, the system will boot and you can enter Setup freely.

- System BIOS Shadow

It determines whether system BIOS will be copied to RAM or the system BIOS is always shadow to support LBA HDD.

| Enabled | System shadow is enabled |
| --- | --- |
| Disabled | System shadow is disabled |

- Video BIOS Shadow

It determines whether video BIOS will be copied to RAM, however, it is optional from chipset design.  Video Shadow will increase the video speed.

| Enabled | Video shadow is enabled |
| --- | --- |
| Disabled | Video shadow is disabled |

4.7   Chipset Features Setup

ROM PCI/ISA BIOS

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Auto Configuration | : Enabled |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAM Timing | : 70 ns |  |  |  |  |  |  |  |
| DRAM RAS# Precharge Time | : 4 |  |  |  |  |  |  |  |
| DRAM R/W Leadoff Timing | : 7 |  |  |  |  |  |  |  |
| Fast RAS To CAS Delay | : 3 |  |  |  |  |  |  |  |
| DRAM Read Burst (EDO/FP) | : x222/x333 |  |  |  |  |  |  |  |
| DRAM Write Burst Timing | : x333 |  |  |  |  |  |  |  |
| Fast MA to RAS# Delay CLK | : 1 |  |  |  |  |  |  |  |
| Fast EDO Path Select | : Disabled |  |  |  |  |  |  |  |
| Refresh RAS# Assertion | : 5 Clks |  |  |  |  |  |  |  |
| ISA Bus Clock | : PCICLK/4 |  |  |  |  |  |  |  |
| System BIOS Cacheable | : Disabled |  |  |  |  |  |  |  |
| Video BIOS Cacheable | : Disabled |  |  |  |  |  |  |  |
| 8 Bit I/O Recovery Time | : 1 |  |  |  |  |  |  |  |
| 16 Bit I/O Recovery Time | : 1 | ESC | : Quit | ¯®¬ |  |  | : Select |  |
| Memory Hole At 15M-16M | : Disabled | F1 | : Help | PU/PD/+/- |  |  | : Modify |  |
| Peer Concurrency | : Enabled | F5 | : Old Values | (Shift)F2 |  |  | : Color |  |
|  |  | F6 | : Load BIOS Defaults |  |  |  |  |  |
|  |  | F7 | : Load Setup Defaults |  |  |  |  |  |

4.8	Power Management Setup

The Power management setup will appear on your screen like this:

ROM PCI/ISA BIOS

POWER MANAGEMENT SETUP

AWARD SOFTWARE, INC.

| Power Management | : Disable | ** Power Down & Resume Events ** |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PM Control by APM | : Yes | IRQ3 |  | (COM2) |  | : ON |  |  |  |
| Video Off Method | : V/H SYNC+ Blank | IRQ4 |  | (COM1) |  | : ON |  |  |  |
| MODEM Use IRQ | : 3 | IRQ5 |  | (LPT2) |  | : ON |  |  |  |
|  |  | IRQ6 |  | (Floppy Disk) |  | : ON |  |  |  |
| Doze Mode | : Disable | IRQ7 |  | (LPT1) |  | : ON |  |  |  |
| Standby Mode | : Disable | IRQ8 |  | (RTC Alarm) |  | : OFF |  |  |  |
| Suspend Mode | : Disable | IRQ9 |  | (IRQ2 Redir) |  | : ON |  |  |  |
| HDD Power Down | : Disable | IRQ10 |  | (Reserved) |  | : ON |  |  |  |
|  |  | IRQ11 |  | (Reserved) |  | : ON |  |  |  |
| ** Wake Up Events In Doze & Standby ** |  | IRQ12 |  | (PS/2 Mouse) |  | : ON |  |  |  |
| IRQ3 (Wake-up Event) | : ON | IRQ13 |  | (Coprocessor) |  | : ON |  |  |  |
| IRQ4 (Wake-up Event) | : ON | IRQ14 |  | (Hard Disk) |  | : ON |  |  |  |
| IRQ8 (Wake-up Event) | : OFF | IRQ15 |  | (Reserved) |  | : ON |  |  |  |
| IRQ12 (Wake-up Event) | : ON |  |  |  |  |  |  |  |  |
|  |  | ESC | : Quit |  | ¯®¬ |  |  | : Select |  |
|  |  | F1 | : Help |  | PU / PD / + / - |  |  | : Modify |  |
|  |  | F5 | : Old Values |  | (Shift)F2 |  |  | : Color |  |
|  |  | F6 | : Load BIOS Defaults |  |  |  |  |  |  |
|  |  | F7 | : Load Setup Defaults |  |  |  |  |  |  |

- Power Management

This category determines how much power consumption for system after selecting below items.  Default value is Disable.  The following pages tell you the options of each item & describe the meanings of each options.

| Item | Options | Descriptions |
| --- | --- | --- |
| A. Power Management | 1. Disable | Global Power Management will be disabled |
|  | 2. User Define | Users can configure their own power management |
|  | 3. Min Saving | Pre-defined timer values are used such that all timers are in their MAX value |
|  | 4. Max Saving | Pre-defined timer values are used such that all timers MIN value |
| B. PM Control by APM | 1. No | System BIOS will ignore APM when power managing the system |
|  | 2. Yes | System BIOS will wait for APM’s prompt before it enter any PM mode e.g. DOZE, STANDBY or SUSPEND Note: If APM is installed, & if there is a task running, even the timer is time out, the APM will not prompt the BIOS to put the system into any power saving mode! |
|  |  | Note: - if APM is no installed, this option has no effect |
| C. Video Off Option | 1. Always On | System BIOS will never turn off the screen |
|  | 2. Suspend -> Off | Screen off when system is in SUSPEND mode |
|  | 3. Susp, Stby -> Off | Screen off when system is in STANDBY or SUSPEND mode |
|  | 4. All Modes -> Off | Screen off when system is in DOZE, STANDBY or SUSPEND mode |
|  |  | Note: The M/B markers are recommended to fix this item to (2) or (3) & hidden it by using MODBIN Utility |

4.9   PNP/PCI Configuration Setup

You can manually configurate the PCI Device’s IRQ. The following pages tell you the options of each item & describe the meanings of each options.

ROM PCI/ISA BIOS

PNP/PCI CONFIGURATION SETUP

AWARD SOFTWARE, INC.

| Resources Controlled By |  | : Manual | PCI IRQ Actived BY |  |  | : Level |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset Configuration Data |  | : Disabled | PCI IDE IRQ Map To |  |  | : PCI-AUTO |  |  |
|  |  |  | Primary IDE INT# |  |  | : A |  |  |
| IRQ-3 | assigned to | : Legacy ISA | Secondary IDE INT# |  |  | : B |  |  |
| IRQ-4 | assigned to | : Legacy ISA |  |  |  |  |  |  |
| IRQ-5 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-7 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-9 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-10 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-11 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-12 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-14 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-15 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| DMA-0 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| DMA-1 | assigned to | : PCI/ISA PnP | ESC | : Quit | ¯®¬ |  |  | : Select Item |
| DMA-3 | assigned to | : PCI/ISA PnP | F1 | : Help | PU/PD/+/ - |  |  | : Modify |
| DMA-5 | assigned to | : PCI/ISA PnP | F5 | : Old Values | (Shift)F2 |  |  | : Color |
| DMA-6 | assigned to | : PCI/ISA PnP | F6 | : Load BIOS Defaults |  |  |  |  |
| DMA-7 | assigned to | : PCI/ISA PnP | F7 | : Load Setup Defaults |  |  |  |  |

The following pages tell you the options of each item & describe the meanings of each options.

4.10   INTEGRATED PERIPHERALS

ROM PCI/ISA BIOS

INTEGRATED PERIPHERALS

AWARD SOFTWARE, INC.

| IDE HDD Block MODE | : Enabled |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IDE Primary Master PIO | : Auto |  |  |  |  |  |  |
| IDE Primary Slave PIO | : Auto |  |  |  |  |  |  |
| IDE Secondary Master PIO | : Auto |  |  |  |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |  |  |  |
| On-Chip Primary PCI IDE | : Enabled |  |  |  |  |  |  |
| On-Chip Secondary PCI IDE | : Enabled |  |  |  |  |  |  |
| PCI Slot IDE 2nd Channel | : Enabled |  |  |  |  |  |  |
| USB Controller | : Disabled |  |  |  |  |  |  |
| Onboard FDD Controller | : Enabled |  |  |  |  |  |  |
| Onboard Serial Port 1 | : Auto |  |  |  |  |  |  |
| Onboard Serial Port 2 | : Auto |  |  |  |  |  |  |
| UART2 Mode | : Standard |  |  |  |  |  |  |
| Onboard Parallel Port | : 378/IRQ7 |  |  |  |  |  |  |
| Onboard Parallel Mode | : SPP | ESC | : Quit | ¯®¬ |  |  | : Select |
|  |  | F1 | : Help | PU/PD/+/- |  |  | : Modify |
|  |  | F5 | : Old Values | (Shift)F2 |  |  | : Color |
|  |  | F6 | : Load BIOS Defaults |  |  |  |  |
|  |  | F7 | : Load Setup Defaults |  |  |  |  |

4.11   Password Setting

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP |  |  |  | INTEGRATED PERIPHERALS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BIOS FEATURES SETUP |  |  |  | SUPERVISOR PASSWORD |  |  |
| CHIPSET FEATURES SETUP |  |  |  | USER PASSWORD |  |  |
| POWER MANAGEMENT SETUP |  |  |  | IDE HDD AUTO DETECTION |  |  |
| PNP/PCI CONFIRURATION |  |  |  | HDD LOW LEVEL FORMAT |  |  |
| LOAD BIOS DEFAULT |  | Enter Password: |  |  |  | T SETUP |
| LOAD SETUP DEFAULTS |  |  | EXIT WITHOUT SAVING |  |  |  |
| Esc | : Quit |  |  | ¯®¬ | : Select Item |  |
| F10 | : Save & Exit Setup |  |  | (Shift)F2 | : Change Color |  |
| Change /set /Disable Password |  |  |  |  |  |  |

When you select this function, the following message will appear at the center of the screen to assist you in creating a password.

ENTER PASSWORD:

Type the password, up to eight characters, and press <Enter>.  The password typed now will clear any previously entered password from CMOS memory.  You will be asked to confirm the password.  Type the password again and press <Enter>.  You may also press <Esc> to abort the selection and not enter a password.

To disable password, just press <Enter> when you are prompted to enter password.  A message will confirm the password being disabled.  Once the password is disabled, the system will boot and you can enter Setup freely.

PASSWORD DISABLED.

If you select System at Security Option of BIOS Features Setup Menu, you will be prompted for the password every time the system is rebooted or any time you try to enter Setup.  If you select Setup at Security Option of BIOS Features Setup Menu, you will be prompted only when you try to enter Setup.

4.12  IDE HDD Auto Detection

The Enhance IDE features was included in all Award BIOS.  Below is a brief description of this feature.

1. Setup Changes

<I> Auto-detection

- BIOS setup will display all possible modes that supported by the HDD including NORMAL, LBA & LARGE.

- If HDD does not support LBA modes, no ‘LBA’option will be shown.

- If no of cylinders is less than or equal to 1024, no ‘LARGE’ option will be show.

- Users can select a mode which is appropriate for them.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| Select Primary Slave Option (N = Skip ) : N OPTIONS SIZE CYLS HEADS PRECOMP LANDZ SECTOR MODE 1(Y) 0 0 0 0 0 0 NORMAL Note: Some OSes (like SCO-UNIX) must use “NORMAL “ for installation HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE Primary Master : 1282 621 64 0 2483 63 LBA Primary Slave : |  |  |
| --- | --- | --- |
| ESC: Skip |  |  |

RMA  FORM

When the motherboard can not work well, please fill up this form to describe related

situations.  If the space is not enough to use, you can attach separate paper.

MODEL:                                                        MODEL NO:

HARDWARE

CPU :  Brand                               , Model                              , Speed                                    MHz

CD-PROCESSOR:  Brand                               , Model                               , Speed                            MHz

SIMM: Brand                         , Speed                           ns, Q’ty                        pcs,Total                   MB

CACHE: Brand                               , Speed                                ns,Total                              K

TAG RAM: Brand                              , Speed                              ns

BIOS DATA CODE:

SYSTEM SPEED RUNNING                                 MHz

VIDEO CARD:   Chip                            , RAM                            , VGA Mode

Bus                                     (ISA,VESA or PCI)

OTHER ADD-ON CARDS:

SOFTWARE

OPERATING SYSTEM                                                 VERSION

SOFTWARE PROGRAM

BIOS SETUP: DRAM Wait State                                     CACHE Wait State

If you change BIOS SETUP, please describe the changes:

<A>ERROR

¨HANG UP                             ¨NO SCREEN                             ¨FLOPPY R/W ERROR

¨HARD DISK R/W ERROR      ¨PARITY MEMORY ERROR

¨OTHER

<B>ERROR MESSAGES ON YOUR SCREEN (PLEASE SHOW US THE WHOLE  SENTENCE)

<C>PROBLEM DESCRIPTION