PENTIUM / P55C

PCI Motherboard

INTELâ 430VX Design

User’s Manual

EXP8J61

451-011600100

REVISION: D

17-Mar-1997

Table of Contents

Chapter  1  Introduction	1

1.1	Overview	1

1.2	System Features	1

1.3	System Specification	2

1.4	System Performance	2

1.5	EXP8J61 Board Layout	3

Chapter  2  Installation	4

2.1	Installing DRAM	4

2.2	Setting CPU Type Jumper	6

2.3	Selecting CPU I/O & CPU Core Voltage	7

2.4	Installing FLASH ROM	8

2.5	Installing Other Jumpers & Connectors	9

Chapter  3  System BIOS Setup	12

3.1	Entering Setup	12

3.2	Control Keys	12

3.3	Getting Help	13

3.4	The Main Menu	13

3.5	Standard CMOS Setup Menu	15

3.6	BIOS Features Setup	18

3.7	Chipset Features Setup	21

3.8	Power Management	21

3.9	Plug and Play/PCI Configuration Setup	23

3.10	Integrated Peripherals	24

3.11	Password Setting	25

3.12	IDE HDD Auto Detection	26

Chapter  1    Introduction

1.1    Overview

The EXP8J61 motherboard is complemented by a maximum 512K L2 write-back cache, providing workstation level computing performance.  SIMM and DIMM sockets can support up to 128MB of DRAM.

The EXP8J61 motherboard offers the outstanding I/O capabilities.  Three PCI Local Bus slots provide a high bandwidth data path for data-movement intensive functions such as graphics acceleration.  Three ISA slots complete the I/O mix.

The EXP8J61 motherboard provides the foundation for cost effective, high performance, and highly expandable platforms which deliver the latest CPU and I/O technologies.

1.2    System Features

The EXP8J61 motherboard supports the following features:

q 	INTEL PENTIUM P54C, P55C(MMX) 100/120/133/150/166/200 MHz CPU,

CYRIX 6x86-120+(100MHz)/ 6x86-133+(110MHz)/6x86-150+(120MHz)/

6x86-166+(133MHz) CPU,

AMD 5K86-P75/5K86-P90/5K86-P100/ K5-PR120 (90MHz) / K5-PR133 (100MHz) /

K5-PR166/ K6-PR200 CPU

q 	3 Master 32-bit PCI Bus

q 	L1/L2 write back/write through cache

q 	256KB/512KB cache size

q 	72-pin SIMM Modules and 168-pin DIMM Module

q 	2 Serial/1 Parallel/1 FDC on board

q 	2-CHANNEL PCI IDE on board

q 	MESI (Modified Exclusive Shared Invalid) protocol to maintain the data coherence for L2 Cache to optimize CPU bus

q 	IrDA function support

1.3    System Specification

Processor:	INTEL PENTIUM P54C,P55C(MMX) 100/120/133/150/166/200 MHz CPU,

CYRIX 6x86-120+(100MHz)/6x86-133+(110MHz)/ 6x86-150+(120MHz)/

6x86-100+ (133MHz) CPU,

AMD 5K86-P75/5K86-P90/ 5K86-P100/ K5-PR120 (90MHz)/ K5-PR133 (100MHz)/

K5-PR166/ K6-PR200 CPU

CPU Clock Speed:	50/55/60/66 MHz

Memory:	8MB to 128MB

SRAM:	256K/512K

BIOS type:	AWARD BIOS

Additional BIOS Feature:	Setup program installed in ROM

Slot types:	Three 16-bit ISA slot

Three 32-bit PCI slot

Dimension:	220X224mm

Additional Features

Miscellaneous Connectors: 	Reset button, Suspend button, Coin battery

Board Design: 	4-layer Implementation for low noise operation

1.4    System Performance

| SOFTWARE CPU TYPE | LANDMARK V2.0 | POWER METER V1.8 MIPS | NORTON V8.0 CPU SPEED |
| --- | --- | --- | --- |
| PENTIUM 90 | 526.85 MHz | 45.9 MIPS | 289.2 |
| PENTIUM 100 | 578.51 MHz | 50.8 MIPS | 317.6 |
| PENTIUM 120 | 702.43 MHz | 61.8 MIPS | 385.6 |
| PENTIUM 133 | 771.33 MHz | 67.7 MIPS | 423.5 |
| PENTIUM 150 | 878.06 MHz | 74.8 MIPS | 482.1 |
| PENTIUM 166 | 964.19 MHz | 83.6 MIPS | 529.3 |
| PENTIUM 200 | 1157.04 MHz | 98.1 MIPS | 635.2 |
| P55C-166 (MMX) | 1093.07 MHz | 83.6 MIPS | 567.1 |
| CYRIX 6x86-120+ (100MHz) | 1209.95 MHz | 66.1 MIPS | 680.3 |
| CYRIX 6x86-133+ (110MHz) | 1353.98 MHz | 74.8 MIPS | 761.2 |
| CYRIX 6x86-150+ (120MHz) | 1469.25 MHz | 81.3 MIPS | 826 |
| CYRIX 6x86-166+ (133MHz) | 1613.31 MHz | 88.9 MIPS | 907 |
| AMD 5K86-P90 | 817.55 MHz | 49.0 MIPS | 361.5 |
| AMD 5K86-P100 | 897.68 MHz | 53.7 MIPS | 397 |
| AMD K5-PR120 (90MHz) | 817.54 MHz | 59.2 MIPS | 394.3 |
| AMD K5-PR133 (100MHz) | 897.72 MHz | 67.7 MIPS | 433.0 |
| AMD K5-PR166 | 1047.34MHz | 79 MIPS | 505.1 |
| AMD K6-PR200 | 2024.95MHz | 118.5 MIPS | 1190.2 |

1.5    EXP8J61 Board Layout

82371SB

82437VX

JPA1

82438VX

82438VX

JPA2

CPU SOCKET

1               5

IDE_LED

J20

1           4

U5

U1

1           4

TURBO RESET  SPEAKER     KEYLOCK

LED

JP19 JP16   J18            J17

BANK0

DIM1

U8

U6

U3

3      1

JP15

3       1

JP14

JP12

JP13

1       3

1       3

1      3

JP18

JP17

JP11

JP2

JP3

JP9

JP10

1

4

1

4

J15

J14

1

6

CN1

KEYBOARD

J2        J1

J3

J6

J4

J5

J8

J9

J10

J11

J12

COM2

COM1

IDE1

IDE2

+

U9

U22

1

4

PS2J

PRN

FDC

SIMM

PB SRAM

PB SRAM

M5125

(37C93X)

TAG

27010

U13

U7

USB CONNECTOR

PCI1

PCI2

PCI3

1        2

9      10

1        2

9      10

1       2

25    26

1       2

33    34

1       2

39    40

1       2

39    40

POWER

CONNECTOR

G

R

B

Y

4

J7

- +

JP9

82371

3

1

Chapter  2    Installation

The hardware must be set up for various functions before the system is ready to operate.  It is a simple task to set up the EXP8J61 motherboard.  The user only has to adjust a few jumpers, connectors and sockets.

2.1    Installing DRAM

The EXP8J61 motherboard can support extended memory of 8MB to 128MB.

n  The layout of the DRAM memory banks on board is shown below:

BANK0

J2        J1

DIM1

Installing DRAM

J1+J2

TOTAL

DIM1

or            =

The 72-pin group includes two SIMMs; each SIMM can be 4, 8,16, or 32MB in size.  Please install the DRAM of the same size in the group.

n   Table 1 (SIMM)

| BANK 0 |  |  |
| --- | --- | --- |
| SIMM1 | SIMM2 | Total Memory |
| 4MB | 4MB | 8MB |
| 8MB | 8MB | 16MB |
| 16MB | 16MB | 32MB |
| 32MB | 32MB | 64MB |

n   Table 2  (DIMM)

| DIM1 (BANK0) | Total Memory |
| --- | --- |
| 8MB | 8MB |
| 16MB | 16MB |
| 32MB | 32MB |

Note: DIMM and SIMM can’t be installed together.

2.2    Setting CPU Type Jumper

| CPU Type | Jumper Setting |
| --- | --- |
| PENTIUM 75MHz/ JP9 JP2 l l l l JP10 JP3 AMD 5k86-P75 |  |
| l l JP9 JP9 JP10 JP2 JP2 JP3 JP3 PENTIUM 90MHz/AMD 5K86-P90/ AMD K5-PR120 (90MHz) |  |
| JP2 JP3 JP9 JP10 PENTIUM100MHz/AMD 5K86-P100/ AMD K5-PR133 (100MHz) |  |
| l l JP3 l l JP9 JP10 JP2 INTEL PENTIUM120MHz |  |
| l l JP3 JP2 INTEL PENTIUM133MHz | JP9 JP10 |
| l l JP2 l l JP10 l l JP3 JP9 INTEL PENTIUM150MHz |  |
| INTEL PENTIUM 166MHz l l l l JP2 JP3 JP9 JP10 / AMD K5-PR166 |  |
| INTEL PENTIUM 200MHz l l JP3 JP2 JP9 JP10 / AMD K6-PR200 |  |
| CYRIX 6x86-120+(100MHz) JP9 l l l l JP10 l l JP3 JP2 |  |
| l l CYRIX 6x86-133+(110MHz) JP9 JP10 l l JP3 JP2 |  |
| l l CYRIX 6x86-150+(120MHz) l l JP3 JP9 JP10 JP2 |  |
| CYRIX 6x86-166+(133MHz) l l JP3 JP9 JP10 JP2 |  |

2.3    Selecting CPU I/O & CPU Core Voltage

1               5

IDE_LED

J20

1           4

U5

U1

1           4

TURBO RESET  SPEAKER     KEYLOCK

LED

JP19 JP16   J18            J17

U6

U3

3      1

JP15

3       1

JP14

JP12

1       3

1       3

1      3

JP18

JP17

JP11

JP2

JP3

U22

PB SRAM

PB SRAM

27010

JPA1

82438VX

82438VX

JPA2

JP13

CPU SOCKET

- +

CPU I/O VOLTAGE SELECT

| 3.3V (P55C,AMD K6) for Dual Voltage CPU | Default (P54C) |
| --- | --- |
| JP14 3 2 1 JP14 3 2 1 l l |  |

CPU CORE VOLTAGE SELECT

| 2.5V | 2.9V | §3.3V for Intel CPU | 3.45V~3.6V for AMD K5 and Cyrix 6x86 (028) CPU |
| --- | --- | --- | --- |
| JPA1 1 2 3 1 2 3 JPA1 1 2 3 JPA1 1 2 3 JPA2 JPA2 1 2 3 JPA2 1 2 3 l l l l l l JPA1 1 2 3 l l 1 2 3 JPA2 |  |  |  |

§ Default setting

NOTE: If JP13 is closed, CPU core voltage and I/O voltage are shorted accordingly, which is 	used for most Intel, AMD, and Cyrix CPUs.  (If you use P55C or AMD K6 CPU, just keep 	this jumper open.)

2.4    Installing FLASH ROM

1               5

IDE_LED

J20

1           4

U5

U1

1           4

TURBO RESET  SPEAKER     KEYLOCK

LED

JP19 JP16   J18            J17

U6

U3

3      1

JP15

3       1

JP14

JP12

JP13

1       3

1       3

1      3

JP18

JP17

JP11

JP2

JP3

U22

PB SRAM

PB SRAM

27010

JPA1

82438VX

82438VX

JPA2

CPU SOCKET

- +

| §5V FLASH ROM | 12V FLASH ROM |
| --- | --- |
| JP8 JP8 3 2 1 3 2 1 · · · · JP18 1 2 3 l l JP18 1 2 3 l l |  |

§ Default Setting

2.5    Installing Other Jumpers & Connectors

J15

J14

CN1

G

R

B

Y

4

U8

1

4

1

4

1

6

KEYBOARD

1

4

USB CONNECTOR

BANK0

J5

PS2J

SIMM

M5125

(37C93X)

J4

+

1       2

25    26

1       2

33    34

J8

J9

J10

J11

1        2

9      10

PRN

FDC

COM2

J6

J12

1        2

9      10

J3

COM1

1       2

39    40

1       2

39    40

POWER

CONNECTOR

J7

IDE1

IDE2

U13

82371SB

PCI1

PCI2

PCI3

U9

U7

J2        J1

82437VX

TAG

DIM1

U6

U3

JP17

82438VX

82438VX

CPU SOCKET

U22

U5

U1

27010

J20

PB SRAM

PB SRAM

1               5

1           4

1           4

JP19 JP16   J18            J17

- +

IDE_LED

TURBO RESET  SPEAKER     KEYLOCK

LED

Other Jumper Description

| Jumper | Description |  |
| --- | --- | --- |
| l l JP17 | AT BUS CLOCK EQUAL PCICLK/3 (FOR FREQUENCY 50MHz) | AT BUS CLOCK PCICLK/4 § ( FOR FREQUENCY 60, 66MHz) |

§ Default Setting

Connector Description

| Connector | Pin Out | Signal Name |
| --- | --- | --- |
| J14, J15 USB CONNECTOR | 1 2 3 4 | +5V DC DATA OUT DATA OUT GROUND |
| JP16 RESET | 1 2 | GROUND RESET IN |
| J17 KEYLOCK CONNECTOR | 1 2 3 4 5 | +5V DC NC GROUND KEYBOARD INHIBITOR GROUND |
| J18 SPEAKER CONNECTOR | 1 2 3 4 | DATA LINE NC GROUND +5V DC |
| 1 4 + - - + J20 IDE_LED |  | 1-2 IDE1 3-4 IDE2 |
| JP11 | Suspend Button |  |
| - + JP19 | Turbo LED |  |
| CN1 IRDA CONNECTOR (See Page 24) | 1 2 3 4 5 6 | VCC NC DATA-RECEIVE GROUND DATA-TRANSMIT RESEVER |
| PS2J PS/2 MOUSE CONNECTOR | 1 2 3 4 | GROUND DATA DATA VCC |

Chapter  3    System BIOS Setup

This chapter will explain how to set up the system configuration (CMOS Setup) under AWARD BIOS.  The SETUP program is contained in the system’s ROM rather than on a diskette.

3.1    Entering Setup

Power on the computer and press <Del> immediately in order to enter Setup.  The other way to enter Setup is to restart the computer and press <Del> or simultaneously press <Ctrl>, <Alt> and <Esc> keys when the following message appears at the bottom of the screen during POST (Power On Self Test):

TO ENTER SETUP BEFORE BOOT PRESS <CTRL-ALT-ESC> OR <DEL> KEY

If the message disappears before you respond and you still try to enter Setup, restart the computer to try again or press the RESET button on the system case.  You may also restart by simultaneously pressing <Ctrl>, <Alt>, and <Del> keys.  If you don’t press the keys in time and the system does not boot, an error message will be displayed:

PRESS <F1> TO CONTINUE, <CTRL-ALT-ESC> OR <DEL> TO ENTER SETUP

At this time, you should take the actions accordingly.

3.2    Control Keys

| Keystroke | Function |
| --- | --- |
| Up | Move to previous item |
| Down | Move to next item |
| Left | Move to the item in the left hand |
| Right | Move to the item in the right hand |
| Esc | Main Menu -- Quit and not save changes into CMOS Status Page Setup Menu and Option Page Setup Menu -- Exit current page and return to Main Menu |
| PgUp / “+” | Increase the numeric value or make changes |
| PgDn / “-“ | Decrease the numeric value or make changes |
| F1 | General help, only for Status Page Setup Menu and Option Page Setup Menu |
| (Shift) F2 | Change color from total 16 colors. F2 to select color forward, (Shift) F2 to select color backward |
| F3 | Reserved |
| F4 | Reserved |
| F8 | Reserved |
| F9 | Reserved |
| F10 | Save all the CMOS changes, only for Main Menu |

3.3    Getting Help

- Main Menu

The on-line description of the highlighted setup item is displayed at the bottom of the screen.

- Status Page Setup Menu/Option Page Setup Menu

Press F1 to pop up a small help window that describes the appropriate keys to use and the possible selections for the highlighted item.  To exit the help window, press <F1> or <Esc>.

3.4    The Main Menu

Once you enter AWARD BIOS CMOS Setup Utility, the Main Menu will appear.  The Main Menu allows you to select from ten setup functions and two exit choices.  Use arrow keys to select among the items and press <Enter> to accept or enter a sub-menu.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PLUG AND PLAY/PCI CONFIGURATION | HDD LOW LEVEL FORMAT |
| LOAD BIOS DEFAULTS | SAVE & EXIT SETUP |
| LOAD SETUP DEFAULTS | EXIT WITHOUT SAVING |
| Esc : Quit | ¯ ® ¬ : Select Item |
| F10 : Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

- Standard CMOS Setup

This category includes all the items in a standard BIOS.

- BIOS Features Setup

This category includes all the items of Award special enhanced features.

- Chipset Features Setup

This category includes all the items of chipset special features.

# Power Management Setup

This category determines how much power consumption for system is activated after selecting the related items.  Default value is Disable.

# Plug and Play/PCI Configuration

This category specifies the value (in units of PCI bus clocks) of the latency timer for this PCI bus master and the IRQ level for PCI device.

# Load  BIOS Defaults

BIOS defaults indicates the most appropriate value of the parameter which makes the system in minimum performance.  The OEM manufacturer may change the defaults through MODBIN before the binary image is burned into ROM.

# Load Setup Defaults

Chipset defaults indicates the values required by the system for maximum performance.  The OEM manufacturer may change the defaults through MODBIN before the binary image is burned into ROM.

# User  Password

Change, set, or disable password.  It allows you to limit access to the System and Setup, or just to Setup.

# IDE HDD Auto Detection

Automatically configure hard disk parameters.

# Save & Exit Setup

Save CMOS value changes to CMOS and exit setup.

# Exit Without Saving

Abandon all CMOS value changes and exit setup.

3.5    Standard CMOS Setup Menu

The items in Standard CMOS Setup Menu are divided into 10 categories.  Each category includes no, one or more than one setup items.  Use the arrow keys to highlight the item and use the <PgUp>/<PgDn> key to select the value you want.

| Date (mm:dd:yy) : Tue, July 23 1996 |  |  |  |
| --- | --- | --- | --- |
| Time (hh:mm:ss) : 10:21:21 |  |  |  |
| HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |  |
| Primary Master : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Primary Slave : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Secondary Master : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Secondary Slave : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Drive A | : 1.44M, 3.5 in. |  |  |
| Drive B | : None |  |  |
| Floppy 3 Mode Support : Disabled |  | Base Memory | : 640K |
|  |  | Extended Memory | : 15360K |
| Video | : EGA/VGA | Other Memory | : 384K |
| Halt On | : All Errors | Total Memory | : 16384K |
| ESC : Quit ¯®¬ : Select Item PU / PD /+ /- : Modify |  |  |  |
| F1 : Help (Shift) F2 : Change Color |  |  |  |

# Date

The date format is <day>,<month> <date> <year>.  Press <F3> to show the calendar.

| day | The day of week, from Sun to Sat, determined by the BIOS, is read only |
| --- | --- |
| month | The month, Jan through Dec |
| date | The date, from 1 to 31 (or the maximum allowed in the month), can key in the numeric / function key |
| year | The year, depend on the year of BIOS |

# Time

The time format is <hour>:<minute>:<second>, which accepts both function key and numeric key.  The time is calculated based on the 24-hour military-time format.  For example, 1 p.m. is 13:00:00.

# Drive C Type/Drive D Type

The category identifies the type of hard disk drive C or drive D that has been installed in the computer.  There are 45 predefined types and 2 user definable types for Normal BIOS.  Type 1 to Type 45 are predefined.  Type User is user-definable.

# Primary Master/Primary Slave/Secondary Master/Secondary Slave

The category identifies the types of 2 channels that have been installed in the computer.  There are 45 predefined types and 4 user definable types for Enhanced IDE BIOS.  Type 1 to Type 45 are predefined.  Type User is user-definable.

Press PgUp/<+> or PgDn/<-> to select a numbered hard disk type, or type the number and press <Enter>.    Note that your drive must match with the drive specification table.  The hard disk will not work properly if you enter improper information.  If your hard disk drive type is not matched or listed, you can use Type User to define your own drive type manually.

If you select Type User, related information is reguired for the following items.  Enter the information directly. This information should be provided in the documentation of your hard disk vendor or the manufacturer.

If the controller of HDD interface is ESDI, the selection shall be “Type 1”.

If the controller of HDD interface is SCSI, the selection shall be “None”.

If the controller of HDD interface is CD-ROM, the selection shall be “None”.

| CYLS. | Number of Cylinders |
| --- | --- |
| HEADS | Number of Heads |
| PRECOMP | Write Precomp |
| LANDZONE | Landing Zone |
| SECTORS | Number of Sectors |
| MODE | HDD Access Mode |

If a hard disk has not been installed, select NONE and press <Enter>.

# Drive A Type/Drive B Type

The category identifies the type of floppy disk drive A or drive B that has been installed in the computer.

| None | No floppy drive installed |
| --- | --- |
| 360K, 5.25 in. | 5-1/4 inch PC-type standard drive; 360 kilobyte capacity |
| 1.2M, 5.25 in. | 5-1/4 inch AT-type high-density drive; 1.2 megabyte capacity |
| 720K,3.5 in. | 3-1/2 inch double-sided drive; 720 kilobyte capacity |
| 1.44M, 3.5 in. | 3-1/2 inch double-sided drive; 1.44 megabyte capacity |
| 2.88M, 3.5 in. | 3-1/2 inch double-sided drive; 2.88 megabyte capacity |

# Video

The category selects the type of adapter used for the primary monitor that must match with your video display card and monitor.  Although secondary monitors are supported, you do not need to select the type in Setup.

You have two ways to boot up the system:

- When VGA is primary and monochrome is secondary, select the video type as  “VGA Mode”.

- When monochrome is primary and VGA is secondary, select the video type as “Monochrome Mode”.

| EGA/VGA | Enhanced Graphics Adapter/video Graphics Array. For EGA, VGA, SEGA, or PGA monitor adapters. |
| --- | --- |
| CGA 40 | Color Graphics Adapter, power up in 40 column mode |
| CGA 80 | Color Graphics Adapter, power up in 80 column mode |
| MONO | Monochrome adapter, includes high resolution monochrome adapters |

# Error Halt

The category determines whether the computer will stop when an error is detected during power up.

| No Errors | Whenever the BIOS detects a non-fatal error the system will be stopped and you will be prompted. |
| --- | --- |
| All, But Keyboard | The system boot will not stop for a keyboard error; it will stop for all other errors. |
| All, But Diskette | The system boot will not stop for a disk error; it will stop for all other errors. |
| All, But Disk/Key | The system boot will not stop for a keyboard or disk error; it will stop for all other errors. |
| All Errors | The system boot will not be stopped for any error that may be detected. |

# Memory

The category is display-only which is determined by BIOS.

Base Memory

The POST of the BIOS will determine the amount of base (or conventional) memory installed in the system.  The size of the base memory is typically 512K for systems with 512K memory installed on the motherboard, or 640K for systems with 640K or more memory installed on the motherboard.

Extended Memory

The BIOS determines how much extended memory is present during POST.  This is the amount of memory located above 1MB in the CPU's address map.

Other Memory

It refers to the memory located in the 640K to 1024K address space.  This is the memory that can be used for different applications.  DOS uses this area to load device drivers to keep as much base memory free as possible for application programs.  Some of this area is used for shadow RAM.

# Total Memory

System memory is the sum of base memory, extended memory, and other memory.

3.6    BIOS Features Setup Menu

| Virus Warning CPU Internal Cache External Cache Quick Power On Self Test Boot Sequence Swap Floppy Drive Boot Up Floppy Seek Boot Up NumLock Status Boot Up System Speed Gate A20 Option Typematic Rate Setting Typematic Rate (Chars/Sec) Typematic Delay (Msec) Security Option PCI/VGA Palette Snoop OS Select For DRAM > 64MB | : Disabled : Enabled : Enabled : Disabled : A ,C : Disabled : Enabled : On : High : Fast : Disabled : 6 : 250 : Setup : Disabled : Non-OS2 | Video BIOS Shadow C8000-CBFFF Shadow CC000-CFFFF Shadow D0000-D3FFF Shadow D4000-D7FFF Shadow D8000-DBFFF Shadow DC000-DFFFF Shadow | : Enabled : Disabled : Disabled : Disabled : Disabled : Disabled : Disabled |
| --- | --- | --- | --- |
|  |  | ESC : Quit ¯ ® ¬ : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |  |

# Virus Warning

This category protects the boot sector and partition table of your hard disk.  Any attempt to write to the boot sector or partition table of the hard disk will halt the system and the following warning message will appear.  At this time, you can run an anti-virus program to locate the problem.

! WARNING ! Disk boot sector is to be modified Type "Y" to accept write or "N" to abort write Award Software, Inc.

| Enabled | Activates automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector or hard disk partition table |
| --- | --- |
| Disabled | No warning message to appear when anything attempts to access the boot sector or hard disk partition table |

Note: This function is available only for DOS and other OSes that do not trap INT13.

# CPU Internal Cache/External Cache

These two categories speed up memory access.  However, it depends on CPU/chipset design.  The default value is Enable.  If your CPU has no Internal Cache then this item “CPU Internal Cache” will not be shown.

| Enabled | Enable cache |
| --- | --- |
| Disabled | Disable cache |

# Quick Power On Self Test

This category speeds up Power On Self Test (POST) after you power on the computer.  If it is set to Enable, BIOS will shorten or skip some check items during POST.

| Enabled | Enable quick POST |
| --- | --- |
| Disabled | Normal POST |

# Boot Sequence

This category determines which drive the computer first searches for an operating system.  Default value is A,C.

| A,C | System will first search for floppy disk drive then hard disk drive |
| --- | --- |
| C,A | System will first search for hard disk drive then floppy disk drive |
| C, CD-ROM, A | System will first search for hard disk drive, CD-ROM then floppy disk drive |
| CD-ROM, C, A | System will first search for CD-ROM, hard disk drivethen floppy disk drive |

Note:  This function is only available for IDE type.  As for SCSI type, the boot sequence starts from A.

# Boot Up Floppy Seek

During POST, BIOS will determine if the floppy disk drive installed has  40 or 80 tracks.  360K type is 40 tracks while 720K, 1.2M and 1.44M are all 80 tracks.

| Enabled | BIOS searches for floppy disk drive to determine if it is 40 or 80 tracks. Note that BIOS can not tell from 720K, 1.2M or 1.44M drive type as they are all 80 tracks |
| --- | --- |
| Disabled | BIOS will not search for the type of floppy disk drive by track number. Note that there will not be any warning message if the drive installed is 360K |

# Boot Up NumLock Status

The default value is On.

| On | Keypad is number keys |
| --- | --- |
| Off | Keypad is arrow keys |

# Boot Up System Speed

It selects the default system speed - the speed that the system will run immediately after power up.

| High | Set the speed to high |
| --- | --- |
| Low | Set the speed to low |

# Gate A20 Option

| Normal | The A20 signal is controlled by keyboard controller or chipset hardware |
| --- | --- |
| Fast | Default : Fast. The A20 signal is controlled by Port 92 or chipset specific method |

# Typematic Rate Setting

| Enabled | Enable typematic rate and typematic delay programming |
| --- | --- |
| Disabled | Disable typematic rate and typematic delay programming. The system BIOS will use default value of this 2 items and the default is controlled by keyboard |

# Typematic Rate (Chars/Sec)

| 6 | 6 characters per second |
| --- | --- |
| 8 | 8 characters per second |
| 10 | 10 characters per second |
| 12 | 12 characters per second |
| 15 | 15 characters per second |
| 20 | 20 characters per second |
| 24 | 24 characters per second |
| 30 | 30 characters per second |

# Typematic Delay (Msec)

It controls the time between the display of the first and second characters.

| 250 | 250 msec |
| --- | --- |
| 500 | 500 msec |
| 750 | 750 msec |
| 1000 | 1000 msec |

# Security Option

This category allows you to limit access to the System and Setup, or just to Setup.

| System | The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt. |
| --- | --- |
| Setup | The system will boot, but access to Setup will be denied if the correct password is not entered at the prompt. |

Note:	To disable security, select PASSWORD SETTING from Main Menu and then you will be asked to enter password.  Do not type anything and just press <Enter>.  It will disable security.  Once the security is disabled, the system will boot and you can enter Setup freely.

# Video BIOS Shadow

It determines whether video BIOS will be copied to RAM; however, it is optional from chipset design.  Video Shadow can increase the video speed.

| Enabled | Video shadow is enabled |
| --- | --- |
| Disabled | Video shadow is disabled |

3.7    Chipset Features Setup

| Auto Configuration | : Enabled | Delayed Transaction |  |  | : Disabled |  |
| --- | --- | --- | --- | --- | --- | --- |
| DRAM Timing | : 70 ns |  |  |  |  |  |
| DRAM RAS# Precharge Time | : 4 |  |  |  |  |  |
| DRAM R/W Leadoff Timing | : 6 |  |  |  |  |  |
| Fast RAS To CAS Delay | : 3 |  |  |  |  |  |
| DRAM Read Burst (EDO/FP) | : x333/x444 |  |  |  |  |  |
| DRAM Write Burst Timing | : x333 |  |  |  |  |  |
| Fast MA to RAS# Delay CLK | : 1 |  |  |  |  |  |
| Fast EDO Path Select | : Disabled |  |  |  |  |  |
| Refresh RAS# Assertion | : 5 Clks |  |  |  |  |  |
| ISA Bus Clock | : PCICLK/4 |  |  |  |  |  |
| System BIOS Cacheable | : Disabled |  |  |  |  |  |
| Video BIOS Cacheable | : Disabled |  |  |  |  |  |
| 8 Bit I/O Recovery Time | : 1 | ESC | : Quit | ¯®¬ |  | : Select |
| 16 Bit I/O Recovery Time | : 1 | F1 | : Help | PU/PD/+/- |  | : Modify |
| Memory Hole At 15M-16M | : Disabled | F5 | : Old Values | (Shift) F2 |  | : Color |
| Peer Concurrency | : Enabled | F6 | : Load BIOS Defaults |  |  |  |
| Passive Release | : Disabled | F7 | : Load Setup Defaults |  |  |  |

3.8    Power Management Setup

The Power Management Setup will appear on your screen like this:

| Power Management | : Disable | ** Power Down & Resume Events ** |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PM Control by APM | : Yes | IRQ3 |  | (COM2) | : ON |  |
| Video Off Method | : V/H SYNC+ Blank | IRQ4 |  | (COM1) | : ON |  |
| MODEM Use IRQ | : 3 | IRQ5 |  | (LPT2) | : OFF |  |
|  |  | IRQ6 |  | (Floppy Disk) | : OFF |  |
| Doze Mode | : Disable | IRQ7 |  | (LPT1) | : OFF |  |
| Standby Mode | : disable | IRQ8 |  | (RTC Alarm) | : OFF |  |
| Suspend Mode | : Disable | IRQ9 |  | (IRQ2 Redir) | : OFF |  |
| HDD Power Down | : Disable | IRQ10 |  | (Reserved) | : OFF |  |
|  |  | IRQ11 |  | (Reserved) | : OFF |  |
| ** Wake Up Events In Doze & Standby ** |  | IRQ12 |  | (PS/2 Mouse) | : ON |  |
| IRQ3 (Wake-up Event) | : ON | IRQ13 |  | (Coprocessor) | : OFF |  |
| IRQ4 (Wake-up Event) | : ON | IRQ14 |  | (Hard Disk) | : ON |  |
| IRQ8 (Wake-up Event) | : ON | IRQ15 |  | (Reserved) | : OFF |  |
| IRQ12 (Wake-up Event) | : ON |  |  |  |  |  |
|  |  | ESC | : Quit |  | ¯®¬ | : Select |
|  |  | F1 | : Help |  | PU / PD / + / - | : Modify |
|  |  | F5 | : Old Values |  | (Shift) F2 | : Color |
|  |  | F6 | : Load BIOS Defaults |  |  |  |
|  |  | F7 | : Load Setup Defaults |  |  |  |

# Power Management

This category determines how much power consumption for system is activated after selecting the below items.  Default value is Disable.  The following will tell you the options of each item and describe the meaning of each option.

| Item | Options | Description |
| --- | --- | --- |
| A. Power Management | 1. Disable | Global Power Management will be disabled |
|  | 2. Min Saving | Pre-defined timer values are used such that all timers are in their MIN value |
|  | 3. Max Saving | Pre-defined timer values are used such that all timers are in their MAX value |
|  | 4. User Define | Users can configure their own power management |
| B. PM Control by APM | 1. No | System BIOS will ignore APM when power managing the system |
|  | 2. Yes | System BIOS will wait for APM’s prompt before it enter any PM mode e.g. DOZE, STANDBY or SUSPEND Note: If APM is installed, & if there is a task running, even the timer is time out, the APM will not prompt the BIOS to put the system into any power saving mode! |
|  |  | Note: if APM is not installed, this option has no effect |
| C. Video Off Method | 1. Blank Screen |  |
|  | 2. V/H SYNC+Blank |  |
|  | 3. DPMS |  |

3.9    Plug and Play/PCI Configuration Setup

You can manually configure the PCI Device’s IRQ.  The following will tell you the options of each item and describe the meaning of each option.

| Resources Controlled By |  | : Manual | PCI IRQ Actived By |  |  | : Level |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reset Configuration Data |  | : Disabled | PCI IDE IRQ Map To |  |  | : PCI-AUTO |  |
|  |  |  | Primary IDE INT# |  |  | : A |  |
| IRQ-3 | assigned to | : Legacy ISA | Secondary IDE INT# |  |  | : B |  |
| IRQ-4 | assigned to | : Legacy ISA |  |  |  |  |  |
| IRQ-5 | assigned to | : PCI/ISA Plug and Play | Used MEM base address : N/A |  |  |  |  |
| IRQ-7 | assigned to | : PCI/ISA Plug and Play |  |  |  |  |  |
| IRQ-9 | assigned to | : Legacy ISA |  |  |  |  |  |
| IRQ-10 | assigned to | : PCI/ISA Plug and Play |  |  |  |  |  |
| IRQ-11 | assigned to | : PCI/ISA Plug and Play |  |  |  |  |  |
| IRQ-12 | assigned to | : PCI/ISA Plug and Play |  |  |  |  |  |
| IRQ-14 | assigned to | : PCI/ISA Plug and Play |  |  |  |  |  |
| IRQ-15 | assigned to | : PCI/ISA Plug and Play |  |  |  |  |  |
| DMA-0 | assigned to | : PCI/ISA Plug and Play |  |  |  |  |  |
| DMA-1 | assigned to | : PCI/ISA Plug and Play |  |  |  |  |  |
| DMA-3 | assigned to | : PCI/ISA Plug and Play | ESC | : Quit | ¯®¬ |  | : Select Item |
| DMA-5 | assigned to | : PCI/ISA Plug and Play | F1 | : Help | PU/PD/+/ - |  | : Modify |
| DMA-6 | assigned to | : PCI/ISA Plug and Play | F5 | : Old Values | (Shift) F2 |  | : Color |
| DMA-7 | assigned to | : PCI/ISA Plug and Play | F6 | : Load BIOS Defaults |  |  |  |
|  |  |  | F7 | : Load Setup Defaults |  |  |  |

3.10    Integrated Peripherals

| IDE HDD Block MODE | : Enabled | Onboard Parallel Port |  |  | : 378/IRQ7 |  |
| --- | --- | --- | --- | --- | --- | --- |
| IDE Primary Master PIO | : Auto | Parallel Port Mode |  |  | : SPP |  |
| IDE Primary Slave PIO | : Auto |  |  |  |  |  |
| IDE Secondary Master PIO | : Auto |  |  |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |  |  |
| On-Chip Primary PCI IDE | : Enabled |  |  |  |  |  |
| On-Chip Secondary PCI IDE | : Enabled |  |  |  |  |  |
| PCI Slot IDE 2nd Channel | : Enabled |  |  |  |  |  |
| USB Controller | : Disabled |  |  |  |  |  |
| Onboard FDD Controller | : Enabled |  |  |  |  |  |
| Onboard Serial Port 1 | : Aoto |  |  |  |  |  |
| UR1 Mode | : Normal |  |  |  |  |  |
| Onboard Serial Port 2 | : Auto | ESC | : Quit | ¯®¬ |  | : Select |
| UR2 Mode | : Normal | F1 | : Help | PU/PD/+/- |  | : Modify |
|  |  | F5 | : Old Values | (Shift) F2 |  | : Color |
|  |  | F6 | : Load BIOS Defaults |  |  |  |
|  |  | F7 | : Load Setup Defaults |  |  |  |

If you want to use IrDA function, you must to be change CMOS setting as following.

| IDE HDD Block Mode | : Enabled | Onboard Paralleld Port |  | : 378/IRQ7 |  |
| --- | --- | --- | --- | --- | --- |
| IDE Primary Master PIO | : Auto | Parallel Port Mode |  | : SPP |  |
| IDE Primary Slave PIO | : Auto |  |  |  |  |
| IDE Secondar;y Master PIO | : Auto |  |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |  |
| On-Chip Primary PCI IDE | : Enabled |  |  |  |  |
| On-Chip Secondary PCI IDE | : Enabled |  |  |  |  |
| PCI Slot IDE 2nd Channel | : Enabled |  |  |  |  |
| USB Controller | : Disabled |  |  |  |  |
| Onboard FDD Controller | : Enabled |  |  |  |  |
| Onboard Serial Port 1 | : Auto |  |  |  |  |
| UR1 Mode | : Normal |  |  |  |  |
| Onboard Serial Port 2 | : Auto | ESC | : Quit | ¯®¬ | : Select Item |
| UR2 Mode | : IrDA | F1 | : Help | PU/PD/+/ - | : Modify |
| UR2 Duplex Mode | : Half | F5 | : Old Values | (Shift) F2 | : Color |
| Use UART2 Pins | : IR-RX2TX2 | F6 | : Load BIOS Defaults |  |  |
| UR2 IR Tr/Re polarity | : Hi/Hi | F7 | : Load Setup Defaults |  |  |

3.11    Password Setting

| STANDARD CMOS SETUP |  |  |  | INTEGRATED PERIPHERALS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BIOS FEATURES SETUP |  |  |  | SUPERVISOR PASSWORD |  |  |
| CHIPSET FEATURES SETUP |  |  |  | USER PASSWORD |  |  |
| POWER MANAGEMENT SETUP |  |  |  | IDE HDD AUTO DETECTION |  |  |
| PLUG AND PLAY/PCI CONFIRURATION |  |  |  | HDD LOW LEVEL FORMAT |  |  |
| LOAD BIOS DEFAULT |  | Enter Password: |  |  |  | T SETUP |
| LOAD SETUP DEFAULTS |  |  | EXIT WITHOUT SAVING |  |  |  |
| Esc | : Quit |  |  | ¯®¬ | : Select Item |  |
| F10 | : Save & Exit Setup |  |  | (Shift) F2 | : Change Color |  |
| Change /set /Disable Password |  |  |  |  |  |  |

When you select this function, the following message will appear at the center of the screen to assist you in creating a password.

ENTER PASSWORD:

Type the password, up to eight characters, and press <Enter>.  The password typed will overwrite any previously entered password from CMOS memory.  You will be asked to confirm the password.  Type the password again and press <Enter>.  You may also press <Esc> to abort the operation and enter no password.

To disable password, just press <Enter> when you are prompted to enter password.  A message will confirm the password being disabled.  Once the password is disabled, you can enter Setup freely.

PASSWORD DISABLED:

If you select System at Security Option of BIOS Features Setup Menu, you will be prompted for the password entry when the system is rebooted or you try to enter Setup.  If you select Setup at Security Option of BIOS Features Setup Menu, you will be prompted only when you try to enter Setup.

3.12    IDE HDD Auto Detection

The Enhanced IDE features are included in all Award BIOS.  Below is a brief description of the features.

If your computer has an IDE hard disk, you can use this ability to detect its parameters and enter them into the Standard CMOS Setup automatically.

- BIOS setup will display all possible modes that are supported by the HDD including NORMAL, LBA & LARGE.

- If HDD does not support LBA modeI, no ‘LBA’option will be shown.

- If the number of cylinders is less than or equal to 1024, no ‘LARGE’ option will be shown.

- Users can select a mode which is appropriate for them.

| HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Primary Master : |  |  |  |  |
| Primary Slave : |  |  |  |  |
|  | Select Primary Slave Option (N = Skip ) : N |  |  |  |
|  | OPTIONS SIZE CYLS HEADS PRECOMP LANDZ SECTOR MODE |  |  |  |
|  | 1(Y) 516 1120 16 65535 1119 59 NORMAL |  |  |  |
|  | 2 516 524 32 0 1119 63 LBA |  |  |  |
|  | 3 516 560 32 65535 1119 59 LARGE |  |  |  |
| Note: Some OSes (like SCO-UNIX) must use “NORMAL“ for installation |  |  |  |  |
| ESC: Skip |  |  |  |  |