451-011500200

REVISION:A

06-Jan-1997

PentiumTM Pro Processor-based

Intel 440FX PCI Motherboard

User’s Manual

EXP8P61

TABLE OF CONTENTS

CHAPTER  1  INTRODUCTION	1

1.1   Overview	1

1.2   System Features	1

1.3   System Specification	2

1.4   System Performance	2

1.5   EXP8P61 Board Layout	3

CHAPTER  2  INSTALLATION	4

2.1   DRAM Installation	4

2.2   CPU Type Jumper Setting	6

2.3   CPU Installation	7

2.4   FLASH ROM Installation	9

2.5   EPP MODE Setting	10

2.6   Other Jumper & Connector Installation	11

CHAPTER  3  SYSTEM BIOS SETUP	13

3.1    Entering Setup	13

3.2    Control Keys	13

3.3    Getting Help	14

3.4    The Main Menu	14

3.5    Standard CMOS Setup Menu	16

3.6    BIOS Features Setup	19

3.7    Chipset Features Setup	22

3.8    Power Management	23

3.9    PNP/PCI Configuration Setup	25

3.10  Integrated Peripherals	26

3.11  Password Setting	27

3.12  IDE HDD Auto Detection	28

CHAPTER   1

INTRODUCTION

1.1	Overview

The EXP8P61  motherboard offers outstanding I/O capabilities.  Four PCI Local Bus slots provide a high bandwidth data path for data-movement intensive function such as Graphics. Four ISA slots complete the I/O mix.

The EXP8P61 motherboard provides the foundation for cost effective, high  performance, highly expandable platforms which deliver the latest EXP8P61 in CPU and I/O technologies.

1.2	System Features

The EXP8P61 motherboard supports the following features:

- Supports INTEL PENTIUM PRO 150/180/200 MHz CPU

- Supports 4 MASTER 32-bit PCI Bus

- Supports L1 Write Back/Write Through Cache Feature

- Supports 72 pin SIMM modules

- Supports 2 Serial 1 Parallel 1 FDC on board

- Supports 2 Channels PCI IDE on board

1.3	System Specifications

Processor:	INTEL PENTIUM PRO 150/180/200 MHz CPU

CPU Clock Speed:	50/60/66 MHz

Memory:	2MB to 128MB

BIOS type:	AWARD BIOS

Additional BIOS Feature:	Set Program Resides in ROM

Slot type:	Four 16-bit ISA Bus

Four 32-bit PCI Bus

Dimension:	28x22 cm

Additional Features

Miscellaneous Connectors: 	Reset button, Suspend button

Board Design: 	4-layer Implementation for Low noise operation

1.4	System  Performance

| SOFTWARE CPU TYPE | LANDMARK V2.0 | POWER METER V1.8 MIPS | NORTON V8.0 CPU SPEED |
| --- | --- | --- | --- |
| PENTIUM PRO 150 | 936.78 MHz | 71.1 MIPS | 376 |
| PENTIUM PRO 180 | 1124.2 MHz | 83.6 MIPS | 451.2 |
| PENTIUM PRO 200 | 1249.14 MHz | 94.8 MIPS | 501.4 |

1.5	EXP8P61 Board Layout

KEYBOARD BIOS

JP20

CPU  SOCKET

PCI3

PCI2

W83768

PS/2 MOUSE CONNECTOR

KEYBOARD CONNECTOR

COM2COM1

POWER CONNECTOR

IDE1      IDE2

FDC

J7       J5       J4       J2

DS12887

27010

U15

PCI1

PCI4

JP10

PRN

JP8

BANK0   BANK0   BANK1   BANK1

39  40       39  40

25 26

1     2  1     2

9   10  9   10

1    2   1    2

1     2         1    2

1

3

U28

U27

1    6

5  10

1           4

JP9

U1

1

3

1

3

JP12

1           4

J12

J13

U12

SB82371SB

SB82441FX

SB82442FX

MS1

33  34

ON   DIP

1    2   3   4

SW1

ON

OFF

ON   DIP

1    2   3   4

SW2

ON

OFF

JP21

JP17

1               5

1           4

1          4

JP19         J24       J22   JP18 J26

+ –

KEYLOCK      SPKER    IDE/LED  RESET TB/LED

JP9

82371

3

1

CHAPTER   2

INSTALLATION

Before the system is ready to operate, the hardware must be set up for various functions of the system.  To set up the EXP8P61 motherboard is a simple task.  The user only has to set a few jumpers, connectors and sockets.

2.1	DRAM Installation

The EXP8P61 motherboard can support expanded memory from 2MB to 128MB.

nThe board layout below shows the locations of the DRAM memory banks:

BANK0 BANK0  BANK1  BANK1

J7       J5       J4       J2

DRAM  INSTALLATION

J5+J7

J2+J4

TOTAL

+            =

Each group includes two SIMMs each SIMM size can be 1, 2, 4, 8,16, 32MB, please install the same DRAM size in one group.

n  TABLE   (SIMM)

| BANK 0 |  | BANK 1 |  |  |
| --- | --- | --- | --- | --- |
| J7 | J5 | J4 | J2 | TOTAL MEMORY |
| 4MB | 4MB | None | None | 8MB |
| 4MB | 4MB | 4MB | 4MB | 16MB |
| 8MB | 8MB | None | None | 16MB |
| 4MB | 4MB | 8MB | 8MB | 24MB |
| 8MB | 8MB | 4MB | 4MB | 24MB |
| 8MB | 8MB | 8MB | 8MB | 32MB |
| 16MB | 16MB | None | None | 32MB |
| 4MB | 4MB | 16MB | 16MB | 40MB |
| 16MB | 16MB | 4MB | 4MB | 40MB |
| 8MB | 8MB | 16MB | 16MB | 48MB |
| 16MB | 16MB | 8MB | 8MB | 48MB |
| 16MB | 16MB | 16MB | 16MB | 64MB |
| 32MB | 32MB | None | None | 64MB |
| 4MB | 4MB | 32MB | 32MB | 72MB |
| 32MB | 32MB | 4MB | 4MB | 72MB |
| 8MB | 8MB | 32MB | 32MB | 80MB |
| 32MB | 32MB | 8MB | 8MB | 80MB |
| 16MB | 16MB | 32MB | 32MB | 96MB |
| 32MB | 32MB | 16MB | 16MB | 96MB |
| 32MB | 32MB | 32MB | 32MB | 128MB |

2.2	 CPU Type Jumper Setting

JP20

CPU  SOCKET

DS12887

27010

1

3

U28

U27

U1

JP21

SB82442FX

JP17

KEYLOCK      SPKER    IDE/LED  RESET TB/LED

1               5

1           4

1          4

JP19         J24       J22   JP18 J26

ON   DIP

1    2   3   4

SW1

ON   DIP

1    2   3   4

SW2

ON

OFF

ON

OFF

+ –

| ON DIP 1 2 3 4 ON OFF SW1 ON DIP 1 2 3 4 ON OFF SW2 CPU type | Jumper Setting |
| --- | --- |
| ON DIP 1 2 3 4 ON OFF SW1 INTEL PENTIUM PRO 150MHz |  |
| ON DIP 1 2 3 4 ON OFF SW2 INTEL PENTIUM PRO 180MHz |  |
| ON DIP 1 2 3 4 ON OFF SW1 ON DIP 1 2 3 4 ON OFF SW2 INTEL PENTIUM PRO 200MHz |  |

2.3	CPU Installation

JP20

1

3

U27

ON   DIP

1    2   3   4

SW1

ON

OFF

27010

BANK1

BANK0

CPU  SOCKET

U28

U1

ON   DIP

1    2   3   4

SW2

DS12887

SB82442FX

ON

OFF

JP21

JP17

JP19         J24       J22   JP18 J26

+ –

KEYLOCK      SPKER    IDE/LED  RESET TB/LED

1               5

1           4

1          4

CPU Frequency Setting

| 50MHz | §60MHz | 66MHz |
| --- | --- | --- |
| · JP16 · · · ON DIP 1 2 ON OFF SW2 ON DIP 1 2 ON OFF SW2 SW2 ON OFF ON DIP 1 2 |  |  |

§ Default Setting

CPU Bus Frequency Setting

| §50/60MHz | 66MHz |
| --- | --- |
| ON DIP 3 4 ON OFF SW2 SW2 ON OFF ON DIP 3 4 |  |

§ Default Setting

JP20

CPU  SOCKET

DS12887

27010

1

3

U28

U27

U1

JP21

SB82442FX

JP17

KEYLOCK      SPKER    IDE/LED  RESET TB/LED

1               5

1           4

1          4

JP19         J24       J22   JP18 J26

ON   DIP

1    2   3   4

SW1

ON   DIP

1    2   3   4

SW2

ON

OFF

ON

OFF

BANK1

BANK0

+ –

CPU Frequency Times

| 2 | 2.5 | §3 | 3.5 | 4 |
| --- | --- | --- | --- | --- |
| SW1 ON OFF ON DIP 1 2 3 4 ON DIP 1 2 3 4 ON OFF SW1 ON DIP 1 2 3 4 ON OFF SW1 SW1 ON OFF ON DIP 1 2 3 4 SW1 ON OFF ON DIP 1 2 3 4 |  |  |  |  |

§ Default Setting

2.4	FLASH ROM Installation

JP20

1

3

U27

ON   DIP

1    2   3   4

SW1

ON

OFF

27010

U28

CPU  SOCKET

U1

ON   DIP

1    2   3   4

SW2

DS12887

SB82442FX

ON

OFF

JP21

JP17

JP19         J24       J22   JP18 J26

KEYLOCK      SPKER    IDE/LED  RESET TB/LED

1               5

1           4

1          4

+ –

| §5V FLASH ROM | 12V FLASH ROM |
| --- | --- |
| JP20 JP8 JP8 3 2 1 · 3 2 1 · · · 1 2 3 · · JP20 · · 1 2 3 |  |

§ Default Setting

3

2

1

2.5	EPP MODE Setting

KEYBOARD BIOS

PS/2 MOUSE CONNECTOR

KEYBOARD CONNECTOR

MS1

W83768

1     2  1     2

POWER CONNECTOR

9   10  9   10

COM2COM1

1    2   1    2

1     2         1    2

JP9

1    6

5  10

1           4

J12

IDE1      IDE2

FDC

PRN

1

3

1           4

J13

1

3

JP8

25 26

JP10

33  34

39  40       39  40

JP12

PCI3

PCI2

PCI1

PCI4

SB82371SB

SB82441FX

U15

U12

DMA CHANNEL CONFIGURATION (FOR EPP DEVICE)

| DMA1 | DMA3 |
| --- | --- |
| JP10 JP8 JP10 JP8 · · · · 1 2 3 · · 1 2 3 · · 1 2 3 1 2 3 |  |

2.6	Other Jumper & Connector Installation

PS/2 MOUSE CONNECTOR

KEYBOARD BIOS

KEYBOARD CONNECTOR

MS1

W83768

1     2  1     2

POWER CONNECTOR

9   10  9   10

COM2COM1

1    2   1    2

1     2         1    2

1           4

J12

FDC

PRN

1           4

J13

IDE1      IDE2

25 26

33  34

39  40       39  40

JP12

PCI3

PCI2

PCI1

PCI4

SB82371SB

SB82441FX

U12

U15

U27

27010

U28

CPU  SOCKET

U1

DS12887

SB82442FX

JP21

JP17

JP19         J24       J22   JP18 J26

+ –

KEYLOCK      SPKER    IDE/LED  RESET TB/LED

1               5

1           4

1          4

Other Jumper Description

| JUMPER | DESCRIPTION |  |
| --- | --- | --- |
| · · · · JP12 | AT BUS CLOCK EQUAL PCICLK/3 (FOR FREQUENCY 50MHZ) | AT BUS CLOCK PCICLK/4 § ( FOR FREQUENCY 60, 66MHZ) |
| · · JP17 | NORMAL | CLEAR CMOS |
| JP21 | NORMAL MODE | SMI MODE |

§ Default Setting

CONNECTOR DESCRIPTION

| CONNECTOR | PIN OUT | SIGNAL NAME |
| --- | --- | --- |
| J12, J13 USB CONNECTOR | 1 2 3 4 | +5V DC DATA OUT DATA OUT GROUND |
| JP18 RESET | 1 2 | GROUND RESET IN |
| JP19 KEYLOCK & POWER LED CONNECTOR | 1 2 3 4 5 | +5V DC NC GROUND KEYBOARD DATA GROUND |
| J24 SPEAKER CONNECTOR | 1 2 3 4 | +5V DC GROUND NC DATA OUT |
| J22 IDE LED | 1 2 3 4 | +5V DC DATA OUT DATA OUT +5V DC |
| MS1 PS/2 MOUSE CONNECTOR | 1 2 3 4 | DATA DATA GROUND +5V DC |

CHAPTER   3

SYSTEM BIOS SETUP

The chapter will explain how to set up the system configuration (CMOS Setup) under AWARD BIOS.  The SETUP program is contained in the system’s ROM rather than on a diskette.

3.1	Entering Setup

Power on the computer and press <Del> immediately in order to enter Setup.  The other way to enter Setup is to restart the computer and press <Del> key or simultaneously press <Ctrl>and<Alt>keys when the following message appears at the bottom of the screen during POST (Power On Self Test):

TO ENTER SETUP BEFORE BOOT PRESS <CTRL-ALT-ESC> OR <DEL> KEY

If the message disappears before you respond and you still try to enter Setup, restart the computer to try again or press the RESET button on the system case.  You may also restart by simultaneously pressing <Ctrl>, <Alt>, and <Delete> keys.  If you do’nt press the keys in time and the system does not boot, an error message will be displayed:

PRESS <F1> TO CONTINUE, <CTRL-ALT-ESC> OR <DEL> TO ENTER SETUP

3.2	Control Keys

| Keystroke | Function |
| --- | --- |
| Up | Move to previous item |
| Down | Move to next item |
| Left | Move to the item in the left hand |
| Right | Move to the item in the right hand |
| Esc | Main Menu -- Quit and not save changes into CMOS Status Page Setup Menu and Option Page Setup Menu -- Exit current page and return to Main Menu |
| PgUp / “+” | Increase the numeric value or make changes |
| PgDn / “-“ | Decrease the numeric value or make changes |
| F1 | General help, only for Status Page Setup Menu and Option Page Setup Menu |
| (Shift)F2 | Change color from total 16 colors. F2 to select color forward, (Shift) F2 to select color backward |
| F3 | Reserved |
| F4 | Reserved |
| F8 | Reserved |
| F9 | Reserved |
| F10 | Save all the CMOS changes, only for Main Menu |

3.3	Getting Help

- Main Menu

The on-line description of the highlighted setup item is displayed at the bottom of the screen.

Status Page Setup Menu/Option Page Setup Menu

Press F1 to pop up a small help window that describes the appropriate keys to use and the possible selections for the highlighted item.  To exit the Help window, press <F1> or <Esc>.

3.4	The Main Menu

Once you enter AWARD BIOS CMOS Setup Utility, the Main Menu will appear.  The Main Menu allows you to select from ten setup functions and two exit choices.  Use arrow keys to select among the items and press <Enter> to accept or enter the sub-menu.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | HDD LOW LEVEL FORMAT |
| LOAD BIOS DEFAULTS | SAVE & EXIT SETUP |
| LOAD SETUP DEFAULTS | EXIT WITHOUT SAVING |
| Esc : Quit | ¯ ® ¬ : Select Item |
| F10 : Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type... |  |

- Standard CMOS Setup

This category includes all the items in a standard BIOS.

- BIOS Features Setup

This category includes all the items of Award special enhanced features.

- Chipset Features Setup

This category includes all the items of chipset special features.

- Power Management Setup

This category determines how much power consumption for system is activated after selecting the related item.  Default value is Disable.

- PNP/PCI Configuration Setup

This category specifies the value (in units of PCI bus clocks) of the latency timer for this PCI bus master and the IRQ level for PCI device.

- Load  BIOS Defaults

BIOS defaults indicates the most appropriate value of the parameter which makes the system in minimum performance.  The OEM manufacturer may change the defaults through MODBIN before the binary image is burned into ROM.

- Load Setup Defaults

Chipset defaults indicates the values required by the system for maximum performance.  The OEM manufacturer may change the defaults through MODBIN before the binary image is burned into ROM.

- User  Password

Change, set, or disable password.  It allows you to limit access to the system and Setup, or just to Setup.

- IDE HDD Auto Detection

Automatically configure hard disk parameters.

- Save & Exit Setup

Save CMOS value changes to CMOS and exit setup.

- Exit Without Save

Abandon all CMOS value changes and exit setup.

3.5	Standard CMOS Setup Menu

The items in Standard CMOS Setup Menu are divided into 10 categories.  Each category includes no, one or more than one setup items.  Use the arrow keys to highlight the item and use the <PgUp>/<PgDn> key to select the value you want.

ROM PCI/ISA BIOS

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| Date (mm:dd:yy) : Tue, Sep 3 1996 |  |  |  |
| --- | --- | --- | --- |
| Time (hh:mm:ss) : 10:21:21 |  |  |  |
| HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDE SECTOR MODE |  |  |  |
| Primary Master : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Primary Slave : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Secondary Master : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Secondary Slave : Auto 0 0 0 0 0 0 AUTO |  |  |  |
| Drive A | : 1.44M, 3.5 in. |  |  |
| Drive B | : None |  |  |
| Floppy 3 Mode Support : Disabled |  | Base Memory | : 640K |
|  |  | Extended Memory | : 15360K |
| Video | : EGA/VGA | Other Memory | : 384K |
| Halt On | : All Errors | Total Memory | : 16384K |
| ESC: Quit ¯®¬: Select Item PU / PD /+ /- : Modify |  |  |  |
| F1 : Help (Shift)F2: Change Color |  |  |  |

- Date

The date format is <day>, <month> <date> <year>.  Press <F3> to show the calendar.

| day | The day of week, from Sun to Sat, determined by the BIOS, is read only |
| --- | --- |
| month | The month, Jan through Dec |
| date | The date, from 1 to 31 (or the maximum allowed in the month), can key in the numerical / function key |
| year | The year, depending on the year of BIOS |

- Time

The time format is <hour>:<minute>:<second>, which accepts both function key and numerical key The time is calculated based on the 24-hour military-time format.  For example, 1 p.m. is 13:00:00.

- Drive C Type/Drive D Type

The category identifies the type of hard disk drive C or drive D that has been installed in the computer.  There are 45 predefined types and 2 user definable types are for Normal BIOS..  Type 1 to Type 45 are predefined.  Type User is user-definable.

- Primary Master/Primary Slave/Secondary Master/Secondary Slave

The category identifies the types of 2 channels that have been installed in the computer.  There are 45 predefined types and 4 user definable types are for Enhanced IDE BIOS.  Type 1 to Type 45 are predefined.  Type User is user-definable.

Press PgUp/<+> or PgDn/<-> to select a numbered hard disk type, or type the number and press <Enter>.    Note that your drive must match the drive specification table.  The hard disk will not work properly if you enter improper information.  If your hard disk drive type is not matched or listed, you can use Type User to define your own drive type manually.

If you select Type User, related information is reguired for the following items.  Enter the information directly.  This information should be provided in the documentation of your hard disk vendor or the  manufacturer.

If the controller of HDD interface is ESDI, the selection shall be “Type 1”.

If the controller of HDD interface is SCSI, the selection shall be “None”.

If the controller of HDD interface is CD-ROM, the selection shall be “None”.

| CYLS. | Number of Cylinders |
| --- | --- |
| HEADS | Nnumber of Heads |
| PRECOMP | Write Precomp |
| LANDZONE | Landing Zone |
| SECTORS | Number of Sectors |
| MODE | HDD Access Mode |

If a hard disk has not been installed, select NONE and press <Enter>.

- Drive A Type/Drive B Type

The category identifies the type of floppy disk drive A or drive B that has been installed in the computer.

| None | No floppy drive installed |
| --- | --- |
| 360K, 5-1/4 in. | 5-1/4 inch PC-type standard drive; 360 kilobyte capacity |
| 1.2M, 5-1/4 in. | 5-1/4 inch AT-type high-density drive; 1.2 megabyte capacity |
| 720K, 3-1/2 in. | 3-1/2 inch double-sided drive; 720 kilobyte capacity |
| 1.44M, 3-1/2 in. | 3-1/2 inch double-sided drive; 1.44 megabyte capacity |
| 2.88M, 3-1/2 in. | 3-1/2 inch double-sided drive; 2.88 megabyte capacity |

- Floppy 3 Mode Support

| Disabled | No 3 Mode floppy drive installed. |
| --- | --- |
| Drive A | Installed 3 mode drive at drive A. |
| Drive B | Installed 3 mode drive at drive B. |
| Both | Installed 3 mode drive at drive A and drive B. |

- Video

The category selects the type of adapter used for the primary monitor that must match your video display card and monitor.  Although secondary monitors are supported, you do not need to select the type in Setup.

You have two ways to boot up the system:

- When VGA is primary and monochrome is secondary, select the video tape as  “VGA Mode”.

- When monochrome is primary and VGA is secondary, select the video type as “Monochrome mode”.

| EGA/VGA | Enhanced Graphics Adapter/video Graphics Array. For EGA, VGA, SEGA, or PGA monitor adapters. |
| --- | --- |
| CGA 40 | Color Graphics Adapter, power up in 40 column mode |
| CGA 80 | Color Graphics Adapter, power up in 80 column mode |
| MONO | Monochrome adapter, includes high resolution monochrome adapters |

- Error Halt

The category determines whether the computer will stop when an error is detected during power up.

| No errors | Whenever the BIOS detects a non-fatal error the system will be stopped and you will be prompted. |
| --- | --- |
| All, But Keyboard | The system boot will not stop for a keyboard error; it will stop for all other errors. |
| All, But Diskette | The system boot will not stop for a disk error; it will stop for all other errors. |
| All, But Disk/Key | The system boot will not stop for a keyboard or disk error; it will stop for all other errors. |
| All Errors | The system boot will not be stopped for any error that may be detected. |

- Memory

The category is for display only which is determined by BIOS.

Base Memory

The POST of the BIOS will determine the amount of base (or conventional) memory installed in the system.  The size of the base memory is typically 512K for systems with 512K memory installed on the motherboard, or 640K for systems with 640K or more memory installed on the motherboard.

Extended Memory

The BIOS determines how much extended memory is present during POST.  This is the amount of memory located above 1MB in the CPU's address map.

Other Memory

It refers to the memory located in the 640K to 1024K address space.  This is the memory that can be used for different applications.  DOS uses this area to load device drivers to keep as much base memory free as possible for application programs.  Some of this area is Shadow RAM.

- Total Memory

System memory is the sum of base memory, extended memory, and other memory.

3.6	BIOS Features Setup Menu

ROM PCI/ISA BIOS

BIOS FEATURES SETUP

AWARD SOFTWARE, INC.

| Virus Warning CPU Internal Cache External Cache Quick Power On Self Test Boot Sequence Swap Floppy Drive Boot Up Floppy Seek Boot Up NumLock Status Boot Up System Speed Gate A20 Option Typematic Rate Setting Typematic Rate (Chars/Sec) Typematic Delay (Msec) Security Option PCI/VGA Palette Snoop OS Select For DRAM > 64MB | : Disabled : Enabled : Enabled : Disabled : A ,C : Disabled : Enabled : On : High : Fast : Disabled : 6 : 250 : Setup : Disabled : Non-OS2 | Video BIOS Shadow C8000-CBFFF Shadow CC000-CFFFF Shadow D0000-D3FFF Shadow D4000-D7FFF Shadow D8000-DBFFF Shadow DC000-DFFFF Shadow | : Enabled : Disabled : Disabled : Disabled : Disabled : Disabled : Disabled |
| --- | --- | --- | --- |
|  |  | ESC : Quit ¯ ® ¬ : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |  |

- Virus Warning

This category protects the boot sector and partition table of you hard disk.  Any attempt to write to the boot sector or partition table of the hard disk will halt the system and the following warning message will appear.  At this time, you can run an anti-virus program to solve the roblem.

! WARNING ! Disk boot sector is to be modified Type "Y" to accept write or "N" to abort write Award Software, Inc.

| Enabled | Activates automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector or hard disk partition table. |
| --- | --- |
| Disabled | No warning message to appear when anything attempts to access the boot sector or hard disk partition table. |

Note: This function is available only for DOS and other OSes that do not trap INT13.

- CPU Internal Cache/External Cache

These two categories speed up memory access.  However, it depends on CPU/chipset design.  The default value is Enable.  If your CPU has no Internal Cache then this item “CPU Internal Cache” will not be show.

| Enabled | Enable cache |
| --- | --- |
| Disabled | Disable cache |

- Quick Power On Self Test

This category speeds up Power On Self Test (POST) after you power on the computer.  If it is set to Enable, BIOS will shorten or skip some check items during POST.

| Enabled | Enable quick POST |
| --- | --- |
| Disabled | Normal POST |

- Boot Sequence

This category determines which drive the computer first searches for an operating system.  Default value is A, C.

| A,C | System will first search for floppy disk drive then hard disk drive. |
| --- | --- |
| C,A | System will first search for hard disk drive then floppy disk drive. |
| C, CD-ROM, A | System will first search for hard disk drive, CD-ROM then floppy drive. |
| CD-ROM, C, A | System will first search for CD-ROM, hard disk drive then floppy disk drive. |

Note: This function is only available for IDE type.  As for SCSI type, the boot sequence starts from A.

- Boot Up Floppy Seek

During POST, BIOS will determine if the floppy disk drive installed has 40 or 80 tracks.  360K type is 40 tracks while 760K, 1.2M and 1.44M are all 80 tracks.

| Enabled | BIOS searches for floppy disk drive to determine if it is 40 or 80 tracks. Note that BIOS can not tell from 720K, 1.2M or 1.44M drive type as they are all 80 tracks. |
| --- | --- |
| Disabled | BIOS will not search for the type of floppy disk drive by track number. Note that there will not be any warning message if the drive installed is 360K. |

- Boot Up NumLock Status

The default value is On.

| On | Keypad is number keys |
| --- | --- |
| Off | Keypad is arrow keys |

- Boot Up System Speed

It selects the default system speed - the speed that the system will run at immediately after power up.

| High | Set the speed to high |
| --- | --- |
| Low | Set the speed to low |

- IDE HDD Block Mode

| Enabled | Enable IDE HDD Block Mode. The BIOS will detect the block size of the HDD and send block command automatically. |
| --- | --- |
| Disabled | Disable IDE HDD Block Mode |

- Gate A20 Option

| Normal | The A20 signal is controlled by keyboard controller or chipset hardware. |
| --- | --- |
| Fast | Default : Fast. The A20 signal is controlled by Port 92 or chipset specific method. |

- Typematic Rate Setting

This determines the typematic rate.

| Enabled | Enable typematic rate and typematic delay programming |
| --- | --- |
| Disabled | Disable typematic rate and typematic delay programming. The system BIOS will use default value of this 2 items and the default is controlled by keyboard. |

- Typematic Rate (Chars/Sec)

| 6 | 6 characters per second |
| --- | --- |
| 8 | 8 characters per second |
| 10 | 10 characters per second |
| 12 | 12 characters per second |
| 15 | 15 characters per second |
| 20 | 20 characters per second |
| 24 | 24 characters per second |
| 30 | 30 characters per second |

- Typematic Delay (Msec)

It controls the time between display of the first and second character.

| 250 | 250 msec |
| --- | --- |
| 500 | 500 msec |
| 750 | 750 msec |
| 1000 | 1000 msec |

- Security Option

This category allows you to limit access to the System and Setup, or just to Setup.

| System | The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt. |
| --- | --- |
| Setup | The system will boot, but access to Setup will be denied if the correct password is not entered at the prompt. |

Note:	To disable security, select PASSWORD SETTING from Main Menu and then you will be asked to enter password.  Do not type anything and just press <Enter>, it will disable security.  Once the security is disabled, the system will boot and you can enter Setup freely.

- System BIOS Shadow

It determines whether system BIOS will be copied to RAM or the system BIOS is always shadow to support LBA HDD.

| Enabled | System shadow is enabled |
| --- | --- |
| Disabled | System shadow is disabled |

- Video BIOS Shadow

It determines whether video BIOS will be copied to RAM, however, it is optional from chipset design.  Video Shadow will increase the video speed.

| Enabled | Video shadow is enabled |
| --- | --- |
| Disabled | Video shadow is disabled |

3.7	Chipset Features Setup

ROM PCI/ISA BIOS

CHIPSET FEATURES SETUP

AWARD SOFTWARE, INC.

| Auto Configuration | : Enabled | 8 Bit I/O Recovery Time |  |  | : 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAM Speed Selection | : 70 ns | 16 Bit I/O Recovery Time |  |  | : 1 |  |  |  |
| DRAM RAS# Precharge Time | : 4 | Memory Hole At 15M-16M |  |  | : Disabled |  |  |  |
| MA Additional Wait State | : Disabled | DRAM Fast Leadoff |  |  | : Disabled |  |  |  |
| RAS# To CAS# Delay | : Enabled | Passive Release |  |  | : Enabled |  |  |  |
| DRAM Read Burst (B/E/F) | : x2 / 3 / 4 | Delayed Transaction |  |  | : Enabled |  |  |  |
| DRAM Write Burst (B/E/F) | : x3 / 3 / 3 |  |  |  |  |  |  |  |
| ISA Bus Clock | : PCICLK/4 |  |  |  |  |  |  |  |
| DRAM Refresh Queue | : Enabled |  |  |  |  |  |  |  |
| DRAM RAS Only Refresh | : Disabled |  |  |  |  |  |  |  |
| DRAM ECC/PARITY Select | : Disabled |  |  |  |  |  |  |  |
| Fast DRAM Refresh | : Disabled |  |  |  |  |  |  |  |
| Read-Around-Write Combine | : Enabled |  |  |  |  |  |  |  |
| PCI Burst Write Combine | : Enabled |  |  |  |  |  |  |  |
| PCI-To-DRAM Pipeline | : Enabled | ESC | : Quit | ¯®¬ |  |  | : Select |  |
| CPU-To-PCI Write Post | : Enabled | F1 | : Help | PU/PD/+/- |  |  | : Modify |  |
| CPU-To-PCI IDE Posting | : Enabled | F5 | : Old Values | (Shift)F2 |  |  | : Color |  |
| System BIOS Cacheable | : Disabled | F6 | : Load BIOS Defaults |  |  |  |  |  |
| Video RAM Cacheable | : Disabled | F7 | : Load Setup Defaults |  |  |  |  |  |

3.8	Power Management Setup

The Power Management Setup will appear on your screen like this:

ROM PCI/ISA BIOS

POWER MANAGEMENT SETUP

AWARD SOFTWARE, INC.

| Power Management | : Disable | ** Power Down & Resume Events ** |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PM Control by APM | : Yes | IRQ3 |  | (COM2) |  | : ON |  |  |  |
| Video Off Method | : V/H SYNC+ Blank | IRQ4 |  | (COM1) |  | : ON |  |  |  |
| MODEM Use IRQ | : 3 | IRQ5 |  | (LPT2) |  | : ON |  |  |  |
|  |  | IRQ6 |  | (Floppy Disk) |  | : ON |  |  |  |
| Doze Mode | : Disable | IRQ7 |  | (LPT1) |  | : ON |  |  |  |
| Standby Mode | : Disable | IRQ8 |  | (RTC Alarm) |  | : OFF |  |  |  |
| Suspend Mode | : Disable | IRQ9 |  | (IRQ2 Redir) |  | : ON |  |  |  |
| HDD Power Down | : Disable | IRQ10 |  | (Reserved) |  | : ON |  |  |  |
|  |  | IRQ11 |  | (Reserved) |  | : ON |  |  |  |
| ** Wake Up Events In Doze & Standby ** |  | IRQ12 |  | (PS/2 Mouse) |  | : ON |  |  |  |
| IRQ3 (Wake-up Event) | : ON | IRQ13 |  | (Coprocessor) |  | : ON |  |  |  |
| IRQ4 (Wake-up Event) | : ON | IRQ14 |  | (Hard Disk) |  | : ON |  |  |  |
| IRQ8 (Wake-up Event) | : OFF | IRQ15 |  | (Reserved) |  | : ON |  |  |  |
| IRQ12 (Wake-up Event) | : ON |  |  |  |  |  |  |  |  |
|  |  | ESC | : Quit |  | ¯®¬ |  |  | : Select |  |
|  |  | F1 | : Help |  | PU / PD / + / - |  |  | : Modify |  |
|  |  | F5 | : Old Values |  | (Shift)F2 |  |  | : Color |  |
|  |  | F6 | : Load BIOS Defaults |  |  |  |  |  |  |
|  |  | F7 | : Load Setup Defaults |  |  |  |  |  |  |

- Power Management

This category determines how much power consumption for system is activated after selecting the below items.  Default value is Disable.  The following will tell you the options of each item and describe the meaning of each option.

| Item | Options | Description |
| --- | --- | --- |
| A. Power Management | 1. Disable | Global Power Management will be disabled |
|  | 2. Min Saving | Pre-defined timer values are used such that all timers MIN value |
|  | 3. Max Saving | Pre-defined timer values are used such that all timers are in their MAX value |
|  | 4. User Define | Users can configure their own power management |
| B. PM Control by APM | 1. No | System BIOS will ignore APM when power managing the system |
|  | 2. Yes | System BIOS will wait for APM’s prompt before it enter any PM mode e.g. DOZE, STANDBY or SUSPEND Note: If APM is installed, & if there is a task running, even the timer is time out, the APM will not prompt the BIOS to put the system into any power saving mode! |
|  |  | Note: - if APM is no installed, this option has no effect |
| C. Video Off Method | 1. Always On | System BIOS will never turn off the screen |
|  | 2. Suspend -> Off | Screen off when system is in SUSPEND mode |
|  | 3. Susp, Stby -> Off | Screen off when system is in STANDBY or SUSPEND mode |
|  | 4. All Modes -> Off | Screen off when system is in DOZE, STANDBY or SUSPEND mode |
|  |  | Note: The M/B makers are recommended to fix this item to (2) or (3) and hide it by using MODBIN Utility |

3.9	PNP/PCI Configuration Setup

You can manually configure the PCI Device’s IRQ. The following will tell you the options of each item and describe the meaning of each option.

ROM PCI/ISA BIOS

PNP/PCI CONFIGURATION SETUP

AWARD SOFTWARE, INC.

| Resources Controlled By |  | : Manual | PCI IRQ Actived BY |  |  | : Level |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reset Configuration Data |  | : Disabled | PCI IDE IRQ Map To |  |  | : PCI-AUTO |  |  |
|  |  |  | Primary IDE INT# |  |  | : A |  |  |
| IRQ-3 | assigned to | : Legacy ISA | Secondary IDE INT# |  |  | : B |  |  |
| IRQ-4 | assigned to | : Legacy ISA |  |  |  |  |  |  |
| IRQ-5 | assigned to | : PCI/ISA PnP | Used MEM base addr |  |  | : N/A |  |  |
| IRQ-7 | assigned to | : Legacy ISA | Used MEM Length |  |  | : 8k |  |  |
| IRQ-9 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-10 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-11 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-12 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-14 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| IRQ-15 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| DMA-0 | assigned to | : PCI/ISA PnP |  |  |  |  |  |  |
| DMA-1 | assigned to | : PCI/ISA PnP | ESC | : Quit | ¯®¬ |  |  | : Select Item |
| DMA-3 | assigned to | : PCI/ISA PnP | F1 | : Help | PU/PD/+/ - |  |  | : Modify |
| DMA-5 | assigned to | : PCI/ISA PnP | F5 | : Old Values | (Shift)F2 |  |  | : Color |
| DMA-6 | assigned to | : PCI/ISA PnP | F6 | : Load BIOS Defaults |  |  |  |  |
| DMA-7 | assigned to | : PCI/ISA PnP | F7 | : Load Setup Defaults |  |  |  |  |

3.10	Integrated Peripherals

ROM PCI/ISA BIOS

INTEGRATED PERIPHERALS

AWARD SOFTWARE, INC.

| IDE HDD Block MODE | : Enabled | USB Controller |  |  | : Disabled |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IDE Primary Master PIO | : Auto |  |  |  |  |  |  |
| IDE Primary Slave PIO | : Auto |  |  |  |  |  |  |
| IDE Secondary Master PIO | : Auto |  |  |  |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |  |  |  |
| On-Chip Primary PCI IDE | : Enabled |  |  |  |  |  |  |
| On-Chip Secondary PCI IDE | : Enabled |  |  |  |  |  |  |
| PCI Slot IDE 2nd Channel | : Enabled |  |  |  |  |  |  |
| Onboard FDD Controller | : Enabled |  |  |  |  |  |  |
| Onboard Serial Port 1 | : Auto |  |  |  |  |  |  |
| Onboard Serial Port 2 | : Auto |  |  |  |  |  |  |
| Onboard Parallel Port | : 378H |  |  |  |  |  |  |
| Onboard Parallel Mode | : Normal |  |  |  |  |  |  |
|  |  | ESC | : Quit | ¯®¬ |  |  | : Select |
|  |  | F1 | : Help | PU/PD/+/- |  |  | : Modify |
|  |  | F5 | : Old Values | (Shift)F2 |  |  | : Color |
|  |  | F6 | : Load BIOS Defaults |  |  |  |  |
|  |  | F7 | : Load Setup Defaults |  |  |  |  |

3.11	Password Setting

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP |  |  |  | INTEGRATED PERIPHERALS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BIOS FEATURES SETUP |  |  |  | SUPERVISOR PASSWORD |  |  |
| CHIPSET FEATURES SETUP |  |  |  | USER PASSWORD |  |  |
| POWER MANAGEMENT SETUP |  |  |  | IDE HDD AUTO DETECTION |  |  |
| PNP/PCI CONFIRURATION |  |  |  | HDD LOW LEVEL FORMAT |  |  |
| LOAD BIOS DEFAULT |  | Enter Password: |  |  |  | T SETUP |
| LOAD SETUP DEFAULTS |  |  | EXIT WITHOUT SAVING |  |  |  |
| Esc | : Quit |  |  | ¯®¬ | : Select Item |  |
| F10 | : Save & Exit Setup |  |  | (Shift)F2 | : Change Color |  |
| Change /set /Disable Password |  |  |  |  |  |  |

When you select this function, the following message will appear at the center of the screen to assist you in creating a password.

ENTER PASSWORD:

Type the password, up to eight characters, and press <Enter>.  The password typed now will overwrite any previously entered password from CMOS memory.  You will be asked to confirm the password.  Type the password again and press <Enter>.  You may also press <Esc> to abort the operation and enter no  password.

To disable password, just press <Enter> when you are prompted to enter password.  A message will confirm the password being disabled.  Once the password is disabled, you can enter Setup freely.

PASSWORD DISABLED.

If you select System at Security Option of BIOS Features Setup Menu, you will be prompted for the password entry when the system is rebooted or you try to enter Setup.  If you select Setup at Security Option of BIOS Features Setup Menu, you will be prompted only when you try to enter Setup.

3.12	IDE HDD Auto Detection

The Enhanced IDE features are included in all Award BIOS.  Below is a brief description of the features.  If your computer has an IDE hard disk, you can use this ability to detect its parameters and enter them into the Standard CMOS Setup automatically.

- BIOS setup will display all possible modes that supported by the HDD including NORMAL, LBA & LARGE.

- If HDD does not support LBA modes, no ‘LBA’option will be shown.

- If no of cylinders is less than or equal to 1024, no ‘LARGE’ option will be show.

- Users can select a mode which is appropriate for them.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| Select Primary Slave Option (N = Skip ) : N OPTIONS SIZE CYLS HEADS PRECOMP LANDZ SECTOR MODE 1(Y) 516 1120 16 65535 1119 59 NORMAL 2 516 524 32 0 1119 63 LBA 3 516 560 32 65535 1119 59 LARGE Note: Some OSes (like SCO-UNIX) must use “NORMAL “ for installation HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE Primary Master : Primary Slave : |  |  |
| --- | --- | --- |
| ESC: Skip |  |  |