Installing the Boot-ROM on the NET-700EC/ET/ECT Board (ODI).

-----------------------------------------------------------------

Let's assume that your NET-700EC/ET/ECT board is already installed

with a boot-ROM and you would like the board's I/O base address

configured at 0x300, Interrupt Assignment IRQ3 and in I/O mode

(NE-2000 compatible), and boot-ROM address 0xD800. You also want to

use the new configuration the next time you boot up your system.

(1). Run the SETMODE Configuration Utility. Insert the

"NET-700EC/ET/ECT Drivers and Utilities" diskette 1 into

driver A: (or B:) and type the following commands that appear

in bold face type:

A:\>CD SETMODE        <Enter>

A:\SETMODE>SETMODE    <Enter>

(2).  The SETMODE Configuration Utility will appear on the screen.

Choose the "Configure Currently Enabled Card" option or

simply press "F1".

(3).  The "Select Boards I/O port Address" screen will appear.

Press "F1" until the "0x300" port address appears, then press

<Enter>.

(4).  The "DFI NET-700 Change Configuration" screen will appear.

(5).  Use the arrow keys to choose the "I/O Base Address" option

and press <Enter> until the "0x300" port address appears on

the rightmost side of this option.

(6).  Move to the "Interrupt Assignment" option and press <Enter>

until "IRQ3" appears on the rightmost side of this option.

(7).  If you are using a connector other than the one shown on the

screen, choose the "Physical Media" option and press <Enter>

until the type of connector you are using appears on the

rightmost side of this option.

(8).  Use the arrow keys to move to the "Boot PROM" option and

ressp <Enter>. The "Boot PROM Select" screen will appear.

Choose the "Boot PROM with size 32K" option and press

<Enter>. The "Boot PROM Address" screen will appear. Now, use

the arrow keys to select the "Set Boot PROM Address to

0xd800." and press <enter>. The last line on the box will

display the address you have selected. Press <ESC> to return

to the "DFI NET-700 Change Configuration" screen.

(9).  Choose the "Update Registers and Save Configuration to

EEPROM" option, then press <Enter>

(10). Now choose "Exit" or press <ESC> to return to the "Main

Functions" screen.

(11). Choose the "Initialize Board and Perform Loopback test"

option and press <Enter>, or simply press "F5". Failure to

pass any of these tests will require you to check the

configuration again for any conflicts. If you pass the tests,

press <ESC> to return to the "Main Functions" screen.

(12). Choose the "Quit and Return to DOS" option and press <Enter>,

or simply press "F10", to leave the SETMODE utility and

return to DOS. The procedures mentioned above will configure

the hardware part of the NET-700EC/ET/ECT board. Now, let's

configure the software part of the NET-700EC/ET/ECT board.

(1).  Prepare a 360K bootable diskette. The diskette must contain

the following files:

COMMAND.COM

AUTOEXEC.BAT

NET.CFG

LSL.COM

RPLODI.COM

NET700.COM

IPXODI.COM

NETX.COM

Note : If you are using DOS v6.0:

1.   Add a statement in CONFIG.SYS as follows:

Content of CONFIG.SYS --

DEVICE=A:\SETVER.EXE

2.   The NETX.COM file which in the 360K bootable diskette

must be upgraded from DOS v5.0 to DOS v6.0 as follows:

SETVER NETX 5.00

3.   Then copy the SETVER.EXE and NETX.COM files into the

360K bootable diskette.

LSL.COM, RPLODI.COM, IPXODI.COM and NETX.COM can be obtained from

the Workstation Driver diskette in the Novell Client Software

package. Copy the NET700.COM from the ODI DOS directory of the

"NET-700EC/ET/ECT drivers and Utilities" diskette 1.

Content of the AUTOEXEC.BAT      Content of the NET.CFG

prompt $p$g                      LINK DRIVER NET700

LSL                                INT 3

RPLODI                             PORT 300

NET700                             FRAME ETHERNET_802.3

IPXODI

NET5 / NETX

F:

Note: The frame type must same between Server and Workstation of

installation.

(2).  Re-boot your system using the above diskette to login to the

server correctly. If it doesn't work, please review the

procedures again (hardware and software part) to make sure

that nothing was missed nor done incorrectly.

(3).  Once you were able to login to the server successfully, type

the following commands:

CD\LOGIN

DOSGEN                           ---  Will create an

NET$DOS.SYS image file

in SYS:\LOGIN directly.

RPLFIX NET$DOS.SYS               ---  Solve "Image file not

compatible if using DOS

v5.0/6.0" problem.

COPY A:AUTOEXEC.BAT \LOGIN       ---  Auto Execution bath

file for remote boot.

FLAG NET$DOS.SYS SRO             ---  Flag the image file to

Sareable, Read Only.

(4).  If you do not have the RPLFIX.COM file, please contact your

local Novell dealer.

(5).  You may now logout and re-boot your system. Once the message

"Boot from Network (Y or N)?" appears, press "Y" to remote

boot your DOS from your Server.

--END--