AMI EISA Configuration Utility - Version 1.4

Release Notes

November 13, 1991

This document outlines the new features that were added and

bugs that were fixed in Version 1.4 of AMI ECU.  The

previous version of AMI ECU was 1.3 which was released on

6/21/91.

A "Define ISA Board" feature was added.  This feature can be

used when a CFG file is not available for an ISA board and

the user wishes to avoid possible resource conflicts between

the ISA board and any of the EISA boards in the system.  An

ISA board definition contains the board's name,

manufacturer, board type (8/16 bit), and the resources that

are used by the ISA board (DMA, IRQ, I/O Ports, and

Memory/Address space).  ISA board definitions are all stored

in one database file and can be used in any configuration

after they are defined.

An  advanced options  screen has  been added  to the  manual

configuration menu.  This screen  provides advanced features

for experienced users.  There are four options available:

1.   Global  Resource  Map: Displays  a  list  of  currently

allocated resources (DMA, IRQ, I/O Ports, and Memory/Address

space) and the boards/functions that own them.

2.  Board Details: Displays information  on any board in the

current  configuration,  including  the  board's  name,  ID,

manufacturer, amperage requirements, etc.

3.  System  Details: Displays  information about  the system

that is  being configured, including  a list of  which slots

are  occupied, have  busmaster capability,  ISA8  / ISA16  /

EISA, etc.   The total amperage  and used amperage  are also

displayed.  When adding a board, a warning is generated

if the system's amperage limit is exceeded.

4.  Physical Board ID Map: Displays  IDs for boards that are

physically installed  in the  system.  This  screen is  also

available directly from the main menu.

Additional overlay  file support  has been  added.  The  ECU

supports AMI overlay file format.

Added  a "Move  Board" option  on  the manual  configuration

screen.  This  option allows  a board to  be moved  from one

slot to another during a configuration

session.

On some  VGA and  EGA cards,  the ECU  caused the  screen to

blink  on  and off.   This  problem  has been  corrected  in

version 1.4.

Version 1.4 maintains tight control  over the memory that is

used  for  buffered  file  I/O.   This  leaves  more  memory

available for loading CFG files.

Browsing  through scrollable  windows is  easier in  version

1.4.   PgUp and  PgDn may  now  be used  to move  scrollable

windows.   Cursor key  usage in  scrollable  windows with  a

lower menu bar was also improved.

The appearance of COMMENT and HELP text has been improved in

version 1.4.  AMI ECU now fully emulates the behavior of MCS

ECU when  text formatting escape  sequences are  embedded in

the text.  Long text lines no longer run off the edge of the

window, they are now word wrapped.

Version 1.4  has the  ability to  include CFG  text with  an

INCLUDE = "xxxxxxxx.CFG" statement.

In version 1.3 the "Board Details" page of the Display/Print

command  displayed  length  and skirt  information  for  the

system  board  (these  fields  are  meaningless  for  system

boards),  and the  READID field  for all  boards was  always

displayed as  "NO" regardless of  its actual  status.  These

problems have been fixed in version 1.4.

Version 1.4  displays switch and jumper  comments correctly,

version 1.3 did  not always display comments  when they were

available.

Version  1.3  programmed  all  IRQs  to  be  edge  triggered

regardless of what the CFG file contained.  This problem was

corrected in version 1.4.

In version 1.3,  if a subfunction contained  a FREE resource

group with a memory statement, the selection bytes generated

were  incorrect.   This  caused the  wrong  settings  to  be

restored when  loading an  old configuration.   This problem

was corrected in version 1.4.

Inside subfunctions, choice statements that had only an INIT

in their FREE resource group  caused problems when modifying

an old  configuration.  The  user's saved  choices were  not

restored properly.  This bug has been fixed in version 1.4.

Version 1.3 filled  the ECU_VERSION field in  EISA CMOS with

0.0.   Version 1.4  now fills  this field  with the  correct

version number.

If version 1.3  detected a serious error  during some stages

of configuration,  the utility would  exit to  DOS.  Version

1.4 handles  these errors in a  more friendly way;  an error

message is  displayed and then the  ECU returns to  the main

menu.

In version  1.3, when a  board with  a long name  string was

removed and  a board with a  short name string was  added in

the same  slot, some of the  first board's name  remained on

the screen.  This was corrected in version 1.4.

Version 1.3's critical error handler  did not work properly.

Version 1.4 now displays a  window with the choices "Abort",

"Retry", and "Fail" available to the user.

Version 1.3's CFG file parser did not close a CFG file if it

encountered an error in the file.  After using the ECU for a

while it  was possible to exceed  the "FILES=" limit  set in

config.sys, in  this state the ECU  could not read  any more

CFG files.  This problem has been corrected in version 1.4.

Version 1.3 may sometimes generate a "Reference to undefined

switch" error while parsing the system board CFG file.  This

is caused  by unitialized variables.   The problem  has been

corrected in version 1.4.

Version  1.3  displayed  switch/jumper  labels  and  factory

settings in  the order  indicated by  the REVERSE  statement

(this is  the method  dictated by the  EISA spec).   MCS ECU

uses the order of the bits in the LOC statement to determine

the display order of these fields.  Version 1.4 now emulates

MCS ECU's display of

switch/jumper settings.

The EISA  spec says  that only one  MEMORY statement  may be

present in a resource group.  MCS ECU allows multiple MEMORY

statements under  a resource group if  that group is  a FREE

type.  Version 1.4 emulates MCS ECU  and does not generate a

syntax error under these conditions.

Some  board manufacturers  have placed  more  than one  HELP

statement in their CFG files even though this is a violation

of the EISA  spec.  MCS ECU does not generate  an error when

it  finds multiple  HELP  statements,  but ignores  all  but

thelast one.  Version 1.4 now emulates MCS ECU's handling of

duplicate statements.