About the file :

This readme file contains Flash ROM installation procedures when

you are need to update the BIOS. You should be able to receive an

updating diskette from your dealer or retail store where you purchased

the P54C-IP motherboard when you file a defective BIOS complain.

###################################################################

#                          WARNING!                               #

# Flash EPROM provides better upgradeability and a convenient way #

# to update sytem BIOS. Please follow the next procedures "How to #

# update the BIOS" to do the correct setp. If your installation   #

# procedures is not completed or encounter unexpected error(s).   #

# The system will not boot anymore.                               #

###################################################################

Flash ROM :

It is a sure item when you purchased a P54C-IP motherboard

from any retail store or dealer(s). The purpose to have a flash ROM

installed in the P54C-IP motherboard is to allow user updates any

defective BIOS without useing EPROM WRITER MACHINE.

How to update the BIOS :

1. Check if you can update the BIOS - turn on your computer and you

will see the following message shown on the screen.

Award Modular BIOS V4.50G, An Energy Star Ally

Copyright (c) 1984-94, Award Software, Inc.

05/20/1995

Compare the date shown in the above message with the date of the new

BIOS file xx-xx-xx.BIN in the diskette. The new BIOS file should be

located in the same diskette as this readme file.

If the BIOS file( xx-xx-xx.BIN ) date is newer than the date shown in

the screen, you can continue the updating procedure. Please do not

update your BIOS with any other files or you may encounter unexpected

error(s) in your computer system.

2. Turn off your computer to adjust jumper setting :

Locate JP6 jumpers - refer to Figure 1-1(P54C-IP)

and section 2-2 "Connectors and Jumpers" in your P54C-IP user's

manual. Chanage the setting to

1-2 : close for SST FLASH EPROM (+5V) ,Default

2-3 : close for INTEL FLASH EPROM (+12V)

3. Turn on your computer and enter the DOS prompt. Please note that

you can not use any protect mode application or driver (such as

EMM386, QEMM386) when updating BIOS. Comment out those command in

the AUTOEXEC.BAT or CONFIG.SYS file and reboot your system again.

If you are useing MS-DOS 6.0 or later, you can do this by press [F8]

function key while the "Staring MS-DOS..." line is on the screen.

The MS-DOS will prompt you to confirm each line of the CONFIG.SYS

and AUTOEXEC.BAT files. The AWDFLASH.EXE file can not work under

protected mode.

4. Insert xx-xx-xx.BIN diskette into drive A or B and type :

A:\AWDFLASH               or    B:\AWDFLASH

A:\AWDFLASH xx-xx-xx.bin  or    B:\AWDFLASH xx-xx-xx.bin

the Falsh Memory Write Utility will ask for the filename of the

new BIOS. Enter "xx-xx-xx.BIN" and press [Enter] key. The utility

will automatically save the ROM BIOS code as "BIOS.OLD" in the

diskette when the utility give a message "Do You Want To Save BIOS

(Y/N)" and press [Y]. The "BIOS.OLD" file is a backup of your

original BIOS from the motherboard. The next message will ask

"Are you sure to program(Y/N)". If you press the [Y] key. The utility

will load the new code into Falsh ROM.

#################################################################

#                           NOTE :                              #

#  DO NOT power off computer when updating flash ROM. This step #

#  may takes about 10 seconds to complete. Repeat step 4 if any #

#  abnormal symptom is observed.                                #

#################################################################

5. When completed loading the new BIOS, you must power off or reboot the

system, and enter the CMOS Setup Utility before going any

further by pressing [DEL] key. In the Setup Utility, choose "LOAD

SETUP DEFAULTS" option and respond "Y(es)" to load the hardware

default value for proper initialization. If you have not LOAD SETUP

DEFAULTS. The mothboard's CMOS value was still the old BIOS.

7. Call your dealer if you have any problem when updating.

***********************       END OF README.DOC       **************************