# Warning

# DO NOT FLASH YOUR BIOS JUST BECAUSE YOU CAN!!!

# DO SO ONLY TO RECTIFY SOME ISSUES OR PROBLEMS

# MAKE DOUBLE SURE YOU ARE USING THE RIGHT BIOS

# INCORRECT USAGE OF THE WRONG BIOS IS NOT COVERED UNDER WARRANTY

# IF IN DOUBT ASK US!

# Please follow this procedure when upgrading the BIOS on the system board.

# 1) Create a bootable floppy disk:

# Place formatted floppy on drive “A”, and from your C:\ prompt type SYS A :, you'll get the following message SYSTEM FILES TRANSFERRED.

# 2) Unzip or decompress the upgrading files to this diskette.

# There should be at least a flash utility and a .bin file.

# 3) Take note of the .bin file name.

# 4) Boot the system up from the floppy.

# DO NOT BOOT FROM THE HARD DRIVE! This may kill the BIOS.

# 5) Run the flash utility or the .BAT file (if available).

# 6) Follow the directions when running the flash utility and type in the name of the .bin file (xxxx.bin) when prompted, where the X represents a letter or a number.

# 7) After you have done this, reboot the system, check for the new BIOS date and press Del to enter the CMOS and select Load Setup Defaults or Optimal Settings on newer Bios and run the IDE HDD Autodetect.  Save & Exit the new settings, and boot through normally. “DONE.”