How to Set the PCI-0646 for ATA PIO w/Flow Control Timing Modes

---------------------------------------------------------------

From the Small Form Factor Committee's proposals and ATA-2 (as of

September 1994), the minimum times in ns for PIO w/flow control modes are:

t1      t2      t2i     t0

addr    active  recover cycle

setup   pulse   pulse   time

(min)   (min)   (min)   (min)

Mode 0  70      165     *       600

Mode 1  50      125     *       383

Mode 2  30      100     *       240

Mode 3  30      80      70      180

Mode 4  25      70      25      120

X address valid

|_______             __________

DIOW-/DIOR-     |       \___________/          \_____

<--t1--> <---t2----> <---t2i--->

<----------t0--------->

NOTE:  t0 = t2 + t2i; however, the minimum values for t2 and t2i sum to

less than t0.  Therefore, t2 and/or t2i must be extended as necessary

to meet the minimum t0 requirement.

Below are the PCI-0646 register settings needed to achieve the various

PIO w/flow control modes at various local bus clock frequencies.  Note that

we have played it conservatively where the timings did not fit nicely into

integer multiples of the clock period.

20Mhz Local Bus -> 50 ns/clock

--------------------------------------------------------

|    REGISTER   | t1      t2      t2i     t0

| ARTTIM  DWRTIM| ns(clks)ns(clks)ns(clks)ns(clks)

--------------------------------------------------------

Mode 0| 40h     47h   |100(2)   200(4)  400(8)  600(12)

Mode 1| 40h     34h   |100(2)   150(3)  250(5)  400(8)

Mode 2| 40h     22h   |100(2)   100(2)  150(3)  250(5)

Mode 3| 40h     21h   |100(2)   100(2)  100(2)  200(4)

Mode 4| 40h     2fh   |100(2)   100(2)   50(1)  150(3)

min   | 40h     1fh   |100(2)    50(1)  1 5(1)  100(2)

--------------------------------------------------------

25Mhz Local Bus -> 40 ns/clock

--------------------------------------------------------

|    REGISTER   | t1      t2      t2i     t0

| ARTTIM  DWRTIM| ns(clks)ns(clks)ns(clks)ns(clks)

--------------------------------------------------------

Mode 0| 40h     59h   | 80(2)   200(5)  400(10) 600(15)

Mode 1| 40h     45h   | 80(2)   160(4)  240(6)  400(10)

Mode 2| 40h     32h   | 80(2)   120(3)  120(3)  240(6)

Mode 3| 40h     22h   | 80(2)    80(2)  120(3)  200(5)

Mode 4| 40h     2fh   | 80(2)    80(2)   40(1)  120(3)

min   | 40h     1fh   | 80(2)    40(1)   40(1)   80(2)

--------------------------------------------------------

33Mhz Local Bus -> 30 ns/clock

--------------------------------------------------------

|    REGISTER   | t1      t2      t2i     t0

| ARTTIM  DWRTIM| ns(clks)ns(clks)ns(clks)ns(clks)

--------------------------------------------------------

Mode 0| 80h     6dh   | 90(3)   180(6)  420(14) 600(20)

Mode 1| 40h     57h   | 60(2)   150(5)  240(8)  390(13)

Mode 2| 40h     43h   | 60(2)   120(4)  120(4)  240(8)

Mode 3| 40h     32h   | 60(2)    90(3)   90(3)  180(6)

Mode 4| 40h     3fh   | 60(2)    90(3)   30(1)  120(4)

min   | 40h     1fh   | 60(2)    30(1)   30(1)   60(2)

--------------------------------------------------------

40Mhz Local Bus -> 25 ns/clock

--------------------------------------------------------

|    REGISTER   | t1      t2      t2i     t0

| ARTTIM  DWRTIM| ns(clks)ns(clks)ns(clks)ns(clks)

--------------------------------------------------------

Mode 0| 80h     70h   | 75(3)   175(7)  425(17) 600(24)

Mode 1| 40h     5ah   | 50(2)   125(5)  275(11) 400(16)

Mode 2| 40h     45h   | 50(2)   100(4)  150(6)  250(10)

Mode 3| 40h     43h   | 50(2)   100(4)  100(4)  200(8)

Mode 4| 40h     31h   | 50(2)    75(3)   50(2)  125(5)

min   | 40h     1fh   | 50(2)    25(1)   25(1)   50(2)

--------------------------------------------------------

50Mhz Local Bus -> 20 ns/clock

--------------------------------------------------------

|    REGISTER   | t1      t2      t2i     t0

| ARTTIM  DWRTIM| ns(clks)ns(clks)ns(clks)ns(clks)

--------------------------------------------------------

Mode 0| 00h     d0h   | 80(4)   260(13) 340(17) 600(30)

Mode 1| 80h     7ch   | 60(3)   140(7)  260(13) 400(20)

Mode 2| 40h     56h   | 40(2)   100(5)  140(7)  240(12)

Mode 3| 40h     44h   | 40(2)    80(4)  100(5)  180(9)

Mode 4| 40h     41h   | 40(2)    80(4)   40(2)  140(7)

min   | 40h     1fh   | 40(2)    20(1)   20(1)   40(2)

--------------------------------------------------------

Example:  To set Mode 3 on the master drive on the primary IDE port on

a 33MHz local bus:

pci64x r 51 23  <- optional - specifies fastest host-side timing

pci64x r 52 32

pci64x r 53 40

pci64x r 54 32

or, simply

pci64x t 0 3

pci64x r 51 23  <- optional - specifies fastest host-side timing

Don't forget to set your drive for Mode 3 operation via:

pci64x d 0 3

How to Set the PCI-0646 for ATA Single Word DMA Timing Modes

---------------------------------------------------------------

From the Small Form Factor Committee's proposals and ATA-2 (as of

September 1994), the minimum times in ns for Single Word DMA modes are:

t2      t2i     t0

active  recover cycle

pulse   pulse   time

(min)   (min)   (min)

Mode 0  480     *       960

Mode 1  240     *       480

Mode 2  120     *       240

Below are the PCI-0646 register settings needed to achieve the various

Single Word DMA modes at various local bus clock frequencies.  Note that

we have played it conservatively where the timings did not fit nicely into

integer multiples of the clock period.

20Mhz Local Bus -> 50 ns/clock

--------------------------------------------------------

| REGISTER| t2       t2i      t0

| DWRTIM  | ns(clks) ns(clks) ns(clks)

--------------------------------------------------------

Mode 0| a9h     | 500(10)  500(10)  1000(20)

Mode 1| 54h     | 250(5)   250(5)    500(10)

Mode 2| 31h     | 150(3)   100(2)    250(5)

min   | 1fh     |  50(1)    50(1)    100(2)

--------------------------------------------------------

25Mhz Local Bus -> 40 ns/clock

--------------------------------------------------------

| REGISTER| t2       t2i      t0

| DWRTIM  | ns(clks) ns(clks) ns(clks)

--------------------------------------------------------

Mode 0| cbh     | 480(12)  480(12)   960(24)

Mode 1| 65h     | 240(6)   240(6)    480(12)

Mode 2| 32h     | 120(3)   120(3)    240(6)

min   | 1fh     |  40(1)    40(1)     80(2)

--------------------------------------------------------

33Mhz Local Bus -> 30 ns/clock

--------------------------------------------------------

| REGISTER| t2       t2i      t0

| DWRTIM  | ns(clks) ns(clks) ns(clks)

--------------------------------------------------------

Mode 0| 00h     | 480(16)  480(16)   960(32)

Mode 1| 87h     | 240(8)   240(8)    480(16)

Mode 2| 43h     | 120(4)   120(4)    240(8)

min   | 1fh     |  30(1)    30(1)     60(2)

--------------------------------------------------------

40Mhz Local Bus -> 25 ns/clock

--------------------------------------------------------

| REGISTER| t2       t2i      t0

| DWRTIM  | ns(clks) ns(clks) ns(clks)

--------------------------------------------------------

Mode 0|         | not supported

Mode 1| a9h     | 250(10)  250(10)   500(20)

Mode 2| 54h     | 125(5)   125(5)    250(10)

min   | 1fh     |  25(1)    25(1)     50(2)

--------------------------------------------------------

50Mhz Local Bus -> 20 ns/clock

--------------------------------------------------------

| REGISTER| t2       t2i      t0

| DWRTIM  | ns(clks) ns(clks) ns(clks)

--------------------------------------------------------

Mode 0|         | not supported

Mode 1| cbh     | 240(12)  240(12)   480(24)

Mode 2| 65h     | 120(6)   120(6)    240(12)

min   | 1fh     |  20(1)    20(1)     40(2)

--------------------------------------------------------

How to Set the PCI-0646 for ATA Multiword DMA Timing Modes

---------------------------------------------------------------

From the Small Form Factor Committee's proposals and ATA-2 (as of

September 1994), the minimum times in ns for Multiword DMA modes are:

t2      t2i     t0

active  recover cycle

pulse   pulse   time

(min)   (min)   (min)

Mode 0  215     *       480

Mode 1   80     *       150

Mode 2   70     *       120

Below are the PCI-0646 register settings needed to achieve the various

Multiword DMA modes at various local bus clock frequencies.  Note that

we have played it conservatively where the timings did not fit nicely into

integer multiples of the clock period.

20Mhz Local Bus -> 50 ns/clock

--------------------------------------------------------

| REGISTER| t2       t2i      t0

| DWRTIM  | ns(clks) ns(clks) ns(clks)

--------------------------------------------------------

Mode 0| 54h     | 250(5)   250(5)    500(10)

Mode 1| 2fh     | 100(2)    50(1)    150(3)

Mode 2| 2fh     | 100(2)    50(1)    150(3)

min   | 1fh     |  50(1)    50(1)    100(2)

--------------------------------------------------------

25Mhz Local Bus -> 40 ns/clock

--------------------------------------------------------

| REGISTER| t2       t2i      t0

| DWRTIM  | ns(clks) ns(clks) ns(clks)

--------------------------------------------------------

Mode 0| 65h     | 240(6)   240(6)    480(12)

Mode 1| 21h     |  80(2)    80(2)    160(4)

Mode 2| 2fh     |  80(2)    40(1)    120(3)

min   | 1fh     |  40(1)    40(1)     80(2)

--------------------------------------------------------

33Mhz Local Bus -> 30 ns/clock

--------------------------------------------------------

| REGISTER| t2       t2i      t0

| DWRTIM  | ns(clks) ns(clks) ns(clks)

--------------------------------------------------------

Mode 0| 87h     | 240(8)   240(8)    480(16)

Mode 1| 31h     |  90(3)    60(2)    150(5)

Mode 2| 3fh     |  90(3)    30(1)    120(4)

min   | 1fh     |  30(1)    30(1)     60(2)

--------------------------------------------------------

40Mhz Local Bus -> 25 ns/clock

--------------------------------------------------------

| REGISTER| t2       t2i      t0

| DWRTIM  | ns(clks) ns(clks) ns(clks)

--------------------------------------------------------

Mode 0| 9ah     | 225(9)   275(11)   500(20)

Mode 1| 41h     | 100(4)    50(2)    150(6)

Mode 2| 31h     |  75(3)    50(2)    125(5)

min   | 1fh     |  25(1)    25(1)     50(2)

--------------------------------------------------------

50Mhz Local Bus -> 20 ns/clock

--------------------------------------------------------

| REGISTER| t2       t2i      t0

| DWRTIM  | ns(clks) ns(clks) ns(clks)

--------------------------------------------------------

Mode 0| bch     | 220(11)  260(13)   480(24)

Mode 1| 43h     |  80(4)    80(4)    160(8)

Mode 2| 41h     |  80(4)    40(2)    120(6)

min   | 1fh     |  20(1)    20(1)     40(2)

--------------------------------------------------------