Board:	LPM30

Part #:	09-00232-xx

·	LPX form factor system board.

·	Micronics’ X30 chipset with an on-chip external level 2 cache controller.

·	One VL slot built onto the system board.  Slot will accommodate a riser card which supports up to one VESA and four ISA peripheral cards.

·	On-board VESA Localbus video adapter, using the 64-bit ATI mach64 graphics accelerator with 1MB of VRAM (2MB optional).

·	On-board 16-bit sound controller.

·	On-board feature connectors accept:

·	14,400 Fax modem module (optional)

·	Ethernet network adapter module (optional)

·	3.3V and 4.0V power modules for low voltage CPU’s

·	Accepts up to 256K of external cache.

·	On-board Localbus IDE controller.

·	Floppy controller (supports 2.88MB, 1.44MB, 1.2MB, 720K, and 360K floppy drive).

·	Two high speed 16550 compatible serial ports.

·	Bi-directional parallel port which is EPP and ECP compatible.

·	Battery-backed real-time clock.

·	Supports the following processors in the CPU ZIF socket:

·	486SX2, 50MHz

·	486DX, 33MHz

·	486DX2, 50 or 66MHz

·	486DX4, 75 or 100 MHz

·	Cyrix DX2/80 or DX/40

·	AMD DX40, DX2/80, or DX4/100

CPU Voltage

µ §

| CPU | Manufacture(s) | Voltage |
| --- | --- | --- |
| 486SX | Intel, AMD | 5.0V |
| 486DX | Intel, AMD | 5.0V |
| 486DX2 | Intel | 5.0V |
| 486DX2 | AMD | 3.3V or 5.0V |
| 486SL-series | Intel | 5.0V |
| Write-Back Enhanced 486DX2 | Intel | 5.0V |
| 486DX4 | Intel, AMD | 3.3V |
| Pentium Overdrive | Intel | 5.0V |
| Cyrix 486DX/DX2 | Cyrix | 5.0V |
| Cyrix 486DX2V-80 | Cyrix | 4.0V |

Jumper Settings:

| CPU | W22 | W23 | W30 | W31 | W32 | W33 | W34 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 486SX | 2-3 | 1-2 | 2-3 | 2-3 | 2-3 | open | open |
| 486DX/DX2 | 2-3 | 1-2 | 1-2 & 3-4 | 2-3 | 2-3 | open | open |
| 486SL-series | 2-3 | 1-2 | 1-2 & 3-4 | 2-3 | 2-3 | open | open |
| Intel Write-Back Enhanced 486DX2 | 1-2 | 1-2 | 1-2 & 3-4 | 2-3 | 1-2 | close | 1-2 |
| 486DX4 | 2-3 | 1-2 | 1-2 & 3-4 | 2-3 | 2-3 | open | open |
| Pentium Overdrive | 1-2 | 1-2 | 1-2 & 3-4 | 2-3 | 1-2 | open | open |
| AMD 486DX/DX2 | 2-3 | 1-2 | 1-2 & 3-4 | 2-3 | 2-3 | open | open |
| AMD DXL4 | 2-3 | 1-2 | 1-2 & 3-4 | open | 2-3 | open | 2-3 |
| Cyrix 486DX/DX2 | 1-2 | 2-3 | 1-2 & 3-4 | 1-2 | 2-3 | open | 1-2 |

| CPU | W35 | W38 | W40 | W41 | W43 | W44 | W45 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 486SX | open | 3-4 | 4-5 | 3-4 | 1-2 | open | open |
| 486DX/DX2 | open | 3-4 | 4-5 | 3-4 | 1-2 | open | open |
| 486SL-series | 1-2 | 3-4 | 4-5 | 3-4 | 1-2 | open | open |
| Intel Write-Back Enhanced 486DX2 | 2-3 | 3-4 | 4-5 | 1-2 & 3-4 | 1-2 | open | open |
| 486DX4 | open | 3-4 | 4-5 | 3-4 | 1-2 | open | open |
| Pentium Overdrive | open | 2-3 | 1-2 | 3-4 | 1-2 | open | open |
| AMD 486DX/DX2 | open | 1-2 & 3-4 | 4-5 | open | 1-2 | close | 1-2 |
| AMD DXL4 | open | 1-2 & 3-4 | 4-5 | open | 1-2 | close | open |
| Cyrix 486DX/DX2 | open | 3-4 | 2-3 | 2-3 | 2-3 | open | open |

Note:  If you are using a 3.3V or 4.0V processor, you will need to install a special Power Module.

CPU Speed

| Jumper | 25Mhz | 33Mhz (default) | 40Mhz |
| --- | --- | --- | --- |
| W26 | open | open | close |
| W27 | open | close | open |

External Cache

| Jumper | Function | Setting |
| --- | --- | --- |
| W39 | 128K 256K | 1-2 & 3-4 2-3 & 4-5 |
| W42 | 128K 256K | open close |

IDE Controller

| Jumper | Function | Setting |
| --- | --- | --- |
| W28 | Enable Disable | 1-2 2-3 |

I/O Controller

| Jumper | Function | Setting |
| --- | --- | --- |
| W11 | Enable Disable | open close |

Game Port

| Jumper | Function | Setting |
| --- | --- | --- |
| W2 | Enable Disable | close open |

Parallel Port IRQ Selection

| Jumper | Function | Setting |
| --- | --- | --- |
| W5 | IRQ7 IRQ5 | 1-2 2-3 |

ECP/EPP Mode DMA Selection

| Jumper | none (default) | DMA1 | DMA3 |
| --- | --- | --- | --- |
| W6 | open | 1-2 | 2-3 |
| W10 | open | 1-2 | 2-3 |

Video Selection

| Jumper | Function | Setting |
| --- | --- | --- |
| W1 | Color Mono | close open |

VGA Controller Selection

| Jumper | Function | Setting |
| --- | --- | --- |
| W14 | On-board Enable Disable | open close |

Enabling the VGA Feature Connector

| Jumper | Function | Setting |
| --- | --- | --- |
| W15 | Enable Disable | open close |

On-Board Sound Controller

| Jumper | Function | Setting |
| --- | --- | --- |
| W4 | Enable Disable | open close |

Sound Controller Address Selection

| Jumper | Function | Setting |
| --- | --- | --- |
| W3 | Address 240 Address 220 | 1-2 2-3 |

Sound Controller IRQ Selection

| Jumper | Function | Setting |
| --- | --- | --- |
| W7 | IRQ5 (default) IRQ9 | 1-2 2-3 |

Sound Controller DQR Selection

| Jumper | DRQ0 | DRQ1 | DRQ3 | None |
| --- | --- | --- | --- | --- |
| W8 | 1-2 | 2-3 | 1-2 | 2-3 |
| W9 | 2-3 | 1-2 | 1-2 | 2-3 |

Buzzer Selection

| Jumper | Function | Setting |
| --- | --- | --- |
| W20 | Enable Disable | close open |

BIOS Operation Selection

| Jumper | Function | Setting |
| --- | --- | --- |
| W18 | Write protected (default) Programmable | open close |

CMOS Reset Jumper

| Jumper | Function | Setting |
| --- | --- | --- |
| W25 | Normal (default) Reset CMOS | 1-2 2-3 |

Reserved Jumpers

| Jumper | Function | Setting |
| --- | --- | --- |
| W13 | Reserved | 1-2 |
| W17 | Reserved | open |
| W19 | Reserved | 1-2 & 3-4 |
| W21 | Standard 5V DRAM Micron 12V DRAM | open close |
| W29 | Reserved | 2-3 |

Case and Peripheral Connections

| Jumper | Function | Setting |
| --- | --- | --- |
| J15 | VL IDE Connector | Primary |
| J16 | ISA IDE Connector | Secondary |
| J17 | Floppy Connector |  |
| J2 | VGA Connector |  |
| J12 | Parallel Port Connector | Can be disabled in CMOS |
| J5 | Serial Port (Com1) | Can be disabled in CMOS |
| J7 | Serial Port (Com2) | Can be disabled in CMOS |
| J1 | Game Port |  |
| J4 | PS/2 Keyboard |  |
| J3 | PS/2 Mouse |  |
| J10 | LAN Connector | Contact Micronics to order |
| J11 | Modem Connector | Contact Micronics to order |
| J14 & J18 | Power Supply Connectors |  |
| SPKR | Speaker | Sound Controller |
| Line Out | Line Level Output | Sound Controller |
| Line In | Line Level Input | Sound Controller |
| MIC In | Microphone Input | Sound Controller |
| J8 | Audio Connector | Connect to CD-ROM |
| J20 | Speaker Connector | 1-Speaker; 2-N/C; 3-Ground; 4-5V DC |
| J22 | Turbo LED | 1-5V DC; 2-Ground |
| J23 | Turbo Switch |  |
| J24 | Reset |  |
| W36 | HD LED |  |
| J19 | Keylock/ Power LED | 1-Power; 2-N/C; 3-Ground; 4-5V DC |
| J21 | Power Module Connector |  |