## LPX30WB

·	Processor:

·	Single socket for 80486 CPUs.

·	Supports Intel 486 SX, DX, DX2, and DX4 processors, AMD 486 DX, DX2, and DX4 processors, and Cyrix 486 DX and DX2 processors

·	P24T (Intel Pentium OverDrive) processors require Interposer kit from Intel

·	3.3V and 4.0V processors require a Power Module

·	Expansion:

·	Riser card accomodating up to one VLB and four ISA slots

·	Chipset:

·	Micronics X30 chipset ASIC

·	Winbond 787 Super I/O

·	Memory (parity and non-parity):

·	Four 32/36-bit 72-pin SIMM sockets, acoomodating 60ns or 70ns SIMMs.

·	Supports 1M x 32/36 (4MB), 2M x 32/36 (8MB), 4M x 32/36 (16MB), and 8M x 32/36 (32MB).

·	Maximum memory is 128MB.

·	Parity and non-parity memory cannot be mixed.

·	Cache:

·	0/128/256K Level 2 cache.

·	PCI IDE:

·	Two IDE connectors on-board

·	Primary (VL) IDE supports Mode 3 and LBA.

·	Secondary (ISA) IDE supports Mode 0 and LBA.

·	Floppy:

·	Supports 360K, 720K, 1.2, 1.44  floppy drives.

·	Serial / Parallel ports:

·	Two on-board 16550 compatible serial ports.

·	One on-board parallel port with ECP and EPP support.

·	BIOS:

·	Phoenix BIOS on 1MB Flash chip.

·	Keyboard:

·	PS/2 style keyboard and mouse connectors.

·	Form factor:

·	Baby AT footprint (8.5” x 9.5”)

LPX30WB Jumper Settings

CPU speed:

| CPU Speed | 486SX | 486DX/ DX2 | DX2 SL Enhanced | Write Back DX2 | DX4 | AMD DXL2 | Cyrix DX/DX2 | P24T |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W11 | 2-3 | 1-2 , 3-4 | 1-2 , 3-4 | 1-2 , 3-4 | 1-2 , 3-4 | 1-2 , 3-4 | 1-2 , 3-4 | 1-2 , 3-4 |
| W12 | open | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 3-4 | 2-3 |
| W18 | open | open | open | open | open | open | open | open |
| W19 | 2-3 | 2-3 | 2-3 | 1-2 | 2-3 | 2-3 | 2-3 | 1-2 |
| W20 | open | open | 2-3 | 2-3 | 2-3 | open | 1-2 | 2-3 |
| W21 | open | open | open | closed | open | open | open | open |
| W22 | open | open | open | 2-3 | open | open | open | open |
| W23 | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 | 2-3 | 1-2 |
| W24 | 2-3 | 2-3 | 2-3 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 |
| W25 | open | open | open | 1-2 | open | open | 2-3 | 1-2 |
| W26 | open | open | 4-5 | 4-5 | 4-5 | open | 2-3 | 1-2 |
| W27 | open | open | open | 1-2 | open | open | 2-3 | 1-2 |
| W28 | open | open | 3-4 | 1-2 , 3-4 | 3-4 | open | 2-3 | 3-4 |

CPU speed:

|  | 25 MHz | 33 MHz | 40 MHz |
| --- | --- | --- | --- |
| W9 | open | close | open |
| W10 | open | close | close |

Cache Size Selection:

|  | No Cache | 128K | 256K |
| --- | --- | --- | --- |
| W13 | open | 1-2 | 2-3 |
| W14 | open | 1-2 | 2-3 |
| W15 | open | 1-2 | 2-3 |
| W17 | open | 1-2 | 2-3 |

VL IDE Interface:

|  | Function | Setting |
| --- | --- | --- |
| # W7 | Enable VL IDE (default) Disable VL IDE | 1-2 2-3 |
| # W33 | VL IDE at IRQ 14 (default) VL IDE at IRQ 15 | 1-2 2-3 |

ISA IDE Interface:

|  | Function | Setting |
| --- | --- | --- |
| # W8 | Enable ISA IDE (default) Disable ISA IDE | 2-3 1-2 |
| W32 | ISA IDE at IRQ 14 (default) ISA IDE at IRQ 15 | 2-3 1-2 |

Video Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W2 | Color (default) Mono | closed open |

On-board VGA:

| Jumper | Enabled | Disabled |
| --- | --- | --- |
| # W34 | 2-3 | 1-2 |
| W39 | 2-3 | 1-2 |

On-board VGA IRQ 9:

| Jumper | Enabled | Disabled |
| --- | --- | --- |
| # W34 | close | open |

BIOS Operation Selection:

| Jumper | Function | Setting |
| --- | --- | --- |
| W4 | Flash BIOS write-protected (default) Flash BIOS programmable | open closed |

BIOS Reset Jumper:

| Jumper | Function | Setting |
| --- | --- | --- |
| W5 | Normal operation (default) Clear CMOS RAM settings | 1-2 2-3 |