# Appendix 1: Quick Jumper Setting Reference

##### JP1A: Audio enable/disable jumper

Use this jumper to enable or disable the audio system integrated on the mainboard.

| Function | Jumper Cap |
| --- | --- |
| 1 2 Enable audio JP1A | Open pins 1-2 |
| Disable audio | Short pins 1-2 |

##### JP1B: Modem enable/disable jumper

Use this jumper to enable or disable the modem integrated on the mainboard.

| 1 2 Function | Jumper Cap |
| --- | --- |
| Enable modem JP1B | Open pins 1-2 |
| Disable modem | Short pins 1-2 |

Note: If you use jumper JP1A to disable the audio system, the modem is automatically disabled as well, even if JP1B is set to enabled.

##### JP2: Keyboard power on jumper

Use this 3-pin jumper to enable keyboard power on with hot keys or password.

3
2
1

| Function JP2 | Jumper Cap |
| --- | --- |
| Disable keyboard power on | Short pins 1-2 |
| Enable keyboard power on | Short pins 2-3 |

##### JP4: Select Slot1 or Socket-370 jumper

Use this jumper to select if you are installing a processor into the Slot1 or a processor into the socket-370.

| 1 2 3 JP4 Function | Jumper Cap |
| --- | --- |
| Select Slot1 | Short Pins 1-2 |
| Select socket-370 | Short pins 2-3 |

##### JP5: Clear CMOS memory jumper

Use this 3-pin jumper to clear all the current data stored in the CMOS memory.

| Function 1 2 3 | Jumper Cap |
| --- | --- |
| Normal operation JP5 | Short pins 1-2 |
| Clear CMOS | Short pins 2-3 |

##### JP6: Select system bus frequency jumper

When this jumper is open, it forces the mainboard to use a 133 MHz system bus, even if the processor requires a 66/100 MHz bus.

| Function 1 2 JP6 | Jumper Cap |
| --- | --- |
| Normal operation (66/100 MHz) | Short pins 1-2 |
| Force 133 MHz | Open pins 1-2 |

Note:.If the chipset is 82C693A, the board supports up 133 MHz FSB. If the chipset is 82C693, it supports only 66/1000 MHz FSB, and the pins of JP6 should be shorted

##### JP7: Select FC-PGA Coppermine jumper

Use these 3-pin jumper sets to set the socket-370 on the board for use with a FC-PGA Coppermine processor.

| 1 2 3 JP7 Function | Jumper Cap |
| --- | --- |
| FC-PGA Coppermine | Short pins 1-2 |
| Normal operation (other processors) | Short pins 2-3 |

##### J10: Panel Connector

The mainboard J10 panel connector has a standard set of switch and indicator connectors that are commonly found on ATX/AT system cases. Use the illustration below to make the correct connections to the case switches and indicators.

| Function | Pins |
| --- | --- |
| Speaker | 1, 3, 5, +7 |
| 21 22 Power LED Pins 2-4-6 Speaker Pins 1-3-5-7 HDD LED Pins 15-16 1 2 Reset Switch Pins 17-18 Power/Suspend Switch Pins 21-22 Suspend LED Pins 19-20 Turbo LED Pins 13-14 Keylock Pins 8-10 Power Indicator | +2, +4, 6 |
| Keylock | +8, 10 |
| Turbo Indicator | +13, 14 |
| Hard Disk Indicator | +15, 16 |
| Reset Switch | 17, 18 |
| Suspend Switch | 19, 20 |
| Power Switch | 21, 22 |